3GPP TSG RAN WG1 #110			     R1-2206537

Toulouse, France, August 22nd – August 26th, 2022

Source:	Intel Corporation

Title:	Discussion on Channel Access Indication within Fall-back and RAR UL Grant DCIs

Agenda item:	8.2

Document for:	Discussion and Decision

# Introduction

In this contribution, in the context of channel access to extend NR up to 71 GHz and based on the agreements and conclusions made during the previous RAN1 meetings along this WI, we discuss the channel access indication within the Fall-back DCIs, where the change requests are provided in [2] and [3].

# Channel Access Indication within Fall-back and RAR UL Grant DCIs

In RAN1 #107 meeting [1], it was agreed that for FR2-2 the field ChannelAccess-CPext carried in the non-fallback DCIs is reinterpreted to carry information indicating which channel access type to use. In particular, the following was agreed:

Agreement For Non-Fallback DCI formats, for FR2-2 operation, for the configuration of the ChannelAccess-CPext field in DCI to indicate the channel access type only, new tables are introduced indicating channel access types for FR2-2, with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

However, no consensus was reach regarding the details of the channel access indication for both the fallback DCI format 0-0 and 1-0, and the RAR UL grant, and as it can be observed in Sec. 7.3.1.1.1, and 7.3.1.2.1 of TS 38.212, and Sec. 8.2 and 8.2A of TS 38.213, these DCIs cannot not be currently used for FR2-2 in unlicensed band, since the gNB won’t be able to indicate to the UE the channel access type that should be applied, while the UE would be unaware of this information and whether it may operate within or outside of gNB’s COT, and more importantly compliance with the regional regulatory requirements won’t be always guaranteed.

In this matter, indication of channel access type to use is required in these DCIs, and in even though type 2 channel access mechanism may be only supported based on the UE’s capability, the gNB should be still allowed to configure/indicate this channel access type when supported. Therefore, for fallback DCI formats 0_0/1_0 and RAR UL grant, similarly as in unlicensed operation for FR1, for FR2-2 operation the field ChannelAccess-Cpext should be also carried, and be composed by two bits, allowing to signal type 1, type 2 or type 3 channel access mechanism.

Proposal 1: For fallback DCI formats 0_0/1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field indicates one of the entries of a table which entries are “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

- Endorse the draft CR for TS38.212 in R1-2206538 and draft CR for TS38.213 in R1-2206539

The following are text proposal for changes required to update the channel access field presence for FR2-2.

## TS38.213

## 8.2 Random access response - Type-1 random access procedure <Unchanged parts omitted> The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213] as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if ChannelAccessMode-r16 = "semistatic" is provided for frequency range 1. For operation in a cell in frequency range 2-2 if LBT-mode is provided and indicates that channel access procedures would need to be performed [15, TS 37.213], the ChannelAccess-CPext field indicates a channel access type as defined in Table 7.3.1.1.1-4B in TS 38.212. Table 8.2-1: Random Access Response Grant Content field size RAR grant field; Number of bits. Frequency hopping flag; 1. PUSCH frequency resource allocation; 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access. PUSCH time resource allocation; 4. MCS; 4. TPC command for PUSCH; 3. CSI request; 1. ChannelAccess-CPext; 0, for operation without shared spectrum channel access, or for operation with shared spectrum access in frequency range 2-2 when LBT-mode indicates that channel access procedures would not need to be performed or LBT-mode is not provided, 2, for operation with shared spectrum channel access in frequency range 1, or in frequency range 2-2 when LBT-mode is provided and indicates that channel access procedures would need to be performed.. <Unchanged parts omitted> ## 8.2A Random access response - Type-2 random access procedure <Unchanged parts omitted> - for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if ChannelAccessMode-r16 = "semistatic" is provided for frequency range 1. For operation in a cell in frequency range 2-2 if LBT-mode is provided and indicates that channel access procedures would need to be performed [15, TS 37.213], the ChannelAccess-CPext field indicates a channel access type as defined in Table 7.3.1.1.1-4B in TS 38.212. - the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission <Unchanged parts omitted>

TS38.212

##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: <Unchanged parts omitted> - ChannelAccess-CPext –2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bit otherwise. <Unchanged parts omitted> The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: <Unchanged parts omitted> - ChannelAccess-CPext –2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bit otherwise. <Unchanged parts omitted> Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 in frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided in frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0. 3; -; -. Table 7.3.1.1.1-4B: Channel access type in frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. 2; Type 3 channel access defined in clause 4.4.3 of 37.213. 3; Reserved. <Unchanged parts omitted> ##### 7.3.1.2.1 Format 1_0 DCI format 1_0 is used for the scheduling of PDSCH in one DL cell. The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: <Unchanged parts omitted> - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bits otherwise <Unchanged parts omitted> The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI: <Unchanged parts omitted> - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bits otherwise <Unchanged parts omitted>

# Conclusions

In this contribution, we derived the following proposal:

Proposal 1: For fallback DCI formats 0_0/1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field indicates one of the entries of a table which entries are “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

- Endorse the draft CR for TS38.212 in R1-2206538 and draft CR for TS38.213 in R1-2206539

# References

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #107 e-Meeting, November, 2021.

- R1-2206539, “[draft CR] correction on support of channel access indication within the UL RAR grant”, Intel Corporation, August 2022.

- R1-2206538, “[draft CR] correction on support of channel access indication within the fall-back DCIs”, Intel Corporation, August 2022.