3GPP TSG RAN WG1 #110	                                     R1-2206879

Toulouse, France, August 22nd – 26th, 2022

Agenda item:		9.2.4.2

Source:		LG Electronics

Title:			Other aspects on AI/ML for positioning accuracy enhancement

Document for:	Discussion and Decision

Introduction

In RAN1#109e meeting, the agreements on sub use cases and potential specification impact of AI/ML for positioning were made as follows.

Agreement Study further on sub use cases and potential specification impact of AI/ML for positioning accuracy enhancement considering various identified collaboration levels. - Companies are encouraged to identify positioning specific aspects on collaboration levels if any in agenda 9.2.4.2. - Note1: terminology, notation and common framework of Network-UE collaboration levels are to be discussed in agenda 9.2.1 and expected to be applicable to AI/ML for positioning accuracy enhancement. - Note2: not every collaboration level may be applicable to an AI/ML approach for a sub use case Agreement For further study, at least the following aspects of AI/ML for positioning accuracy enhancement are considered. - Direct AI/ML positioning: the output of AI/ML model inference is UE location - E.g., fingerprinting based on channel observation as the input of AI/ML model - FFS the details of channel observation as the input of AI/ML model, e.g. CIR, RSRP and/or other types of channel observation - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - AI/ML assisted positioning: the output of AI/ML model inference is new measurement and/or enhancement of existing measurement - E.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement - FFS the details of input and output for corresponding AI/ML model(s) - FFS: applicable scenario(s) and AI/ML model generalization aspect(s) - Companies are encouraged to clarify all details/aspects of their proposed AI/ML approaches/sub use case(s) of AI/ML for positioning accuracy enhancement Agreement Companies are encouraged to study and provide inputs on potential specification impact at least for the following aspects of AI/ML approaches for sub use cases of AI/ML for positioning accuracy enhancement. - AI/ML model training - training data type/size - training data source determination (e.g., UE/PRU/TRP) - assistance signalling and procedure for training data collection - AI/ML model indication/configuration - assistance signalling and procedure (e.g., for model configuration, model activation/deactivation, model recovery/termination, model selection) - AI/ML model monitoring and update - assistance signalling and procedure (e.g., for model performance monitoring, model update/tuning) - AI/ML model inference input - report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) - model input acquisition and pre-processing - type/definition of model input - AI/ML model inference output - report/feedback of model inference output - post-processing of model inference output - UE capability for AI/ML model(s) (e.g., for model training, model inference and model monitoring) - Other aspects are not precluded - Note: not all aspects may apply to an AI/ML approach in a sub use case - Note2: the definitions of common AI/ML model terminologies are to be discussed in agenda 9.2.1

In this contribution, we discuss on other aspects on AI/ML for positioning accuracy enhancements such as representative sub use-cases that are worth considering and related potential specification impacts in terms of RAN1 point of view.

Discussions on potential sub use cases and their specification impact

Based on above agreements, we suggest two sub use cases (i.e. LOS/NLOS indication, PRU prediction) on the perspective of positioning accuracy enhancement and the corresponding potential specification impact in the following sub-section. In sub-section 2.2, additional performance metric such as resource/power efficiency is discussed.

Accuracy improvements

In Rel-17 positioning enhancements, reporting of path specific measurement and providing LOS/NLOS indicator for first arrival path were agreed since LOS/NLOS identification is critical factor for accuracy performance. However, the indication has been without definition and it relies on UE implementation. In this sense, LMF does not know the reliability of the indication and then, it may be difficult to utilize it to improve the performance accuracy. Therefore, it should be required to improve the reliability of LOS indication information and to improve the performance in terms of positioning accuracy and resource efficiency.

Observation #1: In Rel-17, LOS/NLOS indication for first path can be reported but the detailed algorithm is up to UE implementation (reliability issue per UE).

To improve the reliability of LOS/NLOS information, it can be based on AI/ML approach at UE or NW side. When applying the AI/ML method at UE-side, the LOS/NLOS probability value for first arrival path and/or the reliability information such as probability can be derived based on the UE capability and various parameters related to LOS estimation such as UE antenna configurability, capability of PRS measurement, AI/ML model for LOS estimation, indication of PRU, the number of PRS measurement, noise variance, etc. For NW-side AI/ML, based on the measurement results and some assistance information (e.g. LOS probability and/or reliability information) from UE and/or gNB, LOS/NLOS identification can be achieved.

Proposal #1: Consider assistant information including LOS probability and/or reliability information for the AI/ML based LOS/NLOS identification.

Based on the LOS/NLOS related information from the above, NW can collect/process the LOS/NLOS identification at a specific location on coordinate system for supporting UEs. Through this, it is possible to determine which PRS resource is most likely to be LOS for a specific UE in the form of a boundary between specific regions on the coordinate as shown in Figure 1 below. Then, the PRS configuration for the location of the UE can be adaptively configured via the above information.

Figure 1. Example of LOS/NLOS probability and the corresponding PRSs on coordinate system supporting UEs

Proposal #2: Consider PRS priority configuration based on AI/ML based LOS/NLOS indication.

In the Rel-17 positioning enhancement, PRU (positioning reference unit) was introduced by utilizing the exactly known position of a specific UE or gNB (i.e. reference UE or gNB) at LMF for accuracy improvement. Regarding a UE as PRU, there is a limitation since the UE generally has mobility and the location is correspondingly changed. If the LMF can predict the UE location with mobility based on AI/ML method, it would be possible that which UE can be used as a PRU. Then, the LMF can use the UE dynamically as PRU to calculate/estimate the position of target UE.

Observation #2: When LMF can predict UE location with mobility, it is possible that which UE can be used as PRU, the LMF can use the UE dynamically as PRU to calculate the position of target UE.

For LMF-based PRU prediction, based on the measurement report of a normal UE as an input data set of the AI/ML algorithm, it is possible to predict/determine the output whether the normal UE is utilized as a PRU or not at the LMF side. If the UE is determined as the PRU, the position of target UE can be estimated/calculated with the PRU. Meanwhile, for UE-based PRU prediction, it is based on AI/ML model in the UE-side with providing assistance information for AI/ML model at LMF. Depending on the AI/ML model output, the UE can report identification information as PRU and/or assistance information, e.g. PRU expiration time, de-registration request on PRU, etc.

With these consideration, the potential specification impact can be related signaling on measurement report or information on deciding the UE as the PRU. For example, duration and the amount of distance change in duration for sensor based information and/or RSRP based on RAT dependent method can be considered as measurement or assistant information. Based on AI/ML model and assistant information, predicted value of the UE location with the corresponding time instance of the future, PRU related information such as PRU indication, the probability of the conditions, etc can be obtained.

Proposal #3: Consider PRU prediction on NW-/UE-side based on measurement report in addition to PRU identification and/or assistance information utilized for PRU determination.

Efficiency and power saving

In RAN1#109e, performance metrics other than accuracy for AI/ML based positioning was brought up but not discussed much on that. From point of efficiency and power saving for positioning, AI/ML also can be considered. For example, if LMF can configure more adequate resource(s) (or set/TRP) of PRS/SRS for UE in accordance with current channel condition or other many factors that impact on positioning for target UE, there are some resource like as the time/frequency resources that are not used for positioning and it brings out advantage of resource efficiency. Furthermore, if more adequate search window such as expected RSTD/UL-AoA/DL-AoD (including each uncertainty) can be configured based on AI/ML, it also brings out overhead reduction for beamforming and provides a resource saving for measurement report of unnecessary information. From this perspective, RAN1 can consider sub use case for positioning based on AI/ML in terms of efficiency and power saving.

Observation #3: Consider the metric for AI/ML based positioning in terms of efficiency and power saving as in Rel-17.

Conclusion

In this contribution, we discussed about several aspects on AI/ML for positioning enhancement, and our observations and proposals are summarized below:

Observation #1: In Rel-17, LOS/NLOS indication for first path can be reported but the detailed algorithm is up to UE implementation (reliability issue per UE).

Observation #2: When LMF can predict UE location with mobility, it is possible that which UE can be used as PRU, the LMF can use the UE dynamically as PRU to calculate the position of target UE.

Observation #3: Consider the metric for AI/ML based positioning in terms of efficiency and power saving as in Rel-17.

Proposal #1: Consider assistant information including LOS probability and/or reliability information for the AI/ML based LOS/NLOS identification.

Proposal #2: Consider PRS priority configuration based on AI/ML based LOS/NLOS indication.

Proposal #3: Consider PRU prediction on NW-/UE-side based on measurement report in addition to PRU identification and/or assistance information utilized for PRU determination.