3GPP TSG RAN WG1 Meeting #110 	 R1-2207004

Toulouse, France, August 22nd – 26th, 2022

Agenda Item: 7.1

Source: MediaTek Inc.

Title: 	On PUSCH power initialization during BWP switch

Document for: Discussion and decision

Introduction

In this contribution, we discuss the PUSCH power initialization during BWP switch. In 38.213 V16.10.0 [1] 7.1.1, when UE is doing PUSCH power initialization, some used parameters are configured under RACH/PUSCH configuration under UL BWP. If UE establishes RACH process in BWP0, and then switches to BWP1, it is not clear the parameters under which BWP should be used for PUSCH power initialization in BWP1. The intention of this document is to fix this issue in 38.213.

Discussion on PUSCH power initialization during BWP switch

In 38.213 V16.10.0 [1] 7.1.1, when UE is doing PUSCH power initialization, some used parameters, including preambleReceivedTargetPower/msg3-DeltaPreamble/msgA-preambleReceivedTargetPower/msgA-DeltaPreamble, are configured under RACH/PUSCH configuration under UL BWP, as shown below in highlighted sentences:

- 7.1	Physical uplink shared channel

<unchanged parts are omitted>

- 7.1.1 UE behavior

<unchanged parts are omitted>

- PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1.

-	If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in clause 8.3,

j=0, PO_UE_PUSCH,b,f,c0=0, and PO_NOMINAL,PUSCH,f,c0=PO_PRE+∆PREAMBLE,Msg3,

where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble, or ∆PREAMBLE,Msg3=0 dB if msg3-DeltaPreamble is not provided, for carrier f of serving cell c

-	If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A,

j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH,

where PO_PRE is provided by msgA-preambleReceivedTargetPower, or by preambleReceivedTargetPower if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c

<unchanged parts are omitted>

;

The RRC configuration hierarchy of preambleReceivedTargetPower/msg3-DeltaPreamble/msgA-preambleReceivedTargetPower/msgA-DeltaPreamble under UL BWP from 38.331 [2] are shown below for ease of reference:

- BWP-UplinkCommonRACH-ConfigCommonRACH-ConfigGenericpreambleReceivedTargetPower

- BWP-UplinkCommonPUSCH-ConfigCommonmsg3-DeltaPreamble

- BWP-UplinkCommonMsgA-ConfigCommon-r16RACH-ConfigCommonTwoStepRA-r16RACH-ConfigGenericTwoStepRA-r16msgA-preambleReceivedTargetPower-r16

- BWP-UplinkCommonMsgA-ConfigCommon-r16MsgA-PUSCH-Config-r16msgA-DeltaPreamble-r16

If UE establishes RACH process in BWP0, and then switches to BWP1, it is not clear the aforementioned parameters under which BWP should be used for PUSCH power initialization in BWP1.

Observation 1: In 38.213 V16.10.0 [1] 7.1.1, when UE is doing PUSCH power initialization, some used parameters, including preambleReceivedTargetPower/msg3-DeltaPreamble/msgA-preambleReceivedTargetPower/msgA-DeltaPreamble, are configured under RACH/PUSCH configuration under UL BWP. If UE establishes RACH process in BWP0, and then switches to BWP1, it is not clear the aforementioned parameters under which BWP should be used for PUSCH power initialization in BWP1.

Intuitively, the PUSCH power initialization should be based on the RACH process; UE applies the parameters UE used to establishes RACH process as a starting point for PUSCH power initialization, irrelevant of which BWP UE switches to.

Observation 2: The PUSCH power initialization should be based on the RACH process; UE applies the parameters UE used to establishes RACH process as a starting point for PUSCH power initialization, irrelevant of which BWP UE switches to.

Hence, we have the following text proposal:

Proposal 1: Adopt the CR below in 38.213 V16.10.0 [1] to clarify the used parameters for PUSCH power initialization

- 7.1	Physical uplink shared channel

<unchanged parts are omitted>

- 7.1.1 UE behavior

<unchanged parts are omitted>

- PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1.

-	If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in clause 8.3,

j=0, PO_UE_PUSCH,b,f,c0=0, and PO_NOMINAL,PUSCH,f,c0=PO_PRE+∆PREAMBLE,Msg3,

where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] corresponding to the Type-1 random access procedure and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble corresponding to the Type-1 random access procedure, or ∆PREAMBLE,Msg3=0 dB if msg3-DeltaPreamble is not provided, for carrier f of serving cell c

-	If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A,

j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH,

where PO_PRE is provided by msgA-preambleReceivedTargetPower corresponding to the Type-2 random access procedure, or by preambleReceivedTargetPower corresponding to the Type-2 random access procedure if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble corresponding to the Type-2 random access procedure, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c

<unchanged parts are omitted>

Conclusion

In this contribution, we discuss the PUSCH power initialization during BWP switch and have the following observations and proposals:

Observation 1: In 38.213 V16.10.0 [1] 7.1.1, when UE is doing PUSCH power initialization, some used parameters, including preambleReceivedTargetPower/msg3-DeltaPreamble/msgA-preambleReceivedTargetPower/msgA-DeltaPreamble, are configured under RACH/PUSCH configuration under UL BWP. If UE establishes RACH process in BWP0, and then switches to BWP1, it is not clear the aforementioned parameters under which BWP should be used for PUSCH power initialization in BWP1.

Observation 2: The PUSCH power initialization should be based on the RACH process; UE applies the parameters UE used to establishes RACH process as a starting point for PUSCH power initialization, irrelevant of which BWP UE switches to.

Proposal 1: Adopt the CR below in 38.213 V16.10.0 [1] to clarify the used parameters for PUSCH power initialization

- 7.1	Physical uplink shared channel

<unchanged parts are omitted>

- 7.1.1 UE behavior

<unchanged parts are omitted>

- PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1.

-	If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in clause 8.3,

j=0, PO_UE_PUSCH,b,f,c0=0, and PO_NOMINAL,PUSCH,f,c0=PO_PRE+∆PREAMBLE,Msg3,

where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] corresponding to the Type-1 random access procedure and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble corresponding to the Type-1 random access procedure, or ∆PREAMBLE,Msg3=0 dB if msg3-DeltaPreamble is not provided, for carrier f of serving cell c

-	If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A,

j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH,

where PO_PRE is provided by msgA-preambleReceivedTargetPower corresponding to the Type-2 random access procedure, or by preambleReceivedTargetPower corresponding to the Type-2 random access procedure if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble corresponding to the Type-2 random access procedure, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c

<unchanged parts are omitted>

Reference

3GPP TS 38.213 V16.10.0, NR; Physical layer procedures for control

[2]    3GPP TS 38.331 V17.0.0, NR; Radio Resource Control (RRC); Protocol specification