3GPP TSG RAN WG1 #110			R1-2207407

Toulouse, France, August 22nd – 26th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on potential enhancements on dynamic/flexible TDD

Agenda Item:	9.3.3

Document for: 	Discussion

- Introduction

At the RAN#94e meeting, a new SID [1] on “Study on evolution of NR duplex operation” was approved. The detailed objectives are as follows.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss potential enhancements on dynamic/flexible TDD.

- Potential enhancements on dynamic/flexible TDD

At the RAN1#109-e meeting, the CLI handling for duplex enhancement was discussed and following guideline, agreement, and conclusion were made [2].

Guideline for future meetings - Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD. - Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD. Agreement At least, following interference scenarios can be considered for study of dynamic/flexible TDD: - gNB-to-gNB inter-cell co-channel interference - UE-to-UE inter-cell co-channel interference Conclusion The following self-interference scenario and inter-subband CLI scenarios are not considered under AI 9.3.3 (Potential enhancements on dynamic/flexible TDD). - gNB self-interference - UE-to-UE intra-cell co-channel inter-subband CLI - UE-to-UE inter-cell co-channel inter-subband CLI - gNB-to-gNB inter-cell co-channel inter-subband CLI

Based on the outcome of RAN1#109-e, concerning the interference of duplex enhancement, following two cases are considered as shown in Fig.1.

1. Cross link interference at UE

Aggressor signal is UL signal transmitted by other UE, and victim signal is DL signal transmitted by gNB at UE reception. UE-UE CLI measurement and reporting mechanisms, such as CLI-RSSI and SRS-RSRP measurements were introduced in Rel-16, and it can be a baseline for UE-UE CLI for subband non-overlapping full duplex, and further enhancement can be considered, e.g. introducing spatial domain information, layer 1 based measurement and reporting.

2. Cross link interference at gNB

Aggressor signal is DL signal transmitted by other gNB, and victim signal is UL signal transmitted by UE at gNB reception. The inter-gNB interference is expected for dynamic/flexible TDD configurations, and it is necessary to study the impact of inter-gNB interference. Since the impact would be critical for the duplex operation, CLI handling at gNB can be studied.

Figure 1. Example of protentional interference for duplex enhancement.

  - Cross link interference management at UE

At the RAN1#109-e meeting, the UE-to-UE CLI handling for duplex enhancement was discussed and following agreement was made [2].

Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of UE-to-UE CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - Potential enhancements to UE-to-UE CLI measurement/reporting - Coordinated scheduling - Spatial domain enhancements, - Advanced Receiver - UE and gNB transmission and reception timing - Power control based solution - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for UE-to-UE CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancement specific for SBFD will be discussed in 9.3.2

UE-to-UE CLI management is already supported in Rel-16, and CLI measurement window can be flexibly configured to UEs. Therefore, the mechanism of Rel-16 CLI can be a baseline for the interference management. On top of the Rel-16 CLI mechanisms, we can consider several potential enhancements. One of the potential enhancements is to introduce spatial domain information for CLI measurement and reporting. As shown in Fig.3, if the direction of antenna beams of aggressor UE1 and that of victim UE0 match, e.g. “beam b” is the best beam and selected for DL reception of UE0, we may expect UE1 UL signal may interfere UE0 DL signal. On the other hand, if UE0 uses “beam c”, although the beam is the 2nd best beam but sufficient DL received power is expected, we may expect that the interference has little impact on UE0 reception. Therefore, interference management with spatial domain can be considered as a potential enhancement of UE-to-UE CLI.

Second potential enhancement is to introduce layer 1 based measurement and reporting. For the dynamic/flexible TDD or SBFD scenarios, interference environment may vary dynamically, e.g. the transmission direction (UL or DL) may be flexibly configured and scheduled based on the traffic of aggressor UE. Therefore, layer 1 CLI measurement and reporting with instantaneous interference level may be helpful for CLI handling.

Proposal 1: For the enhancement of UE-UE CLI handling for duplex enhancement, following potential enhancements should be studied.

- Introduction of spatial domain information

- Introduction of layer 1 based measurement and reporting

Figure 3. Example of UE-UE CLI.

  - Cross link interference management at gNB

At the RAN1#109-e meeting, the gNB-to-gNB CLI handling for duplex enhancement was discussed and following agreement was made [2].

Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of gNB-to-gNB CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - gNB-to-gNB CLI measurement and reporting - Coordinated scheduling - Spatial domain enhancements - Advanced receiver - UE and gNB transmission and reception timing - Power control based solution - Potential enhancements to Rel-16 RIM - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for inter-gNB CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancements specific for SBFD will be discussed in 9.3.2

gNB-to-gNB CLI is potentially happened, e.g. when the directions of antenna beam of aggressor and victim gNBs match. On the other hand, sufficient distance between gNBs is assumed with typical deployment, so that gNB-to-gNB CLI may be limited, compared to UE-to-UE CLI. Therefore, we may need to carefully study the necessity of gNB-to-gNB CLI handling based on evaluation study.

Concerning to the potential enhancement for gNB-to-gNB CLI handling, CLI measurement and reporting can be considered since it is the straightforward way to recognize the interference level among gNBs. In order to support gNB-to-gNB CLI measurement and reporting, gNBs need to exchange the information of time and frequency resource for the measurement.

Another potential enhancement is to introduce a signaling for exchanging gNB information, e.g. spatial information of gNB antenna beam. For example, if the directions of antenna beam of aggressor and victim gNBs match, CLI may be expected, while if they don’t match, CLI may be avoided. Therefore, of the signaling for exchanging gNB information would be helpful for gNB-to-gNB CLI.

Proposal 2: Introduction of gNB-to-gNB CLI handling is carefully studied based on the evaluation results, and if it is clarified that gNB-to-gNB CLI is critical to the system, following potential enhancements should be studied.

- Introduction of gNB-to-gNB CLI measurement and reporting

- Introduction of signaling for exchanging gNB information

- Conclusion

In this contribution, we discussed potential enhancements on CLI handling for dynamic/flexible TDD. Based on the discussion we made the following proposals and observation.

Proposal 1: For the enhancement of UE-UE CLI handling for duplex enhancement, following potential enhancements should be studied.

- Introduction of spatial domain information

- Introduction of layer 1 based measurement and reporting

Proposal 2: Introduction of gNB-to-gNB CLI handling is carefully studied based on the evaluation results, and if it is clarified that gNB-to-gNB CLI is critical to the system, following potential enhancements should be studied.

- Introduction of gNB-to-gNB CLI measurement and reporting

- Introduction of signaling for exchanging gNB information

References

- 3GPP, RP-213591, CMCC, “Study on evolution of NR duplex operation,” Dec. 2021.

- 3GPP, RAN1#109e, RAN1 Chairman’s Notes, May 2022.