3GPP TSG-RAN WG1 #110bis-e		R1-22xxxxx

e-Meeting, Oct 10 - 29, 2022

Source: 	Moderator (Ericsson)

Title:      	Summary#2 of Email discussion [110bis-e-R17-DSS-01]

Agenda item:	8.13

Document for:	Discussion and Decision

# 1 Introduction

This document summarizes discussions for RAN1#110bis-e for Rel17 NR DSS WI considering below documents [1]-[7] submitted for A.I. 8.13

| Ref# | TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- | --- |
| 1 | R1-2208621 | Corrections on Scell scheduling Pcell | vivo | 8.13 |
| 2 | R1-2209036 | Correction on different SCSs between P(S)Cell and sSCell | Intel Corporation | 8.13 |
| 3 | R1-2209037 | Discussion on different SCSs between P(S)Cell and sSCell | Intel Corporation | 8.13 |
| 4 | R1-2209450 | Discussion on simultaneous PDCCH monitoring between USS set on sSCell and CSS set on PCell | LG Electronics | 8.13 |
| 5 | R1-2209469 | Draft CR for Rel-17 DSS | ZTE | 8.13 |
| 6 | R1-2209851 | Correction for DCI size alignment for Rel-17 DSS | Huawei, HiSilicon | 8.13 |
| 7 | R1-2209962 | Discussion on clarification for cross-carrier scheduling from SCell to P(S)Cell | Qualcomm Incorporated | 8.13 |
|  | R1-2210191 | Disabling EN-DC power split when SCG is deactivated | Nokia, Nokia Shanghai Bell | 8.13 |

# 2. Topics for Discussion

Following topics for DSS WI were discussed in the tdocs

- Alignment of capability parameter names for FG 34-3 and FG 34-4 – [1],[5]

- Clarification related to P(S)Cell SCS > sSCell SCS case – [2],[3]

- Additional clarification on simultaneous PDCCH monitoring between sSCell USS sets and P(S)Cell CSS sets – [4]

- Clarification related to monitoringCapabilityConfig - [5]

- Further clarifications on DCI size alignment – [6]

- Clarification related to CSI-MeasConfig when SCell to PCell scheduling is used [ 7]

Companies are requested to provide comments (if any) on the topics to consider for discussion in RAN1#110bis-e in the Table below.

| Company Name | Comments |
| --- | --- |
| Moderator Notes1 | Topic 1 - can request editor reflect correct parameter names for FG 34-3 and FG 34-4 in 38.213 editor’s alignment CR. Topic 2 – was also discussed in RAN1#110 but no agreement. Topic 3 – is there need for additional clarification(s) considering what is already captured for FG 34-1 and 34-1a? Topics 4, 5, 6 – suggest discussing these in this meeting. |
| OPPO | Topic 1: This should be a category-D change per editorial. Agree with moderator to leave this modification to editor. Topic 2: As mentioned by moderator, this was already discussed in RAN1 #110 with no consensus. From our view, it was not a RAN1 agreement or study in Rel-17 that the comparison of SCS between PCell and sSCell can invalidate or turn over the overall configuration of sSCell scheduling PCell. We are ok to discuss Topics {3,4,5,6} |
| Qualcomm | Agree with OPPO. |
| LG Electronics | Agree with OPPO. |
| ZTE | Agree with moderator, i.e., “ Topics 4, 5, 6 – suggest discussing these in this meeting“. |
| Intel | For Topic 2, if majority companies think dynamic switching between BWPs with μP≤μS and BWPs with μP>μS is not supported in Rel-17 DSS, we prefer to make a conclusion on it. For other topics, agree with Moderator’s views. |
| Huawei | Generally OK with moderator suggestions. For Intel’s conclusion, also acceptable. |
| vivo | OK with moderator suggestions and fine with Intel’s conclusion. |
| Samsung | Agree with suggestion by Moderator/OPPO. For Topic 2, the issue was already settled as follows. We don’t see a need for additional conclusion. Agreement (RAN1#106bis-e) Option A is supported in Rel-17 - … - When P(S)Cell SCS (μ) is larger than sSCell SCS (μ1), for CCS from sSCell to P(S)Cell and, it is not supported Rel-17 DSS. |
| Spreadtrum | OK with moderator suggestions and Intel’s conclusion. |
| Moderator Notes1_2 | Topic 2,3,4,5,6 selected for further discussion in [110bis-e-R17-DSS-01] thread. |

# 3. Discussion

### 2.1 Topic 2 - Handling of P(S)Cell SCS > sSCell SCS

Please provide your input to below questions Q1-Q2 on this topic.

#### Question 1

Q1.  Is it OK to agree to draft CR for 38.213 in R1-2209036

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 2, Q1) |
| --- | --- | --- |

#### Question 2

Q2. If CR is not OK, is there need to make any additional conclusion? (Please also suggest text for the conclusion if needed).

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 2, Q2) |
| --- | --- | --- |

### 2.2 Topic 3 - Simultaneous monitoring between sSCell USS and P(S)Cell CSS

Please provide your input to below question Q1 on this topic.

#### Question 1

Q1. Do you see need for changes proposed in R1-2209450? If yes, should the clarification be included in 38.213 (Alt1 in  R1-2209450) or 38.306 (Alt2 in R1-2209450)

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 3, Q1) |
| --- | --- | --- |

### 2.3 Topic 4 - Clarification for monitoringCapabilityConfig

Please provide your input to question Q1 below

#### Question 1

Q1. Is it OK to agree to below change to 38.213 sub-clause 10.1.1 proposed in R1-2209469?

#### 10.1.1	Self-carrier and cross-carrier scheduling on the primary cell

A UE can be configured for scheduling on the primary cell from the primary cell and from a secondary cell [12, TS 38.331]. The UE is either not provided monitoringCapabilityConfig for the primary cell or for the secondary cell, or the UE is provided only monitoringCapabilityConfig = r15monitoringcapability for the primary cell and for the secondary cell. The UE is not provided coresetPoolIndex on the primary cell or on the secondary cell.

The SCS configuration μP for the active DL BWP on the primary cell is smaller than or equal to the SCS configuration μS for the active DL BWP on the secondary cell.

<Unchanged parts are omitted>

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 4, Q1) |
| --- | --- | --- |

### 2.4 Topic 5 - DCI size alignment

Please provide your input to question Q1 below.

#### Question 1

Q1. Is it OK to agree to draft CR to 38.212 proposed in R1-2209851?

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 5, Q1) |
| --- | --- | --- |

### 2.5 Topic 6 - CSI-MeasConfig when SCell to PCell scheduling is used

Please provide your input to question Q1 below

#### Question 1

Q1. Do you see need for additional clarification related to CSI-MeasConfig when SCell to PCell scheduling is used as discussed in R1-2209962. If yes, please indicate your preferred option for clarification also considering Opt.1,2,3 discussed in R1-2209962

Companies are requested to indicate their view about the above question in the Table below.

| Company Name | Yes/No | Comments (Topic 6, Q1) |
| --- | --- | --- |

# 3 Conclusions

TBU

# 4 References

[1]	R1-2208621 Corrections on Scell scheduling Pcell, vivo, RAN1#110bis-e, Oct 2022.

[2]	R1-2209036 Correction on different SCSs between P(S)Cell and sSCell, Intel Corporation, RAN1#110bis-e, Oct 2022.

[3]	R1-2209037 Discussion on different SCSs between P(S)Cell and sSCell, Intel Corporation, RAN1#110bis-e, Oct 2022.

[4]	R1-2209450 Discussion on simultaneous PDCCH monitoring between USS set on sSCell and CSS set on PCell, LG Electronics, RAN1#110bis-e, Oct 2022.

[5]	R1-2209469 Draft CR for Rel-17 DSS, ZTE, RAN1#110bis-e, Oct 2022.

[6]	R1-2209851 Correction for DCI size alignment for Rel-17 DSS, Huawei, HiSilicon, RAN1#110bis-e, Oct 2022.

[7]	R1-2209962 Discussion on clarification for cross-carrier scheduling from SCell to P(S)Cell, Qualcomm Incorporated, RAN1#110bis-e, Oct 2022.

[1]	R1-22xxxxx Summary#1 of Email discussion [110bis-e-R17-DSS-01], Moderator (Ericsson), RAN1#110bis-e, Oct 2022.