3GPP TSG-RAN WG1 Meeting #110bis-e                                                       R1-22xxxxx

e-Meeting, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on DCI size alignment for Cross-carrier schduling from SCell to PCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), [Huawei, HiSilicon], [Samsung] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DSS-Core |  |  |  |  |  | Date: |  |  | 2022-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN1#110, CR 0119 (R1-2208246) on DCI size alignment for cross-carrier scheduling from SCell to PCell was agreed and following was captured into TS38.212 (only DCI format 0_1 related text from the previous CR shown below as example). - If the SCell is deactivated, the UE determines the number of information bits in DCI format 0_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, the UE determines the number of information bits in DCI format 0_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell. However, the text in previous CR was unclear for the following cases and this needs to be clarified When the SCell is deactivated, and: - firstActiveDownlinkBWP-Id is set to dormant BWP When the active DL BWP of the SCell is a dormant DL BWP, and: - firstWithinActiveTimeBWP-Id is not configured for the SCell (only firstOutsideActiveTimeBWP-Id is configured) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that: - When the SCell is deactivated and if firstActiveDownlinkBWP-Id is not set to dormant BWP - DCI size alignment based on firstActiveDownlinkBWP-Id for the SCell - If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP - DCI size alignment based on firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on firstOutsideActiveTimeBWP-Id for the SCell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification of DCI size alignment for cross-carrier scheduling from SCell to PCell for following cases When the SCell is deactivated, and: - firstActiveDownlinkBWP-Id is set to dormant BWP When the active DL BWP of the SCell is a dormant DL BWP, and: - firstWithinActiveTimeBWP-Id is not configured for the SCell (only firstOutsideActiveTimeBWP-Id is configured) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.2, 7.3.1.1.3, 7.3.1.2.2, 7.3.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

<omitted text>

For a UE configured with scheduling on the primary cell from an SCell, if prior to padding the number of information bits in DCI format 0_1 carried by PDCCH on the primary cell is not equal to the number of information bits in DCI format 0_1 carried by PDCCH on the SCell for scheduling on the primary cell, zeros shall be appended to the DCI format 0_1 with smaller size until the payload size is the same.

- If application of step 4C in clause 7.3.1.0 results in additional zero padding for DCI format 0_1 for scheduling on the primary cell, corresponding zeros shall be appended to both DCI format 0_1 monitored on the primary cell and DCI format 0_1 monitored on the SCell for scheduling on the primary cell.

- If the SCell is deactivated and firstActiveDownlinkBWP-Id is not set to dormant BWP, the UE determines the number of information bits in DCI format 0_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP, the UE determines the number of information bits in DCI format 0_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

<omitted text>

##### 7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

<omitted text>

For a UE configured with scheduling on the primary cell from an SCell, if prior to padding the number of information bits in DCI format 0_2 carried by PDCCH on the primary cell is not equal to the number of information bits in DCI format 0_2 carried by PDCCH on the SCell for scheduling on the primary cell, zeros shall be appended to the DCI format 0_2 with smaller size until the payload size is the same.

- If application of step 4B in clause 7.3.1.0 results in additional zero padding for DCI format 0_2 for scheduling on the primary cell, corresponding zeros shall be appended to both DCI format 0_2 monitored on the primary cell and DCI format 0_2 monitored on the SCell for scheduling on the primary cell.

- If the SCell is deactivated and firstActiveDownlinkBWP-Id is not set to dormant BWP, the UE determines the number of information bits in DCI format 0_2 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP, the UE determines the number of information bits in DCI format 0_2 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

<omitted text>

##### 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

<omitted text>

For a UE configured with scheduling on the primary cell from an SCell, if prior to padding the number of information bits in DCI format 1_1 carried by PDCCH on the primary cell is not equal to the number of information bits in DCI format 1_1 carried by PDCCH on the SCell for scheduling on the primary cell, zeros shall be appended to the DCI format 1_1 with smaller size until the payload size is the same.

- If application of step 4C in clause 7.3.1.0 results in additional zero padding for DCI format 1_1 for scheduling on the primary cell, corresponding zeros shall be appended to both DCI format 1_1 monitored on the primary cell and DCI format 1_1 monitored on the SCell for scheduling on the primary cell.

- If the SCell is deactivated and firstActiveDownlinkBWP-Id is not set to dormant BWP, the UE determines the number of information bits in DCI format 1_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP, the UE determines the number of information bits in DCI format 1_1 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

<omitted text>

##### 7.3.1.2.3	Format 1_2

DCI format 1_2 is used for the scheduling of PDSCH in one cell.

<omitted text>

For a UE configured with scheduling on the primary cell from an SCell, if prior to padding the number of information bits in DCI format 1_2 carried by PDCCH on the primary cell is not equal to the number of information bits in DCI format 1_2 carried by PDCCH on the SCell for scheduling on the primary cell, zeros shall be appended to the DCI format 1_2 with smaller size until the payload size is the same.

- If application of step 4B in clause 7.3.1.0 results in additional zero padding for DCI format 1_2 for scheduling on the primary cell, corresponding zeros shall be appended to both DCI format 1_2 monitored on the primary cell and DCI format 1_2 monitored on the SCell for scheduling on the primary cell.

- If the SCell is deactivated and firstActiveDownlinkBWP-Id is not set to dormant BWP, the UE determines the number of information bits in DCI format 1_2 carried by PDCCH on the primary cell based on a DL BWP provided by firstActiveDownlinkBWP-Id for the SCell. If the active DL BWP of the SCell is a dormant DL BWP, or if the SCell is deactivated and firstActiveDownlinkBWP-Id is set to dormant BWP, the UE determines the number of information bits in DCI format 1_2 carried by PDCCH on the primary cell based on a DL BWP provided by firstWithinActiveTimeBWP-Id for the SCell if provided; otherwise, based on a DL BWP provided by firstOutsideActiveTimeBWP-Id for the SCell.

<omitted text>