3GPP TSG RAN WG1 #110bis-e	R1-22xxxx

# e-Meeting, October 10th – 19th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for multi-slot PDCCH monitoring in FR2-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Lenovo), ZTE, Sanechips, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz |  |  |  |  |  | Date: |  |  | 2022-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN1#108-e and RAN1#109-e agreements were achieved for search space set configuration of multi-slot PDCCH monitoring, where a new parameter monitoringSlotsWithinSlotGroup-r17 is introduced to indicate which slot(s) within a slot group are configured for multi-slot PDCCH monitoring. Different types of search space set correspond to different PDCCH monitoring patterns. However, only the monitoring pattern for some SSs has been specified in TS 38.213 so far, neither TS 38.213 nor TS 38.331 capture PDCCH monitoring pattern indicating slots in a group of slots for Type0/0A/2 CSS and Type1 CSS without dedicated RRC signaling. RAN2 agreed to a CR on field description in TS 38.331 for monitoringSlotsWithinSlotGroup that refers back to TS 38.213 Section 10 for a description on the number of slots configured for multi-slot PDCCH monitoring. TS 38.213 should therefore give a complete picture. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the PDCCH monitoring pattern configured in monitoringSlotsWithinSlotGroup-r17 for Type0/0A/2 CSS and Type1 CSS without dedicated RRC signaling in TS 38.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification does not fully capture RAN1’s agreements. The reference for monitoringSlotsWithinSlotGroup from TS 38.331 points to an imcomplete description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This is to correct the specification to align with previous RAN1 agreements. No additional impact is expected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

10.1	UE procedure for determining physical downlink control channel assignment

<Unrelated part omitted>

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10 search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace:

-	a search space set index s, 0<s<40 , by searchSpaceId

-	an association between the search space set  s and a CORESET p by controlResourceSetId or by controlResourceSetId-v1610

-	a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset or by monitoringSlotPeriodicityAndOffset-r17

-	a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET for PDCCH monitoring within each slot where the UE monitors PDCCH, by monitoringSymbolsWithinSlot

-	a duration of Ts<ks indicating a number of slots that the search space set s exists by duration, or a number of slots in consecutive groups of slots where the search space set s can exist by duration-r17

-	a bitmap, by monitoringSlotsWithinSlotGroup, that applies per group of slots and provides a PDCCH monitoring pattern indicating slots in a group of slots for PDCCH monitoring

-	a size of the group of slots is same as a size of monitoringSlotsWithinSlotGroup

-	for a Type1-PDCCH CSS set provided by ra-SearchSpace in dedicated RRC signaling, or for a Type3-PDCCH CSS set, or for a USS set, the PDCCH monitoring pattern indicates only consecutive slots in the group of slots for PDCCH monitoring and, at least for one combination Xs,Ys indicated by the UE as a capability, a number of the consecutive slots is not larger than Ys

-	for a Type1-PDCCH CSS set provided by ra-SearchSpace in SIB1, the PDCCH monitoring pattern indicates only 1 slot in the group of slots for PDCCH monitoring

-	for a Type0-PDCCH CSS set or for a Type0A-PDCCH CSS set, or for a Type2-PDCCH CSS set, the PDCCH monitoring pattern indicates slots in the group of slots for PDCCH monitoring, and the slots are not restricted to be consecutive, and the number of those slots is not larger than the size of monitoringSlotsWithinSlotGroup

-	a number of PDCCH candidates Ms(L) per CCE aggregation level L by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively

-	an indication that search space set s is either a CSS set or a USS set by searchSpaceType

<Unrelated part omitted>