3GPP TSG RAN WG1 #110bis-e	R1-22xxxxx

e-Meeting, October 10th – 19th, 2022

Title:	[DRAFT] LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility

Response to:

Release:	Rel-18

Work Item:	NR_mob_enh2-Core

Source:	Fujitsu [to be RAN1]

To:	RAN2, RAN3, RAN4

Cc:	RAN2

Contact Person:

Name:	Yosuke Akimoto

E-mail Address:	akimoto.yosuke at fujitsu.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

- L1 Intra-frequency measurement

Regarding L1/L2-based inter-cell mobility for Rel-18 NR further mobility enhancement, RAN1 made the following agreements for L1 intra-frequency measurement:

Agreement

- For Rel-18 L1/L2 mobility, L1 intra-frequency measurement for candidate cell is supported

  - At least the following aspects are for RAN1 further study:

    - RAN1 assumes Rel-17 ICBM CSI measurement as starting point.

    - Whether and how to apply relaxation for the restrictions imposed on the Rel-17 intra-frequency L1 non-serving cell measurement defined in 9.13.2 of TS38.133, where RAN4 impact is foreseen, e.g.

      - SFN offset alignment compared with serving cell

      - BWP setting, i.e. non-serving cell SSB should be covered by serving cell active BWP

      - Introduction of symbol level gap or SMTC for larger Rx timing difference (i.e. larger than CP length)

    - Commonality with intra-frequency L3 measurement

    - Commonality with L1 inter-frequency measurement for measurement configuration

Question 1 (to RAN4): As mentioned in this agreement, while RAN1 assumes Rel-17 ICBM CSI measurement as starting point for L1 intra-frequency measurement for Rel-18 L1/L2 mobility, RAN1 wonders if the restriction on e.g., SFN offset alignment, BWP setting, i.e. non-serving cell SSB should be covered by serving cell active BWP, and Rx timing difference, etc, described in 9.13.2 of TS38.133 for intra-frequency L1 non-serving measurement can be relaxed or not.

- L1 Inter-frequency measurement

In addition, RAN1 made the following agreements for L1 inter-frequency measurement:

Agreement

- For Rel-18 L1/L2 mobility, further study the potential RAN1 spec impact of L1 inter-frequency measurement

  - The definition and scenarios of L1 inter-frequency measurement is determined by RAN4, and RAN1 assumes at least the following until receiving their confirmation

    - The scenarios not included in intra-frequency are regarded as inter-frequency, which includes at least the following scenarios:

      - The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE.

      - The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE

  - At least the following aspect is studied:

    - Commonality with L1 intra-frequency measurement for measurement configuration

Question 2 (to RAN4): As mentioned in this agreement, RAN1 would like to ask RAN4 to confirm our understanding that the supported scenarios not included in intra-frequency are regarded as inter-frequency for L1 measurement, which includes at least the following scenarios:

- The frequency of the measured RS not covered by any of the active BWPs of SpCell and Scells configured for a UE, but covered by some of the configured BWPs of SpCell and Scells configured for a UE.

- The frequency of the measured RS not covered by any of the configured BWPs of SpCell and Scells configured for a UE

Also, RAN1 would like to inform RAN4 of our understanding that the introduction of measurement gap and SMTC for L1 inter-frequency measurement, if any, is expected to be a RAN4 issue.

[

- L1 measurement and TCI state configurations

RAN1 has started the discussion on the configuration for L1 measurement and TCI states for candidate cells. Regarding the following RAN2 agreements captured in RAN2 LS (R1-2208331/R2-2209257), it is not clear for RAN1 which kind of information/configuration for candidate cell(s) are available at a serving cell for inter-DU case for Rel-18 L1/L2 mobility. Thus, companies have different understanding on the implication of the sentence “as much commonality as reasonable” in the LS.

- The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different.

Question 3 (to RAN2 and RAN3): RAN1 respectfully asks RAN2 and RAN3 if the serving DU knows the measurement RS configuration and TCI state configuration of cells served by another DU.

- Other agreements

The agreements achieved in RAN1#110b-e are captured below for information.

Agreement

xxxx

Agreement

xxxx

]

2. Actions:

To RAN4 group.

ACTION: 	RAN1 respectfully asks RAN4 to answer the above questions (Question 1 and 2), and to take the RAN1 agreements into consideration for their work.

To RAN2 and RAN3 group.

ACTION: 	RAN1 respectfully asks RAN2 and RAN3 to answer above question (Question 3), and to take the RAN1 agreements into consideration for their work.

3. Date of Next TSG RAN WG1 Meetings:

TSG RAN WG1 Meeting #111	14th – 18th Nob. 2022		Toulouse, FR.

TSG RAN WG1 Meeting #112	27th Feb. – 3rd Mar. 2022	Athens, GR.