3GPP TSG RAN WG1 #110-bis                                              R1-2208575

e-Meeting, October 10th – 19th, 2022

Agenda Item:     9.14.1

Source:	Spreadtrum Communications

Title:	Discussion on PRACH coverage enhancements

Document for:	Discussion and decision

Introduction

In RAN#96, a revised Work Item [1] was agreed to study UL coverage enhancement, and the objectives include PRACH coverage enhancements, as shown below.

- Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

In this contribution, we present our analysis on PRACH coverage enhancements.

Discussion

Multiple PRACH transmissions with same beam or different beam

In the low frequency network of NR, the DL coverage area and/or UL coverage distance of TRP/UE can be covered by a single beam. However, in the case of high frequency, such as the new radio above 6 GHz, due to the large path loss and penetration loss in high frequency band, channel/signal transmission is heavily dependent on the link quality. In this case, Msg1 needs to be transmitted multiple to increase the possibility of random access, and time diversity provides increased reliability

There are two ways to implement multiple Msg1 transmission, using the same beam or different beam. Considering that the beam correspondence is mandatory in Rel-15, it is natural to use the same UL Tx beam for multiple Msg1 transmissions. Transmissions with same beams can improve UL coverage by multiple repetitions, which would increase the success possibility of random access.

Repetition with different beams increases robustness even for UE with beam correspondence. In the case where the UE’s beam correspondence may not be guaranteed, e.g., in initial access, a better beam may be included than the beam based on SSB reception. PRACH detection may fail due to beam misalignment between the UE and the gNB. In this case, UE’s beam determined based on SSB reception may not be appropriate one for PRACH transmission, and transmitting multiple Msg1 with different beams have better PRACH coverage performance than multiple Msg1 transmission with same beam (the beam based on SSB reception).  Moreover, PRACH for FR2 was identified as a bottleneck in Rel-17 coverage enhancement study. Due to the large path loss and penetration loss, beam may be switched frequently in FR2. Multiple Msg1 transmission with different beams may increase reliability since both the radio channel and the instantaneous interference-plus-noise level may differ significantly between different directions in high frequency band.

Observation 1: Multiple Msg1 transmission with different beams can increase robustness than same beam, especially for the scenario of non-beam correspondence, beam switching frequently in FR2.

Proposal 1: Different beams for multiple Msg1 transmissions can be considered as the same beam.

Multiple PRACH transmissions with same beam

CBRA/CFRA

UE can access network through contention-based random access (CBRA) or contention-free random access (CFRA). Both CBRA and CFRA should be support for multiple Msg1 transmissions.

For CBRA, since many UEs compete for the preambles in a RACH occasion, multiple Msg1 transmissions may increase more collisions. gNB would configure proper random access preamble resources considering the repetition numbers.

For CFRA, dedicated preamble for Msg1 transmission is assigned by the network. If UE transmits only a single Msg1 on the dedicated RACH resource, the other resources will be wasted. UE may transmit multiple Msg1 on the dedicated preamble resources, which makes efficient utilization of RACH resources for multiple Msg1 transmissions. In addition, there are many scenarios triggering CFRA, such as handover, DL data arrival during RRC_CONNECTED when UL synchronisation status is “non-synchronised”, etc. A UE performing handover is usually located at the cell-edge of both the source and target cell with relatively poor uplink coverage conditions. A single Msg1 transmission may cause handover failure and increase the risk of link failure and mobility interruption. Therefore, multiple Msg1 transmissions are a better choice in these cases. It is essential to increase the probability of random access and the transmission reliability.

Observation 2: In contention free random access, multiple Msg1 transmissions may improve the utilization of RACH resources, and increase the probability of random access and the transmission reliability.

Msg.1

Number of multiple Msg1 transmissions

It was agreed to study in UL coverage WID that UE uses the same UE Tx beam for multiple Msg1 transmissions for 4-step RACH transmission. An important issue is to determine whether or not to apply multiple PRACH transmissions and if so how to determine the number of multiple Msg1 transmissions.

For CFRA, network can configure the number based on its deployment policy so that a good balance between the random access probability and latency can be achieved.

For CBRA, the UE Tx beam can be selected based on the measurement results of SSB-RSRP. At the same time, Msg1 repetition number can be implicitly indicated through the level of the beam quality, as shown in the Figure 1, other factors can also be considered. If the PRACH repetition number can be configured to transmit 2, 4, 8 time, etc., and divide the rsrp-ThresholdSSB into different levels, corresponding to different repetition numbers. For example, if UE selects the UL Tx beam corresponding to beam1 to transmit multiple Msg1. The RSRP of beam1 meets the range of [rsrp-ThresholdSSB-Msg1_1, rsrp-ThresholdSSB-Msg1_2], then, the multiple Msg1 transmission number is 8. Similarly, if the RSRP of beam1 meets the range of [rsrp-ThresholdSSB-Msg1_2, rsrp-ThresholdSSB-Msg1_3], the multiple Msg1 transmission number is 4. If the RSRP of beam1 meets the range of [rsrp-ThresholdSSB-Msg1_3, rsrp-ThresholdSSB-Msg1_4], the multiple Msg1 transmission number is 2. In this way, the multiple Msg1 repetition numbers related to the beam quality, which not only configure an appropriate value for PRACH repetition, but also save the signaling overhead through an implicit way.

Figure 1.  The relationship between rsrp-ThresholdSSB and PRACH repetition number.

Proposal 2: For CFRA, the Msg1 repetition number can be configured by network. For CBRA, the Msg1 repetition number can be implicitly indicated through the level of the RSRP and/or other factors.

Whether to share the PRACH resources between multiple and single Msg1 transmission

Legacy UE and Rel-18 UL cov enh. UE can be co-existence in one cell, the supportive of backward compatibility is important. Regarding the PRACH resources division between multiple Msg1 transmissions and single Msg1 transmission (legacy UE), there can be multiple options list in the following.

- Option 1 - Shared RACH occasion and preambles configuration: multiple Msg1 transmissions and single Msg1 transmission share the PRACH resources, RACH occasions and/or preambles can be used for both.

- Option 2 - Shared RACH occasion and separate preambles configuration: multiple Msg1 transmissions and single Msg1 transmission are separated by using different sets of PRACH preambles in the same RACH occasion

- Option 3 - Separate RACH occasion configuration: multiple Msg1 transmissions and single Msg1 transmission are separated by using independent sets of RACH occasions.

For option1, it is difficult to distinguish with multiple Msg1 transmissions and single Msg1 transmission. When a gNB receives multiple Msg1, it cannot identify whether multiple Msg1 are sent by the same UE or different UEs, and gNB has no idea whether the detected preamble has associated multiple transmissions on other RACH occasions. If gNB is required to allocate resource of Msg3 for each received Msg1. The UE with multiple Msg1 transmissions may receive multiple RARs. Only one resource is needed to transmit Msg3, and other resources indicated by multiple Msg2 are unnecessary.

For option 2, it is better than Option 1 in distinguishing multiple Msg1 transmissions from single Msg1 transmissions. However, for legacy UE access through contention-based random access, the number of available preambles decreases, resulting in more intense competition and greater probability of collision.

For option 3, it is an easy method to expand to multiple PRACH transmission, some benefits are list as follows:

- No need to consider the coexistence of legacy UE without multiple PRACH

- Flexible design of association between SSB and RO

- Easy to trace the start point and end point of multiple Msg1 transmissions.

Observation 3: Separate PRACH resource for multiple Msg1 transmissions and single Msg1 transmission may not need to consider the coexistence of legacy UEs.

Msg2

RAR window configuration

For multiple Msg1 transmissions applied to 4-step RACH transmission, a topic that needs to be studied is the determination between RAR window and the multiple Msg1 transmissions, including the starting position and the length of the RAR windows, corresponds between multiple PRACH transmissions and RAR within the RAR window.

There are three options to associate Msg1 transmission with a random access response (RAR) window:

- Option 1: A 1:1 mapping between Msg1 transmission and the RAR window. As is shown in Figure 2, for each Msg1 transmission, there is a RAR window to monitor the corresponding Msg2. In this way, UE needs to monitor Msg2 after transmitting Msg1 at each time, which may increase UE power consumption. In addition, it can be seen from the figure that the position of the RAR window may coincide with the sending time of Msg1. Due to the half-duplex limitation and TDD mode, a UE will not perform transmitting Msg1 and receiving Msg2 simultaneously.

- Option 2: A i:1 mapping between Msg1 transmission and the RAR window. As is shown in Figure 3, where the Msg1 transmission for i times mapping to the same RAR reception window, the range of i value is less than or equal to K, where K is the number of multiple Msg1 transmissions. Similar with option1, the problem of not perform transmitting Msg1 and receiving Msg2 at the same time also exists in this option.

- Option 3: K:1 mapping between Msg1 transmission and the RAR window. A simple method is to configure a RAR window and start the RAR window after UE sends the last Msg1 for all K times. As is shown in Figure 4, where the Msg1 transmission for all K times mapping to a RAR window. Transmitting msg1 and receiving Msg2 will not occur at the same time. In addition, this way has little change on the specification among the three option.

Proposal 3: K:1 mapping between Msg1 transmission and the RAR window should be considered as a baseline.

Figure 2: one to one mapping between the Msg1 transmission and the RAR window

Figure 3: i to one mapping between the Msg1 transmission and the RAR window (i≤K)

Figure 4: K to one mapping between the Msg1 transmission and the RAR window

In addition, if UE receives Msg2 before the K-th transmission of Msg1, it is a waste of RO resources if UE continues to transmit Msg1. So it can contain an implicit indication that once UE receives Msg2, UE is able to terminate the follow up Msg1 transmission in advance and release the corresponding RO resources for other UEs to access.

Proposal 4: Once UE receives Msg2, UE is able to terminate the follow up Msg1 transmission in advance and release the corresponding RO resources for other UEs to access.

RA-RNTI

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during RAR window. The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted, is computed as [2]:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

In the RA-RNTI calculation function, the index of the first OFDM symbol of the PRACH occasion (s_id), the index of the first slot of the PRACH occasion in a system frame (t_id) and the index of the PRACH occasion in the frequency domain (f_id) are all related the PRACH occasion.

If a single Msg1 is transmitted before the end of a monitored RAR window, including the case of transmitting Msg1 only once and one to one mapping between Msg1 transmission and the RAR window (Option1), there is no ambiguity in the position of the PRACH occasion. However, if network configures multiple Msg1 transmission before a RAR window, for example, multiple Msg1 transmissions are mapped to one RAR window as described in Option 2 or Option 3. UE is unable to determine which position of PRACH occasion should be used to calculate RA-RNTI. The position of the PRACH occasion for calculating RA-RNTI should be further studied. A simple way is to specify that the PRACH occasion for calculating RA-RNTI is determined by a specific reference position, such as the position of PRACH occasion corresponds to transmit the first Msg1 or the last Msg1.

Proposal 5: For multiple Msg1 transmission, the position of the RACH occasion for calculating RA-RNTI need to be defined, for example, taking the RACH occasion position corresponds to transmit the first Msg1 or the last Msg1 as a reference.

Msg.3

What’s more, the interaction of PRACH repetition with Msg3 repetition should be studied. For example, if the number of multiple PRACH are transmitted, how to request Msg3 repetition. And according to a PRACH repetition number, gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition with a different repetition factor.

Proposal 6: The interaction of PRACH repetition with Msg3 repetition should be studied.

Conclusion

In this contribution, we discussed PRACH coverage enhancements. The following proposals and observations are achieved:

Observation 1: Multiple Msg1 transmission with different beams can increase robustness than same beam, especially for the scenario of non-beam correspondence, beam switching frequently in FR2.

Observation 2: In contention free random access, multiple Msg1 transmissions may improve the utilization of RACH resources, and increase the probability of random access and the transmission reliability.

Observation 3: Separate PRACH resource for multiple Msg1 transmissions and single Msg1 transmission may not need to consider the coexistence of legacy UEs.

Proposal 1: Different beams for multiple Msg1 transmissions can be considered as the same beam.

Proposal 2: For CFRA, the Msg1 repetition number can be configured by network. For CBRA, the Msg1 repetition number can be implicitly indicated through the level of the RSRP and/or other factors.

Proposal 3: K:1 mapping between Msg1 transmission and the RAR window should be considered as a baseline.

Proposal 4: Once UE receives Msg2, UE is able to terminate the follow up Msg1 transmission in advance and release the corresponding RO resources for other UEs to access.

Proposal 5: For multiple Msg1 transmission, the position of the RACH occasion for calculating RA-RNTI need to be defined, for example, taking the RACH occasion position corresponds to transmit the first Msg1 or the last Msg1 as a reference.

Proposal 6: The interaction of PRACH repetition with Msg3 repetition should be studied.

Reference

- RP-221858, Revised WID on further NR coverage enhancements.

- 38.321, “NR; Medium Access Control (MAC) protocol specification”