3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2208619

# e-Meeting, October 10th – 19th,2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxxx | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on type 2 codebook determination with DG PDSCHs and SPS PDSCHs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022/9/22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For type 2 codebook construction for unicast HARQ-ACK and multicast HARQ-ACK, the following agreements were made in previous meeting, based on which it can concluded that the HARQ-ACK sub-codebook for both unicast SPS PDSCHs and multicast SPS PDSCHs are jointly constructed and concatenated with HARQ-ACK sub-codebook for unicast DG PDSCHs and sub-codebook for multicast DG PDSCHs and is placed after HARQ-ACK sub-codebook for multicast DG PDSCHs. However, the current specification in 9.1.3.1, it captures that the UE separately applies the procedures in this clause per G-RNTI or per G-CS-RNTI, which may lead HARQ-ACK sub-codebook for SPS PDSCHs would be generated in multiple times, since the procedure to generate HARQ-ACK sub-codebook for unicast SPS PDSCHs and multicast SPS PDSCHs is also included in “the procedures in this clause”. Agreement: (RAN1#104) For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1, - DAI for unicast and DAI for multicast are separately counted. - Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported. - FFS details on concatenating the codebooks. - FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast. Agreement: (RAN1#104bis) For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource, - The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast. - FFS: The number of Type-2 HARQ-ACK sub-codebooks for multicast. - Note: The case of SPS PDSCH will be discussed separately. Agreement: (RAN1#107bis) When UE is configured with unicast SPS and multicast SPS with ACK/NACK based feedback for multiplexing on the same PUCCH for the same priority case, the HARQ-ACK codebook is constructed as for multiple SPS PDSCHs regardless of unicast SPS PDSCH or multicast SPS PDSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the HARQ-ACK codebook for SPS PDSCHs is generated only once and is concatenated after the HARQ-ACK codebook for multicast DG PDSCHs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behavior on type 2 codebook construction is not correct in the concerned cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

If a UE is configured to monitor PDCCH for multicast DCI formats with CRC scrambled by one or more G-RNTIs or G-CS-RNTIs that the UE generates a Type-2 HARQ-ACK codebook, the UE separately applies the procedures in this clause except the procedures for SPS PDSCHs per G-RNTI or per G-CS-RNTI and determines the Type-2 HARQ-ACK codebook by concatenating the Type-2 HARQ-ACK codebook for unicast DCI formats, followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-RNTI values, followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-CS-RNTI values, followed by the HARQ-ACK codebook for SPS PDSCHs.

< Unchanged parts are omitted >

*************************************End of TP********************************************