3GPP TSG-RAN WG1 Meeting #110bis-e 	R1-2208625

# eMeeting, October 10 - 19

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | x | rev | x | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on resource pool index for sl-TxPoolScheduling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022/9/24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to R1-2208329 (R2-2209207), in Rel-17, RAN2 introduced mode-1 dedicated discovery resource pool configuration for NR SL discovery transmission, i.e. sl-DiscTxPoolScheduling. However, the current DCI Format 3_0 cannot schedule any resource in the pool(s) indicated by sl-DiscTxPoolScheduling, since the “Resource pool index” field in DCI format 3_0 cannot refer to any pool configured by sl-DiscTxPoolScheduling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the resource pool index field in DCI format 3_0 refers to a resource pool provided by sl-DiscTxPoolScheduling or sl-TxPoolScheduling, and clarify that resource pools provided by sl-TxPoolScheduling are indexed first, then resource pools provided by sl-DiscTxPoolScheduling are indexed, if both sl-TxPoolScheduling and sl-DiscTxPoolScheduling are configured. Clarify that DCI size alignment is performed among all tx pools including the ones provided by sl-DiscTxPoolScheduling if configured, and those provided by sl-TxPoolScheduling if configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB is not allowed to schedule a dedicated discovery resource pool for mode-1 UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.4.1	Format 3_0

DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell.

The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI:

-	Resource pool index –log2I  bits, where I is the number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling, if configured, and sl-DiscTxPoolScheduling, if configured. If both sl-TxPoolScheduling and sl-DiscTxPoolScheduling are provided, the UE determines the resource pool index in the following order:

-	first, in increasing order of resource pools provided in sl-TxPoolScheduling;

-	second, in increasing order of resource pools provided in sl-DiscTxPoolScheduling.

-	Time gap – 3 bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.1.2.1 of [6, TS 38.214]

-	HARQ process number – 4 bits.

-	New data indicator – 1 bit.

-	Lowest index of the subchannel allocation to the initial transmission –log2(N subChannel SL) bits as defined in clause 8.1.2.2 of [6, TS 38.214]

-	SCI format 1-A fields according to clause 8.3.1.1:

-	Frequency resource assignment.

-	Time resource assignment.

-	PSFCH-to-HARQ feedback timing indicator –log2Nfb_timing bits, where Nfb_timing is the number of entries in the higher layer parameter sl-PSFCH-ToPUCCH, as defined in clause 16.5 of [5, TS 38.213]

-	PUCCH resource indicator – 3 bits as defined in clause 16.5 of [5, TS 38.213].

-	Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise 3 bits as defined in clause 8.1.2 of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI.

-	Counter sidelink assignment index – 2 bits

-	2 bits as defined in clause 16.5.2 of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic

-	2 bits as defined in clause 16.5.1 of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static

-	Padding bits, if required

If multiple transmit resource pools are provided in sl-TxPoolScheduling if configured, and sl-DiscTxPoolScheduling if configured, zeros shall be appended to the DCI format 3_0 until the payload size is equal to the size of a DCI format 3_0 given by a configuration of the transmit resource pool resulting in the largest number of information bits for DCI format 3_0.

If the UE is configured to monitor DCI format 3_1 and the number of information bits in DCI format 3_0 is less than the payload of DCI format 3_1, zeros shall be appended to DCI format 3_0 until the payload size equals that of DCI format 3_1.