3GPP TSG RAN WG1 Meeting #110bis-e				R1-2209239

e-Meeting, October 10th – 19th, 2022

Source 	: CAICT

Title 	: Discussions on multi-cell PUSCH/PDSCH scheduling with a single DCI

Agenda Item	: 9.9.1

Document for	: Discussion / Decision

- Introduction

In the previous meetings, multi-cell PUSCH/PDSCH scheduling with a single DCI was discussed and the achieved agreements are listed in the appendix. In this contribution, we will continue to discuss the aspects about this item.

- DCI field design

  - DCI field types

In the previous meeting, it was agreed to discuss DCI format 0_X/1_X as the three types shown below:

Agreement

For discussing field design of DCI format 0_X/1_X which schedules more than one cell, reformulate the types of DCI fields as below:

- Type-1 field:

  - Type-1A field: A single field indicating common information to all the co-scheduled cells

  - Type-1B field: A single field indicating separate information to each of co-scheduled cells via joint indication

  - Type-1C field: A single field indicating an information to only one of co-scheduled cells

- Type-2 field: Separate field for each of the co-scheduled cells

- Type-3 field: Common or separate to each of the co-scheduled cells, or separate to each sub-group, dependent on explicit configuration.

  - Note: One sub-group comprises a subset of co-scheduled cells where a single field is commonly applied to the co-scheduled cell(s) belonging to a same sub-group.

- Note: Handling of any parameters applicable to multi-cell scheduling where corresponding fields are not included in DCI format 0_X/1_X (if any) will be separately discussed.

The CRC bits had better be designed as a type-1 field since separate 24-bit CRC field for each of the co-scheduled cells would make the DCI much redundant. Correspondingly, the CRC bits would be scrambled by one RNTI. It is noticed that in the current specification some of the bit fields are interpreted according to the RNTI. For example, the interpretation of“Modulation and coding scheme”is based on whether the CRC of PDCCH is scrambled by MCS-RNTI or C-RNTI, and the MCS table related higher layer parameter configuration. The meaning of “New data indicator” for PUSCH scheduling depends on whether the CRC is scrambled by C-RNTI or CS-RNTI. Then, one RNTI in the PDCCH means unified interpretation for these fields for each scheduled PDSCH/PUSCH. Then the flexibility of the scheduled multiple PUSCH/PDSCH would be much restricted due to unique RNTI is used. Further enhancements should be considered to overcome this restriction.

Observation 1: If the CRC bit filed is type-1, the flexibility of the scheduled multiple PUSCH/PDSCH would be restricted due to unique RNTI is used.

Proposal 1: Further enhancements are considered to overcome the scheduling restriction if CRC bit filed is designed as a type-1 field.

A possible solution is to consider the CRC bit filed as Type-1B field. Through multiple predefined RNTIs, different combination of the scheduled PUSCH/PDSCHs modes could be indicated. For example, the DCI format 1_X is used to schedule 2 cells. When predefined RNTI-1 is used to scramble the CRC bits, the corresponding scheduled PDSCHs of Cell-1 and Cell-2 are SPS PDSCH and D-PDSCH respectively. While if predefined RNTI-2 is used, the corresponding scheduled PDSCHs of Cell-1 and Cell-2 are both D-PDSCH.

Proposal 2: CRC bit filed in DCI format 0_X/1_X is considered as Type-1B field. Multiple predefined RNTIs are used to discriminate different scheduling modes of the scheduled PDSCH/PUSCH by DCI format 0_X/1_X.

2.2	Indication of scheduled cells

About the indication of scheduled cells, there are the following achieved agreements in the previous meeting:

Agreement

For multi-cell scheduling, the co-scheduled cells are indicated by DCI format 0_X/1_X. At least the following options are considered:

- Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells.

  - The table is configured by RRC signaling.

  - FFS: Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 2: An indicator in the DCI is a bitmap corresponding to a set of configured cells that can be scheduled by the DCI 0_X/1_X

  - FFS: Separate sets of configured cells for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 3: using existing field (e.g., CIF, FDRA) to indicate whether one or more cells are scheduled or not

- Other options are not precluded.

Note: It does not preclude other DCI information fields (e.g., BWP) to be jointly indicated by the indicator of the co-scheduled cells.

Option 2 provides much flexibility about the scheduled cells. However, the corresponding overhead would also be heavy. If the maximum number of co-scheduled cells is 8, one 8-bit field would be needed for the indication. To balance the overhead and scheduling flexibility, option 1 is proposed.

Considering both intra-band and inter-band CA operation, both FR1 and FR2 scheduling are to be supported by the new DCI for multi-cell PUSCH/PDSCH scheduling, the length of bit field for each cell could be quite different. For example, if there are N1 bits and N2 bits required for cell-1 scheduling and cell2 scheduling respectively. The bit length for cell-3 scheduling could be comparable with N1+N2. Therefore, it is beneficial to have a unified DCI bit fields for scheduling of cell-1 and cell-2 with the DCI for scheduling of cell-3 considering PDCCH detection simplicity. Also it is beneficial that most of type-2 bit fields for each scheduled cell are configurable. One indicator in the DCI could indicate the length of type-2 bit fields for each scheduled cell. With proper configuration, the indicator for the row index of scheduled cells and the indicator for the length of type-2 bit fields for each scheduled cell could be jointly designed. The Fig.1 below is an example. The “Flag” field in the DCI indicates a row which includes preconfigured bit length for each scheduled cell. Corresponding to “Flag=0”, there are N10 bits of type-2 fields for cell 1, N20 bits of type-2 fields for cell 2, N30 bits of type-2 fields for cell 3. If 0 bits of type-2 fields are included for one cell, it means this cell is not scheduled. Aligned length of bit fields as N10+N20+N30=N11+N21+N31=N12+N22+N32 for each configuration would facilitate PDCCH blind detection.

Considering the multi-cell PDSCH scheduling and multi-cell PUSCH scheduling are independent with each other, it is proposed separate tables are configured.

Fig. 1 configured bit fields in the DCI for each scheduled cell

Proposal 3: A table defining combinations of the length of bit fields for each cell scheduling is preconfigured. One flag in the DCI indicates one row of the table which implicitly indicates the scheduled cells.

- HARQ enhancements

HARQ enhancements to support multi-cell PUSCH/PDSCH scheduling with a single DCI was discussed in the previous meeting and there is the following working assumption:

Agreement

When UE detects a DCI format 1_X scheduling a set of PDSCHs, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k, where k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format and n is the last UL slot overlapping with the DL slot nD for the reference PDSCH reception for slot-based PUCCH or an UL slot overlapping with the end of the reference PDSCH reception in DL slot nD for sub-slot based PUCCH.

- FFS details of reference PDSCH

The last PDSCH among the scheduled PDSCH is preferred to be the reference PDSCH to narrow the range of K1 value down.

Proposal 4: The reference PDSCH is the last one among these PDSCHs.

Agreement

- For Type-2 HARQ-ACK codebook, two sub-codebooks are generated with a first sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single cell and a second sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling more than one cell.

- Separate DAI counting for DCI(s) with each scheduling a single cell and DCI(s) with each scheduling more than one cell.

- FFS whether a DCI scheduling more than one cell is associated with the first sub-codebook or the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

- Type-2 HARQ-ACK codebook is generated by concatenating the first sub-codebook and the second sub-codebook.

- If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling,

  - FFS: the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell;

- Otherwise, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

- HARQ-ACK information bits for co-scheduled PDSCHs by a DCI format 1_X is ordered based on serving cell indices associated with co-scheduled PDSCHs.

- HARQ-ACK bundling across co-scheduled cells is not supported for multi-cell scheduling.

When the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one, it is preferred the corresponding HARQ-ACK is included in the first sub-codebook since only HARQ-ACK information for the scheduled PDSCH is needed. Otherwise, the number of HARQ-ACK bits associated with this DCI depends on the maximum number of cells which can be co-scheduled by a DCI format 1_X which is inefficient.

Proposal 5: When the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one, the corresponding HARQ-ACK is included in the first sub-codebook.

- Conclusion

In this contribution, we discussed the multi-cell PUSCH/PDSCH scheduling with a single DCI. The following observations and proposals are reached:

Observation 1: If the CRC bit filed is type-1, the flexibility of the scheduled multiple PUSCH/PDSCH would be restricted due to unique RNTI is used.

Proposal 1: Further enhancements are considered to overcome the scheduling restriction if CRC bit filed is designed as a type-1 field.

Proposal 2: CRC bit filed in DCI format 0_X/1_X is considered as Type-1B field. Multiple predefined RNTIs are used to discriminate different scheduling modes of the scheduled PDSCH/PUSCH by DCI format 0_X/1_X.

Proposal 3: A table defining combinations of the length of bit fields for each cell scheduling is preconfigured. One flag in the DCI indicates one row of the table which implicitly indicates the scheduled cells.

Proposal 4: The reference PDSCH is the last one among these PDSCHs.

Proposal 5: When the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one, the corresponding HARQ-ACK is included in the first sub-codebook.

Reference

- R1-2208048, “Feature lead summary #6 on multi-cell PUSCH/PDSCH scheduling with a single DCI”

- RP-220834, “Revised WID on Multi-carrier enhancements”

- 3GPP TS 38.212 V17.0.0, “Multiplexing and channel coding”

Appendix A: Agreements made on the topic of “PUCCH carrier switching for HARQ feedback” in the RAN1 #109-e meeting

Agreement

Agree the following terminologies ONLY for convenience of discussion:

- DCI format 0_X is used for scheduling multiple PUSCHs on multiple cells with one PUSCH per cell

- DCI format 1_X is used for scheduling multiple PDSCHs on multiple cells with one PDSCH per cell.

The above does not imply introducing new DCI format(s) at this point.

Agreement

- Different TBs are scheduled on different cells by DCI format 0_X.

- Different TBs are scheduled on different cells by DCI format 1_X.

Agreement

Fallback DCI (i.e., DCI formats 0_0 and 1_0) does not support multi-cell scheduling.

Agreement

The DCI for multi-cell scheduling is monitored only in USS set.

Agreement

- PDSCH cannot be scheduled by DCI format 0_X.

- PUSCH cannot be scheduled by DCI format 1_X.

Agreement

- All the co-scheduled cells by a DCI format 1_X and the scheduling cell are included in the same PUCCH group.

- FFS: All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same [cell or PUCCH group].

Agreement

- DCI format 0-X/1-X on a scheduling cell can be used to schedule PUSCHs/PDSCHs on multiple cells including the scheduling cell.

- DCI format 0-X/1-X on a scheduling cell can be used to schedule PUSCHs/PDSCHs on multiple cells not including the scheduling cell.

Agreement

- For a UE, the maximum number of cells scheduled by a DCI format 0_X can be same or different to the maximum number of cells scheduled by a DCI format 1_X.

Working Assumption

- All HARQ-ACK codebook types (Type-1/2/3) are applicable when multi-carrier PDSCH scheduling is configured.

Agreement

- One value for the maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is selected from {3, 4, 8}.

- For a UE, the maximum number of co-scheduled cells by a DCI format 0_X can be smaller than or equal to the maximum number supported in Rel-18.

Agreement

- One value for the maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is selected from {3, 4, 8}.

- For a UE, the maximum number of co-scheduled cells by a DCI format 1_X can be smaller than or equal to the maximum number supported in Rel-18.

Agreement

- (Working assumption) DCI format 0_X/1_X is a new DCI format for multi-cell scheduling

- DCI format 0_X can be used for single cell PUSCH scheduling.

- DCI format 1_X can be used for single cell PDSCH scheduling.

- FFS: UE monitors one of or both multi-cell scheduling DCI and legacy single cell scheduling DCI for a scheduled cell.

Agreement

- DCI format 0-X/1-X can be transmitted on PCell.

- DCI format 0-X/1-X can be transmitted on a SCell at least when the DCI format 0-X/1-X does not schedule PUSCH/PDSCH on PCell.

- FFS whether a DCI format 0-X/1-X can be transmitted on an SCell if the DCI format 0-X/1-X schedules PUSCH/PDSCH on PCell.

Agreement

Further study DCI size budget including below options for multi-cell scheduling DCI:

- Option 1: Existing DCI size budget is maintained per scheduled cell.

  - Alt 1-1: DCI size budget is maintained via DCI size alignment and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-2: DCI size budget is maintained via configured size for multi-cell scheduling DCI and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-3: DCI size budget is maintained via DCI size alignment and DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

- Option 2: Existing DCI size budget is not necessarily maintained per scheduled cell.

  - Alt 2-1: DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

  - Alt 2-2: DCI size budget of multi-cell scheduling DCI is not counted per serving cell and not considered in the related serving cell specific DCI size alignment procedure, e.g., for K co-scheduled cells, gNB guarantee the total budget of 3*K DCI sizes is not exceeded.

  - Alt 2-3: voiding the “3+1” limit for multi-cell scheduling

  - Alt 2-4: the DCI size budget for DCI size alignment can be separately configured for each cell

  - Alt 2-5: DCI size budget of the scheduling cell can be increased to account for the DCI format for multi-cell scheduling. Accordingly, the DCI size budget of a scheduled cell can be reduced.

- Other options/alternatives could be considered.

Agreement

Further study BD/CCE counting for multi-cell scheduling DCI based on below options:

- Alt 1: counted on each co-scheduled cell

- Alt 2: counted only in one scheduled cell

- Alt 3: scaled down to each of co-scheduled cell according to the number of co-scheduled cells

- Alt 4: counted as part of the scheduling cell instead of each scheduled cell

- Alt 5: scaled down to each of scheduled cells excluding scheduling cell

- Alt 6: counted on each co-scheduled cell excluding scheduling cell

- Other alternatives could be considered.

Agreement

For multi-cell scheduling, the co-scheduled cells are indicated by DCI format 0_X/1_X. At least the following options are considered:

- Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells.

  - The table is configured by RRC signaling.

  - FFS: Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 2: An indicator in the DCI is a bitmap corresponding to a set of configured cells that can be scheduled by the DCI 0_X/1_X

  - FFS: Separate sets of configured cells for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 3: using existing field (e.g., CIF, FDRA) to indicate whether one or more cells are scheduled or not

- Other options are not precluded.

- Note: It does not preclude other DCI information fields (e.g., BWP) to be jointly indicated by the indicator of the co-scheduled cells.

Agreement

For design of multi-cell scheduling DCI, companies are encouraged to consider following types of DCI fields:

- Type-1 field: A single field indicating common information to all the co-scheduled cells or separate information to each of co-scheduled cells via joint indication or an information to only one of co-scheduled cells

- Type-2 field: Separate field for each of the co-scheduled cells, or each sub-group comprising one or more co-scheduled cells where a single field is commonly applied to the co-scheduled cells belonging to a same sub-group

- Type-3 field: Common or separate to each of the co-scheduled cells or to each sub-group.

  - FFS: whether it is dependent on explicit configuration or implicit condition (e.g., intra or inter band CA, FR1 or FR2).

- Other types are not precluded.

Agreement

All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same PUCCH group.

Agreement

- Confirm below working assumption reached in RAN1#109e meeting.

  - (Working assumption) DCI format 0_X/1_X is a new DCI format for multi-cell scheduling

Working assumption:

For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell.

- The DCI format 0_X/1_X and the legacy DCI format(s) can be monitored simultaneously.

- FFS: whether monitoring of the DCI format 0_X/1_X and the legacy DCI format(s) is supported for one, a subset, or all cells within the set of cells.

- FFS: number of different DCI sizes for 0_X/1_X and for legacy DCI formats

- FFS: whether to support a subset or all legacy DCI format(s) to be monitored with DCI 0_X/1_X

Working assumption:

- The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.

- The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

FFS: The maximum number of configurable cells for co-scheduling

Agreement

For discussing field design of DCI format 0_X/1_X which schedules more than one cell, reformulate the types of DCI fields as below:

- Type-1 field:

  - Type-1A field: A single field indicating common information to all the co-scheduled cells

  - Type-1B field: A single field indicating separate information to each of co-scheduled cells via joint indication

  - Type-1C field: A single field indicating an information to only one of co-scheduled cells

- Type-2 field: Separate field for each of the co-scheduled cells

- Type-3 field: Common or separate to each of the co-scheduled cells, or separate to each sub-group, dependent on explicit configuration.

  - Note: One sub-group comprises a subset of co-scheduled cells where a single field is commonly applied to the co-scheduled cell(s) belonging to a same sub-group.

- Note: Handling of any parameters applicable to multi-cell scheduling where corresponding fields are not included in DCI format 0_X/1_X (if any) will be separately discussed.

Agreement

For DCI format 1_X/0_X which can schedule more than one cell,

- Type-1 fields at least include below:

  - Type-1A:

    - Identifier for DCI formats

    - Downlink assignment index

    - TPC for scheduled PUCCH

    - PUCCH resource indicator

    - PDSCH-to-HARQ timing indicator

    - One-shot HARQ-ACK request

- Type-2 fields at least include below:

  - New data indicator per TB

  - Redundancy version per TB

- FFS: Other fields to be included in DCI format 1_X/0_X and which type of the fields belongs to.

- FFS: size for each field

Agreement

When UE detects a DCI format 1_X scheduling a set of PDSCHs, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k, where k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format and n is the last UL slot overlapping with the DL slot nD for the reference PDSCH reception for slot-based PUCCH or an UL slot overlapping with the end of the reference PDSCH reception in DL slot nD for sub-slot based PUCCH.

- FFS details of reference PDSCH

Agreement

- For Type-2 HARQ-ACK codebook, two sub-codebooks are generated with a first sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single cell and a second sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling more than one cell.

- Separate DAI counting for DCI(s) with each scheduling a single cell and DCI(s) with each scheduling more than one cell.

- FFS whether a DCI scheduling more than one cell is associated with the first sub-codebook or the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

- Type-2 HARQ-ACK codebook is generated by concatenating the first sub-codebook and the second sub-codebook.

- If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling,

  - FFS: the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell;

- Otherwise, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

- HARQ-ACK information bits for co-scheduled PDSCHs by a DCI format 1_X is ordered based on serving cell indices associated with co-scheduled PDSCHs.

- HARQ-ACK bundling across co-scheduled cells is not supported for multi-cell scheduling.

Agreement

UE does not expect to be configured both CBG-based PDSCH/PUSCH transmission and the multi-cell PDSCH/PUSCH scheduling on the same or different cells within a same PUCCH group.

Agreement

- At least cases 1-1 and 1-2 on SCS are supported:

  - Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell.

  - Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.

  - Case 1-3: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and different SCS is used among the co-scheduled cells including the scheduling cell.

  - Case 1-4: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and different SCS is used among the co-scheduled cells.

  - FFS: Whether Case 1-3 or 1-4 is additionally supported.