3GPP TSG RAN WG1 #110bis-e	R1-2209367

e-Meeting, 10-19 October 2022

Agenda item:		9.2.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation of ML for CSI feedback enhancement

Document for:		Discussion and Decision

# Introduction

In RAN#110 [1], the following agreements and conclusions were made regarding CSI feedback enhancement:

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, between GCS and SGCS, SGCS is adopted

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, it is up to companies to choose the error modeling method for realistic channel estimation and report by willingness.

- Note: It is not precluded that companies use ideal channel to calibrate

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, the throughput in the ‘Evaluation Metric’ includes average UPT, 5%ile UE throughput, and CDF of UPT.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the specific quantization/dequantization method, e.g., vector quantization, scalar quantization, etc.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, the capability/complexity related KPIs, including FLOPs as well as AI/ML model size and/or number of AI/ML parameters, are to be reported separately for the CSI generation part and the CSI reconstruction part.

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

  - Note: Companies to report the ratio for dataset mixing

  - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- FFS other cases for generalization verification, e.g.,

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Agreement

For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point:

- Various deployment scenarios (e.g., UMa, UMi, InH)

- Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10)

- Various carrier frequencies (e.g., 2GHz, 3.5GHz)

- Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc.

- Companies to report the selected scenarios for generalization verification

Agreement

For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point:

- Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband)

- Various sizes of CSI feedback payloads, FFS candidate payload number

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc.

- Companies to report the selected configurations for generalization verification

- Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

For the CSI prediction sub-use case evaluation, the following conclusions were reached in RAN#110 [1]:

Conclusion

If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, consider CSI prediction involving temporal domain as a starting point.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for evaluation,

- 100% outdoor UE is assumed for UE distribution.

  - FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles

- UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h

  - Note: Companies to report the set/subset of speeds

- 5 ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, companies are encouraged to report the details of their models for evaluation, including:

- The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

- The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

- The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

In this contribution, we continue the discussion of the evaluation and address open issues. We discuss both CSI feedback compression with autoencoders and CSI prediction.

# Discussion

## CSI feedback with Autoencoders

### Evaluation Methodology

#### Intermediate KPI’s

Table 1:  SLS Parameters

| Parameter | Value |
| --- | --- |
| Simulation scenario | UMa |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Num. cell sites | 7 |
| BS antenna height | 25m |
| Distribution of UEs (indoor, outdoor pedestrian, outdoor car) | 80, 20, 0 |
| UE speed | 3 km/h |
| Macrocell inter-site distance | 200m |
| Channel estimation model | Realistic |
| Traffic model | FTP 1 (20%, 50%, 70% target RU) |

In our previous contribution [3], we presented full buffer SU-MIMO results comparing Methods 1 and 3 for intermediate KPI’s for ranks greater than 2 using the (unsquared) cosine similarity.  In this contribution, we update those results for MU-MIMO with bursty traffic (FTP 1), now using the squared cosine similarity (SGCS).  The system level simulations use the Rel-16 Type II codebook, where we calculate the cosine similarity of the eigenvectors of the ideal channel covariance matrix with the precoding vectors identified by the PMI.  The simulation is performed using a maximum rank of 2 with adaptive rank.  A large percentage of the UE’s are assigned rank 2.  The system level parameters in Table 1 are used with 10 UE’s per sector in each drop.  The SGCS is calculated and averaged over all of the strongest eigenvectors (layer 1) for both rank 1 and rank 2 UE’s, all of the selected eigenvectors for the second layer (from rank 2 UE’s), and averaged over all of the vectors in the fed back PMI’s (both layer 1 and 2).  The first two averages correspond to Method 3 for SGCS and the last corresponds to Method 1.  The results are shown for target resource utilizations of 20%, 50%, and 70% in Table 2, Table 3, and Table 4 where the mean and cell edge user throughputs have been normalized to the results for the first parameter combination in each table.  We find that the first layer has a significantly higher GCS than the second layer.  In Figure 1, the three metrics are plotted for the 50% target resource utilization case.  The 20% and 70% target utilization cases show similar trends.  In the figure, we see that all of the metrics have a similar relationship to throughput.  Since the overall mean SGCS (Method 1) combines all layers into a single metric, we prefer this method compared to the per layer SGCS (Method 3).

Proposal 1: Prefer Method 1 over method 3 for an intermediate KPI for rank > 1.

Table 2:  Intermediate KPI Comparison for Rank > 1 (20% RU target)

| Parameter Combination | Feedback (bits) | Mean Squared Cosine Similarity (SGCS) |  |  | Normalized Throughput |  | Resource Utilization |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Layer 1 | Layer 2 | Overall Mean | Sector Throughput | Cell Edge Throughput |  |
| 1 | 107 | 0.695 | 0.487 | 0.591 | 1.000 | 1.000 | 0.158 |
| 2 | 163 | 0.728 | 0.516 | 0.622 | 1.012 | 1.004 | 0.156 |
| 3 | 201 | 0.779 | 0.623 | 0.701 | 1.033 | 1.066 | 0.152 |
| 4 | 312 | 0.812 | 0.668 | 0.740 | 1.051 | 1.104 | 0.148 |
| 5 | 423 | 0.817 | 0.681 | 0.749 | 1.049 | 1.103 | 0.149 |
| 6 | 531 | 0.830 | 0.701 | 0.766 | 1.054 | 1.103 | 0.149 |

Table 3:  Intermediate KPI Comparison for Rank > 1 (50% RU target)

| Parameter Combination | Feedback (bits) | Mean Squared Cosine Similarity (SGCS) |  |  | Normalized Throughput |  | Resource Utilization |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Layer 1 | Layer 2 | Overall Mean | Sector Throughput | Cell Edge Throughput |  |
| 1 | 107 | 0.691 | 0.483 | 0.588 | 1.000 | 1.000 | 0.500 |
| 2 | 163 | 0.717 | 0.507 | 0.613 | 1.031 | 1.068 | 0.487 |
| 3 | 201 | 0.768 | 0.602 | 0.686 | 1.100 | 1.237 | 0.464 |
| 4 | 313 | 0.796 | 0.640 | 0.719 | 1.132 | 1.321 | 0.453 |
| 5 | 425 | 0.799 | 0.648 | 0.724 | 1.134 | 1.337 | 0.451 |
| 6 | 523 | 0.804 | 0.659 | 0.732 | 1.144 | 1.339 | 0.448 |

Table 4:  Intermediate KPI Comparison for Rank > 1 (70% RU target)

| Parameter Combination | Feedback (bits) | Mean Squared Cosine Similarity (SGCS) |  |  | Normalized Throughput |  | Resource Utilization |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Layer 1 | Layer 2 | Overall Mean | Sector Throughput | Cell Edge Throughput |  |
| 1 | 107 | 0.686 | 0.481 | 0.586 | 1.000 | 1.000 | 0.754 |
| 2 | 163 | 0.710 | 0.501 | 0.607 | 1.052 | 1.088 | 0.737 |
| 3 | 201 | 0.755 | 0.582 | 0.670 | 1.169 | 1.322 | 0.704 |
| 4 | 313 | 0.779 | 0.613 | 0.697 | 1.221 | 1.410 | 0.687 |
| 5 | 425 | 0.781 | 0.620 | 0.701 | 1.225 | 1.446 | 0.685 |
| 6 | 523 | 0.782 | 0.625 | 0.705 | 1.232 | 1.423 | 0.683 |

Figure 1: Comparison of Rank 2 SGCS KPI’s (50% RU Target)

#### SLS evaluation methodology (parameters)

The primary open issue is the type of traffic to use for system level simulations.  Since the system level simulation evaluation methodology for codebook develop has used FTP 1 traffic for past developments, this method should also be used here for a proper comparison.  For early results, we feel that full buffer traffic can be allowed, but final performance comparisons should be made based on bursty traffic using the FTP 1 model.

Proposal 2:  Adopt the FTP 1 traffic model for system level simulation evaluation of throughput performance of AI/ML-based CSI feedback enhancement.

#### Generalization

Generalization is the testing of items not included in the training and testing data set with the purpose of assessing the performance of the model when new data is encountered.  Since it is difficult for the data set used to develop the model to cover all possible situations that the model may encounter when used for inference, it is important in this study item to test the performance of models when presented with situations not included in the training dataset.  Such testing can include both additional examples from the same types of scenarios used to create the dataset as well as other scenarios.  For example, a training dataset for CSI compression might be generated from a UMa scenario.  Generalization testing should consider the performance of the model when used with additional UMa samples not included in the training dataset, but also with samples from InH, RMa, or UMi scenarios.  Generalization results can be reported for each of these cases to see how the generalization performance varies.  This kind of testing also provides information about the diversity required in the training dataset in order to provide the required performance in the field.

Initial generalization results have been obtained using the network in Figure 2 by varying the indoor/outdoor UE mix.  The training data set used a mix of 80% indoor and 20% outdoor UE’s with additional SLS training parameters and hyperparameters shown in Table 5 and Table 6.  We compare performance on data sets using a 50/50 and 20/80 mix.  Therefore, these results can be considered an example of Case 2 generalization where training has been trained with Scenario#A/Configuration#A (UMa/80% indoor, 20% outdoor) and inference has been performed with Scenario#A/Configuration#B (UMa/50% indoor, 50% indoor) and Scenario#A/Configuration#C (UMa/20% indoor, 80% outdoor).  Additional generalization tests should also be performed with data from other scenarios.  The results of these tests are shown in Table 7 and Table 8 where the GCS has been used as the KPI (all data entries in the table are mean GCS values).  Results are reported for the 80/20 split used in training as well as the 50/50 and 20/80 splits used for the generalization tests.  In addition, networks trained using the MSE and SGCS objective functions are shown (as indicated by the column headers).  The results in Table 7 are for rank 1 and the results in Table 8 are for rank 2 where the Method 1 GCS metric has been used for rank 2.  The results indicate that the network generalizes well across the indoor/outdoor splits.  However, additional tests with data from additional scenarios is desired to assess the generalization performance for data which varies more from the training data.

Proposal 3:  Include generalization tests from scenarios or configurations outside the training data set.

Figure 2: Neural network architecture for eigenvector compression

Table 5:  SLS Parameters for Training Data

| Parameter | Value |
| --- | --- |
| Simulation scenario | UMa |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Num. cell sites | 7 |
| BS antenna height | 25m |
| Distribution of UEs (indoor, outdoor pedestrian, outdoor car) | 80, 20, 0 |
| UE speed | 3 km/h |
| Macrocell inter-site distance | 200m |

Table 6:  Hyperparameters

| Parameter | Value |
| --- | --- |
| Number of training samples | 114000 |
| Learning Rate | 10-4 – 10-3 |
| Compression Rate | 4, 8, 16, 32 |
| Feedback bits | 26, 52, 104, 208, 416 |
| Epochs | 1000 |
| Batch size | 100 |
| Quantization | Scalar (Uniform) |
| Objective function | MSE, SGCS |

Table 7: Rank 1 generalization results for various indoor/outdoor splits

| Feedback (bits) | 80/20 |  | 50/50 |  | 20/80 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MSE | SGCS | MSE | SGCS | MSE | SGCS |
| 416 | 0.9642 | 0.9247 | 0.9535 | 0.9138 | 0.9735 | 0.9449 |
| 208 | 0.9202 | 0.9089 | 0.9156 | 0.9007 | 0.9409 | 0.9245 |
| 104 | 0.8831 | 0.8757 | 0.8671 | 0.8708 | 0.9105 | 0.9023 |
| 52 | 0.8110 | 0.8522 | 0.8311 | 0.8531 | 0.8515 | 0.8832 |
| 26 | 0.7677 | 0.8305 | 0.7645 | 0.8326 | 0.8358 | 0.8676 |

Table 8: Rank 2 generalization results for various indoor/outdoor splits

| Feedback (bits) | 80/20 |  | 50/50 |  | 20/80 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MSE | SGCS | MSE | SGCS | MSE | SGCS |
| 416 | 0.9624 | 0.9127 | 0.9526 | 0.8998 | 0.9362 | 0.8875 |
| 208 | 0.9178 | 0.8851 | 0.9118 | 0.8644 | 0.8962 | 0.8340 |
| 104 | 0.8756 | 0.8621 | 0.8462 | 0.8396 | 0.8211 | 0.8088 |
| 52 | 0.8281 | 0.8327 | 0.7840 | 0.7965 | 0.7396 | 0.7465 |
| 26 | 0.7835 | 0.8037 | 0.7275 | 0.7508 | 0.6706 | 0.7076 |

## CSI prediction

CSI prediction can be used to enable advanced use cases which are sensitive to CSI aging.  These use cases can include improving performance at higher UE speeds, MU-MIMO precoding, or coherent JT-CoMP.  Prediction can also be used to reduce overhead in both feedback and reference signal transmissions by reducing the number of occasions for both.  Thus, the main goal of CSI prediction is to achieve a high prediction horizon (predicting as far into the future as possible) with as low as possible prediction error.

In this section, we discuss the evaluation methodology aligned to the latest EVM agreements for CSI prediction, provide our simulation assumptions, propose a way forward for the generalization of channel prediction models, list the key performance indicators (KPIs), and evaluate some related simulation results.

### Evaluation Methodology

For channel prediction, link level as well as system level evaluations have been agreed on. The bases for all evaluations are UE tracks over a few hundreds of milliseconds or even over multiple seconds. We use either simple link-level (LL) channel models like EVA7 or EVA70 to assess specific issues or use the radio channels as agreed for the latest system-level (SL) simulations.

In accordance with the EVM agreements we evaluate the NMSE as well as cosine similarity over the channel prediction horizon. Note that the first focus of channel prediction is often related to the smooth evolution of the radio channel components and its prediction. The related Type II CSI feedback for the prediction time is then calculated at the UE based on the predicted channel components. Note that for more general evaluations this step might be omitted, and the cosine similarity is then calculated not for the precoding vectors, but for the real and imaginary parts of the channel coefficients.

One can consider different prediction methods, where either the channel predictor neural network (NN) is applied individually per each of the 16 or 32 antenna ports (APs), or a larger NN infers the prediction for all the APs in one single step. In both cases the Type II CSI feedback is then calculated as defined, e.g., in Release 16, but for the predicted instead of the current radio channels.

In another approach one might directly infer the CSI potentially using one specific NN, which would save the extra post processing step of Type II CSI calculation. At the same time, the different rules for generating Type II CSI might be a challenge with respect to ML based training of a neural network.

#### Performance Baseline

According to the latest EVM agreement, the Rel-16 eMIMO Type II codebook is a legacy feedback mode which can be used as a baseline for comparing the channel prediction performance.  At the system level the user throughput, spectral efficiency, feedback overhead, or complexity for the predicted CSI can be compared against the Rel-16 codebook without prediction at the cost of corresponding channel aging. The overhead, in this case, should include both the reference signal overhead as well as the feedback overhead, since one of the desired effects of CSI prediction is to reduce not only the number of CSI feedback occasions, but also the number of CSI-RS transmissions required to meet a certain performance level.

Kalman filtering can also be considered as a performance baseline for non-ML based channel prediction.

Proposal 4: Adopt the Rel-16 eType II codebook without prediction as the baseline for performance comparisons of ML-based CSI prediction, considering both throughput and overhead, where the overhead includes both reference signal and feedback overhead.  Kalman filtering can also be considered as a performance baseline.

#### Generalization

Generally, mobile radio cells experience strong variations in radio channel conditions including indoor, outdoor, LOS, NLOS, or obstructed LOS scenarios, etc. One option to adapt to these strong channel variations is to use scenario-specific trained AI/ML models. This leads to a more complex system design and requires maintaining a potentially large number of trained and verified AI/ML models.

Alternatively, one can try to train generalized AI/ML models with data sets from all potential and expected channel conditions to reduce the number of required AI/ML models. In that case one must expect some performance degradations compared to ML models trained by specialized scenario specific data sets.

The latest agreement regarding generalization defines three cases of scenario and configuration variations for verification of such generalized AI/ML models.

For channel prediction we propose a specific variant of generalization as agreed in the last meeting, which is not only for the verification process of the generalized AI/ML model, but results in a specialized inference method relying on one more or less well generalized AI/ML model.

We refer to the case that the generalized AI/ML is first trained on Scenario#A/Configuration#A and then updated based on a fine-tuning data set different than Scenario#A/Configuration#A.

Conventional channel prediction such as Kalman filtering typically requires a long channel observation time over ideally hundreds of ms to learn the state space model for the current UE location. Only after that will a suitable channel prediction based on this state space model become possible. One could say that Kalman filters must learn the radio channel from scratch for every new UE activation.

Here, we observe one interesting benefit of AI/ML based channel prediction as it can be applied after only a few, e.g., three channel estimates based on semi-persistent CSI RSs with a repetition rate of, for example, 5 ms. This enables channel prediction already after, e.g. 15 ms. But, corresponding to our observations, the channel prediction quality of the generalized AI/ML model varies over different radio channel conditions. Even if the training data set had been large including already similar channels from Scenario#A/Configuration#A we see such variations. More importantly, the generalized AI/ML model for Scenario#A/Configuration#A has often a performance gap compared to a fully specialized AI/ML model for the specific current channel conditions of a certain UE. With a specialized AI/ML model we refer to an AI/ML model which has been overfitted to the current radio channel evolution of the UE by using the channel estimates of the last 50 ms, 100 ms, or several hundreds of ms as a data set for fine tuning the AI/ML model. Here, we see then the close relation to Kalman filtering, which always requires such long channel observations, while for the AI/ML based approach it can be used for optimizing the channel prediction quality.

The performance gap between the generalized AI/ML model and the specialized model varies for different UE positions within the radio cell; i.e., for some UEs there is almost no gain due to fine tuning, while other UEs observe large gaps between the generalized and the specialized AI/ML model. For that reason, we propose that the UE first estimates the potential performance gain due to fine tuning, which can be done based on the latest radio channel estimates. The UE might then report to the gNB an indication which allows the gNB to adjust the CSI-RS repetition rate based on how close the generalized AI/ML model performs relative to the best possible AI/ML model. The gNB can then configure, depending on the indication, a semi static set of CSI RS transmissions, where the length and the repetition rate is increased when performance is now as close to the best possible model so that the UE has a longer data set for fine tuning. This single data set for fine tuning is then configured to allow for a fast and efficient fine tuning of the generalized AI/ML model over a single epoch.

The proposed method provides benefits similar to conventional Kalman filtering from the optimal adaptation to the current UE channel conditions by specialization of the AI/ML model. At the same time, it minimizes the required channel observation time as far as possible so that in cases where the performance is close to the best possible only short or even no fine tuning is needed at all. Correspondingly, the overhead for transmission of CSI-RSs and the execution latency for the channel prediction is minimized and can be adapted based on higher layer criteria.

The concept allows a UE to cover a wide range of Scenarios/Configurations with a single or very few generalized AI/ML models due to the built-in adaptation capabilities.

We should note that specialization can be seen as an overfitting to the current channel conditions of a single UE. For that reason, the purpose of the fine tuning of the generalized AI/ML model is not to update the generalized AI/ML model as such, but to use it for channel prediction of this specific UE under the current channel conditions.

Proposal 5:  Consider specialized AI/ML models based on one or few generalized AI/ML models to achieve highest channel prediction performance with minimum number of AI/ML model versions.

#### KPIs

As already indicated, both throughput and overhead are KPI’s for system level performance of CSI prediction. Another important KPI for CSI prediction is the prediction horizon, which is typically a measure of the normalized mean square error (NMSE) of the predicted CSI over time and/or spatial movement of the UE relative to one RF wavelength. A typical target value might be –20 dB at 10 ms, which would support high-end precoding schemes at least for nomadic users, or users with moderate mobility of less than 15 km/h.

Another relevant KPI for channel prediction is the execution latency, as CSI information is outdated extremely fast.  The execution latency includes not only the inference time, but also the time required to do any preprocessing of the CSI-RS signal and post-processing after inference (see also our discussion in [2]). Obviously, the execution latency should be significantly smaller than the intended, or achievable prediction time. Therefore, ideal values would be in the range of one to less than 10 ms for the FR1 frequency range below 6 GHz. Channel prediction for the FR2 bands becomes even more challenging due to the shorter wavelength, but also less relevant due to lower number of multipath components per narrow beam.

A third relevant KPI is then the processing complexity, which is related to the latency and to the required CSI accuracy. Relative complexity of an ML-based predictor should be compared to conventional methods, such as legacy (Rel-16) codebook-based methods for obtaining CSI and methods such as Kalman filtering.

Another sub-use case specific KPI is the required observation time tobserve, which might range for different prediction methods from a few ms to 500 ms or even seconds. Note that methods which achieve a high prediction horizon with few channel observations reduce the overhead for CSI-RS and are more practical than methods which require a channel observation over 500 ms.

The proposed KPIs for CSI prediction are summarized in Table 9.

Table 9: Proposed KPIs for CSI Prediction

| KPI | Measured in | Target Value |
| --- | --- | --- |
| System-level throughput (mean and cell-edge) | Spectral efficiency (bits/sec/Hz) |  |
| Feedback overhead | Bits |  |
| Reference signal overhead | Transmissions per second |  |
| Channel prediction horizon | NMSE over the prediction time or, alternatively, cosine similarity in case of PMI prediction | -20dB at 10 ms (for NMSE) |
| Execution latency | Time (ms) between CSI-RS transmission to transmission of the predicted CSI | < 1 to 10 ms |
| Processing complexity | FLOPS, tic toc (MATLAB), memory size, number of NN weights, quantization of NN weights, … |  |
| Observation time | Minimum channel observation time needed, e.g., minimum number of CSI-RS measurements needed |  |

Proposal 6: Adopt specific KPIs related to channel prediction such as throughput, overhead, channel prediction horizon, observation time, complexity, and execution latency.

### Simulation Results

In the following, we provide a few simulation results to discuss some specific aspects like the impact of the number of observation time steps, the number of observed subcarriers in the frequency domain, and the benefit of interpolation in the time domain.

The most basic assumption for channel prediction is to do it per AP or per beam pair, where the best beam pairs between UE and gNB are then inferred from the related beam management, which is not part of this sub-use case. To each AP, the gNB regularly transmits the AP-specific CSI-RS. Ideally, for optimum channel estimation and prediction performance, the gNB transmits the CSI-RS over the full RF bandwidth, i.e., over all PRBs.

Obviously, this needs a sequence of consecutive CSI-RS signals tracked by the moving UE, which should follow some predefined spatial tracks.

Some initial channel prediction methods have been implemented based on a single neural network as described above and given in Table 10.

Table 10: CSI Prediction Parameters

| Parameter | Value |
| --- | --- |
| Training data | Simulated channels EVA7 using the MATLAB LTE Toolbox |
| ML algorithm | RNN / LSTM: 10 LSTM cells followed by one dense layer Activation function: tanh Training epochs: 25 Batch size: 1 sample Loss function: MSE |
| Input | Estimated downlink channel H, k time instance |

In Figure 3 we present the results for CSI prediction with the LSTM NN and compare its performance with a Kalman filter, which is claimed to be the optimum theoretic solution for channel prediction. The channels are simulated using the MATLAB LTE Toolbox. The R.13 channel configuration is assumed, with a single antenna at the gNB and UE, and channel model EVA7, maximum Doppler frequency of 7Hz. A total of 50 random seeds are used and 50 OFDM frames are collected per seed. The LSTM is trained to do a prediction of one time-step ahead after watching the last three time-steps.

The Kalman filter uses an autoregressive model of order three and updates the state-space equations for the prediction. As can be observed in Figure 3, the LSTM has a better performance for the 5 ms prediction time instance (one time-step ahead). For the second time-step prediction, the LSTM has slightly worse performance than the Kalman filter. However, the LSTM was not trained for a two-step prediction.

Figure 3: Channel prediction with Kalman filter (blue) and LSTM (red) considering a SISO-OFDM channel with maximum Doppler frequency of 7Hz.

Evaluation of data preprocessing

For evaluation of the impact of the input data processing we reuse the EVA7 channel model as provided by the MATLAB LTE Toolbox for some LL simulations. Specifically, we examine the effect of decreasing the sample time of the channel response (more channel samples over time) and increasing the observation bandwidth.  We have a channel sample time of 0.5 ms and use either 20 or 30 time-domain input samples. As long as the Nyquist criterion is satisfied for the UE speed, sampling rates such as this can be reached from coarser sampling through interpolation. Figure 4 illustrates the related convolutional LSTM NNs, which have been adapted to the different input sequences.  Since the NN can predict the channel evolution at multiples of the channel sample time, 10 outputs are required to get a prediction at the intended 5 ms prediction horizon.

From Figure 5 we can identify the impact of the channel observations in time and frequency. In line with theoretical expectations the Fisher information increases with increasing number of channel observations. Correspondingly, the theoretical Kramer Rao Lower bound of the minimum channel estimation error for any unbiased channel estimator is improved as well. For example, increasing the number of frequency domain input subcarriers from 16 to 100 so that the observation bandwidth increases from 1.44 MHz to 9 MHz improves the NMSE by almost 7 dB. Increasing the number of time domain samples from 20 to 30 brings another 2 dB.

Observation 1:  Channel prediction performance improves significantly as the observation bandwidth increases and as the time step between measurements decreases.

The related complexity increased for 20 versus 30 time-domain input samples from 3624 to 5208 trainable parameters, which is an increase of 43%. The number of frequency domain samples does not have any impact on the number of trainable parameters.

Observation 2: The complexity due to higher number of time domain and frequency domain channel samples increases only moderately.

Especially, a high frequency domain observation bandwidth is beneficial due to its low impact on the number of trainable parameters and due to the low related latency. By a proper design with cell specific CSI RSs the CSI RS overhead for a high bandwidth can be small.

Proposal 7: Support high number of frequency-domain and of time-domain channel samples for training and inference of NNs for channel prediction.

Figure 4: High level illustration of convolutional LSTM NN models adapted for 20 input time domain signals (left) and 30 input domain signals (right). The output signals are in both cases 10 time-domain signals.

Figure 5: CDF of the NMSE for either 20 or 30 input time domain signals as well as either 16 or 100 input frequency domain CSI reference signals. The CSI reference signals are spaced by 6 subcarriers so that for a subcarrier spacing of 15 kHz we have an input bandwidth of either 1.44 MHz or of 9 MHz.

Evaluation of channel prediction in UMa Scenario

Given in Table 11 are the simulation parameters which follow the latest EVM agreement for SL simulations for the UMa scenario as well as the main ML parameters.

Table 11: CSI Prediction Parameters for UMa scenario

| Parameter | Value |
| --- | --- |
| Training data | 21 tracks corresponding to EVM agreement for Rel 16 UMa, RF = 2GHz, ISD = 200m, UE mobility = 30kmph, Channel sampling period 5 ms |
| ML algorithm | RNN / convolutional LSTM Activation function: tanh Training epochs: 25 Batch size: 1 sample Loss function: MSE, cosine similarity on real and imaginary channel coefficients |
| Input | Estimated downlink channel H, k time instance |

We have the following dataset pre-processing:

- Every Rx-Tx pair, i.e., every antenna port (AP) out of overall 16 APs for estimating the massive MIMO antenna, is considered a new channel realization: SISO-OFDM assumption.

- Each UE track is up-sampled by a factor of 5 to increase the time resolution of the input signal from the channel sample period of tsample= 5 ms to 1 ms. Note that with tsample= 5 ms the Nyquist criterion for the UE mobility of 30 kmph is still fulfilled.

- 50% of all tracks are used for training, the other 50% are for testing.

- 20% of the training dataset is used for validation.

For clarity Figure 6 illustrates how the radio channels of 21 UE tracks over 5s each have been allocated to training validation and testing.

Figure 6: 21 tracks of mobile UEs used for training, validation and testing of the convolutional LSTM NN.

Description of the overall evaluation setup:

- The input signals for training, validation, and testing consist of 15 ms of channel coefficients – 15 channel coefficients, 16 PRBs, 2 real/imaginary parts.

- In Figure 7 we see – compared to the LSTM in the previous section – the slightly updated convolutional LSTM NN structure, which avoids the last dense layer and has been optimized for the UMa channel conditions as defined for Rel 16 and partly given in Table 9.

- The channel prediction horizon is 5 ms for the channel coefficients, i.e., five channel coefficients with 1 ms resolution.

- The number of trainable parameters for the architecture in Figure 7 is 4368.

- Note that the number of PRBs used does not affect the number of trainable parameters.

- According to TensorFlow the number of FLOPS is 129472.

Figure 7: AI/ML architecture for channel prediction adapted to UMa channels corresponding to latest EVM agreement.

The channel prediction quality per AP for the maximum prediction time of 5 ms is given in Figure 8, where the NMSE is on average better than -15dB and the cosine similarity close to 0.99.

Observation 3: Performance results for CSI prediction using the agreed evaluation methodology conditions continue to indicate promising performance for this use case.

Figure 8: Channel prediction with convolutional LSTM for one single AP. Left: CDF of NMSE and right cosine similarity of real and imaginary part of the channel coefficients.

Evaluation of channel prediction with specialized AI/ML models in UMa Scenario

We take the same UMa scenario as agreed for the EVM and given in Table 11, but now with 400 user tracks. For illustration of the potentially large effect of the specialization operation as discussed above in section 2.2.1.2, we evaluate first a single UE. In Error! Reference source not found. we see the CDF of the normalized squared error for a channel prediction 5 ms ahead and the AI/ML architecture as well as data pre- and post-processing as given in the previous subsection.

The solid lines in Figure 9 are for the channel prediction with the generalized AI/ML model but evaluated only for one single UE track. We observe that the red line is somewhat improved compared to the blue line, which is due to the longer time used for training of 4.875s versus otherwise only 125 ms.

The dashed lines are for the same UE track, but now including the specialization of the AI/ML model again over a time of 4.875s versus 125 ms. We see that fine tuning of the generalized AI/ML model improves the performance significantly by more than ten dB in this case and longer time durations for the fine tuning data set helps as well. Note that this UE has been selected as it shows such large performance gains, while there are other UEs which are already close to optimum without any specialization.

Observation 4: Additional fine tuning of the CSI prediction model can substantially improve the performance of the CSI prediction for certain UEs.

Figure 9: Cumulative distribution function (CDF) of the NSE when specializing the weights of the CSI predictor to a certain UE track that was unseen during pre-training. A channel duration of 125 ms is taken to re-train the AI/ML model, the remaining 4.875s of the UE track simulation is used for testing. Solid lines denote the output of the CSI predictor before fine tuning and dashed lines represent the output performance after fine tuning with 125 ms channel duration.

To get an idea of the potential effect of channel prediction on the system level we used the UMa scenario as described in the EVM with the NMSE as the intermediate KPI once with zero order hold (ZOH) and once with a 5 ms prediction with results shown in Figure 10. These results already include the specialization operation for a single generalized AI/ML model. Similarly, Figure 11 provides the CDFs of the NMSE for a channel prediction time of 10 ms, where the channel prediction provides a gain of roughly 7 dB over ZOH.

Observation 5: CSI prediction using AI/ML provides substantial performance gain at 5 and 10 ms prediction times compared with zero-order hold.

Figure 10: CDF of the NMSE with channel prediction 5 ms ahead compared to ZOH over 5 ms.

Figure 11: CDF of the NMSE with channel prediction 10 ms ahead compared to ZOH over 10 ms.

# Conclusion

In this contribution, we have addressed evaluation issues for both CSI feedback with autoencoders and CSI prediction with AI/ML. Our observations and proposals are:

For CSI feedback with autoencoders:

Proposal 1: Prefer Method 1 over method 3 for an intermediate KPI for rank > 1.

Proposal 2:  Adopt the FTP 1 traffic model for system level simulation evaluation of throughput performance of AI/ML-based CSI feedback enhancement.

Proposal 3:  Include generalization tests from scenarios or configurations outside the training data set.

For CSI prediction:

Proposal 4: Adopt the Rel-16 eType II codebook without prediction as the baseline for performance comparisons of ML-based CSI prediction, considering both throughput and overhead, where the overhead includes both reference signal and feedback overhead.  Kalman filtering can also be considered as a performance baseline.

Proposal 5:  Consider specialized AI/ML models based on one or few generalized AI/ML models to achieve highest channel prediction performance with minimum number of AI/ML model versions.

Proposal 6: Adopt specific KPIs related to channel prediction such as throughput, overhead, channel prediction horizon, observation time, complexity, and execution latency.

Observation 1:  Channel prediction performance improves significantly as the observation bandwidth increases and as the time step between measurements decreases.

Observation 2: The complexity due to higher number of time domain and frequency domain channel samples increases only moderately.

Proposal 7: Support high number of frequency-domain and of time-domain channel samples for training and inference of NNs for channel prediction.

Observation 3: Performance results for CSI prediction using the agreed evaluation methodology conditions continue to indicate promising performance for this use case.

Observation 4: Additional fine tuning of the CSI prediction model can substantially improve the performance of the CSI prediction for certain UEs.

Observation 5: CSI prediction using AI/ML provides substantial performance gain at 5 and 10 ms prediction times compared with zero-order hold.

# References

- “Draft Report of 3GPP TSG RAN WG1 #110 v0.2.0 (Toulouse, France, 22nd-26th August 2022),” MCC Support.

- R1-2209366, “Further discussion on the general aspects of ML for Air-interface,” Nokia, Nokia Shanghai Bell, 3GPP RAN1#110bis-e, 10-19 October 2022.

- R1-2206968, “Evaluation of ML for CSI feedback enhancement,” Nokia, Nokia Shanghai Bell, 3GPP RAN1#110, Toulouse, France, 22-26 August 2022.