3GPP TSG RAN WG1 #110-bis-e			R1-2209721

e-Meeting, October 10th– October 19th, 2022

Agenda item:	9.2.1

Source:	Samsung

Title:	General aspects of AI ML framework and evaluation methodology

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, Rel-18 new study item on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” is endorsed. The objective of the study item is as follows.

AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

Furthermore, in the first RAN 1 meeting, i.e., RAN1#109-e, some basic terminologies are defined. Moreover, an initial categorization for network-UE collaboration levels is agreed.

Agreement Take the following network-UE collaboration levels as one aspect for defining collaboration levels - Level x: No collaboration - Level y: Signaling-based collaboration without model transfer - Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary

# General aspect of AI/ML framework

## Definition and characterization of AI/ML

### 2.1.1 Remaining description of terminologies

As this study item (SI) is the first attempt to characterize AI/ML operations in RAN 1, RAN1#109-e and RAN1#110 spent some efforts on describing/defining some common notations and terminologies [2]. In our view, having a common understanding among companies on at least some important notations and terminologies is essential for the progress of the study item. In this regard, we provide our views on the remaining important notations and terminologies that have yet been defined/described.

RAN1#110 additionally made the following description of terminologies as a working assumption.

| Terminology | Description |
| --- | --- |
| Online training | An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples. Note: the notion of (near) real-time vs. non real-time is context-dependent and is relative to the inference time-scale. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as online training by commonly accepted conventions. Note: Fine-tuning/re-training may be done via online or offline training. (This note could be removed when we define the term fine-tuning.) |
| Offline training | An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions. |
| AI/ML model delivery | A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc. |

In the following, we provide proposals to describe the remaining terminologies.

One controversial terminology in RAN1#110 was “model deployment”. Some view model deployment in conjunction with model delivery. For example, a model can be converted to an executable form and be delivered to a target device. Alternatively, a model can be converted to an executable form and be delivered from one logical entity to another logical entity within a device. In light of the above, we want to define model deployment independently without associating it with model delivery. Moreover, it is preferred to avoid terms such as “conversion to an executable form” as those may lead to ambiguity.

Proposal 1: RAN1 can consider the following terminologies and their description:

| Terminology | Description |
| --- | --- |
| Model deployment | Process of preparation of a trained AI/ML model for inference. A deployed model is ready for inference at the target device. |
| Model registration | Assignment of an identification for an AI/ML model. The identification can be used to identify a model for its life-cycle management. |
| Model configuration | A process of setting the tuneable aspects of an AI/ML model to be used for model inference or model training. |

## AI/ML Model management

### 2.2.1 Discussion on Collaboration Levels, Impacts

In RAN1#109-e, the following network-UE collaboration levels were defined

Agreement RAN1#109-e Take the following network-UE collaboration levels as one aspect for defining collaboration levels. - Level x: No collaboration - Level y: Signaling-based collaboration without model transfer - Level z: Signaling-based collaboration with model transfer Note: Other aspect(s), for defining collaboration levels is not precluded and will be discussed in later meetings, e.g., with/without model updating, to support training/inference, for defining collaboration levels will be discussed in later meetings FFS: Clarification is needed for Level x-y boundary

In fact, other aspects could also be considered to further categorize the different collaboration levels. In our view, the main intention of defining such collaboration levels is to identify the different levels of specification impact. Moreover, a further detailed definition of collaboration levels would facilitate the discussion of the AI/ML model operations for different sub-use cases. In this respect, whether it is a one-sided model or a two-sided model would entail different levels of specification impact. In particular, different signalling and collaboration levels are required between one-sided and two-sided models for their life cycle management such as model activation/deactivation, model monitoring, and model update. In this regard, we propose further classification of collaboration levels with respect to one/two-sided models.

Proposal #2:  Further define sub-levels for Level x and Level y for one-sided and two-sided models as

Level x: No collaboration

Level y-1: Signalling-based collaboration for one-sided model without model transfer

Level y-2: Signalling-based collaboration for two-sided model without model transfer

Level z-1: Signalling-based collaboration for one-sided model with model transfer

Level z-2: Signalling-based collaboration for two-sided model with model transfer

Furthermore, RAN1#109-e described model transfer as follows

| Terminology | Description |
| --- | --- |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |

One issue, RAN1 is planning to discuss for RAN1#110b-e, is the boundary between level x, y and z. For example, a model can be transferred via either user plane or control plane. Obviously, a model transfer via control plane has more specification impact as compared to model transfer via user plane. Model transfer could also mean a transfer of a model in its executable format or in a standardized model transfer format to be converted to executable format at the target device. In our view, what matters is whether the model transfer is

Proposal #3:  If an AI/ML model is delivered via the air interface and the delivery is visible from RAN1 specification point of view, then the corresponding collaboration is Level z. A model transfer can be via a user plane or control plane, or in an executable format or standardized transfer format.

Model transfer, especially from the network to the UE, has various advantages as the

- Network/gNB may be better positioned to provide model

- gNB has bird’s eye view of operations within its site

- training based on data from multiple UEs

- more resources (dataset, computational) for model training

- site specific applications, e.g., beam prediction, positioning

- site specific optimization objective, e.g., MU-MIMO performance

- UE may be better suited to do inference for certain tasks

- UE has a better local view

- more inputs (measurements) can be leveraged, e.g., beam measurement

- some sub use cases work better with model exchange, e.g., CSI compression

However, the model transfer also introduces various challenges. In our view, various issues should be studied on the practicality/feasibility of AI/ML model transfer over the air interface including:

-    Interoperability: does a model transferred from one node to another node work in a plug-and-pay manner, i.e., without extensive receiving node specific optimization, compiling and testing?

-     Proprietary issues: If AI/ML models are considered as proprietary assets, model transfer discloses them.

-    Model transfer format (MTF): does 3GPP need to agree on a common MTF so that a model exchanged between two nodes from different vendors compile and run?

-    Performance guarantee: If AI/ML model is transferred from one node to other, which entity guarantees performance, e.g., inference latency?

Proposal #4: Concerning with the feasibility and practicality of AI/ML model transfer, RAN1 should study aspects such as

-  Interoperability: does a model transferred from one node to another node work in a plug-and-pay manner, i.e., without extensive receiving node specific optimization, compiling and testing?

-    Proprietary issues: If AI/ML models are considered proprietary assets, model transfer discloses them.

-   Model transfer format (MTF): does RAN1 need to adopt a common MTF so that a model exchanged between two nodes from different vendors compiles and runs?

-  Performance guarantee: If AI/ML model is transferred from one node to other, which entity guarantees performance, e.g., inference latency?

Moreover, the model transfer can be categorized in to two categories based on the level of transfer and requirements as follows:

Cat1: Model transfer for a partially known model at the receiving node, e.g., the structure of AI/ML model known.

Cat2: Model transfer for an unknown model to the receiving node.

The above two categories for model transfer may entail different levels of requirements. For example, if the model is partially known at the receiving node, e.g., only the weights of the neural network are needed to be updated by the network, with relatively less requirements on the node-specific optimization, compiling and testing at the receiving node. This may allow that the partially transferred model can be run immediately without additional effort while achieving the aforementioned advantages of model transfer.

Proposal #5:  Further consider two categories for model transfer

Cat1: Model transfer for a partially known model at the receiving node, e.g., the structure of AI/ML model known.

Cat2: Model transfer for a completely new model to the receiving node.

### 2.2.2. Life cycle management (LCM)

RAN1#110 agreed the following aspects to be studied under model life cycle management (LCM) .

Agreement RAN1#110 Study the following aspects, including the definition of components (if needed) and necessity, in Life Cycle Management - Data collection - Note: This also includes associated assistance information, if applicable. - Model training - [Model registration] - Model deployment - Note: Terminology is to be defined. This includes process of compiling a trained AI/ML model and packaging it into an executable format and delivering to a target device. - [Model configuration] - Model inference operation - Model selection, activation, deactivation, switching, and fallback operation - Note: some of them to be refined - Model monitoring - Model update - Note: Terminology is to be defined. This includes model finetuning, retraining, and re-development via online/offline training. - Model transfer - UE capability Note: Some aspects in the list may not have specification impact. Note: Aspects with square brackets are tentative and pending terminology definition. Note: More aspects may be added as study progresses.

Fig. 3 LCM for UE-side model

In our view, LCMs for one-sided and two-sided models entail different requirements and specification impact. In this regard, we present our views on LCM of one-sided and two-sided models, respectively.

- LCM for One-sided (UE-side) Models

LCM of one-sided model poses different requirements for UE-side and network-side models. We focus our discussion on the LCM of UE-side model as it is of more interest than the network-side model. LCM for UE-side model may include the blocks as depicted in Fig. 3. For example, a model might be transferred via air-interface to the UE, if collaboration Level z-1 is considered. In another example, it can be UE's own model, which is not the one transferred from the network, if collaboration Level z-1 is considered. Even in such case, UE-side models may be registered to the network such that the network can be aware of the models available at the UE’s side. This would help the network to provide appropriate assistance information including model (de)activation, and switching. In some systems, a model might be updated or fine-tuned after being activated. A such online model update may also require assistance information from the network. An updated model can then be re-deployed and its inference output can be utilized by the UE to make decisions. The inference output can also be reported to the network depending on the use case or for the purpose of performance monitoring. Monitoring the performance of a UE-side model may also require some assistance information from the network.

Table 1: Signalling requirement for LCM of one sided and two sided models.

| No. | LCM function | Signaling requirement for one-sided model LCM | Signaling requirement for two-sided model LCM |
| --- | --- | --- | --- |
| 1. | Model registration | Optional | Compulsory |
| 2. | Model (de)activation, switching | Optional | Compulsory |
| 3. | Model monitoring | Optional | Compulsory |
| 4. | Model update | Optional | Compulsory |

- LCM for Two-sided Models

LCM for a two-sided model has more stringent requirements as compared to one-sided models. As an example, activation, deactivation and switching of one part of the model in a transparent manner without informing the other side is not possible. Moreover, model monitoring at the UE with direct performance metrics based on the output of the model is not possible, if one part of the model, including model structure and inference output, at the network side is not accessible by the UE. In Table 1, we summarized LCM functions for two-sided models that require compulsory signalling between the UE and the network. On the other hand, for one-sided model, these LCM functions can be carried out in a specification transparent manner and the corresponding signalling is an optional engineering.

Proposal #6: Study different levels of requirements involved with the life cycle management for one-sided and two-sided models, respectively.

### 2.2.3. Two-sided Models Development

RAN1#110 under the agenda item 9.2.2.2  agreed on  the following three types of AI/ML model training collaborations for two-sided model based CSI compression.

Agreement RAN1#110 under AI 9.2.2.2 In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, repectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW - Other collaboration types are not excluded.

In our view, the discussion for two-sided model training/development falls under the general aspects of AI/ML framework as the framework discussion includes characterization of defining stages of AI/ML algorithm and associated complexity, UE-gNB collaboration, life cycle management, dataset(s), and notation/terminology.

Proposal #7: Study the various types of AI/ML model training collaborations under agenda item 9.2.1: general aspects of AI/ML framework.

In general, two-sided models can be developed either by a single vendor (Type 1) or by two or more vendors through collaboration (Type 2 and 3).  In all the three types, the two-sided models can be either developed in an offline setup or online setup wherein in the latter the collection of training inputs (data, gradient values etc.) is via the air-interface. However, online model development and update requires sharing extensive training dataset and other quantities such as backpropagation gradient values for training.

Proposal #8: Deprioritize two-sided model training collaboration that requires extensive training, validation and testing dataset sharing in this study item.

Moreover, the model delivery in the above three types can be via model transfer over the air-interface or based on transparent methods to the physical layer, e.g., preinstalled models. In RAN1#109-e, some companies raised concerns on the proprietary aspects of AI/ML models. This imposes further constraints on model sharing between UE and network vendors.

Table 2: Various two-sided model training collaborations

| No. | Two-sided model training collaborations | Model development | Model delivery | Can a model be proprietary |
| --- | --- | --- | --- | --- |
| 1. | Developed by UE shared to the network | Offline or online | Transparent or model transfer | No |
| 2. | Developed by network shared to the UE | Offline or online | Transparent or model transfer | No |
| 3. | Developed via multi-vendor collaboration | Offline or online | Transparent or model transfer | Possible |

Among the agreed three types for two-sided model training collaborations, Type 2 and Type 3 can protect the model to be proprietary. Additionally, Type 2 and 3 allow the model to be developed while considering optimization for the target node’s hardware and implementations. However, Type 2 and Type 3 also pose a concern in terms of scalability in model development. It is not practical for each vendor to setup a training session or exchange training dataset or reference models with each potential collaborating vendor.

Table 3: Three types of training collaborations agreed to be studied for two-sided model development for CSI compression

|  | Can model be proprietary? | Optimization for UE/gNB hardware/implementation | Model development/training scalability | Model management scalability (storing, monitoring, updating, etc.) | Possible overhead |
| --- | --- | --- | --- | --- | --- |
| Type-1: Joint training at one side | No | Not supported | Scalable | - No issues for over-the air-transfer (use and discard). - Otherwise, UE/gNB has to manage multiple models. | - Model transfer overhead, if it is over-the-air-interface |
| Type-2: Joint training at the two sides | Yes | Supported | Not scalable - Two vendors train the network in one session. - A vendor has to contact each collaborating vendors for offline engineering. | - Has to be verified whether a single a UE-side model works with multiple gNB-side models, and vice versa. | - Dataset, forward and backward propagation values delivery overhead, if over the air interface. |
| Type-3: Separate training at the two sides | Yes | Supported | Not scalable - Independent training sessions. - Vendors has to receive training data or reference model from collaborating vendors. | - Has to be verified whether a single UE-side model works with multiple gNB-side models, and vice versa. | - Dataset delivery overhead, if over-the-air-interface. - Reference model delivery overhead, if over the air interface. |

The above types can be utilized for both offline and online model development, update or fine-tuning. As an example, in Type 2, two-vendors can collaborate for offline engineering outside the 3GPP’s framework (possibly on a private server). These vendors may share training dataset and backpropagation gradient values without disclosing their respective models. However, this method may still require two-vendors to have the same structure of the model, and the input 
(the batch) of the training dataset also needs to be aligned. That is, the models from both sides need to be paired and based on the same baseline architecture, e.g., ResNet, DenseNet, etc., at the minimum to achieve expected performance. In this case, the model proprietary issue cannot be fully resolved. On the other hand, for online model development, update or fine-tuning, the backpropagation gradient values and the training dataset has to be shared online via either air-interface or in a transparent manner to the physical layer. However, such online update will be inefficient due to sharing large size training dataset and backpropagation values.

Fig. 4 Different approaches for two-side model development without disclosing models from each side

Fig. 5 Different examples for Type 3

Type 3, i.e., separate training of the two sides at the UE and network side, can assume multiple flavours.  As illustrated in Fig. 5, Type 3-1 Sequential training, can start at the network side or UE side. When the network side starts the training, the network trains the ENC* and DEC as an example, and generate the labelled dataset, {V, c}, for the input space and latent-space based on the trained model. This labelled dataset can then be shared with the UE-side for the training of its model, e.g., ENC., based on a supervised learning. Then, the UE and network would deploy ENC. and DEC., respectively. As shown in Fig. 5, a similar procedure can be followed to train the two-sides sequential starting from UE-side.

Fig. 6 One realization of Type 3-3 [Parallel training]

Similarly in Type 3-2, collaborating vendors may share reference models. As an example, for an auto-encoder based CSI compression, a UE and network vendors provide their reference decoder and encoder models, respectively. The reference models may not be considered as proprietary to the vendors, thus, can be shared with other vendors without concerns. Finally, the two vendors train their respective proprietary models with respect to the shared reference models (e.g., a UE vendor trains its proprietary encoder with respect to the shared reference decoder). The performance might be impacted by the reference models, which requires further investigation. In order for the two proprietary models from UE and network vendors to match, they might needs to be trained with the same dataset or, at least, a dataset with the same distribution. Moreover, if two proprietary models are well generalized across several different datasets (e.g., in case that the distributions of datasets are quite similar), they might be trained with the datasets with similar but not exactly identical distribution. For this purpose, the collaborating vendors have to share information (metadata) on the dataset or the training dataset between themselves. Otherwise, if different training datasets are used by different vendors, it might lead to performance degradation.

Finally, a third approach, Type 3-3, can be considered in which vendors independently train their respective models by keeping some structure in the latent space. As an example, vendors may agree on reference dataset to guide (alugn among proprietary implementations)  the mapping of the input space, V, to the latent space, c. Then, they may agree on a general mapping principle, as an example conserving distance in the input and latent space. This may require an agreement on how to measure distance in the two spaces. Finally, each side can train their respective model separately based on the agreed reference datasets and agreed upon mapping philosophy.  However, the feasibility and performance of such model development is yet to be studied.

In order to mitigate the aforementioned scalability issue in the model development for Type 3, training inputs/aspects that are shared by the collaborating vendors can be standardized. As an example, reference datasets, reference models, mapping principles can be standardized.

For the above three approaches, the models are assumed to be pre-stored in the hardware. Without model transfer, it cannot be updated or fine-tuned for different scenarios. Therefore, the generalization performance needs to be carefully evaluated.

Table 4: Challenges for the three types of training collaboration

| No. | Two-sided model development approach | Model development | Exchanged quantities | Challenges |
| --- | --- | --- | --- | --- |
| 1. | Type 2 via gradient values sharing. | Offline or online | - Training dataset - Inference output - Backpropagation gradient values | - Scalability - Overhead for online development (dataset sharing and gradient values sharing) - Not aligned with 3GPP’s philosophy of open development |
| 3 | Type 3-1 via labeled-data sharing [Sequential] | Offline or online | - Labeled training dataset | - Scalability - Overhead for online development (dataset sharing and reference model sharing) - Not aligned with 3GPP’s philosophy of open development |
| 2. | Type 3-2 via reference model sharing [Parallel] | Offline or online | - Information on training dataset or metadata - Reference models | - Scalability - Overhead for online development (dataset sharing and reference model sharing) - Not aligned with 3GPP’s philosophy of open development |
| 3. | Type 3-3 via separate training based on structured latent space [Parallel] | Offline or online | - Reference dataset and distance measurement metric, if not standardized | - Performance has not yet been verified. - Overhead to align dataset |

Proposal #9: Consider the following and study their impacts for the two-side model development approaches,

  - Requirements on privacy-sensitive dataset sharing

  - Scalability, i.e., whether the number of models one vendor should develop increases with the collaborating vendors

  - Whether the model development approaches adhere to 3GPP’s open and fair framework

# Evaluation methodology

## General discussion on KPIs

In RAN1#110, the following initial list common KPIs are agreed for evaluating performance benefits of AI/ML.

Agreement The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Performance - Intermediate KPIs - Link and system level performance - Generalization performance - Over-the-air Overhead - Overhead of assistance information - Overhead of data collection - Overhead of model delivery/transfer - Overhead of other AI/ML-related signaling - Inference complexity - Computational complexity of model inference: FLOPs - Computational complexity for pre- and post-processing - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte) - Training complexity - LCM related complexity and storage overhead - FFS: specific aspects - FFS: Latency, e.g., Inference latency Note: Other aspects may be added in the future, e.g. training related KPIsNote: Use-case specific KPIs may be additionally considered for the given use-case.

A starting point would be identifying the KPIs relevant for AI/ML based operations. To simplify the discussion on the determination of KPIs and their use in evaluating AI/ML based solutions, we propose to categorize KPIs into two categories

- Performance-related KPIs: These are KPIs that are directly related to the performance of an AI/ML based solutions for the considered use case. Taking the CSI feedback enhancement use case as example, this category may include direct performance indicators such as UPT, inference latency, and feedback overhead.

- Capability/complexity-related KPIs: This is a category of KPIs that are related to AI/ML operation but does not directly relate to the performance of AI/ML algorithm. The KPIs in this category may rather indicate the required capability for UE to operate a given AI/ML model. An example of KPIs that may fall under this category includes computational complexity, overhead associated with AI/ML model life-cycle management, power consumption, memory storage and other associated hardware requirements (including for given processing delays).

For capability-related KPIs, the following aspects needs to be studied.

- Size of model (storage requirement)

Since UE or gNB need to store the AI model for inference and the required storage size for AI/ML models can vary significantly, the size of AI models need to be considered as one of the capability-related KPIs, at least for AI/ML at the UE side. Moreover, this may be one of KPIs if AI/ML models need to be exchanged between UE and gNB, depending on how to transfer the AI/ML models.

- Inference/training complexity and latency

Inference complexity and training complexity when applicable. Both AI model training and inference require computation. If online training is a part of the study, the training complexity needs to be considered. The number of FLOPs is widely used to evaluate the computational complexity of AI/ML inferencing. Besides, latency is another key metric for AI model evaluation since this has an impact to the system performance. Latency is related to the computational complexity of the UE. Moreover, different AI models may allow different levels of parallel operation resulting in different latency even if two models have the same size. Latency can be studied together with the inference / training complexity.

- Generalization

Generalization is one of the main aspects to evaluate an AI model. The generalization performance of a model is a particularly useful metric if a single model is employed across multiple deployment scenarios, e.g., UMa, UMi, etc., or other varying scenarios such as UE speed, or even configuration, e.g., various antenna configurations. As an example, a degree of the performance degradation over the co-scenario performance (i.e., the performance when the distributions of training and inference datasets are the same) can be used. If the degree of the performance degradation is marginal, this implies that the trained AI model generalize well across different scenarios.

- Model management complexity

For some use cases, the model needs to be updated frequently, or multiple models are needed for different scenarios or for different UEs. The model management complexity needs to be considered in the evaluation together with specification impact or collaboration levels.

Another issue regarding evaluation is whether models are needed to be disclosed for fair evaluation (fair comparison and reproducibility of results. If so what aspect of the model should be disclosed, e.g., only model architecture, or entire model parameters, etc. It our view, it is not efficient to disclose the entire model and its parameters. However, if companies disclose some higher level description of their models such as the type of model, number of layers etc., it would help to compare results and assist on drawing some conclusions.  One important observation is that the inference latency is related type of neural network due to different level of parallelization they allow. As an example, CNN and LSTM networks with a comparable number of parameters do not incur the same inference latency.

Proposal #10: For evaluation purpose, companies should report their results with, at least, a higher level description of their AI/ML model. Higher level description includes

- Types of neural network for AI/ML model, e.g., CNN, LSTM, transformer, etc.

- Number of layers

### 3.2.1 Generalization Performance

In RAN1#110, the following agreements were taken about verification of the generalization performance of AI/ML models under agenda items 9.2.2.1 and 9.2.4.1, respectively.

Agreement RAN1#110 under AI. 9.2.2.1 The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two - FFS the detailed set of scenarios/configurations - FFS other cases for generalization verification, e.g., - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. Agreement RAN1#110 under AI 9.2.4.1 To investigate the model generalization capability, at least the following aspect(s) are considered for the evaluation for AI/ML based positioning: - Different drops - Training dataset from drops {A0, A1,…, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1,…, AN-1}). Here N>=1. - Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m}); - Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error; - Other aspects are not excluded. Note: It’s up to participating companies to decide whether to evaluate one aspect at a time, or evaluate multiple aspects at the same time.

In our view, it is better if a general framework for verification of the generalization performance of AI/ML model is agreed. In this respect, we believe the agreement taken under 9.2.2.1 is general enough and can be adopted as a general guideline for the use-cases considered in this SI and future endeavours. As an example, UE drops, clutter parameters and network synchronization error fall under evaluation scenarios.

Proposal #11: The following cases for verifying the generalization performance of an AI/ML model over various scenarios/configurations:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

In RAN1#109-e and RAN1#110, the following approaches were highlighted as potential approaches towards generalization performance of AI/ML models. The following list up the respective potential issues for the approaches.

| No. | Approach | Potential issues |
| --- | --- | --- |
| 1. | Training with mixed dataset | - Mixing scenarios/configs for training dataset generation is combinatorial and complex |
| 2. | Switch through a family of AI/ML models for a single task | - Requires scenario discovery or assistance information from gNB - Overhead (storage, LCM) of keeping multiple models - Scalability with respect to a new scenario/config |
| 3. | Model update (transfer learning) | - Partial (subset of layers) and full model update with site (scenario/config) related data - Updating overhead - How to update a two-sided model without model sharing? |

Proposal #12: For the case when a single model has to be employed across multiple scenarios and/or configurations, RAN1 should study various approaches that would improve generalization performance and their specification impact. Approaches may include

- Training with mixed dataset

- Switching through a family of AI/ML models for a certain task

- Model update via transfer learning

# Conclusion

In this contribution the following proposals are made:

Proposal 1: RAN1 can consider the following terminologies and their description:

| Terminology | Description |
| --- | --- |
| Model deployment | Process of preparation of a trained AI/ML model for inference. A deployed model is ready for inference at the target device. |
| Model registration | Assignment of an identification for an AI/ML model. The identification can be used to identify a model for its life-cycle management. |
| Model configuration | A process of setting the tuneable aspects of an AI/ML model to be used for model inference or model training. |

Proposal #2:  Further define sub-levels for Level x and Level y for one-sided and two-sided models as

Level x: No collaboration

Level y-1: Signalling-based collaboration for one-sided model without model transfer

Level y-2: Signalling-based collaboration for two-sided model without model transfer

Level z-1: Signalling-based collaboration for one-sided model with model transfer

Level z-2: Signalling-based collaboration for two-sided model with model transfer

Proposal #3:  If an AI/ML model is delivered via the air interface and the delivery is visible from RAN1 specification point of view, then the corresponding collaboration is Level z. A model transfer can be via a user plane or control plane, or in an executable format or standardized transfer format.

Proposal #4: Concerning with the feasibility and practicality of AI/ML model transfer, RAN1 should study aspects such as

-  Interoperability: does a model transferred from one node to another node work in a plug-and-pay manner, i.e., without extensive receiving node specific optimization, compiling and testing?

-    Proprietary issues: If AI/ML models are considered proprietary assets, model transfer discloses them.

-   Model transfer format (MTF): does RAN1 need to adopt a common MTF so that a model exchanged between two nodes from different vendors compiles and runs?

-  Performance guarantee: If AI/ML model is transferred from one node to other, which entity guarantees performance, e.g., inference latency?

Proposal #5:  Further consider two categories for model transfer

Cat1: Model transfer for a partially known model at the receiving node, e.g., the structure of AI/ML model known.

Cat2: Model transfer for a completely new model to the receiving node.

Proposal #6: Study different levels of requirements involved with the life cycle management for one-sided and two-sided models, respectively.

Proposal #7: Study the various types of AI/ML model training collaborations under agenda item 9.2.1: general aspects of AI/ML framework.

Proposal #8: Deprioritize two-sided model training collaboration that requires extensive training, validation and testing dataset sharing in this study item.

Proposal #9: Consider the following and study their impacts for the two-side model development approaches,

  - Requirements on privacy-sensitive dataset sharing

  - Scalability, i.e., whether the number of models one vendor should develop increases with the collaborating vendors

  - Whether the model development approaches adhere to 3GPP’s open and fair framework

Proposal #10: For evaluation purpose, companies should report their results with, at least, a higher level description of their AI/ML model. Higher level description includes

- Types of neural network for AI/ML model, e.g., CNN, LSTM, transformer, etc.

- Number of layers

Proposal #11: The following cases for verifying the generalization performance of an AI/ML model over various scenarios/configurations:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Proposal #12: For the case when a single model has to be employed across multiple scenarios and/or configurations, RAN1 should study various approaches that would improve generalization performance and their specification impact. Approaches may include

- Training with mixed dataset

- Switching through a family of AI/ML models for a certain task

- Model update via transfer learning

# Reference

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm (Moderator)

[2] Chair's notes RAN1#109-e

[3] Chair's notes RAN1#110