3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209834

# e-meeting, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.213 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on coexistance between Rel-16 and Rel-17 H/S/NA configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eIAB_enh-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At RAN1#110, the following working assumption was made: Working Assumption If Rel-16 H/S/NA resource configuration and Rel-17 H/S/NA resource configuration are both provided for a given RB set within a slot: - Alt 3b. A resource configured with Rel-16 H or Rel-16 S with dynamic indication of availability overrides the Rel-17 H/S/NA configuration, otherwise the child node follows the Rel-17 H/S/NA configuration for the resource. The above working assumption has not been captured in the specification yet. In addition, there are some other cases which are not covered by the working assumption. Case 1: If both Rel-16 H/S/NA and Rel-17 H/S/NA resource configuration are both provided for a given RB set within a slot, and both Rel-16 and Rel-17 dynamic indication of resource availability are received by child node: - A symbol configured with Rel-16 S with Rel-16 dynamic indication indicating available overrides the Rel-17 H/S/NA configuration - For a symbol configured with Rel-16 S with Rel-16 dynamic indication indicating “no indication of availability”, the child node follows the Rel-17 H/S/NA configuration and Rel-17 dynamic indication to determine resource availability for the RB set in the symbol Case 2: If only Rel-16 H/S/NA resource configuration is provided for a given a slot: - If Rel-16 dynamic indication of availability is not configured by RRC or not provided by DCI, the child node ignores the Rel-17 dynamic availability indication, if any. - If both Rel-16 and Rel-17 dynamic indication of availability are configured and received, the child node ignores the Rel-17 dynamic availability indication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the working assumption in the specification. Capture the coexistence resolution for the case when both Rel-16 and Rel-17 dynamic resource availability are configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Parent node mistakenly assumes the H/S/NA configuration applied by child node DU when both Rel-16 and Rel-17 H/S/NA configuration or both Rel-16 and Rel-17 dynamic resource availability provided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: No inter-operatability issue is identified. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 14	Integrated access-backhaul operation

========================= Unchanged parts =========================

With reference to slots of an IAB-DU cell, the IAB-DU can be provided an indication of hard, soft or unavailable type per RB set for symbols configured as downlink, uplink or flexible in a slot by Frequency-Domain HSNA Configuration List [16, TS 38.473]. The RB set size and the number of RB sets are configured by RB-Set-Configuration [16, TS 38.473]. If an indication of hard, soft or unavailable type is not provided for an RB set of a symbol in a slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot.

If an IAB-DU is provided an indication of hard, soft or unavailable type for a symbol in a slot by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] and the IAB-DU is also provided an indication of hard, soft or unavailable type for each RB set of the symbol in the slot by Frequency-Domain HSNA Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473], the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the symbol in the slot if the symbol is configured as soft by HSNA Slot Configuration List and the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available, otherwise, the IAB-DU applies the configuration of hard, soft or unavailable type provided by Frequency-Domain HSNA Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the symbol in the slot only if

  - the symbol is configured as soft by HSNA Slot Configuration List, and the IAB-MT does not detect a DCI format 2_5 with an AI index field value indicating the availability of the soft symbol in the slot

  - the symbol is configured as soft by HSNA Slot Configuration List, and the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol in the slot as no indication of availability

  - the symbol is configured as hard or unavailable by HSNA Slot Configuration List

If an IAB-DU is provided an indication of hard, soft or unavailable type for a symbol in a slot by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] and the IAB-DU is also provided an indication of hard, soft or unavailable type for each RB set of the symbol in the slot by Frequency-Domain HSNA Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473], and if the IAB-DU is provided with a set of availability combinations by both availabilityCombinations and availabilityCombinationsRBGroups

  - if the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available in the slot, the IAB-DU applies the configuration of hard, soft or unavailable type provided by HSNA Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the symbol in the slot

  - if the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft symbol in the slot as no indication of availability, the IAB-DU applies the configuration of hard, soft or unavailable type provided by Frequency-Domain HSNA Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473] for the RB set of the symbol in the slot

If an IAB-DU is only provided an indication of hard, soft or unavailable type for a symbol in a slot by HSNA Slot Configuration List in gNB-DU Cell Resource Configuration [16, TS 38.473], the IAB-MT shall ignore a DCI format 2_5 with an AI index field value indicating availability of soft RB sets of the symbol in the slot.

< Unchanged parts are omitted >