3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2210205

e-Meeting, 10-19 October, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on RAN2 LS on resource pool index in DCI format 3_0

Document for:	Discussion and Decision

- Introduction

RAN2 informed RAN1 of some agreements they made relating to discovery resource pools for SL relay [1]:

In Rel-17, RAN2 introduced mode-1 dedicated discovery resource pool configuration for NR SL discovery transmission, i.e. sl-DiscTxPoolScheduling, as now specified in TS 38.331. However, RAN2 found that current DCI Format 3_0, as specified in TS 38.212, cannot schedule any resource in the pool(s) indicated by sl-DiscTxPoolScheduling, since the “Resource pool index” field in DCI format 3_0 is currently unable to refer to any pool configured by sl-DiscTxPoolScheduling.

RAN2 discussed above issue and reached the agreement that the parameter “I” related to the “Resource pool index” field in DCI Format 3_0 in TS 38.212 is the number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling, if configured, and sl-DiscTxPoolScheduling, if configured. Also, according to TS 38.331, the total number of the resource pools configured in sl-TxPoolScheduling and sl-DiscTxPoolScheduling is no larger than 8.

RAN1 was requested to take the above RAN2 agreement into account and make any necessary specification change.

- Discussions

RAN2’s understanding on usage of “resource pool index” field in DCI format 3_0 is correct, and thus with newly introduced resource pools dedicated for discovery instead of communication, RAN1 specification change is necessary. Thus a TP as proposed below should be captured in TS 38.212:

< Unchanged parts are omitted >

##### 7.3.1.4.1	Format 3_0

DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell.

The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI:

-	Resource pool index –log2I  bits, where I is the total number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling if configured, and sl-DiscTxPoolScheduling, if configured.

< Unchanged parts are omitted >

Proposal 1: Confirm that RAN1 is to update “resource pool index” field in DCI format 3_0 of TS38.212 to capture RAN2 agreement.

- Conclusions

Following proposals were made:

Proposal 1: Confirm that RAN1 is to update “resource pool index” field in DCI format 3_0 of TS38.212 to capture RAN2 agreement.

# References

- R2-2209207, “LS on resource pool index in DCI Format 3_0”, RAN2, RAN2#119-e, August, 2022.