3GPP TSG-RAN WG1 Meeting #111	R1-2212481

Toulouse, France, November 14 – 18, 2022

Agenda Item:	8.10

Source:	Huawei, HiSilicon

Title:	Remaining issues on resource multiplexing for IAB

Document for:	Discussion and Decision

Introduction

In this paper, we discuss some remaining issues on resource multiplexing for Rel-17 IAB.

Coexistence issue for Rel-16/17 dynamic indication of availability

In RAN1#110bis-e, the working assumption from RAN1#110 was confirmed for the case when Rel-16 H/S/NA resource configuration and Rel-17 H/S/NA resource configuration are both provided for a resource of an IAB node:

Agreement Confirm the WA from RAN1#110: If Rel-16 H/S/NA resource configuration and Rel-17 H/S/NA resource configuration are both provided for a given RB set within a slot: Alt 3b. A resource configured with Rel-16 S with dynamic indication of availability overrides the Rel-17 H/S/NA configuration, otherwise the child node follows the Rel-17 H/S/NA configuration for the resource.. Correction on coexistence of Rel-17 and Rel-16 HSNA configuration for TS38.213 is agreed. Final CR in R1-2210757.

According to the CR, if an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by Frequency-Domain HSNA Configuration List unless the symbol is configured as soft type in HSNA Slot Configuration List in gNB-DU Cell Resource Configuration and the IAB-DU cell can transmit or receive in the symbol.

There are two cases which are not covered by the confirmed working assumption.

- Case 1: Only Rel-16 H/S/NA resource configuration is provided for a given slot, and only Rel-17 dynamic indication of availability indicates the availability of RB sets of this slot. It is possible that IAB MT is configured with Rel-17 dynamic resource availability by RRC and corresponding indication is received in DCI 2_5, while only Rel-16 H/S/NA is configured for DU. It was also not prohibited that Rel-17 dynamic resource availability is indicated by DCI 2_5 in case of only Rel-16 H/S/NA is configured. The behavior of child node is not clear in this case.

- Case 2: Only Rel-16 H/S/NA resource configuration is provided for a given slot, and both Rel-16 and Rel-17 dynamic indication of availability indicates the availability of this slot. IAB MT may receive DCI 2_5 which includes both dynamic resource availability indication for RB set groups of the slot and for the slot, as both AvailabilityCombination can be configured by RRC.

For Case 1 and Case 2, the current specification needs further clarification on the interpretation of Rel-17 dynamic indication of availability received and applied.

All possible cases of coexistence involving Soft resources are summarized in the following table:

Table 1: All possible cases considering Rel-16/17 resource configuration/availability

|  | Rel-16 dynamic indication of availability | Rel-17 dynamic indication of availability | Rel-16 + Rel-17 dynamic indication of availability |
| --- | --- | --- | --- |
| Rel-16 S for a slot + Rel-17 S for RB set(s) | Covered by agreed WA | Covered by agreed WA | Covered by agreed WA |
| Rel-16 S for a slot | Specified in Rel-16 | Case 1 | Case 2 |

For Case 1, if Rel-16 dynamic indication of availability is not configured by RRC or not provided by DCI, the child node ignores the Rel-17 dynamic availability indication. Alternatively, the child node may follow the Rel-17 indication of first RB set group to determine resource availability for Rel-16 S.

For Case 2, if both Rel-16 and Rel-17 dynamic indication of availability are configured and received, one simple solution is that the child node ignores the Rel-17 dynamic availability indication. Alternatively, if Rel-16 dynamic indication indicates “no indication of availability”, the child node follows the Rel-17 indication of first RB set group to determine resource availability for Rel-16 S.

Based on the analysis and observations above, we propose the following

Proposal 1: If only Rel-16 H/S/NA resource configuration is provided for a given a slot:

- If Rel-16 dynamic indication of availability is not configured by RRC or not provided by DCI, the child node ignores the Rel-17 dynamic availability indication, if any.

- If both Rel-16 and Rel-17 dynamic indication of availability are configured and received, the child node ignores the Rel-17 dynamic availability indication.

Based on the above, one CR is submitted in [3].

The misalignment and ambiguity of DL TX power adjustment

For the DL TX power adjustment mechanism, the TS 38.213, TS 38.214 and TS 38.321 captured related descriptions as following:

3GPP TS 38.213 V17.3.0

<omit un-related part> For a serving cell of an IAB-MT, the IAB-MT can be provided a set of TCI states or a set of RS resource indexes corresponding to a SS/PBCH block or to a CSI-RS resource index for a slot where a PDSCH EPRE adjustment is indicated by DL Tx Power Adjustment MAC CE as described in [11, TS 38.321]. The PDSCH EPRE can be derived from a downlink CSI-RS EPRE as described in [6, TS 38.214] and a PDSCH power offset provided by powerControlOffsetIAB as described in [11, TS 38.321]. For a downlink DM-RS and/or PT-RS associated with a PDSCH, the IAB-MT may assume that the ratio of PDSCH EPRE to DM-RS EPRE, and/or PT-RS EPRE to PDSCH EPRE, is obtained as for a "UE" in [6, TS 38.214]. If no TCI state or RS resource index is provided to the IAB-MT, the IAB-MT may assume that a same PDSCH EPRE adjustment applies to all TCI states or RS resource indexes configured for the IAB-MT. A PDSCH EPRE adjustment provided by DL Tx Power Adjustment MAC CE may be associated with <omit un-related part>

3GPP TS 38.214 V17.3.0

If configured to report CQI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI: - The first 2 OFDM symbols are occupied by control signaling. - The number of PDSCH and DM-RS symbols is equal to 12. - The same bandwidth part subcarrier spacing configured as for the PDSCH reception - The bandwidth as configured for the corresponding CQI report. - The IAB-MT shall only assume the frequency resources as indicated by the DL TX power adjustment MAC CE, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321]. - The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception - No resource elements used by primary or secondary synchronization signals or PBCH. - Redundancy Version 0. - The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Clause 5.2.2.3.1. - In addition, the IAB-MT shall apply the provided DL TX power adjustment, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321]. <omit un-related part>

3GPP TS 38.321 V17.2.0

The DL TX Power Adjustment MAC CE and Desired DL TX Power Adjustment MAC CEs are identified by MAC subheaders with eLCIDs as specified in Table 6.2.1-1b and Table 6.2.1-1b, respectively. Otherwise the format of the two MAC CEs is identical. Each has a variable size with following fields (Figure 6.1.3.63-1): - Resource Configuration ID: This field indicates the RRC configuration which applies to this MAC CE and corresponds to iab-ResourceConfigID parameter in IAB-ResourceConfig as specified in TS 38.331 [5]. The length of the field is 16 bits; - DL Tx Power adjustment: For the case of the Desired DL TX Power Adjustment MAC CE, this field contains desired DL TX power adjustment sent by an IAB-MT to assist with the parent node's DL TX power allocation. For the case of the DL TX Power Adjustment MAC CE, this fields contains adjustment to a node's DL TX power sent by its parent node. The value of the power adjustment is in both cases contained in the 5 right-most bits of the field, while the remaining 3 bits are set to zero. The length of the field is 8 bits; - Reference CSI-RS ID: This field indicates the CSI-RS used as reference for power adjustment, which is identified by NZP-CSI-RS-ResourceId as specified in TS 38.331 [5]. The indicated DL TX power adjustment is in terms of a relative offset to the CSI-RS TX power of the indicated CSI-RS.The length of this field is 8bits; <omit un-related part> - IAB-MT beam type IDi: This field determines the type of information used as an indication of a downlink IAB-MT beam, indicated with the IAB-MT Resource set IDi. If the value of the field is 00, the information included in the IAB-MT Resource set IDi field is the TCI state ID (contained in the 7 rightmost bits of the field). If the value of the field is 01, the information included in the IAB-MT Resource set IDi field is the SSB ID (contained in the 6 rightmost bits of the field). If the value of the field is 10, the information included in the IAB-MT Resource set IDi field is the CSI-RS index (comprising all 8 bits). The length of the field is 2 bits; - IAB-MT Resource set IDi: an indication of the IAB-MTs downlink beams. IAB-MT Resource set ID0 field refers to the first IAB-MT downlink beam, IAB-MT Resource set ID1 field refers to the second IAB-MT downlink beam, and so on. The length of the field is 8 bits; <omit un-related part>

The three parts of the specification on DL TX power adjustment are not fully aligned and the procedures on how an IAB-MT shall derive the PDSCH EPRE based on the MAC CE provided in TS 38.321 is still missing.

In TS 38.213, there are three issues:

- In TS 38.213, it says “a PDSCH power offset provided by powerControlOffsetIAB as described in [11, TS 38.321]”, however, there is no “powerControlOffsetIAB” MAC CE in TS 38.321.

- For the beam which shall apply the DL TX power adjustment, a CSI-RS resource index can be provided. In detail, if the “IAB-MT beam type ID” is set to “10”, then “IAB-MT Resource set ID” refers to a CSI-RS resource ID as described in TS 38.321. However, the reference CSI-RS whose EPRE would be used to derive the PDSCH EPRE is also provided by “Reference CSI-RS ID” in the same MAC-CE. To avoid the ambiguity, it is preferred to clarify that the downlink CSI-RS EPRE refers to the CSI-RS indicated by “Reference CSI-RS ID” in DL Tx Power Adjustment MAC CE in TS 38.213.

- The relations among “PDSCH EPRE”, “ratio of PDSCH EPRE to CSI-RS EPRE” and “DL TX power adjustment” is missing. “PDSCH power offset” is used in TS 38.213, but it is not clear what “PDSCH power offset” refers to. Based on our understanding, the DL TX power adjustment provided by DL TX Power Adjustment MAC-CE is actually the ratio of PDSCH EPRE to CSI-RS EPRE assuming DL TX power adjustment applied.

Based on the above, we propose the following update to TS 38.213:

TP 1: TS 38.213 V17.3.0 For a serving cell of an IAB-MT, the IAB-MT can be provided a set of TCI states or a set of RS resource indexes corresponding to a SS/PBCH block or to a CSI-RS resource index for a slot where a PDSCH EPRE adjustment is indicated by DL Tx Power Adjustment MAC CE as described in [11, TS 38.321]. The PDSCH EPRE can be derived from a downlink CSI-RS EPRE and a ratio of PDSCH EPRE to CSI-RS EPRE as described in [6, TS 38.214] and a PDSCH power offset provided by powerControlOffsetIAB as described in [11, TS 38.321]. The downlink CSI-RS EPRE refers to the CSI-RS indicated by Reference CSI-RS ID in DL Tx Power Adjustment MAC CE as described in [11, TS 38.321]. The ratio of PDSCH EPRE to CSI-RS EPRE is provided by DL Tx Power adjustment in DL Tx Power Adjustment MAC CE as described in [11, TS 38.321]. For a downlink DM-RS and/or PT-RS associated with a PDSCH, the IAB-MT may assume that the ratio of PDSCH EPRE to DM-RS EPRE, and/or PT-RS EPRE to PDSCH EPRE, is obtained as for a "UE" in [6, TS 38.214]. If no TCI state or RS resource index is provided to the IAB-MT, the IAB-MT may assume that a same PDSCH EPRE adjustment applies to all TCI states or RS resource indexes configured for the IAB-MT. A PDSCH EPRE adjustment provided by DL Tx Power Adjustment MAC CE may be associated with - multiplexing mode of the IAB-node, if provided, and/or - when simultaneous reception by the IAB-MT and transmission/reception by an IAB-DU cell occur in non-overlapping frequency resources, if provided, or when simultaneous reception by the IAB-MT and transmission/reception by an IAB-DU cell occur in overlapping frequency resources, if provided, and/or - slots indicated by slot index, if provided.

The ratio of PDSCH EPRE to CSI-RS EPRE is required for MT to derive to the PDSCH EPRE. In 38.214, it says the IAB-MT shall apply the provided DL TX power adjustment, but it is still not clear how the IAB-MT can apply the provided DL TX power adjustment. The unclear part is whether “the ratio of PDSCH EPRE to CSI-RS EPRE” or the “PDSCH EPRE” or the “CSI-RS EPRE” shall apply the DL TX power adjustment.

Based on above, we suggest the following update for 38.214

TP 2: TS 38.214 V17.3.0 - The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Clause 5.2.2.3.1. - In addition, the IAB-MT shall apply the ratio of PDSCH EPRE to CSI-RS EPRE where the CSI-RS EPRE refers to the Reference CSI-RS ID provided by DL TX power adjustment, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321].

In conclusion, we have the following proposal:

Proposal 2: Adopt TP1 and TP2 for 38.213 and 38.214 respectively.

Summary and conclusion

Proposal 1: If only Rel-16 H/S/NA resource configuration is provided for a given a slot:

- If Rel-16 dynamic indication of availability is not configured by RRC or not provided by DCI, the child node ignores the Rel-17 dynamic availability indication, if any.

- If both Rel-16 and Rel-17 dynamic indication of availability are configured and received, the child node ignores the Rel-17 dynamic availability indication.

Proposal 2: Adopt TP1 and TP2 for 38.213 and 38.214 respectively.