3GPP TSG RAN WG1 #112bis-e			                           R1-2303884

e-Meeting, April 17th – April 26th, 2023

Source:	Moderator (NTT DOCOMO)

Title:	FL summary on DMRS#1

Agenda item:	9.1.3.1

Document for:	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are two objectives for DMRS enhancements, as shown below.

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS […] - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

This document contains summary of the company’s tdocs and FL proposals.

- Objective #3 (increasing DMRS ports)

  - Antenna ports table for PDSCH

2.1.1 eType1, maxLength1

In previous RAN1 meetings, we have made agreements or working assumption for the following rows in the table for Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH. We discuss the remaining issue in this section.

Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes |
| 0 | 1 | 0 | Cat. 1 | 0 | 2 | 0,1,2,3,8 | Rank 5-8 with one DMRS symbol (Working assumption) |
| 1 | 1 | 1 |  | 1 | 2 | 0,1,2,3,8,10 |  |
| 2 | 1 | 0,1 |  | 2 | 2 | 0,1,2,3,8,9,10 |  |
| 3 | 2 | 0 |  | 3 | 2 | 0,1,2,3,8,9,10,11 |  |
| 4 | 2 | 1 |  |  |  |  |  |
| 5 | 2 | 2 |  |  |  |  |  |
| 6 | 2 | 3 |  |  |  |  |  |
| 7 | 2 | 0,1 |  |  |  |  |  |
| 8 | 2 | 2,3 |  |  |  |  |  |
| 9 | 2 | 0-2 |  |  |  |  |  |
| 10 | 2 | 0-3 |  |  |  |  |  |
| 11 | 2 | 0,2 |  |  |  |  |  |
| 12 | 1 | 8 | Cat.2 |  |  |  |  |
| 13 | 1 | 9 |  |  |  |  |  |
| 14 | 1 | 8,9 |  |  |  |  |  |
| 15 | 2 | 8 |  |  |  |  |  |
| 16 | 2 | 9 |  |  |  |  |  |
| 17 | 2 | 10 |  |  |  |  |  |
| 18 | 2 | 11 |  |  |  |  |  |
| 19 | 2 | 8,9 |  |  |  |  |  |
| 20 | 2 | 10,11 |  |  |  |  |  |
| 21 | [2] | [8-10] |  |  |  |  |  |
| 22 | [2] | [8-11] |  |  |  |  |  |
| 23 | [2] | [8, 10], [9, 11] |  |  |  |  |  |
| 24 | 1 | 0,1,8 | Cat.3 |  |  |  |  |
| 25 | 1 | 0,1,8,9 |  |  |  |  |  |
| 26 | 2 | 0,1,8 |  |  |  |  |  |
| 27 | 2 | 0,1,8,9 |  |  |  |  |  |
| 28 | 2 | 2,3,10 |  |  |  |  |  |
| 29 | 2 | 2,3,10,11 |  |  |  |  |  |

Row 21-22 (1CW)

The benefit of supporting the row 21-22 is that it can be multiplexed with row 9-10, so that 2 UEs can share DMRS ports in two CDM groups. However, such MU-MIMO is precluded. Hence, FL suggestion is to remove the row 21-22.

Row 23 (1CW)

Huawei/HiSilicon[3] and Xiaomi[14] propose to support row 23 of {9,11}. On the other hand, ZTE/China Telecom[4], Qualcomm[23], Docomo[25] propose to remove the row 23. Considering we have discussed in several meetings, and we could not have consensus to support the row 23, FL suggestion is to remove row 23.

Huawei/HiSilicon[3]:

Unlike other layer combinations that can be achieved by different compositions of DMRS port combinations, {3+3+2} can only be enabled by composition {#28 + #26 + #23}, which means entry #23 is irreplaceable for supporting all layer combinations. The system-level simulation is conducted to reflect the probability of different layer combinations, where the maximum number of MU pairing layer is limited to 8. Other detailed simulation assumptions can be found in Appendix A The PDF of layer combinations is shown in Figure 1, from which it can be clearly observed that layer combination {3+3+2} has a relatively high probability to be scheduled. By further concentrating on the cases with 8 MU pairing layers, the probability of layer combination {3+3+2} can reach more than 1/3. Considering that replacing {3+3+2} by {3+3+1+1} (or other layer combinations corresponding to more than 3 paired UEs) subjects to practical MU traffic, while replacing {3+3+2} by {4+3+1} (or other layer combinations corresponding to at least one 4-layer UE) may bring MU performance degradation, layer combination {3+3+2} plays an important role to achieve the maximum number of MU pairing layer under this DMRS configuration. To enable this particular layer combination, entry#23 in Table 1, DMRS port combination {9, 11}, should undoubtedly be supported. Figure.1 The PDF of layer combinations

Qualcomm [23]:

Regarding the proposal to use DMRS ports [9,11] with row 23, the intention of the proponent is for MU enhancements, i.e., allow a particular MU scheduling scenario with 3 users with rank 3+3+2. However, we don’t see DMRS ports [9,11] is suitable for MU, based on the following reasons. - Channel estimation performance of the UE with port [9,11] will be quite bad as it sees channel interference from both CDM groups. - This is not aligned with MU design principle since Rel-15, which is allocate DMRS ports of a same UE into a same CDM group. - It creates serious trouble for UE implementation, as UE is assigned with DMRS ports [9,11] will have to effectively estimate 8 DMRS ports (for MU detection and noise estimation), which is not doable on a UE which only supports 1 CW PDSCH. The justification from the proponent of the proposal to support DMRS ports [9,11] is that it can support 3 users with rank 3+3+2, i.e., [0,1,8]+[2,3,10]+[9,11], to fully utilize the potential of rank 8 DL MU-MIMO capability for 1 symbol type-1 DMRS. However, we believe this case of rank 3+3+2 is just one corner case of MU scheduling, because of the following reasons. - MU scheduling fully utilize 8 layers is rare, given mutual interference between MU layers. More reasonable gNB should schedule MU with less than 8 layers. - Even in rank 8 MU scheduling, we listed all possible scheduling cases as in the following table. As there are in total 21 cases, the case of rank 3+3+2 is just one out of 21 cases. - Even if rank 3+3+2 is not supported without DMRS ports [9, 11], gNB can switch to 4 UEs MU with [0,1,8]+[2,3,10]+[9]+[11], or 3 UEs MU with [0,1,8]+[2,3,10]+[9]. There is no devastating outcome because of not supporting DMRS ports [9, 11]. Case number; Rank combination; Row indices to support the rank combination. Case 1; 2 UEs with 1+7 layer; row 2 (two CWs) + row 18 (1CW). Case 2; 2 UEs with 2+6 layer; Not supported with current agreed table. Case 3; 2 UEs with 3+5 layer; Not supported with current agreed table. Case 4; 2 UEs with 4+4 layer; row 27 (1 CW) + row 29 (1CW). Case 5; 3 UEs with 1+1+6 layers; row 1 (2 CWs)+ row 16 & 18 (1CW). Case 6; 3 UEs with 1+2+5 layers; row 0 (2 CWs)+ row 16 & 20 (1CW). Case 7; 3 UEs with 2+2+4 layers; row 27+ 8 + 20 (1CW). Case 8; 3 UEs with 1+3+4 layers; row 27 + 28 + 18 (1CW). Case 9; 3 UEs with 2+3+3 layers; Not supported with current agreed table. Case 10; 4 UEs with 1+1+1+5 layers; row 0 (2CWs) + row 16+17+18(1CW). Case 11; 4 UEs with 1+1+2+4 layers; 27 + 8 + 17 +18 (1CW). Case 12; 4 UEs with 1+2+2+3 layers; row 26+16+8+20 (1CW). Case 13; 4 UEs with 1+1+3+3 layers; row 26+28+16+18 (1CW). Case 14; 4 UEs with 2+2+2+2 layers; row 7+8+19+20 (1CW). Case 15; 5 UEs with 1+1+1+1+4 layers; row 27+5+6+17+18(1CW). Case 16; 5 UEs with 1+1+1+2+3 layers; row 26+8+16+17+18(1CW). Case 17; 5 UEs with 1+1+2+2+2 layers; row 7+8+19+17+18 (1CW). Case 18; 6 UEs with 1+1+1+1+1+3 layers; row 26+16+5+6+17+18(1CW). Case 19; 6 UEs with 1+1+1+1+2+2 layers; row 19+20+3+4+5+6(1CW). Case 20; 7 UEs with 1+1+1+1+1+1+2 layers; row 7+5+6+15+16+17+18 (1CW). Case 21; 8 UEs each with 1 layer; row 3+4+5+6+15+16+17+18 (1CW).

2 CWs

Huawei/HiSilicon, ZTE/China Telcom, vivo, Spreadtrum, Intel, Xiaomi (can live), LGE, NEC propose to confirm the WA (Alt.1), and Qualcomm and MediaTek propose to add new rows so that different CW is mapped to different CDM group (Alt.2). Nokia/NSB propose to add new rank5 DMRS combinations of {0,2,3,8,9} to Alt.1. FL suggestion is to discuss and down select in this meeting.

For at least for S-TRP

FL Proposal 2.1.1A

- For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case,

  - For 1 CW,

    - Do not support row 21-22

    - Do not support row 23

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 21 | [2] | [8-10] |
| 22 | [2] | [8-11] |
| 23 | [2] | [8, 10], [9, 11] |

  - For 2 CWs,

      - Alt.1: Confirm the working assumption in RAN1#112 without modification.

        - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0.

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |

      - Alt.2: Confirm the working assumption in RAN1#112 with the following modification.

        - Alt.3-1: Support at least row 0-37 for 2 CWs in Table 4-0.

          - Subject to UE capability, UE can indicate supporting values {4,5,6,7} only, or values {0,1,2,3,4,5,6,7}.

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |
| 4 | 2 | 0,1,2,3,10 |
| 5 | 2 | 0,1,8,2,3,10 |
| 6 | 2 | 0,1,8,2,3,10,11 |
| 7 | 2 | 0,1,8,9,2,3,10,11 |

Summary of companies’ view

|  | Support/fine | Concern |
| --- | --- | --- |
| Delete row 21-22 | Docomo, Google, OPPO, Nokia/NSB, Huawei/HiSilicon, Lenovo (can live), Sharp, Apple, New H3C, China Telcom, Samsung, Spreadtrum | CATT (slightly prefer), Ericsson |
| Delete row 23 | Docomo, Google, OPPO, Nokia/NSB, Lenovo (can live) , Sharp, Apple, New H3C, China Telcom, Samsung, Spreadtrum | CATT (slightly prefer), Huawei/Hisilicon, Ericsson, Xiaomi, CMCC |
| Alt.1 | Docomo, Nokia/NSB (Add FFS: {0,2,3,8,9} for rank 5) , Huawei/HiSilicon, Lenovo, Intel, ZTE, Ericsson, China Telcom, Samsung, Spreadtrum, CMCC | QC (UE complexity), MTK |
| Alt.2 | CATT, QC (UE complexity), MTK, Xiaomi | Intel, ZTE (out of scope) |

FL: If we remove “values {0,1,2,3} only” from Alt.2, the concern from Docomo/Ericsson/China Telcom can be solved.

For S-DCI M-TRP

In RAN1#112 online, LGE pointed out that we have not agreed to reuse all rows of DMRS ports table of S-TRP to S-DCI based M-TRP.

Google[16] mentions Cat.3 is useful for TDM/FDM/SFN based sDCI mTRP operation and Cat.1-2 are useful for SDM based sDCI mTRP operation. In the following proposal, gNB can select any of Cat.1-3 for sDCI mTRP operation. Hence we don’t need to discuss separate handling between TDM/FDM/SFN based sDCI mTRP operation and SDM based sDCI mTRP operation.

FL Proposal 2.1.1B

- For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP,

  - Support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data for Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-TRP, in addition to row 30 for 1CW in RAN1#112 agreement.

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | FL Proposal 2.1.1A: Support. For 2 CWs, support Alt.1. We have concern on Alt.2, if UE can report either “values {0,1,2,3} only” or “values {4,5,6,7} only”, because it causes market fragmentation. Hence, either “values {0,1,2,3} only” or “values {4,5,6,7} only” should be removed in Alt.2. FL Proposal 2.1.1B: Support. |
| Google | 2.1.1A: Support in principle, but the UE capability part can be decided later. 2.1.1B: Support |
| OPPO | FL Proposal 2.1.1A: Support. FL Proposal 2.1.1B: Support. |
| Nokia/NSB | FL Proposal 2.1.1A: Support. For 2CWs, we are fine with Alt 1, also we propose to add FFS: {0,2,3,8,9} for rank 5. We will provide the evaluation result by May meeting, so please add it for last checking. FL: Alt.1 says “at least” and it does not preclude to add another combination. FL Proposal 2.1.1B: Support. |
| CATT | Proposal 2.1.1A: For 1 CW, we slightly prefer to keep row 21-23, since the size of antenna port(s) field is not increased due to these 3 rows. For 2 CWs, Alt.2 is preferred and UE capability part is for further study. Proposal 2.1.1A: Support. |
| Huawei, HiSilicon | FL Proposal 2.1.1A: Not support. For 1CW, row 23 should be supported. Thanks to FL, the analysis and simulation in our contribution are attached above, which shows the important role of layer combination 3+3+2, especially under 8 layers scenario. Thanks QC’s great effort in detailed analyzing. In terms of each specific reason, following are the corresponding response: - The DMRS channel estimation is performed per port, we haven’t observe any special channel estimation degradation that [9, 11] will suffer. - Please see row 20, 21 for “Type2, maxLength1” DMRS in current spec., which crosses multiple CDM groups without any MU restriction. - Leave the different possible implementation alone, even if following QC’s implementation logic, [9, 11] is doable for UE supporting 2CW. - The whole WID is targeting higher-layer MU-MIMO, which we think should be a consensus among companies. - Among all scheduling cases listed by QC, except Case 2&3, which cannot be utilized for MU-MIMO according to the Rel.15 restriction and the performance requirement of MU scheduling, {3+3+2} is the only layer combination that hasn’t been supported by current spec. yet, which for completeness and realistic value attached above should be supported. - As discussed in our contribution, replacing {3+3+2} by {3+3+1+1} (or other layer combinations corresponding to more than 3 paired UEs) subjects to practical MU traffic, replacing {3+3+2} by {4+3+1} (or other layer combinations corresponding to at least one 4-layer UE) may bring MU performance degradation, furthermore, replacing {3+3+2} by {3+3+1} (or other layer combinations corresponding to less total layers) will obviously incur spectrum efficiency decrease. Now that {3+3+2} can bring performance benefit and scheduling flexibility for gNB side without incurring any devastating outcome, [9, 11] or other possible DMRS port combination facilitating {3+3+2} should be supported. Based on our analysis and simulation, we suggest to modify the 1CW part of FL Proposal 2.1.1A as below: FL Proposal 2.1.1A - For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, - For 1 CW, - Do not support row 21-22 - Support row 23 For 2CW, support Alt.1. FL Proposal 2.1.1B: Support. |
| Lenovo | FL Proposal 2.1.1A: For 1 CW, we prefer to include them to increase scheduling flexibility but can live up with the proposal on account of progress if it is the majority view. For 2 CW, support Alt.1 since for Alt.2 it will increase the UE complexity and the performance benefit by introduced rows is not a common view. FL Proposal 2.1.1B: Support. |
| Intel | Proposal 2.1.1A: For 2 CWs, do not support Alt-2. We have already concluded this discussion in the last meeting and similar behavior exists in legacy case as well. We only agree with Alt-1 and propose to confirm the WA. There is no major technical issue to support Alt-1. We also agree with DOCOMO that current formulation of Alt-2 may lead to unnecessary fragmentation. Proposal 2.1.1B: OK |
| QC | FL Proposal 2.1.1A: We support the proposal. For 2 CW, we support Alt 2. Functionality-wise, “values {0,1,2,3}” is the same as “values {4,5,6,7}”. We don’t see if a UE can report either “values {0,1,2,3} only” or “values {4,5,6,7} only” will cause market fragmentation. From UE implementation point of view, implementing “values {4,5,6,7}” is much easier than “values {0,1,2,3}”, which is why we introduce UE capability for “values {4,5,6,7} only” to accelerate the deployment of future Rel-18 UE supporting 8-layer DL MIMO. @Intel: Given Rel-15 8L DL-MIMO feature is not implemented/deployed in field yet. There is no backward compatibility issue if one just choosing to implement Rel-18 2 CWs PDSCH with Alt 2 and not implementing Rel-15 2 CWs PDSCH. Again, there is major implementation issue to support Alt-1 as we indicated in R1-2303576. Alt 2 is easier to implement on UE side, which will accelerate deployment 8L DL MIMO. FL Proposal 2.1.1B: If I understand the intention of this FL proposal correct, this proposal means the DMRS port table for S-TRP is reused for M-TRP (of course, with one additional row in M-TRP table, as agreed before). If that is the case, we can support the spirit of this proposal. But the wording of the proposal might need some clarification. |
| MediaTek | FL Proposal 2.1.1A: Support FL Proposal 2.1.1B: Support |
| ZTE | FL Proposal 2.1.1A: Support. For 2CWs case, we support Alt 1 but do NOT support Alt 2. We fail to see the logic that the legacy rule from Rel-15 (i.e., DMRS ports 0-4 for rank = 5 or DMRS ports 0-6 for rank 7 when Type 1+ double-symbol) cannot be reused in Rel-18. In other words, the added values in Alt 2 is the restriction over the legacy, which is out of scope from our perspective. FL Proposal 2.1.1B: Support. Given that DMRS ports {0, 2, 3} has already agreed in the last meeting, either to capture it repeatedly herein or not is fine. |
| Ericsson | FL Proposal 2.1.1A: For 1 CW, we prefer to keep row 21-23. For the antenna port combination of row 23 we are fine to support [9,11]. For 2 CWs, support Alt.1. For Alt.2 we are fine with adding the new values 4-7, but we have concern on different UE capabilities. We can support Alt2 if there’s consensus on basic UE capability for 2 CWs, either 0-3 or 4-7 can be considered as basic UE capability for 2 CWs. FL Proposal 2.1.1B: Support |
| Sharp | FL Proposal 2.1.1A: Support. FL Proposal 2.1.1B: Support. |
| Xiaomi | FL Proposal 2.1.1A: For 1 CW. As mentioned by Huawei, the intention to introduce larger number of orthogonal DMRS ports is to support more layers/UEs in MU-MIMO. Hence, we slightly prefer to support row 23. For 2 CWs, support Alt.-2. It is a good compromise if we can not reach an agreement on the working assumption. FL Proposal 2.1.1B: Support |
| Apple | FL Proposal 2.1.1A: Support FL Proposal 2.1.1B: Support |
| New H3C | FL Proposal 2.1.1A: Support. FL Proposal 2.1.1B: Support. |
| China Telecom | FL Proposal 2.1.1A: Support. For 2CWs case, we don’t support Alt2. The values row 0-3 should be considered first and be supported, which is align with the current Rel-15 in principle. Therefore, the new values row 4-7 are unnecessary and will cause the UE capabilities fragmentation if UE can only support the row [4-7]. Therefore, to avoid such fragmentation and values 0-3 must be supported, the values 4-7 are not needed. FL Proposal 2.1.1B: Support. |
| Samsung | FL Proposal 2.1.1A: Support, and for 2CWs, we support Alt1. FL Proposal 2.1.1B: We can live with the proposal. |
| Spreadtrum | FL Proposal 2.1.1A: Support. For 2CW case, we prefer to confirm the WA without modification. FL Proposal 2.1.1B: Support. |
| CMCC | FL Proposal 2.1.1A: For 1 CW, we are fine to support row 23 to keep the rank combination of {3+3+2} and left the scheduling flexibility to network. For 2 CWs, support Alt.1. FL Proposal 2.1.1B: Support. |
| LGE | FL Proposal 2.1.1A: Support. For 1 CW, do not support row 21-23. For 2 CW, support Alt.1. FL Proposal 2.1.1B: Support. |
| vivo | FL Proposal 2.1.1A: For 1 CW, we support. For 2 CWs, we support Alt 1. Firstly, there is no need to support redundant entries for the same target for rank=5~8. Secondly, in our understanding, channel estimation of DMRS ports and equalizer can be performed for all scheduled layers together, instead of being performed per CW. We fail to see the complexity of UE for this when using value {0,1,2,3}. Thirdly, association between CDM group and CW is not required for R15 DMRS for Rank=5~8, the legacy principle can be reused. FL Proposal 2.1.1B: Support. |
| FL | FL Proposal 2.1.1B is moved to [112bis-e-R18-MIMO-04] DMRS - EMAIL ENDORSEMENT 1. |
| Nokia/NSB2 | For 2CW, there is big performance difference in channel estimation when two ports or 3 ports are multiplexed into one CDM group. So, having worse 2 ports for CW0 of rank 5 (0,1/,2,3,8) should be avoided. (0,1,8 from CDM group 0, and two ports 0,1 are used for demodulation of CW0). We propose to mix them for each CW. With (0,2/3,8,9), each CW has at lease one port from CDM group 1 where only two ports are multiplexed. So, we propose to support (0,2,3,8,9) for rank 5. |
| QC | We thank FL very much for updating the UE capability proposal for 2 CWs. We fully support that update as it addresses companies concern on market fragmentation issue. @VIVO and all, please check the following figure which illustrate the UE complexity issue. Mapping in entries 4-7 are much more friendly to UE processing pipeline. As shown in following figure, for a UE does not have two channel estimation nor two demodulation modules (due to hardware limitation), using entries 0-3 cannot start demodulation for a CW until the channel estimation for CDM group 2 is completed, which causes additional delay to UE processing time. While entries 4-7 can start demodulation for CW 1 right away, after channel estimation for CDM group 1. If RAN1 as a community care about acceleration the deployment of 8L/2CWs DL MIMO, easier UE implementation of 2 CWs should be allowed. Regarding Rel-15 2CWs already mapped CW across two CDM groups, it is true. But given no one has implemented/deployed Rel-15 2CW feature, we don’t think there is backward compatible issue if one chooses to implement 2CW starting from Rel-18 spec. |

2.1.2 eType1, maxLength2

Next step is to make an agreement of DMRS port table for eType1, maxLength2.

Based on the agreements of eType1 maxLength1, we can observe the following principle.

- All rows of Cat.1 (Rel.15 legacy ports) are agreed.

  - Rows of rank 3-4 in Cat.1 (Rel.15 legacy ports) have MU-MIMO restriction.

  - Rows of rank 3-4 in Cat.2 (Rel.18 new ports) are with [ ] (probably, these rows are not supported).

- All rows of Number of DMRS CDM group(s) without data = 1 are agreed.

Hopefully, we can follow this principle to eType1, maxLength2.

The following table is the same table as sect. 2.1.2 in FL summary#2 in RAN1#112 (R1-2301775), and [ ] of some rows are removed based on the above principle.

- For the blue highlighted rows (rows with Number of DMRS CDM group(s) without data = 1), the benefit is for dynamic switching between MU-MIMO and SU-MIMO.

- For the yellow highlighted rows (rank 3-4 in Cat.1-2), row 9-10 is beneficial for sDCI mTRP. Row 26-30 and row 57-60 are useful to be multiplexed between two UEs with rank 3-4 for double symbol DMRS.

- Cat.3 is useful for lower DMRS overhead.

In RAN1#112, multiple companies commented that the max number of rows should not exceed 64, so that the DCI size is increased up to 1-bit. By removing row 40-42 and 61, the total number of rows are 64 now. Let’s try whether the following proposal can be agreed.

For 2CWs, it is pointed out that row 4-7 and row 8-11 are equivalent from DMRS overhead perspective. Hence, I add [] to row 8-11. Considering 2 CWs for eType1 maxLength1 is working assumption, row 4-7 for 2 CW is proposal for working assumption.

At least for S-TRP case

FL Proposal 2.1.2A

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-2-X.

  - FFS: For row 9-11 in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

  - FFS: The total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64.

Table 7.3.1.2.2-2-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 2 | 0-4 | 2] |
| 1 | 1 | 1 | 1 | [1 | 2 | 0,1,2,3,4,6 | 2] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,4,5,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,4,5,6,7 | 2] |
| 4 | 2 | 1 | 1 | [4 | 2 | 0,1,2,3,8 | 1] |
| 5 | 2 | 2 | 1 | [5 | 2 | 0,1,2,3,8,10 | 1] |
| 6 | 2 | 3 | 1 | [6 | 2 | 0,1,2,3,8,9,10 | 1] |
| 7 | 2 | 0,1 | 1 | [7 | 2 | 0,1,2,3,8,9,10,11 | 1] |
| [8 | 2 | 2,3 | 1] | [8 | 1 | 0,1,4,5,8 | 2] |
| [9 | 2 | 0-2 | 1] | [9 | 1 | 0,1,4,5,8,12 | 2] |
| [10 | 2 | 0-3 | 1] | [10 | 1 | 0,1,4,5,8,9,12 | 2] |
| [11 | 2 | 0,2 | 1] | [11 | 1 | 0,1,4,5,8,9,12,13 | 2] |
| 12 | 2 | 0 | 2 | [12 | 2 | 0,1,4,5,8 | 2] |
| 13 | 2 | 1 | 2 | [13 | 2 | 0,1,4,5,8,12 | 2] |
| 14 | 2 | 2 | 2 | [14 | 2 | 0,1,4,5,8,9,12 | 2] |
| 15 | 2 | 3 | 2 | [15 | 2 | 0,1,4,5,8,9,12,13 | 2] |
| 16 | 2 | 4 | 2 | [16 | 2 | 2,3,6,7,10 | 2] |
| 17 | 2 | 5 | 2 | [17 | 2 | 2,3,6,7,10,14 | 2] |
| 18 | 2 | 6 | 2 | [18 | 2 | 2,3,6,7,10,11,14 | 2] |
| 19 | 2 | 7 | 2 | [19 | 2 | 2,3,6,7,10,11,14,15 | 2] |
| 20 | 2 | 0,1 | 2 |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |
| [24 | 2 | 0,4 | 2] |  |  |  |  |
| [25 | 2 | 2,6 | 2] |  |  |  |  |
| [26 | 2 | 0,1,4 | 2] |  |  |  |  |
| [27 | 2 | 2,3,6 | 2] |  |  |  |  |
| [28 | 2 | 0,1,4,5 | 2] |  |  |  |  |
| [29 | 2 | 2,3,6,7 | 2] |  |  |  |  |
| [30 | 2 | 0,2,4,6 | 2] |  |  |  |  |
| 31 | 1 | 8 | 1 |  |  |  |  |
| 32 | 1 | 9 | 1 |  |  |  |  |
| 33 | 1 | 8,9 | 1 |  |  |  |  |
| 34 | 2 | 8 | 1 |  |  |  |  |
| 35 | 2 | 9 | 1 |  |  |  |  |
| 36 | 2 | 10 | 1 |  |  |  |  |
| 37 | 2 | 11 | 1 |  |  |  |  |
| 38 | 2 | 8,9 | 1 |  |  |  |  |
| 39 | 2 | 10,11 | 1 |  |  |  |  |
| [40 | 2 | 8-10 | 1] |  |  |  |  |
| [41 | 2 | 8-11 | 1] |  |  |  |  |
| [42 | 2 | 8,10 | 1] |  |  |  |  |
| 43 | 2 | 8 | 2 |  |  |  |  |
| 44 | 2 | 9 | 2 |  |  |  |  |
| 45 | 2 | 10 | 2 |  |  |  |  |
| 46 | 2 | 11 | 2 |  |  |  |  |
| 47 | 2 | 12 | 2 |  |  |  |  |
| 48 | 2 | 13 | 2 |  |  |  |  |
| 49 | 2 | 14 | 2 |  |  |  |  |
| 50 | 2 | 15 | 2 |  |  |  |  |
| 51 | 2 | 8,9 | 2 |  |  |  |  |
| 52 | 2 | 10,11 | 2 |  |  |  |  |
| 53 | 2 | 12,13 | 2 |  |  |  |  |
| 54 | 2 | 14,15 | 2 |  |  |  |  |
| [55 | 2 | 8,12 | 2] |  |  |  |  |
| [56 | 2 | 10,14 | 2] |  |  |  |  |
| [57 | 2 | 8,9,12 | 2] |  |  |  |  |
| [58 | 2 | 10,11,14 | 2] |  |  |  |  |
| [59 | 2 | 8,9,12,13 | 2] |  |  |  |  |
| [60 | 2 | 10,11,14,15 | 2] |  |  |  |  |
| 61 | 2 | 8,10,12,14 | 2 |  |  |  |  |
| 62 | 1 | 0,1,8 | 1 |  |  |  |  |
| 63 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 64 | 2 | 0,1,8 | 1 |  |  |  |  |
| 65 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 66 | 2 | 2,3,10 | 1 |  |  |  |  |
| 67 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| [69 | 1 | 0,1,8] | 2 |  |  |  |  |
| [70 | 1 | 0,1,8,9] | 2 |  |  |  |  |
| [71 | 1 | 4,5,12 | 2] |  |  |  |  |
| [72 | 1 | 4,5,12,13 | 2] |  |  |  |  |
| [73 | 2 | 0,1,8 | 2] |  |  |  |  |
| [74 | 2 | 0,1,8,9 | 2] |  |  |  |  |
| [75 | 2 | 4,5,12 | 2] |  |  |  |  |
| [76 | 2 | 4,5,12,13 | 2] |  |  |  |  |
| [77 | 2 | 2,3,10 | 2] |  |  |  |  |
| [78 | 2 | 2,3,10,11 | 2] |  |  |  |  |
| [79 | 2 | 6,7,14 | 2] |  |  |  |  |
| [80 | 2 | 6,7,14,15 | 2] |  |  |  |  |

------- end of the proposal --------

FL note: there is additional proposal to add the following rows for 1CW (Cat.3 with 2 symbols). MU capacity of supporting row 26-30 and row 57-60 are the same as supporting row 73-80. However, it seems row 73-80 seems to have lower UE complexity, because different TD-OCC is used for different UE for MU multiplexing.

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 69 | 1 | 0,1,8 | 2 |
| 70 | 1 | 0,1,8,9 | 2 |
| 71 | 1 | 4,5,12 | 2 |
| 72 | 1 | 4,5,12,13 | 2 |
| 73 | 2 | 0,1,8 | 2 |
| 74 | 2 | 0,1,8,9 | 2 |
| 75 | 2 | 4,5,12 | 2 |
| 76 | 2 | 4,5,12,13 | 2 |
| 77 | 2 | 2,3,10 | 2 |
| 78 | 2 | 2,3,10,11 | 2 |
| 79 | 2 | 6,7,14 | 2 |
| 80 | 2 | 6,7,14,15 | 2 |

For M-TRP case

FL Proposal 2.1.2B

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

  - All rows for Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - For one CW, add new row 68 in Table 7.3.1.2.2-2A-X.

Table 7.3.1.2.2-2A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 68 | 2 | 0,2,3 | 1 |

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Proposal 2.1.2A: Support. Proposal 2.1.2B: Support. But, the total number of rows is 65 for sDCI mTRP, and at least one row can be removed in case of sDCI mTRP to keep 64 rows. For example, some rows of Cat.3 may not be useful for CDM based sDCI mTRP. However, Cat.3 is useful for TDM/FDM/SFN based sDCI mTRP operation. We believe row 30 is not useful for sDCI mTRP, and suggest to remove row 30 for sDCI mTRP to keep 64 rows. |
| Google | 2.1.2A: Support 2.1.2B: Support |
| OPPO | For proposal 2.1.2A: 1. we think Row 8-11 are not needed, which have similar overhead and performance as Row 4-7. 2. Considering a UE with two CWs is not likely to be scheduled with MU-MIMO, the use case of Row 12-19 for two CWs is also unclear to us. 3. For S-DCI based M-TRP, the number of rows would exceed 64 and would increase one more bit. It is proposed to remove row 55 and 56 to avoid the overhead. Row 55-56 is equivalent to Row 24-25, and cannot provide any additional use case. Proposal 2.1.2B: Support. |
| Nokia/NSB | Proposal 2.1.2A: We don’t support for increasing DCI size by 2bits, so the additional rows should be minimized. - Row 9,10, 30: Do not support - 2CWs, we don’t see need for the row 0-3 (2 symbol). Does this for dynamic switching of FD-OCC2 and FD-OCC4? 0-3 need double overhead. So, we propose to use the same table as “maxLength=1” for two CWs. Proposal 2.1.2B: Support the proposal. |
| CATT | Proposal 2.1.2A: Support the proposal without FL note. Cat.3 port combinations with 2 symbols will increase the overhead of antenna port(s) field. Proposal 2.1.2B: Support. |
| Huawei, HiSilicon | FL Proposal 2.1.2A: Generally fine. Some rows need to be deleted or discussed. - Remove row 55&56. The benefit of row 24 and 25 is that only TD-OCC dispreading is adopted during channel estimation, which means other co-scheduled DMRS ports with different FD-OCC are not expected to be within the same CDM group. Following this principle, entry 55 and 56 are useless. - Row 57~60 should be further discussed to facilitate more layer combinations. - For 2CW, at least row 8~11 is also needed to facilitate supporting rank>4 with only 1 symbol, which can improve the efficiency of resource utilization and scheduling flexibility. FL Proposal 2.1.2B: Support. |
| Lenovo | Proposal 2.1.2A: We prefer to delete one additional row, such as row 60, to make it compatible with MTRP case with 1 additional bit. Proposal 2.1.2B: Support |
| Intel | Proposal 2.1.2A: We prefer to support Rows 73-80 (reason is already clarified by FL) Proposal 2.1.2B: OK |
| QC | Proposal 2.1.2A: Do not support. For 1 CW, specifically, we think rows 24-30, 55-61 should be removed or kept with MU restriction. We are fine with other rows. If those rows are used for MU, we don’t think it is a good MU design from system performance point of view, because the ports are distributed into two TD-OCC codes which see interference from other MU from both TD-OCCs. A better design should put the ports in a TD-OCC as much as possible to minimize the interference between MU, which is similar to the Rel-15 principle to put ports of a UE in a CDM group as much as possible. More critically, from UE implementation point of view, putting DMRS ports into 2 TD-OCC codes forces a UE to estimate 8 DMRS ports (in both TD-OCCs in one CDM group) for MU detection and noise/interference estimation. For example, a UE is signaled with row 24 {0,4}. For the UE to detect existence of MU, it must estimate channel of ports {1,8,9, 5,12,13} to see if there is MU on any of them, which effectively requires UE to estimate 8 DMRS ports. However, this row is for 1CW and a UE only supporting 1 CW can only estimate 4 DMRS ports, so those rows (intended for 1 CW) do not work on UE side. If those rows are used for SU, then we need add restriction to exclude MU for them. For rows 69-80, we support them and we agree with FL’s notes (except there is typo “different CDM group is used” should be “different TD-OCC is used”). Actually, if we comparing rows 69-80 with rows 24-30, 55-61, they have same functionality, i.e., for rank 3-4 with 1 CW. But, rows 69-80 are better design than rows 24-30, 55-61 because putting ports in one TD-OCC code can reduce MU interference, which is aligned with Rel-15 principle. Also, it does not create MU detection issue for UE implementation. Therefore, we think we should remove rows 24-30, 55-61 and adopt rows 69-80. For 2CWs, we don’t support the entries in current proposal because they map layers of 1 CW to two CDM groups. We still prefer map layers of 1 CW to one CDM group. We think we can defer the decision on 2CW after we decide whether/how to confirm the WA for 2CWs type 1 1-symbol DMRS. Proposal 2.1.2B: Support. |
| MediaTek | Proposal 2.1.2A: Do not support. With regards to two CV entries, we believe we should strive to have a one-to-one mapping between CDM and CW, i.e., one CDM group should be at most map to one CW. This would allow for much simpler UE implementation without loss of performance. We also agree with the comment made by QC on rows 24-30, 55-61 and prefer to add MU MIMO restrictions. Proposal 2.1.2B: Support |
| ZTE | FL Proposal 2.1.2A: Support w/o the additional proposal. In principle, we think two aspects should be guaranteed for eType1 + maxLength2: i) The legacy rules should be completely captured, i.e. all rows in terms of Cat. 1; ii) The same rules of eType1 + maxLength1 should be reused as much as possible. Regarding the highlighted rows for 1 CW case seems kind of controversial, we’d like to share further view as follows: - Rows 0-2 and 9-10 are in line with aspect i) in terms of rows 0-2 and 9-10 when Rel-15 Type1 + maxLength2, these should be supportive. - Rows 26-30 are in line with aspect i) in terms of rows 26-30 when Rel-15 Type1 + maxLength2, these should be supportive. - Someone roughly argued that rows 26-30 revert the legacy restriction of MU-MIMO, hence these should be removed or replaced. To our understanding of the legacy as in TS 38.214, it is crystal clear that only row 30 herein is not to be used in MU-MIMO. It is more accurate to reuse the legacy MU-MIMO restriction on this single row 30 instead of removing all of rows 26-30 which align with the legacy rule. - In addition, someone also argued that rows 26-30 will lead to interference between two TD-OCC codes so that the reverting of TD-OCC cannot be fulfilled anymore to alleviate performance loss especially in large delay spread scenario (that needs to be handled by rows 69-80 as listed in the additional proposal). Recalling that someone insisted in previous meetings the length of FD-OCC de-spreading can be implemented by UE randomly via some advance processing to handle the performance loss in case of large delay spread scenario, it is quite confusing why limit DMRS ports within one symbol could be more reasonable for this purpose??? We fail to see the concern of the so-called UE complexity according to companies arguments so far. For DM-RS configuration type 1, - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 30} in Table 7.3.1.2.2-1 and Table 7.3.1.2.2-2 of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 12} in Table 7.3.1.2.2-1A and {2, 9, 10, 11, 30 or 31} in Table 7.3.1.2.2-2A of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with two codewords, the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.. - Rows 31-33 are in line with aspect ii) in terms of the agreed rows 12-14 when Rel-18 eType1 + maxLength1, these should be supportive. - Rows 57-60 are the same to rows 26-29 in principle, these should be supportive. Notably, rows 57-60 can be used in MU-MIMO as legacy (i.e. rows 26-29 when Rel-15 Type1 + maxLength2), the use case is crystal clear to us. - Rows 62-63 are in line with aspect ii) in terms of the agreed rows 24-25 when Rel-18 eType1 + maxLength1, these should be supportive. - Rows 69-80 in the additional proposal, do NOT support as the above elaboration of rows 26-30. FL Proposal 2.1.2B: Support. In particular, we do believe scheduling flexibility/completeness is the higher priority over DCI increased with 1-bit (also noticed there are even more than 1 bit reserved/unused for PUSCH DMRS indication tables in Rel-15), removing the rows of SDCI MTRP case for avoiding DCI increased with 1-bit is unconvincing to us. |
| Ericsson | Proposal 2.1.2A: Support. We also support rows 69-80 if those are preferred by many UE chipset vendors to simplify the UE implementation. On MU-MIMO restriction, we don’t prefer to add MU restriction on each row consisting of ports from more than one CDM group or more than one TD-OCC code, we’d like to understand if such restriction is essential for most of the UE implementations. Proposal 2.1.2B: Support. |
| QC | To ZTE: the challenge on UE implementation is quite clear. A UE will need to estimate the channel of all ports in a CDM group or a TD-OCC (i.e., a box) in the following figure to detect whether MU exist and estimate the interference from the MU (if exist). Those rows 24-30, 55-61 will require UE to estimate 8 DMRS ports. Please notice that this is infeasible to do for a UE only supports 1 CW. Please also notice that UE can decode 2 CW is still quite futuristic in deployment. I hope the above is clear and answers your question on UE implementation. |
| Sharp | Proposal 2.1.2A: We may remove Row 30 because Row 10 can be used for fallback into SU-MIMO with 4 layers. Even if Row 30 is not removed, at least MU-MIMO restriction is needed for Row 30. Proposal 2.1.2B: Support. |
| Xiaomi | FL Proposal 2.1.2A: Generally fine with proposal 2.1.2A. FL Proposal 2.1.2B: As discussed in our contribution, DMRS ports (0,2,3) is additionally added to support flexible layers combination for 3 layers PDSCH in MTRP NCJT in R16. It could be (2+1) when (0,1,2) is indicated or (1+2) when (0,2,3) is indicated. However, for MTRP CJT, all the three layers of PDSCH will be mapped to these TRPs. From our perspective, there is no need to support (0,2,3) for MTRP CJT, which could reuse the same antenna port(s) combinations for STRP. Therefore, we suggest the following modification to Proposal 2.1.2B: FL Proposal 2.1.2B For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-DCI based M-TRP NCJT case, … |
| Apple | FL Proposal 2.1.2A: Fine with proposal and are also fine to consider additional MU-MIMO restriction such as proposed by QC for 1 CW FL Proposal 2.1.2B: Support |
| New H3C | Proposal 2.1.2A: Support the proposal without FL note. Proposal 2.1.2B: Support. |
| China Telecom | Proposal 2.1.2A: We support the proposal but not the rows 69-80. The values in row 26-30 have already supported in the Rel-15 Table 7.3.1.2.2-2A, which so we wonder why the row 73-80 will reduce the UE complexity since the capability should already be supported by the UE. But to remove the row 69-80 can reduce the UE’s capability since the purpose of introducing the rows can already be achieved with the existing rows. Proposal 2.1.2B: Support. |
| CMCC | FL Proposal 2.1.2A: For 1 CW, we don’t agree to add MU restriction on rows consisting of ports from only 1 CDM group. One of the motivations to increasing DMRS ports is for MU-MIMO, if adding MU restriction on each row consisting of ports from more than one TD-OCC, the performance of increasing DMRS ports is degraded. For 2 CW, row 8~11 is needed to support rank>4 transmission with 1 symbol. FL Proposal 2.1.2B: Support. |
| FL | I added [ ] to the rows based on your inputs. Since multiple companies support row 69-80 instead of row 26-30/57-60, both set of rows are kept with []. Re Xiaomi’s comment of NCJT, based on the current spec, switching between the two DMRS table is done by the number of active TCIs per TCI codepoint. It is not switched by M-TRP NCJT vs. S-TRP or SFN vs S-TRP. Hence, I didn’t capture your comment. - Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4 or Tables 7.3.1.2.2-1A/2A/3A/4A. When a UE receives an activation command that maps at least one codepoint of DCI field 'Transmission Configuration Indication' to two TCI states, the UE shall use Table 7.3.1.2.2-1A/2A/3A/4A; otherwise, it shall use Tables 7.3.1.2.2-1/2/3/4. The UE can receive an entry with DMRS ports equals to 1000, 1002, 1003 when two TCI states are indicated in a codepoint of DCI field 'Transmission Configuration Indication'.. |
| LGE | Proposal 2.1.2A: Support. MU-MIMO restriction issue for UE implementation. Therefore, we think we should remove rows 40-42, 61. For 1 CW(Cat.3 with 2 symbols), we do not support row 69-80 instead of row 26-30/57-60. Proposal 2.1.2B: Support. |
| Xiaomi2 | @FL, thanks for your response. From our understanding, MTRP CJT can use the same antenna ports combinations with STRP, maybe the same antenna ports table. We understand that the switching between the two DMRS table is done by the number of active TCIs per TCI codepoint. But it does not mean MTRP CJT and NCJT should use the same antenna ports combinations. Whether MTRP CJT and NCJT should use the same antenna ports combinations/table can be decided first. Then, if they need to use different antenna ports tables, new ways to perform switching between the two DMRS table for MTRP CJT and NCJT can be introduced. Well, in short, we believe that at least whether MTRP CJT and MTRP NCJT should use the same antenna ports table should be discussed. |
| FL | After some offline discussion, Xiaomi kindly accepted the Proposal 2.1.2B. Based on the current spec., Tables 7.3.1.2.2-1A/2A/3A/4A (DMRS tables for sDCI mTRP) can be used whenever two TCI states are activated for PDSCH. However, the new entry can be indicated only when the following UE capability is reported. Similarly, we can discuss which scenario the new entry of sDCI mTRP will be used in UE feature session. 16-2b-1b; Single-DCI based SDM scheme – Support of new DMRS port entry; 1. Support of new DMRS port entry {0, 2, 3}; 16-2b-1; supportNewDMRS-Port-r16; MIMO-ParametersPerBand; n/a; n/a; Optional with capability signaling. FL: I added FFS to MU-MIMO restriction, because row 9-11 is now FFS. |
| ZTE2 | Re the updated FL Proposal 2.1.2A, the motivation of the newly added bullet is unreasonable to us. We think indication field needs to be increased to guarantee the scheduling flexibility. We think it should be marked as FFS. FFS: The total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64. FL: I added FFS to the limit of 64 rows. To QC: Why this kind of restriction you emphasized is specially needed to Rel-18 UE but not for Rel-15 UE? Notably, the current spec does NOT have such restriction as we pointed out above. |
| FL | FL Proposal 2.1.2B is moved to [112bis-e-R18-MIMO-04] DMRS - EMAIL ENDORSEMENT 1. |
| QC3 | @ZTE, please read our input/comments provided above and in section 2.6, which has been provided 24 hours ago. Please let us know if you have any technical question to our input. @FL, for this proposal, we commented above that we want to delete or add MU restriction to row 24-30. But [] so far are missed on row 24-25. Can you please add [] for those two rows as well, similar as other rows which are controversial. For 2CWs, we thank FL for updating the proposal by adding [] on certain entries. But we don’t support the entries in current proposal because they map layers of 1 CW to two CDM groups. We still prefer map layers of 1 CW to one CDM group. We think we can defer the decision on 2CW after we decide whether/how to confirm the WA for 2CWs type 1 1-symbol DMRS. |
| FL(v40) | For 2.1.2A, I put [ ] to all rows of 2 CWs and row 24-25 of 1CW (Sorry QC for missing your comment). |

2.1.3 eType2, maxLength1

For eType2 maxLength1, the following principle of eType1 maxLength1 can be reused.

- All rows of Cat.1 (Rel.15 legacy ports) are agreed.

- All rows of Number of DMRS CDM group(s) without data = 1 are agreed.

For MU-MIMO restriction of Type2 in Rel.15, only row 23 has MU-MIMO restriction in Rel.15. If we follow this MU-MIMO restriction, there is no point to support row 47 (12,14). Hence, FL suggestion is to remove the row 47.

For row 33-34 and row 44-46 (i.e. rank 3-4 in Cat.2), it has no use-case because Cat.3 is better in terms of DMRS overhead and MU multiplexing. Hence, these rows can be removed (For now, these rows are with [ ] in the table).

At least for S-TRP case

FL Proposal 2.1.3A

- For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-3-X.

  - FFS: For row 23 in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-3-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | [2 | 3 | 12-16] |
| 3 | 2 | 0 | [3 | 3 | 12-17] |
| 4 | 2 | 1 | [4 | 2 | 0,1,2,3,12] |
| 5 | 2 | 2 | [5 | 2 | 0,1,2,3,12,14] |
| 6 | 2 | 3 | [6 | 2 | 0-3,12-14] |
| 7 | 2 | 0,1 | [7 | 2 | 0-3,12-15] |
| 8 | 2 | 2,3 | [8 | 3 | 0,1,2,3,12] |
| [9 | 2 | 0-2] | [9 | 3 | 0,1,2,3,12,14] |
| [10 | 2 | 0-3] | [10 | 3 | 0-3,12-14] |
| 11 | 3 | 0 | [11 | 3 | 0-3,12-15] |
| 12 | 3 | 1 | [12 | 2 | 0,2,3,12,13] |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| [20 | 3 | 0-2] |  |  |  |
| [21 | 3 | 3-5] |  |  |  |
| [22 | 3 | 0-3] |  |  |  |
| [23 | 2 | 0,2] |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| [33 | 2 | 12-14] |  |  |  |
| [34 | 2 | 12-15] |  |  |  |
| 35 | 3 | 12 |  |  |  |
| 36 | 3 | 13 |  |  |  |
| 37 | 3 | 14 |  |  |  |
| 38 | 3 | 15 |  |  |  |
| 39 | 3 | 16 |  |  |  |
| 40 | 3 | 17 |  |  |  |
| 41 | 3 | 12,13 |  |  |  |
| 42 | 3 | 14,15 |  |  |  |
| 43 | 3 | 16,17 |  |  |  |
| [44 | 3 | 12-14] |  |  |  |
| [45 | 3 | 15-17] |  |  |  |
| [46 | 3 | 12-15] |  |  |  |
| [47 | 2 | 12,14] |  |  |  |
| 48 | 1 | 0,1,12 |  |  |  |
| 49 | 1 | 0,1,12,13 |  |  |  |
| 50 | 2 | 0,1,12 |  |  |  |
| 51 | 2 | 0,1,12,13 |  |  |  |
| 52 | 2 | 2,3,14 |  |  |  |
| 53 | 2 | 2,3,14,15 |  |  |  |
| 54 | 3 | 0,1,12 |  |  |  |
| 55 | 3 | 0,1,12,13 |  |  |  |
| 56 | 3 | 2,3,14 |  |  |  |
| 57 | 3 | 2,3,14,15 |  |  |  |
| 58 | 3 | 4,5,16 |  |  |  |
| 59 | 3 | 4,5,16,17 |  |  |  |

For M-TRP case

FL Proposal 2.1.3B

- For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

  - All rows for Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH for S-TRP.

  - For one CW, add new row 60 in Table 7.3.1.2.2-3A-X.

Table 7.3.1.2.2-3A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 60 | 2 | 0,2,3 |

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Proposal 2.1.3A: Support. Proposal 2.1.3B: Support. |
| Google | Proposal 2.1.3A: Support. Proposal 2.1.3B: Support. |
| OPPO | Proposal 2.1.3A: 1. We agree that Row 33-34,Row 44-47 are not needed. 2. Considering a UE with two CWs is not likely to be scheduled with MU-MIMO, the use case of Row 2-3 and Row 8-11 for two CWs is unclear to us. Proposal 2.1.3B: Support. |
| Nokia/NSB | Proposal 2.1.3A: - For 1CW, not support row 9,10,20,21,22, 33,34, 44-46 - For 2CWs, - Not support row 2,3,8 and 9 - propose to add {0,2,3,12,13} # of CDM group without data=2. We don’t see need for Number of DMRS CDM group(s) without data; DMRS port(s). 2; 0,2,3,12,13. Proposal 2.1.3B: Support |
| CATT | Proposal 2.1.3A: Support in principle. We slightly prefer to keep rows for SU-MIMO (e.g., row 44-47) if the size of antenna port(s) field is not increased. Proposal 2.1.3B: Support. |
| Huawei, HiSilicon | FL Proposal 2.1.3A: For 1CW, support. For 2CW, considering it is used for SU scenario, row 8~11 is not needed. FL Proposal 2.1.3B: Support. |
| Lenovo | Proposal 2.1.3A: Support. Proposal 2.1.3B: Support. |
| Intel | Proposal 2.1.3A: OK to support. We also think all rows from legacy tables should be kept. Proposal 2.1.3B: OK |
| QC | Proposal 2.1.3A: We don’t support this proposal. For 1 CW, specifically, we think rows 9-10, 20-23,33-34,44-47, should be either removed or kept with MU scheduling restriction. The reason is because they have DMRS ports distributed across two CDM groups. It is the same issue as discussed for Type 1 single symbol DMRS. RAN1 can treat them in a same way as Type 1 DMRS. For 2CWs, we don’t support the entries in current proposal because they map layers of 1 CW to two CDM groups. We still prefer map layers of 1 CW to one CDM group. We think we can defer the decision on 2CW after we decide whether/how to confirm the WA for 2CWs type 1 1-symbol DMRS. Proposal 2.1.3B: Support. |
| ZTE | FL Proposal 2.1.3A: Support. - Regarding row 47, we tend to agree with FL’s assessment that it seems useless on top of the legacy MU-MIMO restriction. - Regarding rows 33-34 and 44-46, these should be reserved for MU-MIMO scenario in addition to the intention of DMRS overhead saving and MU multiplexing. Notably, there is no any MU-MIMO restriction of rows 9-10 and 20-22 for rank = 3/4 when Rel-15 Type2 + maxlength2. FL Proposal 2.1.3B: Support. |
| Ericsson | Proposal 2.1.3A: Support. On MU-MIMO restriction, we don’t prefer to add MU restriction on each row consisting of ports from more than one CDM group, we’d like to understand if such restriction is essential for most of the UE implementations. Proposal 2.1.3B: Support. |
| Sharp | Proposal 2.1.3A: Support Proposal 2.1.3B: Support |
| Xiaomi | FL Proposal 2.1.3A: Support FL Proposal 2.1.3B: At least for S-DCI based M-TRP NCJT, we are OK with this proposal. But, whether (0,2,3) is needed for MTRP CJT needs more discussion. |
| Apple | FL Proposal 2.1.3A: Do not support entries 33,34,44,45,46. Similar principle as for eType 1 should be applied here , i.e. no new ports entries with DMRS ports spanning multiple CDM groups. Or at least MU-MIMO restriction should be agreed for these entries. Also for entries 9,10,20,21,22,23, MU-MIMO restriction should be agreed FL Proposal 2.1.3B: Support |
| New H3C | Proposal 2.1.3A: Support Proposal 2.1.3B: Support |
| China Telecom | Proposal 2.1.3A: Support. Proposal 2.1.3B: Support |
| CMCC | Proposal 2.1.3A: Support. Proposal 2.1.3B: Support. |
| LGE | Proposal 2.1.3A: Support. Proposal 2.1.3B: Support. |
| FL | I added FFS to MU-MIMO restriction, because row 23 is now FFS. |
| FL | FL Proposal 2.1.3B is moved to [112bis-e-R18-MIMO-04] DMRS - EMAIL ENDORSEMENT 1. |
| QC3 | For 2CWs, we thank FL for updating the proposal by adding [] on certain entries. But we don’t support the entries in current proposal because they map layers of 1 CW to two CDM groups. We still prefer map layers of 1 CW to one CDM group. We think we can defer the decision on 2CW after we decide whether/how to confirm the WA for 2CWs type 1 1-symbol DMRS. |
| FL(v40) | For 2.1.2A, I put [ ] to all rows of 2 CWs. |

2.1.4 eType2, maxLength2

For eType2 maxLength2, the following principle of eType1 maxLength1 can be reused.

- All rows of Cat.1 (Rel.15 legacy ports) are agreed.

- All rows of Number of DMRS CDM group(s) without data = 1 are agreed.

For MU-MIMO restriction of Type2 in Rel.15, only row 23 has MU-MIMO restriction in Rel.15. If we follow this MU-MIMO restriction, there is no point to support row 81 (12,14). Hence, FL suggestion is to remove the row 81.

For row 67-68 and row 78-80 (i.e. rank 3-4 in Cat.2), it has no use-case because Cat.3 is better in terms of DMRS overhead and MU multiplexing. Hence, these rows can be removed (For now, these rows are with [ ] in the table).

At least for S-TRP case

FL Proposal 2.1.4A

- For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-4-X.

  - FFS: For row 23 in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-4-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 3 | 0-4 | 1] |
| 1 | 1 | 1 | 1 | [1 | 3 | 0-5 | 1] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,6,8 | 2] |
| 4 | 2 | 1 | 1 | [4 | 2 | 0,1,2,3,6,7,8 | 2] |
| 5 | 2 | 2 | 1 | [5 | 2 | 0,1,2,3,6,7,8,9 | 2] |
| 6 | 2 | 3 | 1 | [6 | 2 | 0,1,2,3,12 | 1] |
| 7 | 2 | 0,1 | 1 | [7 | 2 | 0-3,12,14 | 1] |
| 8 | 2 | 2,3 | 1 | [8 | 2 | 0-3,12-14 | 1] |
| [9 | 2 | 0-2 | 1] | [9 | 2 | 0-3,12-15 | 1] |
| [10 | 2 | 0-3 | 1] | [10 | 3 | 0,1,2,3,12 | 1] |
| 11 | 3 | 0 | 1 | [11 | 3 | 0-3,12,14 | 1] |
| 12 | 3 | 1 | 1 | [12 | 3 | 0-3,12-14 | 1] |
| 13 | 3 | 2 | 1 | [13 | 3 | 0-3,12-15 | 1] |
| 14 | 3 | 3 | 1 | [14 | 1 | 0,1,6,7,12 | 2] |
| 15 | 3 | 4 | 1 | [15 | 1 | 0,1,6,7,12,18 | 2] |
| 16 | 3 | 5 | 1 | [16 | 1 | 0,1,6,7,12,13,18 | 2] |
| 17 | 3 | 0,1 | 1 | [17 | 1 | 0,1,6,7,12,13,18,19 | 2] |
| 18 | 3 | 2,3 | 1 | [18 | 2 | 0,1,6,7,12 | 2] |
| 19 | 3 | 4,5 | 1 | [19 | 2 | 0,1,6,7,12,18 | 2] |
| [20 | 3 | 0-2 | 1] | [20 | 2 | 0,1,6,7,12,13,18 | 2] |
| [21 | 3 | 3-5 | 1] | [21 | 2 | 0,1,6,7,12,13,18,19 | 2] |
| [22 | 3 | 0-3 | 1] | [22 | 2 | 2,3,8,9,14 | 2] |
| [23 | 2 | 0,2 | 1] | [23 | 2 | 2,3,8,9,14,20 | 2] |
| 24 | 3 | 0 | 2 | [24 | 2 | 2,3,8,9,14,15,20 | 2] |
| 25 | 3 | 1 | 2 | [25 | 2 | 2,3,8,9,14,15,20,21 | 2] |
| 26 | 3 | 2 | 2 | [26 | 3 | 0,1,6,7,12 | 2] |
| 27 | 3 | 3 | 2 | [27 | 3 | 0,1,6,7,12,18 | 2] |
| 28 | 3 | 4 | 2 | [28 | 3 | 0,1,6,7,12,13,18 | 2] |
| 29 | 3 | 5 | 2 | [29 | 3 | 0,1,6,7,12,13,18,19 | 2] |
| 30 | 3 | 6 | 2 | [30 | 3 | 2,3,8,9,14 | 2] |
| 31 | 3 | 7 | 2 | [31 | 3 | 2,3,8,9,14,20 | 2] |
| 32 | 3 | 8 | 2 | [32 | 3 | 2,3,8,9,14,15,20 | 2] |
| 33 | 3 | 9 | 2 | [33 | 3 | 2,3,8,9,14,15,20,21 | 2] |
| 34 | 3 | 10 | 2 | [34 | 3 | 4,5,10,11,16 | 2] |
| 35 | 3 | 11 | 2 | [35 | 3 | 4,5,10,11,16,22 | 2] |
| 36 | 3 | 0,1 | 2 | [36 | 3 | 4,5,10,11,16,17,22 | 2] |
| 37 | 3 | 2,3 | 2 | [37 | 3 | 4,5,10,11,16,17,22,23 | 2] |
| 38 | 3 | 4,5 | 2 |  |  |  |  |
| 39 | 3 | 6,7 | 2 |  |  |  |  |
| 40 | 3 | 8,9 | 2 |  |  |  |  |
| 41 | 3 | 10,11 | 2 |  |  |  |  |
| [42 | 3 | 0,1,6 | 2] |  |  |  |  |
| [43 | 3 | 2,3,8 | 2] |  |  |  |  |
| [44 | 3 | 4,5,10 | 2] |  |  |  |  |
| [45 | 3 | 0,1,6,7 | 2] |  |  |  |  |
| [46 | 3 | 2,3,8,9 | 2] |  |  |  |  |
| [47 | 3 | 4,5,10,11 | 2] |  |  |  |  |
| 48 | 1 | 0 | 2 |  |  |  |  |
| 49 | 1 | 1 | 2 |  |  |  |  |
| 50 | 1 | 6 | 2 |  |  |  |  |
| 51 | 1 | 7 | 2 |  |  |  |  |
| 52 | 1 | 0,1 | 2 |  |  |  |  |
| 53 | 1 | 6,7 | 2 |  |  |  |  |
| 54 | 2 | 0,1 | 2 |  |  |  |  |
| 55 | 2 | 2,3 | 2 |  |  |  |  |
| 56 | 2 | 6,7 | 2 |  |  |  |  |
| 57 | 2 | 8,9 | 2 |  |  |  |  |
| 58 | 1 | 12 | 1 |  |  |  |  |
| 59 | 1 | 13 | 1 |  |  |  |  |
| 60 | 1 | 12,13 | 1 |  |  |  |  |
| 61 | 2 | 12 | 1 |  |  |  |  |
| 62 | 2 | 13 | 1 |  |  |  |  |
| 63 | 2 | 14 | 1 |  |  |  |  |
| 64 | 2 | 15 | 1 |  |  |  |  |
| 65 | 2 | 12,13 | 1 |  |  |  |  |
| 66 | 2 | 14,15 | 1 |  |  |  |  |
| [67 | 2 | 12-14 | 1] |  |  |  |  |
| [68 | 2 | 12-15 | 1] |  |  |  |  |
| 69 | 3 | 12 | 1 |  |  |  |  |
| 70 | 3 | 13 | 1 |  |  |  |  |
| 71 | 3 | 14 | 1 |  |  |  |  |
| 72 | 3 | 15 | 1 |  |  |  |  |
| 73 | 3 | 16 | 1 |  |  |  |  |
| 74 | 3 | 17 | 1 |  |  |  |  |
| 75 | 3 | 12,13 | 1 |  |  |  |  |
| 76 | 3 | 14,15 | 1 |  |  |  |  |
| 77 | 3 | 16,17 | 1 |  |  |  |  |
| [78 | 3 | 12-14 | 1] |  |  |  |  |
| [79 | 3 | 15-17 | 1] |  |  |  |  |
| [80 | 3 | 12-15 | 1] |  |  |  |  |
| [81 | 2 | 12,14 | 1] |  |  |  |  |
| 82 | 3 | 12 | 2 |  |  |  |  |
| 83 | 3 | 13 | 2 |  |  |  |  |
| 84 | 3 | 14 | 2 |  |  |  |  |
| 85 | 3 | 15 | 2 |  |  |  |  |
| 86 | 3 | 16 | 2 |  |  |  |  |
| 87 | 3 | 17 | 2 |  |  |  |  |
| 88 | 3 | 18 | 2 |  |  |  |  |
| 89 | 3 | 19 | 2 |  |  |  |  |
| 90 | 3 | 20 | 2 |  |  |  |  |
| 91 | 3 | 21 | 2 |  |  |  |  |
| 92 | 3 | 22 | 2 |  |  |  |  |
| 93 | 3 | 24 | 2 |  |  |  |  |
| 94 | 3 | 12,13 | 2 |  |  |  |  |
| 95 | 3 | 14,15 | 2 |  |  |  |  |
| 96 | 3 | 16,17 | 2 |  |  |  |  |
| 97 | 3 | 18,19 | 2 |  |  |  |  |
| 98 | 3 | 20,21 | 2 |  |  |  |  |
| 99 | 3 | 22,23 | 2 |  |  |  |  |
| [100 | 3 | 12,13,18 | 2] |  |  |  |  |
| [101 | 3 | 14,15,20 | 2] |  |  |  |  |
| [102 | 3 | 16,17,22 | 2] |  |  |  |  |
| [103 | 3 | 12,13,18,19 | 2] |  |  |  |  |
| [104 | 3 | 14,15,20,21 | 2] |  |  |  |  |
| [105 | 3 | 16,17,22,23 | 2] |  |  |  |  |
| 106 | 1 | 12 | 2 |  |  |  |  |
| 107 | 1 | 13 | 2 |  |  |  |  |
| 108 | 1 | 18 | 2 |  |  |  |  |
| 109 | 1 | 19 | 2 |  |  |  |  |
| 110 | 1 | 12,13 | 2 |  |  |  |  |
| 111 | 1 | 18,19 | 2 |  |  |  |  |
| 112 | 2 | 12,13 | 2 |  |  |  |  |
| 113 | 2 | 14,15 | 2 |  |  |  |  |
| 114 | 2 | 18,19 | 2 |  |  |  |  |
| 115 | 2 | 20,21 | 2 |  |  |  |  |
| 116 | 1 | 0,1,12 | 1 |  |  |  |  |
| 117 | 1 | 0,1,12,13 | 1 |  |  |  |  |
| 118 | 2 | 0,1,12 | 1 |  |  |  |  |
| 119 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 120 | 2 | 2,3,14 | 1 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 122 | 3 | 0,1,12 | 1 |  |  |  |  |
| 123 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 124 | 3 | 2,3,14 | 1 |  |  |  |  |
| 125 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 126 | 3 | 4,5,16 | 1 |  |  |  |  |
| 127 | 3 | 4,5,16,17 | 1 |  |  |  |  |
| [129 | 1 | 0,1,12 | 2] |  |  |  |  |
| [130 | 1 | 0,1,12,13 | 2] |  |  |  |  |
| [131 | 1 | 6,7,18 | 2] |  |  |  |  |
| [132 | 1 | 6,7,18,19 | 2] |  |  |  |  |
| [133 | 2 | 0,1,12 | 2] |  |  |  |  |
| [134 | 2 | 0,1,12,13 | 2] |  |  |  |  |
| [135 | 2 | 6,7,18 | 2] |  |  |  |  |
| [136 | 2 | 6,7,18,19 | 2] |  |  |  |  |
| [137 | 2 | 2,3,14 | 2] |  |  |  |  |
| [138 | 2 | 2,3,14,15 | 2] |  |  |  |  |
| [139 | 2 | 8,9,20 | 2] |  |  |  |  |
| [140 | 2 | 8,9,20,21 | 2] |  |  |  |  |
| [141 | 3 | 0,1,12 | 2] |  |  |  |  |
| [142 | 3 | 0,1,12,13 | 2] |  |  |  |  |
| [143 | 3 | 6,7,18 | 2] |  |  |  |  |
| [144 | 3 | 6,7,18,19 | 2] |  |  |  |  |
| [145 | 3 | 2,3,14 | 2] |  |  |  |  |
| [146 | 3 | 2,3,14,15 | 2] |  |  |  |  |
| [147 | 3 | 8,9,20 | 2] |  |  |  |  |
| [148 | 3 | 8,9,20,21 | 2] |  |  |  |  |
| [149 | 3 | 4,5,16 | 2] |  |  |  |  |
| [150 | 3 | 4,5,16,17 | 2] |  |  |  |  |
| [151 | 3 | 10,11,22 | 2] |  |  |  |  |
| [152 | 3 | 10,11,22,23 | 2] |  |  |  |  |

------- end of the proposal --------

FL note: there is additional proposal to add the following rows for 1CW (Cat.3 with 2 symbols). MU capacity of supporting row 42-47 and row 100-105 are the same as supporting row 141-152. However, it seems row 141-152 seems to have lower UE complexity, because different TD-OCC is used for different UE for MU multiplexing.

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 129 | 1 | 0,1,12 | 2 |
| 130 | 1 | 0,1,12,13 | 2 |
| 131 | 1 | 6,7,18 | 2 |
| 132 | 1 | 6,7,18,19 | 2 |
| 133 | 2 | 0,1,12 | 2 |
| 134 | 2 | 0,1,12,13 | 2 |
| 135 | 2 | 6,7,18 | 2 |
| 136 | 2 | 6,7,18,19 | 2 |
| 137 | 2 | 2,3,14 | 2 |
| 138 | 2 | 2,3,14,15 | 2 |
| 139 | 2 | 8,9,20 | 2 |
| 140 | 2 | 8,9,20,21 | 2 |
| 141 | 3 | 0,1,12 | 2 |
| 142 | 3 | 0,1,12,13 | 2 |
| 143 | 3 | 6,7,18 | 2 |
| 144 | 3 | 6,7,18,19 | 2 |
| 145 | 3 | 2,3,14 | 2 |
| 146 | 3 | 2,3,14,15 | 2 |
| 147 | 3 | 8,9,20 | 2 |
| 148 | 3 | 8,9,20,21 | 2 |
| 149 | 3 | 4,5,16 | 2 |
| 150 | 3 | 4,5,16,17 | 2 |
| 151 | 3 | 10,11,22 | 2 |
| 152 | 3 | 10,11,22,23 | 2 |

For M-TRP case

FL Proposal 2.1.4B

- For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

  - All rows for Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - For one CW, add new row 128 in Table 7.3.1.2.2-4A-X.

Table 7.3.1.2.2-4A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 128 | 2 | 0,2,3 | 1 |

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Proposal 2.1.4A: Support. Proposal 2.1.4B: Support. |
| Google | Proposal 2.1.4A: Support. Proposal 2.1.4B: Support. |
| OPPO | Proposal 2.1.4A: 1. Agree that row 67-68 and row 78-80 are not needed. 2. Row 14-17 and Row 6-9 for two CWs have similar overhead and performance (FDM vs. TDM), so we think Row 14-17 are not needed. 3. Considering a UE with two CWs is not likely to be scheduled with MU-MIMO, the use case of Row 18-37 for two CWs is unclear to us. Proposal 2.1.4B: Support. |
| Nokia/NSB | Proposal 2.1.4A: 1. We need more reduction of the rows. . 2. We support the same table for 2CWs as in “maxLength=1” Proposal 2.1.4B: Support. (value should be less than 128, for not increasing DCI bit size) |
| CATT | Proposal 2.1.4A: Support in principle. We slightly prefer to keep rows for SU-MIMO (e.g., row 78-81) if the size of antenna port(s) field is not increased. Besides, we suggest to remove FL note, since Cat.3 port combinations with 2 symbols will increase the overhead of antenna port(s) field. Proposal 2.1.4B: Support. |
| Huawei, HiSilicon | FL Proposal 2.1.4A: Generally fine. Some rows need to be discussed. - Row 100~105 should be further discussed to facilitate more layer combinations. - For 2CW, considering it is used for SU scenario, row 10~13 is not needed. Furthermore, at least row 14~17 is also needed to facilitate supporting rank>4 with only 1 symbol, which can improve the efficiency of resource utilization and scheduling flexibility. FL Proposal 2.1.4B: Support. |
| Lenovo | Proposal 2.1.4A: Support. Proposal 2.1.4B: Support. |
| Intel | Looks fine. Probably need more discussion after maxLength=1 tables are finalized. |
| QC | Proposal 2.1.4A: Do not support the proposal. For single CW, we don’t support rows 9-10,20-23,42-47,67-68,78-81,100-105, based on the same reasons as mentioned for eType 1. - Rows 9-10, 20-23, 67-68,78-81: DMRS ports distributed in 2 CDM groups - Rows 42-47, 100-105: DMRS distributed in 2 TD-OCC For rows 129-152, we support them and we agree with FL’s notes (except there is typo “different CDM group is used” should be “different TD-OCC is used”). Actually, if we comparing rows 42-47, 100-105 with rows 141-152, they have same functionality, i.e., for rank 3-4 with 1 CW. But, rows 141-152 are better design than rows 42-47, 100-105 because putting ports in one TD-OCC code can reduce MU interference, which is aligned with Rel-15 principle. Also, it does not create MU detection issue for UE implementation. Therefore, we think we should remove rows 42-47, 100-105 and adopt rows 141-152. For 2CWs, we don’t support the entries in current proposal because they map layers of 1 CW to two CDM groups. We still prefer map layers of 1 CW to one CDM group. We think we can defer the decision on 2CW after we decide whether/how to confirm the WA for 2CWs type 1 1-symbol DMRS. Proposal 2.1.4B: Support. |
| ZTE | FL Proposal 2.1.4A: Support w/o the additional proposal. - Row 81 can be removed but rows 67-68 and 78-80 should be kept as our elaboration on Rel-18 eType2 + maxLength2 in section 2.1.3. - The additional proposal is not unreasonable as we explained for rows 26-30 and 57-60 of Rel-18 eType1 + maxLength2 in section 2.1.2. FL Proposal 2.1.4B: Support. |
| Ericsson | Proposal 2.1.4A: Support. On MU-MIMO restriction, we don’t prefer to add MU restriction on each row consisting of ports from more than one CDM group or more than one TD-OCC code, we’d like to understand if such restriction is essential for most of the UE implementations. Proposal 2.1.4B: Support. |
| Sharp | Proposal 2.1.4A: Support Proposal 2.1.4B: Support |
| Xiaomi | Proposal 2.1.4A: Fine Proposal 2.1.4A: Same concern. Whether (0,2,3) is needed for MTRP CJT needs more discussion. |
| Apple | FL Proposal 2.1.4A: We do not agree to include entries 67,68 and 78,79,80,81 as the entries are spread across multiple CDM groups with new ports. At least MU-MIMO restriction should be agreed if these entries are to be agreed. Also for port entries 9,10, 20,21,22,23, MU-MIMO restriction should be agreed. Otherwise, we can also discuss this table once max_length=1 is finalized FL Proposal 2.1.4B: Support |
| New H3C | Proposal 2.1.4A: Support Proposal 2.1.4B: Support |
| China Telecom | Proposal 2.1.4A: Support except the additional proposal. For the additional proposal, we think it should follow the same principle as that of proposal 2.1.2A. Proposal 2.1.4B: Support |
| CMCC | FL Proposal 2.1.4A: For 1 CW, we don’t agree to add MU restriction on rows consisting of ports from only 1 or 2 CDM groups. One of the motivations to increasing DMRS ports is for MU-MIMO, if adding MU restriction on each row consisting of ports from more than one TD-OCC, the performance of increasing DMRS ports is degraded. FL Proposal 2.1.4B: Support. |
| LGE | Proposal 2.1.4A: Support. Proposal 2.1.4B: Support. |
| FL | I added FFS to MU-MIMO restriction, because row 23 is now FFS. |
| FL | FL Proposal 2.1.4B is moved to [112bis-e-R18-MIMO-04] DMRS - EMAIL ENDORSEMENT 1. |
| QC3 | For 2CWs, we thank FL for updating the proposal by adding [] on certain entries. But we don’t support the entries in current proposal because they map layers of 1 CW to two CDM groups. We still prefer map layers of 1 CW to one CDM group. We think we can defer the decision on 2CW after we decide whether/how to confirm the WA for 2CWs type 1 1-symbol DMRS. |
| FL(v40) | For 2.1.2A, I put [ ] to all rows of 2 CWs. |

  - DCI size of antenna ports field for PDSCH/PUSCH

For the size of antenna ports field, since the number of rows in DMRS ports table is increased, it is inevitable to increase the size of DCI field for the antenna ports indication. Ericsson proposes to use RRC configuration to select the actual needed row indexes in the antenna ports table. Some companies propose to introduce new DCI field of “offset indicator” to assist DMRS port indication by the antenna ports field, while other companies think “offset indicator” is not suitable to indicate DMRS ports of Cat.3 and just increasing 1-bit of antenna ports field is better.

Let’s discuss the following proposals for PDSCH and PUSCH.

FL Proposal 2.2A (for PDSCH)

- For Rel.18 eType1/eType2 DMRS ports with maxLength=1/2 for PDSCH, if Rel.18 eType1/eType2 DMRS ports is configured by RRC, the size of DCI field for antenna ports indication in DCI format 1_1/1_2 is down-selected from the following:

  - Alt.1: The DCI size of DMRS port indication is increased by 1-bit from Rel.17.

    - FFS: Whether existing antenna ports field is increased by 1-bit or new 1-bit DCI field is added.

  - Alt.2: The DCI size of DMRS port indication is increased by M (M = {0, 1}) bit, and M is configured by RRC.

    - RRC signaling can configure the actual rows in the antenna ports table, which can be indicated by the DCI of antenna port indication.

    - Note: antenna ports field can be increased by M-bit, or new M-bit DCI field (e.g. [DMRS port(s) offset indicator]) can be introduced to assist DMRS port indication by the existing antenna ports field.

FL Proposal 2.2B (for PUSCH)

- For Rel.18 eType1/eType2 DMRS ports with maxLength=1/2 for PUSCH, if Rel.18 eType1/eType2 DMRS ports is configured by RRC, the size of DCI field for antenna ports indication in DCI format 0_1/0_2 is down-selected from the following:

  - Alt.1: The DCI size of DMRS port indication is increased by 1-bit from Rel.17.

    - FFS: Whether existing antenna ports field is increased by 1-bit or new 1-bit DCI field is added.

  - Alt.2: The DCI size of DMRS port indication is increased by M (M = {0, 1}) bit, and M is configured by RRC.

    - RRC signaling can configure the actual rows in the antenna ports table, which can be indicated by the DCI of antenna port indication.

    - Note: antenna ports field can be increased by M-bit, or new M-bit DCI field (e.g. [DMRS port(s) offset indicator]) can be introduced to assist DMRS port indication by the existing antenna ports field.

Summary of companies’ inputs for PDSCH (in RAN1#112)

|  | Support/fine | Concern |
| --- | --- | --- |
| Alt.1 | Fraunhofer IIS/HHI (1st pref.), NEC, Intel, CMCC, Apple (no new field), Docomo, ZTE, CATT, Lenovo, Sharp, Huawei/HiSilicon, Nokia/NSB, Futurewei, Samsung, QC, MTK, China Telecom, Spreadtrum, vivo, OPPO, New H3C |  |
| Alt.2 | Google, Docomo, Ericsson, OPPO, ZTE, CMCC, Xiaomi, Fraunhofer IIS/HHI (2nd pref.), Nokia/NSB, LGE, China Telecom (can live) | QC (UE complexity), OPPO, Nokia/NSB (support only M=0, but not alt 2) |

| Company | Comment |
| --- | --- |
| Docomo | FL Proposal 2.2A: Since the size of antenna ports table is larger than that in Rel.17, it is natural to increase the DCI size of antenna ports field. Hence, we support Alt.1, and we are also file with Alt.2. FL Proposal 2.2B: We prefer the same rule is applied to both PDSCH and PUSCH. |
| Google | Support Alt2 for both proposals. We can consider negative value for M as well for further overhead reduction. This could be something similar to codebook subset restriction. |
| Futurewei | FL Proposal 2.2A: We prefer Alt. 1 which is simple and straightforward to accommodate a larger antenna ports table. FL Proposal 2.2B: We prefer Alt. 1, same reason as for FL Proposal 2.2A. |
| OPPO | FL Proposal 2.2A: Alt.1. FL Proposal 2.2B: Alt.1. |
| Nokia/NSB | Proposal 2.2A: either Alt 1 or Alt2 (M=0 only, RRC configuration is mandatory) Proposal 2.2B Alt.1 |
| CATT | FL Proposal 2.2A: Support and Alt.1 is preferred. FL Proposal 2.2B: Support and Alt.1 is preferred. |
| Huawei, HiSilicon | FL Proposal 2.1.4A: Prefer Alt.1. FL Proposal 2.1.4B: Prefer Alt.1. |
| Lenovo | FL Proposal 2.2A and FL Proposal 2.2B: Support Alt.1. For Alt.2, we think it is complicated for defining two cases for M=0 and 1. Also, we have the similar view with FL to have the same rule for both PDSCH and PUSCH. |
| Intel | Proposal 2.1.4A/B: Prefer Alt-1. Can also live with Alt-2 but without the note. We do not support the idea of complicating DM-RS antenna port indication by further introducing port offset indication. |
| QC | FL Proposal 2.2A: support Alt 1. We don’t support Alt 2. We don’t think adding 1 bit in DCI will be showstopper for Rel-18 DMRS. Alt 2 is unnecessarily complicated. FL Proposal 2.2B: support Alt 1. We don’t support Alt 2. We don’t think adding 1 bit in DCI will be showstopper for Rel-18 DMRS. Alt 2 is unnecessarily complicated. |
| MediaTek | FL Proposal 2.2A: We believe Alt 1 is much simpler and cleaner solution and hence our preference is Alt 1. FL Proposal 2.2A: We believe Alt 1 is much simpler and cleaner solution and hence our preference is Alt 1. |
| ZTE | To our understanding, it is natural to increase DCI size in terms of DMRS indication due to the increased DMRS ports in Rel-18, DMRS indication field should be increased as it is due to the enhancement in Rel-18. To guarantee the completeness and flexibility of gNB scheduling is the most significant thing rather than DCI size saving. We think this issue is invalid, but if deemed necessary, our standpoint is: FL Proposal 2.2A (for PDSCH): Support Alt. 1 by following updates. - Conclusion: Alt.1: The DCI size of DMRS port indication is increased in Rel-18 by 1-bit from Rel.17. FL Proposal 2.2A (for PUSCH): Support Alt. 1 by following updates. - Conclusion: Alt.1: The DCI size of DMRS port indication is increased in Rel-18 by 1-bit from Rel.17. |
| Ericsson | Support Alt2 for both proposals. |
| Sharp | FL Proposal 2.2A: Support Alt 1. FL Proposal 2.2B: Support Alt 1. |
| Xiaomi | FL Proposal 2.2A and FL Proposal 2.2B: Support Alt.1. Increase the size of antenna ports field DCI is a more appropriate way to indicate R18 DMRS port(s). |
| Apple | FL Proposal 2.2A: Support Alt 1 as it is much simpler FL Proposal 2.2A: Support Alt 1 as it is much simpler |
| New H3C | FL Proposal 2.2A: Alt.1. FL Proposal 2.2B: Alt.1. |
| China Telecom | Support Alt1 for both proposals since they are the much more simpler solutions. |
| Samsung | Support Alt1 for both proposals. It is natural to increase 1-bit if more entries are supported. |
| Spreadtrum | FL Proposal 2.2A: We prefer Alt.1 to avoid complicated design. Increasing 1 bit should be enough. FL Proposal 2.2B: We prefer the same solution as for PDSCH, i.e. Alt1. |
| CMCC | Support Alt1 for both PDSCH and PUSCH. |
| LGE | FL Proposal 2.2A: We slightly prefer Alt.2 and we are also fine with Alt.1 if it is the majority view. FL Proposal 2.2B: We prefer the same rule is applied to both PDSCH and PUSCH. |
| vivo | FL Proposal 2.2A: Support Alt.1. There is no doubt that we need an extended table for R18 DMRS. A direct way is to indicate using legacy table or R18 table by gNB. FL Proposal 2.2B: Support Alt.1. |

  - Antenna ports field for PUSCH (rank 1-4)

2.3.1 eType1, maxLength1

We made the following agreement in RAN1#112. Remaining issue for eType1 maxLentgh1 is whether to support the rows with [].

Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-8-X, Table 7.3.1.1.2-9-X, Table 7.3.1.1.2-10-X, and Table 7.3.1.1.2-11-X are supported. - FFS: Whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not. Table 7.3.1.1.2-8-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=1, rank = 1 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0. 1; 1; 1. 2; 2; 0. 3; 2; 1. 4; 2; 2. 5; 2; 3. 6; 1; 8. 7; 1; 9. 8; 2; 8. 9; 2; 9. 10; 2; 10. 11; 2; 11. 12-15; Reserved; Reserved. Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0,1. 1; 2; 0,1. 2; 2; 2,3. 3; 2; 0,2. 4; 1; 8,9. 5; 2; 8,9. 6; 2; 10,11. [7]; [2]; [8,10]. 8-15; Reserved; Reserved. Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-2. [1]; [2]; [8-10]. 2; 1; 0,1,8. 3; 2; 0,1,8. 4; 2; 2,3,10. 5-15; Reserved; Reserved. Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-3. [1]; [2]; [8-11]. 2; 1; 0,1,8,9. 3; 2; 0,1,8,9. 4; 2; 2,3,10,11. 5-15; Reserved; Reserved.

Multiple companies propose to remove the rows with [ ], considering these rows would be not supported for PDSCH.

FL Proposal 2.3.1A

- For RAN1#112 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH,

  - Do not support row 7 for rank2, row1 for rank3, row 1 for rank4.

FL Proposal 2.3.1B (Opposite proposal)

- For RAN1#112 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH,

  - Support row 7 for rank2, row1 for rank3, row 1 for rank4.

| Company | Comment |
| --- | --- |
| Docomo | Support. For row 1 of rank 3 and row 1 of rank 4, we don’t see any use-case, because DMRS ports of Cat.3 is more useful in terms of DMRS overhead. For row 7 of rank 2, although MU MIMO restriction is not applied to PUSCH, we don’t think row 7 of rank 2 is useful. |
| Google | OK |
| Ericsson | Do not support FL proposal. These rows don’t use legacy ports are very useful for increasing the MU-MIMO capacity in uplink. Network can schedule Rel-18 UE using new ports and keep the legacy ports to schedule Rel-15 UE. |
| OPPO | Support. If the use case is MU-MIMO, the ports within the same CDM group should be used. |
| Nokia/NSB | Support the proposal. But, we can live with the rows with brace. |
| CATT | Do not support the proposal. The same port combination(s) as that for PDSCH is used. |
| Huawei, HiSilicon | Agree with Ericsson’s assessment. |
| Lenovo | We prefer to align the DMRS design for PDSCH and PUSCH. So we can live up with the proposal. |
| Intel | We think the rows may be beneficial for increasing UL MU-MIMO capacity |
| QC | We don’t have strong opinion on this proposal. If gNB vendors don’t see issues to implement receiver to support those rows, we can support keeping them. |
| MediaTek | Fine |
| ZTE | Do not support. We share similar view to E/// that rows with [] are needed for MU-MIMO scenario, which is in line with the motivation as stated in WID. |
| Sharp | We are fine with the proposal. |
| Xiaomi | Fine. |
| Apple | Fine |
| New H3C | OK |
| China Telecom | Not support. The rows with brackets are needed. |
| LGE | Support. We think the rows may be beneficial for increasing UL MU-MIMO capacity within the same CDM group. |
| vivo | Support FL Proposal 2.3.1A. |
| ZTE2 | Support FL Proposal 2.3.1B. |
| FL(v40) | Multiple companies suggested to support the rows. Since there is no MU-MIMO restriction for PUSCH, and there should be no technical concern at UE vendors to support the new rows (because receiver of PUSCH is gNB), and multiple gNB vendors/operator think the new row are beneficial, my suggestion is to consider FL Proposal 2.3.1B. |

2.3.2 eType1, maxLength2 (discuss later)

Based on reviewing tdocs, multiple companies propose to reuse the same DMRS ports combinations of PDSCH to PUSCH with rank 1-4. Following tables captures all DMRS port combinations of PDSCH (including rows with [ ]).

FL: Since DMRS ports combinations for PDSCH is not decided yet, we will discuss this later.

Table 7.3.1.1.2-12-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType2, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 2 | 0 | 2 |
| 7 | 2 | 1 | 2 |
| 8 | 2 | 2 | 2 |
| 9 | 2 | 3 | 2 |
| 10 | 2 | 4 | 2 |
| 11 | 2 | 5 | 2 |
| 12 | 2 | 6 | 2 |
| 13 | 2 | 7 | 2 |
| 14 | 1 | 8 | 1 |
| 15 | 1 | 9 | 1 |
| 16 | 2 | 8 | 1 |
| 17 | 2 | 9 | 1 |
| 18 | 2 | 10 | 1 |
| 19 | 2 | 11 | 1 |
| 20 | 2 | 8 | 2 |
| 21 | 2 | 9 | 2 |
| 22 | 2 | 10 | 2 |
| 23 | 2 | 11 | 2 |
| 24 | 2 | 12 | 2 |
| 25 | 2 | 13 | 2 |
| 26 | 2 | 14 | 2 |
| 27 | 2 | 15 | 2 |
| 28-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10 | 1 | 8,9 | 1 |
| 11 | 2 | 8,9 | 1 |
| 12 | 2 | 10,11 | 1 |
| 13 | [2 | 8,10 | 1] |
| 14 | 2 | 8,9 | 2 |
| 15 | 2 | 10,11 | 2 |
| 16 | 2 | 12,13 | 2 |
| 17 | 2 | 14,15 | 2 |
| 18 | 2 | 8,12 | 2 |
| 19 | 2 | 10,14 | 2 |
| 20-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 2 | 8-10 | 1 |
| 4 | 2 | 8,9,12 | 2 |
| 5 | 2 | 10,11,14 | 2 |
| 6 | 1 | 0,1,8 | 1 |
| 7 | 2 | 0,1,8 | 1 |
| 8 | 2 | 2,3,10 | 1 |
| 9-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2] |
| 4 | 2 | 8-11 | 1 |
| 5 | 2 | 8,9,12,13 | 2 |
| 6 | 2 | 10,11,14,15 | 2 |
| 7 | 2 | 8,10,12,14 | 2 |
| 8 | 1 | 0,1,8,9 | 1 |
| 9 | 2 | 0,1,8,9 | 1 |
| 10 | 2 | 2,3,10,11 | 1 |
| 11-31 | Reserved | Reserved | Reserved |

2.3.3 eType2, maxLength1 (discuss later)

Based on reviewing tdocs, multiple companies propose to reuse the same DMRS ports combinations of PDSCH to PUSCH with rank 1-4. Following tables captures all DMRS port combinations of PDSCH (including rows with [ ]).

FL: Since DMRS ports combinations for PDSCH is not decided yet, we will discuss this later.

Table 7.3.1.1.2-16-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType2, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 3 | 0 |
| 7 | 3 | 1 |
| 8 | 3 | 2 |
| 9 | 3 | 3 |
| 10 | 3 | 4 |
| 11 | 3 | 5 |
| 12 | 1 | 12 |
| 13 | 1 | 13 |
| 14 | 2 | 12 |
| 15 | 2 | 13 |
| 16 | 2 | 14 |
| 17 | 2 | 15 |
| 18 | 3 | 12 |
| 19 | 3 | 13 |
| 20 | 3 | 14 |
| 21 | 3 | 15 |
| 22 | 3 | 16 |
| 23 | 3 | 17 |
| 24-31 | Reserved | Reserved |

Table 7.3.1.1.2-17-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 3 | 0,1 |
| 4 | 3 | 2,3 |
| 5 | 3 | 4,5 |
| 6 | 2 | 0,2 |
| 7 | 1 | 12,13 |
| 8 | 2 | 12,13 |
| 9 | 2 | 14,15 |
| 10 | 3 | 12,13 |
| 11 | 3 | 14,15 |
| 12 | 3 | 16,17 |
| 13 | 2 | 12,14 |
| 14-31 | Reserved | Reserved |

Table 7.3.1.1.2-18-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3 | [2 | 12-14] |
| 4 | [3 | 12-14] |
| 5 | [3 | 15-17] |
| 6 | 1 | 0,1,12 |
| 7 | 2 | 0,1,12 |
| 8 | 2 | 2,3,14 |
| 9 | 3 | 0,1,12 |
| 10 | 3 | 2,3,14 |
| 11 | 3 | 4,5,16 |
| 12-31 | Reserved | Reserved |

Table 7.3.1.1.2-19-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 3 | 0-3 |
| 2 | [2 | 12-15] |
| 3 | [3 | 12-15] |
| 4 | 1 | 0,1,12,13 |
| 5 | 2 | 0,1,12,13 |
| 6 | 2 | 2,3,14,15 |
| 7 | 3 | 0,1,12,13 |
| 8 | 3 | 2,3,14,15 |
| 9 | 3 | 4,5,16,17 |
| 10-31 | Reserved | Reserved |

2.3.4 eType2, maxLength2 (discuss later)

Based on reviewing tdocs, multiple companies propose to reuse the same DMRS ports combinations of PDSCH to PUSCH with rank 1-4. Following tables captures all DMRS port combinations of PDSCH (including rows with [ ]).

FL: Since DMRS ports combinations for PDSCH is not decided yet, we will discuss this later.

Table 7.3.1.1.2-20-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType2, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 3 | 0 | 1 |
| 7 | 3 | 1 | 1 |
| 8 | 3 | 2 | 1 |
| 9 | 3 | 3 | 1 |
| 10 | 3 | 4 | 1 |
| 11 | 3 | 5 | 1 |
| 12 | 3 | 0 | 2 |
| 13 | 3 | 1 | 2 |
| 14 | 3 | 2 | 2 |
| 15 | 3 | 3 | 2 |
| 16 | 3 | 4 | 2 |
| 17 | 3 | 5 | 2 |
| 18 | 3 | 6 | 2 |
| 19 | 3 | 7 | 2 |
| 20 | 3 | 8 | 2 |
| 21 | 3 | 9 | 2 |
| 22 | 3 | 10 | 2 |
| 23 | 3 | 11 | 2 |
| 24 | 1 | 0 | 2 |
| 25 | 1 | 1 | 2 |
| 26 | 1 | 6 | 2 |
| 27 | 1 | 7 | 2 |
| 28 | 1 | 12 | 1 |
| 29 | 1 | 13 | 1 |
| 30 | 2 | 12 | 1 |
| 31 | 2 | 13 | 1 |
| 32 | 2 | 14 | 1 |
| 33 | 2 | 15 | 1 |
| 34 | 3 | 12 | 1 |
| 35 | 3 | 13 | 1 |
| 36 | 3 | 14 | 1 |
| 37 | 3 | 15 | 1 |
| 38 | 3 | 16 | 1 |
| 39 | 3 | 17 | 1 |
| 40 | 3 | 12 | 2 |
| 41 | 3 | 13 | 2 |
| 42 | 3 | 14 | 2 |
| 43 | 3 | 15 | 2 |
| 44 | 3 | 16 | 2 |
| 45 | 3 | 17 | 2 |
| 46 | 3 | 18 | 2 |
| 47 | 3 | 19 | 2 |
| 48 | 3 | 20 | 2 |
| 49 | 3 | 21 | 2 |
| 50 | 3 | 22 | 2 |
| 51 | 3 | 24 | 2 |
| 52 | 1 | 12 | 2 |
| 53 | 1 | 13 | 2 |
| 54 | 1 | 18 | 2 |
| 55 | 1 | 19 | 2 |
| 56-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 3 | 0,1 | 1 |
| 4 | 3 | 2,3 | 1 |
| 5 | 3 | 4,5 | 1 |
| 6 | 2 | 0,2 | 1 |
| 7 | 3 | 0,1 | 2 |
| 8 | 3 | 2,3 | 2 |
| 9 | 3 | 4,5 | 2 |
| 10 | 3 | 6,7 | 2 |
| 11 | 3 | 8,9 | 2 |
| 12 | 3 | 10,11 | 2 |
| 13 | 1 | 0,1 | 2 |
| 14 | 1 | 6,7 | 2 |
| 15 | 2 | 0,1 | 2 |
| 16 | 2 | 2,3 | 2 |
| 17 | 2 | 6,7 | 2 |
| 18 | 2 | 8,9 | 2 |
| 19 | 1 | 12,13 | 1 |
| 20 | 2 | 12,13 | 1 |
| 21 | 2 | 14,15 | 1 |
| 22 | 3 | 12,13 | 1 |
| 23 | 3 | 14,15 | 1 |
| 24 | 3 | 16,17 | 1 |
| 25 | 2 | 12,14 | 1 |
| 26 | 3 | 12,13 | 2 |
| 27 | 3 | 14,15 | 2 |
| 28 | 3 | 16,17 | 2 |
| 29 | 3 | 18,19 | 2 |
| 30 | 3 | 20,21 | 2 |
| 31 | 3 | 22,23 | 2 |
| 32 | 1 | 12,13 | 2 |
| 33 | 1 | 18,19 | 2 |
| 34 | 2 | 12,13 | 2 |
| 35 | 2 | 14,15 | 2 |
| 36 | 2 | 18,19 | 2 |
| 37 | 2 | 20,21 | 2 |
| 38-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6 | 2 | 12-14 | 1 |
| 7 | 3 | 12-14 | 1 |
| 8 | 3 | 15-17 | 1 |
| 9 | 3 | 12,13,18 | 2 |
| 10 | 3 | 14,15,20 | 2 |
| 11 | 3 | 16,17,22 | 2 |
| 12 | 1 | 0,1,12 | 1 |
| 13 | 2 | 0,1,12 | 1 |
| 14 | 2 | 2,3,14 | 1 |
| 15 | 3 | 0,1,12 | 1 |
| 16 | 3 | 2,3,14 | 1 |
| 17 | 3 | 4,5,16 | 1 |
| 18-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 3 | 0-3 | 1 |
| 2 | 3 | 0,1,6,7 | 2 |
| 3 | 3 | 2,3,8,9 | 2 |
| 4 | 3 | 4,5,10,11 | 2 |
| 5 | 2 | 12-15 | 1 |
| 6 | 3 | 12-15 | 1 |
| 7 | 3 | 12,13,18,19 | 2 |
| 8 | 3 | 14,15,20,21 | 2 |
| 9 | 3 | 16,17,22,23 | 2 |
| 10 | 1 | 0,1,12,13 | 1 |
| 11 | 2 | 0,1,12,13 | 1 |
| 12 | 2 | 2,3,14,15 | 1 |
| 13 | 3 | 0,1,12,13 | 1 |
| 14 | 3 | 2,3,14,15 | 1 |
| 15 | 3 | 4,5,16,17 | 1 |
| 16-63 | Reserved | Reserved | Reserved |

  - Signaling of Rel.18 DMRS ports

In RAN1#112, we made the following conclusion.

Conclusion Dynamic switching between R15 DMRS port and R18 DMRS port by a scheduling DCI is not supported in Rel-18

In this meeting, Huawei/HiSilicon[3], ZTE/China Telcom[4], Samsung[18] propose MAC CE based switching between Rel.15 DMRS ports and Rel.18 DMRS ports. The benefit is it can avoid RRC re-configuration, and it enables faster switching. Nokia/NSB[15] proposes DMRS type configuration per search space so that DCI-based dynamic switching between Rel.15 DMRS ports and Rel.18 for PDSCH is allowed by selecting search space of the scheduling DCI.

These proposals are not explicitly precluded by the conclusion, and it is good to discuss.

FL Proposal 2.4A (MAC CE based switching)

- Support MAC CE based switching between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH

FL Proposal 2.4B (Search-space based switching)

- Support configuration of Rel-18 DMRS per search space to indicate what DMRS type is supported for PDSCH/PUSCH scheduling in the search space.

Google [16] proposes dynamic indication of co-scheduled UE in the same CDM group to handle the similar issue.

FL Proposal 2.4C (dynamic indication of co-scheduled UE)

- Support dynamic indication of information of co-scheduled UE in the indicated CDM group(s) to facilitate the FD-OCC length selection in UE side

  - The information is whether new port index(es) (eType 1: p=8~15, eType 2: p=12~23) is/are used for co-scheduled UE in the same indicated CDM group for the scheduled UE.

Proposal 2.4D: (Proposed by ZTE)

Rel-18 PDSCH/PUSCH DMRS ports can be dynamically generated with length 2/4 FD-OCC to guarantee the performance especially in large delay spread scenario.

- Note: It is common understanding in RAN1 that dynamic switching between length 2/4 FD-OCC can be performed dynamically by UE implementation.

| Company |  | Comment |
| --- | --- | --- |
| Docomo |  | FL proposal 2.4A: We are fine. FL proposal 2.4B: In Rel.15-17, DMRS-DownlinkConfig is configured in PDSCH-Config and DMRS-UplinkConfig is configured in PUSCH-Config. PDSCH DMRS configuration per Search Space is new concept and its impact to TS38.331 is not small. Even if it enables indication of the dynamic switching of FD-OCC, from gNB perspective, we cannot ensure UE switches FD-OCC length for de-spreading (because not testable), and the benefit is not clear. Hence, we don’t support FL proposal 2.4B. FL proposal 2.4C: Support. |
| Google |  | In our view, we do not need to dynamically switch between R15 DMRS and R18 DMRS. But the NW only needs to provide some information for the UE to determine the FD-OCC despreading length. We suggest the following proposal: Support to indicate the maximum number of co-scheduled DMRS ports per CDM group in the scheduling DCI when R18 DL DMRS is configured. |
| Futurewei |  | FL Proposal 2.4A: Support. FL Proposal 2.4B: It is not clear to us the benefit of this proposal. FL Proposal 2.4C: We don’t see this proposal is necessary. |
| OPPO |  | FL proposal 2.4A: Not support. We think RRC based switching is sufficient. FL proposal 2.4B: Not support. It would increase the UE complexity to dynamically switch the OCC length. FL proposal 2.4C: As discussed in previous meetings, we don’t think it is needed. |
| Nokia/NSB |  | FL proposal 2.4A: Not support. MAC-CE based has little flexibility. FL proposal 2.4B: Support. Rel-18 DMRS configuration should be additional indiation to Rel-15 DMRS configuration. So, just simple indication of whether to use Rel-18 DMRS or not is enough. Rel-18 DMRS is more useful for low data rate IoT service. FL proposal 2.4C: The proposal is including dynamic switching of FD-OCC2/4. Overhead is almost similar, but less flexible. Dynamic switching is simpler and better. |
| CATT | Support Proposal 2.4A. |  |
| Huawei, HiSilicon |  | FL Proposal 2.4A: Support. FL Proposal 2.4B: Not support. The benefit is not clear. FL Proposal 2.4C: Fine with the principle. Details can be further discussed. |
| Lenovo |  | FL proposal 2.4A: Support. From our previous simulation results, Rel-15 DMRS ports can provide better performance than Rel-18 DMRS ports; and Rel-18 DMRS ports may occupy less resources when the transmission rank is higher since only one CDM group may be enough. We think faster DMRS port type switching than RRC is beneficial and should be supported. FL Proposal 2.4B: Not support since it will increase UE realization complexity. FL Proposal 2.4C: Not support. |
| Intel |  | Proposal 2.4A: Do not support dynamic switching of Rel-15 and Rel-18 ports. This issue was discussed at length and concluded in the last meeting. No need to further revisit again. Proposal 2.4B: Do not support this proposal. This has major specification impact without major benefits. As we commented there is no fundamental performance issue with length-4 OCC. Proposal 2.4C: The issue is not simply about new ports. Same issue exists in legacy DM-RS where the UE does not know if there are other co-scheduled UEs within a CDM group. In Rel-15, this indication was not agreed. From our perspective, if we want to make this useful, we should indicate whether there are co-scheduled ports within the same CDM group which can potentially help UEs with appropriate receivers. |
| QC |  | We don’t see much need to introduce 1 bit signaling to switch Rel-15 and Rel-18 DMRS, because of the same reasons we gave in previous meetings. - There is no much performance difference between Rel-15 and Rel-18 DMRS (see simulation), if UE implement channel estimation properly. Someone not implementing channel estimation properly does not justify the need to introduce this signaling. - Dynamic switching between Rel-15 and Rel-18 DMRS would significant increase UE implementation complexity. We are open to discuss MAC-CE. However, we still don’t support this proposal, because it is a bad design, due to the following reasons. - If MAC-CE based switch is introduced, it is clearly a waste of a whole MAC-CE to deliver just 1-bit (to switch between Rel-15 and Rel-18), consider the header of MAC-CE. Since this switching is essentially some MU scheduling information (to tell a Rel-18 UE with Rel-15 DMRS ports whether other MU exist or not), it is natural to include more MU scheduling information in the MAC-CE, such as the following Modified FL Proposal 2.4A. By the way, there is a RAN4 WI (RP-222300) on NW assisted advance UE canceling inter-user interference for MU-MIMO. It can be seen that the following list of MU information in Modified FL Proposal 2.4A are proposed by many companies in RAN4 as well. - Even if we just introduce 1 bit in MAC-CE, using this bit to indicate switch between Rel-15 and Rel-18 DMRS is a bad design. A better design is using this bit indicate whether there is co-scheduled MU with this target UE or not. The following are the reasons. - (The original proposal) Indicating switch between Rel-15 and Rel-18 DMRS only help channel estimation of target UE 1 in one scenario (Scenario 1). It does not work for scenario 2 and 3. While (the modified proposal) using 1 bit to indicate MU exist or not can help channel estimation in all three scenarios. - The modified proposal has more benefits. Functionalities 2 and 3 are not even achievable by the original proposal. In summary, we still object include only 1-bit to switch Rel-15 and Rel-18 in MAC CE. We are open to discuss the following Modified FL Proposal 2.4A. By the way, for PUSCH, we don’t see the need to introduce this 1-bit switch at all, even with MAC-CE based switch. As the switch is providing MU information to help receiver channel estimation, which is not needed for PUSCH. Modified FL Proposal 2.4A Support using MAC CE to indicate at least the following MU scheduling related information to a target UE for PDSCH. based switching between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH/PUSCH - 1 bit to indicate if co-scheduled UE exist or not. - 1 bit to indicate whether PRG of co-scheduled UEs (if exist) are aligned with target UE. - 1 bit to indicate whether PDSCH staring and symbol of co-scheduled UEs (if exist) are aligned with target UE. - 1 bit to indicate whether DMRS sequences of co-scheduled UEs (if exist) are aligned with the target UE. - DMRS to PDSCH power ratio of co-scheduled UEs, if exist. FFS number of bits. - Supported Max QAM (i.e., 64QAM, 256QAM, or 1024QAM) of co-scheduled UEs, if exist. FFS number of bits. FFS: including other MU scheduling information in the MAC-CE. |
| MediaTek |  | Proposal 2.4A: Not support. This was discussed earlier and decided RRC switching is sufficient. FL Proposal 2.4B: Not Support. This introduced UE complexity. Any DCI/PDCCH based DRMS type selection should be excluded based on the previous discussions, this proposal is basically introducing the same concept but with different mechanism. FL proposal 2.4C: Fine to discuss further on the details. |
| ZTE |  | Proposal 2.4A: Support. In general, this switching is to compensate the performance loss issue caused by length 4 FD-OCC in large delay spread scenario as captured in the WA in RAN1#110 meeting. Working Assumption To increase the number of DMRS ports for PDSCH/PUSCH, support at least Opt.1 (introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6)). - FFS: FD-OCC length for Rel.18 DMRS type 1 and type 2. - FFS: Whether it is needed to handle potential performance issues of Opt 1. For example, study if there is performance loss in case of large delay spread scenario. If needed, how (e.g. additionally support other options).. Basically, we do believe indication of the switching between length 2/4 FD-OCC is very essential for both UE side and gNB side. - For UE side, Rel-18 DMRS ports will be more sensitive to delay spread variation than the Rel-15 DMRS ports due to the sparser allocation of length 4 FD-OCC basis in frequency domain. Besides, companies (at least including vivo, Ericssion, Huawei, Lenovo and ZTE) provided simulation results in the previous meeting have already proved that performance loss is indeed existing for length 4 FD-OCC when compared with length 2 FD-OCC large delay spread scenario (DS = 1000ns). The only reason used by opponents is that UE can be implemented based on the special but also fancy assumption of transmitter in gNB side (e.g., power scaling of path/cluster with larger delays by precoder, in R1-2209970 from QC in RAN1#110b-e). Apparently, we do believe this assumption is over-demanded and also unfair to gNB implementation. - For gNB side, this switching is very essential for optimizing system performance via guaranteeing transmission scheduling in SU-MIMO and MU-MIMO scenarios as we elaborated in in our tdoc (R1-2302419). In particular, if some companies insist the switching can be performed dynamically based on their UE implementation with the “advanced” processing, it should allow that gNB can generate Rel-18 DMRS ports (at least for Cat. 1) with length 2/4 FD-OCC in the same sense. This point should be common understanding to RAN1 and also be captured in Rel-18. We proposed: Proposal 2.4D: Rel-18 PDSCH/PUSCH DMRS ports can be dynamically generated with length 2/4 FD-OCC to guarantee the performance especially in large delay spread scenario. - Note: It is common understanding in RAN1 that dynamic switching between length 2/4 FD-OCC can be performed dynamically by UE implementation. Proposal 2.4B: Do not support. We share similar view with companies, it will lead to UE complexity of SS monitoring in addition. Proposal 2.4C: Fine to further study, but we do believe it is not irrelevant to the selection of FD-OCC length. More precisely, it is to indicate the interference from co-scheduled UE with different FD-OCC length, whether this kind of scheduling can be supported should be discussed at first. On the other hand, this issue is the very similar to co-existence between Rel-15 and Rel-18 DMRS in MU-MIMO, which should be discussed in case 3) in section 2.5 instead. |
| Ericsson |  | Proposal 2.4A: Fine with this proposal. Proposal 2.4B: Fine with the proposal. Proposal 2.4C: Fine with the proposal. |
| Sharp |  | FL Proposal 2.4A: Support. FL Proposal 2.4B: We are open to discuss. FL Proposal 2.4C: We are fine with the proposal. |
| Xiaomi |  | Proposal 2.4A: Not support. Our first choice is RRC based switching. Proposal 2.4B: Not support. Proposal 2.4C: Ok to further discuss. But we don’t get the intention of proposal 2.4C. |
| Apple |  | FL Proposal 2.4A: We do Not support and as discussed earlier, RRC switching is sufficient. FL Proposal 2.4B: We do not Support. Share similar view as QC and Mediatek that this introduced UE complexity. FL proposal 2.4C: Fine to discuss further |
| New H3C |  | OK for proposal 2.4A |
| China Telecom |  | Proposal 2.4A: Support the proposal. From the simulation before, we think most of us agree that the FD-OCC 4 will bring an extra performance loss. The reason why indication of dynamic switching with DCI was not supported in the last meeting is that some companies thought that the capability can be supported by the UE according to the implement. But the question is, we don’t think all the UEs will have the capability to avoid such performance degradation by themselves. If so, we think Proposal 2.4D by ZTE should also be supported to ensure the performance. Even the problem can be solved by UEs, the increase of complexity can be even larger than that brought by the indication. And if the MAC CE is not flexible, why RRC based switching can be a good solution? And it is interesting that why adding 1 bit for DMRS switching is a bad design and unnecessary, but and more bits for indicating the information about UE co-existence is needed. Since DCI based solution is precluded in the last meeting, we think the best solution currently is to support the MAC CE based switching solution. Proposal 2.4B: Not support. Proposal 2.4C: We are fine to further discuss, but we don’t understand the motivation of discussing it here, we think the proposal is actually about the co-existence of Rel-15 and Rel-18 DMRS.. |
| Samsung |  | Proposal 2.4A: We have similar view with ZTE and China Telecom. Not all UEs cannot have a capability freely switching between length-2/4 FD-OCC based on UE’s own decision. If a UE does not have such implementation and a capability, then as mentioned by many companies, the channel estimation performance would be degraded. So, for such UEs, such switching indication at least via MAC-CE would be beneficial. Proposal 2.4B: Do not support. Proposal 2.4C: Similar view with Nokia and ZTE, this issue is not exactly related to switching between FD-OCC length. |
| Fraunhofer |  | FL proposal 2.4A: No support. We also think that RRC-based switching is sufficient. FL proposal 2.4B: No support. FL proposal 2.4C: We don’t think it is needed. |
| Spreadtrum |  | Proposal 2.4A: We prefer RRC based switching. Proposal 2.4B: This feature will change the way of DMRS type determination of current spec. However, we think it’s not worthy since the benefit is not clear. Proposal 2.4C: If our memory is correct, this proposal was originally brought for MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports. We are OK to further discuss either in this section or in section 2.5. |
| CMCC |  | FL proposal 2.4A: Support. Length-4 FD-OCC is more sensitive to the frequency selectivity fading, it is better to use length-2 FD-OCC for channel estimation under large delay spread scenario. While for higher layers MU-MIMO, length-4 FD-OCC is needed. So, MAC CE based switching of length 2 and length 4 FD-OCC is benefit for SU and MU-MIMO switching. For MU-MIMO scheduling, if the layers of MU-MIMO is less than 8 or 12 layers, then length-2 FD-OCC can be used with better performance. While when network schedules higher layers MU-MIMO, then length-4 FD-OCC is needed. So, MAC CE based switching of length 2 and length 4 FD-OCC is benefit for MU-MIMO scheduling. FL Proposal 2.4B: Not support. Configuration of DMRS type per search space will reduce the flexibility of search space. FL Proposal 2.4C: Fine with the proposal. |
| LGE |  | Agree with Google, we do not need to dynamically switch between R15 DMRS and R18 DMRS because semi-static switching is sufficient. |
| vivo |  | FL proposal 2.4A: No support, and RRC is fine. Since there is no consensus to support dynamic switching, we think RRC-based switching is fine. If MAC CE is used, which table is used and which FD-OCC length is assumed would be more complex than direct RRC configuration. FL proposal 2.4B: No support. FL proposal 2.4C: Since there is no consensus to support dynamic switching, we don’t understand the motivation of this proposal at this stage. |
| ZTE2 |  | @FL, we think it is critical to reach the common understanding as in the above Proposal 2.4D. Could it be treated for discussion/clarification? FL: I captured it. Companies, please check FL proposal 2.4D. |
| QC3 |  | Regarding the following proposal by ZTE, we don’t know what it means by “Rel-18 PDSCH/PUSCH DMRS ports can be dynamically generated with length 2/4 FD-OCC”. Can ZTE please clarify? Also, about the note, as we commented multiple times, UE can choose not to perform any dynamic switch. We don’t know what is the intention of the note and don’t agree with it. Last but not least, what is the spec impact of this proposal? Can ZTE please clarify? |

  - MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports

In RAN1#110, we agreed to study MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports within a CDM group for PDSCH. Note that for PUSCH, there should be no restriction. For PDSCH, some companies claims that there is no need to make an agreement of “UE does not expect such MU-MIMO in a CDM group”, because the current specification already allows MU-MIMO by allocating different PN sequence, TCI state, etc. between different UEs by gNB implementation, and it does not make sense to preclude such existing gNB implementation by RAN1 spec. Hence, for 3) and 4), unless Alt.2 in 4) is agreed, we may not need any agreement (i.e. it is up to gNB responsible to avoid such MU operation).

Nevertheless, if we remove whole 3) and 4), some companies may have concern, because it looks like UE should handle such MU operation. Hence, slight wording change is suggested for 3) and 4).

FL Proposal 2.5A

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 1) For PUSCH, there is no restriction.

  - 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15/18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec.

    - Note: MU-MIMO restriction in Rel.17 is applied.

  - 3) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group,

    - UE is not required to handle such MU-MIMO in a CDM group (No spec. impact).

  - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, down select from the following.

    - Alt.1: UE is not required to handle such MU-MIMO in a CDM group (No spec. impact).

    - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

      - Dedicated UE capability is introduced.

      - The signaling is at least by RRC (FFS: whether to support DCI based signaling).

Objected by QC/Huawei/HiSilicon

FL Proposal 2.5B (Proposal from QC/HW)

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 1) For PUSCH, there is no restriction.

  - 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15/18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec.

    - Note: MU-MIMO restriction in Rel.17 is applied.

  - 3) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group,

    - Rel.15 UE2 may assume that the [orthogonal] Rel-18 New ports in the CDM group are not associated with transmission of PDSCH to another UE.

  - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, down select from the following.

    - Alt.1: Rel.15 UE2 may assume that the [orthogonal] Rel-18 New ports in the CDM group are not associated with transmission of PDSCH to another UE.

    - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

      - Dedicated UE capability is introduced.

      - The signaling is at least by RRC (FFS: whether to support DCI based signaling).

| Company | Comment |
| --- | --- |
| Docomo | Support. For 4), we support Alt.2 because UE configured with R15 can receive DMRS with FD-OCC length 4 to handle MU operation. If Alt.2 (RRC can configure increased M = {0,1} bits of antenna ports field) is supported in FL Proposal 2.2A, Alt.2 in the above proposal may be not beneficial. However, if not, Alt.2 in the above proposal is beneficial because we don’t need to increase the size of antenna ports field for some UEs for MU-MIMO. |
| Google | Support |
| Futurewei | On 4), we prefer Alt. 1. It is up to gNB implementation whether to schedule such MU-MIMO in a CDM group. |
| OPPO | Support the proposal and prefer Alt.1 for 4). It is up to gNB implementation to schedule such case and gNB should ensure the orthogonality among co-scheduled DMRS ports. |
| Nokia/NSB | Fine without specification impact. |
| CATT | Support. Alt.1 is preferred for 4). |
| Huawei, HiSilicon | For 3), the current version seems confusing. The original version aims to guarantee the performance of Rel.15 UE is not affected by explicitly adding scheduling restriction, the effect of which cannot be achieved by the current version. To address some companies’ concern that “the current specification already allows MU-MIMO by allocating different PN sequence between different UEs by gNB implementation”, similar to the MU MIMO restriction in current spec. which consists of wording “orthogonal”, we can introduce some wording meaning that aforementioned limitation between Rel.18 UE and Rel.15 UE applies to DMRS ports with same PN sequence (corresponds to the “orthogonal” in current MU restriction). The detailed capture way can be left to editor. FL: Thanks, but for non-orthogonal case (e.g. different PN sequence), it is up to gNB implementation that “orthogonal” DMRS ports may or may not be used to other UEs. Majority companies think there is no need to specify such thing. Also, when we make an agreement, we should clarify whether it has spec impact or not, so that spec editors can correctly capture the agreements. For 4), ensuring the orthogonality of length-2 FD-OCC between co-scheduled Rel.18 new DMRS ports and Rel.15 DMRS ports by introducing a restriction can also be treated as candidate direction. In this way the channel estimation performance of Rel.15 DMRS port can be guaranteed to the most extent. As a result we suggest to add an alternative for 4) as below: FL Proposal 2.5A - For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports, - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, down select from the following. Alt.3: Introduce restriction that the UE indicated with Rel.15 DMRS ports is not expected to be co-scheduled with a UE indicated with Rel.18 DMRS ports if the orthogonality of length-2 FD-OCC between the co-scheduled DMRS ports cannot be satisfied. |
| Lenovo | Support. For item 4, we support Alt.1 since for Alt.2 it may increase DCI bit. The application scenario for item 4 is not common and the motivation for introducing additional DCI signalling is not so strong. |
| Intel | For 4) we are OK with Alt-1 |
| QC | For 1)-3) in the FL Proposal 2.5A, we support. For 4), we suggest discussing it after MU restriction (in section 2.6) is settled. If MU restriction is agreed, we think Alt 2 is acceptable with MU restriction on certain rows; Otherwise, taking Alt 1 is more reasonable from our perspective. Furthermore, Alt 2 needs clarification. The signaling whether is based on DCI (This should be MAC-CE, as DCI based signaling is excluded in last meeting) or RRC is related to the discussion in section 2.4. Again, our view is that using 1 bit just to deliver Rel-18 UE 1 is with Rel-18 new ports is a not a design, based on the reasons we gave in previous section. In summary, we think 4) will be automatically settled after discussion in 2.4 and 2.6 are concluded. |
| MediaTek | Support. For item 4, we prefer Alt 1. |
| ZTE | Support. For item 4), Alt 3 raised by HW is also valid to in our views. |
| Ericsson | Support. For 4) we support Alt-2. |
| Sharp | For 4), we prefer Alt 2. |
| Xiaomi | Fine with proposal 2.5A. |
| Apple | Support 1,2,3 and for 4), our first preference is Alt.1, but would also be fine with Alt.2 as long as Alt-2 is the default UE capability. |
| New H3C | Support. For item 4, we prefer Alt 1. |
| China Telecom | Support. We prefer Alt.2 for 4). |
| Samsung | Support in principle, and support Alt1 for 4). |
| Fraunhofer | Support and we prefer ALT1 for item 4. |
| Spreadtrum | Support. For 4) we support Alt.1. If such case is scheduled, gNB should guarantee the orthogonality of all the DMRS ports. |
| CMCC | Support 1), 2) and 3). For 4), if MAC CE or DCI based switching between Rel.15 DMRS ports and Rel.18 DMRS ports is supported in FL proposal 2.4A, then Alt2 is not needed. |
| LGE | Support the proposal and prefer Alt.1 for 4). |
| vivo | Support no spec impact. We would like to mention again that even in Rel-15, there is no spec restriction that two co-scheduled UE should use the orthogonal ports in MU-MIMO. Please note that the different DMRS scrambling IDs can be configured for different UEs in MU-MIMO. Therefore, it’s up to gNB implementation. |
| QC4 | Sorry for a late comment on 3). We missed the wording change of 3) comparing to previous meeting. Regarding this wording, “UE is not required to handle such MU-MIMO in a CDM group (No spec. impact)”, we think the wording in previous meeting “UE does not expect such MU-MIMO in a CDM group” is better. With current wording, the two phrases in red actually conflict with each other. If there is no spec impact, gNB can schedule MU in that way, Rel-15 UE2’s performance will degrade. This is very against the design principle of 3GPP where a newly added feature should not degrade the performance of existing legacy feature. Of course, Rel-15 does not exclude implicit MU via different DMRS sequences, when gNB are confident the correlation of the two sequences are small, thus expecting small interference between them. But case 3 is same DMRS sequence with non-orthgonal FD-OCC code, these two ports would see large cross-correlation and Rel-15 UE2 performance will surely be degraded. Therefore, we think current wording is very problematic. We will object current wording. |
| FL | I added FL Proposal 2.5B. The restriction from HW/QC means “Rel.15 UE2 may assume that the [orthogonal] Rel-18 New ports in the CDM group are not associated with transmission of PDSCH to another UE”. However, RAN1 Rel.18 can only discuss impact of Rel.18 RAN1 spec. The suggestion from HW/QC does not impact to Rel.18 RAN1 spec., because Rel.15 UE does not read Rel.18 spec. I think HW/QC’s intention is to restrict gNB implementation, but RAN1 cannot explicitly restrict gNB behavior. I think this is why most of companies think there should be no spec. impact on this issue. Any wording suggestion to Proposal 2.5B would be appreciated. |

  - MU-MIMO scheduling restriction within a CDM group

In section 5.1.6 in TS38.214, MU-MIMO scheduling restriction is specified as following.

For DM-RS configuration type 1, - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 30} in Table 7.3.1.2.2-1 and Table 7.3.1.2.2-2 of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 12} in Table 7.3.1.2.2-1A and {2, 9, 10, 11, 30 or 31} in Table 7.3.1.2.2-2A of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with two codewords, the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE. For DM-RS configuration type 2, - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10 or 23} in Table 7.3.1.2.2-3 and Table 7.3.1.2.2-4 of Clause 7.3.1.2 of [5, TS38.212], or - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10, 23 or 24} in Table 7.3.1.2.2-3A and {2, 10, 23 or 58} in Table 7.3.1.2.2-4A of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with two codewords, the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

After Rel.18 eType1/eType2 DMRS ports tables are defined, which row of DMRS port combination requires MU-MIMO restriction will be specified. It is not proper to just reuse the indexes of rows which require MU-restriction in current Rel.17 spec. For example, for Rel.15 Type 1, if row 2 (i.e. {0,1} with number of CDM group without data = 1) is indicated, remaining DMRS ports are not used to another UE. This is because there is no remaining orthogonal DMRS ports in Rel.15 Type 1. However, in Rel.18 eType 1, if row 2 (i.e. {0,1} with number of CDM group without data = 1) is indicated, there are remaining DMRS ports of {8,9}, and it is straightforward to allow DMRS ports combination of {8,9} with number of CDM group without data = 1 to another UE. Hence, we cannot reuse the indexes of rows which require MU-restriction in current Rel.17 spec.

Qualcomm [23] etc. proposes the following principle.

FL Proposal 2.6A

- Adopt the following MU scheduling restriction for Rel.18 DMRS ports for PDSCH:

  - If the DMRS ports of a UE are in more than one CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

    - The above applies to both single symbol and dual symbol DMRS.

  - Furthermore, for dual symbol DMRS, if the DMRS ports of a UE are associated with more than one TD-OCC codes in one CDM group, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

| Company | Comment |
| --- | --- |
| Docomo | Not support. For Type1 in R15, if two CDM groups are used for a UE, there is MU-MIMO restriction. However, for Type2 in R15, even if more than one CDM groups are used for a UE, there is no MU-MIMO restriction for some cases. Hence, the above proposal looks too restrictive compared to Rel.15, and we prefer to discuss which row requires MU-MIMO restriction row by row, after antenna ports table for PDSCH is agreed. |
| Google | We failed to see the benefit for the proposal |
| Futurewei | Not support. In our view, this restriction is unnecessary and will reduce the MU scheduling flexibility and degrade MU-MIMO performance, which this agenda item is intended to enhance. |
| OPPO` | Support. We think similar restriction as in Rel-15 should be extended to Rel-18 to ensure the channel estimation performance. We can further discuss which Row needs such restriction in the next step. |
| Nokia/NSB | We are fine with the first bullet. |
| CATT | Not support. The number of orthogonal ports in one CDM is doubled in Rel.18 and the restriction is not needed. |
| Huawei, HiSilicon | Not support. Considering the whole WID is targeting higher-layer MU-MIMO and there does exist some DMRS port combinations crossing multiple CDM groups without any MU restriction already as discussed in section 2.1.1, aforementioned MU scheduling restriction is not needed. |
| Lenovo | Not support. It is too restrictive. We agree with FL to discuss together with antenna port indication table. |
| Intel | Such restriction is not needed. |
| QC | We support the proposal. It does not change the technical fact of the problem, whether we discussed row by row in DMRS port table or we discuss it here as a general principle. The key point is that a UE with capability of decoding 1 CW PDSCH cannot estimate more than 4 DMRS ports. While some of the rows in the DMRS ports table for 1 CW require UE to estimate more than 4 ports, if MU exist. That is the fundamental reason the above MU restriction is needed for those rows. Those rows are listed as below. - DMRS ports distributed into two CDM groups, for both single symbol and dual symbol DMRS - DMRS ports distributed into two TD-OCC codes, for dual symbol DMRS. To DCM: whether DMRS is with type 1 or type 2, it does not change the technical problem. If we agree with the MU restriction for type 1 DMRS due to the technical reason we mentioned above, we should agree the same MU restriction for type 2 DMRS. |
| MediaTek | Support |
| ZTE | Do NOT support. This proposal does not accurately reflect the legacy MU-MIMO restriction as mentioned by companies. Several rows in the legacy are in line with the first and second bullets but not restricted in MU-MIMO scenario, i.e.: - For the first bullet, - Rows 9 and 20-22 when Type2 + maxlength1 in Table 7.3.1.2.2-3 and Table 7.3.1.2.2-3A of TS 38.212. - Rows 9 and 20-22 when Type2 + maxlength2 in Table 7.3.1.2.2-4 and Table 7.3.1.2.2-4A of TS 38.212. - For the second bullet, - Rows 26-29 when Type2 + maxlength1 in Table 7.3.1.2.2-3 and Table 7.3.1.2.2-3A of TS 38.212. - Rows 26-29 when Type2 + maxlength2 in Table 7.3.1.2.2-4 and Table 7.3.1.2.2-4A of TS 38.212. If it is to make the legacy MU-MIMO restriction of Rel-18 DMRS ports corresponding to the Rel-15 DMRS ports with same rules of combination, it only needs to list those rows of each case as the current specification. Besides, note that Rel-18 DMRS ports indication tables are still pending, this issue should be postponed accordingly. TS 38.214, section 5.1.6.2 For DM-RS configuration type 1, - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 30} in Table 7.3.1.2.2-1 and Table 7.3.1.2.2-2 of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 12} in Table 7.3.1.2.2-1A and {2, 9, 10, 11, 30 or 31} in Table 7.3.1.2.2-2A of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with two codewords, the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE. For DM-RS configuration type 2, - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10 or 23} in Table 7.3.1.2.2-3 and Table 7.3.1.2.2-4 of Clause 7.3.1.2 of [5, TS38.212], or - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10, 23 or 24} in Table 7.3.1.2.2-3A and {2, 10, 23 or 58} in Table 7.3.1.2.2-4A of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with two codewords, the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.. |
| Ericsson | We prefer to not apply such MU restriction for Rel-18 DMRS for PDSCH. |
| QC2 | To ZTE: The MU problem on Rel-18 is more challenging than Rel-15. We understand ZTE prefer to follow Rel-15 for everything. But please look at the problem from technical perspective. On this MU issue, things change from Rel-15 to Rel-18. Rel-15 spec only allows at most 4 DMRS ports with two CDM groups (for 1-symbol DMRS) or two TD-OCC codes in one CDM group (for 2-symbol DMRS). UE only need to estimate at most 4 MU ports for MU detection and noise/interference estimation. In Rel-18, in the same scenarios, Rel-18 specs might allow up to 8 DMRS ports, A UE with capability of decoding 1 CW can only estimate up to 4 DMRS ports, hence cannot handle these MU scenarios, as shown in the figure below. From gNB MU scheduling perspective, we don’t see what are the advantages of distributing <=4 ports of each UE into two CDM groups or two TD-OCCs, rather than keep one UE’s <=4 ports in a single CDM group or a single TD-OCC. Using CDM group to TD-OCC to separate UEs are natural way to minimize interference between MU. For UEs with rank<=4, can ZTE or any proponent who like MU scheduling with a UE’s DMRS ports across CDM groups or TD-OCCs please explain what are the benefits to do so, comparing to confining <=4 DMRS ports of a UE within a CDM group and a TD-OCC. |
| Xiaomi | Not support. We get the intention of this proposal 2.6A, but it is too strict to R18 DMRS. The intention of this WI is to support more layers/UEs in MU-MIMO. This restriction put a little too much limitation on the use cases of R18 DMRS. |
| Apple | Support the first bullet and are also fine with second bullet |
| New H3C | This restriction isn’t required |
| China Telecom | Not support. We don’t understand why the restrictions are needed. |
| Samsung | We don’t support additional MU scheduling restriction within a CDM group. The purpose of this agenda item is to increase the number of MU-MIMO layer. |
| Fraunhofer | We fail to see the benefit of this proposal. |
| Spreadtrum | Support. MU scheduling restriction is important to reduce UE complexity. |
| CMCC | Not support. One of the motivations to increasing DMRS ports is for MU-MIMO, if adding MU restriction on the rows listed in the proposal, the performance of increasing DMRS ports is degraded. |
| LGE | Support. We think similar restriction as in Rel-15 should be extended to Rel-18. |
| vivo | Support in principle. We can understand that the principle in this proposal is similar with what for legacy DMRS. But it can be discussed in section 2.1 case by case, like the restriction in the previous agreements. |
| ZTE2 | To QC: Why this kind of restriction you emphasized is specially needed to Rel-18 UE but not for Rel-15 UE? Notably, the current spec does NOT have such restriction as we pointed out above. |
| QC3 | @ZTE, please read our input/comments above, which has been provided 24 hours ago. Please let us know if you have any technical question to our input. |

  - Sequence mapping

CATT [10] discuss the following issue:

With length 4 FD-OCC, patterns of Rel.18 eType 1 DMRS and eType 2 DMRS can be designed as same as that in Rel.15 type 1 DMRS and type 2 DMRS. In Rel.15 DMRS, sequence rm is mapped to resource elements k,lp,μ according to equation (1): αk,lp,μ=βPDSCHDMRSwfk'wtl'r2n+k'k=4n+2k'+∆ Configuration type 16n+k'+∆ Configuration type 2k'=0,1l=l+l'n=0,1,… (1) Parameter k' is included in wfk' , k and r2n+k'. In wfk', k' is the index of OCC weighting. In parameter k, k' determines the frequency resources (subcarriers) used for DMRS transmission. In Rel.18 DMRS with length 4 FD-OCC, four values of k'(e.g., k0, k1, k2 and k3) are needed, and they are corresponding to four resource elements in frequency domain, respectively. In Rel.18 eType 2 DMRS, four values of k' can be 0, 1, 6 and 7 to facilitate resource mapping in frequency domain. Take CDM group 0 as an example, these values can ensure that DMRS occupies the 1st, 2nd, 7th, and 8th REs in one RB. However, these values of k' will complicate sequence generation of rm. For example, UE may need to generate longer PN sequence than needed if r8n+k' is used in resource mapping equation, since not all values in the generated sequence are used for DMRS transmission. On the other hand, sequence orthogonality between multiple ports may be an issue due to the same reason (discontinuous values in the generated sequence are used for DMRS transmission). Four values of k' can also be 0, 1, 2 and 3 to simplify sequence generation, and r4n+k' is used in resource mapping equation. In order to ensure that DMRS occupies the 1st, 2nd, 7th, and 8th REs in one RB for CDM group 0, parameter k can be modified, and equation (2) is used in sequence mapping. αk,lp,μ=βPDSCHDMRSwfk'wtl'r4n+k'k=12n+k'+∆,k'=0,1 12n+k'+4+∆,k'=2,3k'=0,1,2,3l=l+l'n=0,1,… (2) In Rel.18 eType 1 DMRS, four values of k' can also be 0, 1, 2 and 3, and equation (3) can be used in sequence mapping. αk,lp,μ=βPDSCHDMRSwfk'wtl'r4n+k'k=8n+2k'+∆k'=0,1,2,3l=l+l'n=0,1,… (3)

FL Proposal 2.7A

The following sequence mapping equations are adopted for Rel.18 eType 1 DMRS and Rel.18 eType 2 DMRS, respectively:

- Rel.18 eType 1 DMRS:

αk,lp,μ=βPDSCHDMRSwfk'wtl'r4n+k'k=8n+2k'+∆k'=0,1,2,3l=l+l'n=0,1,…

- Rel.18 eType 2 DMRS:

αk,lp,μ=βPDSCHDMRSwfk'wtl'r4n+k'k=12n+k'+∆,k'=0,1 12n+k'+4+∆,k'=2,3k'=0,1,2,3l=l+l'n=0,1,…

| Company | Comment |
| --- | --- |
| Docomo | Fine with the proposal. |
| Google | I am not sure whether we need to agree on this, but it looks this can be handled by editor based on our previous agreement. |
| Futurewei | Support in principle. |
| OPPO | Maybe it can be handled by editor when we finish the design. |
| Nokia/NSB | Up to editor. |
| CATT | Support. |
| Huawei, HiSilicon | Seems the detailed mapping equations can be left to editor. |
| Lenovo | Support in principle and it is up to editor. |
| Intel | Do not need an agreement probably. This is the natural outcome of current agreements and is up to the editor. |
| QC | We think it can be up to editor how to capture agreements made for Rel-18 DMRS. The proposal is not needed. |
| MediaTek | Agree with comment made by OPPO/Huawei this can be left to editor. |
| ZTE | It is sufficient to be up to editor. |
| Ericsson | Shall be up to editor to decide. |
| Sharp | Support |
| Xiaomi | Support |
| Apple | Agree that this can left up to editor |
| New H3C | OK in principal |
| China Telecom | Fine with the proposal and it can be up to editor. |
| Samsung | We are fine with the proposal but also okay to be up to editor. |
| Fraunhofer | Do not support. |
| Spreadtrum | Support in principle. Up to the editor is also fine. |
| CMCC | Support |
| LGE | Support and it is up to editor. |
| vivo | Support in principle, and it can be up to editor. |
| FL | Multiple companies comment that it can be up to editors. Proponents, please comment if you have different views. |

  - Scheduling restrictions of PDSCH among MU-MIMO UEs

Qualcomm [23] discuss the following:

In Rel-15, there were some restrictions already introduced (as listed in detail below). However, in the deployment, implementation, and operating of 5G in the past a few years, a few new cases which requires more alignment among MU were identified. Given Rel-18 MIMO supporting more MU scenarios, and this WI is targeting MU enhancement, it is preferred to consider potentially aligning transmission parameters of MU in a better way. There are three enhanced MU alignments that can be considered in Rel-18. - Alignment 1: aligning the number of CDM groups without data among MU. - Alignment 2: aligning the PRG boundary for MU in different CDM groups - Alignment 3: aligning the staring and ending PDSCH symbol for MU For alignment 1, one can notice that in Rel-15, there is already a paragraph in TS38.214 in this line to aligning number of CDM groups. However, the highlighted text (in yellow) is not clear. There could be two interpretations. - Interpretation 1: the “CDM groups without data” are not used for data transmission only for this target UE. While co-scheduled UEs may still use them for data transmission. When receiving PDSCH scheduled by DCI format 1_1, the UE shall assume that the CDM groups indicated in the configured index from Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-4 of [5, TS. 38.212] contain potential co-scheduled downlink DM-RS and are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) in Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-4 of [5, TS. 38.212] correspond to CDM group 0, {0,1}, {0,1,2}, respectively. Interpretation 2: the “CDM groups without data” are not used for data transmission for all MU. If interpretation 1 is assumed, then depends on MU using them for data or not, there could be 3dB or 4.77dB difference on interfering MU’s DMRS to data power ratio, which is unknown to target UE. Target UE will need to do blind detection to figure that out. If interpretation 2 is assume, there is no ambiguity. With the above analysis, we have the following proposal. Proposal 2: A UE is expected the same “number of CDM groups without data” for co-schedule MU in Rel-18 DL DMRS. The UE does not expect the precoding of the potential co-scheduled UE(s) in other DM-RS ports of the same CDM group to be different in the PRG-level grid configured to this UE with PRG =2 or 4. The UE does not expect the resource allocation of the potential co-scheduled UE(s) in other DM-RS ports of the same CDM group to be misaligned in the PRG-level grid to this UE with PRG=2 or 4. For alignment 2, as shown in the following, Rel-15 actual has PRG boundary (and allocation in occupied PRGs) alignment across MU UEs, but only for MU within the same CDM groups. For channel estimation of target UE, the below restriction in Rel-15 is fine, as the channel is only performed for the assigned CDM group for the target UE. However, for interference/noise estimation, Rel-15 restriction is not sufficient, as target UE need to estimate interference/noise based on all tones which includes the other CDM group. With misaligned PRG boundary, target UE’s interference estimation is not accurate, unless UE blindly detect the potential PRG boundary mis-alignment across MU. With the above analysis, we have the following proposal. Proposal 3: A UE is expected the same PRG boundary and the same RB assignment (in PRG-level grid) for co-schedule MU in same and different CDM groups in Rel-18 DL DMRS. Similar to frequency alignment as in alignment 2, alignment 3 for time domain is desired for target UE’s interference/noise estimation. Again, in Rel-15, some restriction (as cited below) along this line is made from channel estimation perspective. However, without aligned PDSCH starting and ending symbol, although DMRS symbols are aligned with can make sure a good channel estimation for target UE, but the mis-aligned PDSCH starting and ending symbol can still break the target UE’s interference/noise estimation. To solve this issue, the following proposal is made. Proposal 4: A UE is expected the same staring OFDM symbol and the same ending OFDM symbol for PDSCH of co-schedule MU in Rel-18 DL DMRS.

Since we have not discussed scheduling restriction of PDSCH among MU-MIMO UEs, we can discuss whether such restriction should be introduced for Rel.18 DMRS. New restriction from Rel.15-17 is noted with underline in the proposal.

FL Proposal 2.8A

- Adopt the following MU scheduling restriction for Rel.18 DMRS ports for PDSCH:

  - 1) A UE is expected the same “number of CDM groups without data” for co-schedule MU in Rel-18 DL DMRS.

  - 2) A UE is expected the same PRG boundary and the same RB assignment (in PRG-level grid) for co-schedule MU in same and different CDM groups in Rel-18 DL DMRS.

  - 3) A UE is expected the same staring OFDM symbol and the same ending OFDM symbol for PDSCH of co-schedule MU in Rel-18 DL DMRS.

| Company | Comment |
| --- | --- |
| Docomo | Not support to introduce additional MU restriction from Rel.15-17. 1) Our understanding of the existing spec. is interpretation2, and co-scheduled UEs may use “CDM group without date” for the UE’s DMRS. We are fine to clarify the interpretation2, but it can be done in Rel.15-17 CR in AI7.1 or 7.2. 2) In Rel.15-17, the restriction of PRG boundary is only applied to UEs with the same CDM group, and we prefer to keep the same scheduling flexibility for Rel.18. 3) In Rel.15-17, there is no restriction that PDSCH between MU should be fully overlapped (except for mDCI mTRP in Rel.16). Although we understand the interference/noise can be different if PDSCHs are not fully overlapped, we assume UE can demodulate PDSCH as long as DMRS symbols are aligned. We prefer to keep the same scheduling flexibility for Rel.18. |
| Futurewei | We share the same view as Docomo that these additional MU scheduling restrictions are unnecessary. |
| OPPO | We think the scheduling restriction in Rel-17 is sufficient and can be extended to Rel-18. No new rule needs to be introduced. |
| Nokia/NSB | We didn’t have such restriction in Rel-15. Up to NW scheduling. |
| CATT | For 1), support to discuss. For 2) and 3), we share the same view as Docomo and Futurewei. These additional scheduling restrictions are not necessary. |
| Huawei, HiSilicon | Share the same view with Docomo. |
| Lenovo | We think it is not necessary to introduce additional MU scheduling restrictions. |
| Intel | Share same understanding as DOCOMO. Legacy rules should be sufficient. For 3) legacy rules should already ensure DM-RS symbols are aligned even if PDSCH length may be different. |
| QC | We support the proposals. For 1), we can see what are companies’ understandings of current spec. For 2) and 3), we understand there were no such restriction in Rel 15-17. But the issue exists, which causes UE cannot estimate noise/interference of co-scheduled UE correctly. DMRS alignment does not solve this issue. Please see the following figures for 3) of misaligned PDSCH in time domain. The similar mis-alignment of PRG in freq domain motivates restriction 2). It is not true that “UE can demodulate PDSCH as long as DMRS symbols are aligned”. Demodulation, especially for high MCS, easily fail due to in-accurate noise/interference Rnn estimation. |
| MediaTek | Support. Similar comments as QC. We believe the restriction added by 2) and 3) will allow for better UE demod performance. |
| ZTE | Do not support. It is out of scope in Rel-18. |
| Ericsson | We don’t support this proposal. The legacy MU restrictions are sufficient. However, it is good to be aware of such UE demodulation limitations on MU scheduling. |
| Apple | Support and similar view as QC and Mediatek |
| New H3C | We have the same view with Docomo. |
| ZTE | Not support just like Proposal 2.6A. |
| Samsung | We don’t see any necessity on having additional spec. based MU restriction, legacy rules are sufficient. |
| Fraunhofer | Same view than Docomo. |
| Spreadtrum | We agree with some companies view that such MU restriction can achieve better demodulation performance. |
| LGE | We have same understanding as DOCOMO. Legacy rules should be sufficient. For 3) legacy rules should already ensure DM-RS symbols are aligned. |
| vivo | Don’t support. We prefer to follow the legacy principle. |

  - Other proposals

Following proposals are also proposed.

| Proposals | Companies |
| --- | --- |
| - Study on OCC disabling scheme for new DMRS type (Rel.17 feature in above 52.6GHz). | Samsung |
| - Reusing low PAPR design for Rel.18 DMRS port(s) | Huawei/HiSilicon |
| - support eType1 DMRS for MsgA PUSCH. | Lenovo |
| - Additional scheduling restriction of orphan RE issue for eType1 | Vivo, CATT, Lenovo, Google |
| - Orphan RB issue for eType2 | Qualcomm |
| - PTRS power boosting for PDSCH with Rel-18 DMRS ports | Lenovo |

Please provide your views on the above proposals, or other aspects which are not included in the summary, if any.

| Company | Comment |
| --- | --- |
| FL | For 2), the proposal is to reuse existing specification of low PAPR sequence. From FL perspective, if we don’t make any agreement to update the spec., the existing Rel.17 spec. (in this case, both Rel.15 DMRS sequence and Rel.16 DMRS sequence) is automatically applied, if UE supports its UE capability. Hence, we don’t need to discuss proposal to “reuse existing spec.”. |
| Docomo | For 5), it seems the issue (orphan RB in PRG) also exists in Rel.15. We are not sure why we need to discuss it for Rel.18 DMRS. |
| Google | For 5), could Docomo clarify a bit how such issue exists in R15? |
| Docomo2 | Re Google’s question to 5), if we correctly understand the proposal, the issue is when PRB boundary of the scheduled PDSCH is not aligned with PRG boundary (as in figure), channel estimation performance may degrade for “eType2” DMRS, because UE may de-spread DMRS in PRG level. But, it seems this is also true for Rel.15 Type 1 or Type 2 DMRS, and this is not special issue of Rel.18 DMRS. Orphan RB issue for eType 2 in R1- 2303576 |
| Nokia/NSB | - Not needed. We didn’t have it for Rel-15 - Override existing spec is enough. - DMRS port 0 is always used for MsgA @Lenovo, thanks for correcting me. Confused with Msg3. Can we use Rel-15 DMRS for the case? - Not need. Up to network - Not need. Up to network |
| Huawei, HiSilicon | Thanks FL for reply. We share the same view with FL that the existing Rel.17 spec. should be automatically inherited. Given that DMRS root sequence is one of the most fundamental factors that will influence the implementation, here we only want to quickly check whether all companies share the same view and derive a conclusion. |
| Lenovo | We propose to study PTRS power boosting for PDSCH with Rel-18 DMRS ports. For item 3, To Nokia/NSB: For Msg-A, the mapping is defined between preambles of a PRACH slot and PUSCH occasions associated with DMRS resource, where one DMRS resource is associated one DMRS port index and/or DMRS sequence. One example is shown as follows: Fig.x Example of the mapping between PRACH transmission and PUSCH transmission Also, please refer to the following description in TS38.214 For MsgA PUSCH transmission, if the UE is not configured with msgA-PUSCH-DMRS-CDM-group, the UE shall assume that 2 DM-RS CDM groups are configured. Otherwise, msgA-PUSCH-DMRS-CDM-group indicates which DM-RS CDM group to use from the set of {0,1}. For MsgA PUSCH transmission, if the UE is not configured with msgA-PUSCH-NrofPorts, the UE shall assume that 4 ports are configured per DM-RS CDM group for double-symbol DM-RS. Otherwise, msgA-PUSCH-NrofPorts with value of 0 indicates the first port per DM-RS CDM group, while a value of 1 indicates the first two ports per DM-RS CDM group. |
| QC | For 5), the issue does not exist in Rel-15. In Rel-15, because one DMRS symbol only supports 6 DMRS ports, in the orphan RB, each DMRS port can have 12/6=2 observations/looks of channel in frequency domain. It is possible to estimate slope of the channel with 2 observations. However, in Rel-18, each DMRS symbol supports 12 DMRS ports. In Orphan RB, each DMRS port only has 12/12=1 observation/look of channel in frequency domain. It is impossible to estimate slope of the channel in orphan RB. In other words, we can only estimate the DC component of the channel (i.e., assuming channel is flat in the RB). Channel estimation performance of the orphan RB will be pretty bad, which makes the orphan RB performance bottleneck for the whole PDSCH assignment, especially with high MCS. To overcome this performance issue, UE can do precoder blind detection to align orphan RB with other PRGs. But this would need extra implementation complexity which justifies a dedicated UE capability. |

- Specifying objective #5 (>4 layers PUSCH DMRS)

  - Antenna port(s) table for PUSCH (rank 5-8)

In RAN1#112, we made the following working assumption. In this meeting, multiple companies propose to confirm the WA. However, since joint indication or separate indication of TRI and TPMI is still not decided for CB MIMO in AI 9.1.4.2, it is safer to keep the working assumption for CB MIMO based PUSCH.

Working Assumption To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables. - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table). - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

However, for NCB, it seems only joint indication of TRI and SRI is assumed in AI 9.1.4.2, and there should be no issue to confirm the WA for NCB based PUSCH. WA for CB can be confirmed after the outcome of the discussion of joint/separate indication of TRI/TPMI in AI9.1.4.2.

FL Proposal 3.1A

- Confirm the following Working Assumption in RAN1#112 at least for NCB based PUSCH:

  - To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables.

    - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

      - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

  - Note: The above Working Assumption for CB based PUSCH may be confirmed later.

Whether separate DMRS ports table is needed for partial coherent UL codebook.

In RAN1#112, the FL proposal 3.1B was discussed, and following comments were made.

ZTE: Support Alt. 2. According to the legacy rules, DMRS port combinations can be from the same or different CDM groups no matter full/partial/non-coherent UL codebook. We fail to see the reason the make the new restriction in Rel-18 specially. CMCC: Support Alt 2. DMRS ports combination(s) that the same antenna group into different DMRS CDM groups is supported in Rel-15, we don’t see more restriction is needed for Rel-18. InterDigital: Support Alt.1. We’re concerned with the loss of coherency if DMRS ports of the same CDM group are mapped to different antenna groups. This would lead to CHEST degradation and poor demodulation. Nokia/NSB: Support Alt 1. For partial coherent case, the precoded layers may have different power according to the number antenna ports to layers mapping, and multiplexing non-coherent DMRS ports with different power causes performance degradation.

FL Proposal 3.1B (Suspended)

- For > 4 layers PUSCH with Rel.15 Type1/Type2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports, for partial coherent UL codebook, down select from the following:

  - Alt.1: DMRS ports combination(s) that the same antenna group into the same DMRS CDM group.

  - Alt.2: DMRS ports combination(s) for full/non-coherent UL codebook is reused.

    - Note: DMRS ports combination(s) that the same antenna group into the same or different DMRS CDM group.

FL: Since this issue is related to the outcome of CW to antenna port group mapping in AI 9.1.4.2, we will wait to discuss in this agenda.

Table. Summary of companies’ views for FL Proposal 3.1B (in RAN1#112)

| Support Alt.1 | Support Alt.2 |
| --- | --- |
| IDC, Nokia/NSB, Sharp, Docomo, Lenovo, | OPPO, Xiaomi, CATT, CMCC, Google, ZTE, Huawei, HiSilicon, Fraunhofer IIS/HHI, LGE, Ericsson, vivo, Spreadtrum,New H3C |

| Company | Comment |
| --- | --- |
| Docomo | Proposal 3.1A: Support. Proposal 3.1B: Support Alt.1. |
| Google | Proposal 3.1A: Support. Proposal 3.1B: Support Alt2. Now even for STxMP, there is no restriction like Alt1. |
| InterDigital | Proposal 3.1B: Support Alt. 1. We have antenna group definition that is based on relative coherency between different antenna elements which also is driving precoder type for uplink transmission. Therefore, there is no reason not to respect the coherency of the TX chain for DMRS CDM mapping. In our view, for partial coherent UEs, each CDM group should be mapped to a different antenna group to avoid potential loss due to inaccurate channel estimation. |
| OPPO | Proposal 3.1A: Support. Proposal 3.1B: Support Alt2. We think it can be implemented by gNB scheduling. |
| Lenovo | Proposal 3.1A: Support. Proposal 3.1B: Support Alt1. |
| QC | Proposal 3.1A: Support. We are also fine to hold on confirming the WA until 9.1.4.2 is concluded. Proposal 3.1B: Support Alt 2, which is the legacy design. To interdigital/Nokia: We don’t see mapping DMRS ports in a same CDM group to PUSCH/SRS ports in different antenna group would impact channel estimation performance much. |
| MediaTek | Proposal 3.1A: Support. Proposal 3.1B: Support. Our preference is Alt 2. |
| ZTE | Proposal 3.1A: Support. Proposal 3.1B: Support Alt.2, we also share the same with Google in terms of STxMP UL. |
| Sharp | Proposal 3.1A: We are OK with the proposal in terms of NCB. Proposal 3.1B: We prefer to support all DMRS port combinations in both Alt 1 and Alt 2. |
| Xiaomi | Proposal 3.1A: Support. Proposal 3.1B: Support Alt2. |
| Apple | Proposal 3.1A: Support. Proposal 3.1B: Support and our preference is Alt 2. |
| OPPO | Proposal 3.1A: Support. Proposal 3.1B: Support Alt2. |
| Fraunhofer | Proposal 3.1A: Support. Proposal 3.1B: Support Alt2. |
| Spreadtrum | Proposal 3.1A: Support. Proposal 3.1B: Support Alt 2. |
| LGE | Proposal 3.1A: Support. Proposal 3.1B: Support Alt2. |
| vivo | Proposal 3.1A: Support. Proposal 3.1B: Support Alt 2. In the discussion of STxMP, there is no such restriction. |
| Nokia/NSB2 | @Google, STxMP is rather intended to transmit them into different TRP/Panels. In DL NCJT, we have such restriction. (Do not multiplex DMRS ports with different TCI states in a CDM group). Non-coherent Tx with different propagation channel may cause the interference. For SU-MIMO, why don’t we use the safer scheme. For high-rank reception, channel estimation performance is quite important. Also, the transmit power of each layer are quite different. For example, Ng=2, rank 5 with 4(group 0) + 1 (group1), then the layer from group 1 has transmitted with 6dB more power, which is very high even with different channel gain. If possible, it is preferable to agree this principle only for Ng=2. Ng=4 has complexity but less impact performance impact because only 1 or 2 DMRS ports per group. FL Proposal 3.1B1 - For > 4 layers PUSCH with Rel.15 Type1/Type2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports, for partial coherent UL codebook with Ng=2, support - Alt.1: DMRS ports combination(s) that the same antenna group into the same DMRS CDM group. FL: For FL Proposal 3.1B, we should wait the discussion in AI 9.1.4.2. |
| FL | Since FL Proposal 3.1B is related to the outcome of CW to antenna port group mapping in AI 9.1.4.2, we will wait to discuss in this agenda. If you have comment, please do so to AI 9.1.4.2. FYI: we have the following agreement in AI9.1.4.2 on Monday GTW in this week. Working Assumption For partially coherent uplink precoding by an 8TX UE, Ng=2, - At least the following combinations of layer splitting are supported - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3).. |
| FL | FL Proposal 3.1A is moved to [112bis-e-R18-MIMO-04] DMRS - EMAIL ENDORSEMENT 1. |

3.1.1 For Rel.15 DMRS ports

In RAN1#111, following was agreed.

- For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following: - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook.

Based on the agreement for R15 DMRS ports, DMRS ports combination(s) for rank = 5-8 PDSCH is simply reused for PUSCH. In RAN1#112, some companies proposed to select one row for each table. However, we have agreement of “The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH”, and it seems we should reuse exactly the same DMRS combinations.

FL Proposal 3.1.1A (for Working Assumption)

- Adopt Table 7.3.1.1.2-12B/13B/14B/15B/16B/17B/20B/21B/22B/23B to support signalling >4 ranks PUSCH with Rel-15 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH.

- FFS: Whether/how some of bits in the antenna ports field can be reused for other purpose for >4 ranks PUSCH.

Table 7.3.1.1.2-12B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-16B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-17B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-20B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 12-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support. Some companies may prefer to select one row for each table, but in our understanding, the one or two DMRS port combination(s) in the above tables are already agreed because of “The same DMRS port combination(s)” in the agreement. |
| Google | Support in principle |
| OPPO | Fine. |
| Nokia/NSB | Support in principle for full-coherent/non-coherent only. |
| CATT | Support. |
| Huawei, HiSilicon | Fine with the DMRS port combination(s). Whether a joint table or multiple separate tables are needed depends on whether the WA in section 3.1 is confirmed for CB-based PUSCH |
| Lenovo | Support. |
| Intel | OK |
| QC | We don’t prefer the proposal, which map one CW’s layers to two CDM groups. We still prefer the other design which map one CW’s layers to one CDM group, which can simplify gNB receiver. We think we can defer the decision on this proposal after we decide whether/how to confirm the WA for 2CWs type 1 1-symbol DMRS. FL: But, it is against the previous agreement. |
| ZTE | Support. This is in line with the same rule as we elaborated in section 2.1. Again, we fail to see the logic that any restriction over the legacy (even it is for Rel-15 UE) is needed, which is out of scope from our perspective. |
| Ericsson | We are open to add or modify combinations as QC proposed to map one CW’s layers to one CDM group. |
| Sharp | Support |
| Xiaomi | Fine with the proposal |
| Apple | Fine |
| New H3C | OK |
| China Telecom | Support. |
| Spreadtrum | Support |
| Google | Support. |
| LGE | Support. |
| Vivo | Support. |
| FL | Most of companies are ok with the proposal. I added “at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH ” based on the previous agreement. Since separate DMRS port tables is WA, this proposal is for WA. |

3.1.2 For Rel.18 DMRS ports

Vivo, CATT, CMCC, Docomo, etc. propose to reuse DMRS port combinations for PDSCH with Rel.18 DMRS ports. I’d like to check whether the following principle is agreeable.

FL Proposal 3.1.2A

- For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, support at least the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports at least for full or non-coherent UL codebook.

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support. |
| Google | It seems the DMRS port indication has nothing to do with the UL precoder. We think the common design could be applicable for all types of precoders. |
| OPPO | Agree with Google. |
| Nokia/NSB | Support in principle. But, we prefer using at most two combinations per rank. |
| CATT | Support. |
| Huawei, HiSilicon | Support. |
| Lenovo | Support. |
| Intel | OK |
| QC | One question for clarification: if we have proposals in section 3.1.2.1/2/3/4 agreed, do we still need this proposal? FL: Now I added “at least”, and it is possible to add other combination(s). So, we can discuss separately. |
| ZTE | Support. |
| Ericsson | Don’t support. For PUSCH there’s no MU-MIMO restriction as PDSCH, only reuse the PDSCH will limit the advantage of Rel-18 DMRS design with double orthogonal ports. More ports combinations for PUSCH shall be supported to increase the MU-MIMO capacity in uplink. FL Proposal 3.1.2A - For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, reuse support at least the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports at least for full or non-coherent UL codebook. FL: I reflected. |
| Sharp | Support |
| Xiaomi | support |
| Apple | Support |
| New H3C | OK |
| China Telecom | Support. |
| Spreadtrum | Support |
| CMCC | The DMRS port combinations can be used for full, partial and non-coherent UL codebook. |
| LGE | Support. |
| vivo | Support |

3.1.2.1 eType1, maxLength1

Since DMRS ports table for PDSCH for eType1 maxLength1 is almost agreed, I’d like to discuss the following proposal. Since DMRS ports combinations for 2 CWs for PDSCH are working assumption, the following proposal can be working assumption.

FL Proposal 3.1.2.1A (for working assumption)

- For Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH with rank 5-8, following Table 7.3.1.1.2-11-X-1, Table 7.3.1.1.2-11-X-2, Table 7.3.1.1.2-11-X-3, and Table 7.3.1.1.2-11-X-4 are supported.

  - FFS: The size of antenna ports field in DCI format 0_1/0_2.

Table 7.3.1.1.2-11-X-1 Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=1, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8 |
| [1 | 2 | 3,8,9,10,11] |
| [2 | 2 | 0,1,8,10,11] |
| 3-15 | Reserved | Reserved |

Table 7.3.1.1.2-11-X-2: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,10 |
| [1 | 2 | 2,3,8,9,10,11] |
| 2-15 | Reserved | Reserved |

Table 7.3.1.1.2-11-X-3: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9,10 |
| [1 | 2 | 1,2,3,8,9,10,11] |
| 2-15 | Reserved | Reserved |

Table 7.3.1.1.2-11-X-4: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0,1,2,3,8,9,10,11 |
| 1-15 | Reserved | Reserved |

| Company | Comment |  |
| --- | --- | --- |
| Docomo | Support. |  |
| Google | Support in principle |  |
| OPPO | Support. |  |
| Nokia/NSB | Fine with the proposal |  |
| CATT |  | Support. |
| Huawei, HiSilicon | Support in principle. |  |
| Lenovo | Support. |  |
| Intel | OK |  |
| QC | We don’t prefer the proposal, which map one CW’s layers to two CDM groups. We still prefer the other design which map one CW’s layers to one CDM group, which can simplify gNB receiver. We think we can defer the decision on this proposal after we decide whether/how to confirm the WA for 2CWs type 1 1-symbol DMRS. |  |
| ZTE |  | Support. This is in line with the same rule as we elaborated in section 2.1. Again, we fail to see the logic that any restriction over the legacy is needed, which is out of scope from our perspective. |
| Ericsson | We don’t support the proposal. For PUSCH there’s no MU-MIMO restriction as PDSCH, only reuse the PDSCH will limit the advantage of Rel-18 DMRS design with double orthogonal ports. More ports combinations for PUSCH shall be supported to increase the MU-MIMO capacity in uplink. To allow Rel-15 UE being co-scheduled using legacy port 0,1,2,3, we propose to Add port combination (3,8,9,10,11) and/or (0,1,8,10,11) into the rank 5 table, Add port combination (2,3,8,9,10,11) into the rank 6 table, Add port combination (1,2,3,8,9,10,11) into rank 7 table. |  |
| Sharp | Support |  |
| Apple | Fine |  |
| New H3C | OK |  |
| China Telecom | Support. |  |
| Samsung | Support. |  |
| Spreadtrum | Support |  |
| CMCC | Support |  |
| LGE | Support. |  |
| vivo | Support |  |

3.1.2.2 eType1, maxLength2 (discuss later)

FL: Since DMRS ports combinations for PDSCH is not decided yet, we will discuss this later.

Table 7.3.1.1.2-15-X-1: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1 | 2 | 0,1,2,3,8 | 1 |
| 2 | 1 | 0,1,4,5,8 | 2 |
| 3 | 2 | 0,1,4,5,8 | 2 |
| 4 | 2 | 2,3,6,7,10 | 2 |
| 5-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15-X-2: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1 | 2 | 0,1,2,3,8,10 | 1 |
| 2 | 1 | 0,1,4,5,8,12 | 2 |
| 3 | 2 | 0,1,4,5,8,12 | 2 |
| 4 | 2 | 2,3,6,7,10,14 | 2 |
| 5-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15-X-3: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1 | 2 | 0,1,2,3,8,9,10 | 1 |
| 2 | 1 | 0,1,4,5,8,9,12 | 2 |
| 3 | 2 | 0,1,4,5,8,9,12 | 2 |
| 4 | 2 | 2,3,6,7,10,11,14 | 2 |
| 5-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15-X-4: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 2 | 1 | 0,1,4,5,8,9,12,13 | 2 |
| 3 | 2 | 0,1,4,5,8,9,12,13 | 2 |
| 4 | 2 | 2,3,6,7,10,11,14,15 | 2 |
| 5-31 | Reserved | Reserved | Reserved |

3.1.2.3 eType2, maxLength1 (discuss later)

FL: Since DMRS ports combinations for PDSCH is not decided yet, we will discuss this later.

Table 7.3.1.1.2-19-X-1: Antenna port(s), transform precoder is disabled, dmrs-Type=eType2, maxLength=1, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1 | 3 | 12-16 |
| 2 | 2 | 0,1,2,3,12 |
| 3 | 3 | 0,1,2,3,12 |
| 4-31 | Reserved | Reserved |

Table 7.3.1.1.2-19-X-2: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1 | 3 | 12-17 |
| 2 | 2 | 0,1,2,3,12,14 |
| 3 | 3 | 0,1,2,3,12,14 |
| 4-31 | Reserved | Reserved |

Table 7.3.1.1.2-19-X-3: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3,12-14 |
| 1 | 3 | 0-3,12-14 |
| 2-31 | Reserved | Reserved |

Table 7.3.1.1.2-19-X-4: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3,12-15 |
| 1 | 3 | 0-3,12-15 |
| 2-31 | Reserved | Reserved |

3.1.2.4 eType2, maxLength2 (discuss later)

FL: Since DMRS ports combinations for PDSCH is not decided yet, we will discuss this later.

Table 7.3.1.1.2-23-X-1: Antenna port(s), transform precoder is disabled, dmrs-Type=eType2, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 2 | 2 | 0,1,2,3,12 | 1 |
| 3 | 3 | 0,1,2,3,12 | 1 |
| 4 | 1 | 0,1,6,7,12 | 2 |
| 5 | 2 | 0,1,6,7,12 | 2 |
| 6 | 2 | 2,3,8,9,14 | 2 |
| 7 | 3 | 0,1,6,7,12 | 2 |
| 8 | 3 | 2,3,8,9,14 | 2 |
| 9 | 3 | 4,5,10,11,16 | 2 |
| 10-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23-X-2: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2 | 2 | 0-3,12,14 | 1 |
| 3 | 3 | 0-3,12,14 | 1 |
| 4 | 1 | 0,1,6,7,12,18 | 2 |
| 5 | 2 | 0,1,6,7,12,18 | 2 |
| 6 | 2 | 2,3,8,9,14,20 | 2 |
| 7 | 3 | 0,1,6,7,12,18 | 2 |
| 8 | 3 | 2,3,8,9,14,20 | 2 |
| 9 | 3 | 4,5,10,11,16,22 | 2 |
| 10-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23-X-3: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1 | 2 | 0-3,12-14 | 1 |
| 2 | 3 | 0-3,12-14 | 1 |
| 3 | 1 | 0,1,6,7,12,13,18 | 2 |
| 4 | 2 | 0,1,6,7,12,13,18 | 2 |
| 5 | 2 | 2,3,8,9,14,15,20 | 2 |
| 6 | 3 | 0,1,6,7,12,13,18 | 2 |
| 7 | 3 | 2,3,8,9,14,15,20 | 2 |
| 8 | 3 | 4,5,10,11,16,17,22 | 2 |
| 9-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23-X-4: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1 | 2 | 0-3,12-15 | 1 |
| 2 | 3 | 0-3,12-15 | 1 |
| 3 | 1 | 0,1,6,7,12,13,18,19 | 2 |
| 4 | 2 | 0,1,6,7,12,13,18,19 | 2 |
| 5 | 2 | 2,3,8,9,14,15,20,21 | 2 |
| 6 | 3 | 0,1,6,7,12,13,18,19 | 2 |
| 7 | 3 | 2,3,8,9,14,15,20,21 | 2 |
| 8 | 3 | 4,5,10,11,16,17,22,23 | 2 |
| 9-63 | Reserved | Reserved | Reserved |

  - Max number of PTRS ports

We discussed the following proposal since RAN1#110bis-e, however, there was no consensus.

FL proposal#3.2A:

- For 8Tx PUSCH, support up to 4 ports PTRS for CP-OFDM.

Support/fine: InterDigital, CATT, Lenovo, Apple, Qualcomm, Xiaomi, LGE Docomo, MediaTek, CMCC, Ericsson

No: Samsung, OPPO, Nokia/NSB, vivo, Spreadtrum,

If the above proposal is not acceptable, following will be the consequence. It means 2-port PTRS in the current spec.is reused.

FL proposal#3.2B: (for conclusion)

- For 8Tx PUSCH, no consensus to support up to 4 ports PTRS for CP-OFDM.

Support/fine: ZTE, Spreadtrum, OPPO, vivo, Samsung, Nokia/NSB, Docomo,

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support FL Proposal 3.2A, because Ng (antenna coherent groups) was agreed with 1, 2, 4. If different antenna groups do not share the same PA, different phase noise would be observed for different antenna groups. Hence, each DMRS port(s) should be associated with one PTRS port, and the total number of PTRS ports should be up to 4. But, considering the situation, and 8 PTRS ports will be not used for 8Tx non-coherent codebook, we can accept FL proposal#3.2B. |
| Google | Support 3.2B. Ng does not mean number of panels. Currently only 2 panels are supported for STxMP. |
| InterDigital | Support Proposal 3.2A. To properly support Ng=4, that may represent antenna units pointed to four different directions, 4 PTRS ports should be supported. |
| OPPO | Support Proposal 3.2B. We don’t think we need N PTRS ports for N antenna groups. |
| Nokia/NSB | Support the proposal. 4 PTRS require high overhead but no clear gain. (we can come back if we support simultaneous TX to 4 TRPs in FR2) |
| Lenovo | Support FL Proposal 3.2A. |
| QC | We support FL Proposal 3.2A, based on the reason as DOCOMO mentioned. And we don’t think it is feasible for antenna groups to share a same PA/Oscillator. We don’t agree with FL Proposal 3.2B, which effectively excluding Ng=4 for 8 Tx in Rel-18. If we have to take the conclusion due to controversial views, we request to add a note to the conclusion. Updated FL proposal#3.2B: (for conclusion) - For 8Tx PUSCH, no consensus to support up to 4 ports PTRS for CP-OFDM. Note: This conclusion effectively excludes the support of Ng=4 for 8 Tx PUSCH with the use of PTRS. |
| MediaTek | Proposal 3.2A: Support |
| ZTE | Support FL’s proposal#3.2B. Note that 8Tx UL aims for PUSCH transmission in FR1, we fail to see the motivation of supporting 4 PTRS ports in Rel-18. Subsequently, we do not agree the Note suggested by QC. |
| Ericsson | Support proposal #3.2A. |
| Sharp | Support FL proposal 3.2B. |
| Xiaomi | Support FL Proposal 3.2A. |
| Apple | Proposal 3.2A: Support. As discussed earlier, if we support up to 8 Tx UL partial-coherent operation over 4 port groups (panels) with up to 8 layers, we need 4 port PTRS |
| New H3C | Support FL proposal 3.2B. |
| China Telecom | Support FL proposal 3.2B. |
| Samsung | Support FL proposal#3.2B. Supporting 4 PTRS ports may have bad effect on UL throughput which is the key motivation to have up to 8 layers. And also, up to 8-layer would be appropriate on FR1 which PTRS is optionally used. Hence, we think increasing the maximum number of PTRS ports is not needed. |
| Spreadtrum | Support proposal#3.2B. The note suggested by QC is not always true. |
| CMCC | Support FL proposal#3.2A. If four antenna groups are assumed for UE’s antenna layouts, then maximal PT-RS ports to be configured may be 4. |
| LGE | Support FL Proposal 3.2A, For 8 Tx antenna ports, there can be no coherence at all among 8 antenna ports or 4 pairs of coherent antenna ports can be considered depending on antenna implementation. In this case, 2 PT-RS ports may not be sufficient for phase noise estimation from more than two phase noise sources. |
| vivo | Support proposal#3.2B. |

  - PTRS-DMRS association

One port PTRS

In RAN1#112, we made the following agreement.

Agreement - For full-coherent PUSCH with rank 5-8 with one port PTRS, support Alt.1 in the RAN1#111 agreement with the following update - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2. - FFS: Association with The CW with the higher MCS is selected in case of two CWs. - If the MCS is the same for two CWs, the PTRS port is associated with the first CW. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW with the higher MCS. 1; 2nd scheduled DMRS port the CW with the higher MCS. 2; 3rd scheduled DMRS port the CW with the higher MCS. 3; 4th scheduled DMRS port the CW with the higher MCS.

ZTE [4] points out there is a case that one port PTRS is configured for partial/non-coherent PUSCH. ZTE’s proposal is to use 3-bit PTRS-DMRS association for one port PTRS for partial/non-coherent PUSCH. However, considering that 2-bit PTRS-DMRS association is agreed for full coherent codebook, from FL perspective, it is more reasonable to reuse the agreement of 2-bit PTRS-DMRS association to 1 port PTRS for partial/non-coherent PUSCH as well.

FL note: However, it seems the case of one port PTRS for partial/non-coherent codebook is missing in TS38.214?

6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled (TS38.214) The maximum number of configured PT-RS ports is given by the higher layer parameter maxNrofPorts in PTRS-UplinkConfig. The UE is not expected to be configured with a larger number of UL PT-RS ports than it has reported need for. […] For partial-coherent and non-coherent codebook-based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI(s) and/or number of layers which are indicated by 'Precoding information and number of layers' field(s) in DCI format 0_1 and DCI format 0_2 or configured by higher layer parameter precodingAndNumberOfLayers: - if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI(s) as: - PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1 and DCI format 0_2 described in Clause 7.3.1 of [5, TS38.212].

FL proposal#3.3A: (one port PTRS for partial/non-coherent PUSCH)

- For partial/non-coherent PUSCH, if one PTRS ports is configured, PTRS-DMRS association for PUSCH with up to 8 layers is the following.

  - The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

    - The CW with the higher [MCS or SE] is selected in case of two CWs.

    - If the [MCS or SE] is the same for two CWs, the PTRS port is associated with the first CW.

Table 7.3.1.1.2-25: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port with the CW |
| 1 | 2nd scheduled DMRS port with the CW |
| 2 | 3rd scheduled DMRS port with the CW |
| 3 | 4th scheduled DMRS port with the CW |

FL proposal#3.3A2: (one port PTRS for partial/non-coherent PUSCH)

- For partial/non-coherent PUSCH, if one PTRS ports is configured, PTRS-DMRS association for PUSCH with up to 8 layers is the following.

  - The size of PTRS-DMRS association field is 3-bit in DCI format 0_1/0_2.

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port |
| 4 | 5th scheduled DMRS port |
| 5 | 6th scheduled DMRS port |
| 6 | 7th scheduled DMRS port |
| 7 | 8th scheduled DMRS port |

Two port PTRS

For 2 PTRS ports for partial/non-coherent PUSCH, 3 options can be discussed. For Alt.3, there are at least unused 2-bit of antenna ports field for rank 5-8 based on sect. 3.1.1 and 3.1.2.

FL note: The following is only applied to 2-port PTRS. 4-port PTRS (if supported) can be discussed separately.

FL proposal#3.3B: (two port PTRS for partial/non-coherent PUSCH)

- For two PTRS ports for partial/non-coherent PUSCH, PTRS-DMRS association for PUSCH with up to 8 layers is down selected from the following.

  - Alt.1: The size of PTRS-DMRS association field is 4-bit in DCI format 0_1/0_2.

Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

  - Alt.2: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

    - The CW with the higher MCS is selected in case of two CWs.

    - If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

  - Alt.3: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

    - For PUSCH with rank 5-8, 2-bit of antenna ports field is reused in addition to 2-bit PTRS-DMRS association in DCI format 0_1/0_2, and total 4-bit is used for PTRS-DMRS association.

Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

  - Alt.4: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

| Company | Comment |
| --- | --- |
| Docomo | FL proposal#3.3A: If one port PTRS for partial/non-coherent PUSCH is supported in R17, we are fine. FL proposal#3.3B: Support Alt.1. |
| Google | 3.3A: As we discussed in our contribution, the higher MCS does not mean higher SE, as there are some reserved MCS as follows. So, we think the “higher MCS” should be replaced by “MCS with higher SE”. For reserved MCS, the SE can be calculated based on the TB size and allocated REs. 3.3B: We suggest adding Alt4 as follows. If the DCI overhead is a concern, we can consider partial indication. - Alt.4: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. |
| OPPO | FL proposal#3.3A: Support. FL proposal#3.3B: We support to add Alt.4 as Google proposed. We think there would be some issues with Alt.2. Depended on the codebook design, it is possible that two CWs are mapped to different antenna groups, and associated with two different PTRS ports. In this case, “The CW with the higher MCS is selected in case of two CWs.” cannot work at all. Alt.4 is a better solution. |
| Nokia/NSB | FL proposal#3.3A: Share view with DOCOMO FL proposal#3.3B: prefer Alt.4 but also fine with Alt 1. |
| CATT | FL proposal#3.3A: Support. FL proposal#3.3B: Support Alt.3. For PUSCH transmission with rank = 5, 6, 7, 8, there is only one or two DMRS port combinations for each rank. Since the bitwidth of Antenna port filed in DCI format 0_1/0_2 is 3 to 5 bits, 2-bits of the Antenna port filed can be used to indicate the mapping of PTRS ports and DMRS ports, and the overhead of DCI would not be increased. |
| Huawei, HiSilicon | FL Proposal 3.3A: Support. FL Proposal 3.3B: In order to harvest similar overhead reduction benefit to one-port-PTRS case, the DCI overhead of PTRS-DMRS association should remains 2 bit. Detailed design can be discussed after TPMI is decided in 9.1.4.2. |
| Lenovo | FL proposal#3.3A: Support. FL proposal#3.3B: Support Alt.1. |
| QC | FL proposal#3.3A: We are fine with the proposal. FL proposal#3.3B: Support Alt.1. Alt 2 does not support the scenario where one PTRS port for CW1 and another PTRS port for CW2. With Alt 2, both PTRS ports are used for a same CW, which seems a problem/restriction. By the way, Alt 3 needs some clarification. Does it mean the size of PTRS-DMRS association varies with rank? If so, it seems cause dynamic DCI size which does not work. |
| MediaTek | FL proposal#3.3A: Support. FL proposal#3.3B: We prefer Alt.1. |
| ZTE | FL proposal#3.3A: Do Not support. For partial/non-coherent codebook and also non-codebook based transmission, if the max number of PTRS port is configured as one, then one out of 8 DMRS ports is associated to this one PTRS. Taking rank = 8 as an example, if the antenna port group is configured as 2, one DMRS port is associated with only one antenna port group, the DMRS port with the CW of higher MCS may not be associated with antenna port group with heavier phase noise. To guarantee the PTRS port is associated with the DMRS port with heavier phase noise, 3-bit indication should be used when one DMRS port is used. FL proposal#3.3B: Support Alt 1. |
| Sharp | FL proposal 3.3A: Support FL proposal 3.3B: Support Alt 1. |
| Xiaomi | FL proposal#3.3A: Fine with the proposal FL proposal#3.3B: Support Alt.1. |
| Apple | FL proposal#3.3A: Support. FL proposal#3.3B: Prefer Alt.1. |
| New H3C | FL proposal#3.3A: Support. FL proposal#3.3B: Prefer Alt.1. |
| China Telecom | FL proposal 3.3A: Not support. We share the similar view as ZTE that 1 more bit for indicating CW is needed to ensure the performance. FL proposal 3.3B: Support Alt 1. |
| Samsung | FL proposal#3.3A: We are fine with the proposal. FL proposal#3.3B: We are open to discuss and prefer Alt2 to have same DCI overhead comparing with 1-port PTRS. We are fine with discussing after finalizing TPMI design from AI 9.1.4.2. |
| Spreadtrum | FL proposal#3.3A: Support. FL proposal#3.3B: We prefer Alt.1 to achieve full signaling flexibility. |
| CMCC | FL proposal#3.3B: Support Alt.1. For codebook-based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI and/or number of layers as: PUSCH antenna port 1000 and 1004 in indicated TPMI share PT-RS port 0, and PUSCH antenna port 1001 and 1005 in indicated TPMI share PT-RS port 1, PUSCH antenna port 1002 and 1006 in indicated TPMI share PT-RS port 2, and PUSCH antenna port 1003 and 1007 in indicated TPMI share PT-RS port 3. Then, 4 bits of PTRS-DMRS association field is needed to indicate the association between PT-RS port and DMRS port pair. |
| LGE | FL proposal#3.3A: Support. FL proposal#3.3B: Support Alt.1. |
| vivo | FL proposal#3.3A: Support. FL proposal#3.3B: We prefer Alt 2 with less overhead. This proposal is associated with the design of partial/non-coherent codebook. Since the codebook is not determined yet, we suggest postponing this proposal before the outcome of AI 9.1.4.2. |
| FL | For FL proposal#3.3A, I added FL proposal#3.3A2 (3-bit PTRS-DMRS association). I’d like to ask companies whether to support FL proposal#3.3A. Also, please comment to Google’s proposal of changing “higher MCS” to “MCS with higher SE” |
| ZTE2 | In case of one port PTRS for partial/non-coherent PUSCH Support FL proposal#3.3A. On proposal#3.3A, it relies on the relationship between CW with higher MSC and antenna port/port group with heavier phase noise when PUSCH with rank > 4. We evaluate simulation for 6 and 8 layers transmission of two CWs (more details can be found in our tdoc R1-2302419). As a result, it proves that the PTRS ports will not always associated with the antenna port group with heavier phase noise. Hence the principle of proposal#3.3A cannot be workable for this case. By comparison, it is very clear proposal#3,3A2 can be used anyways with very great flexibility, and it won’t cause crosstalk with the final design of 8Tx partial/non-coherent codebook in the ongoing discussion of AI 9.1.4.2. In particular, we fail to see the motivation to save 1-bit but sacrifice the schedule flexibility. In case of two port PTRS for partial/non-coherent PUSCH Support either Alt 1 or Alt 3. According to our simulation result and elaboration above, there is no relationship between CW with higher MSC and antenna port/port group with heavier phase in this case noise neither. Besides, it can be noted that even though without the outcome of 8Tx partial/non-coherent codebook in AI 9.1.4.2, either Alt 1 or Alt 3 can be workable. |
| QC3 | We prefer FL proposal#3.3A2 (3-bit PTRS-DMRS association). We think it is fine to use higher MCS, which is the behavior in current spec anyway. For FL proposal#3.3B, besides our comment to Alt 1/2/3, for the newly added Alt 4, we don’t support it as it restricts PTRS only can be associated to 4 out of the 8 DMRS ports. |

  - PTRS power boosting

InterDigital, Huawei/HiSilicon, ZTE, vivo, OPPO, Spreadtrum, CATT, Lenovo, Ericsson, and Google discuss PTRS power boosting for PUSCH with rank>4 in their tdocs. Most of companies seems ok with FL proposal 3.4A in RAN1#112, but it was pointed out that non codebook scenario was missing. Hence, the case of con-codebook is added.

FL proposal#3.4A:

- For 8Tx PUSCH, specify PUSCH to PTRS power ratio per layer per RE () based on the following principles.

  - Principle 1: When the ptrs-Power configures 01, the PTRS to PUSCH power ratio is 10log10(L), where L is the total number of PUSCH layers.

  - Principle 2: When the ptrs-Power configures 00, the PTRS to PUSCH power ratio is determined as the following

    - Principle 2.1: For fully coherent TPMIs, PTRS to PUSCH power ratio is 10log10(L), where L is the total number of PUSCH layers.

    - Principle 2.2: For non-coherent TPMIs, PTRS to PUSCH power ratio is 10log10(Qp), where Qp is the number of PTRS ports configured to the UE.

    - Principle 2.3: For non-codebook PUSCH, PTRS to PUSCH power ratio is 10log10(Qp), where Qp is the number of PTRS ports configured to the UE.

    - FFS: The PTRS to PUSCH power ratio for partial coherent TPMIs

  - Send LS to RAN4 to inform that RAN1 made the above agreement and ask if any impact to UL MIMO requirements in RAN 4 (such as PAPR, MPR, EVM, intermod, etc.) when the power of Tx port transmitting PTRS on PTRS RE is 12 dB boosted over the power of the same Tx port on PUSCH REs for 8 layer PUSCH.

| Company | Comment |
| --- | --- |
| Docomo | Support. |
| Google | We think we can make the proposal more general with regard to the upper bound. We think the EPRE ratio should not exceed 9dB with regard to the ICI similar to LTE. But we could ask for RAN4’s advice. - For 8Tx PUSCH, specify PUSCH to PTRS power ratio per layer per RE () based on the following principles. - Principle 1: When the ptrs-Power configures 01, the PTRS to PUSCH power ratio is min(10log10(L), T), where L is the total number of PUSCH layers. - Principle 2: When the ptrs-Power configures 00, the PTRS to PUSCH power ratio is determined as the following - Principle 2.1: For fully coherent TPMIs, PTRS to PUSCH power ratio is min(10log10(L), T), where L is the total number of PUSCH layers. - Principle 2.2: For non-coherent TPMIs, PTRS to PUSCH power ratio is min(10log10(Qp), T), where Qp is the number of PTRS ports configured to the UE. - Principle 2.3: For non-codebook PUSCH, PTRS to PUSCH power ratio is min(10log10(Qp), T), where Qp is the number of PTRS ports configured to the UE. - FFS: The PTRS to PUSCH power ratio for partial coherent TPMIs - Send LS to RAN4 to ask for their advice on the value of T |
| InterDigital | Support |
| OPPO | Fine with the proposal. We don’t think T as proposed by google is necessary. It depends on the output of RAN4. |
| Nokia/NSB | Support the original proposal. |
| CATT | Support FL proposal#3.4A. For partial-coherent TPMIs, when the ptrs-Power configures 00, the PTRS to PUSCH power ratio can be 10log10minLxQp,L dB, where Lx is the number of PUSCH layers sharing the same non-zero power antenna ports as the PUSCH layer the PTRS port is associated to, Qp is the number of PTRS ports configured to the UE and L is the total number of PUSCH layers. |
| Huawei, HiSilicon | Support in general. The definition of Qp should be modified from “the number of PTRS ports configured to the UE” to “the number of scheduled PTRS ports”. |
| Lenovo | Support the original proposal. |
| QC | Support the FL proposal, except the last bullet on LS may need some update on wording. Regarding this part “and ask if any impact to PAPR when PTRS RE is 12 dB boosting over the PUSCH REs for L=8”, we assume it means the PAPR of the port where PTRS is transmitted. We actually don’t have strong concern that this power boost will increase PAPR. But we are OK to ask RAN4 to confirm. Also, besides PAPR, the power boost on the Tx port where PTRS is transmitted might impact other RAN4 requirements such as MPR, intermod, EVM, etc. If we send LS, we might need to ask a question on generic RAN4 impact rather than just PAPR. With the above, suggest the following wording for the LS related. Send LS to RAN4 to inform that RAN1 made the above agreement and ask if any impact to UL MIMO requirements in RAN 4 (such as PAPR, MPR, EVM, intermod, etc) when the power of Tx port transmitting PTRS on PTRS RE is 12 dB boosting boosted over the power of the same Tx port on PUSCH REs for 8 layer PUSCH L=8. FL: Thank you. I reflected. Regarding the cap of T that google proposed, we are not sure RAN1 spec need to capture RAN4 requirements. But we are open to discuss if there is really any issue with the cap. Maybe we can also add that cap issue into the LS. |
| MediaTek | Support |
| ZTE | Support FL’s proposal#3.4A. |
| Ericsson | Support |
| Xiaomi | Support in principle |
| Apple | Support |
| New H3C | OK |
| China Telecom | Support |
| Spreadtrum | Support |
| LGE | Support |
| vivo | Support |
| FL | FL Proposal 3.4A is moved to [112bis-e-R18-MIMO-04] DMRS - EMAIL ENDORSEMENT 1. |

  - Other proposals

Following proposals are also proposed.

| Proposals | Companies |
| --- | --- |
| - UE indicates the number of PTRS ports associated to Ng as part of its capability | IDC |
| - For PTRS-DMRS association, if MCS is reserved MCS, the associated DMRS port is based on the MCS with highest SE. | Google |

Please provide your views on the above proposals, or other aspects which are not included in the summary, if any.

| Company | Comment |
| --- | --- |
| Nokia/NSB (RAN1#112) | One more issue we have to resolve is, for Rel-15 DMRS, we cannot use PT-RS with TD-OCC. For Rel-18, we can further discuss on the options. TS38.214 If a UE transmitting PUSCH scheduled by DCI format 0_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 or dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2, or a UE transmitting PUSCH scheduled by DCI format 0_0 or DCI format 0_1 is configured with the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB, the UE may assume that the following configurations are not occurring simultaneously for the transmitted PUSCH - any DM-RS ports among 4-7 or 6-11 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and PT-RS is transmitted from the UE. |
| Google | For 2), as we mentioned previously, the higher MCS does not mean higher SE, as there are some reserved MCS as follows. So, we think the “higher MCS” should be replaced by “MCS with higher SE”. For reserved MCS, the SE can be calculated based on the TB size and allocated REs. Based on current agreements, a worst case is that there might be no PT-RS if the gNB indicates MCS=29 for one CW, but MCS=27 for the second CW. |

- Conclusion

Following FL proposals are made.

To be updated:

References

| [1] | R1-2302302 | Remaining Details on DMRS Enhancements | InterDigital, Inc. |
| --- | --- | --- | --- |
| [2] | R1-2302313 | On increasing the number of orthogonal DM-RS ports for MU-MIMO | FUTUREWEI |
| [3] | R1-2302373 | Discussion on DMRS enhancements in Rel-18 | Huawei, HiSilicon |
| [4] | R1-2302419 | DMRS enhancement for UL/DL MU-MIMO and 8 Tx UL SU-MIMO | ZTE, China Telecom |
| [5] | R1-2302428 | Discussions on increased number of orthogonal DMRS ports | New H3C Technologies Co., Ltd. |
| [6] | R1-2302472 | Further discussion on DMRS enhancements | vivo |
| [7] | R1-2302535 | DMRS enhancement for Rel-18 MIMO | OPPO |
| [8] | R1-2302588 | Discussion on increased number of orthogonal DMRS ports | Spreadtrum Communications |
| [9] | R1-2302634 | On increased number of orthogonal DMRS ports | Fraunhofer IIS |
| [10] | R1-2302683 | DMRS enhancements in Rel-18 | CATT |
| [11] | R1-2302726 | Discussion of increased number of orthogonal DMRS ports | Lenovo |
| [12] | R1-2302767 | On increased number of orthogonal DMRS ports for MU-MIMO and 8 Tx UL SU-MIMO | Ericsson |
| [13] | R1-2302783 | DMRS Enhancements for Rel-18 NR | Intel Corporation |
| [14] | R1-2302962 | Discussion on DMRS enhancement | Xiaomi |
| [15] | R1-2303008 | Rel-18 UL and DL DMRS Enhancements | Nokia, Nokia Shanghai Bell |
| [16] | R1-2303045 | On DMRS Enhancement | Google |
| [17] | R1-2303071 | Increased number of orthogonal DMRS ports | LG Electronics |
| [18] | R1-2303115 | Views on DMRS enhancements | Samsung |
| [19] | R1-2303180 | Increased number of orthogonal DMRS ports | Sharp |
| [20] | R1-2303219 | Discussion on increased number of orthogonal DMRS ports | CMCC |
| [21] | R1-2303329 | Increased number of orthogonal DMRS ports | MediaTek Inc. |
| [22] | R1-2303470 | Views on supporting increased number of orthogonal DMRS ports | Apple |
| [23] | R1-2303576 | Design for increased number of orthogonal DMRS ports | Qualcomm Incorporated |
| [24] | R1-2303678 | Discussion on increased number of orthogonal DMRS ports | NEC |
| [25] | R1-2303700 | Discussion on DMRS enhancements | NTT DOCOMO, INC. |

Appendix

RAN1#109e agreements:

EVM Agreement - LLS is used for objective #3 (increasing DMRS ports for MU-MIMO) in Rel.18 MIMO, while SLS can be used optionally. Agreement - No EVM discussion is needed for objective #5 (>4 layers PUSCH DMRS) in AI 9.1.3.1 (DMRS) in Rel.18. Agreement - LLS for increasing DMRS ports in AI 9.1.3.1 in Rel.18: - Evaluated channel: PDSCH as baseline (Companies can additionally submit evaluation results of PUSCH). - Evaluation metric: - BLER for fixed MCS and rank as baseline - User throughput for adaptive MCS and rank as optional - MSE or NMSE of DMRS as optional - Evaluation baseline (i.e. compared with): - For evaluation of enhanced single-symbol DMRS, baseline refers to Rel.15 single-symbol DMRS or Rel.15 double-symbol DMRS. - For evaluation of enhanced double-symbol DMRS, baseline refers to Rel.15 double-symbol DMRS. Agreement - Following evaluation assumptions are used for LLS for increasing DMRS ports in AI 9.1.3.1 in Rel.18. Parameter; Value. Duplex, Waveform; TDD, OFDM Note: FDD, OFDM is not precluded. Carrier Frequency; 4 GHz. Subcarrier spacing; 30kHz. Channel Model; CDL-B or CDL-C in TR 38.901 with 30ns or 300ns delay spread as baseline for MU-MIMO and SU-MIMO Note: Other delay spread is not precluded. Note: Simulation using TDL-A with 30ns or 300ns for MU-MIMO is not precluded.. Delay spread; Baseline: 30ns, 300ns Optional: 1000ns. UE velocity; Baseline: 3km/h, 30km/h Optional: 60km/h, 120km/h. Allocation bandwidth; 20MHz Note: Other bandwidth smaller than 20MHz is not precluded. MIMO scheme; Baseline: MU-MIMO Optional: SU-MIMO. BS antenna configuration; Companies can select and need to report which option(s) are used between - 32 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded.. UE antenna configuration; Companies can select and need to report which option(s) are used between 4RX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded.. MIMO Rank; 1, 2, or 4 per UE (rank fixed or rank adaptation). UE number for MU-MIMO; 1, 2, 4, 8, or 12. Precoding and precoding granularity; For PDSCH: Companies can select and need to report which option(s) are used between - [ZF or SVD] based sub-band precoding (with 4PRB precoding granularity) on ideal channel knowledge - CSI codebook based sub-band precoding (with 4PRB precoding granularity) on ideal CSI feedback. For PUSCH: Companies can select and need to report which option(s) are used between - [ZF or SVD] based wide-band precoding on ideal channel knowledge - Codebook based wide-band precoding on ideal CSI feedback.. Feedback delay for precoding; 5ms. DMRS type; Type 1E and/or Type 2E, which are enhanced DMRS that are based on the legacy RE mappings of DMRS Type 1/2, where the enhanced DMRS support larger DMRS ports. Note: The terminology of Type 1E and/or Type 2E is for discussion purpose.. DMRS configurations; Baseline: - Single symbol DMRS without additional DMRS symbols and 1 additional DMRS symbol - Double symbol DMRS without additional DMRS symbols. Note: evaluation of other additional DMRS symbol(s) are not precluded.. DMRS mapping type; Mapping type A (slot based) for PDSCH. Mapping type A (slot based) for PUSCH.. Link adaptation; - Fixed modulation, coding and rank for BLER evaluation as baseline. - Adaptation of both MCS and rank for throughput evaluation as optional.. HARQ; Baseline: Off Optional: On (HARQ with max. 4 re-transmissions) for throughput evaluation. Channel estimation; Realistic channel estimation with ideal info of frequency sync, SNR, doppler and delay spread. Receiver type; MMSE as baseline. EVM; No radio impairments. Agreement - For LLS assumptions for increasing DMRS ports in AI 9.1.3.1 in Rel.18: - Precoding assumption of PUSCH, “[ZF or SVD]” in RAN1#109e agreement is updated by - Alt.2-2: SVD Agreement For LLS assumptions for increasing DMRS ports in AI 9.1.3.1 in Rel.18: - Precoding assumption of PDSCH, “[ZF or SVD]” in RAN1#109e agreement is updated by SVD. Agreement - For MU-MIMO LLS of PDSCH, for evaluation of SVD/CSI-codebook based sub-band precoding, companies shall report the pre-coding assumption of interference of co-scheduled UEs from the following: - Alt.1: calculated by pre-coder of channel of each co-scheduled UE. - For precoding assumption of PDSCH, precoder of target UE and precoder of co-scheduled UE are generated independently. - Companies can report a set of azimuth and zenith angle offset used for evaluation (For example, azimuth angle offsets from [30 o, 60 o, 90 o] and zenith angle offset from [3o, 6o] can be considered). - Alt.2: calculated by random pre-coder (i.e. precoder selected randomly from a predefined set of precoders) which is different from the pre-coder of target UE. - For precoding assumption of PDSCH, only the channel of one target UE, i.e. Hd, needs to be modelled. Precoder is generated based on Hd to obtain the precoder for this UE only. The interference from co-scheduled UEs can be modelled as, , wherein Wi can be randomly selected from a predefined set of precoders - Companies shall report how to generate the predefined set of precoders for simulation. - Alt.3: the same pre-coder as scheduled UE. - PDSCH interference and interfering DMRS ports are emulated using the same pre-coder as for the scheduled UE. - Power offset of the co-scheduled UE is one value from {0dB, -3dB, -6dB} as fixed evaluation parameter. Other values are not precluded. - For precoding assumption of PDSCH, only the channel of one target UE, i.e. Hd, needs to be modelled. Precoder for the target UE (denoted as Wd) is generated based on Hd only. Denote the precoding matrix/vector of the ith co-scheduled UEs as Wi, and Wi=Wd (Wi for all th co-scheduled UEs are same). Then the interference from co-scheduled UEs can be modelled as .​ For the above Alt.1-3, only PDSCH performance of the target UE is evaluated, while interference of both PDSCH and DMRS of co-scheduled UE(s) is simulated. Agreement - For SLS assumption for increasing DMRS ports in AI 9.1.3.1 in Rel.18, - Scenario: Dense Urban (Macro only) at 4GHz is a baseline. Other scenarios (e.g. Umi, Uma) are not precluded. - Following evaluation assumptions are used for SLS. Parameter; Value. Scenario; Dense Urban (macro only). Carrier frequency; 4GHz. Duplex, Waveform; TDD, OFDM Note: FDD, OFDM is not precluded. Multiple access; OFDMA. Frequency Range; FR1 only.. Inter-BS distance; 200 m. Channel model; According to the TR 38.901. Antenna setup and port layouts at gNB; Companies need to report which option(s) are used between - 32 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded.. Antenna setup and port layouts at UE; 4RX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configurations are not precluded.. BS Tx power; 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz. BS antenna height; 25 m. BS noise figure; 5 dB. UE noise figure; 9 dB. UE antenna height & gain; Follow TR36.873. Modulation; Up to 256 QAM. Coding on PDSCH; LDPC Max code-block size=8448bit. Numerology; Slot/non-slot; 14 OFDM symbols per slot. SCS; 30 kHz. Simulation bandwidth; 20 MHz. Number of RBs; 52 for 30 kHz SCS. Frame structure; Slot Format 0 (all downlink) for all slots. MIMO scheme; SU/MU-MIMO with rank adaptation is a baseline For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed. MIMO layers; For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12). CSI feedback; Feedback assumption at least for baseline scheme CSI feedback periodicity (full CSI feedback): 5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms. Overhead; Companies shall provide the downlink overhead assumption. Traffic model; Baseline: FTP1 with 50% Resource Utilization Optional: Full buffer. UE distribution; [80%] indoor (3km/h), [20%] outdoor (30km/h). UE receiver; MMSE-IRC as the baseline receiver. Feedback assumption; Realistic. Channel estimation; Realistic. For increasing orthogonal DMRS ports Agreement - Specify to increase the max. number of DMRS ports for PDSCH/PUSCH larger than Rel.15 for CP-OFDM without increasing the DMRS overhead. - Strive to have common design of DMRS enhancement for PDSCH and PUSCH for a given DMRS Type. Agreement - The maximum number of enhanced DMRS ports in Rel.18 is doubled from Rel.15 DMRS ports: - For DMRS type 1, the max. number of enhanced DMRS ports in Rel.18 for PDSCH/PUSCH is - Single symbol DMRS: 8 DMRS ports. - Double symbol DMRS: 16 DMRS ports. - For DMRS type 2, the max. number of enhanced DMRS ports in Rel.18 for PDSCH/PUSCH is - Single symbol DMRS: 12 DMRS ports. - Double symbol DMRS: 24 DMRS ports. Agreement - To increase the number of DMRS ports for PDSCH/PUSCH, evaluate and, if needed, specify one or more from the following options: - Opt.1 (enhance FD-OCC): Introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6). - Study aspect includes potential performance degradation in large delay spread, potential scheduling restriction, backward compatibility. - Opt.2 (enhance TD-OCC): Utilize TD-OCC over non-contiguous DMRS symbols (e.g. TD-OCC across front/additional DMRS symbols) - Study aspect includes potential performance degradation in high UE velocity, potential scheduling restriction (e.g. how to apply freq. hopping), potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - Opt.3 (Sparser frequency allocation): increase the number of CDM groups (e.g. larger number of comb/FDM). - Study aspect includes potential performance degradation in large delay spread, backward compatibility. - Opt.4 (using TDMed DMRS symbol): reusing additional DMRS symbols to increase orthogonal DMRS ports - Study aspect includes potential performance degradation in high UE velocity, potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - Opt.5 TD-OCC over non-contiguous DMRS symbols combined with FD-OCC or FDM: reusing additional DMRS symbol(s) to improve channel estimation performance. - Study aspect includes potential performance degradation in high UE velocity, potential scheduling restriction (e.g. how to apply freq. hopping), potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - The same option can be applied to both single symbol DMRS and double symbol DMRS. Agreement - To increase the max. number of DMRS ports for PDSCH/PUSCH compared to Rel.15 DMRS for CP-OFDM without increasing the DMRS overhead, - Study whether/how to enable MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports, as well as whether/how to enable MU-MIMO among Rel.18 DMRS ports, in the same or different CDM group. Agreement - To increase the max. number of orthogonal DMRS ports for PDSCH/PUSCH larger than Rel.15 - Study whether/how to support DCI-based dynamic antenna ports indication of Rel.18 DMRS ports and/or Rel.15 DMRS ports. - Study whether/how to reuse the antenna port indication table in 38.212 as much as possible for both PDSCH and PUSCH - Study the potential need for MU scheduling restrictions in the design of the enhanced antenna port indication table in 38.212 for DL PDSCH. For 8 Tx UL SU-MIMO Agreement - Study the following potential DMRS enhancement for potential support of more than 4 layers SU-MIMO PUSCH. - Extend DMRS port allocation table for rank 5~8 - Note: DL DMRS table can be a reference - Enhancement for DMRS to PTRS mapping - Study whether to utilize Rel.18 DMRS ports for more than 4 layers SU-MIMO PUSCH. - Note: the above study does not imply more than 4 layers SU-MIMO PUSCH is supported. - Note: other study for potential DMRS enhancement for potential support of more than 4 layers SU-MIMO PUSCH is not precluded.

RAN1#110 agreements:

For increasing orthogonal DMRS ports Working Assumption - To increase the number of DMRS ports for PDSCH/PUSCH, support at least Opt.1 (introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6)). - FFS: FD-OCC length for Rel.18 DMRS type 1 and type 2. - FFS: Whether it is needed to handle potential performance issues of Opt 1. For example, study if there is performance loss in case of large delay spread scenario. If needed, how (e.g. additionally support other options). Agreement - For enhanced FD-OCC length for DMRS of PDSCH/PUSCH, support the following FD-OCC length: - For Rel.18 DMRS type 1, down select from the following in RAN1#110bis-e: - Opt.1-1: Length 6 FD-OCC is applied to 6 REs of DMRS within a PRB within an CDM group - Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group - For Rel.18 DMRS type 2: - Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB within an CDM group - FFS: Support of length 6 FD-OCC Agreement - Support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports. - For MU-MIMO by different CDM groups, no MU-MIMO scheduling restriction of PUSCH/PDSCH (i.e. MU-MIMO between Rel.15 UE and Rel.18 UE is allowed). - For MU-MIMO within a CDM group, study whether and how to support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH. - Note: the study includes MU-MIMO between Rel.15 UE and Rel.18 UE, and between Rel.18 UEs. - Note: PUSCH above is CP-OFDM waveform. Agreement For increased DMRS ports for enhanced FD-OCC, study whether/how to support DCI based switching between DMRS port(s) associated with length 2 FD-OCC and DMRS port(s) associated with length M FD-OCC (where M > 2). For 8 Tx UL SU-MIMO Agreement - For support of more than 4 layers SU-MIMO PUSCH, study the following potential enhancements for PTRS-DMRS association. - Whether to support more than 2-port UL PTRS. - Whether to increase the DCI size of PTRS-DMRS association field in DCI format 0_1/0_2. Agreement For > 4 layers PUSCH, support rank = 5,6,7,8 for both DMRS type 1/2, and for both single-symbol/double-symbol DMRS.

RAN1#110bis-e agreements:

For increasing orthogonal DMRS ports Conclusion - For discussion purpose, definition of Rel.15 DMRS ports and Rel-18 DMRS ports are: - Rel.15 Type 1/Type 2 DMRS ports: DMRS ports with FD-OCC length =2. - Rel.18 eType 1/eType 2 DMRS ports: DMRS ports with FD-OCC length >2. - Following figure as an example shows difference between Rel.15 Type 1 DMRS ports and Rel.18 eType 1 DMRS ports. Agreement Confirm the working assumption in RAN1#110 with the following update: - To increase the number of DMRS ports for PDSCH/PUSCH, support at least Opt.1 (introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6)). - FFS: FD-OCC length for Rel.18 DMRS type 1 and type 2. - FFS: Whether it is needed to handle potential performance issues of Opt 1. For example, study if there is performance loss in case of large delay spread scenario. If needed, how (e.g. additionally support other options). Agreement For enhanced FD-OCC length for DMRS of PDSCH/PUSCH for Rel.18 eType 1 DMRS, support - Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group Agreement For FD-OCC length 4 for DMRS of PDSCH/PUSCH for Rel.18 eType 1/eType 2 DMRS, support one from the following FD-OCCs (to be selected in RAN1#111): - Opt.1-1: Walsh matrix (Hadamard code): FD-OCC index; wf(0); wf(1); wf(2); wf(3). 0; +1; +1; +1; +1. 1; +1; -1; +1; -1. 2; +1; +1; -1; -1. 3; +1; -1; -1; +1. - Opt.1-2: Cyclic shift with {0, π, π/2, 3π/2}: FD-OCC index; wf(0); wf(1); wf(2); wf(3). 0; +1; +1; +1; +1. 1; +1; -1; +1; -1. 2; +1; +j; -1; -j. 3; +1; -j; -1; +j. Agreement For Rel.18 eType 1/eType 2 DMRS ports of PDSCH/PUSCH with FD-OCC length 4, association between DMRS port indexes, CDM group index, FD-OCC index, and TD-OCC index (across consecutive DMRS symbols, if any) are determined by the following Table 1 and Table 2. - The p in Table 1 and Table 2 corresponds to DMRS port index for PUSCH. - DMRS port index for PDSCH is determined by p +1000 in Table 1 and Table 2. Table 1. Rel.18 eType 1 DMRS ports for PUSCH p; CDM group index; FD-OCC index; TD-OCC index. 0; 0; 0; 0. 1; 0; 1; 0. 2; 1; 0; 0. 3; 1; 1; 0. 4; 0; 0; 1. 5; 0; 1; 1. 6; 1; 0; 1. 7; 1; 1; 1. 8; 0; 2; 0. 9; 0; 3; 0. 10; 1; 2; 0. 11; 1; 3; 0. 12; 0; 2; 1. 13; 0; 3; 1. 14; 1; 2; 1. 15; 1; 3; 1. Table 2. Rel.18 eType 2 DMRS ports for PUSCH p; CDM group index; FD-OCC index; TD-OCC index. 0; 0; 0; 0. 1; 0; 1; 0. 2; 1; 0; 0. 3; 1; 1; 0. 4; 2; 0; 0. 5; 2; 1; 0. 6; 0; 0; 1. 7; 0; 1; 1. 8; 1; 0; 1. 9; 1; 1; 1. 10; 2; 0; 1. 11; 2; 1; 1. 12; 0; 2; 0. 13; 0; 3; 0. 14; 1; 2; 0. 15; 1; 3; 0. 16; 2; 2; 0. 17; 2; 3; 0. 18; 0; 2; 1. 19; 0; 3; 1. 20; 1; 2; 1. 21; 1; 3; 1. 22; 2; 2; 1. 23; 2; 3; 1. Agreement For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH, support the following: - Introduce UE capability to report whether UE can be scheduled PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS. - If this capability is not supported by the UE, UE expects that gNB shall apply the scheduling restriction for PDSCH for FD-OCC length 4 in Rel.18 eType 1 DMRS. - The scheduling restriction above means satisfying all of the following at least for other than M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - The number of consecutively scheduled PRBs for PDSCH is even. - The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even. - FFS: Restriction on scheduling of different UEs in case of MU-MIMO. - FFS: Scheduling restriction for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - Note1: Up to UE how to implement DMRS channel estimation. - Note2: No further RAN1 specification enhancement is introduced to handle the orphan REs (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs) for UE that is scheduled PDSCH without the scheduling restriction. - Note 3: Other scheduling restrictions, if identified in future meetings, are not precluded. Conclusion For FD-OCC length 4 in Rel.18 eType 1 DMRS for PUSCH, - No spec. enhancement is needed to handle orphan RE issue (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs), because gNB (receiver) can decide whether the scheduling restriction is needed or not. For 8 Tx UL SU-MIMO Agreement For more than 4 layers SU-MIMO PUSCH, support - Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports. - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports. - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed. - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only.

RAN1#111 agreements:

For increasing orthogonal DMRS ports Agreement For FD-OCC length 4 for PDSCH/PUSCH, select the following: - Opt.1-1 (Walsh matrix) for PDSCH - Opt.1-2 (Cyclic shift) for PUSCH Agreement - For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, all of the following port combinations can be indicated: - Cat. 1) Legacy port indexes (eType 1: p=0~7, eType 2: p=0~11) - Cat. 2) New port indexes (eType 1: p=8~15, eType 2: p=12~23) - Cat. 3) Legacy port indexes and New port indexes at least within a CDM group at least for maxLength=1 (eType 1: up to 4 ports from {0, 1, 8, 9} and/or up to 4 ports from {2, 3, 10, 11}, eType 2: up to 4 ports from {0, 1, 12, 13} and/or up to 4 ports from {2, 3, 14, 15} and/or up to 4 ports from {4, 5, 16, 17}) at least for S-TRP case, - For up to 4 ranks, only one CDM group is used per UE. For larger than 4 ranks, more than one CDM groups can be used per UE. - FFS: Whether to increase the size of antenna ports field in DCI format 1_1/1_2, or introduce new DCI field for antenna ports indication, or not. - FFS: Whether the new antenna port(s) table is specified or not. - FFS: MU restrictions for certain entries. e.g., DMRS ports = {0,2}, or {8,10}, etc. - FFS: Cat.3 for M-TRP case. - Note: DMRS port index for PDSCH is determined by p +1000 Agreement For length 2 TD-OCC (across consecutive DMRS symbols, if any) for DMRS of PDSCH/PUSCH for Rel.18 eType 1/2 DMRS, support Opt.1: - Opt.1: TD-OCC index; Wt(0); Wt(1). 0; +1; +1. 1; +1; -1. Agreement - For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - FFS: Antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for M-TRP case. Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes. 0; [1]; [0]; Cat. 1; [0]; [2]; [0,1,2,3,8]; [Rank 5-8 with one DMRS symbol]. 1; [1]; [1]; [1]; [2]; [0,1,2,3,8,10]. 2; [1]; [0,1]; [2]; [2]; [0,1,2,3,8,9,10]. 3; 2; 0; [3]; [2]; [0,1,2,3,8,9,10,11]. 4; 2; 1. 5; 2; 2. 6; 2; 3. 7; 2; 0,1. 8; 2; 2,3. 9; [2]; [0-2]. 10; [2]; [0-3]. 11; [2]; [0,2]. 12; [1]; [8]; Cat.2. 13; [1]; [9]. 14; [1]; [8,9]. 15; 2; 8. 16; 2; 9. 17; 2; 10. 18; 2; 11. 19; 2; 8,9. 20; 2; 10,11. 21; [2]; [8-10]. 22; [2]; [8-11]. 23; [2]; [8, 10], [9, 11]. 24; [1]; [0,1,8]; Cat.3. 25; [1]; [0,1,8,9]. 26; 2; 0,1,8. 27; 2; 0,1,8,9. 28; 2; 2,3,10. 29; 2; 2,3,10,11. For 8 Tx UL SU-MIMO Agreement - For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS. - For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following: - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook. - For Rel.18 eType1/eType2 DMRS ports, - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details). - Note: Whether the DMRS port combination allows to use single symbol DMRS for rank = 5,6,7,8 should be checked. - FFS: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group. - FFS: One or more than one DMRS port combination(s) for each rank and TPMI - Note: New DMRS port combinations above does not preclude the new antenna ports tables including the current DMRS port combination(s) for PDSCH for rank = 5,6,7,8 in Rel.15-17. - FFS: Whether the antenna ports combinations for rank = 5,6,7,8 can be indicated by the reserved entries of existing antenna ports tables for rank =1,2,3,4, if the rank is indicated together with DMRS antenna ports. Agreement - For full-coherent PUSCH with rank 5-8, UE shall expect only one port PTRS to be configured. - Down select from the following in RAN1#112: - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2. - FFS: Association with the CW with the higher MCS. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW with the higher MCS. 1; 2nd scheduled DMRS port the CW with the higher MCS. 2; 3rd scheduled DMRS port the CW with the higher MCS. 3; 4th scheduled DMRS port the CW with the higher MCS. - Alt.2: The size of PTRS-DMRS association field is 3bit in DCI format 0_1/0_2, and the following PTRS-DMRS association for UL PTRS port 0 is specified in TS38.212. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port. 1; 2nd scheduled DMRS port. 2; 3rd scheduled DMRS port. 3; 4th scheduled DMRS port. 4; 5th scheduled DMRS port. 5; 6th scheduled DMRS port. 6; 7th scheduled DMRS port. 7; 8th scheduled DMRS port.

RAN1#112 agreements:

For increasing orthogonal DMRS ports Agreement - For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support at least support the following rows: - For 1 CW, - 1) Row 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1) Agreement For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, at least support the following rows: - For 1 CW, - 2) Row 9-11 - For the above rows, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). Working Assumption For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs, - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0. Table 4-0: DMRS ports for 2CWs. Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8. 1; 2; 0,1,2,3,8,10. 2; 2; 0,1,2,3,8,9,10. 3; 2; 0,1,2,3,8,9,10,11. Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, support at least the following row(s): - For one CW, support at least row 30 in the following table. - For the above row, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). - FFS: other rows are not precluded Table 7.3.1.2.2-1A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). …; …; …. 30; 2; 0,2,3. Agreement For Rel.18 eType1/eType2 DMRS ports for PDSCH/PUSCH, support Alt.1 for PTRS RE mapping. - Alt 1: Different RE offsets set for different Rel.18 DMRS port indexes as shown in Table 4 Table 4 Different RE offsets set for different Rel.18 DMRS port indexes DM-RS antenna port p (p for PUSCH, p+1000 for PDSCH). DM-RS Configuration type 1; DM-RS Configuration type 2. resourceElementOffset; resourceElementOffset. offset00; offset01; offset10; offset11; offset00; offset01; offset10; offset11. 0; 0; 2; 6; 8; 0; 1; 6; 7. 1; 2; 4; 8; 10; 1; 6; 7; 0. 2; 1; 3; 7; 9; 2; 3; 8; 9. 3; 3; 5; 9; 11; 3; 8; 9; 2. 4; -; -; -; -; 4; 5; 10; 11. 5; -; -; -; -; 5; 10; 11; 4. 8; 4; 6; 10; 0; -; -; -; -. 9; 6; 8; 0; 2; -; -; -; -. 10; 5; 7; 11; 1; -; -; -; -. 11; 7; 9; 1; 3; -; -; -; -. 12; -; -; -; -; 6; 7; 0; 1. 13; -; -; -; -; 7; 0; 1; 6. 14; -; -; -; -; 8; 9; 2; 3. 15; -; -; -; -; 9; 2; 3; 8. 16; -; -; -; -; 10; 11; 4; 5. 17; -; -; -; -; 11; 4; 5; 10. For 8 Tx UL SU-MIMO Working assumption - To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables. - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table). - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1]. Agreement - For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-8-X, Table 7.3.1.1.2-9-X, Table 7.3.1.1.2-10-X, and Table 7.3.1.1.2-11-X are supported. - FFS: Whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not. - Note: Antenna ports tables for Rel.18 eType2 DMRS ports with maxLength = 1/2 and eType1 DMRS ports with maxLength = 2 for PUSCH are to be discussed separately. Table 7.3.1.1.2-8-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=1, rank = 1 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0. 1; 1; 1. 2; 2; 0. 3; 2; 1. 4; 2; 2. 5; 2; 3. 6; 1; 8. 7; 1; 9. 8; 2; 8. 9; 2; 9. 10; 2; 10. 11; 2; 11. 12-15; Reserved; Reserved. Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0,1. 1; 2; 0,1. 2; 2; 2,3. 3; 2; 0,2. 4; 1; 8,9. 5; 2; 8,9. 6; 2; 10,11. [7]; [2]; [8,10]. 8-15; Reserved; Reserved. Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-2. [1]; [2]; [8-10]. 2; 1; 0,1,8. 3; 2; 0,1,8. 4; 2; 2,3,10. 5-15; Reserved; Reserved. Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-3. [1]; [2]; [8-11]. 2; 1; 0,1,8,9. 3; 2; 0,1,8,9. 4; 2; 2,3,10,11. 5-15; Reserved; Reserved. Agreement - For full-coherent PUSCH with rank 5-8 with one port PTRS, support Alt.1 in the RAN1#111 agreement with the following update - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2. - FFS: Association with The CW with the higher MCS is selected in case of two CWs. - If the MCS is the same for two CWs, the PTRS port is associated with the first CW. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW with the higher MCS. 1; 2nd scheduled DMRS port the CW with the higher MCS. 2; 3rd scheduled DMRS port the CW with the higher MCS. 3; 4th scheduled DMRS port the CW with the higher MCS.