3GPP TSG RAN WG1 #112bis-e	R1-230xxxx

e-Meeting, April 17th – 26th, 2023

Title:	[Draft] LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK

Response to:	-

Release:	Rel-18

Work Items:	NR_NTN_enh-Core

Source:	RAN WG1

To:	RAN WG2

CC:	-

Contact Person:

- Name:	Shohei Yoshioka

- E-mail Address:	syouhei.yoshioka.py@nttdocomo.com

Send any reply LS to:	3GPP Liaisons Coordinator:  mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 has discussed how UE reports information related to PUCCH repetition for Msg4 HARQ-ACK and reached the following working assumption.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE. - Option B: Higher layer signaling in Msg3 PUSCH

It is noted that either repetition request or capability report is transmitted from UE based on the following working assumption.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

Furthermore, it is noted that one bit information in total is assumed for the repetition request or capability report as the following agreement.

Proposal 1-2_vb For PUCCH repetition for Msg4 HARQ-ACK, repetition factor is not requested/indicated from UE, as in R17 Msg3 repetition feature. - i.e., UE transmits one bit for repetition request or capability report.

2. Actions

To TSG RAN WG2

ACTION: 	RAN1 respectfully asks RAN2 to provide feedback on whether or not Option B is feasible from RAN2 perspective, and if feasible, to specify Option B in RAN2 specification. the feasibility of Option B, and if feasible, to specify the details of Option B.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN1 Meeting #113				May 22nd-26th, 2023		Incheon

TSG-RAN1 Meeting #114				Aug. 21st-25th, 2023		Toulouse