3GPP TSG RAN WG1 #113	R1-2305855

Incheon, Korea, May 22 – May 26, 2023

Agenda item:		9.16.8

Source:	Nordic Semiconductor ASA

Title:					On eRedCap feature set

Document for:		Discussion and Decision

# Introduction

R17 default feature set is defined as the following:

| 28. NR_RedCap | 28-1 | RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included 6. 1 UE-specific RRC configured DL BWP per carrier 7. 1 UE-specific RRC configured UL BWP per carrier 8. RRC reconfiguration of any parameters related to BWP 9. UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB 10. NCD-SSB based measurements in RRC-configured DL BWP |
| --- | --- | --- | --- |

# Capability Discussion

## Default eRedCap capability

The first aspect to discuss in this AI is what is eRedCap’s default feature. In our opinion, main characteristic of eRedCap are

- those components inherited from R17 RedCap default features, by removing component 2 related to FR2

- capability to process at most 25/12PRBs per slot for data channels.

- reduced [minimal] peak rate, where square bracket being controversial.

When it comes to optional features, the following should be considered

- capability to process more than 25/12PRBs per slot for data channels.

- half-duplex capability

Proposal-1: R18 RedCap default feature adopts R17 RedCap default feature components, except of component #2 (FR2).

Proposal-2: R18 RedCap default feature, includes “Capability to process at most 25/12PRBs per slot for data channels”.

Proposal-3: R18 RedCap UE separate optional feature includes “Capability to process more than 25/12PRBs per slot for data channels.

## eRedCap Half-duplex

In TDD, a UE is mandatorily capable of receiving two UL unicast DCIs per slot, as part of FG3-1. However, the same does not apply to FDD. The reason here is that for FD-FDD such capability is not beneficial, while in TDD there is a clear benefit.

1) One configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - TCI state(s) for a CORESET configuration 2) CSS and UE-SS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - UP to 3 search space sets in a slot for a scheduled SCell per BWP This search space limit is before applying all dropping rules. - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD

However, similar benefit as in TDD exists for HD-FDD, where if two PUSCH scheduling DCI are allowed, and in case where single UE is scheduled in the cell, HD-FDD UE throughput can be increased from 50->75% of that of FD-FDD, this directly improving the spectral efficiency of HD-FDD scheduler. Scheduling is illustrated in Figure 1, where 7bit bitmap corresponds to symbol allocation within a half-slot. This scheduling scheme supports TA for up to 30km (4symbol gap), while keeps max K2=4.

Figure 1 Improving spectral efficiency for HD-FDD with allowing 2 UL unicast DCI per slot

Proposal-4: A R18 RedCap UE supporting half-duplex, should be capable to process one additional unicast DCI scheduling UL [per slot per scheduled CC] for FDD.

| X. NR_e-redcap | X-Y | Half-duplex FDD operation for RedCap UE | - Half-duplex FDD operation (instead of full-duplex FDD operation) for RedCap UE - Processing one additional unicast DCI scheduling UL per slot [per scheduled CC] for FDD | X-1 |
| --- | --- | --- | --- | --- |

# Conclusions

In this contribution we discussed issues related to further reduced complexity NR UE and we had the following observations and proposals:

Proposal-1: R18 RedCap default feature adopts R17 RedCap default feature components, except of component #2 (FR2).

Proposal-2: R18 RedCap default feature, includes “Capability to process at most 25/12PRBs per slot for data channels”.

Proposal-3: R18 RedCap UE separate optional feature includes “Capability to process more than 25/12PRBs per slot for data channels.

Proposal-4: A R18 RedCap UE supporting half-duplex, should be capable to process one additional unicast DCI scheduling UL [per slot per scheduled CC] for FDD.

| X. NR_e-redcap | X-Y | Half-duplex FDD operation for RedCap UE | - Half-duplex FDD operation (instead of full-duplex FDD operation) for RedCap UE - Processing one additional unicast DCI scheduling UL per slot [per scheduled CC] for FDD | X-1 |
| --- | --- | --- | --- | --- |