3GPP TSG RAN WG1 #113		                                                       R1-2306220

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.16

Source: 	Moderators (AT&T, NTT DOCOMO, INC.)

Title: 	Initial RAN1 UE features list for Rel-18 LTE after RAN1#113

Document for:	Endorsement

- Introduction

This contribution includes the initial RAN1 UE features list for Rel-18 LTE after RAN1 #113 meeting.

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. NR_SL_enh2 | 1-x |  |  |  |  |  |  |  |  |  |  |  |
| 1. NR_SL_enh2 | 1-y |  |  |  |  |  |  |  |  |  |  |  |
| 1. NR_SL_enh2 | 1-z |  |  |  |  |  |  |  |  |  |  |  |

[Placeholder: No discussion in RAN1#113]

- IoT_NTN_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_enh | 2-1a | Semi-static HARQ feedback disabling for eMTC CE mode B | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in CE mode B Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1b | Dynamic HARQ feedback disabling for eMTC CE mode B | 1. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 in CE mode B Note: this applies to single-TB case FFS: whether to merge 2-1a and 2-1b FFS: whether to have a separate FG for the combination of FGs 2-1a and 2-1b [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1d | Semi-static HARQ feedback disabling for eMTC CE mode A | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode A cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in C mode A Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-2 | Semi-static HARQ feedback disabling for SPS PDSCH for eMTC CE Mode A | UE reports ACK/NACK for the first SPS PDSCH after activation if enabled, and follow per-process HARQ feedback enabled/disabled configuration otherwise | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE Mode A cannot disable HARQ feedback for SPS PDSCH | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in C mode A FFS: whether to merge 2-1d and 2-2 [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1e | Semi-static HARQ feedback disabling for NB-IoT | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 Note: this applies to single-TB case FFS: whether to merge 2-1f and 2-1e FFS: whether to have a separate FG for the combination of FGs 2-1e and 2-1f [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1f | Dynamic HARQ feedback disabling for NB-IoT | 1. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-3a | GNSS position fix in RRC Connected state for eMTC—triggered | 1. UE reports GNSS position fix time duration for measurement and validation duration at least [during the initial access stage and in connected mode] 2. UE receives eNB GNSS measurement trigger 3. UE re-acquires GNSS in RRC Connected state 4. UE re-acquires GNSS position fix within a configured gap | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE cannot get triggered GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] Note: This applies to non-DRX | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-4a | GNSS position fix in RRC Connected state for eMTC—autonomous | [1. UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger] 2. UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | [Rel. 18 2-3a] | Yes | N/A | Release 18 eMTC UE cannot get autonomous GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | Note: This applies to non-DRX FFS: merge 2-4a with 2-3a [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-3b | GNSS position fix in RRC Connected state for NB-IoT—triggered | 1. UE reports GNSS position fix time duration for measurement and validation duration at least [during the initial access stage and in connected mode] 2. UE receives eNB GNSS measurement trigger 3. UE re-acquires GNSS in RRC Connected state 4. UE re-acquires GNSS position fix within a configured gap | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot get triggered GNSS position fix in RRC Connected state [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-4b | GNSS position fix in RRC Connected state for NB-IoT—autonomous | [1. UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger] 2. UE reports GNSS position fix time duration for measurement at least during the initial access stage [and in connected mode] | [Rel. 18 2-3b] |  |  | Release 18 NB-IoT UE cannot get autonomous GNSS position fix in RRC Connected state [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] FFS: merge 2-3b with 2-4b | Optional with capability signalling |