3GPP TSG RAN WG1 #113                                                                                  R1-2304377

Incheon, Korea, May 22nd – May 26th, 2023

Source:	New H3C

Title:	Discussions on increased number of orthogonal DMRS ports

Agenda Item:	9.1.3.1

Document for:	Discussion and Decision

- Introduction

In the RAN1 #112bis meeting, increased number of orthogonal DMRS ports in Rel.18 NR MIMO evolution was discussed. Some agreements were made as below [1].

Agreement (eType1, maxLength1)

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP,

- Support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data for Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-TRP, in addition to row 30 for 1CW in RAN1#112 agreement.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

Agreement (eType1, maxLength2)

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 68 in Table 7.3.1.2.2-2A-X.

  - For row 68, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-2A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 68 | 2 | 0,2,3 | 1 |

Agreement (eType2, maxLength1)

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 60 in Table 7.3.1.2.2-3A-X.

  - For row 60, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-3A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 60 | 2 | 0,2,3 |

Agreement (eType2, maxLength2)

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is also applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 128 in Table 7.3.1.2.2-4A-X.

  - For row 128, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-4A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 128 | 2 | 0,2,3 | 1 |

PUSCH with rank >4

Agreement

Confirm the following Working Assumption in RAN1#112 at least for NCB based PUSCH:

- To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables.

  - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

    - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

- Note: The above Working Assumption for CB based PUSCH may be confirmed later.

This contribution focuses on further discussion of increased number of orthogonal DMRS ports.

- Discussion

Based on the agreement in last meeting[1], we have below work assumption:

Working Assumption - Adopt Table 7.3.1.1.2-12B/13B/14B/15B/16B/17B/20B/21B/22B/23B to support signalling >4 ranks PUSCH with Rel-15 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH. - FFS: Whether/how some of bits in the antenna ports field can be reused for other purpose for >4 ranks PUSCH. Table 7.3.1.1.2-12B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0-4; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-13B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 6 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,4,6; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-14B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 7 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,4,5,6; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-15B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 8 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,4,5,6,7; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-16B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=5 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 3; 0-4. 1-15; Reserved; Reserved. Table 7.3.1.1.2-17B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=6 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 3; 0-5. 1-15; Reserved; Reserved. Table 7.3.1.1.2-20B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=5 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 3; 0-4; 1. 1; 2; 0,1,2,3,6; 2. 12-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-21B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=6 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 3; 0-5; 1. 1; 2; 0,1,2,3,6,8; 2. 2-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-22B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=7 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,6,7,8; 2. 1-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-23B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=8 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,6,7,8,9; 2. 1-31; Reserved; Reserved; Reserved.

From technical perspective, we don’t see any critical issue to reuse Rel-15 DMRS ports for full or non-coherent UL codebook to support signaling >4 ranks PUSCH. So we suggest confirming this work assumption

Proposal 1:  The work assumption on supporting signaling >4 ranks PUSCH with Rel-15 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH should be confirmed.

For the antenna ports indication in Rel.18 eType 1/eType 2 DMRS ports, we have the following 3 agreements in last meeting [1].

Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-2-X. FFS: For row 9-11, 24-30, 55-60, and 81-83 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability. FFS: The total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64. Table 7.3.1.2.2-2-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2 One Codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 1; 0; 1; [0; 2; 0-4; 2]. 1; 1; 1; 1; [1; 2; 0,1,2,3,4,6; 2]. 2; 1; 0,1; 1; [2; 2; 0,1,2,3,4,5,6; 2]. 3; 2; 0; 1; [3; 2; 0,1,2,3,4,5,6,7; 2]. 4; 2; 1; 1; 4; 2; 0,1,2,3,8; 1. 5; 2; 2; 1; 5; 2; 0,1,2,3,8,10; 1. 6; 2; 3; 1; 6; 2; 0,1,2,3,8,9,10; 1. 7; 2; 0,1; 1; 7; 2; 0,1,2,3,8,9,10,11; 1. [8; 2; 2,3; 1]; [8; 1; 0,1,4,5,8; 2]. [9; 2; 0-2; 1]; [9; 1; 0,1,4,5,8,12; 2]. [10; 2; 0-3; 1]; [10; 1; 0,1,4,5,8,9,12; 2]. [11; 2; 0,2; 1]; [11; 1; 0,1,4,5,8,9,12,13; 2]. 12; 2; 0; 2; [12; 2; 0,1,4,5,8; 2]. 13; 2; 1; 2; [13; 2; 0,1,4,5,8,12; 2]. 14; 2; 2; 2; [14; 2; 0,1,4,5,8,9,12; 2]. 15; 2; 3; 2; [15; 2; 0,1,4,5,8,9,12,13; 2]. 16; 2; 4; 2; [16; 2; 2,3,6,7,10; 2]. 17; 2; 5; 2; [17; 2; 2,3,6,7,10,14; 2]. 18; 2; 6; 2; [18; 2; 2,3,6,7,10,11,14; 2]. 19; 2; 7; 2; [19; 2; 2,3,6,7,10,11,14,15; 2]. 20; 2; 0,1; 2; [20; 2; 0,1, 2,3,10; 1]. 21; 2; 2,3; 2; [21; 2; 0,1,8,2,3,10; 1]. 22; 2; 4,5; 2; [22; 2; 0,1,8, 2,3,10,11; 1]. 23; 2; 6,7; 2; [23; 2; 0,1,8,9,2,3,10,11; 1]. [24; 2; 0,4; 2]; [24; 1; 0,1,4,5,12; 2]. [25; 2; 2,6; 2]; [25; 1; 0,1,8,4,5,12; 2]. [26; 2; 0,1,4; 2]; [26; 1; 0,1,8,4,5,12,13; 2]. [27; 2; 2,3,6; 2]; [27; 1; 0,1,8,9,4,5,12,13; 2]. [28; 2; 0,1,4,5; 2]; [28; 2; 0,1,4,5,12; 2]. [29; 2; 2,3,6,7; 2]; [29; 2; 0,1,8,4,5,12; 2]. [30; 2; 0,2,4,6; 2]; [30; 2; 0,1,8,4,5,12,13; 2]. 31; 1; 8; 1; [31; 2; 0,1,8,9,4,5,12,13; 2]. 32; 1; 9; 1; [32; 2; 2,3,6,7,14; 2]. 33; 1; 8,9; 1; [33; 2; 2,3,10,6,7,14; 2]. 34; 2; 8; 1; [34; 2; 2,3,10,6,7,14,15; 2]. 35; 2; 9; 1; [35; 2; 2,3,10,11,6,7,14,15; 2]. 36; 2; 10; 1; [36; 2; 0,2,3,8,9; 1]. 37; 2; 11; 1; [37; 2; 0,1,2,3,8,9; 1]. 38; 2; 8,9; 1. 39; 2; 10,11; 1. [40; 2; 8-10; 1]. [41; 2; 8-11; 1]. [42; 2; 8,10; 1]. 43; 2; 8; 2. 44; 2; 9; 2. 45; 2; 10; 2. 46; 2; 11; 2. 47; 2; 12; 2. 48; 2; 13; 2. 49; 2; 14; 2. 50; 2; 15; 2. 51; 2; 8,9; 2. 52; 2; 10,11; 2. 53; 2; 12,13; 2. 54; 2; 14,15; 2. [55; 2; 8,12; 2]. [56; 2; 10,14; 2]. [57; 2; 8,9,12; 2]. [58; 2; 10,11,14; 2]. [59; 2; 8,9,12,13; 2]. [60; 2; 10,11,14,15; 2]. 61; 2; 8,10,12,14; 2. 62; 1; 0,1,8; 1. 63; 1; 0,1,8,9; 1. 64; 2; 0,1,8; 1. 65; 2; 0,1,8,9; 1. 66; 2; 2,3,10; 1. 67; 2; 2,3,10,11; 1. [69; 1; 0,1,8; 2]. [70; 1; 0,1,8,9; 2]. [71; 1; 4,5,12; 2]. [72; 1; 4,5,12,13; 2]. [73; 2; 0,1,8; 2]. [74; 2; 0,1,8,9; 2]. [75; 2; 4,5,12; 2]. [76; 2; 4,5,12,13; 2]. [77; 2; 2,3,10; 2]. [78; 2; 2,3,10,11; 2]. [79; 2; 6,7,14; 2]. [80; 2; 6,7,14,15; 2]. [81; 2; 5,8,9; 2]. [82; 2; 7,10,11; 2]. [83; 2; 7,12,13; 2]. Agreement For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-3-X. FFS: For rows 9, 10, 20-23, 33,34, 44-46, 60-62 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability. Table 7.3.1.2.2-3-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1 One codeword: Codeword 0 enabled, Codeword 1 disabled; Two codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0; 0; 3; 0-4. 1; 1; 1; 1; 3; 0-5. 2; 1; 0,1; [2; 3; 12-16]. 3; 2; 0; [3; 3; 12-17]. 4; 2; 1; 4; 2; 0,1,2,3,12. 5; 2; 2; 5; 2; 0,1,2,3,12,14. 6; 2; 3; 6; 2; 0-3,12-14. 7; 2; 0,1; 7; 2; 0-3,12-15. 8; 2; 2,3; [8; 3; 0,1,2,3,12]. [9; 2; 0-2]; [9; 3; 0,1,2,3,12,14]. [10; 2; 0-3]; [10; 3; 0-3,12-14]. 11; 3; 0; [11; 3; 0-3,12-15]. 12; 3; 1; [12; 2; 0,2,3,12,13]. 13; 3; 2; [13; 2; 0,1,2,3,14]. 14; 3; 3; [14; 2; 0,1,12,2,3,14]. 15; 3; 4; [15; 2; 0,1,12,2,3,14,15]. 16; 3; 5; [16; 2; 0,1,12,13,2,3,14,15]. 17; 3; 0,1; [17; 3; 0,1,2,3,14]. 18; 3; 2,3; [18; 3; 0,1,12,2,3,14]. 19; 3; 4,5; [19; 3; 0,1,12,2,3,14,15]. [20; 3; 0-2]; [20; 3; 0,1,12,13,2,3,14,15]. [21; 3; 3-5]. [22; 3; 0-3]. [23; 2; 0,2]. 24; 1; 12. 25; 1; 13. 26; 1; 12,13. 27; 2; 12. 28; 2; 13. 29; 2; 14. 30; 2; 15. 31; 2; 12,13. 32; 2; 14,15. [33; 2; 12-14]. [34; 2; 12-15]. 35; 3; 12. 36; 3; 13. 37; 3; 14. 38; 3; 15. 39; 3; 16. 40; 3; 17. 41; 3; 12,13. 42; 3; 14,15. 43; 3; 16,17. [44; 3; 12-14]. [45; 3; 15-17]. [46; 3; 12-15]. [47; 2; 12,14]. 48; 1; 0,1,12. 49; 1; 0,1,12,13. 50; 2; 0,1,12. 51; 2; 0,1,12,13. 52; 2; 2,3,14. 53; 2; 2,3,14,15. 54; 3; 0,1,12. 55; 3; 0,1,12,13. 56; 3; 2,3,14. 57; 3; 2,3,14,15. 58; 3; 4,5,16. 59; 3; 4,5,16,17. [60; 3; 13,15,17]. [61; 3; 13,15]. [62; 2; 13,15]. Agreement For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-4-X. FFS: For rows 9, 10, 20-23, 42-47, 67, 68, 78-80, 100-105, and 153-158 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability. Table 7.3.1.2.2-4-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2 One codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 1; 0; 1; [0; 3; 0-4; 1]. 1; 1; 1; 1; [1; 3; 0-5; 1]. 2; 1; 0,1; 1; [2; 2; 0,1,2,3,6; 2]. 3; 2; 0; 1; [3; 2; 0,1,2,3,6,8; 2]. 4; 2; 1; 1; [4; 2; 0,1,2,3,6,7,8; 2]. 5; 2; 2; 1; [5; 2; 0,1,2,3,6,7,8,9; 2]. 6; 2; 3; 1; 6; 2; 0,1,2,3,12; 1. 7; 2; 0,1; 1; 7; 2; 0-3,12,14; 1. 8; 2; 2,3; 1; 8; 2; 0-3,12-14; 1. [9; 2; 0-2; 1]; 9; 2; 0-3,12-15; 1. [10; 2; 0-3; 1]; [10; 3; 0,1,2,3,12; 1]. 11; 3; 0; 1; [11; 3; 0-3,12,14; 1]. 12; 3; 1; 1; [12; 3; 0-3,12-14; 1]. 13; 3; 2; 1; [13; 3; 0-3,12-15; 1]. 14; 3; 3; 1; [14; 1; 0,1,6,7,12; 2]. 15; 3; 4; 1; [15; 1; 0,1,6,7,12,18; 2]. 16; 3; 5; 1; [16; 1; 0,1,6,7,12,13,18; 2]. 17; 3; 0,1; 1; [17; 1; 0,1,6,7,12,13,18,19; 2]. 18; 3; 2,3; 1; [18; 2; 0,1,6,7,12; 2]. 19; 3; 4,5; 1; [19; 2; 0,1,6,7,12,18; 2]. [20; 3; 0-2; 1]; [20; 2; 0,1,6,7,12,13,18; 2]. [21; 3; 3-5; 1]; [21; 2; 0,1,6,7,12,13,18,19; 2]. [22; 3; 0-3; 1]; [22; 2; 2,3,8,9,14; 2]. [23; 2; 0,2; 1]; [23; 2; 2,3,8,9,14,20; 2]. 24; 3; 0; 2; [24; 2; 2,3,8,9,14,15,20; 2]. 25; 3; 1; 2; [25; 2; 2,3,8,9,14,15,20,21; 2]. 26; 3; 2; 2; [26; 3; 0,1,6,7,12; 2]. 27; 3; 3; 2; [27; 3; 0,1,6,7,12,18; 2]. 28; 3; 4; 2; [28; 3; 0,1,6,7,12,13,18; 2]. 29; 3; 5; 2; [29; 3; 0,1,6,7,12,13,18,19; 2]. 30; 3; 6; 2; [30; 3; 2,3,8,9,14; 2]. 31; 3; 7; 2; [31; 3; 2,3,8,9,14,20; 2]. 32; 3; 8; 2; [32; 3; 2,3,8,9,14,15,20; 2]. 33; 3; 9; 2; [33; 3; 2,3,8,9,14,15,20,21; 2]. 34; 3; 10; 2; [34; 3; 4,5,10,11,16; 2]. 35; 3; 11; 2; [35; 3; 4,5,10,11,16,22; 2]. 36; 3; 0,1; 2; [36; 3; 4,5,10,11,16,17,22; 2]. 37; 3; 2,3; 2; [37; 3; 4,5,10,11,16,17,22,23; 2]. 38; 3; 4,5; 2; [38; 2; 0,1,2,3,14; 1]. 39; 3; 6,7; 2; [39; 2; 0,1,12,2,3,14; 1]. 40; 3; 8,9; 2; [40; 2; 0,1,12,2,3,14,15; 1]. 41; 3; 10,11; 2; [41; 2; 0,1,12,13,2,3,14,15; 1]. [42; 3; 0,1,6; 2]; [42; 3; 0,1,2,3,14; 1]. [43; 3; 2,3,8; 2]; [43; 3; 0,1,12,2,3,14; 1]. [44; 3; 4,5,10; 2]; [44; 3; 0,1,12,2,3,14,15; 1]. [45; 3; 0,1,6,7; 2]; [45; 3; 0,1,12,13,2,3,14,15; 1]. [46; 3; 2,3,8,9; 2]; [46; 1; 0,1,6,7,18; 2]. [47; 3; 4,5,10,11; 2]; [47; 1; 0,1,12,6,7,18; 2]. 48; 1; 0; 2; [48; 1; 0,1,12,6,7,18,19; 2]. 49; 1; 1; 2; [49; 1; 0,1,12,13,6,7,18,19; 2]. 50; 1; 6; 2; [50; 2; 0,1,6,7,18; 2]. 51; 1; 7; 2; [51; 2; 0,1,12,6,7,18; 2]. 52; 1; 0,1; 2; [52; 2; 0,1,12,6,7,18,19; 2]. 53; 1; 6,7; 2; [53; 2; 0,1,12,13,6,7,18,19; 2]. 54; 2; 0,1; 2; [54; 2; 2,3,8,9,20; 2]. 55; 2; 2,3; 2; [55; 2; 2,3,14,8,9,20; 2]. 56; 2; 6,7; 2; [56; 2; 2,3,14,8,9,20,21; 2]. 57; 2; 8,9; 2; [57; 2; 2,3,14,15,8,9,20,21; 2]. 58; 1; 12; 1; [58; 3; 0,1,6,7,18; 2]. 59; 1; 13; 1; [59; 3; 0,1,12,6,7,18; 2]. 60; 1; 12,13; 1; [60; 3; 0,1,12,6,7,18,19; 2]. 61; 2; 12; 1; [61; 3; 0,1,12,13,6,7,18,19; 2]. 62; 2; 13; 1; [62; 3; 2,3,8,9,20; 2]. 63; 2; 14; 1; [63; 3; 2,3,14,8,9,20; 2]. 64; 2; 15; 1; [64; 3; 2,3,14,8,9,20,21; 2]. 65; 2; 12,13; 1; [65; 3; 2,3,14,15,8,9,20,21; 2]. 66; 2; 14,15; 1; [66; 3; 4,5,10,11,22; 2]. [67; 2; 12-14; 1]; [67; 3; 4,5,16,10,11,22; 2]. [68; 2; 12-15; 1]; [68; 3; 4,5,16,10,11,22,23; 2]. 69; 3; 12; 1; [69; 3; 4,5,16,17,10,11,22,23; 2]. 70; 3; 13; 1. 71; 3; 14; 1. 72; 3; 15; 1. 73; 3; 16; 1. 74; 3; 17; 1. 75; 3; 12,13; 1. 76; 3; 14,15; 1. 77; 3; 16,17; 1. [78; 3; 12-14; 1]. [79; 3; 15-17; 1]. [80; 3; 12-15; 1]. [81; 2; 12,14; 1]. 82; 3; 12; 2. 83; 3; 13; 2. 84; 3; 14; 2. 85; 3; 15; 2. 86; 3; 16; 2. 87; 3; 17; 2. 88; 3; 18; 2. 89; 3; 19; 2. 90; 3; 20; 2. 91; 3; 21; 2. 92; 3; 22; 2. 93; 3; 23; 2. 94; 3; 12,13; 2. 95; 3; 14,15; 2. 96; 3; 16,17; 2. 97; 3; 18,19; 2. 98; 3; 20,21; 2. 99; 3; 22,23; 2. [100; 3; 12,13,18; 2]. [101; 3; 14,15,20; 2]. [102; 3; 16,17,22; 2]. [103; 3; 12,13,18,19; 2]. [104; 3; 14,15,20,21; 2]. [105; 3; 16,17,22,23; 2]. 106; 1; 12; 2. 107; 1; 13; 2. 108; 1; 18; 2. 109; 1; 19; 2. 110; 1; 12,13; 2. 111; 1; 18,19; 2. 112; 2; 12,13; 2. 113; 2; 14,15; 2. 114; 2; 18,19; 2. 115; 2; 20,21; 2. 116; 1; 0,1,12; 1. 117; 1; 0,1,12,13; 1. 118; 2; 0,1,12; 1. 119; 2; 0,1,12,13; 1. 120; 2; 2,3,14; 1. 121; 2; 2,3,14,15; 1. 122; 3; 0,1,12; 1. 123; 3; 0,1,12,13; 1. 124; 3; 2,3,14; 1. 125; 3; 2,3,14,15; 1. 126; 3; 4,5,16; 1. 127; 3; 4,5,16,17; 1. [129; 1; 0,1,12; 2]. [130; 1; 0,1,12,13; 2]. [131; 1; 6,7,18; 2]. [132; 1; 6,7,18,19; 2]. [133; 2; 0,1,12; 2]. [134; 2; 0,1,12,13; 2]. [135; 2; 6,7,18; 2]. [136; 2; 6,7,18,19; 2]. [137; 2; 2,3,14; 2]. [138; 2; 2,3,14,15; 2]. [139; 2; 8,9,20; 2]. [140; 2; 8,9,20,21; 2]. [141; 3; 0,1,12; 2]. [142; 3; 0,1,12,13; 2]. [143; 3; 6,7,18; 2]. [144; 3; 6,7,18,19; 2]. [145; 3; 2,3,14; 2]. [146; 3; 2,3,14,15; 2]. [147; 3; 8,9,20; 2]. [148; 3; 8,9,20,21; 2]. [149; 3; 4,5,16; 2]. [150; 3; 4,5,16,17; 2]. [151; 3; 10,11,22; 2]. [152; 3; 10,11,22,23; 2]. [153; 3; 7,12,13; 2]. [154; 3; 9,14,15; 2]. [155; 3; 11,16,17; 2]. [156; 3; 9,18,19; 2]. [157; 3; 18,19,20; 2]. [158; 3; 21,22,23; 2].

For the antenna ports indication on row 9-11, 24-30, 55-60, and 81-83 in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH,  we suggest keeping them in order to support MU-MIMO  for one CW case and introducing the restriction to MU-MIMO case or UE capability.

Proposal 2: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, row 9-11, 24-30, 55-60, and 81-83 are kept with MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) in one CW or UE capability.

In order reduce DCI indication overhead, the total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64.

Proposal 3: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH the total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64.

For the antenna ports indication on rows 9, 10, 20-23, 33,34, 44-46, 60-62  in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, we suggest keeping them in order to support MU-MIMO  for one CW case and introducing the restriction to MU-MIMO case or UE capability.

Proposal 4: For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, rows 9, 10, 20-23, 33,34, 44-46, 60-62  are kept with MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability in one CW.

For the antenna ports indication on rows 9, 10, 20-23, 42-47, 67, 68, 78-80, 100-105, and 153-158 in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, we suggest keeping them in order to support MU-MIMO  for one CW case and introducing the restriction to MU-MIMO case or UE capability.

Proposal 5: For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, rows 9, 10, 20-23, 42-47, 67, 68, 78-80, 100-105, and 153-158 are kept with MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability in one CW.

- Conclusion

In this contribution, we discussed about increased number of orthogonal DMRS ports. The proposals are as follows:

Proposal 1:  The work assumption on supporting signaling >4 ranks PUSCH with Rel-15 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH should be confirmed.

Proposal 2: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, row 9-11, 24-30, 55-60, and 81-83 are kept with MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) in one CW or UE capability.

Proposal 3: For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH the total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64.

Proposal 4: For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, rows 9, 10, 20-23, 33,34, 44-46, 60-62  are kept with MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability in one CW.

Proposal 5: For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, rows 9, 10, 20-23, 42-47, 67, 68, 78-80, 100-105, and 153-158 are kept with MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability in one CW.

- Reference

- Chairman’s Notes, RAN1#112bis, final, April 17th –April 26th, 2023