3GPP TSG RAN WG1 #113		 R1-2304530

Incheon, Korea, May 22nd – May 26th, 2023

# Source: 	ZTE, Sanechips

# Title:	Evaluation on LP-WUS

# Agenda item: 	9.11.1

# Document for:	Discussion and decision

Introduction

In RAN #94 meeting¸ new study item on low-power Wake-up Signal and Receiver for NR [1] was approved and the latest revision was approved in [2].

The study item includes the following objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In RAN1#112bis e-meeting, some agreements are captured in the chairman notes [3]. In this contribution, we further discuss evaluation details on LP-WUS and provide some initial simulation results.

KPIs target

- 2.1 Coverage target

In last RAN1#112bis-e meeting, we have the agreement regarding the coverage target.

Agreement RAN1 further study the designs [target]/techniques of LP-WUS to have a comparable coverage as NR channel X. The NR channel X is - Option #1: PDCCH for paging - Option #2: PUSCH for message3 - FFS other options, e.g., between option1and option2 (better than PUSCH, worse than PDCCH) - The final design will jointly consider the coverage with other KPIs - FFS additional detail assumptions for NR channels, e.g., the message size for MSG3 and etc.

It is understood the LP-WUS should have a comparable coverage with NR channel. However, it is not fair to require LP-WUS has similar coverage with PDCCH, because whether the coverage compensation is needed depends on whether it has better performance than bottleneck channel, e.g., PUSCH. Even the LP-WUS has better performance, the bottleneck channel would still be the main factor impacting the system efficiency. Therefore, better LP-WUS coverage than bottle neck channel cannot improve system efficiency but cause more resource overhead.

Observation 1: If the coverage target for LP-WUS is PDCCH, it would cause more resource overhead.

The following scenarios are focused for LP-WUS and the bottleneck channel for those scenarios are presented in TR 38.830 as following Table 1.

Table 1. MIL for reference channel in TR38.830

| Scenarios | Channels (and Frame format) | Representative value | Standard Deviation (w/o outlier) | Relative difference between channels |
| --- | --- | --- | --- | --- |
| MIL for Urban 4GHz TDD scenario with BS Tx power of 33 dBm/MHz | PUSCH for eMBB DDDSU | 139.01 | 0.97 | 0.00 |
|  | PUSCH of Msg.3 | 152.10 | 2.00 | 13.09 |
|  | Broadcast PDCCH (PDCCH of Msg.2) | 158.54 | 2.12 | 19.53 |
| MIL for Urban 2.6GHz TDD scenario | PUSCH for eMBB DDDDDDDSUU | 139.66 | 1.12 | 0.00 |
|  | PUSCH of Msg.3 | 153.51 | 1.75 | 13.85 |
|  | Broadcast PDCCH (PDCCH of Msg.2) | 159.07 | 1.44 | 19.41 |
| MIL for Rural 700MHz FDD NLOS O2I scenario | PUSCH for eMBB UUUUU | 144.76 | 2.45 | 0.22 |
|  | PUCCH Format 3 22bit | 144.54 | 1.92 | 0.00 |
|  | PUSCH of Msg.3 | 147.50 | 1.58 | 2.96 |
|  | PDSCH for Msg.2 | 155.83 | 3.40 | 11.29 |

If PUSCH msg3 is assumed as the target channel, for 2.6GHz and 4GHz scenario, the MIL performance gap between bottleneck channel and msg3 would be more than 13dB.

Observation 2: For 4GHz and 2.6GHz scenario, MIL performance gap between NR bottleneck channel and msg3 is more than 13dB.

Observation 3: For 4GHz and 2.6GHz scenario, MIL performance gap between NR bottleneck channel and PDCCH is more than 19dB.

If the target coverage channel is the bottleneck channel, it would be beneficial to save the system overhead. There is no impact on the NW deployment, since the current NW deployment is based on the bottleneck channel. Therefore, bottleneck channel also should be considered for the coverage target. To make it simple, setting PUSCH as the target channel would be enough. Therefore, the following proposal is made.

Proposal 1: For LP-WUS coverage target, option 3 (PUSCH) should be added.

Different with LP-WUS, the LP-SS are provided for sync or measurement, instead of wake-up and data transmission. If the LP-SS has better performance, it benefits more UE for power saving because more UEs can use the LP-SS for sync and measurement.

Proposal 2: For LP-SS, separated coverage target should be studied.

- 2.2 Latency target

For the latency, we need to further discuss which kind of latency is assumed here. In connected mode, the latency may refer to the scheduling delay. In idle/inactive mode, the latency may refer to the paging delay. So, the latency here needs clarification. From our perspective, the latency means the time interval between LP-WUS and PRACH in idle/inactive mode. In connected mode, the latency means the time interval between LP-WUS and PDSCH with data.

Additionally, in last RAN1 112bis-e meeting, the following proposal has been discussed and harvested many supports.

The latency for the target use cases are considered as follows: - Latency for RRC CONNECTED mode is in the order of milliseconds - Latency for RRC IDLE/INACTIVE mode is in the order of seconds

However, based on above proposal, it does not mean for idle/inactive mode, the latency is not needed to be further reduced, since the current requirement is the minimum requirement, instead of the final target. Therefore, we have following proposal.

Proposal 3: Clarify the latency definition for different RRC states.

- Minimum latency requirement for RRC CONNECTED mode is in the order of milliseconds

- Minimum latency requirement for RRC IDLE/INACTIVE mode is in the order of seconds

- 2.3 Other KPIs target

For the power saving gain, according to the SID, substantial gains compared to the existing Rel-15/16/17 UE power saving mechanisms should be provided. However, it would be difficult to define what is the exact target for the substantial gains. As a starting point, at least 10% should be the minimum requirement.

For the increased system overhead by introducing LP-SS or LP-WUS, the increased system overhead should be no more than 5%. As for the NW power consumption, it can be discussed, for example, the increased power should not exceed 5%.

Proposal 4: Discuss whether to set a target or minimum requirement for power saving gain, system overhead and NW power consumption.

Evaluation for UE power consumption in idle mode

For RRC idle/inactive mode, the existing UE power consumption evaluation method in Rel-17 PEI, e.g., numerical calculation, can be reused to evaluate the power consumption for LP-WUS. In section 3.1, the power model and system parameters for power consumption evaluation are discussed and the initial results on power consumption are shown in section 3.2.

- 3.1 Power model

In RAN1#112bis meeting [3], the following agreements were achieved for ultra-deep sleep of main radio.

Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) as baseline - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases.

For LP-WUS, the main radio is powered off until the LP-WUS is received in idle/inactive mode which means main radio can be powered off for a long time. Therefore, the relative power of ultra-deep sleep should be far less than the relative power of deep sleep. Considering the relative power of deep sleep was defined as 1 or 0.8 unit in existing power consumption model, it is acceptable the relative power of ultra-deep sleep is 0.015 unit for LP-WUS power consumption evaluation.

Proposal 5: The relative power of MR ultra-deep sleep is 0.015 unit for LP-WUS power consumption evaluation.

For LP-WUS, the sync/re-sync may be required after main radio is powered on from UDS. For the power consumption evaluation of PEI, it is assumed that there are up to 3 SSBs before the PO from DS. Considering the sync/re-sync may be achieved via LP-WUS or other signals such as TRSs, it is proposed there are at least 3 SSBs used for sync/re-sync when main radio is waked up from the state of ultra-deep sleep.

Proposal 6: For LP-WUS power consumption evaluation, at least 3 SSBs are required for sync/re-sync when main radio wakes up from the state of ultra-deep sleep.

In RAN1#112 bis meeting [3], the following were discussed for WUR power model, but no agreement is achieved.

- Power State; - Relative Power (unit); - Transition energy: - (unit multiplied by ms); - Ramp-up time TLR, ramp-up (ms). - Off[1]; - 0.001/ [Y, e.g., Y>=0.1 or 0.01]; - [TLR, ramp-up *(PON+POFF)/2]; - TLR, ramp-up = FFS, and company to report TLR, ramp-up - FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. - On[2]; - 0.01/0.05/0.1/0.5/1/2/4/10/20/30 - FFS: If other values are needed. - FFS: whether further categorization/sub-categorization is needed and how. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation - 10/20/30 for LP-WUR ON power state are used for OFDM receiver - FFS: LP-WUR power consumption values for FR2. - Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. - Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. - Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) - Note4: - For cat1a, clock error option 3 or 4 is assumed, - FFS for cat1b, clock error option 1 or 2 is assumed - For WUR Off value 0.001, oscillator option 1, 2, 3, 4 are not assumed and only RTC is maintained; For WUR Off value Y, option 1,2,3,4 can be assumed. - Note5: Up to companies to report whether same or different values are assumed for WUS monitoring and time/frequency synchronization.

The relative power of WUR depends on the LP-WUS architecture which is discussed in AI 9.11.2. The details are discussed in our contribution [4]. For power consumption evaluation, the following values are used:

- for OOK/FSK-based receiver, the relative power of WUR on is 0.01,0.1,0.5 or 1 and relative power of WUR off is 0.001.

- for OFDM-based receiver, the relative power of WUR on could be 10 or 30 and relative power of WUR off is 0.6 if the formula [TLR, ramp-up *(PON-POFF)/2] is still applied.

Proposal 7: The relative power of OOK/FSK/OFDM based LP-WUS architecture is suggested as below.

- for OOK/FSK-based receiver, the relative power of WUR on is 0.01,0.1,0.5 or 1 and relative power of WUR off is 0.001.

- for OFDM-based receiver, the relative power of WUR on is 10 or 30 and relative power of WUR off is 0.6.

It is known that the complexity of WUR is lower than MR, therefore the ramp-up time of WUR is less than MR. 10ms ramp-up time of WUR are used as the starting point for LP-WUS power consumption evaluation. As the WUR-on relative power increases, the ramp-up time also should be increased. However, for OFDM based receiver, the ramp-up time and transition energy should be aligned with the MR, since they have the similar architecture design.

Proposal 8: For LP-WUS power consumption evaluation, the 10ms ramp-up time of WUR could be used as the starting point.

- FFS the transition energy and ramp-up time for receiver for OFDM based receiver.

The following agreements was achieved in RAN1#112bis e-meeting:

Agreement The period of synchronization signal that LP-WUR used for at least power evaluation can be - Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR - For evaluations assuming LP-SS - {320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms} - Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy - Other values are not precluded Note: companies to report the purpose of the synchronization signal along with evaluations, e.g. can be for LR synchronization (i.e., time and/or frequency tracking) and/or measurement.

For 30KHz case, we think half slot or one slot for one LP-SS transmission may be the starting point, and other values, e.g., 1,2,4ms also can be considered. To satisfy the latency requirement, e.g., order of seconds, the periodicity for duty cycled LP-WUS monitoring could be 1280ms. To have more UEs monitoring in the same duration, larger on-duration periods are assumed, e.g., 4ms, 16ms.

Additionally, for the synchronization of OFDM based receiver, the measurement may last few milliseconds, therefore, duration of 2ms for measurement or sync is assumed. And synchronization signal may be SSB, or other signal introduced for OFDM based LP WUS. As for the LP-WUS receiving, to handle the time domain offset and support more UEs monitoring in the same time duration, 1 or 2ms are assumed for a monitoring window. Therefore, the evaluation assumptions of OOK-based/OFDM-based SS and LP-WUR are summarized as below.

OOK-based SS and LP-WUS: the period of LP-SS is 160, 320ms and on duration is 0.25ms. The period of LP-WUS is 1280ms and on duration is 1/4/16ms.

OFDM based SS and LP-WUS: for SS, the period of SS is 160,320 and on duration is 2ms, the period of LP-WUS is 1280ms and on duration is 1/2.

Proposal 9: For power evaluation,

- the monitoring window for LP-SS could be assumed as {0.25ms, 0.5ms, 1ms, 2,ms, 4ms} and the monitoring window for SSB/PSS/SSS could be assumed as {2ms, 4ms}.

- the monitoring window for duty cycled LP-WUS could be assumed as {1ms, 2ms, 4ms, 16ms} and the monitoring window for LP-WUS based on OFDM receiver for NR signal could be assumed as {1ms ,2ms}

- the monitoring periodicity for duty cycled LP-WUS could be assumed as 1.28s.

- 3.2 Power consumption results

Based on the of power model and previous agreements, the following values are used in power consumption evaluation:

| Baseline related | DRX cycle | 1.28s |
| --- | --- | --- |
|  | eDRX cycle | 60.44s |
|  | R_E | 1%, 0.1%, 0.01%, 0.001% |
|  | Number of UE per group | 1,10 |
| MR related | transition energy, ramp-up time | 400ms, 15000 |
| WUR related | WUR on, WUR off and ramp-up time | Alt 1: PWUR_on=0.01, PWUR_off= 0.001, TLR, ramp-up=10 Alt 2: PWUR_on=0.1, PWUR_off=0.001, TLR, ramp-up=10 Alt 3: PWUR_on=0.5, PWUR_off=0.001, TLR, ramp-up=10 Alt 4: PWUR_on=1, PWUR_off=0.001,TLR, ramp-up=10 Alt 5: PWUR_on=10, PWUR_off=0.6, TLR, ramp-up=14 Alt 6: PWUR_on=30, PWUR_off=0.6, TLR, ramp-up=18 |
|  | Monitoring mode | Mode 1: Always on Mode 2: on-off duty cycled For OOK/FSK based receiver Mode 2-1: period of LP-SS=160ms, on duration of LP-SS=0.25ms, period of LP-WUS=1.28s, on duration of LP-WUS=1ms Mode 2-2: period of LP-SS=320ms, on duration of LP-SS=0.25ms, period of LP-WUS=1.28s, on duration of LP-WUS=1ms Mode 2-3: period of LP-SS=160ms, on duration of LP-SS=0.25ms, period of LP-WUS=1.28s, on duration of LP-WUS=4ms Mode 2-4: period of LP-SS=320ms, on duration of LP-SS=0.25ms, period of LP-WUS=1.28s, on duration of LP-WUS=4ms Mode 2-5: period of LP-SS=160ms, on duration of LP-SS=0.25ms, period of LP-WUS=1.28s, on duration of LP-WUS=16ms Mode 2-6: period of LP-SS=320ms, on duration of LP-SS=0.25ms, period of LP-WUS=1.28s, on duration of LP-WUS=16ms For OFDM-based receiver Mode 2-7: period of SS=160ms, on duration of SS=2ms, period of LP-WUS=1.28s, on duration of LP-WUS=1ms Mode 2-8: period of SS=320ms, on duration of SS=2ms, period of LP-WUS=1.28s, on duration of LP-WUS=1ms Mode 2-9: period of SS=160ms, on duration of SS=2ms, period of LP-WUS=1.28s, on duration of LP-WUS=2ms Mode 2-10: period of SS=320ms, on duration of SS=2ms, period of LP-WUS=1.28s, on duration of LP-WUS=2ms |

The initial power consumption evaluation results are shown in Figure 1 and Figure 2.

(a)Alt-1 (UE Number=1)                       (b) Alt-1 (UE Number=10)

(c)Alt-2 (UE Number=1)                       (d) Alt-2 (UE Number=10)

(e)Alt-3 (UE Number=1)                       (f) Alt-3 (UE Number=10)

(g)Alt-4(UE Number=1)                       (h) Alt-4 (UE Number=10)

Figure 1 Power consumption evaluation results

Based on the above results, we have following observations:

Observation 4: For always on monitoring, compared with DRX/eDRX with PEI, the maximum power saving gain is 98.14% and 93.6%.

Observation 5: For duty cycled monitoring, compared with DRX/eDRX with PEI, the maximum power saving gain is 99.9% and 99.6% .

- Alt-5(UE Number=1)                       (b) Alt-5 (UE Number=10)

- Alt-6(UE Number=1)                       (b) Alt-6 (UE Number=10)

Figure 2 Power consumption evaluation results

Based on the above results, we have following observations:

Observation 6: For always on monitoring, the LP-WUS of Alt 5/Alt 6 bring significantly more power than DRX/eDRX with/without PEI.

Observation 7: For on-off duty cycled monitoring, compare with DRX with PEI, maximum power saving gain is 31.6%.

Observation 8: For on-off duty cycled monitoring，LP-WUS of Alt 6 has no power saving compared with DRX/eDRX with/without PEI.

Proposal 10: For receiver for NR signals/channels, if the relative power of WUR off is 0.6 and the transition energy of WUR is calculated by [TLR, ramp-up *(PON-POFF)/2], the power saving gain is not attractive and this case is not further considered.

- FFS: the relative power of WUR off is 0.1 or 0.01

- 3.3 Power consumption evaluation for RRM measurement

For LP-WUS, the power consumption also is impacted by different RRM measurement schemes. In order to evaluate the impact of RRM measurement on power consumption, the following three Opts are used in power consumption evaluation.

Opt 1: The serving cell is measured by LP-SS and neighbor cell measurement is performed only after MR is waked up by LP-WUS;

Opt 2: The serving cell is measured by LP-SS and neighbor cell measurement is performed by MR and relaxed to N DRX cycle;

For option2, if the measurement period for serving cell is equal or larger than that for neighbor cell measurement, the serving cell measurement can be performed together with neighbor cell measurement. The power consumption has ignorable impacts compared with the MR transition energy.

In Figure 3, the power consumption evaluation results of different RRM measurement are shown wherein R_E=1% and UE number=1 within the legacy RRM measurement is used for DRX with PEI.

Figure 3. Power consumption evaluation results based on RRM measurement

From the power consumption results from in Figure 3, it can be observed that:

Observation 9: With relative power <=0.5, compared with DRX with PEI, if the serving cell measurement is offloaded to LP-SS totally, or the measurement period for serving cell measurement is equal or larger than that for neighbor cell measurement, the always-on monitoring has power saving gain when neighbor cell measurement is relaxed to 9 DRX cycles.

Observation 10: With relative power <=1, compared with DRX with PEI, if the serving cell measurement is offloaded to LP-SS totally, or the measurement period for serving cell measurement is equal or larger than that for neighbor cell measurement, the duty cycled monitoring has power saving gain when neighbor cell measurement is relaxed to 7 DRX cycles

Observation 11: If the serving cell measurement is offloaded to LP-SS totally, and the neighbor cell measurement is performed only after MR is waked up by LP-WUS, then the power saving gain would be quite significant.

Based on above, if the serving cell measurement and neighbor cell measurement can be relaxed as a same measurement period, to have power saving gain compared with DRX with PEI, then the measurement period should be larger than X DRX cycles, e.g, 7 or 9.

If the measurement period for serving cell is less than that for neighbor cell measurement, the serving cell measurement is performed more frequently and the neighbor cell measurement can be performed together with serving cell measurement to avoid the MR transition energy from UDS. In this case, the measurement period for serving cell should be X-N1 DRX cycles, and the measurement period for neighbor cell should be X+N2 DRX cycles then, wherein N2>>N1. Therefore, to satisfy the requirement for neighbor cell measurement, the N2 should not be quite large and then the N1 also has limited value range.

Observation 12: If the measurement period for serving cell is less than that for neighbor cell measurement, the minimum number of relaxed DRX cycles for serving cell measurement is limited, and the power saving gain is mainly dominated by MR wake-up via serving cell measurement.

Proposal 11: For RRM measurement evaluation, to obtain the relaxed measurement period,

- Option1: the serving cell measurement and neighbor cell measurement are always performed together.

- Option2: the serving cell measurement and neighbor cell measurement difference for power consumption are clarified.

- 3.4 FAR impacts

Once LP-WUS false detection happens, the MR will be powered on. More power consumption is wasted since the procedure, such as SSB reception and PEI/PO monitoring, would be implemented on MR. When false detection happens, only when the UE has finished PEI/PO monitoring but does not receive its paging message, the UE can identify that a false detection happens. The results of power consumption evaluation with FAR impact are shown in Figure 4 wherein the FAR=X% indicates the probability of extra MR power on caused by FAR is X% in one DRX cycle, which is defined in details in section 5.

Figure 4. Power consumption evaluation results based on FAR

From the power consumption results from Figure 4, we have following observation:

Observation 13: For Mode-1, compared with DRX with PEI, the LP-WUS with relative power<=1 has power saving gain when the probability of extra MR power on caused by FAR is 0.1%/1%.

- 4. Evaluation for latency

## 4.1 Dynamic PO for latency

In RAN1#111 meeting [3], the following agreements were achieved for latency evaluation:

For IDLE/INACTIVE state, the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can [monitor/detect] the paging message - FFS: if UE is not required to monitor a PO after wake-up, e.g., latency is the time interval between the data arrival time at the gNB and the time UE transmits the PRACH after LP-WUS detection. - sync/re-sync for main radio is included

For LP-WUS, if the first PO is the legacy PO which is defined in TR38.804, there are three parts of the latency, i.e., time for main radio ramp up, time for sync/re-sync and the delay from sync/re-sync to legacy PO. One example is shown in Figure 5 wherein the latency T1=Tx+Tramp up+T sync/re-sync.

Figure 5. Latency for LP-WUS based on legacy PO mechanism

If the first PO is dynamic, i.e., the PO is located after main radio ramp up and sync/re-sync, there are two parts of the latency. One example is shown in Figure 6 wherein the latency T2=Tramp up+T sync/re-sync. It is assumed the gap between the last SSB and PO is 8ms.

Figure 6. Latency for LP-WUS based on dynamic PO mechanism

The latency results based on DRX/eDRX with/without PEI are shown in Table 2, wherein the paging is uniform arrived, DRX cycle is 1.28s, eDRX cycle is 61.44s, PTW is 5.12s, the Tramp up is 400ms /800ms and T sync/re-sync is 50ms.

Table 2. Latency evaluation

|  | Latency | LP-WUS based on legacy PO (T1) | LP-WUS based on dynamic PO (T2) |
| --- | --- | --- | --- |
|  |  | Latency Reduction | Latency Reduction |
| DRX/DRX+PEI | 0.64s | -115% | 29.6% |
| eDRX/eDRX+PEI | 26.46s | 94.7% | 98.2% |
| LP-WUS based on legacy PO (T1) | 1.38s | - | 67.3% |
| LP-WUS based on legacy PO (T1) | 0.45s | -206% | - |

Based on Table 2, it can be observed:

Observation 14: Compared with DRX/eDRX, the latency reduction of LP-WUS based on legacy PO is -115%/94.7%.

Observation 15: Compared with DRX/eDRX, the latency reduction of LP-WUS based on dynamic PO is 29.6%/98.2%.

Observation 16: Compared with LP-WUS based on legacy PO, the latency reduction of LP-WUS based on dynamic PO is 67.3%.

Once LP-WUS miss detection happens, the MR is not powered on to receive paging message and send PRACH. The eNB can identify that a miss detection happens only after the eNB has not received PRACH. Then eNB pages the UE again in the next first PO. Therefore, the latency caused by LP-WUS miss detection should be considered. One example is shown in Figure 7. Considering the miss detection of LP-WUS, the latency is T=T1/T2+m%*(Toffset+T1/T2), wherein m% is miss detection rate, Toffset includes two parts, one is the gap between PO and RO and the other is the gap between RO and next LP-WUS. Obviously, the latency impact caused by MDR is small if dynamic PO is used.

Figure 7. Latency for LP-WUS when miss detection happens

Observation 17: The latency impact caused by miss detection is small if dynamic PO is used.

Proposal 12: For latency evaluation, both legacy PO and dynamic PO should be considered for LP-WUS.

## 4.2 Monitoring mechanism

If the LP-WUS is discontinuously monitored by UE, The latency caused by on-and-off cycle should be added for T1 and T2. Obviously, the latency for on-off duty cycle monitoring mechanism would be larger than that for always on monitoring. Moreover, different duty cycle monitoring schemes have different impacts on the latency. The following table provides the evaluation results of latency of LP-WUS with different monitoring schemes.

Table 3. Latency evaluation

|  | T1 | T2 |
| --- | --- | --- |
| Always on monitoring | 1.38s | 0.45s |
| on-off duty cycle monitoring (duty cycle=1.28s and on duration=16ms) | 1.90s | 1.07s |
| Increased latency (duty cycle vs always-on) | 37% | 137% |

Based on the Table 3, it can be observed:

Observation 18: Based on always-on monitoring and dynamic PO, the minimum latency from data arrival to detection of PO would be as low as 0.45s, and power saving gain also can be achieved.

Observation 19: Compare with always on monitoring, the latency increase of duty cycle monitoring may be up to 137%.

Proposal 13: For latency evaluation, both continuous monitoring and duty cycle monitoring can be considered for LP-WUS.

- 5. Evaluation for coverage

The assumptions for LLS are shown in Table 4.

Table 4. The assumptions for LLS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz for OOK |
| Waveform | OOK |
| Channel structure | For OOK, the following two Options are considered in LLS - Option 2: Sequence only, - Option 3: Payload+CRC, |
| Payload size | 24bits data + 8 bits CRC |
| Sequence | 32-length sequence by the repetition of [0 1 0 1 0 1 0 1] |
| Coder scheme | Manchester coding 1/2 |
| SCS of OFDM generator for NR signal | 30kHz |
| Configuration for LP-WUS signal | For OOK waveform, - Option 2b: M =4 for SCS = 30 kHz (same as NR signal) |
| WUS duration | Based on payload size and coding scheme or the length of preamble sequence |
| MDR/FAR assumption | - The miss-detection rate (MDR) of LP-WUS 1%, - The false-alarm rate (FAR) of LP-WUS:0.1% |
| gNB Channel BW | 20MHz and with Normal CP=2.344us。 |
| LP-WUS BW | Option 1: - 5MHz including subcarriers for guard band - 4.32MHz (i.e.,12 RBs) for LP-WUS transmission for 30kHz SCS - 11 subcarriers with 30KHz SCSs are used for guardband on each side of LP-WUS |
| Filter | 3rd order Butterworth LPF - Filter bandwidth =4.32 MHz, - Cutoff frequency = 4.98/2 MHz |
| Adjacent subcarrier interference | PDSCH with randomly modulated QPSK mapped on resources other than that for WUS and guard band; EPRE of LP-WUS / EPRE of PDSCH =0 dB |
| Sampling Rate | 7.68 MHz |
| ADC bit width | Ideal ADC |
| Channel Model | TDL-C 300ns |
| Number of Rx for LP-WUS | 1 Rx |
| UE speed | 3 km/h |
| Inter-cell interference | No |
| Phase noise modeling | No |
| Oscillator max frequency error | No |
| Oscillator max time error | No |

The initial results are given in Figure 8.

Figure 8. Initial coverage comparison between OOK signal and PUSCH

By referring to the Table 1. MIL for reference channel in TR38.830, and the above figure, we have the following observations and proposal.

Observation 20: Both LP-WUS sequence and sequence+data coverage is worse than PUSCH of msg3 and PDCCH.

Observation 21：For the coverage performance, LP-WUS sequence>LP-WUS data>PUSCH.

Proposal 14: The coverage of LP WUS can be better than PUSCH, and whether coverage enhancement is necessary needs to be further studied.

- Evaluation for system overhead

- 6.1 General aspects

According to the discussion, the system overhead can be expressed as percentage of the used resources for LP-WUS.

| Performance Metric | Note |
| --- | --- |
| System overhead | expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations. |

In frequency domain, the bandwidth for LP-WUS includes the guardband bandwidth and signal bandwidth, which can be donated as NLP-WUS RBs. In time domain, the occupied symbols include the symbols for LP-WUS and guard time if any, where the total occupied symbols can be donated as SLP-WUS. Then one transmission of LP-WUS occupies NLP-WUS*SLP-WUS*12 REs in total.

Then the largest system overhead percentage (only in frequency domain) could be calculated as

PMax=NLP-WUS*12/Nband

where the Nband means the total RBs for a band.

If we consider to extend the above formula in the time domain, then M times transmission of LP-WUS in duration T occupies the total RE resources M*NLP-WUS*SLP-WUS*12. And the percentage can be calculated as

P=M*NLP-WUS*SLP-WUS*12/(Nband*T*12)

Satisfying M*SLP-WUS<=T

In a time duration of T, assuming that M times LP-WUS transmission carries X*M bits information, then the data rate is

R1=X*M/T (bit/symbol)

The peak data rate can be

RMax=X/SLP-WUS (bit/symbol)

Satisfying M*SLP-WUS<=T

The value of M can be related to the paging rate, UE number PF, or PO. Of course, if additional LP-WUS is transmitted, e.g., for sync/measurement, the system overhead also needs to be counted. And the following formula also can be considered. Based on above, we calculate the system overhead percentage P based on the following formula

If each LP-WUS transmission has the same resource occupation, it can be written as

P=M*NLP-WUS*SLP-WUS/(Nband*T)

Where

- Nband means the total RBs for a band or carrier in a cell

- NLP-WUS, i means the number of RBs for ith LP-WUS transmission including the guardband bandwidth and signal bandwidth

- SLP-WUS,i means the number of symbols for ith LP-WUS transmission including guard time if any

- Assuming that LP-WUS is transmitted M times in duration T, FFS how to determine M

Additionally, the system overhead increasement may also be brought by the false wake-up. If the UE is falsely waked up and initiates the RACH procedure without PO monitoring, the resource occupation during the RACH procedure and RRC connection setup would cause additional system overhead. It can be seen that higher false alarm rate would bring more serious system overhead wasting.

Proposal 15: Discuss the LP-WUS transmission assumption in idle/inactive mode for evaluation and capture how to calculate the system overhead in the TR.

- 6.2 Evaluation assumptions

In RAN1 meetings, the following agreements are achieved, which helps for evaluating the system overhead and BS power consumption.

Agreement The period of synchronization signal that LP-WUR used for at least power evaluation can be - Existing SSB periodicity can be used from gNB transmission perspective for evaluations assuming SSB, companies to report how often used for LP-WUR - For evaluations assuming LP-SS - {320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms} - Companies to report other important assumptions if any, e.g., durations of LP-SS to achieve enough T/F accuracy - Other values are not precluded

Agreement Update the IDLE/INACTIVE state traffic model option 1 as follows and remove traffic model option 2, - The traffic arrival is modeled as a Poisson Arrival Process where inter-arrival times are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - FFS: Value of N

According to TR37.910, two different ISD, i.e., 500ms or 1732m, can be assumed for evaluation with the connection target 100000/km2.

Table 7.1.1-1 Evaluation results of connection density for NR FDD (Full buffer system level simulation followed by link level simulation packet arrival rate: 1 packet / 2 hour / device) (a) Evaluation configuration A (ISD=500 m) Scheme and antenna configuration; Sub-carrier spacing; ITU Requirement (device/km2); Channel model A; Channel model B. Number of samples; Connection density (device/km2); Required bandwidth (kHz); Number of samples; Connection density (device/km2); Required bandwidth (kHz). 1x2 SIMO OFDMA; 15 kHz; 1,000,000; 4; 36,323,844; 180; 4; 36,007,832; 180. (b) Evaluation configuration B (ISD=1 732 m) Scheme and antenna configuration; Sub-carrier spacing; ITU Requirement (device/km2); Channel model A; Channel model B. Number of samples; Connection density (device/km2); Required bandwidth (kHz); Number of samples; Connection density (device/km2); Required bandwidth (kHz). 1x2 SIMO OFDMA; 15 kHz; 1,000,000; 4; 1,267,406; 180; 4; 1,503,394; 180.

Moreover, we have defined the paging rate which is corresponding to the packet arrival rate. Based on that, we have the following assumptions for system overhead evaluation.

Table 5. Assumptions for system overhead evaluation

| Connections | 1,000,000 (device/km2) |
| --- | --- |
| System bandwidth | 100M |
| ISD with 3 sectors | 500m, 1732m |
| SCS | 15KHz, 30KHz |
| LP-SS assumption | Periodicity{160ms, 320ms} - Time domain resources:7 symbols - Frequency bandwidth: 5MHz |
| LP-WUS assumption | - Time domain resources:N2 symbols:1, 7, 14 - Frequency bandwidth 5MHz |
| Assumption for UE specific wake-up | - Number of beams:1,4,8 - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28 |
| Assumption for group specific wake-up | - Number of beams:1,4,8 - Number of UEs in a group: 4, 8 |
| Note: If always-on monitoring is assumed, the LP-SS may not be needed |  |

Proposal 16: Consider the above assumptions in the Table 5 for system overhead evaluation.

- 6.3 Initial results

For LP-SS, the resource occupation for different periodicity is calculated based on the following formula

For 15/30KHz SCS case, one slot occupies 1/0.5 ms. The resource occupation rate in duration T is

Table 6. System overhead for LP-SS

| SCS | Periodicity | Number of beams | Bandwidth | P in T=periodicity |
| --- | --- | --- | --- | --- |
| SCS=15KHz | Periodicity=160ms | 4 | 100M, 20M | 0.125%, 0.625%, |
| SCS=15KHz | Periodicity=160ms | 8 | 100M, 20M | 0.25%, 1.25% |
| SCS=15KHz | Periodicity=320ms | 4 | 100M, 20M | 0.0625%, 0.3125% |
| SCS=15KHz | Periodicity=320ms | 8 | 100M, 20M | 0.125%, 0.625% |
| SCS=30KHz | Periodicity=160ms | 4 | 100M, 20M | 0.0625%, 0.3125% |
| SCS=30KHz | Periodicity=160ms | 8 | 100M, 20M | 0.125%, 0.625% |
| SCS=30KHz | Periodicity=320ms | 4 | 100M, 20M | 0.03125%, 0.15625% |
| SCS=30KHz | Periodicity=320ms | 8 | 100M, 20M | 0.0625%,0.3125% |

Based on the Table 6, it can be observed:

Observation 22: For the LP-SS periodicity 160ms, the system overhead is up to 1.25% in a time duration 160ms.

Observation 23: For the LP-SS periodicity 320ms, the system overhead is up to 0.625% in a time duration 320ms.

For LP-WUS, determining number of transmitted LP-WUS could be the key to calculate the system overhead. The following table summarizes the maximum number of transmitted LP-WUS for different cases, which satisfies that the system overhead does not exceed 5% in a time duration T=1.28s.

Table 7. Number of transmitted LP-WUS to satisfy system overhead <=5%

| System bandwidth (MHz) | SCS u=0 or 1 | N2: the number of symbols of LP-WUS | N1: the number of beams | number of wake-up times | number of transmitted LP-WUS M=number of beams* number of wake-up times |
| --- | --- | --- | --- | --- | --- |
| 20 | 0 | 1 | 1 | 3585 | 3585 |
| 20 | 0 | 1 | 4 | 897 | 3588 |
| 20 | 0 | 1 | 8 | 449 | 3592 |
| 20 | 0 | 7 | 1 | 512 | 512 |
| 20 | 0 | 7 | 4 | 128 | 512 |
| 20 | 0 | 7 | 8 | 64 | 512 |
| 20 | 0 | 14 | 1 | 256 | 256 |
| 20 | 0 | 14 | 4 | 64 | 256 |
| 20 | 0 | 14 | 8 | 32 | 256 |
| 20 | 1 | 1 | 1 | 7169 | 7169 |
| 20 | 1 | 1 | 4 | 1793 | 7172 |
| 20 | 1 | 1 | 8 | 897 | 7176 |
| 20 | 1 | 7 | 1 | 1024 | 1024 |
| 20 | 1 | 7 | 4 | 256 | 1024 |
| 20 | 1 | 7 | 8 | 128 | 1024 |
| 20 | 1 | 14 | 1 | 512 | 512 |
| 20 | 1 | 14 | 4 | 128 | 512 |
| 20 | 1 | 14 | 8 | 64 | 512 |
| 100 | 0 | 1 | 1 | 17921 | 17921 |
| 100 | 0 | 1 | 4 | 4481 | 17924 |
| 100 | 0 | 1 | 8 | 2241 | 17928 |
| 100 | 0 | 7 | 1 | 2560 | 2560 |
| 100 | 0 | 7 | 4 | 640 | 2560 |
| 100 | 0 | 7 | 8 | 320 | 2560 |
| 100 | 0 | 14 | 1 | 1280 | 1280 |
| 100 | 0 | 14 | 4 | 320 | 1280 |
| 100 | 0 | 14 | 8 | 160 | 1280 |
| 100 | 1 | 1 | 1 | 35841 | 35841 |
| 100 | 1 | 1 | 4 | 8961 | 35844 |
| 100 | 1 | 1 | 8 | 4481 | 35848 |
| 100 | 1 | 7 | 1 | 5120 | 5120 |
| 100 | 1 | 7 | 4 | 1280 | 5120 |
| 100 | 1 | 7 | 8 | 640 | 5120 |
| 100 | 1 | 14 | 1 | 2560 | 2560 |
| 100 | 1 | 14 | 4 | 640 | 2560 |
| 100 | 1 | 14 | 8 | 320 | 2560 |

Observation 24: For 20M system bandwidth with SCS 15KHz, to satisfy system overhead <5%, with 14 symbols in time domain, the expected number of transmitted LP-WUS is no more than 256 times, including repetition and multi-beam based operation according to the assumptions in Table 7.

Observation 25: For 100M system bandwidth with SCS 15KHz, to satisfy system overhead <5%, with 14 symbols in time domain, the expected number of transmitted LP-WUS is no more than 1280 times, including repetition and multi-beam based operation according to the assumptions in Table 7.

Additionally, M can be obtained based on PO, the expected number of paged UEs could be obtained based on:  reference time duration/one frame*number of POs per PF*number of paged UEs per PO*RE

Table 8. example M based on PO

|  | Per UE | 4 UEs in a group | 8UEs in a group |
| --- | --- | --- | --- |
| 1280/10*4*32*1%=163.84 | 164 | 41 | 21 |
| 1280/10*4*32*0.1%=16.384 | 17 | 11 | 6 |
| 1280/10*4*32*0.01%=1.6384 | 2 | 1 | 1 |
| 1280/10*4*32*0.001%=0.16384 | 1 | 1 | 1 |

Table 9. configurations for system overhead evaluation

| Reference (%) | Final results |
| --- | --- |
| per UE, 15KHz with different paging rate (1%, 0.1%, 0.01%, 0.001%), beams=1, 1 symbol, 100M P= 164*5M*1/14 ms/(100m*1280ms) =0.0458 17*5M*1/14 ms/(100m*1280ms) =0.0047; 5.5804*10^-4 2.7902*10^-4 | where u means the numerology, {0,1} N1 means the number of beams, {1,4,8} N2 means the number of symbols of LP-WUS, {1,7,14} B is the bandwidth and B0 is the reference bandwidth 100M{20,100} G means the number of UEs in a group{1,4,8} H means the number of POs per PF{1,2,4} |

The number of total configurations in table 9 is 2*3*3*2*3*4*3=1296, and the CDF curve is shown as follows:

Figure 9. CDF of system overhead

Observation 26: More than 98% cases has <5% system overhead based on assumptions in Table 9.

Observation 27: Larger bandwidth, larger SCS, less beams, larger number of UEs in a group, less OFDM symbols for LP-WUS can reduce system overhead.

- Evaluation for BS power consumption

- 7.1 Evaluation assumptions

Due to the LP-WUS or LP-SS transmission, it may cause additional network power consumption. It should be carefully considered that the network power saving gain is negated by the LP-WUS transmission. Therefore, evaluation for BS power consumption should be considered. Then the percentage for increased BS power consumption by introducing LP-WUS can be evaluated based on the following formula:

Where r is the percentage for increased BS power consumption by introducing LP-WUS, P2 is the total BS power consumption after introducing LP-WUS and LP-SS if applicable, P1 is the total BS power consumption for baseline scheme without introducing LP-WUS and LP-SS.

More specifically, the following scenarios are considered for NWES.

For evaluation purpose, - a load (L) % of a cell is a percentage of resources used for UE specific PDSCH / PUSCH - The following load scenarios are considered Load scenario; Characteristics. Idle/empty load; - Include cell-specific signals and channels, and - L = 0. low load; - Include cell-specific signals and channels, and - 0 < L≤15. Light load; - Include cell-specific signals and channels, and - 15 < L≤ 30. Medium load; - Include cell-specific signals and channels, and - 30 < L≤ 50. For CA, the companies report whether the load is defined per CC or across all CCs..

Additionally, since the offset between LP-SS and SSB also may have a significant impact on BS power consumption, it also should be determined for BS power consumption evaluation. For example, at least offset=0 should be considered for NW power saving. Whether other offset values are considered can be further discussed. For example, for power boosting, 5ms power offset, similar like NCD-SSB may also be a potential value.

For the LP-WUS transmission, if duty cycle is assumed, the periodicity and the offset also should be considered for evaluating. There are mainly two options for considering.

- For LP-WUS monitoring with duty cycle,

  - Option 1: offset is same with LP-SS, the periodicity may be same or different

  - Option 2: offset is different with LP-SS. The periodicity may be same or different

For option 1, it benefits for UE power saving and NW power saving, since the resources may be continuously transmitted. However, if the LP-WUS is UE specific and different UE share the same monitoring occasion, the collision may happen and the resource for UE multiplexing in frequency domain may be also not enough. Therefore, option2 is also needed. For always-on monitoring, it is flexible for the gNB to transmit the LP-WUS on any occasion.

As for the detailed assumption for LP-WUS transmission, it depends on the paging rate. For example, for 1% paging rate in 1.28s, the periodicity for duty cycle could be 1.28s, 2.56s, 5.12s, 10.24s, 20.48s, 40.96s, 81.92s. as for the offset relative to SSB, it also could be 0, 5, 10 ms.

Table 10. BS power consumption assumptions

| BS assumption | Reuse from NES |
| --- | --- |
| Scenarios | Empty load, low load, light load, medium load |
| SSB transmission | 20ms |
| SCS | 15KHz, 30KHz |
| LP-SS assumption | Periodicity{160ms, 320ms, 640ms} - Time domain resources:7 symbols - Frequency bandwidth: 5MHz Offset with SSB: 0ms, 10ms Number of beams: same as SSB. |
| LP-WUS assumption | - Time domain resources:7 symbols or N2 symbols，e.g.,14symbols - Frequency bandwidth 5MHz - For always-on monitoring, LP-WUS can be transmitted together with SSB or SIB, which means they are continuous or overlapped in time domain. - For duty cycle monitoring, - Periodicity: {0.64s, 1.28s, 2.56s, 5.12s, 10.24s, 20.48s, 40.96s, 81.92s} - Offset with SSB: 0,10ms |
| Assumption for UE specific wake-up | - Number of beams:1,4,8 - Traffic model: - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s - FFS number of UEs or LP-WUS |
| Assumption for group specific wake-up | - Number of beams:4,8 - Number of UEs in a group: 4, 8 |
| Note: If always-on monitoring is assumed, the LP-SS may not be needed and LP-WUS can be transmitted at any slot/symbol. Note: for empty load scenario, no LP-WUS is transmitted and LP-SS is transmitted |  |

Proposal 17: Discuss the evaluation assumptions for BS power consumption

- Whether introduce Offset for LP-SS with SSB.

- Periodicity or offset for duty cycled LP-WUS

- Evaluation for RRC connected mode

In this section, LP WUS used for XR traffic and eMBB traffic in RRC connected mode are evaluated. For SLS of XR, the assumptions are in line with TR 38.838.

- 8.1 Evaluation for XR traffic

In this section, XR DL video traffic of (30Mbps, 60fps) is assumed. The packet size follows truncated Gaussian distribution. Both jitter range of [-4,4]ms and [-8,8]ms are evaluated.

Rel-17 UE power saving schemes including PDCCH skipping and SSSG switching are used to compare with LP WUS scheme.

In LP WUS simulation, Main Radio is in micro sleep state if LP-WUS does not wake up the MR. If LP WUS wakes up the MR, the MR will start a drx-onDurationTimer at the next slot after the detection of LP-WUS. After drx-onDurationTimer and drx-inactivityTimer expire, MR will switch to micro sleep state. In the evaluation, it is assumed drx-onDurationTimer is 4ms and drx-inactivityTimer is 4ms, MR will return to micro sleep state after the active time period. LP WUS is always been monitored when MR is in micro sleep state. The power of LP-WUS is ignored in simulation, because it is negligible compared with the power of micro sleep (45 units). The procedure is shown in Figure 10 as below.

Figure 10 Scheme of LP-WUS simulation

In Rel-17 PS scheme (i.e., PDCCH skipping and SSSG switching) simulation, a SSSG with sparse monitoring occasions (monitor PDCCH every 2 slots) is used in the beginning of DRX onDuration. Once UE detects a data scheduling, UE switches to another SSSG with dense monitoring occasions (i.e., per-slot configuration). When UE finishes data transmission, gNB indicates UE to skip PDCCH monitoring for a duration in the slot that is 2ms after the last packet via dummy DCI. The candidate PDCCH skipping periods are {4ms, 8ms}. CDRX setting (CDRX cycle, drx-onDurationTimer, drx-inactivityTimer) is set to (10, 8, 4). The procedure is shown in Figure 11 as below.

Figure 11 PDCCH skipping and SSSG switching

The simulation results for XR traffic are shown as below.

Table 11 DL only, 30Mbps, 60fps, jitter range = [-4,4]ms, InH

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all UEs (%) | UPT(Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | - | - | - | 11 | 11 | 93.9% | - | 324 |
| LP WUS | / | 5 | 4 | 11 | 11 | 93.9% | 26% | 242 |
| ## PDCCH skipping+ SSSG switching | ## 10 | ## 8 | ## 4 | ## 11 | ## 11 | ## 90.3% | ## 17% | 235 |

Table 12 DL only, 30Mbps, 60fps, jitter range = [-8,8]ms, InH

| Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | #UE /cell | floor (Capacity) | Percentage of satisfied UE | Mean PSG of all UEs (%) | UPT(Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | - | - | - | 11 | 11 | 93.9% | - | 325 |
| LP WUS | / | 5 | 4 | 11 | 11 | 90.18% | 26% | 238 |
| ## PDCCH skipping+ SSSG switching | ## 10 | ## 8 | ## 4 | ## 11 | ## 11 | ## 90.9% | ## 16% | 246 |

According to the simulation results, both LP-WUS and Rel-17 PS scheme have UPT loss. For LP-WUS, the UPT loss is mainly caused by the latency of the processing time for MR to wake up, which is 1 slot in this evaluation. For PDCCH skipping and PDCCH SSSG switching scheme, the UPT loss is mainly caused by the latency caused by sparse PDCCH monitoring and DRX setting.

It is observed that the power saving gain from the power saving techniques in the configurations of jitter range of [-4,4]ms and [-8,8]ms is quite close. The reason is that C-DRX alignment is not considered in the evaluation so that the C-DRX active time will be asynchronized with data arrival for both cases.

LP WUS can provide 26% PSG for XR traffic with 30Mbps in InH. PDCCH skipping and SSSG switching can provide 16%-17% PSG for XR traffic with 30Mbps in InH.

Observation 28: LP-WUS can provide 26% PSG for XR traffic with 30Mbps in InH.

- 8.2 Evaluation for eMBB traffic

In this section, FTP 3 traffic (packet size: 0.5Mbyte, average interval time: 200ms) is used.

For the evaluation of PDCCH skipping-SSSG switching-DCI 2-6 scheme, UE skips a duration of 20ms when the UE finishes a packet reception. After PDCCH skipping duration, UE switches to a SSSG with sparse monitoring occasions (monitors PDCCH every 2 slots). If UE receives a data scheduling, UE switches to a SSSG with dense monitoring occasions (monitors PDCCH every 1 slot).

For LP WUS scheme, if the wake-up delay is 3ms, it is assumed that MR can enter into light sleep. If the wake-up delay is 10ms, MR can enter into deep sleep. Moreover, we assume UE always monitors LP-WUS. If LP-WUS indicates to wake up, MR wakes up after a wake-up delay, and starts a DRX On-duration with 4ms. The inactivity timer of DRX is 2ms.

For Genie scheme, UE power consumption is calculated assuming that UE is in a sleep state (e.g., micro/light/deep sleep as defined in TR38.840) whenever there is neither DL data reception nor UL transmission.

Table 13. Compared with AlwaysOn, evaluation results for FTP 3 traffic for 2 UEs in a cell

| Scheme | relative power of LP-WUR “ON” | PSG (Compared to Baseline) | UPT loss (compared to Baseline) |
| --- | --- | --- | --- |
| Baseline-AlwaysOn | - | - | - |
| CDRX (160-8-100) ms | - | 59.25% | 57.63% |
| PDCCH skipping-switching- DCI 2-6 | - | 73.92% | 69.07% |
| LPWUS (wake-up delay 3ms) | 1 | 71.04% | 42.59% |
|  | 10 | 63.59% |  |
| LPWUS (wake-up delay 10ms) | 1 | 90.02% | 68.69% |
|  | 10 | 82.57% |  |
| Genie |  | 92.04% | - |

According to the evaluation results, the PSG from LP-WUS is close to genie with the UPT loss of 68.69%. And LP WUS has less UPT loss than PDCCH skipping-switching-DCI 2-6 scheme and CDRX scheme.

Observation 29: Compared with always-on scheme, LP-WUS can provide 63.59%-90.02% PSG with 42.59%-68.69% UPT loss for FTP 3 traffic.

Table 14. Compared with CDRX, evaluation results for FTP 3 traffic for 2 UEs in a cell

| Scheme | relative power of LP-WUR “ON” | PSG | UPT gain |
| --- | --- | --- | --- |
| LPWUS (wake-up delay 3ms) | 1 | 28.94% | 35.51% |
|  | 10 | 10.64% |  |
| LPWUS (wake-up delay 10ms) | 1 | 75.52% | -26.11% |
|  | 10 | 57.22% |  |

According to the evaluation results, compared with CDRX scheme, LP WUS has higher PSG and/or higher UPT gain.

Observation 30: Compared with CDRX scheme, LP WUS provides higher PSG and/or higher UPT gain.

- Conclusions

In this contribution, we have discussed evaluation on LP-WUS. We make the following observations and proposals:

Observation 1: If the coverage target for LP-WUS is PDCCH, it would cause more resource overhead.

Observation 2: For 4GHz and 2.6GHz scenario, MIL performance gap between NR bottleneck channel and msg3 is more than 13dB.

Observation 3: For 4GHz and 2.6GHz scenario, MIL performance gap between NR bottleneck channel and PDCCH is more than 19dB.

Observation 4: For always on monitoring, compared with DRX/eDRX with PEI, the maximum power saving gain is 98.14% and 93.6%.

Observation 5: For duty cycled monitoring, compared with DRX/eDRX with PEI, the maximum power saving gain is 99.9% and 99.6%.

Observation 6: For always on monitoring, the LP-WUS of Alt 5/Alt 6 bring significantly more power than DRX/eDRX with/without PEI.

Observation 7: For on-off duty cycled monitoring, compare with DRX with PEI, maximum power saving gain is 31.6%.

Observation 8: For on-off duty cycled monitoring，LP-WUS of Alt 6 has no power saving compared with DRX/eDRX with/without PEI.

Observation 9: With relative power <=0.5, compared with DRX with PEI, if the serving cell measurement is offloaded to LP-SS totally, or the measurement period for serving cell measurement is equal or larger than that for neighbor cell measurement, the always-on monitoring has power saving gain when neighbor cell measurement is relaxed to 9 DRX cycles.

Observation 10: With relative power <=1, compared with DRX with PEI, if the serving cell measurement is offloaded to LP-SS totally, or the measurement period for serving cell measurement is equal or larger than that for neighbor cell measurement, the duty cycled monitoring has power saving gain when neighbor cell measurement is relaxed to 7 DRX cycles

Observation 11: If the serving cell measurement is offloaded to LP-SS totally, and the neighbor cell measurement is performed only after MR is waked up by LP-WUS, then the power saving gain would be quite significant.

Observation 12: If the measurement period for serving cell is less than that for neighbor cell measurement, the minimum number of relaxed DRX cycles for serving cell measurement is limited, and the power saving gain is mainly dominated by MR wake-up via serving cell measurement.

Observation 13: For Mode-1, compared with DRX with PEI, the LP-WUS with relative power<=1 has power saving gain when the probability of extra MR power on caused by FAR is 0.1%/1%.

Observation 14: Compared with DRX/eDRX, the latency reduction of LP-WUS based on legacy PO is -115%/94.7%.

Observation 15: Compared with DRX/eDRX, the latency reduction of LP-WUS based on dynamic PO is 29.6%/98.2%.

Observation 16: Compared with LP-WUS based on legacy PO, the latency reduction of LP-WUS based on dynamic PO is 67.3%.

Observation 17: The latency impact caused by miss detection is small if dynamic PO is used.

Observation 18: Based on always-on monitoring and dynamic PO, the minimum latency from data arrival to detection of PO would be as low as 0.45s, and power saving gain also can be achieved.

Observation 19: Compare with always on monitoring, the latency increase of duty cycle monitoring may be up to 137%.

Observation 20: Both LP-WUS sequence and sequence+data coverage is worse than PUSCH of msg3 and PDCCH.

Observation 21：For the coverage performance, LP-WUS sequence>LP-WUS data>PUSCH.

Observation 22: For the LP-SS periodicity 160ms, the system overhead is up to 1.25% in a time duration 160ms.

Observation 23: For the LP-SS periodicity 320ms, the system overhead is up to 0.625% in a time duration 320ms.

Observation 24: For 20M system bandwidth with SCS 15KHz, to satisfy system overhead <5%, with 14 symbols in time domain, the expected number of transmitted LP-WUS is no more than 256 times, including repetition and multi-beam based operation according to the assumptions in Table 7.

Observation 25: For 100M system bandwidth with SCS 15KHz, to satisfy system overhead <5%, with 14 symbols in time domain, the expected number of transmitted LP-WUS is no more than 1280 times, including repetition and multi-beam based operation according to the assumptions in Table 7.

Observation 26: More than 98% cases has <5% system overhead based on assumptions in Table 9.

Observation 27: Larger bandwidth, larger SCS, less beams, larger number of UEs in a group, less OFDM symbols for LP-WUS can reduce system overhead.

Observation 28: LP-WUS can provide 26% PSG for XR traffic with 30Mbps in InH.

Observation 29: Compared with always-on scheme, LP-WUS can provide 63.59%-90.02% PSG with 42.59%-68.69% UPT loss for FTP 3 traffic.

Observation 30: Compared with CDRX scheme, LPWUS provides higher PSG and/or higher UPT gain.

Proposal 1: For LP-WUS coverage target, option 3 (PUSCH) should be added.

Proposal 2: For LP-SS, separated coverage target should be studied.

Proposal 3: Clarify the latency definition for different RRC states.

- Minimum latency requirement for RRC CONNECTED mode is in the order of milliseconds

- Minimum latency requirement for RRC IDLE/INACTIVE mode is in the order of seconds

Proposal 4: Discuss whether to set a target or minimum requirement for power saving gain, system overhead and NW power consumption.

Proposal 5: The relative power of MR ultra-deep sleep is 0.015 unit for LP-WUS power consumption evaluation.

Proposal 6: For LP-WUS power consumption evaluation, at least 3 SSBs are required for sync/re-sync when main radio wakes up from the state of ultra-deep sleep.

Proposal 7: The relative power of OOK/FSK/OFDM based LP-WUS architecture is suggested as below.

- for OOK/FSK-based receiver, the relative power of WUR on is 0.01,0.1,0.5 or 1 and relative power of WUR off is 0.001.

- for OFDM-based receiver, the relative power of WUR on is 10 or 30 and relative power of WUR off is 0.6.

Proposal 8: For LP-WUS power consumption evaluation, the 10ms ramp-up time of WUR could be used as the starting point.

- FFS the transition energy and ramp-up time for receiver for OFDM based receiver.

Proposal 9: For power evaluation,

- the monitoring window for LP-SS could be assumed as {0.25ms, 0.5ms, 1ms, 2,ms, 4ms} and the monitoring window for SSB/PSS/SSS could be assumed as {2ms, 4ms}.

- the monitoring window for duty cycled LP-WUS could be assumed as {1ms, 2ms, 4ms, 16ms} and the monitoring window for LP-WUS based on OFDM receiver for NR signal could be assumed as {1ms ,2ms}

- the monitoring periodicity for duty cycled LP-WUS could be assumed as 1.28s.

Proposal 10: For receiver for NR signals/channels, if the relative power of WUR off is 0.6 and the transition energy of WUR is calculated by [TLR, ramp-up *(PON-POFF)/2], the power saving gain is not attractive and this case is not further considered.

- FFS: the relative power of WUR off is 0.1 or 0.01

Proposal 11: For RRM measurement evaluation, to obtain the relaxed measurement period,

- Option1: the serving cell measurement and neighbor cell measurement are always performed together.

- Option2: the serving cell measurement and neighbor cell measurement difference for power consumption are clarified.

Proposal 12: For latency evaluation, both legacy PO and dynamic PO should be considered for LP-WUS.

Proposal 13: For latency evaluation, both continuous monitoring and duty cycle monitoring can be considered for LP-WUS.

Proposal 14: The coverage of LP WUS can be better than PUSCH, and whether coverage enhancement is necessary needs to be further studied.

Proposal 15: Discuss the LP-WUS transmission assumption in idle/inactive mode for evaluation and capture how to calculate the system overhead in the TR.

Proposal 16: Consider the above assumptions in the Table 5 for system evaluation.

Proposal 17: Discuss the evaluation assumptions for BS power consumption

- Whether introduce Offset for LP-SS with SSB.

- Periodicity or offset for duty cycled LP-WUS

- References

RP-213645, “New SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#94-e

RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”, vivo, RAN#97-e.

3GPP, RAN1 Chairman’s Notes, RAN1#112bis-e

R1-2304531, LP-WUS receiver architectures, ZTE, RAN1#113