3GPP TSG RAN WG1 #113                                                                R1-2304575

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.9.4

Source:	Spreadtrum Communications

Title:	Discussion on improved GNSS operations for IoT NTN

Document for:	Discussion and decision

Introduction

The WID on IoT-NTN performance enhancements in Rel-18 is approved in [1].

4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

In this contribution, we share our views on improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption.

Schemes for GNSS measurement in connected

In RAN1#112b-e, it was agreed that on when the aperiodic GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot [2].

Agreement On when the aperiodic GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot - FFS: details of X, e.g. predefined value or configured value, considering HARQ feedback for the MAC CE, etc

In the current NB-IoT specification, the application timing of TA adjustment carried in MAC CE for UL transmission is after n+12 DL subframe where the ending of reception of MAC CE in DL subframe n. A similar timing can be applied for the MAC CE that triggers GNSS position fix. UE determines the start of GNSS position fix gap from the end of n+12 DL subframe where n is the end of the MAC CE that triggers GNSS position fix.

Proposal 1: The start time should be at n+ 12, where n is the end of MAC CE receiving subframe/slot.

GNSS measurement triggering in connected

In previous RAN1 meetings, the following agreements on GNSS measurement triggering in connected had been achieved.

Agreement Further study on whether there is a need for potential enhancements on the following for long connection time - UE triggered GNSS measurement. - Network triggered GNSS measurement. Agreement - Support eNB to at least aperiodically trigger UE to make GNSS measurement.

For UE triggered GNSS measurement, UE can send a request for network to trigger GNSS measurement aperiodically. In this way, there are two possible UE behaviors.

- Case 1: After sending a GNSS measurement request, the UE receives a GNSS measurement trigger indication sent by the network. Perform a GNSS measurement based on the received GNSS measurement trigger instruction.

- Case 2: After sending the GNSS measurement request, the UE directly starts the GNSS measurement.

Considering that the network side may not be able to successfully receive the GNSS measurement request message sent by the UE, we prefer that after sending a GNSS measurement request, the UE receives a GNSS measurement trigger indication sent by the network.

Proposal 2: UE can send a request for network to trigger GNSS measurement aperiodically.

Closed Loop control of UL synchronization

In RAN1#112, the following agreement on reducing the number of GNSS fixes required during a connection had been achieved [3].

Agreement At least for the case when frequency error is within frequency error requirements, study the mechanisms and conditions to allow UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration. - FFS: with legacy closed loop time correction or enhanced closed loop time correction - This mechanism is enabled/configured by eNB - FFS: whether such mechanism will be specified depends on the outcome of this study

If the frequency error is within frequency error requirements and the timing error is also within timing error requirements, or the timing error can be adjusted through the existing closed-loop adjustment mechanism, then UE can also perform UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration. The following two methods had been discussed in last meeting.

- Alt 1: the duration is based on remaining timeAlignmentTimer

- Alt 2: the duration is one value/timer configured by network

For Alt 1, the TAC should be indicated to UE before original validity duration expires. In our view, reusing TAT is the simplest way as it reflects whether the UL TA can be regarded as valid. If the duration is one value/timer configured by network, it is hard to determine whether such relaxation can be meet the detection requirement from RAN4 perspective.  For UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration, the duration is one value/timer configured by network.

Proposal 3: If the frequency error is within frequency error requirements and the timing error is also within timing error requirements, or the timing error can be adjusted through the existing closed-loop adjustment mechanism, then UE can also perform UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration.

Proposal 4: For UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration, the duration is one value/timer configured by network.

If the original GNSS validity duration expires, but the frequency error is within frequency error requirements, but the timing error is not within timing error requirements and cannot be adjusted through the existing closed-loop adjustment mechanism, in this case, UE needs to perform GNSS measurement. Therefore, in our opinion, there is no need to enhance the existing closed-loop timing adjustment mechanism

Proposal 5: legacy closed loop time correction does not need to be enhanced.

Conclusion

In this contribution, we have following proposals:

Proposal 1: The start time should be at n+ 12, where n is the end of MAC CE receiving subframe/slot.

Proposal 2: UE can send a request for network to trigger GNSS measurement aperiodically.

Proposal 3: If the frequency error is within frequency error requirements and the timing error is also within timing error requirements, or the timing error can be adjusted through the existing closed-loop adjustment mechanism, then UE can also perform UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration.

Proposal 4: For UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration, the duration is one value/timer configured by network.

Proposal 5: legacy closed loop time correction does not need to be enhanced.

Reference

- RP-223519, “Revised WID on IoT NTN enhancements”, MediaTek Inc.

- 3GPP RAN1#112b-e Chairman’s Notes, April 17th – April 26th, 2023.

- 3GPP RAN1#112 Chairman’s Notes, February 27th – March 3rd, 2023.