3GPP TSG RAN WG1  #113 	    R1-2304703

# Incheon, Korea, May 22nd – May 26th, 2023

Title:	Draft reply LS on 2TA for multi-DCI multi-TRP

Response to: 	R1-2304326/R2-2304342

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	CATT

To:	RAN2

Contact Person:

Name:	Su Xin

Tel. Number:

E-mail Address:	 suxin@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description

RAN1 would like to thank RAN2 for LS R2-2304342 on 2TA for multi-DCI multi-TRP. Please find RAN1 answers on the questions below:

Question 1 on TAG grouping

Q1a:   For the 2TA operation, are there any restrictions on the association of serving cells and/or TRPs to the TAGs?

Answer 1a: No, there is no restriction on the association of serving cells and/or TRPs to the TAGs in RAN1.

Q1b: NR currently supports up to 4 TAGs per cell group. Are the 4 TAGs enough or does RAN1 see a need to increase the number of TAGs per cell group?

Answer 1b: On the basis of answer 1a, since there is no restriction on the association of serving cells and/or TRPs to the TAGs in RAN1, and considering that two TAGs are needed for each serving cell, the 4 TAGs per cell group in current configuration will not be enough and need to be increased.

Question 2 on operation

Q2: When the time-alignment timer associated with one of the TRPs of a serving cell expires, are certain UL or DL operation only impacted towards that TRP while they are not impacted towards the another TRP? If so, which UL or DL operation?

Answer 2: Yes, CATT thinks that only UL or DL operation associated with the TRP of the expired Time-alignment timer are impacted. The impacted DL or UL operation include: UE will release PUSCH/PUCCH/SRS transmission towards the TRP with expired TAG; UE will clear any configured downlink assignments and configured uplink grants towards the TRP with expired TAG; UE will flush HARQ buffers toward the TRP with expired TAG

2. Actions:

To RAN2 group:

ACTION: 	RAN1 respectfully asks RAN2 to take the above answers into consideration in the future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #114			Aug. 21 to Aug. 25, 2023		Toulouse, FR

TSG-RAN WG1 Meeting #114-bis			Oct. 9 to Oct. 13, 2023		Xiamen, CN