3GPP TSG RAN WG1 #113		                                                       R1-2304835

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.16.2

Source: 	Intel Corporation (Rapporteur)

Title: 	UE features for Rel-18 positioning

Document for:	Discussion/Decision

- Introduction

This contribution includes an initial list of RAN1 UE features list for Rel-18 WI on Expanded and Improved NR Positioning considering the decisions up until RAN1 #112bis-e meeting.

NOTE 1: Although some details/rows are marked with brackets and/or with yellow highlighting, all contents should be considered as up for further discussions at least until the first baseline is agreed.

NOTE 2: Any potential consideration of “basic FGs” for SL positioning can be taken up once the FGs are stable, likely not before the first baseline is agreed.

- NR_pos_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41. NR_pos_enh2 | 41-1-1 | Common SL PRS Processing Capability | - Maximum SL PRS bandwidth in MHz, which is supported and reported by UE. FR1 bands: {5, 10, 20, 40, 50, 80, 100} - SL PRS buffering capability: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering - Duration of SL PRS symbols N in units of ms a UE can process every T ms assuming maximum SL PRS bandwidth in MHz, which is supported and reported by UE. T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms - Max number of SL PRS resources that UE can process in a slot FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz |  | No | Yes | UE does not support SL positioning | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. Notes for component 3: A UE reports one combination of (N, T) values per band, where N is a duration of SL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b. A UE is not expected to support SL PRS bandwidth that exceeds the reported SL PRS bandwidth value c. A UE’s SL PRS processing capability is agnostic to SL PRS comb factor configuration d.The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-2 | Receiving SL-PRS in a shared resource pool | UE can receive SL-PRS in a shared resource pool | 15-1, 41-1-1 | Yes | Yes | Receiving SL-PRS in a shared resource pool is not supported | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. | Optional with capability signaling. [UE indicating support of FG 41-1-1 must indicate either this feature group or feature group 41-1-3 is supported.] |
| 41. NR_pos_enh2 | 41-1-3 | Receiving SL-PRS in a dedicated resource pool | - UE can receive SL-PRS in a dedicated resource pool [SL PRS and/or PSCCH]-based RSRP reporting in case of unicast | 15-1, 41-1-1 | Yes | Yes | Receiving SL-PRS in a dedicated resource pool is not supported | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. | Optional with capability signaling. [UE support of FG 41-1-1 must indicate either this feature group or feature group 41-1-2 is supported.] |
| 41. NR_pos_enh2 | 41-1-4 | Transmitting SL-PRS scheme 1 in a shared resource pool | UE can transmit SL-PRS using dynamic scheduling or configured grant type 1 and 2 in NR sidelink positioning scheme 1 scheduled by NR Uu in a shared resource pool | 15-2, 41-1-2 | Yes | Yes | Transmitting SL-PRS scheme 1 in a shared resource pool is not supported | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-5 | Transmitting SL-PRS scheme 1 in a dedicated resource pool | - UE can transmit SL-PRS using dynamic scheduling or configured grant type 1 and 2 in NR sidelink positioning scheme 1 scheduled by NR Uu in a dedicated resource pool Support sidelink pathloss based open loop power control | 15-2, 41-1-3 | Yes | Yes | Transmitting SL-PRS scheme 1 in a dedicated resource pool is not supported | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-6 | Transmitting SL-PRS scheme 2 in a shared resource pool | UE can transmit SL-PRS using NR sidelink positioning scheme 2 configured by NR Uu or preconfiguration in a shared resource pool. | 15-3, 41-1-2 | Yes | Yes | Transmitting SL-PRS scheme 2 in a shared resource pool is not supported | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. | Optional with capability signaling. [For UE supports NR sidelink positioning in a shared resource pool, UE must indicate this FG is supported.] |
| 41. NR_pos_enh2 | 41-1-7 | Transmitting SL-PRS scheme 2 in a dedicated resource pool | UE can transmit SL-PRS using NR sidelink positioning scheme 2 configured by NR Uu or preconfiguration in a dedicated resource pool. | 15-3, 41—1-3 | Yes | Yes | Transmitting SL-PRS scheme 2 in a dedicated resource pool is not supported | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. | Optional with capability signaling. [For UE supports NR sidelink positioning in a dedicated resource pool, UE must indicate this FG is supported.] |
| 41. NR_pos_enh2 | 41-1-8 | SL-PRS congestion control in a dedicated resource pool | - UE can report CBR measurement to gNB when operating in Mode 1 and mode 2 - UE can adjust its radio parameters based on CBR measurement and CRlimit. UE can process CBR and CR within the time it indicates | 41-1-3 and either 41-1-5 or 41-1-7 | Yes | Yes | SL-PRS congestion control in a dedicated resource pool is not supported | Per band | N.A. | N.A. | N.A. |  | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-9 | SL PRS Resources for SL-RSTD | TBD on SL PRS resource and resource multiplexing final agreement | 41-1-2 or 41-1-3 | No | Yes |  | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. Note: if the UE does not indicate this capability for a band, the UE does not support this positioning method in this band. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-10 | SL PRS Resources for SL-TDOA | TBD on SL PRS resource and resource multiplexing final agreement | 41-1-2 or 41-1-3 | No | Yes |  | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. Note: if the UE does not indicate this capability for a band, the UE does not support this positioning method in this band. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-11 | SL PRS Resources for SL-AOA | TBD on SL PRS resource and resource multiplexing final agreement | 41-1-2 or 41-1-3 | No | Yes |  | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. Note: if the UE does not indicate this capability for a band, the UE does not support this positioning method in this band. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-12 | SL PRS Resources for Multi-RTT | TBD on SL PRS resource and resource multiplexing final agreement | 41-1-2 and (41-1-4 or 41-1-6) or 41-1-3 and (41-1-5 or 41-1-7) | No | Yes |  | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. Note: if the UE does not indicate this capability for a band, the UE does not support this positioning method in this band. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-13 | SL PRS measurement report for SL-RSTD | - SL-RSTD measurements per pair of UEs (target and reference UE). Values = TBD - Support SL PRS-RSRP measurements. Values = {0, 1} - Support SL PRS-RSRPP repoting for first path. Values = {0, 1} - Support LOS/NLOS indication. Values = {0, 1}. TBD: LOS/NLOS indicator type/granularity indication | 41-1-2 or 41-1-3 | No | Yes |  | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-14 | SL PRS measurement report for SL-TDOA | - SL-RTOA measurements per UE. Values = TBD - Support SL PRS-RSRP measurements. Values = {0, 1} - Support RSRPP repoting for first path. Values = {0, 1} - Support LOS/NLOS indication. Values = {0, 1}. TBD: LOS/NLOS indicator type/granularity indication | 41-1-2 or 41-1-3 | No | Yes |  | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-15 | SL PRS measurement report for SL-AOA | - SL-AOA measurements per UE. Values1 = TBD (e.g., # of measurements). Values2 = {Azimuth, Zenith, Azimuth and Zenith}; Values3 = {GCS, LCS}; - Support SL PRS-RSRP measurements. Values = {0, 1} - Support RSRPP repoting for first path. Values = {0, 1} - Support LOS/NLOS indication. Values = {0, 1}. TBD: LOS/NLOS indicator type/granularity indication | 41-1-2 or 41-1-3 | No | Yes |  | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-16 | UE Rx-Tx measurement report for SL Multi-RTT | - Max number of UE Rx–Tx time difference measurements corresponding to a single SL PRS Tx resource with each measurement corresponding to a single SL PRS Rx resource. - Value for component 1: [{1,2,3,4}] - Support RSRP measurements. Values = {0, 1} - Support RSRPP repoting for first path. Values = {0, 1} - Support LOS/NLOS indication. Values = {0, 1}. TBD: LOS/NLOS indicator type/granularity indication | 41-1-2 and (41-1-4 or 41-1-6) or 41-1-3 and (41-1-5 or 41-1-7) | No | Yes |  | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-1-17 | SL PRS reception in case of TDM-based multiplexing of SL PRS from different UEs in a slot | SL PRS reception in case of TDM-based multiplexing of SL PRS from different UEs in a slot at least for dedicated resource pools | 41-1-3 | No | Yes |  | Per band | N.A. | N.A. | N.A. | Need for location server or server UE to know if the feature is supported. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-2-1 | Support DL RSCP reporting based on DL PRS | DL RSCP is reported along with UE Rx-Tx Measurement Report for Multi-RTT | 13-11 | No | N.A. |  | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-2-2 | Support of DL RSCPD reporting based on DL PRS | DL RSCPD is reported along with measurement report for DL-RTOA | 13-6 | No | N.A. |  | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-2-3 | Support of DL RSCP reporting based on DL PRS in RRC_INACTIVE | DL RSCP is reported along with UE Rx-Tx Measurement Report for Multi-RTT | 27-18c | No | N.A. |  | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-2-4 | Support of DL RSCPD reporting based on DL PRS in RRC_INACTIVE | DL RSCPD is reported along with measurement report for DL-RTOA | 27-18a | No | N.A. |  | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-2-5 | DL PRS processing capabilities for DL RSCP or DL RSCPD measurements | 1. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE | 13-1 |  | N.A. |  | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. Component 1 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-2-6 | DL PRS processing capabilities for DL RSCP or DL RSCPD measurements in RRC_INACTIVE | 1. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE | 27-6 |  | N.A. |  | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. Component 1 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-2-7 | Measurements on DL PRS on [indicated] DL RS resources occuring within idnicated time windows | Supported measurements on DL PRS on [indicated] DL RS resources occuring within idnicated time windows. Values = [{RSTD, RSRP, RSRPP, RSCP, RSCPD}] | 13-1 | No | N.A. |  | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. FFS: Conditioning based on reported capabilities for different UE-assisted DL positioning methods | Optional with capability signaling. |
| 41. NR_pos_enh2 | 41-3-1 | Support of SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state | - SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state with common configuration of SRSPos resource set and SRSPos resources. - SRS for positioning validity area-specific TA timer - Spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state may be present or absent in the configuration. - For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, pathloss RS may be present or absent in the configuration. - UL timing - TBD | 27-15 | Yes | N.A. |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| 41. NR_pos_enh2 | 41-4-1 | Support of PRS bandwidth aggregation in RRC_CONNECTED | - Support of PRS bandwidth aggregation across multiple PFLs in RRC_CONNECTED - Applicable for DL-TDOA and Multi-RTT positioning methods | 13-1 | No |  | PRS bandwidth aggregation across multiple PFLs in RRC_CONNECTED state is not supported | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-2 | Support of PRS bandwidth aggregation in RRC_INACTIVE | - Support of PRS bandwidth aggregation across multiple PFLs in RRC_INACTIVE - Applicable for DL-TDOA and Multi-RTT positioning methods | 27-18 | [Yes] |  | PRS bandwidth aggregation across multiple PFLs in RRC_INACTIVE state is not supported | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-3 | Support of PRS bandwidth aggregation in RRC_IDLE | - Support of PRS bandwidth aggregation across multiple PFLs in RRC_IDLE - Applicable for DL-TDOA and Multi-RTT positioning methods | TBD | TBD |  | PRS bandwidth aggregation across multiple PFLs in RRC_IDLE state is not supported | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-4 | Support of positioning SRS bandwidth aggregation in RRC_CONNECTED | - Support of positioning SRS bandwidth aggregation across intra-band contiguous carriers in RRC_CONNECTED - Applicable for UL-TDOA and Multi-RTT positioning methods | 13-8, 6-2 | Yes |  | Positioning SRS bandwidth aggregation across intra-band contiguous carriers in RRC_CONNECTED state is not supported | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-5 | Support of positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED | - Support of positioning SRS bandwidth aggregation in RRC_CONNECTED that is decoupled from the UE support of communication CA - Applicable for UL-TDOA and Multi-RTT positioning methods | 13-8 | Yes |  | Positioning SRS bandwidth aggregation in RRC_CONNECTED state that is decoupled from the UE support of communication CA is not supported | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-4-6 | Support of positioning SRS bandwidth aggregation in RRC_INACTIVE | - Support of positioning SRS bandwidth aggregation across intra-band contiguous carriers in RRC_INACTIVE - Applicable for UL-TDOA and Multi-RTT positioning methods | 27-15 | Yes |  | Positioning SRS bandwidth aggregation across intra-band contiguous carriers in RRC_INACTIVE state is not supported | Per band | N.A. | N.A. | N.A. | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-5-1 | Support of PRS with Rx frequency hopping for RedCap UEs | - Support of PRS with Rx frequency hopping for Redcap UEs - For FR1, switching time of [{70us, 140us}] and PRS Rx frequency hopping range up to 100MHz - For FR2, switching time of [{35us, 70us, 140us}] and PRS Rx frequency hopping range up to 400MHz | 13-1 | Yes |  | PRS with Rx frequency hopping for Redcap UEs is not supported | Per band | N.A. | N.A. | N.A. |  | Optional with capability signaling |
| 41. NR_pos_enh2 | 41-5-2 | Support of positioning SRS with Tx frequency hopping for RedCap UEs | - Support of positioning SRS with Tx frequency hopping for Redcap UEs - For FR1, switching time of [{70us, 140us}] and positioning SRS Tx frequency hopping range up to 100MHz - For FR2, wwitching time of [{35us, 70us, 140us}] and positioning SRS Tx frequency hopping range up to 400MHz | 13-8 | Yes |  | Positioning SRS with Tx frequency hopping for Redcap UEs is not supported | Per band | N.A. | N.A. | N.A. |  | Optional with capability signaling |