3GPP TSG RAN WG1 #113			R1-2304906

Incheon, Korea, May 22nd – May 26th, 2023

Source: 	     Xiaomi

Title:	Discussion on sidelink positioning reference signal

Agenda item: 	9.5.1.1

Document for:  Discussion

Introduction

In RAN#98-e meeting, the WID on expanded and improved NR positioning has been approved [1], including the following objectives:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]:

  - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1].

    - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum.

    - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified.

  - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

  - Specify support of resource allocation for SL PRS:

    - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1].

      - For resource allocation mechanism for SL PRS in Scheme 2:

        - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1].

        - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1].

    - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1].

      - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

  - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1].

  - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

  - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]:

    - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)).

    - Specify the protocol and procedures for SL positioning between UEs and LMF.

  - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2]

  - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, the issues and solutions on design of sidelink positioning reference signal is discussed, and our opinions are given.

Sidelink reference signal design

SL PRS SN design

In RAN1#112bis-e meeting, the following agreement was made:

Agreement For SL PRS sequence generation, one of the following options is down-selected to define the parameter nID,seqSL-PRS nID,seqSL-PRS: - Option 1:nID,seqSL-PRS nID,seqSL-PRS is a higher layer parameter. - FFS: How the higher layer parameter is obtained, e.g., (pre-)configuration or via LPP/SLPP, etc. - Option 2:nID,seqSL-PRS nID,seqSL-PRS is based on 12 LSB bits CRC of PSCCH associated with the SL PRS. - Option 3: based on a combination of higher layer parameter from a configured ID list and 12 LSB bits of CRC of PSCCH associated with the SL PRS. - FFS: How the higher layer parameter/ID list is determined/obtained, e.g., (pre-)configuration or via LPP/SLPP, etc.

For DL PRS signal, the initialization parameter value is determined by higher layer configurations; while for SL CSI-RS signal, the initialization parameter value is determined by the CRC of associated PSCCH. The reason for using CRC of associated PSCCH is to guarantee sufficient randomization even when the UEs are out of coverage (where centralized coordination is not possible). Even if two UEs selects the same SL PRS resource for  transmission, as the content of the associated PSCCH/SCI cannot be the same, the initialization parameter of SL PRS SN can still be different. Furthermore, even if the TX UE is in NW coverage, TX UEs can be in RRC idle state, and the RX UE may be out of coverage and thus cannot directly receive configuration from gNB. Without a centralized coordinator such as gNB or LMF, the configuration value can be the same and collision may happen when the same SL PRS resource is selected. Therefore, the solution used for SL CSI-RS shall be reused to mitigate potential interference by randomization, that is, nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission.

One of the concerns on option 2 is that it may have privacy or security issue as all the neighbouring UEs can know the value of initialization parameter by decoding the SCI, which is not cyphered. Knowing the parameter, a UE which is not the intended SL-PRS receiver would be able to perform SL PRS measurement on the SL PRS signal. However, physical layer security has not been supported in SA3 requirement, there is no privacy or security issue if a third party knows there is a UE located nearby but does not know which UE it is. Therefore, we propose:

Proposal 1: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission

SL PRS bandwidth

For DL PRS, the same DL PRS bandwidth is configured for the same frequency layer. However, for SL positioning, the accuracy requirement may be different depending on various of services and application scenarios. For example, for V2X use case, a pedestrian UE may not always transmit SL PRS with full bandwidth to save energy consumption; for commercial use case, a ranging measurement for direction may not need to use full bandwidth SL PRS signal transmission. Therefore, supporting SL PRS transmission with flexible bandwidth in the same SL PRS resource pool would be beneficial.

For dedicated resource pool, the granularity of frequency domain resource allocation for SL PRS transmission can be defined as PRS subchannel, similar as the subchannel defined for SL communication pool. A UE can transmit SL PRS in one or multiple contiguous PRS subchannels. The subchannel size defined for SL communication can be the starting point. By reusing the concept of subchannel, the candidate resource concept in Mode 2 resource allocation can be reused for scheme 2 SL PRS resource allocation; and the frequency domain resource indication used in SCI format 1-A can be maximally reused as well. Note that full bandwidth only SL PRS can also be supported by configuring the subchannel size to be the same as the bandwidth of the resource pool.

Comparing setting multiple SL PRS resource pools with different SL PRS bandwidths, a resource pool with flexible SL PRS bandwidth can achieve better resource utilization efficiency without further resource fragmentation. In addition, congestion control can be used to enable large SL PRS bandwidth when the congestion level is low, and reduce the resource utilization by limiting the usable SL PRS bandwidth when the congestion level is high.

Proposal 2: SL PRS transmission with flexible bandwidth is supported in both dedicated resource pool

- PRS subchannel is defined as the frequency domain resource allocation granularity

SL PRS resource in a dedicated resource pool

In RAN1#112bis-e, SL PRS resource is defined to uniquely identify a resource for SL PRS transmission within a slot in a dedicated resource pool:

Agreement - A SL PRS resource refers to a time-frequency resource within a slot of a dedicated SL PRS resource pool that is used for SL PRS transmission. - FFS: for a shared resource pool - Characteristics associated with a SL PRS resource include at least: - SL PRS resource ID, - SL PRS comb offset and associated SL PRS comb size (N), - SL PRS starting symbol and number of SL PRS symbols (M), - SL PRS frequency domain allocation, - Note: Additional parameters can be included as/when identified. - FFS: other time domain aspects, if any - A SL PRS resource is identified by a SL PRS resource ID that is unique within a slot of a dedicated SL PRS resource pool. NOTE 1: The above does not imply need for signalling/(pre-)configuration of all these parameters

The comb size of SL PRS shall be (pre)configured for a dedicated resource pool. If different comb sizes can be used by different UEs in the same dedicated resource pool, SL PRS signals may have collisions on only part of REs in a PRB, which would greatly increase the complexity of SL PRS resource allocation.

The set of starting symbols of SL PRS in slot shall also be (pre)configured in a dedicated resource pool. If different UEs can dynamically select the starting symbol, there will be AGC issue at the receiving UE side. The number of OFDM symbols for a SL PRS resource can be inferred from the starting symbol positions in a dedicated resource pool, which is the number of symbols between two adjacent starting symbol positions, excluding the AGC and gap symbol(s) if there exists.

Proposal 3: The comb size, starting symbol(s) in a slot and number of symbols for a SL PRS transmission is determined by (pre)configuration in a SL PRS dedicated resource pool.

In RAN1#112bis-e, the following agreement is made:

Agreement For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot. - FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH

Since there is no 2nd stage SCI for SL PRS in a dedicated resource pool, there is no need to support PSSCH channel in a dedicated resource pool. Only PSCCH and SL PRS signal need to be considered in a dedicated resource pool. In addition, as the information in SCI will be useful for decoding of the associated SL PRS, PSCCH shall occupy the first several symbols of the slot.

Proposal 4: Only PSCCH and SL PRS need to be supported in a dedicated SL PRS resource pool.

- PSCCH occupies the first symbol(s) of the slot, following the SL PRS resource(s)

In a slot of dedicated resource pool, a PSCCH conveying the single stage SCI can have a much smaller bandwidth compared to the associated SL PRS transmission in the same slot. As the PSCCH and the associated SL PRS are TDMed in a slot, there are two options for the transmission of PSCCH and the associated SL PRS:

Option 1: same TX power for PSCCH and SL PRS;

As the bandwidth of PSCCH and SL PRS can be different, the PSD of the transmission of PSCCH and SL PRS will be different. To adapt to different PSD of transmission, a gap symbol between PSCCH and the time domain adjacent SL PRS resource is needed.

Option 2: same PSD for PSCCH and SL PRS;

As the bandwidth of PSCCH and SL PRS can be different, the overall TX power of the transmission of PSCCH and SL PRS will be different. To solve the AGC issue of different transmission power of PSCCH and SL PRS, an AGC symbol between PSCCH and the time domain adjacent SL PRS resource is needed.

Proposal 5: RAN1 down-selects between the following two options:

- An AGC symbol is used between PSCCH and the time domain adjacent SL PRS resource if the same TX PSD for PSCCH and the associated SL PRS is assumed in a slot

- A gap symbol is used between PSCCH and the time domain adjacent SL PRS resource if the same TX power of PSCCH and the associated SL PRS is assumed in a slot.

In a slot of dedicated resource pool, if there are more than 1 time domain multiplexing SL PRS resource, two options for PSCCH resources can be considered:

Option 1: If different SL PRS resources are TDMed in a slot, the associated PSCCHs are also TDMed in a slot;

Option 2: The PSCCHs are always FDMed in a slot;

For option1, multiple TDMed PSCCHs need to be supported in a slot, which would introduce additional AGC symbols and gap symbols. The resource efficiency would be degraded compared with option2. Therefore, option 2 is more preferred. The number of PRBs and OFDM symbols occupied by a single PSCCH can be (pre)configured in a dedicated resource pool to accommodate sufficient number of PSCCH resources.

Proposal 6: For SL PRS dedicated resource pool, the PSCCHs are always FDMed in a slot.

- The number of PRBs and the number of OFDM symbols of a PSCCH resource in a resource pool is (pre)configured.

In addition, if there are multiple TDM SL PRS resources in a slot, a SL PRS TX UE may transmit only in part of the TDM SL PRS resources. Therefore, at least an AGC symbol shall be inserted between the time domain adjacent SL PRS resources.

Proposal 7: If there are multiple time domain multiplexing SL PRS resources, at least an AGC symbol is used between time domain adjacent SL PRS resources.

An example of PSCCH and SL PRS resources with and without TDMed SL PRS resource in a slot can be seen as in Figure 1.

Figure 1: example of slot structure for SL PRS dedicated resource pool

SL PRS resource in a Shared resource pool

For shared resource pool, SL PRS can be transmitted together with SL PSCCH/PSSCH transmission, as shown in Figure 2. SL PRS has the same bandwidth as the PSCCH/PSSCH transmission of SL communication, and thus the guard symbol and AGC symbol are not needed.

Figure 2 Example of slot structure for shared resource pool of SL PRS

Proposal 8: For shared resource pool design, SL PRS transmission is transmitted together with SL PSCCH/PSSCH transmission in a slot.

- The bandwidth of SL PRS is the same as that of PSSCH

Open loop power control for SL PRS transmission

For dedicated resource pool for SL positioning, an independent set of power control parameters can be (pre)configured for SL PRS transmission. Similar as SL communication, DL based open loop power control and SL based open loop power control can be supported. To simplify the specification impact, SL based open loop power control for SL PRS is only supported for SL PRS unicast transmission. The pathloss for SL based open loop power control is estimated based on the RSRP measurement reported from the receiver UE. The RSRP report from the receiver UE can be based on the measurements from SL PRS transmitted from the transmit UE.

Proposal 9: For SL based open loop power control, only unicast of SL PRS is supported

Proposal 10: the RSRP report used in SL based open loop power control is measurement obtained from SL PRS reception.

For shared resource pool for SL positioning, the transmission power of a given SL transmission shall remain constant across different OFDM symbols. Otherwise, some transition duration or AGC symbol may be necessary, which will increase the overhead of SL communication. Therefore, the power of SL PRS transmission shall follow the power control of PSSCH. The PSD of SL PRS transmission shall be the same as that of PSSCH which is transmitted together with SL PRS signal.

Proposal 11: For SL transmission in a shared resource pool, the transmission power of SL PRS follows the transmission power of the PSSCH to keep constant TX power in all the OFDM symbols.

Conclusion

In this contribution, the design of sidelink PRS signal is discussed. Based on the discussion, the following proposals are proposed:

Proposal 1: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission

Proposal 2: SL PRS transmission with flexible bandwidth is supported in both dedicated resource pool

- PRS subchannel is defined as the frequency domain resource allocation granularity

Proposal 3: The comb size, starting symbol(s) in a slot and number of symbols for a SL PRS transmission is determined by (pre)configuration in a SL PRS dedicated resource pool.

Proposal 4: Only PSCCH and SL PRS need to be supported in a dedicated SL PRS resource pool.

- PSCCH occupies the first symbol(s) of the slot, following the SL PRS resource(s)

Proposal 5: RAN1 down-selects between the following two options:

- An AGC symbol is used between PSCCH and the time domain adjacent SL PRS resource if the same TX PSD for PSCCH and the associated SL PRS is assumed in a slot

- A gap symbol is used between PSCCH and the time domain adjacent SL PRS resource if the same TX power of PSCCH and the associated SL PRS is assumed in a slot.

Proposal 6: For SL PRS dedicated resource pool, the PSCCHs are always FDMed in a slot.

- The number of PRBs and the number of OFDM symbols of a PSCCH resource is resource pool based (pre)configured.

Proposal 7: If there are multiple time domain multiplexing SL PRS resources, at least an AGC symbol is used between time domain adjacent SL PRS resources.

Proposal 8: For shared resource pool design, SL PRS transmission is transmitted together with SL PSCCH/PSSCH transmission in a slot.

- The bandwidth of SL PRS is the same as that of PSSCH

Proposal 9: For SL based open loop power control, only unicast of SL PRS is supported

Proposal 10: the RSRP report used in SL based open loop power control is measurement obtained from SL PRS reception.

Proposal 11: For SL transmission in a shared resource pool, the transmission power of SL PRS follows the transmission power of the PSSCH to keep constant TX power in all the OFDM symbols.

# Reference

[1] RP-223549, Expanded and Improved NR Positioning, Intel, RAN#98-e, Dec. 2022.