3GPP TSG RAN WG1 #113			R1-2305275

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.16.1

Source: 	Apple Inc.

Title: 	Views on UE features for Rel-18 NR MIMO evolution

Document for:	Discussion/Decision

- Views on UE features for Rel-18 NR MIMO evolution

In this contribution, we present our high level views on UE features for Rel-18 NR MIMO evolution. Note that, since RAN1#113 is the first meeting to discuss UE features for Rel-18 NR MIMO evolution and we still have 2 meetings left before RAN1 Rel-18 funtional freeze, UE features in this contribution focus on the high level FG definition and components definition with some placeholder pending further RAN1 discussion. At high level, the FGs are organized in the following way to handle different objectives/agendas in NR Rel-18 MIMO evolution.

- FG40-1x covers the extension of unified TCI framework for multi-TRP.

- FG40-2x covers the two TAs for multi-DCI multi-TRP.

- FG40-3x covers the CSI enhancement.

  - FG40-3-1x covers Type-II codebook refinement for multi-TRP CJT (Coherent Joint Transmission)

  - FG40-3-2x covers Type-II codebook refinement exploiting channel time domain properties

  - FG40-3-3x covers TDCP (Time Domain Channel Properties) report

- FG40-4x covers the increased number of DMRS ports.

  - FG40-4-1x covers the increased number of DMRS ports for DL with CP-OFDM

  - FG40-4-2x covers the increased number of DMRS ports for UL with CP-OFDM

- FG40-5x covers the SRS enhancement

  - FG40-5-1x covers the SRS comb offset/cyclic shift hopping

  - FG40-5-2x covers the TDM SRS ports

  - FG40-5-3x covers the 8 SRS ports

- FG40-6x covers the STxMP (Simultaneous Transmission across Multiple Panels) enhancement

  - FG40-6-1x covers the single-DCI SDM STxMP PUSCH

  - FG40-6-2x covers the single-DCI SFN STxMP PUSCH

  - FG40-6-3x covers the multi-DCI STxMP PUSCH

  - FG40-6-4x covers the single-DCI SFN STxMP PUCCH

- FG40-7x covers 8Tx UL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-1-1 | Unified TCI with joint DL/UL TCI framework extension for single-DCI intra-cell multi-TRP operation | - The maximum number of configured joint TCI state per BWP per CC in a band. - Two MAC-CE activated joint TCI-states per CC in a band. - MAC-CE based TCI-state indication for two active TCI-states. - The maximum number of MAC-CE activated joint TCI-states across all CC(s) in a band. - The maximum number of MAC-CE activated joint TCI states per CC in a band |  |  |  |  |  |  |  |  | Component 1 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate value {2, 4, 8, 16, 32} Component 5 candidate value {2, 4, 8, 10, 12, 14, 16} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-1a | Unified TCI with joint DL/UL TCI update for multi-DCI intra-cell and inter-cell multi-TRP operation | - Support unified TCI with joint DL/UL TCI update for multi-DCI intra-cell and inter-cell multi-TRP operation - The maximum number of configured joint TCI state per BWP per CC in a band. - One MAC-CE activates one joint TCI-states per CC in a band for a TRP assocaited with a ‘CORESETpoolIndex’ value. - The maximum number of MAC-CE activated joint TCI-states across all CC(s) per ‘CORESETpoolIndex’ value. - The maximum number of MAC-CE activated joint TCI states per CC in a band per ‘CORESETpoolIndex’ value |  |  |  |  |  |  |  |  | Component 1 candidate values {Intra-cell, intra-cell and inter-cell} Component 2 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate value {1, 2, 4, 8, 16} Component 5 candidate value {1, 2, 4, 8 } | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-2 | Unified TCI with separate DL/UL TCI framework extension for single-DCI intra-cell multi-TRP operation | - The maximum number of configured separate DL TCI state per BWP per CC in a band. - The maximum number of configured separate UL TCI state per BWP per CC in a band. - Two MAC-CE activated DL TCI-states per CC in a band. - Two MAC-CE activated UL TCI-states per CC in a band. - MAC-CE based TCI-state indication. - The maximum number of MAC-CE activated DL TCI states across all CC(s) in a band - The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band - The maximum number of MAC-CE activated DL TCI states per CC in a band - The maximum number of MAC-CE activated UL TCI states per CC in a band |  |  |  |  |  |  |  |  | Component 1 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 6 candidate value {2, 4, 8, 16, 32} Component 7 candidate value {2, 4, 8, 16, 32} Component 8 candidate value {2, 4, 8, 10, 12, 14, 16} Component 9 candidate value {2, 4, 8, 10, 12, 14, 16} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-2a | Unified TCI with separate DL/UL TCI update for multi-DCI intra-cell and inter-cell multi-TRP operation | - Support unified TCI with separate DL/UL TCI update for multi-DCI intra-cell and inter-cell multi-TRP operation - The maximum number of configured separate DL TCI state per BWP per CC in a band. - The maximum number of configured separate UL TCI state per BWP per CC in a band. - One MAC-CE activated DL TCI-state per CC in a band for a TRP assocaited with a ‘CORESETpoolIndex’ value. - One MAC-CE activated UL TCI-state per CC in a band for a TRP assocaited with a ‘CORESETpoolIndex’ value. - The maximum number of MAC-CE activated DL TCI states across all CC(s) in a band per ‘CORESETpoolIndex’ value - The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band per ‘CORESETpoolIndex’ value - The maximum number of MAC-CE activated DL TCI states per CC in a band per ‘CORESETpoolIndex’ value - The maximum number of MAC-CE activated UL TCI states per CC in a band per ‘CORESETpoolIndex’ value |  |  |  |  |  |  |  |  | Component 1 candidate values {Intra-cell, intra-cell and inter-cell} Component 2 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 3 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 6 candidate value {1, 2, 4, 8, 16} Component 7 candidate value {1, 2, 4, 8, 16} Component 8 candidate value {1, 2, 4, 8} Component 9 candidate value {1, 2, 4, 8} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-3 | Unfied TCI update for CJT (Coherent Joint Transmission) PDSCH transmission scheme. | - Support of unified TCI with joint TCI update for CJT (Coherent Joint Transmission) PDSCH transmission scheme - Maximum number of DL TCI states that can be indicated by MAC-CE/DCI and applied to CJT PDSCH reception in a BWP/CC configured with joint DL/UL TCI mode. |  |  |  |  |  |  |  |  | Component 1 candidate value: {Joint UL/DL, Separate UL/DL, both} Component 2 candidate value: {1, 2 } | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-4 | Common multi-CC joint UL/DL TCI state ID update and activation for multi-TRP operation | - Common multi-CC TCI state ID update and activation with joint TCI upate. - The multi-TRP mode for CCs in a common list. - Maximum number of configured CC lists per cell group for common multi-CC TCI state ID activation/update. |  |  |  |  |  |  |  |  | Component 2 candidate value: {single-DCI multi-TRP only, multi-DCI multi-TRP only} Component 3 candidate value: {1, 2, ,3, 4 } | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-5 | Aperiodic CSI-RS resource QCL configuration with unified TCI framework extension for Single-DCI based Multi-TRP | - Supported configuration of mode, on unified TCI framework extension for Single-DCI based Multi-TRP, RRC configuration in CSI-AssociatedReportConfigInfo of CSI-AperiodicTriggerState in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state. |  |  |  |  |  |  |  |  | Component 1 candidate value: {per CSI-RS resource set, per CSI-RS resource, both } | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-6 | Unified TCI framework for Single-DCI Multi-TRP, spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission | - For unified TCI framework for Single-DCI Multi-TRP, support of spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission that is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission. |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-7 | Unified TCI framework for Single-DCI Multi-TRP, DCI format 1_1/1_2 configured without the [TCI selection field] | - For unified TCI framework for Single-DCI Multi-TRP, support of DCI format 1_1/1_2 configured without the [TCI selection field] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2 | 2 TA for multi-DCI multi-TRP | - Support of 2 TA/TAG within a serving cell for multi-DCI multi-TRP - Rx timing difference between the two DL reference timings is no larger than CP length - The UL/joint TCI states of UL signals/channels associated to one CORESET Pool Index correspond to one TAG |  |  |  |  |  |  |  |  | Component 1 candidate value: {intra-cell multi-TRP, inter-cell multi-TRP, both} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-1 | Maximum number of n-TimingAdvanceOffset value per serving cell | - Maximum number of n-TimingAdvanceOffset value supported per serving cell |  |  |  |  |  |  |  |  | Component 1 candidate value: {1, 2 } | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-2 | Maximum Rx timing difference between the two DL reference timings larger than CP | - Support of maximum Rx timing difference between the two DL reference timings larger than CP |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-3 | Activated UL/joint TCI states[of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs | - Support of activated UL/joint TCI states[of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-4 | For intra-cell multi-DCI based Multi-TRP operation with two TA, a PDCCH order sent by TRP X triggers RACH procedure towards a different TRP Y | - For intra-cell multi-DCI based Multi-TRP operation with two TA, support of a PDCCH order sent by TRP X triggers RACH procedure towards a different TRP Y |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-5 | The maximum number of TAGs all CCs | - Maximum number of TAGs that can be supported across all intra-frequency and inter-frequency CCs |  |  |  |  |  |  |  |  | Component 1 candidate value: { 2, 3, 4[, 5, 6, 7, 8]} Note: the UE only supports the configuration where all UL CCs of the same frequency band are configured with up to 2 Timing Advance Group ID. | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a | Rel-16 eType-II regular codebook refinement for multi-TRP CJT (Coherent Joint Transmission) | - Support of Rel-16 eType-II regular codebook refinement for multi-TRP CJT with PMI subband R=1. - Support of parameter combinations with L=2,4 - Support of rank 1,2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources - Supported frequency basis selection mode - Maximum number of ports across all TRPs in the same CSI report setting for multi-TRP CJT. - Maximum number of lists for spatial basis selection, i.e., NL |  |  |  |  |  |  |  |  | Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4} Component 5 candidate values: {mode 2, both mode 1 and mode 2} Component 6 candidate values: {32, 64, 96, 128} Component 7 candidate values: {1, 2, 4} Note: Frequency basis selection mode 1 is the common frequency basis selection with frequency basis offset reporting, mode 2 is only the common frequency basis selection. | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-1 | Rel-16 eType-II regular codebook refinement for multi-TRP CJT, R=2 | - Support of Rel-16 eType-II regular codebook refinement for multi-TRP CJT with PMI subbands R=2. - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources - Supported frequency basis selection mode |  |  |  |  |  |  |  |  | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4} Component 3 candidate values: {mode 2, both mode 1 and mode 2} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-2 | Rel-16 eType-II regular codebook refinement for multi-TRP CJT, support of parameter combination with L=6 | - Support of Rel-16 eType-II regular codebook refinement for multi-TRP CJT with parameter combination with L=6. |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-3 | Rel-16 eType-II regular codebook refinement for multi-TRP CJT, rank 3,4 | - Support of Rel-16 eType-II regular codebook refinement for multi-TRP CJT with rank 3,4 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1a-4 | Active CSI-RS resources and ports in the presence of multi-TRP NCJ eType-II regular | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination |  |  |  |  |  |  |  |  | FFS | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b | Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT (Coherent Joint Transmission) | - Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=1. - Support of parameter combinations with M=1 - Support of rank 1,2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources - Supported frequency basis selection mode - Maximum number of ports across all TRPs in the same CSI report setting for multi-TRP CJT. - Maximum number of lists for spatial basis selection, i.e., NL |  |  |  |  |  |  |  |  | Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {1, 2,3,4 … 64} c) {4, …, 256} d) {2,3,4} Component 5 candidate values: {mode 2, both mode 1 and mode 2} Component 6 candidate values: {32, 64, 96, 128} Component 7 candidate values: {1, 2, 4} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-1 | Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT, R=2 | - Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources - Supported frequency basis selection mode |  |  |  |  |  |  |  |  | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4} Component 3 candidate values: {mode 2, both mode 1 and mode 2} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-2 | Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT, M=2 and R=1 | - Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1. - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT - Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT - Number of TRP/TRP groups, i.e., NZP CSI-RS resources - Supported frequency basis selection mode |  |  |  |  |  |  |  |  | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4} Component 3 candidate values: {mode 2, both mode 1 and mode 2} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-3 | Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT, rank 3,4 | - Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with rank 3,4. |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1b-4 | Active CSI-RS resources and ports in the presence of multi-TRP NCJ FeType-II port selection | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination |  |  |  |  |  |  |  |  | FFS | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1c | Slection of N CSI-RS resource by UE for Type II codebokk refinement for multi-TRP CJT | - Support of slection of N CSI-RS resource by UE for Type II codebokk refinement for multi-TRP CJT including Rel-16 eType-II regular codebook and Rel-17 FeType-II portseletion codebook refinement. |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1d | [Place Holder] W2 qualitzation group for Type II codebokk refinement for multi-TRP CJT | - Supported W2 qualitzation group for Type II codebokk refinement for multi-TRP CJT including Rel-16 eType-II regular codebook and Rel-17 FeType-II portseletion codebook refinement. |  |  |  |  |  |  |  |  | Component 1 candidate values: {mode 1, mode 1 and mode 2} Mode 1: One group comprises one polarization across all N CSI-RS resources for each layer Mode 2: One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources for each layer. | Optional with capability signalling |
| NR_MIMO_evo_DL_UL | 40-3-1e | Fractional frequency basis selection offset for mode 1 frequency baiss selection for multi-TRP CJT | - Support of fractional frequency basis selection offset, i.e., for mode 1 frequency baiss selection for multi-TRP CJT including Rel-16 eType-II regular codebook and Rel-17 FeType-II portseletion codebook refinement |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| NR_MIMO_evo_DL_UL | 40-3-1f | Unequal number of spatial basis selection for multi-TRP CJT | - Support of unequal number of spatial basis selection for multi-TRP CJT including Rel-16 eType-II regular codebook and Rel-17 FeType-II portseletion codebook refinement |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a | Rel-16 eType-II regular codebook refinement exploiting channel time domain properties | - Support of Rel-16 eType-II regular codebook refinement exploiting channel time domain properties with PMI subband R=1. - Support of parameter combinations with L=2,4 - Support of rank 1,2 - Support X=1 CQI associated with the first/earliest slot of the CSI reporting window and the first PMI prediction - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources - Number of predicted CSI (N4) |  |  |  |  |  |  |  |  | Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {1, 2, 4, 8} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a-1 | Rel-16 eType-II regular codebook refinement exploiting channel time domain properties, R=2 | - Support of Rel-16 eType-II regular codebook refinement exploiting channel time domain properties with PMI subbands R=2. - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources - Number of predicted CSI (N4) |  |  |  |  |  |  |  |  | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {1, 2, 4} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a-2 | Rel-16 eType-II regular codebook refinement exploiting channel time domain properties, support of parameter combination with L=6 | - Support of Rel-16 eType-II regular codebook refinement exploiting channel time domain properties with parameter combination with L=6. |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a-3 | Rel-16 eType-II regular codebook refinement exploiting channel time domain properties, rank 3,4 | - Support of Rel-16 eType-II regular codebook refinement exploiting channel time domain properties with rank 3,4 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a-4 | Active CSI-RS resources and ports in the presence of eType-II regular refinement exploiting channel time domain properties | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination |  |  |  |  |  |  |  |  | FFS | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2a-5 | CQI prediction mode | - Supported additional CQI prediction mode for Rel-16 eType-II regular codebook refinement exploiting channel time domain properties |  |  |  |  |  |  |  |  | Component 1 candidate values: {mode 1, mode 2, both} Mode 1: X=1 CQI and the CQI is associated with: - The first/earliest slot of the CSI reporting window and the first/earliest predicted PMI, and - The last slot of the CSI reporting window and the last predict PMI Mode 2: X=2 CQI and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI, and - The 2nd CQI is associated with the middle slot of the CSI reporting window and the predicted PMI in the middle | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2b | Rel-17 FeType-II port selection codebook refinement exploiting channel time domain properties | - Support of Rel-17 FeType-II port selection codebook refinement exploiting channel time domain properties with PMI subband R=1. - Support of parameter combinations with M=1 - Support of rank 1,2 - Support X=1 CQI associated with the first/earliest slot of the CSI reporting window and the first PMI prediction - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources |  |  |  |  |  |  |  |  | Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {1, 2,3,4 … 64} c) {4, …, 256} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2b-1 | Rel-17 FeType-II port selection codebook refinement exploiting channel time domain properties, R=2 | - Support of Rel-17 FeType-II port selection codebook refinement exploiting channel time domain properties with PMI subbands R=2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources |  |  |  |  |  |  |  |  | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2b-2 | Rel-17 FeType-II port selection codebook refinement exploiting channel time domain properties, M=2 and R=1 | - Support of Rel-17 FeType-II port selection codebook refinement exploiting channel time domain properties with M=2 and PMI subband R=1. - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources |  |  |  |  |  |  |  |  | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2b-3 | Rel-17 FeType-II port selection codebook refinement exploiting channel time domain properties, rank 3,4 | - Support of Rel-17 FeType-II port selection codebook refinement exploiting channel time domain properties with rank 3,4. |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2b-4 | Active CSI-RS resources and ports in the presence of FeType-II port selection refinement exploiting channel time domain properties | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination |  |  |  |  |  |  |  |  | FFS | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2c | CSI reference resource as time domain location for CSI prediction | - Support of CSI reference resource as time domain location for CSI prediction including Rel-16 eType-II regular codebook and Rel-17 FeType-II portseletion codebook refinement |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3 | TDCP (Time Domain Channel Properties) report | - Support of TDCP (Time Domain Channel Properties) report based on TRS meassuremnt. - Maximum number of TRS resource set configured in each CSI report setting. - Maximum of D, i.e., time offset for time domain correlation estimate. - Maximum number of reported time domain correlation Y - TDCP report quantities |  |  |  |  |  |  |  |  | Component 2 candidate values: {1, 2} Component 3 candidate values: {4 symbols, 1 slot, 2 slots, 3 slots, 4 slots, 5 slots} Component 4 candidate values: {1, 2, 3, 4, 7 } Component 5 candidate values: {amplitude, amplitude and phase} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1 | Enhanced DMRS with FD-OCC 4 for PDSCH | - Support of enhanced DMRS with FD-OCC 4 for PDSCH |  |  |  |  |  |  |  |  | Component 1 candidate value: {eType 1, eType 2, both} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1a | [Place Holder] MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH within a CDM group | - Support of MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH within a CDM group. |  |  |  |  |  |  |  |  | Depending on whether explicit restriction would be agreed in the spefiicaiton | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1b | [Place Holder] MU-MIMO with Rel.18 DMRS ports for PDSCH when antenna ports are in more than one CDM group | - Support of MU-MIMO between Rel.18 DMRS ports and Rel.15/Rel.18 DMRS ports for PDSCH when antenna ports are in more than one CDM group |  |  |  |  |  |  |  |  | Depending on whether explicit restriction would be agreed in the spefiicaiton | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1c | PDSCH scheduling without restriction for eType I DMRS with FD-OCC 4 | - Support of PDSCH scheduling without restriction for eType I DMRS with FD-OCC 4 |  |  |  |  |  |  |  |  | Note: If this feature is not supported. UE expects that gNB shall apply at least the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even. 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-2 | Enhanced DMRS with FD-OCC 4 for PUSCH with CP-OFDM | - Support of enhanced DMRS with FD-OCC 4 for PUSCH with CP-OFDM |  |  |  |  |  |  |  |  | Component 1 candidate value: {eType 1, eType 2, both} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-2a | [Place Holder] 4 port UL PTRS | - Support of 4 port UL PTRS |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1 | SRS comb offset and/or cyclic shift hopping | - Support of SRS comb offset and/or cyclic shift hopping |  |  |  |  |  |  |  |  | Component 1 candidate value: {comb offset hopping, cyclic shift hopping, both } | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1a | SRS comb offset hopping pattern with repetition factor R>1 | - Support of different SRS comb offset hopping pattern with repetition factor R>1 |  |  |  |  |  |  |  |  | Component 1 candidate value: {pattern 1, pattern 2, both} Note: Pattern 1: within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol. Pattern 2: within a slot, the time-domain hopping behavior depends on the first symbol across the R repetitions. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1b | SRS comb offset hopping combined with group / sequence hopping on an SRS resource | - Support of SRS comb offset hopping combined with group / sequence hopping on an SRS resource. |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1c | [Place Holder] SRS cyclic shift hopping combined with group / sequence hopping on an SRS resource | - Support of SRS cyclic shift hopping combined with group / sequence hopping on an SRS resource. |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2 | TDM SRS ports for 8 port SRS | - Support of different SRS ports mapped onto different OFDM symbols for 8 port SRS |  |  |  |  |  |  |  |  | Component 1 candidate value for s: {2, [2 and 4], [2 and 4 and 8]} Note: 8 ports are equally partitioned into s subsets with each subset having 8/s different ports | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-3 | 8 port SRS | - Support of 8 port SRS |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-3a | 8T8R SRS antenna switching | - Support of 8T8R SRS antenna switching. - Report the entry number of the first-listed band with UL in the band combination that affects this DL. - Report the entry number of the first-listed band with UL in the band combination that switches together with this UL. |  |  |  |  |  |  |  |  | Component 1 candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} Note: Compnent 1 indicates the supported xTyR in addition to 8T8R. Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} Note: component 2 and Component 3 are optional. If reported, the reported values for component 2 and component 3 are not valid for the same values of xTyR in component 1 reported with Rel-15/16/17 UE capability reporting | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1a | Single-DCI STxMP (Simultaneous Transmission across Multiple Panel) SDM scheme - codebook based | - Support of Single-DCI STxMP (Simultaneous Transmission across Multiple Panel) with SDM for codebook based PUSCH. - Support of two SRS resource sets with usage set to 'codebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of SRS antenna ports for each SRS resource in each SRS resource set - Maximum number of layers of each panel for Single-DCI STxMP with SDM - Maximuml total number of layers across both panels for Single-DCI STxMP with SDM |  |  |  |  |  |  | FR2 only |  | Component 3 candidate values: {1, 2 ,4} Component 4 candidate values: {1, 2 [,4]} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1b | Single-DCI STxMP (Simultaneous Transmission across Multiple Panel) SDM scheme - non-codebook based | - Support of Single-DCI STxMP (Simultaneous Transmission across Multiple Panel) with SDM for non-codebook based PUSCH. - Support of two SRS resource sets with usage set to 'nonCodebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of simultaneous transmitted SRS at one symbol - Maximum number of layers of each panel for Single-DCI STxMP with SDM - Maximuml total number of layers across both panels for Single-DCI STxMP with SDM |  |  |  |  |  |  | FR2 only |  | Component 3 candidate values: {1, 2 ,3, 4} Component 4 candidate values: {1, 2, 3, 4} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1c | Dynamic switching between Single-DCI STxMP SDM scheme and Single-TRP scheme - codebook based | - Support of dynamic switching between Single-DCI STxMP SDM scheme and Single-TRP scheme for codebook based PUSCH by DCI formats 0_1/0_2 |  |  |  |  |  |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1d | Dynamic switching between Single-DCI STxMP SDM scheme and Single-TRP scheme - non-codebook based | - Support of dynamic switching between Single-DCI STxMP SDM scheme and Single-TRP scheme for non-codebook based PUSCH by DCI formats 0_1/0_2 |  |  |  |  |  |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2a | Single-DCI STxMP (Simultaneous Transmission across Multiple Panel) SFN scheme - codebook based | - Support of Single-DCI STxMP (Simultaneous Transmission across Multiple Panel) with SFN for codebook based PUSCH. - Support of two SRS resource sets with usage set to 'codebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of SRS antenna ports for each SRS resource in each SRS resource set - Maximum number of layers for Single-DCI STxMP with SFN |  |  |  |  |  |  | FR2 only |  | Component 3 candidate values: {1, 2 ,4} Component 4 candidate values: {1, 2 [,4]} Component 5 candidate values: {1, 2} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2b | Single-DCI STxMP (Simultaneous Transmission across Multiple Panel) SFN scheme - non-codebook based | - Support of Single-DCI STxMP (Simultaneous Transmission across Multiple Panel) with SFN for non-codebook based PUSCH. - Support of two SRS resource sets with usage set to 'nonCodebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of simultaneous transmitted SRS at one symbol - Maximum number of layers for Single-DCI STxMP with SFN |  |  |  |  |  |  | FR2 only |  | Component 3 candidate values: {1, 2 ,3, 4} Component 4 candidate values: {1, 2, 3, 4} Component 5 candidate values: {1, 2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2c | Dynamic switching between Single-DCI STxMP SFN scheme and Single-TRP scheme - codebook based | - Support of dynamic switching between Single-DCI STxMP SFN scheme and Single-TRP scheme for codebook based PUSCH by DCI formats 0_1/0_2 |  |  |  |  |  |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2d | Dynamic switching between Single-DCI STxMP SDM scheme and Single-TRP scheme - non-codebook based | - Support of dynamic switching between Single-DCI STxMP SFN scheme and Single-TRP scheme for non-codebook based PUSCH by DCI formats 0_1/0_2 |  |  |  |  |  |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2e | DMRS ports {0, 2, 3} for Single-DCI STxMP SDM scheme | - Support of DMRS ports {0, 2, 3} for Single-DCI STxMP SDM scheme. |  |  |  |  |  |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3a | Multi-DCI STxMP (Simultaneous Transmission across Multiple Panel) scheme - codebook based | - Support of Multi-DCI STxMP (Simultaneous Transmission across Multiple Panel) for codebook based PUSCH with fully overlapping in both frequency and time. - Support of two SRS resource sets with usage set to 'codebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of SRS antenna ports for each SRS resource in each SRS resource set - Maximum number of layers of each overlapping PUSCH - Maximuml total number of layers across two overlapping PUSCH |  |  |  |  |  |  | FR2 only |  | Component 1 candidate values: a combination from the set {DG PUSCH, Type I CG PUSCH, Type II CG PUSCH} Component 3 candidate values: {1, 2 ,4} Component 4 candidate values: {1, 2 [,4]} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3b | Multi-DCI STxMP (Simultaneous Transmission across Multiple Panel) scheme - non-codebook based | - Support of Multi-DCI STxMP (Simultaneous Transmission across Multiple Panel) for non-codebook based PUSCH with fully overlapping in both frequency and time. - Support of two SRS resource sets with usage set to 'nonCodebook' - Maximum number of SRS resources in each SRS resource set - Maximum number of simultaneous transmitted SRS at one symbol - Maximum number of layers of each overlapping PUSCH - Maximuml total number of layers across two overlapping PUSCH |  |  |  |  |  |  | FR2 only |  | Component 1 candidate values: a combination from the set {DG PUSCH, Type I CG PUSCH, Type II CG PUSCH} Component 3 candidate values: {1, 2 ,3, 4} Component 4 candidate values: {1, 2, 3, 4} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3c | Multi-DCI STxMP scheme – fully overlapping in frequency, partially overlapping in time | - Support of Multi-DCI STxMP schedule with two PUSCHs fully overlapping in frequency and partially overlapping in time. |  |  |  |  |  |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3d | Multi-DCI STxMP scheme – partially or non-overlapping in frequency, partially or fully overlapping in time | - Support of Multi-DCI STxMP schedule with two PUSCHs partially or non-overlapping in frequency and partially or fully overlapping in time. |  |  |  |  |  |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-4 | Single-DCI STxMP (Simultaneous Transmission across Multiple Panel) SFN scheme for PUCCH | - Support of Single-DCI STxMP (Simultaneous Transmission across Multiple Panel) with SFN for PUCCH. |  |  |  |  |  |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1 | 8 Tx codebook based PUSCH operation | - Maximum number of supported layers for codebook based PUSCH operation with 8 Tx. - Maximum number of SRS resources in each SRS resource set with usage set to 'codebook’. - Support of 2 codewords if the maximum number of supported layers is larger than 4 |  |  |  |  |  |  |  |  | Component 1 candidate values: {1, 2, [3,] 4, [5, 6, 7,] 8} Component 2 candidate values: {1, 2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1a | PUSCH codebook coherency subset for 8 Tx | - Supported codebook coherency subset type when UE is configured with 8Tx codebook based PUSCH operation. |  |  |  |  |  |  |  |  | Component 1 candidate values: FFS {non-coherent, non-coherent and partial coherent Ng=4, non-coherent and partial coherent Ng=2 or Ng=4, non-coherent and partial coherent Ng=2 or Ng=4 and full-coherent N1=4, N2=1, non-coherent and partial coherent Ng=2 or Ng=4 and full-coherent N1=2, N2=2} Note: for 8 Tx codebook based PUSCH operation, there are total 5 different coherency modes (1) non-coherent (2) partial-coherent Ng=4, i.e., 4 coherent port groups with 2 ports each group, while non-coherent between different port groups (3) partial-coherent Ng=2, i.e., 2 coherent port groups with 4 ports each group, while non-coherent between different port groups (4) full-coherent N1=4, N2=1, N1 and N2 is the number of antenna elements in vertical and horizontal direction, respectively. Each antenna element contains both V-Pol and H-Pol. (5) full-coherent N1=2, N2=2. Note: For codebook based PUSCH operation, the number of Tx is the maximum number of SRS ports of SRS resource(s) configured in the SRS resource set with usage set to “codebook”. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1a-1 | PUSCH codebook coherency subset for 8 Tx when UE is configured with 2 Tx codebook based PUSCH operation | - Supported codebook coherency subset type when UE is configured with 2 Tx codebook based PUSCH operation while UE supports 8 Tx. |  |  |  |  |  |  |  |  | Component 1 candidate values: {non-coherent, non-coherent and full-coherent} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1a-2 | PUSCH codebook coherency subset for 8 Tx when UE is configured with 4 Tx codebook based PUSCH operation | - Supported codebook coherency subset type when UE is configured with 4 Tx codebook based PUSCH operation while UE supports 8 Tx. |  |  |  |  |  |  |  |  | Component 1 candidate values: {non-coherent, non-coherent and partial coherent, non-coherent and partial coherent and full-coherent} FFS: whether to resuse legacy FG2-13 assuming UE reports that UE supports 4Tx in legacy | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1b | UL full power transmission mode of fullpower for 8 Tx | - Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1c | [Place Holder] UL full power transmission mode of fullpowerMode1 for 8 Tx | - Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation |  |  |  |  |  |  |  |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1d | [Place Holder] UL full power transmission mode of fullpowerMode2 for 8 Tx | - Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2 |  |  |  |  |  |  |  |  | Component 1 candidate values: {1, 2, 4, 8} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1e | [Place Holder] UL full power transmission mode of fullpowerMode2 for 8 Tx - SRS resource | - The supported SRS configurations with different number of antenna ports per SRS resource for full power Mode 2 when UE is capable of and configured with 8 Tx codebook based PUSCH operation |  |  |  |  |  |  |  |  | Component (1) candidate values: {1_8, 1_2_8, 1_4_8, 1_2_4_8} 1st state (1_8): each SRS resource can be configured with 1 port or 8 ports 2nd state (1_2_8): each SRS resource can be configured with 1 port or 2 ports or 8 ports 3rd state (1_4_8): each SRS resource can be configured with 1 port or 4 ports or 8 ports 4th state (1_2_4_8): each SRS resource can be configured with 1 port or 2 port or 4 ports or 8 ports | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1f | [Place Holder] UL full power transmission mode of fullpowerMode2 for 8 Tx - full power TPMP groups | - TPMI group(s) which delivers full power when UE is capable of and configured with 8 Tx codebook based PUSCH operation |  |  |  |  |  |  |  |  | FFS | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1g | UL full power transmission mode of fullpowerMode2 | FFS How to support downgrade configuration, i.e., UE is configured with 2/4 Tx while UE supports 8Tx Whether and how to use the legacy FG16-5c, FG16-5c-2, FG16-5c-3 |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-7-2 | 8 Tx non-codebook based PUSCH operation | - Maximum number of supported layers for non-codebook based PUSCH operation with 8 Tx. - Maximum number of SRS resources in each SRS resource set with usage set to 'nonCodebook’. - Maximum number of simultaneous transmitted SRS at one symbol |  |  |  |  |  |  |  |  | Component 1 candidate values: {1, 2, [3, ]4, [5, 6, 7, ]8} Component 2 candidate values: {5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-3 | CBG-based re-transmission for UL using CBGTI for 2 CW | - Support of CBG-based re-transmission for UL using CBGTI when NW configures a maximum rank larger than 4 for PUSCH operation. |  |  |  |  |  |  |  |  |  | Optional with capability signaling |