3GPP TSG RAN WG1 #113	R1-2305435

Incheon, Korea, May 22-26, 2023

Title:	[Draft] reply LS on Cell DTX/DRX activation/deactivation

Release:	Rel-18

Work Item:	Netw_Energy_NR

Source:	OPPO [to be RAN1]

To:	RAN2

Cc:	RAN4

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

RAN1 would like to thank RAN2’s LS on Cell DTX/DRX activation/deactivation and RAN2’s LS raised the following questions and asked for RAN1’s responses.

RAN2 kindly requests RAN1 to provide information regarding feasibility and reliability of using dedicated and group common L1 signalling for Cell DTX/DRX activation and deactivation. Our question is related only to Cell DTX/DRX activation and deactivation and we would like to focus on a single Cell DTX/DRX configuration, as agreed in our previous meeting. Once L1 signalling for activation and deactivation of Cell DTX/DRX for a single configuration is decided in RAN1 please inform us about the decision and design details.

RAN1 response

For reliability, the L1 signaling could be carried in a PDCCH, because the performance requirement of PDCCH mis-detection probability is 1%. The impact of not receiving an activation/deactivation command via PDCCH is small. Furthermore, if group common PDCCH is adopted, this reliability of PDCCH can be further enhanced by increasing the CCE aggregation level for PDCCH transmission. Moreover, it is to note that the L1 signaling has already beenn adopted for reliability sensitive applications, e.g. XR and URLLC. Therefore, from RAN1 point of view, using L1 signaling for cell DTX/DRX activation/deactivation is reliable.

For feasibility, in legacy specification one group common PDCCH can be transmitted to multiple UEs for different purposes, e.g., for notifying the slot format, for notifying the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE, for the transmission of TPC commands for PUCCH and PUSCH, for notifying the PRB(s) and OFDM symbol(s) where UE cancels the corresponding UL transmission, and for notifying the power saving information outside DRX Active Time, etc.. A new group common DCI format can be designed for Cell DTX/DRX activation and deactivation if needed. Therefore, from RAN1 point of view, using L1 signaling for cell DTX/DRX activation/deactivation is feasible.

2	Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above responses into consideration in their future work.

3	Date of next TSG-RAN WG1 meeting

TSG-RAN WG1 Meeting #114	August 21 -  25, 2023	Toulouse, France

TSG-RAN WG1 Meeting #114bis	October 09-  13, 2023	Xiamen, China