3GPP TSG RAN WG1 #113			R1-2305543

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.16.4

Source:	Samsung

Title:	UE features for NR NCR

Document for:	Discussion and Decision

- Introduction

In this contribution, our views on capabilities for NR network-controlled repeater (NCR) are presented.

- Discussion

In RAN1#112b-e [1], the initial framework of for NR NCR feature is agreed. Further change / revision based on the framework is discussed as follows.

| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. [Support of] Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd | FFS | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 [Component 5 candidate values: FFS] | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In RAN1#112bis-e, FG 43-1 is agreed as above. One FFS part is the aspect of internal delay. Basically, the DL transmitting timing of the NCR-Fwd is delayed after the DL receiving timing of the NCR-MT by the internal delay; and UL receiving timing of the NCR-Fwd is advanced before the UL transmitting timing of the NCR-MT by the internal delay. For the internal delay, it is beneficial for gNB to acquire the information on DL/UL internal delay of NCR. By knowing the value of DL/UL internal delay, gNB can identify whether a UE is being served by the NCR more accurately. Hence, it is preferable to introduce the indication of internal delay in FG 43-1 component 5. For simplicity, same DL internal delay and UL internal delay can be assumed. Also, a number of candidate values for internal delay are provided.

Proposal 1: Adopt FG 43-3 with the following modification (i.e., confirm component 2 and the corresponding candidate values).

| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. Support of Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd | FFS | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 Component 5 candidate values: {0, 5, 10} microseconds for both DL internal delay and DL internal delay. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

As agreed in RAN1#112bis-e, FG 43-2, FG 43-3 and FG 43-4 is agreed as follows.

| 43. NR_netcon_repeater | 43-2 | Periodic beam indication for access link | 1.Support periodic beam indication for access link 2. Priority flag for periodic indication | 43-1 | N/A | Yes |  | NCR-MT cannot decode the periodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link [2. Supported slot-offset k values for reference slot] | 43-1 | N/A | Yes |  | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one FFS: component 2 candidate values | Optional with capability signaling |
| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication | 43-1 | N/A | Yes |  | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |

For FG 43-3, a further study point is whether k is defined by NCR-MT capability and/or declared by vendor (k is the slot offset between the reference of slot offset for each time resource and the slot that NCR-MT receive the DCI carrying the indication). In our view, k should be subject to NCR-MT capability. A value set for k can be larger than that of a similar UE capability for DCI processing (e.g., parameter N_pdsch) in order to support simplified implementation of NCR and/or to incorporate beam switching delay of the NCR-Fwd (e.g., parameters timeDurationForQCL or beamAppTime). Hence the proposed modification for FG 43-3 and the candidate values for k are provided as below.

Proposal 2: Adopt FG 43-3 with the following modification (i.e., confirm component 2 and the corresponding candidate values).

| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link 2. Supported slot-offset k values for reference slot per SCS of NCR-MT | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one Component 2 candidate values: {0, 1, 2, 3, 4, 5, 6} slots for the SCS of 15 kHz and 30 kHz; {1,2, 3, 4, 5, 6, 7, 8, 9, 16, 17, 24, 25} slots for the SCS of 60kHz; {2,3,4, 5, 6, 7, 8, 9, 16, 17, 24, 25} slots for the SCS of 120kHz. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 43-4, the separation of FG 43-4 and FG 43-4a is unnecessary. Beam index update is a natural part of semi-persistent beam indication and it is up to gNB to decide whether to use this feature or not. Hence, we propose the following modification of FG 43-4 to include FG 43-4a.

Proposal 3: Adopt FG 43-4 with the following modification (i.e., adding component 3).

| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication 3. Beam index updates for semi-persistent beam indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Another FFS point for FG 43-2, FG 43-3 and FG 43-4 is that whether/how these FGs can be merged to FG 43-1. In our view, to avoid the fragmentation of NCR capability, all of FG 43-2, FG 43-3 and FG 43-4 should be merged into FG 43-1.

Proposal 4: Merge all of FG 43-2, FG 43-3 and FG 43-4 into FG 43-1.

In RAN1#112bis-e, FG 43-6 and FG 43-8 were agreed as follows.

| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication |  | 43-1, 43-8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | [Component candidate values: {Rel-15/16, Rel-17, both}] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | FFS: Component candidate values: {Rel-15/16, Rel-17, both} | Optional with capability signaling |

The remaining discussion point for FG 43-6 and FG 43-8 is the corresponding component candidate values. For FG 43-8, it is preferred to confirm the FFS part since it is straightforward to use explicit indication rather than implicit determination of the supported beam indication framework from legacy MIMO FGs. For FG 43-6, it is preferred to confirm the FFS part similar as FG 43-8. In addition, a note can be added to FG 43-6 that the reported candidate values of FG 43-6 should be same as or a subset of the reported candidate values of FG 43-8. Hence, we propose the following modifications.

Proposal 5: Adopt FG 43-6 and FG 43-8 with the following modification.

| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication |  | 43-1, 43-1a8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | Component candidate values: {Rel-15/16, Rel-17, both} Note: the reported value should be same as or a subset of reported value of FG 43-8. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | Component candidate values: {Rel-15/16, Rel-17, both} | Optional with capability signaling |

As agreed in RAN1#110b-e, it is subject to NCR capability to support simultaneous transmission of the UL of C-link and UL of backhaul link. In addition, TDMed UL transmission of C-link and backhaul link was agreed as default capability. For an NCR that supports simultaneous transmission of the UL of C-link and UL of backhaul link, an additional capability for NCR power needs to be considered. The NCR power capability is to indicate whether the NCR: (i) is not subject to a joint power limit across the C-link and BH-link (e.g., using separate PAs / power resources for the C-link and BH-link), or (ii) is subject to a joint power limit across the C-link and BH-link and supports power sharing when power limited, or (iii) is subject to a joint power limit across the C-link and BH-link and does not support power sharing when power limited (e.g., transmit only one of C-link or BH-link when power limited). In our view, NCR capability (i), herein referred to as Type-1 NCR, should not be considered as the only/baseline implementation for an NCR with simultaneous transmission of the UL of C-link, as it is not economical to allocate a dedicated PA / power resource for NCR-MT which is infrequently used for control signaling only. A more reasonable NCR implementation can use a shared PA / power resource for both NCR-MT and NCR-Fwd with basic power sharing capability, such as NCR capabilities (ii) and (iii), herein referred to as Type-2 and Type-3 NCR, respectively. Corresponding NCR behavior can be further discussed.

Proposal 6: Introduce a new component in FG 43-5 to indicate the supported power allocation / sharing method between the C-link and the backhaul link.

| 43. NR_netcon_repeater | 43-5 | 1. Simultaneous UL transmission of backhaul link and C-link 2. Power allocation/sharing between C-link and B-link of an NCR | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | N/A | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes | Component candidate values: {Type 1, Type 2, Type 3} Note: For Type 1, NCR is not subject to a joint power limit across the C-link and BH-link. For Type 2, is subject to a joint power limit across the C-link and BH-link and supports power sharing when power limited. For Type 3, is subject to a joint power limit across the C-link and BH-link and does not support power sharing when power limited. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

The observations and proposals made in this contribution are summarized below.

Proposal 1: Adopt FG 43-3 with the following modification (i.e., confirm component 2 and the corresponding candidate values).

| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. Support of Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd | FFS | N/A | Yes | NCR is not supported | Per NCR-MT | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 Component 5 candidate values: {0, 5, 10} microseconds for both DL internal delay and DL internal delay. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Adopt FG 43-3 with the following modification (i.e., confirm component 2 and the corresponding candidate values).

| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link 2. Supported slot-offset k values for reference slot per SCS of NCR-MT | 43-1 | N/A | Yes | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one Component 2 candidate values: {0, 1, 2, 3, 4, 5, 6} slots for the SCS of 15 kHz and 30 kHz; {1,2, 3, 4, 5, 6, 7, 8, 9, 16, 17, 24, 25} slots for the SCS of 60kHz; {2,3,4, 5, 6, 7, 8, 9, 16, 17, 24, 25} slots for the SCS of 120kHz. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Adopt FG 43-4 with the following modification (i.e., adding component 3).

| 43. NR_netcon_repeater | 43-4 | Semi-persistent beam indication for access link | 1.Support semi-persistent beam indication for access link 2. Priority flag for semi-persistent indication 3. Beam index updates for semi-persistent beam indication | 43-1 | N/A | Yes | NCR-MT cannot decode the semi-persistent beam indication | Per NCR-MT | No | No | Yes | Note: at least one of FGs 43-2, 43-3, 43-4 will be merged with 43-1, FFS which one | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: Merge all of FG 43-2, FG 43-3 and FG 43-4 into FG 43-1.

Proposal 5: Adopt FG 43-6 and FG 43-8 with the following modification.

| 43. NR_netcon_repeater | 43-6 | Dedicated signalling for backhaul link beam indication |  | 43-1, 43-1a8 | N/A | Yes | Dedicated signalling for backhaul link beam indication is not supported | Per NCR-MT | No | No | Yes | Component candidate values: {Rel-15/16, Rel-17, both} Note: the reported value should be same as or a subset of reported value of FG 43-8. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-8 | Adaptive beam for NCR backhaul link/C-link | Support of backhaul link beam determination based on predefined rule | 43-1, 2-2,2-4, 2-4a | N/A | Yes | The beam for backhaul link and C-link is fixed. | Per NCR-MT | No | No | Yes | Component candidate values: {Rel-15/16, Rel-17, both} | Optional with capability signaling |

Proposal 6: Introduce a new component in FG 43-5 to indicate the supported power allocation / sharing method between the C-link and the backhaul link.

| 43. NR_netcon_repeater | 43-5 | 1. Simultaneous UL transmission of backhaul link and C-link 2. Power allocation/sharing between C-link and B-link of an NCR | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | N/A | Yes | NCR only supports TDMed UL transmission of C-link and backhaul link | Per NCR-MT | No | No | Yes | Component candidate values: {Type 1, Type 2, Type 3} Note: For Type 1, NCR is not subject to a joint power limit across the C-link and BH-link. For Type 2, is subject to a joint power limit across the C-link and BH-link and supports power sharing when power limited. For Type 3, is subject to a joint power limit across the C-link and BH-link and does not support power sharing when power limited. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

References

- Chair notes, RAN1#112bis-e.