3GPP TSG RAN WG1 #113													R1-2307529

Incheon (KR), 22nd – 26th May, 2023

Agenda item:		9.16.16

Source:	Nokia, Nokia Shanghai Bell

Title:	On TEI18 UE features

Document for:		Discussion and Decision

# Introduction

RAN1 started discussion Rel-18 UE features for selected topics in RAN1#112bis-e [1], also successfully concluding on the RAN1#112-agreed TEI18 item related FGs [2]. In this contribution we suggest a new FG for the RAN1#112bis-agreed TEI18 item [3].

# Discussion

RAN1#112bis agreed on the following for TEI18

Agreement Introduce UE feature(s) for multi-PUSCH scheduling with single DCI 0_1 for non-contiguous slots in FR1 for all defined SCSs - Note: there is no RAN1 impact

The corresponding Rel-17 capabilities extending the feature to FR2_1 and FR2_2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported |  | Consequence if the feature is not supported by the UE | Type (...) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1a | Yes |  | Multiple PUSCH scheduling by single DCI for 120kHz is not supported in FR2-2 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 24. NR_ext_to_71GHz | 24-1g | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS with non-contiguous allocation |  | Yes |  | Multiple PUSCH scheduling by single DCI for 120kHz is not supported in FR2-1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

The new capability extending this to FR1 would be straightforward:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported |  | Consequence if the feature is not supported by the UE | Type (...) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55 TEI18 | 55-x | Multiple PUSCH scheduling by single DCI in FR1 | 1. Multi-PUSCH scheduling by single DCI for FR1 with non-contiguous allocation |  | Yes |  | Multiple PUSCH scheduling by single DCI is not supported on FR1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

# Conclusion

Proposal: In order to complete the RAN1#112bis-agreed TEI18 for multi-PUSCH scheduling support with single DCI 0_1 for non-contiguous slots in FR1, introduce the following new TEI18 FG:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported |  | Consequence if the feature is not supported by the UE | Type (...) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55 TEI18 | 55-x | Multiple PUSCH scheduling by single DCI in FR1 | 1. Multi-PUSCH scheduling by single DCI for FR1 with non-contiguous allocation |  | Yes |  | Multiple PUSCH scheduling by single DCI is not supported on FR1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

# References

- R1-2304282, “Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e,” Moderators (AT&T, NTT DOCOMO, INC.), Apr. 2023

- R1-2304226, “Session Notes for AI 9.17.16”, Ad-Hoc Chair (NTT DOCOMO, INC.)

- R1-2304077, “Summary #1 on Rel-18 TEIs”, Moderator (NTT DOCOMO, INC.)