3GPP TSG- RAN WG1 Meeting #112bis-e                                                                                R1-2306042

Incheon, Korea, May 22th – May 26th, 2023

Agenda Item:	9.2.2.2

Source:	Moderator (Apple)

Title:	Summary #1 on other aspects of AI/ML for CSI enhancement

# Introduction

This paper summarizes the discussion for agenda item 9.2.2.2.

## Contact information

Please provide your contact information.

| Company | Name | Email |
| --- | --- | --- |
| Moderator (Apple) | Huaning Niu | huaning_niu@apple.com |
| NTT DOCOMO | Haruhi Echigo | haruhi.echigo.fw@nttdocomo.com |
| Qualcomm | Jay Kumar Sundararajan | jsundara@qti.qualcomm.com |
| NEC | Zhen He Caroline Liang | he_zhen@nec.cn caroline.liang@emea.nec.com |
| CATT | Yongqiang Fei | feiyongqiang@catt.cn |
| LG Electronics | Haewook Park | haewook.park@lge.com |
| Spreadtrum | Hualei Wang | Hualei.wang@unisoc.com |
| Lenovo | Ahmed Hindy | ahmedhindy@motorola.com |
| Samsung | Ameha | amehat.abebe@samsung.com |
| CMCC | Yuhua Cao | caoyuhua@chinamobile.com |
| MediaTek | Gyu Bum Kyung Pedram Kheirkhah Sangdeh | Gyubum.kyung@mediatek.com Pedram.kheirkhah@mediatek.com |
| Panasonic | Tetsuya Yamamoto | yamamoto.tetsuya001@jp.panasonic.com |
| Xiaomi | Zhengxuan Liu | liuzhengxuan@xiaomi.com |
| Sony | Hiroki Matsuda | hiroki.matsuda@sony.com |
| Sony | Sam Atungsiri | sam.atungsiri@sony.com |
| Intel | Victor Sergeev | victor.sergeev@intel.com |
| AT&T | Isfar Tariq Salam Akoum | Isfar.tariq@att.com Salam.akoum@att.com |
| OPPO | Wenqiang Tian | tianwenqiang@oppo.com |
| Google | Yushu Zhang | yushuzhang@google.com |
| Huawei, HiSilicon | Yuan Li Keyvan Zarifi | liyuan3@huawei.com Keyvan.Zarifi@huawei.com |
| Fujitsu | Xin Wang Qun Zhang | wangxin@fujitsu.com zhangqun@fujitsu.com |
| CAICT | Xiaofeng Liu | Liuxiaofeng1@caict.ac.cn |
| Futurewei | Baoling Sheen | bsheen@futurewei.com |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| InterDigital | Moon-il Lee | Moonil.lee@interdigital.com |
| ZTE | Lun Li Guozeng Zheng | li.lun1@zte.com.cn zheng.guozeng@zte.com.cn |
| vivo | Jianming Wu | jianming.wu@vivo.com |
| Ericsson | Mattias Frenne | Mattias.frenne@ericsson.com |
| Fraunhofer | Ebrahim Amiri | ebrahim.amiri@iis.fraunhofer.de |

# Potential specification impact for CSI compression with two-sided model

## Training collaboration

Three types of training collaboration were agreed in RAN1 110. Following table summarize company’s proposals and observations related to each type of training collaboration.

| Company | View |
| --- | --- |
| Futurewei | Observation 1: For CSI compression using two-sided model sub use case, our views on the pros and cons across training collaboration types for the identified aspects are summarized in Table 2.1-1 based on the Proposed observation 2-1-1(v3) discussed during RAN1#112bis-e. Observation 2: For CSI compression using two-sided model sub use case, pros and cons of each training type may depend on various factors like how training dataset is designed, whether additional information is available, what training technique is leveraged, etc. Thus, many of the pros and cons, or feasibility may be subjective and implementation-dependent, not objective. Observation 3: For CSI compression using two-sided model sub use case, no specific training type can provide all the desirable benefits based on the aspects analyzed and it may not be practical to prioritize based on the aspects identified so far across different views from companies. Proposal 1: Combine the “feasibility” and “extendibility” aspects in the pros and cons table into one as follows. Training types Characteristics; Type 1; Type 2; Type 3. NW-sided; UE-sided; NW-sided; UE-sided. Feasibility of allowing UE side and NW side to develop/update models separately, including train a new UE/NW-side model compatible with NW/UE-side model in use; Feasible but limited (Note 2); Feasible but limited (Note 2); Infeasible; Feasible/Support; Feasible/Support. Proposal 2: For CSI compression using two-sided model sub use case, if prioritization among various training types is needed for Rel-18 and agreed among companies, prioritize the training type(s) that involve less specification impact and/or have less LCM and interworking complexity for Rel-18. |
| vivo | - Do not further categorize NW-sided type 1 training into device-specific and device-agnostic training, as further categorization will make the table too complicated for making observations - Clarify in note 3 how to guarantee the performance of type 3 training if assuming information on model structure is not required to be disclosed in training collaboration type 3. - Clarify the reason to say “Restricted” for type 1 training for “Whether gNB/device specific optimization is allowed”. - Clarify the reason to emphasize “with assisted information signaling” for UE-sided type1 training and UE-first type3 training for “Flexibility to support cell/site/scenario/configuration specific model” and “Model update flexibility after deployment”. - For the issue “Whether training data distribution can match the inference device”, the expression for NW-sided type 1 training should be “Conditional, with assisted information from UE”. - Clarify the reason to say “Conditional, with assisted information from UE” for type 2 training for the issue “Whether training data distribution can match the inference device”. - For the issue “Software/hardware compatibility (Whether device capability can be considered for model development)”, the expression for NW-sided type 1 training should be “Yes for z4”. |
| Ericsson | - As training will not be part of RAN1 specifications (since training will be performed between vendors outside 3GPP realm), it is unnecessary to spend a lot of time discussing finer details of such training collaboration types in a table. |
| ZTE | Proposal 1: Deprioritize the discussion of the pros and cons of NW-sided Type 1 training from two types of device agnostic and device specific. If needed, some conditions can be added to reflect this issue. Proposal 2: The gradient-exchange based sequential training should be categorized into Type 2 training, and then Type 2 training can be classified as ‘Joint/Sequential gradient-exchange training’ in the table. Proposal 3: Prioritize Type 1 joint training at NW side for further study and model transfer/delivery can be further discussed in agenda item 9.2.1. Proposal 4: For training Type 3, NW-first training should be prioritized over UE-first training. |
| Spreadtrum | Proposal 1: To facilitate the discussion, views on Pros and Cons of all of Training types are needed to be aligned. What shown in Table 1 can be considered. Proposal 2: For AI/ML model training type 2 and type 3, data delivery is needed. Proposal 3: For AI/ML model training Type 1, data collection may be not needed to be specified other than assisted signalling, e.g, antenna layout for one CSI-RS resource. Proposal 4: Offline AI/ML model training is the first priority. |
| Huawei | Observation 5: For CSI compression with two-sided model, training Type 1 may suffer software/hardware compatibility issue, and the following restrictions/issues may need to be considered to relieve the compatibility issue: - Network may have to interoperate with various UE vendors/UE versions to dedicatedly train the CSI generation part for UE, which harms the engineering isolation. - Network, in particular gNB, may have to maintain/store multiple CSI generation parts trained for different UE vendors/UE versions. - Network vendor may not freely develop the CSI generation part for UE, which may restrict the pairing with the CSI reconstruction part and thereby result in a sub-optimal performance. Observation 6: For training Type 1 of CSI compression, compared with joint training at Network side, performing joint training at UE side and delivering the model to the Network incur extra challenges for Network due to the following reasons: - Inconvenience of training cell/scenario specific models. - Inflexible model update. - Burden of inference/storing/running multiple Network part models at gNB delivered from different UE vendors/UE versions. Observation 7: For training Type 2 of CSI compression and model update, it relies on complex design to support real-time interaction of FP/BP iterations between Network side and UE side, which causes major challenges to engineering isolation especially for the case of multi-Network vendors to multi-UE vendors. Observation 8: For training Type 3 of CSI compression, - The shared dataset is comprised of the CSI-related data which may be irrelevant to the user privacy (e.g., user position, etc.). - The dataset sharing/delivery can be performed under the contract agreement between the Network vendors/MNOs/UE vendors to mitigate the data ownership problem. Observation 9: For training Type 3 of CSI compression, compared with NW first training, performing UE first training incurs extra challenges for the Network due to the following reasons: - Inconvenience of training cell/scenario specific models. - Inflexible model update. - Burden of maintaining/storing multiple Network part models at gNB to pair with multiple UE vendors/UE versions. Proposal 11: For training Type 1 of CSI compression, deprioritize the mode of joint training at UE side and delivering the model to the Network. Proposal 12: Gradient exchange sequential training is an implementation case of training Type 2. Thus, it can be categorized into training Type 2 for the pros/cons analysis. |
| CATT | Observation 1: In CSI compression using two-sided model use case, for training collaboration Type 1, it has the following pros and cons: - Pros: - Optimal performance can be achieved; - For joint training at UE side, - Dataset sharing might not be needed. - Maintaining/storing a single/unified model at UE side can be supported. - For joint training at NW side, - Cell/site/scenario specific model can be supported easily. - Maintaining/storing a singl e/unified model at NW side can be supported. It is possible that UE does not need to maintaining/storing models for lots of gNBs. - Cons: - Model transfer is needed. - Model updating is lack of flexibility after deployment. - For joint training at UE side, - It is challenging for a UE to support cell/site/scenario specific model. - A gNB has to maintain/store multiple models for multiple UEs. - gNB specific optimization is not supported. - For joint training at NW side, - UE specific optimization is not supported. Observation 2: In CSI compression using two-sided model use case, for training collaboration Type 2, it has the following pros and cons: - Pros: - gNB/device specific optimization is supported. - Model transfer is not needed, which can keep model proprietary. - Cons: - The latency on model training is large. - There is heavy burden in air interface on real-time information exchange between NW side and UE side. - Model updating is lack of flexibility after deployment. - Further study is needed on maintaining/storing a single/unified model at both sides of network and UE. - It is not easy to support cell/site/scenario/configuration specific model. Observation 3: In CSI compression using two-sided model use case, for training collaboration Type 3, it has the following pros and cons: - Pros: - Model transfer is not needed. - gNB/device specific optimization is supported. - For sequential training starting with NW side, - Good extendibility. - Cell/site/scenario specific model can be supported by NW side easily. - It is feasible to maintaining/storing a single/unified model at NW side. - For sequential training starting with UE side, - It is feasible to maintaining/storing a single/unified model at UE side. - Model transfer is not needed, which can keep model proprietary. - Cons: - Dataset transfer from the starting with side to the other side requires extra data transfer overhead. - For sequential training starting with NW side, - Further study is needed on the feasible of maintaining/storing a single/unified model at UE side to adapt to various NW sides. - For sequential training starting with UE side, - Bad extendibility. - It is difficult to support cell/site/scenario specific model. - Further study is needed on the feasible of maintaining/storing a single/unified model at NW side to adapt to various UEs. |
| Panasonic | Observation 10: Type 1 training involves the exchange of AI/ML model and then, requires some common AI/ML inference algorithm and common reference for model inference. Observation 11: For Type 2 with offline training, if the consideration on the air interface specification impact on FP/BP interaction is not needed, there might be no Type 2 specific specification impact. Observation 12: For Type 3 training collaboration with network-first training, at least the option that network generates training dataset to enable UE side supervised learning should be studied. Observation 13: For Type 3, 3GPP may need to define some kind of requirement of CSI encoding by input and output relation, performance test or something else. The input for the training can be 3GPP specified channel model or field raw data. The output for the training can be something 3gpp defined output or network vendor specific information. The UE model performance can be checked by 3gpp specification or inter-operability test (IOT). Observation 15: Type 3 with network-first separate training might be feasible options at least Re.18/19 timeline from standardization effort perspective. Type 1 with network sided training can be potential in the long-term. |
| Xiaomi | Proposal 1: Device-specific for type 1 can be considered to train two-sided AI/ML model. Observation 1: NW may store and manage a lot of NW-sided part models for joint training of the two-sided model at UE side for type 1 and UE-first separate training for type 3. Proposal 2: The pros and cons of the joint training of two-sided model at NW side for type 1 with device-specific and NW-first separate training for type 3 should be firstly studied and discussed. |
| AT&T | Proposal 1: In CSI compression using two-sided model use case, discuss the pros/cons of different offline training collaboration types and select the training collaboration type(s) for CSI compression that will be prioritized and later studied in the WI. |
| CAICT | Proposal 1: Proposed observation 2-1-1(v3) in last meeting is agreeable. Proposal 2: If further proposals based on the proposed table, Type 1 NW-side and type 3 NW/UE first training should be considered with higher priority. |
| Fraunhofer | Observation 1: The same AI-model can be trained by Type 1 or Type 2, therefore Type 1 and Type 2 training may result in the same performance. However, in Type2, two vendors located at different places are used for the training, this causes a lot of offline overhead for data exchange and synchronization. Observation 2: If the confidentiality of the details of the two-sided model is crucial, which means the UE or gNB vendors concern about revealing their data and/or implementation to each other, Type 3 is the only applicable option. Observation 3: Some of the AI models, which require joint training, cannot be implemented using separate training, therefore Type 3 or any separate training method does not outperform the joint training approaches e.g., Type 1, 2. Proposal 1: Separate training Type 3 is applied when there is no possibility for full coordination between the UE and gNB vendors. |
| CMCC | Observation 1: For CSI compression using Type 1 training collaboration, the model could not be kept proprietary; for CSI compression using Type 2 training collaboration, the model could be kept proprietary; for CSI compression using Type 3 training collaboration, the model could be kept proprietary. Observation 2: For CSI compression using Type 1, Type 2 and Type 3 training collaboration, the dataset for sharing is not privacy-sensitive. Observation 3: For Type 1 training, it is flexible to support cell/site/scenario/configuration specific model, but UE-sided training needs some assisted information signaling; for Type 2 training, it is difficult to support cell/site/scenario/configuration specific model; for Type 3 training, it is semi-flexible to support cell/site/scenario/configuration specific model, but UE first training needs some assisted information signaling. Observation 4: For Type 1 training, it is restricted to support gNB/device specific optimization; for Type 2 and Type 3 training, it is feasible to support gNB/device specific optimization. Observation 5: For Type 1 training, it is flexible to model update after deployment, while it is semi-flexible for Type 3 training; for Type 2 training, it is not flexible to model update after deployment. Observation 6: For Type 3 training, it is flexible to allow UE side and NW side to develop/update models separately, while it is inflexible for Type 2 training; for Type 1 training, it is limited to allow UE side and NW side to develop/update models separately. Observation 7: For Type 1 NW-sided training, gNB could maintain/store a single/unified model while UE device could not; for Type 1 UE-sided training, UE device could maintain/store a single/unified model while gNB could not. Observation 8: For Type 3 training, it is supported to train new UE-side model compatible with NW-side model in use or to train new NW-side model compatible with UE-side model in use (extendibility); for Type 1 and Type 2 training, it is limited. Observation 9: For Type 1 UE-sided training and Type 3 UE first training, the training data distribution can match the inference device; while for Type 1 NW-sided training and Type 3 NW first training, it needs some assisted information from UE. Observation 10: For Type 2 training, Type 3 training and Type 1 UE-sided training, the software/hardware compatibility could be considered; while for Type 1 NW-sided training, it is conditional with some assisted information from UE. |
| NVIDIA | Proposal 5: In CSI compression using two-sided model use case, study the pros and cons of the following AI/ML model training collaborations: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, respectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. |
| Lenovo | - Study the training collaboration types considering the communication overhead and/or the corresponding latency, based on whether the communication between the network and UE sides during model training and model adaptation occurs over the NR air interface or via proprietary signaling - Study the advantages/disadvantages of joint training at the UE side vs. joint training at the network side with Type 1 training collaboration - Study the performance of iterative separate training as one of the methods to improve the performance of sperate training when multiple vendors are involved in training on the two sides of communication |
| Qualcomm | Observation 1: For the AI/ML-based CSI feedback enhancement use case, the use of an AI/ML model for inference within a device would require prior offline device-specific optimization and testing. Observation 2: Type 1 training with device-agnostic encoder would result in a UE-side model that: - is not optimized in a device-specific manner for the intended UE-side device, - assumes a structure and input format that is not compatible with the UE-side implementation capabilities, and - may have sub-optimal performance due to a discrepancy between the training and inference data distribution due to device-side variations. Observation 3: Type 1 training performed on the NW-side with involvement of the UE-side vendor requires the UE-side to provide information (such as model structure, pre-processing, post-processing, datasets and ground truth) to the training entity to ensure that the trined models are suitable for inference. Observation 4: For NW-side type 1 training with UE-side involvement, developing a new model for a new UE device type or vendor can result in a large engineering effort across multiple vendors. Observation 5: It is feasible to train a two-sided AI/ML model using an offline Type 2 (multi-vendor) training approach with performance comparable to Type 1 training. Observation 6: For type 2 training, developing a new model for a new UE device type or vendor can result in a large engineering effort across multiple vendors if the NW-side or UE-side use a common model for multiple models on the opposite side. Observation 7: As compared to Type 2 training, the Type 3 offline training approach is more flexible as it does not require coordination during the training process. Observation 8: For Type 3 separate training, the engineering effort of adding a new UE type or new UE-side vendor is contained and does not propagate to other vendors even if the NW-side or UE-side use a common model for multiple models on the opposite side. Observation 9: For NW-first sequential training, the training based on gradient exchange provides several benefits in terms of flexibility in the input type, better alignment between the UE-side and NW-side model training, aligned dataset and avoiding disclosure of proprietary information. Observation 10: It is feasible to train a common NW-side model that is compatible with multiple UE-side models using Type 2 or Type 3 training approach with performance comparable to Type 1 training. Observation 11: Training type 1 (with device-specific encoder), training type 2 and training type 3 are applicable to both collaboration level y and level z. Proposal 1: Model development and training options should consider the need for the UE-part of two-sided AI/ML models to be designed based on the UE capabilities and optimized in a device-specific manner. Proposal 2: Model development and training options should strive for the principle of engineering isolation, i.e., confining engineering effort needed for a new chipset/UE development to the given chipset/UE vendor. Proposal 3: Model development and training options need to consider whether the model is developed for common use across a group of UEs or is developed for an individual UE. Proposal 4: Model development and training options need to consider feasibility of disclosing proprietary model information to the other side. Proposal 5: For AI/ML-based CSI feedback enhancement use-case, take offline training as a starting point. Proposal 6: Deprioritize Type 1 training with device-agnostic encoder in the R18 study. Proposal 7: Adopt the following two-sided model development/training framework: - Case 1: Initial (non-backward-compatible) development/training of “nominal encoder + nominal decoder” - The use of the nominal encoder at the UE-side is not mandated - If needed, UE-side may implement a different proprietary encoder based on this decoder using Case 2. - As the encoders are only nominal, input used in the training process is only a nominal input. The actual input to the CSI encoders may be different and of proprietary choice. - The use of the nominal decoder at the NW-side is not mandated - If needed, NW-side may implement a different proprietary decoder based on this encoder using Case 3. - Case 2: Encoder development/training to be interoperable with existing decoders (e.g., encoders for new UEs or updating encoders for existing UEs): - UE-side vendor trains new encoders based on the existing decoders. - Infra vendor should make the existing decoders available (via either a run-time image or an API for training) for the encoder training. - Case 3: Decoder development/training to be interoperable with existing encoders (e.g., decoders for new cell sites or updating decoders for existing cell sites): - Network-side vendor trains new decoders based on the existing encoders. - FFS: Need for encoder availability for decoder training Proposal 8: Include rows in the training types pros and cons table to indicate the defining assumptions of each training type for the following aspects: collaboration level, device-specific/agnostic encoder, encoder training vendor, decoder training vendor, model transfer, multi-vendor assumption for joint training, data collection for encoder training and decoder training. |
| oppo | Observation 1: For training collaboration types 1 and type 3, the conclusions for most of the questions listed below are the same, but the implementation methods are different. These two training collaboration types are not exclusive, and both can be considered in subsequent research Observation 2: If restrictive descriptions are needed to differentiate the pros and cons for different training types and different characteristics in a table or in the TR eventually, clarifications/explanations for all restrictive terms as well as the scope (e.g. for the whole table, or for a given characteristic, or a given training type) should be added e.g. in notes. |
| Samsung | Proposal 2-8: For Type 3 training collaboration, study performance impact of training/testing an encoder with a reference decoder or dataset. Proposal 2-9: For AI/ML based CSI compression sub-use case, study and verify model update of the encoder at the UE, where the gNB’s training strategy is not disclosed while transferring/configuring the AE. |
| NTT Docomo | Observation 1: Characteristics of type 4 training procedure can be summarized as Table 1. Proposal 1: Categorize type 3 and type 4 training procedure (sequential training) as follows. - Type 3: sequential training via the dataset delivery - Step 1 Joint training at one side - Step 2 Delivery of the dataset produced by trained encoder/decoder to the other side - Step 3 Training encoder/decoder based on the delivered dataset within the other side - Type 4: sequential training via the gradient exchange - Step 1: Joint training at one side - Step 2: Share the common dataset for training at Step 3 - Step 3: Training encoder/decoder at the other side via FP/BP exchange with the frozen decoder/encoder Proposal 2: Study pros and cons of the type 4 training procedure in the aspects agreed to study. |
| MediaTek | - Keeping a single universal AI/ML model at NW for training type 1 should be deprioritized. - Further categorize NW-side training type 1 based on the number of available AI/ML models and their target UE devices. - Discuss feasibility of synchronization/alignment required for different update scheduling in training type 2. |
| ITL | Observation 1: For CSI compression using two-sided model use case, Type 1 NW side joint training may have following restrictions/issues mainly on the compatibility related to model and device: - NW vendor may not freely develop the CSI generation model for UE, which may be restricted by UE device capabilities, model structure and pairing with the CSI reconstruction part and thereby result in sub-optimal performance of CSI compression based on AI/ML model - NW may be restricted to maintain/store a single/unified model if there is no proper assisted information from multiple UE sides Observation 2: For CSI compression using two-sided model use case, Type 1 UE side joint training incurs more challenges compared to that of NW side joint training Observation 3: For training Type 3 of CSI compression, it seems be most flexible and compatible training Type among all the Types with the acceptable signaling overhead (e.g., With assisted information signaling) Observation 4: For training Type 3 of CSI compression, it is important to study how to perform the dataset sharing/delivery between NW vendors and UE vendors, to avoid conflict on the data ownership |
| IIT Kanpur | Proposal-1: (Option-a) Type-1 NW-side device-agnostic model should be trained/deployed to achieve a minimum x threshold performance for at least y% of user devices; (Option-b) Type-1 NW-side device-agnostic model should be trained/deployed only in scenarios/cell/sites where a minimum x threshold performance by at least y% of user devices is achievable. |

### Summary:

Metrics to facilitate pros/cons discussion of each training collaboration type were captured in RAN1 112. In RAN1 112bis-e, companies have provided views on how to capture the discussion.

- Proposals that do not further categorize NW-side type 1 training into device specific and device agnostic training, as further categorization will make the table too complicated for making observations.

- Proposal to further break down training collaboration type 1 based on model transfer level y, z4/z5 etc.

- Proposals to limit the overall discussion since the discussion does not have specification impact.

- On the gradient exchange sequential training, there are proposals to capture it as part of training collaboration type 2, where training type 2 including both joint training and sequential training.

- There are proposals to get it separately as training type 4.

Given the conflicting proposals and the fact those observations of pros/cons do not have specification impact, therefore FL recommend keeping the discussion on high level. Before we work on the proposed observation, let us agree on the definition and structure first.

On how to capture gradient exchange sequential training, we have two options: one is to modify type 2 to include sequential training. The other is to define training collaboration type 4. FL would like to get companies’ input on which way is the preferred way. The current type 2 definition is copied below for easier reference.

- Type 2: Joint training of the two-sided model at network side and UE side, respectively.

On how to capture device agnostic and device specific training under type 1 NW first, given the comments not to further complicate the table, FL recommend prioritizing device specific model, where model structure is known and model transfer is mainly parameter update.  Device agnostic aspects can be captured in notes if needed.

### Discussion 2-1-1:

In CSI compression using two-sided model use case, to capture the gradient exchange sequential training methods, please indicate which is the preferred methods:

- Option 1: modify type 2 training collaboration to include sequential training.

  - Type 2: Joint or sequential training of the two-sided model at network side and UE side respectively, through gradient exchange between nodes.

- Option 2: define a new training type:

  - Type 4: Sequential training of the two-sided model at network side and UE side respectively, through gradient exchange between nodes.

Please indicate your preference below:

| Option 1 | ZTE |
| --- | --- |
| Option 2 | Qualcomm |

Additional view if any:

| Company | View |
| --- | --- |
| Lenovo | Do not support adding a new collaboration type. Also, there was a conclusion in RAN1#111 to deprioritize Type 2 collaboration, we do not understand the basis under which Type 2 is being expanded |
| Futurewei | We agree with Lenovo’s feedback on not adding additional collaboration type and gradients exchange over air-interface is agreed to be deprioritized. |
| Lenovo 2 | For the sake of both progress and clarity, can we suggest the following rewording: - Type 2: Joint or sequential training of the two-sided model at network side and UE side respectively, through gradient exchange of information between nodes in each epoch. |
| NTT DOCOMO | Even if the Option1 is adopted, the pros and cons should be discussed separately for joint training and sequential training in type 2. As the characteristics, e.g., extendibility, are different for type 2 and type 4, it is better to differentiate them. @Lenovo and Futurewei The conclusion states deprioritizing it over air-interface. That conclusion is not about offline training where the training is performed outside 3GPP. |
| Huawei, HiSilicon | We do not support Option 2. Current definition of Type 2 training refers to “joint training” at NW and UE sides which includes both “simultaneous” training of NW and UE parts and “sequential” training of NW and UE parts. For Option 1, the changes seem unnecessary either. If need be, we can add a note to the current definition of Type 2 as follows: - Type 2: Joint training of the two-sided model at network side and UE side, respectively. - Note: Joint training includes both simultaneous training and sequential training. |
| Google | OK with HW’s revision |
| ZTE | We prefer Option 1. For the training types of joint gradient exchange and sequential gradient change, we think the characteristics for both types may be similar and these two types can be incorporated in Type 2 training. In addition, based on the previous two conclusions for deprioritizing joint gradient exchange and sequential gradient change over the air, so we think option 1 is more reasonable for further discussion. |
| vivo | We agree with Lenove’s comment as well. The gradient exchange sequential training method belongs to Type-2 and relies on the air-interface, which is already deprioritized. |
| Qualcomm | We prefer Option 2. Our main concern is that the pros and cons for the sequential gradient exchange method are quite different from joint Type 2 training. For example, unlike Type 2, it allows separate model development on UE-side and NW-side – it is an example of NW-first training. Also, it allows extensibility – a new UE-side model can be trained that it compatible with an already trained NW-side model. Calling it Type 4 would help to avoid confusion in the pros and cons discussion by capturing it in a different column. In terms of the terminology, in our understanding, “joint” training is when both models are trained together and does not include “sequential” training where the training happens one after the other. |
| OPPO | Share similar view with Lenovo. In 3GPP, it is already concluded to deprioritize type 2 training with gradient exchange. If here is discussing 3GPP transparent training, we do not think we should handle this issue in 3GPP discussion. |
| NEC | We have the similar view as Lenovo, Futurewei, vivo and OPPO. It should be noted that gradients exchange over air-interface for Type 2 was agreed to be deprioritized in RAN1#111. |
| CATT | We already have joint-training and separate (sequential) training, and now it is getting to joint + separate training. There may be no much inner difference between these two options. We are open to either way. But exchanging gradient is an iterative progress, which is time consuming and involving many feedback procedures. In this sense this method is not attractive, regardless OTA or offline, and regardless how to categorize it. |
| LG Electronics | Fine with HW’s revision. |
| Xiaomi | We prefer to deprioritizing the discussion on this training type. This type has been discussed in AI 9.2.2.1. We can wait for the preliminary conclusion in AI 9.2.2.1 before discussing it. |

### Proposal 2-1-2:

In CSI compression using two-sided model use case, for training collaboration type 1 at NW side, prioritize the pros/cons discussion assuming a known model structure at UE.

| Objecting companies |  |
| --- | --- |
| Supporting companies | Futurewei, Panasonic, NTT DOCOMO, vivo, Qualcomm, Xiaomi |

Additional view if any:

| Company | View |
| --- | --- |
| Lenovo | Based on company views and preferences provided in the training collaboration comparison table, we believe discussion on Type-I should be deprioritized. |
| Huawei, HiSilicon | Not support. We don’t think device specific training should be prioritized to the device agnostic as, in our view, the device agnostic training method also has its own pros compared to the device specific counterpart including engineering isolation and maintaining a unified model at the NW. At the same time, since both device agnostic and device specific training methods share many similar pros and cons, it may not be needed to add a new column in the Table. For the few entries of the table that device specific and agnostic training methods have different properties, the property of device agnostic and device specific methods should be both mentioned (eg, for Engineering isolation, it is isolable for device agnostic and non-isolable for device specific). Finally, device specific and device agnostic training collaboration type 1 are respectively related to the Model delivery/transfer case z4 and z5 that are being discussed in 9.2.1. We think that if a prioritization is necessary, it should be discussed in 9.2.1. |
| Google | OK |
| ZTE | Agree with Huawei. |
| vivo | We are fine with the proposal. |
| Qualcomm | Device agnostic Type 1 training may not be feasible as the model does not account for the UE’s capability. Device-specific Type 1 training at NW side allows the NW vendor to take the UE capability and UE model structure into account when training the UE-side model. We support prioritizing the device-specific version for the pros/cons discussion. If both are captured, they should be captured in different columns, as otherwise it would be difficult to interpret the pros and cons as several answers differ. In addition, Level y Type 1 training should also be captured, as it is not necessarily covered by NW-side or UE-side, and could also be at a neutral site. |
| OPPO | We should discuss the pros/cons for both known model structure and new model structure. To HW, does device specific model equal to the known model structure in this proposal? From our understanding, seems they are two different discussion. |
| CATT | OK if this is for spec impact discussion, but if just for pros/cons analysis we agree with HW that both can be analyzed. |

### Proposed observation 2-1-1 (To be updated based on 2-1-1 and 2-1-2):

In CSI compression using two-sided model use case, the following table capture the pros/cons of different offline training collaboration types:

## Data collection

Following table summarize company’s proposals related to data collection.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| vivo | - Meta information reporting for data collection should be studied to facilitate the development of scenario-/area-/configuration-specific models. - The necessity of reporting certain kind of meta information in data collection depends on model’s generalization ability on it. - Enhanced legacy codebook can be used for data collection (CSI measurement), and enhancements for different data collection purpose can be different - RAN1 could send LS to RAN2 to clarify the requirement of data collection in CSI compression (and other use cases). |
| Ericsson | - For CSI compression use case, it is required that standardized procedures and associated data format for UE to gNB data collection of a high-resolution CSI (target CSI) is supported to enable model monitoring and to provide data for enabling decoder fine tuning. RAN2 can assume a non-real time latency requirement for collecting training data for training of CSI compression models. The corresponding latency for monitoring of such models is FFS. - It is beneficial if the UE can report data from multiple measurement occasions in a single data collection instance which can be scheduled in a suitable time instance considering network load. - RAN2 can assume a collection periodicity/frequency of days for initial training of generalized CSI compression models. The corresponding collection periodicity/frequency for monitoring of such generalized models is FFS. - For data collection for CSI compression, support reporting of assistance information associated with the MIMO channel measurement data. The data size for collection in the CSI compression use is initially estimated to 13 kbit which is a ballpark value RAN2 can use in their further studies. |
| ZTE | Proposal 5: For training Type 3, further study potential specification impact on the dataset used for the model training at the other side/entity. Observation 1: When model training or monitoring is performed at network side, the overhead of the ground-truth label transmitted over the air-interface from UE to network is a huge concern if the ground-truth CSI is an ideal CSI (e.g., raw channels, eigenvectors). Observation 2: The overhead of enhanced Type II CB (i.e., PC10) for one training sample increases by 50% compared with the maximal payload of Rel-16 TypeII CB (i.e., PC8) but keeps similar model performance as ideal CSI, which can be acceptable to be carried on UCI. Proposal 6: For network side data collection, support to further study - Enhanced Rel-16 TypeII codebook to get high-resolution CSI; - PHY signaling or RRC signaling to report the high-resolution CSI. Proposal 7: To enable high-quality data collection, at least support - UE reports associated information to NW, e.g., SINR, CQI, positioning information - NW configures a threshold of data quality to UE and UE only reports the qualified data to NW Observation 3: For Type 3 training collaboration of a two-sided model, common understanding on the dataset used for model training is necessary, which can facilitate the pairing of CSI generation part and CSI reconstruction part. Observation 4: Dataset alignment between UE and network can be used for testing/monitoring the model/functionality performance. Observation 5: Dataset ID can avoid sharing the proprietary information explicitly across vendors during data collection. Proposal 8: Support to use dataset ID to identify the delivered dataset of Type 3 training collaboration. |
| Spreadtrum Communication | Proposal 5: If model transfer supported, data collection procedure may be not needed. Proposal 6: If model transfer not supported, for UE side, data collection procedure may be needed. Proposal 7: If model transfer not supported, for NW side, data collection procedure may be needed or not depending on whether SRS can be utilized. |
| Huawei | Observation 3: The usefulness of the categorization ID for assisting UE side data collection needs to be further clarified considering the following points: - UE can train a generalized model that is applicable to multiple scenarios/antenna layouts. - UE can autonomously sense the scenario without the need for gNB notification. - The categorization or granularity of the scenarios identified by Network may not match the categorization principle of the UE side. Observation 4: Specifying the dataset delivery for training Type 3 over air-interface can alleviate the per vendor basis offline interoperation and customization of the dataset delivery type/format during the development of the AI/ML feature/models. Proposal 2: For the enhancement of CSI-RS configurations for Network/UE side data collection under CSI compression, separate CSI-RS resources/CSI reports can be adopted for generating ground-truth CSI labels (e.g., measured with higher power/density CSI-RS) and model inputs (e.g., measured with lower power/density CSI-RS) to support the use of low resolution input to infer high resolution output. Proposal 3: For the Network side data collection under CSI compression, - Both L1 and RRC signaling can be supported for model training. - At least L1 signaling should be supported for model monitoring to enable a fast identification of the AI/ML model performance/failure. Proposal 4: For the Network side monitoring, it is necessary to enable ground-truth CSI based monitoring rather than only relying on UE side monitoring and report based on the proxy model, which has the following drawbacks: - The imbalanced generalization performances between the proxy model at UE and the actual CSI reconstruction part at gNB will lead to a degraded monitoring accuracy at the UE side when the channel environment changes. - UE side proxy model is likely to operate under collaboration level x, since its additional LCM will impose huge burden on gNB, including model/functionality identification, monitoring, activation/deactivation/switching/fallback, etc., of the UE side proxy model. Without such additional LCM, the performance and robustness of the proxy model are not trustable at gNB. - In particular, how to monitor the performance of the UE side proxy model is not clear. Proposal 5: For data sample format of Network side data collection under CSI compression: - Both scalar and codebook-based quantization can be supported for model training. - Only codebook-based quantization should be supported for model monitoring. Proposal 6: For the Network side data collection of ground-truth CSI, to enable gNB to efficiently collect number of data samples for the wanted layers, the rank and the index(es) of layer(s) for the report of ground-truth CSI can be determined by the gNB rather than autonomously calculated and reported by the UE. Proposal 7: The data categorization ID, if justified, should be determined by Network as a virtualized ID without specifying the physical meaning. - The physical meaning and the granularity of such ID is up to the Network implementation and is not indicated to the UE. Proposal 8: In CSI compression using two-sided model with training collaboration Type 3, further study the following aspects of potential specification impact of dataset delivery over the air-interface: - CSI reconstruction model training dataset and/or other information delivery from UE side to Network side for UE first training. - CSI generation model training dataset and/or other information delivery from Network side to UE side for NW first training. - The specification impact includes dataset ID, dataset size, data sample format, data sample type(s), quantization/de-quantization related information, etc. Proposal 9: For the dataset delivery of CSI compression over air-interface, the following approaches can be considered to substantially reduce per UE overhead/power consumption: - Quantization of the ground-truth CSI with high resolution quantization format, e.g., R16 Type II-like method with new parameters. - Network splits the overall dataset into many subsets each with a limited number of data samples (e.g., with an overhead comparable to the RRC signaling). The subsets can be separately sent to different UEs, and all subsets are associated with a common dataset ID for the UE side recombination. Proposal 10: For the dataset delivery of CSI compression over air-interface, the dataset ID associated with the delivered dataset can be used to achieve the pairing of the Network part model and the UE part model. For instance, for NW first separate training, UEs receive the dataset associated with a dataset ID to perform the training; after the UE part model is trained, UE and gNB will use the dataset ID to achieve the pairing. |
| CATT | Observation 4: In CSI compression using two-sided model use case, for data collection for model training, enhancement on CSI-RS is not needed. Proposal 1: In CSI compression using two-sided model use case, it is necessary to support NW side data collection. Proposal 2: In CSI compression using two-sided model use case, for NW side data collection for model training, focus on studying CSI-RS measurement based data collection. Proposal 3: In CSI compression using two-sided model use case, on ground-truth CSI reporting for NW side data collection for model training, study potential specification impact on the following schemes: - Option 1: Ground-truth CSI samples are reported by physical layer signaling, with legacy CSI feedback framework reused; - Option 2: Ground-truth CSI samples are reported by RRC signaling, with a batch of ground-truth CSI samples reported together. Proposal 4: In CSI compression using two-sided model use case, for NW side data collection for model training, collecting ground-truth data in type of precoding matrix is supported. Proposal 5: In CSI compression using two-sided model use case, for NW side data collection for model training, NW determines the number of layers for ground-truth CSI data collection. |
| China Telecom | Observation 1: The latency requirement of different data collection purpose generally different. E.g., model training are generally non real time, real time reporting is necessary for model performance monitoring. Proposal 1: In CSI compression using two-sided model use case, Ground-truth CSI reporting for NW side data collection for model training can be supported by both of physical layer signalling and RRC signalling. Proposal 2: For RRC-based ground-truth CSI reporting, new triggering mechanism is needed. Proposal 3: The input or output data based monitoring can be used as an assistance information for model switching/selection, and need FFS. |
| Panasonic | Observation 1: In CSI compression using two-sided model use case, it is necessary to use the ground-truth CSI of realistic DL channel measured by UE and report to network. Observation 2: Data collection for model training is not required to be real-time and then latency requirement can be larger. Observation 3: At least for data collection for performance monitoring, in order to handle multiple UE vendors and/or UE models, the reporting of ground-truth CSI should be performed using 3GPP signaling to avoid the complexity of handling multiple formats. Observation 4: Depending on the requirement of latency, grouped reporting could be realized through MAC-CE RRC or U-plane, and sample-by-sample reporting is better to be implemented via UCI. Observation 5: On data sample type / format for ground-truth CSI reporting, high resolution codebook-based format e.g., legacy codebook (e.g., eType II codebook) with potential enhancements such as extend more configurations in some parameters, should be studied. Observation 6: For network-side data collection, at least time stamps/cell ID and UE location should be considered as the assistance information. Observation 7: For network-side data collection, the necessity and feasibility of UE reporting Rx antenna spacing and Rx RF gain imbalance to network should be studied. Observation 8: For UE-side data collection, to identify the scenario / configuration in which the data is being collected, virtualized configuration ID should be studied as the assistance information. Observation 9: If CSI-RS / SRS configurations in current NR specification is not sufficient for higher accuracy measurement, enhanced CSI-RS and/or SRS may be considered for the data collection. |
| Xiaomi | Observation 2: For UE side and network side collection, it is not necessary to enhance CSI-RS resource since higher accuracy channel measurement can be obtained by current CSI-RS resource configuration. Observation 3: For UE side and network side data collection, the data ID is not necessary to report as an assistance information considering that the data ID can be obtained in a proprietary way or network configuration. Proposal 3: For network side data collection, cell-specific CSI-RS resource configuration can be supported to reduce configuration signalling overhead. Proposal 4: For network side data collection for model performance monitoring, the potential specification impact on the ground truth CSI format, e.g., either scalar quantization or codebook-based quantization, can be studied. |
| AT&T | Proposal 2: In CSI compression using two-sided model use case, further study the necessity, complexity, overhead, latency and potential specification impact on ground truth CSI report for NW side data collection for model performance monitoring: - Scalar quantization for ground-truth CSI - FFS: any processing applied to the ground-truth CSI before scalar quantization - Codebook-based quantization for ground-truth CSI - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1 - L1 signaling procedure to enable fast identification of AI/ML model failure performance. |
| CAICT | Proposal 3: The detail format of ground truth CSI for NW could refer to the design of collaboration Type 3. Proposal 4: NW could inform UE about the maximum number of layers for ground truth CSI feedback. |
| NEC | Proposal 1: Study the mechanism of obtaining RSs specific for data collection in model training, model update and model monitoring, e.g., explicit configuration, implicit acquirement. |
| CMCC | Proposal 1: In CSI compression using two-sided model use case, regarding the ground truth CSI format for NW side data collection for model training, Rel-18 codebook for high/medium velocities can be used as a starting point. Proposal 2: In CSI compression using two-sided model use case, regarding the ground truth CSI format for NW side data collection for model training, the basic codebook structure could be reused, along with the basic concept of spatial domain, frequency domain and Doppler domain basis. Proposal 3: In CSI compression using two-sided model use case, regarding the ground truth CSI format for NW side data collection for model training, the exact supported values of codebook parameters can be studied to make sure high resolution data report. Proposal 4: In CSI compression using two-sided model use case with training collaboration type 3, for sequential training, the following can be further studied: - CSI reconstruction model training dataset and/or other information delivery from UE side to NW side - CSI generation model training dataset and/or other information delivery from NW side to UE side - Data sample format/type/assistance information - Quantization/de-quantization related information |
| Lenovo | - For FDD systems with network-based Type-1 model training as well as Type-3 training collaboration, signaling the CSI training data from the UE to the network is needed - Evaluate schemes related to transfer of CSI dataset for different stages of the LCM - Evaluate the following CSI training data signaling techniques: - Alt1. Proprietary signaling via non-3GPP techniques - Alt2. Legacy CSI dataset feedback where the NR codebook-based CSI is utilized as CSI training data - Alt3. Explicit CSI-dataset feedback via enhanced 3GPP-based signaling of the CSI training data - Evaluate the following CSI training data formats: - Alt-A. Legacy codebook-based dataset points generated via multiple occasions of NR codebook-based CSI feedback - Alt-B. High-resolution codebook-based dataset points generated via high-resolution variants of NR-based CSI codebooks - Alt-C. Floating point representation of raw CSI data, e.g., raw channel matrices or sets of channel eigenvectors |
| Qualcomm | Proposal 9: For data collection for model training, RAN1 should focus on what data should be collected. Mechanism for training data collection needs architectural considerations and should be handled by other working groups. |
| Samsung | Proposal 2-6: Deprioritize two-sided model training collaboration that requires extensive sharing of training, validation and testing datasets over the air-interface in this study item. |
| MediaTek | - Discuss the quantization of CSI sample for data collection in the following aspects: - Decisioning entity about configuration - Incorporation of non-quantized CSI for possible finetuning - Quantizable information (CSI samples and assistant information) - Configuration granularity (per sample or per dataset) - For NW-side data collection, while NW is main entity in establishing data collection procedure, UE should provide NW with a range of possible options for configurations of the data collection procedure including but not limited to: - Types of input CSI - Types of assistant information - Quantization parameters - Periodicity of data collection - Maximum amount of data collected per period - To relax the overhead of using air interface for NW-side data collection, NW can use SRS for CSI estimation. Usage of CSI-RS-based CSI can be limited to finetuning purposes. |
| ITL | Proposal 1: For the network/UE side data collection, separate CSI-RS resources/report configurations can be considered for the improved model training and thereby derive the CSI with high resolution by inputting the CSI with low resolution. Proposal 2: It would be beneficial for NW to determine the number of layers for the NW side data collection of ground-truth CSI for model training |
| Apple | Proposal 2: Consider L1 signaling procedure for NW side data collection for performance monitoring. |

### Summary:

For training collaboration type 3, additional dataset needs to be delivered from NW to UE in NW first training, and from UE side to NW side in UE first training.  Whether the dataset delivery are done between servers through offline approach, or over the air interface has been discussed and not agreed in RAN1 112bis-e. Further discussion is required.

Dataset collection for performance monitoring is another main open issue.

### Proposal 2-2-1:

In CSI compression using two-sided model use case, further study the necessity, complexity, overhead, latency and potential specification impact on ground truth CSI report for NW side data collection for model performance monitoring, including:

- Scalar quantization for ground-truth CSI

  - FFS: any processing applied to the ground-truth CSI before scalar quantization

- Codebook-based quantization for ground-truth CSI

  - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1

- RRC signaling and/or L1 signaling procedure to enable fast identification of AI/ML model performance.

Please provide your view below:

| Company | View |
| --- | --- |
| Lenovo | Support |
| Futurewei | We are ok with the proposal. |
| Panasonic | We are fine with the proposal. |
| NTT DOCOMO | Support the proposal. |
| Huawei, HiSilicon | We think it is also important to study and compare the gNB-triggered and periodic ground truth CSI report. A triggered monitoring has an on-demand basis and does not need to be always “ON” in a periodic manner. In particular, the UE report can be switched off after the monitoring window expires. We suggest the following addition to the Proposal: ### Proposal 2-2-1 (modified): In CSI compression using two-sided model use case, further study the necessity, complexity, overhead, latency and potential specification impact on ground truth CSI report for NW side data collection for model performance monitoring, including: - Scalar quantization for ground-truth CSI - FFS: any processing applied to the ground-truth CSI before scalar quantization - Codebook-based quantization for ground-truth CSI - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1 - RRC signaling and/or L1 signaling procedure to enable fast identification of AI/ML model performance. - gNB-triggered or periodic ground-truth CSI report. |
| Google | OK in principle. But we do not think the eType2 codebook should be a pre-requisite FG for ML based CSI compression. |
| ZTE | We are fine with this proposal. |
| vivo | Support |
| OPPO | Support |
| CATT | Support. |
| ETRI | Support |
| LG Electronics | Support |
| Xiaomi | Support |

In addition, proxy model has been proposed to calculate CQI and for performance monitoring. To enable UE to train a proxy model, the dataset from NW need to include proper labeling for training.

### Proposal 2-2-2

In CSI compression using two-sided model use case with training collaboration type 3, for NW first training, further study feasibility and potential specification impact on

- Proxy CSI reconstruction model training dataset and/or other information delivery from NW side to UE side

- Proxy CSI monitoring model training dataset and/or other information delivery from NW side to UE side

Please provide your view below:

| Company | View |
| --- | --- |
| Lenovo | We suggest the following wording: In CSI compression using two-sided model use case with training collaboration type 3, for NW first training, further study feasibility and potential specification impact on - Proxy CSI reconstruction model training dataset and/or other information delivery from between NW side to UE side Proxy CSI monitoring model training dataset and/or other information delivery from between NW side to UE side |
| Panasonic | We think the difference between proxy CSI reconstruction model and proxy CSI monitoring model training should be clarified. |
| Huawei, HiSilicon | We don’t support the proposal. We think that relying on a proxy model at the UE side to monitor the performance has a few drawbacks and, especially at this late stage of the study item, we prefer to conclude many of the current study topics than to introduce a new study topic that does not seem to be promising. As discussed in our t-doc to 9.2.2.2 and further verified by our simulation results in our t-doc to 9.2.2.1, since the proxy CSI reconstruction model is different from the actual network part CSI reconstruction model, e.g., with smaller size/simpler architecture and weaker learning capability, the generalization performance of the proxy model will be different from the actual NW part CSI reconstruction model. It is shown in our t-doc to 9.2.2.1 that, the proxy model monitoring accuracy would degrade when the scenario changes from UMa to InH. On the other hand, the network side monitoring with ground-truth CSI of enhanced parameters show a much higher and more stable monitoring performance when the scenario changes. Additionally, considering the proxy model is used to monitor the performance of actual model, another challenge is how to monitor the performance of the proxy model. Since the proxy model is one-sided model at UE side, one potential way is that it is up to UE implementation and transparent to the network side. That is to say, even if the proxy model is adopted, it is very likely that it will operate under level x collaboration manner, i.e., NW side will not perform model/functionality identification, monitoring, activation/deactivation/switching/fallback of the UE side proxy model. As a result, the performance and the robustness of the proxy model cannot be fully trusted/controlled by gNB. |
| Google | Agree with HW |
| vivo | Support |
| Qualcomm | The term ‘proxy CSI monitoring model’ is unclear. If this is referring to the model that directly estimates SGCS, then ‘intermediate KPI estimator model’ may be a better term to use. The specification impact of dataset delivery is not clear, as this transfer is expected to happen between training servers offline, and not over the air-interface. Please see our comment in Proposal 2-2-3. |
| OPPO | Support |
| CATT | Not support. In training collaboration type 3 NW first training, the training dataset is assumed to be delivered to UE. Do not see the need to additionally transmit/deliver other dataset to UE for any purpose of training. |
| ETRI | We have similar view with HW. Using proxy models may incur subsequential problems, such as accuracy/generalization performance of proxy models. |
| Xiaomi | Agree with HW’s views. The feasibility of proxy model at UE side needs to further study. |

### Proposal 2-2-3

In CSI compression using two-sided model use case with training collaboration type 3, for sequential training, further study necessity, feasibility, and potential specification impact on:

- CSI reconstruction model training dataset and/or other information delivery from UE side to NW side

- CSI generation model training dataset and/or other information delivery from NW side to UE side

- Data sample format/type

- Quantization/de-quantization related information

- Other aspects are not precluded.

- Note: other information includes assisted information.

- Note: Delivery of dataset through 3GPP transparent way or over the air interface through 3GPP signaling can be separated discussed.

Please provide your view below:

| Company | View |
| --- | --- |
| Lenovo | We suggest the following change - CSI reconstruction model training dataset and/or other information delivery from between UE side to NW side - CSI generation model training dataset and/or other information delivery from between NW side to UE side |
| Panasonic | We are fine with the proposal. |
| NTT DOCOMO | Support the proposal. |
| Huawei, HiSilicon | Support |
| Google | OK |
| ZTE | Support |
| vivo | Support |
| Qualcomm | The reason to study air-interface standardization for dataset exchange between two training servers is unclear. If gNB sends the dataset to the UEs and the UEs send the dataset to the server, then the data has to traverse the air-interface once on the downlink and then again on the uplink (Option 2 in figure below). For effective training, the training dataset should be large. Since training data is typically voluminous, this could cause unnecessary burden on air-interface resources, and power consumption on the UE-side and NW-side. This could be avoided if the transfer happens directly from one server to the other without involving the UE or the air-interface (Option 1 in figure below). Considering this issue, unless a clear benefit is identified, we propose to deprioritize this aspect. |
| OPPO | Support |
| CATT | Support. |
| ETRI | Support the proposal. |
| LG Electronics | Support |
| Xiaomi | We are fine with the proposal. |

## Inference related spec impact

Following table summarize company’s proposals related to inferencing.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| Futurewei | Proposal 3: For CSI compression using two-sided model sub use case, study potential specification impact to support NW configuring at least the following for AI/ML based approach: - The maximum CSI payload size - A set of supported CSI payload sizes - Constraints/restrictions if applicable, e.g., rank restriction Proposal 4: For CSI compression using two-sided model sub use case, study potential specification impact of reporting/decomposing CSI in/into multiple parts as in the legacy CSI report, i.e., CSI part 1 and CSI part 2, at least as a starting point. Proposal 5: For CSI compression using two-sided model sub use case, potential CSI collision case can be handled by adding a priority value in each AI/ML-based CSI report (as in the legacy CSI report) as starting point. |
| vivo | - Study model ID based LCM procedure for CSI compression with two-sided models. - For ID based model management, study the following options for signaling design for model switching/activation/deactivation among multiple models: RRC-based, MAC CE-based, DCI-based. - Avoid using “logical model ID” in proposal 2-3-3 (v1) to prevent potential misalignments in companies. The terminology of “Model ID” or “indication” is ok for us. - Quantization-non-aware training for CSI compression would suffer from a significant performance loss compared with Quantization-aware training. - If quantization method at CSI generation part and dequantization method at CSI reconstruction part are not aligned, there will be an unacceptable performance loss for AI/ML models. - Study the potential specification impact of the alignment of quantization method at UE side and dequantization method at NW side based on different training collaboration types for CSI compression. |
| Ericsson | Quantization alignment between the encoder and the decoder is needed in two-sided CSI compression. Quantization alignment can be obtained via standardized quantization or via information exchanges, e.g., during the training phase. If quantization method is shared via bilateral agreements, there can be too many quantization methods that should be handled by the NW if the quantization is determined by the UE. - The number of quantization methods that should be handled by the NW should be limited to a single or a small set, using either standardized quantization or NW-determined quantization (NW-first training). - If the distribution of the quantization point of the scalar quantization is to be standardized, uniform quantization should be used as the starting point. - In scalar quantization, the different encoder outputs in the output layer should be quantized with the same granularity, unless having different granularities for different encoder outputs shows a clear gain. - RAN1 to study whether the number of quantization levels per encoder output should be fixed or configurable by the network in CSI report configuration The pre-processing by the UE may remove channel subspace (DFT vectors or eigenvectors), then information about the remaining subspace needs to be reported to the network side along with the encoder output bits which impacts the CSI report payload size. - Re-use the legacy CSI reporting principle with CSI Part 1 and Part 2 where Part 1 has a network configured fixed size and Part 2 size is dynamic, determined by information in Part 1. - The UCI for an AI-CSI report consists of bAUX bits carried in CSI part 1 for the auxiliary information common across all the transmission layers, bmodel bits carried in CSI part 2 used to complete the interpretation of the output CSI, and bAE bits carried in CSI part 2, representing the quantized latent space output of the encoder. - Support Option 1 with CQI being calculated based on a hypothetical CSI which is derived, in a standardized fashion, from target CSI. Further study the details of mechanisms for CQI adjustments. - If target CSI being an explicit channel tensor is supported (i.e. full Tx * Rx MIMO channel), an alternative solution is that the CSI report doesn’t contain CQI and RI but contains an interference plus noise (IpN) report. The importance of CBSR will increase due to more complicated interference situations in coming deployments and bands A benefit of a Target CSI definition based on eType-II is that CBSR can straightforwardly be applied by gNB to UE configuration of the target. Since a CSI-RS measurement may be used for multiple purposes (monitoring, inference, data collection), and processed by different hardware in the UE, RAN1 can consider discussing CPU and measurement processing units (MPU) as two decoupled entities used to define the UE processing load. |
| ZTE | Proposal 9: In CSI compression using two-sided model use case, further study the methods and potential specification impact on mapping priority and omission rule for AI/ML CSI report, - Dynamic quantization resolution to reduce payload - Divide the CSI into multiple groups with different priority and omit the CSI groups with low priority, e.g., according to layer, subband and port - CSI reporting is separated into multiple reports, e.g., to establish the association among the multiple reports Observation 6: For CQI calculation based on target CSI with realistic channel measurement, UE may over-estimate the channel condition and reconstructed PMI and CQI are not matched. Our simulation results show that the system performance loss is obvious if no advanced CQI adjustment algorithm is used. Observation 7: For CQI calculation based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by NW: - The output of CSI reconstruction part needs to be provided to UE from NW side, which will lead to additional latency and specification impacts. - The channel condition may already change a lot (e.g., interference) so that PMI and CQI mismatch is unavoidable. - The recovered CSI should be quantized (e.g., by eType II codebook), which will lead to additional quantization loss. Observation 8: For CQI calculation based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by NW, NW can construct a CQI adjustment table according to some channel characteristics based on some priori information at gNB side. Then, UE can calculate the similarity-related metrics between measured channel and the channel characteristics to do corresponding CQI adjustment. Observation 9: For CQI calculation based on legacy codebook, UE may not support traditional codebook and AI/ML codebook simultaneously, which will largely increase the UE complexity. Meanwhile, PMI and CQI mismatching is also unavoidable. If traditional codebook can already get accurate PMI, it is not necessary to implement AI/ML models. Observation 10: For CQI calculation based on CSI reconstruction output, where CSI reconstruction part at the UE is the same as the actual CSI reconstruction part used at the NW. UE may also be not expected to have CSI reconstruction model as it increases UE computation/storage/power consumption burden to a large extent. In addition, the CSI reconstruction model is generally a proprietary design by network side. Observation 11: For CQI calculation based on the output of proxy CSI reconstruction model at UE, this method can be applicable for CSI compression using two-sided model sub use case and shows that the average system UPT can be achieved almost the same as the case of CQI calculation based on the output of actual CSI reconstruction model (i.e., performance upper-bound for all options). Observation 12: For CQI calculation using two stage approach, it is already supported (i.e., when the report quantity is cri-RI-CQI) and less specification impact is foreseen. Besides, the two-step procedure increases the time span of the CQI determination process, which may face the channel variation/aging so that the current CQI cannot match the previous CSI. Proposal 10: The performance of different CQI determination options should be evaluated in agenda item 9.2.2.1. The pros and cons of all the options should be concluded in 9.2.2.2. Proposal 11: Further categorize the Option 1b as following: - Option 1b-1: CQI is calculated based on target CSI with realistic channel measurement and adjusted by previous CSI reconstruction output provided by gNB - Option 1b-2: CQI is calculated based on target CSI with realistic channel measurement and adjusted by CQI adjustment table provided by gNB. Proposal 12: According to initial evaluations on performance and specification impacts, the following down-selections are proposed: - Further study the specification impacts (including the feasibility and necessity) on Option 1a, Option 1b-2 and Option 2a-2. - No further discussion on specification impacts for Option1b-1, Option 1c, Option 2a-1 and Option 2b. Proposal 13: In CSI compression using two-sided model use case, LI determination should be studied along with CQI determination. Proposal 14: In CSI compression using two-sided model use case, if RI is configured to be reported, legacy RI determination can be reused as a starting point. Proposal 15: Further study potential specification impact on more channel information reported for MU-MIMO scheduling, e.g., full rank report based on the AI/ML model. Proposal 16: In CSI compression using two-sided model use case, further study at least the following options: - Option 1: - For network configuration to determine CSI payload size, gNB can configure the UE with an identifier - For UE determination/reporting of the actual CSI payload size, UE reports the selected RI. - Option 2: - For network configuration to determine CSI payload size, gNB can configure a list of identifiers - For UE determination/reporting of the actual CSI payload size, UE reports the selected RI and the identifier Proposal 17: In CSI compression using two-sided model use case, further study how to map RI, CQI, identifier(s) of actual CSI payload size per layer under the current CSI part 1 and part 2 framework. |
| Spreadtrum Communication | Proposal 8: Aperiodic CSI reporting should be considered firstly. Proposal 9: The configuration of CSI-ResourceConfig and/or CSI-ReportConfig should be enhanced to make UE aware of AI/ML related operation. Proposal 10: CQI/RI still should be included in the CSI report. Proposal 11: Regarding CQI calculation, option 1a and/or option 1b can be considered. Proposal 12: The priority for AI/ML based CSI feedback needs to be considered. Proposal 13: Introducing k>1 for CSI reports carrying CSI compression information enabled by AI/ML operation in the priority rule for CSI reports. Proposal 14: How to define/reflect the complexity of the AI/ML operation in the specification should be considered. Observation 2: Codebook subset restriction can be not considered in CSI compression and recovery using two-sided model use case. |
| Huawei | Proposal 13: For CQI determination of CSI compression, consider Option 1 (CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation) as a starting point. Proposal 14: For Network configuration to determine CSI payload size, gNB can configure a set of candidate CSI payload sizes. - Each candidate CSI payload size is represented by the configurations including at least one of the factors including: UE part model(s), rank value, quantization method/granularity, and/or size of latent space from scalable dimensions for each layer, etc. - FFS whether the identifier(s) for the above factors are separately configured (i.e., each identifier to represent one factor) or jointly configured (i.e., one identifier to represent more than one factor). Proposal 15: For UE determination/reporting of the actual CSI payload size, UE reports the identifier(s) to represent the selected CSI payload size (among the set of candidate CSI payload sizes). - Each identifier is associated with one or more factors including: UE part model(s), rank value, quantization method/granularity, and/or size of latent space from scalable dimensions for each layer, etc. - FFS how to map the CSI report on the two parts CSI. Proposal 16: In CSI compression using two-sided model, further study potential specification impact on quantization alignment using 3GPP aware mechanism. - For vector quantization, - Configuration/reporting/updating of the quantization dictionary. - For scalar quantization, - The configuration of the quantization granularity. Proposal 17: For the CSI priority rules of CSI compression, on top of the legacy CSI reporting principles, the AI/ML specific enhancements include: - The priority rules for different LCM procedures of training data collection, inference, and monitoring data collection. - The priority rules within the latent space of per CSI report. Proposal 18: For the CSI processing Unit, on top of the legacy CSI reporting principles, the AI/ML specific enhancements include: - The required CPU value of CSI calculation is reported for per AI/ML model basis. |
| Nokia | Proposal 5: To make the quantization alignment easier, it is necessary to limit the quantization options. RAN1 may decide on some possibilities for encoder output size, segment size, and quantization bits per segment. Proposal 6: We propose to limit the possibilities for encoder output size to 16X , X= 1, 2, 4, 8. These options can provide a wide range of feedback payload sizes. Proposal 7: For uniform scalar quantization (USQ), RAN1 may consider a mid-rise quantizer with quantization range [-1, 1] and 2^B quantization levels. UE or gNB may allocate 1 to 4 bits for quantization of each element ({1, 2, 3, 4}). Proposal 8: For vector quantization (VQ), RAN1 may limit the options for segment size (S) and possibilities for allocated quantization bits per segment (B). Proposal 9: We propose to consider consecutive grouping of every S elements of encoder outputs as the segmentation method. |
|  | Proposal 11: In CSI compression using a two-sided model, consider the following for the data collection, - Data collection shall be mainly focused on performance monitoring or model fine-tuning, and considerations on the data collection for model training shall not be the main focus. - UE-sided data collection, - Existing CSI-RS configuration shall be used as the starting point for any form of data collection - NW-sided data collection, - Enhancement of CSI reporting to enable higher accuracy reporting - FFS: Assistance information reporting Proposal 12: RAN1 shall study the possible use of CSI part 1 and CSI part 2 like approach for the compressed CSI reporting. Proposal 13: Study UCI format design scheme which provides flexibility to cope with various subband/port configuration, rank, CR, quantization scheme. It is desired that the UCI format can be easily augmented to convey channel eigenvalues as an optional feature, if configured. Proposal 14: Investigate impact of differential quantization resolution per layer on performances, to make use of its result for efficient bit allocation of UCI format for AI/ML CSI feedback. Proposal 15: Take a hybrid CSI reporting mechanism for the port selection codebook as the starting point of AIML-enabled CSI feedback operation framework, to explore possibilities to reduce UE complexity and to reduce CSI feedback signalling overhead. RAN1 should identify the required modifications and study their specification impact. - Note: hybrid CSI reporting mechanism refer to a two-step CSI reporting operation, i.e., long-term, low-resolution channel information reporting (step 1), and a short-term, high-resolution channel information reporting (step 2). Proposal 16: RAN1 shall investigate the appropriate dataset sharing without disclosing the mapping from (quantized) latent representation to the codeword. Proposal 17: RAN1 may study the opportunity of allowing adaptive compression ratio CR at the UE, where the UE selects the best CR in the range that satisfies the gNB requirement on SGCS while minimizing the payload feedback size. Observation 1: A study on the effects of interference and noise on the performance of CSI-compression task and how new CSI-RS patterns can help in this process may be needed. The study should assess whether the auto-encoder architecture can deal with interference and noise in the estimated CSI. |
| CATT | Proposal 16: For AI/ML based CSI feedback, the overheads of CSI feedback for rank 3 and rank 4 are expected to be comparable with that of rank 2. Proposal 17: In CSI compression using two-sided model use case, if CQI in CSI report is configured, for CQI determination in CSI report, one of the sub options of Option 1 is adopted: - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook. Proposal 18: For CQI reporting in CSI compression using two-sided model use case, the same quantization schemes as that in Rel-17 for codebook based CSI feedback is considered. Proposal 19: In CSI compression using two-sided model use case, quantization alignment using 3GPP aware mechanism is needed at least for training collaboration Type 3. |
| Fujitsu | Proposal-1: For the sub use case of CSI compression using two-sided AI/ML models, study the mechanism and potential specifications that UE and NW align their supported AI/ML models in the multi-vendor collaborations. Proposal-2: For the sub use case of CSI compression using two-sided AI/ML models, study using local model IDs as the identifiers in CSI configuration and CSI report after model alignment between UE and NW, which reduces the overhead compared to global model IDs. Proposal-3: For the sub use case of CSI compression using two-sided AI/ML models, global model ID is sufficient for model alignment, and there is no need to introduce pairing IDs. Proposal-4: For the sub use case of CSI compression using two-sided AI/ML models, study the following options of the structure of CSI. - The CSI is divided into multiple parts, where at least one part has a fixed bitwidth, which indicates the size(s) of the other(s). - The CSI is not divided into more than one part. The first option is preferred. Proposal-5: For the sub use case of CSI compression using two-sided AI/ML models, study the rules for setting the CSI priority. As an example, the spatial layer indices can be an option. Proposal-6: In the NW-side AI/ML model performance monitoring, prioritize the study of using the codebook-based quantization of the ground-truth CSI, e.g., Rel-16 e-type II-like codebook with new parameter values. |
| Interdigital | Proposal 1: Perform a trade-off analysis of the performance, complexity and standardization impacts of both precoding matrix and explicit channel matrix before prioritizing. Observation 1: Different pre-processing types are beneficial under different deployment scenarios and channel characteristics. Observation2: Different pre-processing types lead to different AI/ML encoder outputs which need to be known at the decoder. Proposal 2: Study support of multiple pre-processing options. Proposal 3: Study UE selection and reporting of pre-processor type. Proposal 4: Study UE determination and reporting of the RI and CQI based on the input to the AI/ML model at the UE. Observation 3: A UE without an up-to-date AI/ML decoder cannot independently detect CQI mismatch. Proposal 5: Study means to detect and identify when there is mismatch between a UE’s AI/ML encoder input and the NW’s AI/ML decoder output. Proposal 6: Study methods to enable CQI adjustment based on detected CQI mismatch. Proposal 7: Study specification impacts of CSI compression using AI/ML including: CSI compression type, support of multiple AI/ML models, new CSI reporting mechanisms and fallback to legacy CSI reporting. Proposal 16: Study quantizer/dequantizer updating separate from AI/ML model switching. Proposal 17: Study different alignment levels between quantizer and dequantizer. Proposal 18: For models with quantization non-aware training, study non-uniform quantization as means to determine actual CSI payload size within the NW configured constraints. |
| Intel | Proposal 3: - It is expected that AI/ML model is trained assuming a particular pre/post processing - If an AI/ML model is configured at the UE for inference, information on pre-processing for that model should be provided to the UE (e.g. specified, configured, downloaded etc.) - Pre/post-processing may include at least linear transforms (DFT across different dimensions), down selection of matrix elements and normalization Proposal 4: - The dimensions of the input are defined by parameters similar to parameters L/M for Enhanced Type II PMI codebook (considering that input corresponds to the neural network input after pre-processing) - In some cases, information from pre-processing step shall be reported by the UE together with CSI bits generated by the neural network (e.g., selected basis vectors, basis rotation factor, etc.) Proposal 5: - Consider existing principles for RI and CQI reporting for spatial-frequency domain CSI compression using two-sided AI model sub-use case Proposal 6: - The following alternatives for CQI adjustment determination can be considered for Option 1b CQI determination - CQI adjustment is configured via higher layers - CQI adjustment is determined by the UE based on reference CQI (e.g., measured from precoder CSI-RS) - CQI is calculated using precoding matrix corresponding to the target CSI with added AWGN |
| Google | Proposal 1: The input of CSI compression based on the eigenvectors of the raw channel with a wideband precoder selected as SD basis, e.g. HW1. Proposal 2: The output of CSI compression should be the compressed eigenvectors for the raw channel with a wideband precoder selected as SD basis, e.g. HW1. Proposal 3: The CSI report for CSI compression should comprise the beam index(es) for W1 selection and compressed eigenvectors for the raw channel with a wideband precoder selected as SD basis, e.g. HW1. Proposal 4: If the input of the ML is the frequency domain channel, the UE reports L1-SINR only instead of reporting RI/CQI. Proposal 5: If the input of the ML is the channel eigenvector or W2, the UE reports a list of CRIs and CQI based on a set of port selection CSI-RS resources. - The gNB applies the decompressed precoders to each CSI-RS resource Proposal 6: The priority for non-ML based CSI report should be higher than the priority of ML based CSI report. Proposal 7: Support the CPU occupancy rule for ML based CSI based on two types processing unit - Type1 CPU: a measurement processing unit (MPU) used for channel estimation and pre-processing - Type2 CPU: an inference processing unit (IPU) used for inference for ML based CSI Proposal 8: Study the AI/ML model adaptation for CSI compression in case of CSI omission, where different AI/ML models may be with different compression ratio. |
| Panasonic | Observation 16: For each option of training collaboration, handling of rank of AI/ML model should studied. Observation 17: Both quantization non-aware training and quantization-aware training should be studied. Observation 18: For CQI determination in CSI report, further study following options. - Option 1: CQI is NOT calculated based on the output CSI reconstruction part from the realistic channel estimation. - Option 1a: CQI is calculated based on target CSI with realistic channel measurement. - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment. - Option 2: CQI is calculated based on the output on CSI reconstruction part from the realistic channel estimation. - Option 2a: CQI is calculated based on CSI reconstruction output if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment. - The CSI reconstruction part for CQI calculation at the UE is a proxy model, which is different from the actual CSI reconstruction part at the network. Observation 19: Legacy CSI reporting mechanism, i.e., mapping of compressed CSI into fixed/configurable/known-payload part (similar to CSI part 1) and variable/predictable size (similar to CSI part 2) may also be required for CSI compression using two-sided models. Observation 20: Further study Direction 1 and Direction 3 with proxy model framework. - Direction 1: Network-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE side. - Direction 3: UE-side monitoring based on the output of the CSI reconstruction model at the UE side. - The CSI reconstruction part for performance monitoring at the UE is a proxy model, which is different from the actual CSI reconstruction part at the network. |
| Xiaomi | Proposal 5: For CSI compression using two-sided model, the following potential specification impact for CSI configuration and report should be studied. - The maximum CSI payload size is configured by network. - The actual CSI payload size is determined by the CSI generation part model configured or selected by UE if one or more CSI generation part models are configured. Proposal 6: The legacy priority rule can be reused to define the priority the AI/ML based CSI reporting, and a priority value PriiCSIy,k,c,sPriiCSIy,k,c,s PriiCSIy,k,c,swith new parameter value or introducing new parameter PriiCSIy',y,k,c,s is used to indicate the priority of CSI reporting. Proposal 7: CSI reporting with two parts, i.e., Part 1 and Part 2 or only one part for AI/ML based CSI feedback with two-sided model can be supported. Proposal 8: The compressed quantization information is divided into N>1 groups for CSI omission, where the values N and how to divide into N groups needs to further study. Observation 4: The CBSR can be implement when input-CSI-NW the precoding matrix represented using angular-delay domain projection. |
| Sony | Proposal 4: RAN1 should study whether the compressed channel information is treated as a new PMI type or new CSI feedback information. Proposal 5: RAN1 should study specification impact of new PMI type for the CSI compression using two-sided model use case. Proposal 6: RAN1 should support periodic/aperiodic feedback of the raw data of CSI which is not compressed using AI/ML model from UE to NW for NW-side monitoring in CSI compression using the two-sided model use case. |
| Fraunhofer | Proposal 2: For CQI calculation in Type 3 network-first training, side-information and a set of parameters of the decoder e.g., the accuracy or loss of the reconstruction should be revealed to the UE. |
| NEC | Proposal 5: Study the priority rules for AI/ML based CSI reporting, at least the following cases should be considered: - Non-AI/ML based CSI report vs. AI/ML based CSI report - AI/ML based CSI report vs. AI/ML based CSI report Proposal 6: Study feasibility and potential specification impact of CSI omission based on truncation approach. |
| CMCC | Proposal 5: For CSI compression using two-sided model use case, the enhancement on CSI processing time and the definitions of Z and Z’ could be studied. |
| NVIDIA | Proposal 7: For AI/ML based CSI feedback, study potential specification impact related to assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection. Proposal 8: For AI/ML based CSI feedback, study potential specification impact related to assistance signalling and procedure for model performance monitoring and model update/tuning. Proposal 9: For AI/ML based CSI feedback, study potential specification impact related to report/feedback of model input for inference (e.g., quantization and feedback message size), type of model input, and model input acquisition and pre-processing. Proposal 10: For AI/ML based CSI feedback, study potential specification impact related to report/feedback of model inference output (e.g., quantization and feedback message size) and post-processing |
| Lenovo | - Study the potential specification impact corresponding to quantization method of CSI feedback data using both scalar and vector quantizers - Study different alternatives of reporting the AI-based CSI framework configuration parameters based on the design details of the AI-based CSI compression framework - Study potential CSI report characteristics for AI-based CSI compression under different network-UE training collaboration levels - Prioritize Option 1a and Option 2a for CSI compression format in spatial-frequency domain - For the mapping order of CSI fields corresponding to AI-based spatial-frequency CSI compression, the CSI feedback is composed into two parts: - Part 1: comprising RI, CQI and size of CSI Part 2, where the size of CSI Report Part 1 is fixed - Part 2: comprising the AI encoder output, where the size of Part 2 is indicated in CSI Report Part 1 - Strive to design the AI-based spatial-frequency CSI compression codebook so that (i) the overall CSI feedback is fixed for different RI values and/or different channel conditions, or (ii) the CSI fields are mapped in an order that enables partial UCI omission of the CSI feedback without jeopardizing the un-omitted CSI feedback - Assuming two-sided AI models for CSI compression under training collaboration Type 3, further enhancements are needed to ensure precise CQI characterization due to mismatch between the nominal decoder at the UE side and the actual decoder at the network side - Consider Option 1b for CQI reporting, where the UE appends side information to the CQI calculated based on the nominal decoder, such that the side information helps quantify the encoder/decoder mismatch to enable more accurate CQI adjustment to the actual CQI value - CBSR is supported for AI-based CSI reporting - Reuse legacy DFT-based CBSR, where a DFT-based restricted vector r implies that no precoding vector v within a pre-determined angular distance from the vector r can be selected |
| LGE | Proposal #1: For CSI reporting for AI/ML based CSI compression, two-part encoding can be considered where # of actual bits for AI/ML generated CSI can be included in Part 1 CSI. FFS on which type of information (explicit or implicit) for representing actual # of feedback bits by taking into account of applicability, versatility, and feedback overhead. Proposal #2: For CQI determination of AI/ML based CSI compression, prioritize option 1 (CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation). Proposal #3: Consider CSI compression ratio information as CSI reporting contents. Proposal #4: Consider enhancement of CSI restriction at least followings - Configuration associated with form of ids such as configuration id, site id, zone id, etc. - Dynamic configuration switching Proposal #5: Consider defining new CSI processing unit to handle the AI/ML based CSI feedback. Proposal #6: Consider at least following aspects for fallback operation - Condition of Fallback mode - NW initiated Fallback mode |
| Qualcomm | Observation 12: For codebook-based CSI feedback, gNB does not use a single identifier to configure the maximum CSI payload size. Instead, the CSI payload is determined by the PMI codebook indication, subband, antenna port, rank, and parameter combination configurations. The PMI codebook type can be considered as an identifier for the UE and gNB to align the PMI reporting format. Observation 13: A paired model(s) is analogous to a PMI codebook which is designed for a specific purpose or scenario and also support a wide range of functionalities and features including subbands, antenna port layout, rank and payload size. Observation 14: The max payload size can be jointly determined by the paired model identifier, and other configurations such as subband configuration, antenna port layout, rank and/or payload configuration. Observation 15: CSI report configurations are multi-factors, it is infeasible to use a single identifier to represent the CSI payload. It would yield too many identifiers and would cause configuration ambiguity. Observation 16: Configuring a list of identifiers for UE to report a selected one requires the UE to either implement CSI reconstruction parts or proxy model for selection. The complexity is much higher than counting non-zeros in eType II CSI calculation. Observation 17: Only UCI and final format of the reported CSI (e.g., the precoding matrix) are specified in legacy CSI feedback framework. The PMI search algorithm and its input are proprietary. Observation 18: In CSI feedback via two-sided model, PMI searching algorithm is replaced by UE-side model while PMI codebook is replaced by NW-side model. The general principle for specification impact should be preserved. The need for specifying UE-side input and pre-processing is not clear. Observation 19: Post-processing of NW-side model output into the final CSI format can be absorbed into the specification of the final CSI format. Observation 20: Channel matrix feedback (i.e., H-in-H-out) creates additional and unnecessary complexity for multi-vendor operation. Observation 21: Eigen-value or soft-rank feedback, along with precoder, achieves similar merit as the channel matrix feedback in terms of flexibility for network scheduling without causing significant increase in implementation complexity. Observation 22: Quantization non-aware training (case-1) leads to noticeable performance degradation compared with quantization aware training (case-2). Observation 23: Trainable quantization offers more flexibility and better performance compared to fixed quantization, e.g., trainable vector quantization can improve the performance. Proposal 10: For AI/ML-based CSI feedback using two-sided model, the procedure used to process the downlink measurements and derive the input to the UE-side model during inference should be left to UE implementation. Proposal 11: While generating the training dataset, the target CSI corresponding to a downlink measurement should be derived by the UE side to reflect the UE processing during inference (e.g., channel estimation, eigen-vector derivation, etc.). Proposal 12: Study assistance signalling for UE’s data collection in the form of a zone ID, scenario ID, and configuration ID. Proposal 13: Reuse current CSI report configuration framework with new signaling of pairing ID and necessary information related to the CSI feedback, e.g., subband configuration, rank restriction, antenna port configuration, payload information. Note: A pairing ID is a logical ID that indicates compatibility between the UE-side and NW-side model of a two-sided model. For example, all encoders developed from a two-sided multi-vendor training session may be associated with a single pairing ID. As another example, in NW-side first training, UE-side encoders trained based on the same NW-side model may be associated with a single pairing ID. Proposal 14: Study payload scalability with number of subbands, number of ports and rank. Proposal 15: For CSI configuration, it is sufficient to configure one identifier representing paired model. UE-side actual payload determination should be based on only reported rank for two-sided ML-CSI feedback. Proposal 16: The input to the UE-side model should be left to UE implementation, the output at the NW-side model can be specified. Proposal 17: Preprocessing at UE-side is upto UE-implementation and should not be specified. Proposal 18: For AI-based CSI feedback, the size of the UCI payload and the final CSI format can be specified. Proposal 19: Study reporting the precoding matrix together with eigen-values or soft-rank for two-sided AI/ML CSI feedback. Proposal 20: Deprioritize channel matrix feedback for the R18 study item. Proposal 21: Quantization method should be considered a part of the UE-side model and dequantization method should be considered a part of the NW-side model. The quantization method should be aligned for good performance, but there is no need for separate specification support to align the quantization method. |
| Oppo | Proposal 1: In training collaboration type 3, - For NW first training, NW needs to be able to provide UE with training data sets that meet different requirements, e.g. on model performance, transmission cost, data characteristics and CSI input types - For UE first training, UE needs to be able to provide NW with training data sets that meet different requirements, e.g. on model performance, transmission cost and data characteristics Proposal 2: CQI should be calculated based on target CSI with realistic channel measurement Proposal 3: Regarding the CSI input, - when UE obtains the CSI generation part from NW in a 3GPP non-transparent way, the network needs to explicitly or implicitly indicate the input interface format of the CSI generation part, e.g. data type, dimension size, normalization/quantification schemes. - when UE obtains the CSI generation part in a 3GPP transparent way, no need to indicate the input interface through 3GPP protocols. Proposal 4: The performance gain, training complexity, inference complexity, signaling cost for indication and standardization impact of different quantization/dequantization methods need to be evaluated. - If the quantization/dequantization scheme is not a key contributor to CSI compression/recovery performance, the quantization/dequantization scheme(s) that is relatively simple, easy to indicate and have less standardization impact(e.g. SQ in case 2-1) should be selected first. Proposal 5: In CSI compression using two-sided model use case, further study potential specification impact on the quantization/dequantization method for the compressed CSI, including - At least for training collaboration type3, quantization/dequantization methods should be specified and aligned to ensure the encoder and encoder to be well trained and could work together - For NW first training, network should indicate the quantization [or the dequantization] method for the compressed CSI to UE. - For UE first training, UE should indicate the dequantization [or the quantization] method for the compressed CSI to NW. - Study potential signaling and procedure to indicate the quantization/dequantization method |
| Samsung | Proposal 2-2: For AI/ML based CSI compression sub-use case, study the specification impact of UCI format for quantized output of CSI generation part. Proposal 2-3: For AI/ML based CSI compression sub-use case, study flexible configuration of quantization method and quantization resolution that enables the network to 1) Adapt to different AI/ML models and channel environments/scenarios 2) Control the feedback payload size. Proposal 2-4: For AI/ML based CSI compression sub-use case, study the specification impact of adaptable CSI feedback payload size that enables the UE to adapt to available size of uplink resources. FFS: whether priority and CSI dropping rules have to be introduced. Proposal 2-5: For AI/ML based CSI compression sub-use case, study methods to configure and apply codebook subset restriction (CBSR) including: - Whether the legacy SD basis vectors based restriction applies - How to apply CBSR for when Output-CSI-UE is in 1) spatial-frequency domain 2) angle-delay domain - Whether soft amplitude restriction is possible Proposal 2-10: In CSI compression using two-sided model, adopt Option 1a: CQI is calculated based on target CSI with realistic channel measurement. Observation#1 In case of MU-MIMO, the network may not directly apply the precoder based on reported PMI, e.g., for interference nulling, etc. Thus, even in legacy systems, some level of mismatch exists between the PMI (precoder network reconstructs from PMI) and the precoder network applies for data transmission. Observation#2 In CSI compression using two-sided model, for CQI determination in CSI report, for Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Is computationally friendly as UE does not require to perform CSI reconstruction or additional measurements for CQI calculation - The mismatch between CQI determined conditioned on target CSI (precoder) and CQI determined conditioned on the reconstructed CSI (precoder) is insignificant when CSI reconstruction loss is insignificant Observation#3 In CSI compression using two-sided model, for CQI determination in CSI report, for Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment: - The adjustment can be handled in a spec. transparent manner. Observation#4 In CSI compression using two-sided model, for CQI determination in CSI report, for Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment: - The availability of Network’s reconstruction output at the UE is not guaranteed, as network may be willing to share it, thus, may not be feasible. - Network may use heavier model, which may not fit in to UE’s computational capability, thus, may not be feasible. Observation#5 In CSI compression using two-sided model, for CQI determination in CSI report, for Option 2b: CQI is calculated using two stage approach in which UE derives CQI using precoded CSI-RS transmitted with a reconstructed precoder: - It incurs additional CSI-RS overhead - The delay between CSI (precoder) generation and CQI determination introduces mismatch. |
| NTT DOCOMO | Observation 7: There is another mechanism to help MCS selection, such as HARQ-ACK mechanism, in addition to CQI reporting. Observation 8: For CSI compression, the constraint on channel for CSI reporting can be the same as subband type II codebook. Observation 9: For CSI compression, CSI reporting can consist of two parts; CSI part 1 including RI/encoder model ID/CQI, and CSI part 2 including compressed bits. Observation 10: For CSI compression, the legacy priority rules for CSI collision handling and CSI omission can be reused except for the priority reporting level within the compressed bits. Proposal 3: Clarify what model is identified by model ID in the two-sided model. Until the clarification is made, it is better to introduce paired model ID, encoder model ID, and decoder model ID for the discussion purpose. Proposal 4: Study what aspects should be considered as conditions indicated from UE to NW for CSI compression. At least the following information can be considered as potential conditions. - RS configuration for nominal input. E.g., numerologies, carrier frequency, bandwidth, frequency density, and number of antenna port - Applicable NW deployment. E.g., gNB antenna/beam configuration, gNB antenna radiation pattern, paired decoder model information - Reportable information. E.g., payload size of compressed CSI feedback, maximum rank of reported CSI, frequency granularity (sub-band size) - Applicable channel property. E.g., received signal strength (RSRP/SINR), interference signal strength, LOS/NLOS condition, doppler information (UE speed) Proposal 5: Study the mechanism to align the paired models for two-sided models. |
| MediaTek | - Study alignment requirement and influence of different training awareness techniques for enabling backpropagation between quantizers and dequantizers. - Study signalling and ID assignment procedure for AI/ML models generalized over multiple input, output, and latent dimensions. - Prioritize option 1, “CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation” as the starting point. - Prioritize option 1a and option 1b for CQI calculation. |
| ETRI | - Proposal 1: Consider further studies on performance improvement of AI models with training datasets from realistic channel estimation. - Proposal 2: In CSI compression using two-sided AI model, further study the following potential specification impacts on UCI configuration. - NW configures UE to generate the UCI payload in a certain size. - UE generates the UCI payload within the maximum UCI payload size. UE delivers to or shares details of the UCI payload (including quantization-related information) - Proposal 3: Consider further studies on potential specification impacts of model selection using the performance monitoring result. - Proposal 5: For AI/ML-based CSI compression sub-use case, study the operation of codebook subset restriction (CSR) including: - Whether the AI/ML-based CSI compression can support efficient codebook restrictions (i.e., achieving similar SGCS with smaller CSI payload sizes when CSR is applied) - Whether the AI/ML-based CSI compression can support all codebook restrictions. When AI/ML model cannot support all codebook restrictions, how to represent the supported codebook restrictions |
| IIT Kanpur | Proposal 2: Study the necessity and potential specification impact on quantization alignment, with scalar quantization scheme as starting point. Proposal 3: For scalar quantization, study variation in quantization granularity for different possible CSI feedback payload sizes. Proposal 4: For scalar quantization, study variation in quantization granularity per layer if layer specific model is used and maximum payload size is configured by gNB. Proposal 5: For scalar quantization, study variation in quantization granularity per rank if rank specific model is used and maximum payload size is configured by gNB. Observation 2: For different training collaboration types, different format/size of CSI feedback and different configuration may be required. Proposal 6: Study the effect of different collaboration types on CSI reporting and configuration in terms of at least. - UCI Payload size and format - CSI configuration size and format - Extra parameters for alignment of Pre/ Post Processing, quantization/dequantization etc. - CQI measurement and reporting type Note: Other options are not precluded. |
| Apple | Proposal 3: In CSI compression using two-sided model use case, further study the potential specification impact for CSI configuration and report: - For network configuration to determine CSI payload size, gNB can configure the UE with one or more identifiers - For UE determination/reporting of the actual CSI payload size, UE reports the selected RI and/or selected identifier. - An identifier may be associated with the information of factors that represent a specific CSI payload size and/or model, e.g., UE part model compatible with the NW part model used by the gNB, rank value, quantization method/granularity, size of latent space from scalable dimensions for each layer, etc. Proposal 4: Further study other payload related aspects including how payload scales with number of subbands, number of ports, different payload configs, etc Proposal 5: When domain transformation pre-processing is used, legacy CSI report principle can be applied to input CSI directly. Proposal 6: When domain transformation pre-processing is not used, - Prioritization rule is indirectly support by selecting different AI model with different UCI bit size. - CBSD can be supported by projecting the input CSI in the subspace orthogonal to restricted sub-space before AI model. Proposal 7: Further discuss whether legacy CPU principle can be reused for AI based CSI processing, as legacy CPU does not differentiate actual computation complexity of different CSI report quantity. |

### Summary:

CSI configuration and reporting have been discussed in RAN1 112 and 112bis-e. There are many aspects related to the signaling including:

- How the NW part model and UE part model are paired together and how to indicate which UE part model or NW part model to use.

- Whether e-type 2 like principle is supported where rank 2, 3 and 4 has similar payload size. If support, how to support it.

- Whether e-type 2 like prioritization and omission rule can be supported in AI based CSI feedback.

- How to support the wide choices of models including layer common rank common, layer common rank specific, layer specific rank common, layer specific rank specific, rank specific and rank common models.

- The wide range methods have been evaluated to achieve different CSI payload size:

  - By different AI models

  - By same AI model with additional adaptation layer

  - By different quantization size, codebook

  - By puncturing/padding bits

To support all the design choices, the CSI configuration and report needs to be flexible enough. RRC configured identifier, which indicate a list of configurations was discussed in RAN1 112bis-e. The latest proposal is proposed here.

### Proposal 2-3-1:

In CSI compression using two-sided model use case, further study the potential specification impact for CSI configuration and report:

- For network configuration to determine CSI payload size, gNB can configure the UE with one or more identifiers

- For UE determination/reporting of the actual CSI payload size, UE reports the selected RI and/or selected identifier.

- An identifier may be associated with the information of factors that represent a specific CSI payload size and/or model, e.g., UE part model compatible with the NW part model used by the gNB, rank value, quantization method/granularity, size of latent space from scalable dimensions for each layer, etc.

- FFS: other payload related aspects including how payload scales with number of subbands, number of ports etc

Please provide your view below:

| Company | View |
| --- | --- |
| Lenovo | For network configuration sub-bullet, rather than configuring the UE with multiple identifiers, it suffices that the gNB configures the UE with one of the multiple identifiers corresponds to the largest CSI payload size. Our preference is to merge this proposal with Proposal 2-3-5 below, as follows: - Two parts of CSI report are configured: Part 1 and Part 2 - Network configures the exact payload size of Part 1 - Network further configures maximum payload size of Part 2 - UE determines the exact payload size of Part 2 that satisfies the network-configured constraint |
| Futurewei | In general, we are ok with the proposal but we suggest adding an option to allow NW to configure maximum CSI payload size as well. |
| Panasonic | We are fine with the proposal. |
| Huawei, HiSilicon | We think “selected RI and/or” should be removed from the second bullet since, according to the third bullet, it is already one of the factors that are associated with the identifier. Therefore, reporting RI may be redundant. Also, a single identifier may not uniquely represent a CSI payload size as it may be associated only with one or a subset of factors that represent the CSI payload size. We suggest the following modification: ### Proposal 2-3-1 (modified): In CSI compression using two-sided model use case, further study the potential specification impact for CSI configuration and report: - For network configuration to determine CSI payload size, gNB can configure the UE with one or more identifiers - For UE determination/reporting of the actual CSI payload size, UE reports the selected RI and/or selected identifier(s). - An identifier may be associated with the information of one or more factors that represent a specific CSI payload and/or model, e.g., UE part model compatible with the NW part model used by the gNB, rank value, quantization method/granularity, size of latent space from scalable dimensions for each layer, etc. - FFS: other payload related aspects including how payload scales with number of subbands, number of ports etc |
| Google | Support. Also OK with HW’s revision. |
| ZTE | We are fine with proposal. |
| vivo | Support |
| Qualcomm | We prefer to explicitly list the options for more clarity on how they work. For example, RI is required in the single identifier case. Only for the multiple identifier case, RI may be implied by the selected identifier. We propose to update option 1 to make it more complete in terms of possible configurations that would be used to determine the CSI payload size, as in the version below. For option 2 where network configures multiple identifiers, does it mean it is the only signaling that is used for max payload size determination, or is the legacy signaling of subband/port-layout configuration also used to determine CSI payload size? We prefer Option 1. In option 2, the need for the UE to dynamically select from multiple options needs justification. Proposed wording for option 1: In CSI compression using two-sided model use case, further study the potential specification impact for CSI configuration and report: Option 1: - For network configuration to determine CSI payload size, gNB can configure the UE with: - an identifier that enables the UE to choose a CSI generation model compatible with the CSI reconstruction model used by the gNB - Subband configuration, port layout configuration, rank restriction - other payload configuration if needed (e.g., size of the latent message in case of model scalable across multiple latent sizes) - For UE determination/reporting of the actual CSI payload size, UE reports the selected RI. Option 2: - <…. option with multiple idenifiers …. > |
| NEC | We prefer to HW’s modification. |
| CATT | Support in principle. It is close to current CSI report mechanism. For Futurewei’s proposal, maybe the identifier is of a payload configuration. |
| ETRI | We support. |
| LG Electronics | Support |
| Xiaomi | We have similar view with Futurewei. The maximum payload size can be configured by NW, which has been supported for legacy CSI reporting. |

### Proposal 2-3-2:

In CSI compression using two-sided model use case, further study at least the following methods to support the priority rule regarding CSI collision handling and CSI omission:

- Layer based priority rule for layer specific and layer common model.

- Model adaptation including different quantization method/granularity, different size of latent space, or CSI puncturing.

Please provide your view below:

| Company | View |
| --- | --- |
| Lenovo | Support |
| Huawei, HiSilicon | ### We think to meet the CPU budget at the time of collision, both inter layer CSI report prioritization and per layer CSI payload reduction are viable solution and need to be considered. We are not sure about the intention of the second bullet and we would like to ask the proponents for clarification. Based on above, we suggest the following: ### Proposal 2-3-2: In CSI compression using two-sided model use case, further study at least the following methods to support the priority rule regarding CSI collision handling and CSI omission: - Layer based priority rule or CSI payload reduction for a layer for layer specific and layer common model. - [Model adaptation including different quantization method/granularity, different size of latent space, or CSI puncturing.] |
| Google | ### Support |
| ZTE | ### We are fine with the first bullet, but we are not clear about the second bullet, which needs further clarification. |
| vivo | ### Support |
| Qualcomm | ### CSI collision and CSI omission are two different things. Priority rule for CSI collision is to order the CSI reports, the low priority CSI report may be dropped or omitted. Priority rule for CSI omission is about ordering the UCI contents of one CSI report, if omission occurs, UE omits low priority UCI content. Based on this, the two bullets seem nothing about priority rule of CSI collision, but more related to CSI omission context. ### We assume the 1st bullet is about ordering the layers? However, whether model is layer-specific or layer-common is UE implementation, we should only discuss the latent message - whether it is layer-by-layer latent message, or contain information of all layers as a whole. Before having a clear view of how latent message is structured, it is premature to discuss priority rule. ### We assume the 2nd bullet means to say payload adaptation. However, it is not priority rule, it is about the UE action once priority is determined. Regarding the listed options, we think they violate the CSI omission principle. CSI omission is an emergency action, does not require any recalculation/quantization of the CSI. It's just simple omission operation, e.g., omit part of UCI part 2. The listed options are just ways for UE to adapt the CSI payload, they fall in the scope of option 2 of proposal 2-3-1. ### Based on above, we believe the proposal is premature, it should be discussed in WI after the latent message structure is decided. Note that in R15/R16/R17 CSI omission discussion occurred in late stage of WI after codebook and UCI were determined. |
| NEC | ### Support in general. |
| CATT | Fine with the 2nd bullet. ### But for the 1st bullet, we think how to design model (layer specific or layer common) is up to vendors, and the RAN1 spec should not differentiate the CSI collision handling rule/behavior for different model design. Don’t know what to study. |
| LG Electronics | In case of rank common/specific model, how the priority / omission rule can be handled? In our view, CSI reporting and CSI omission is generic CSI framework, so if supported, it should be applied commonly. |
| Xiaomi | ### In our view, the two sub-bullets are used to implement CSI omission. They do not have relationship with CSI collision. Once CSI collision occurs, the entity CSI reporting will be dropped. It is not necessary to use the provided methods in the two bullets for dropping CSI reporting. Therefore, we suggest the proposal is reworded as ### Proposal 2-3-2: In CSI compression using two-sided model use case, further study at least the following methods to support the priority rule regarding CSI collision handling and CSI omission: - Layer based priority rule for layer specific and layer common model. Model adaptation including different quantization method/granularity, different size of latent space, or CSI puncturing. |

### Proposal 2-3-3:

In CSI compression using two-sided model use case, further study at least the following methods to support codebook subset restriction:

- If input-CSI-NW/output-CSI-UE is in angular-delay domain, legacy SD basis vector-based restriction can be applied to input CSI directly.

- If input-CSI-NW/output-CSI-UE is in spatial-frequency domain, CBSD can be supported by projecting the input CSI in the subspace orthogonal to restricted sub-space before AI model.

- Other options are not precluded.

Please provide your view below:

| Company | View |
| --- | --- |
| Lenovo | We do not agree with the second bullet. Even if the UE projects the CSI before the AI model in the sub-space orthogonal to the restricted sub-space, this does not ensure that the output-CSI-UE would also be orthogonal to the restricted sub-space |
| Huawei, HiSilicon | OK with the first bullet. For the second bullet, we understand the intention but we are not sure whether projecting the input CSI to the orthogonal subspace is feasible. We suggest to rewrite the second bullet as FFS: if input-CSI-NW/output-CSI-UE is in spatial-frequency domain. ### Proposal 2-3-3 (modified): In CSI compression using two-sided model use case, further study at least the following methods to support codebook subset restriction: - If input-CSI-NW/output-CSI-UE is in angular-delay domain, legacy SD basis vector-based restriction can be applied to input CSI directly. - FFS: If input-CSI-NW/output-CSI-UE is in spatial-frequency domain, CBSD can be supported by projecting the input CSI in the subspace orthogonal to restricted sub-space before AI model. - Other options are not precluded. |
| Google | Support |
| ZTE | A minor wording comment, we suggest rewording as In CSI compression using two-sided model use case, further study at least the following methods to support codebook subset restriction: - If input-CSI-NW/output-CSI-UE is in angular-delay domain, legacy SD basis vector-based restriction can be applied to input CSIinput-CSI-NW directly. - If input-CSI-NW/output-CSI-UE is in spatial-frequency domain, CBSDCBSR can be supported by projecting the input CSIinput-CSI-NW in the subspace orthogonal to restricted sub-space before AI model. - Other options are not precluded. |
| vivo | Support |
| Qualcomm | We do not support the second bullet, projecting the input CSI does not ensure the output CSI at gNB side is restricted. Also, this is WI aspect, we prefer to defer the discussion to WI stage. |
| ETRI | We support. |
| LG Electronics | For angular-delay domain case, could anyone elaborate more how the legacy spatial domain vector restriction is applied? |
| Xiaomi | We are fine with the proposal updated by Huawei. |

### Proposal 2-3-4:

In CSI compression using two-sided model use case, further study the following options on top of legacy CPU principle:

- The required CPU value of CSI calculation is reported for per AI/ML model basis.

- Decouple CPU and measurement processing units (MPU) to define the UE processing load.

- Other options are not precluded.

Please provide your view below:

| Company | View |
| --- | --- |
| Lenovo | What does (MPU) refer to? Can the moderator (or proponents) provide definition of it? |
| NTT DOCOMO | Support the first bullet. As of the second bullet, it seems not the specific issue for CSI compression. |
| Huawei, HiSilicon | OK with the first bullet. For the second bullet, we have a similar question as Lenovo regarding the definition of MPU. |
| Google | Support. The ML may be based on a dedicated HW, that is the reason to introduce MPU. |
| ZTE | Please clarify the MPU since there is no such definition in current spec. |
| vivo | Support |
| Qualcomm | We should first discuss what are the additional criteria or rules to follow (and their necessity), not how to report the CPU value. Same comment for active resource/port – which is also an important processing criterion. Is MPU a new terminology? |
| OPPO | Support, and for AI/ML, can we just say XPU, e.g. it may be CPU, GPU, NPU, etc. |
| CATT | We think the proposal is OK at least for model-ID-based LCM. But is the first bullet still holds for functionality-based LCM? |
| LG Electronics | Agree with Docomo. |
| Xiaomi | We are not clear why should MPU be introduced. |

### Proposal 2-3-5:

In CSI compression using two-sided model use case, re-use the legacy CSI reporting principle with CSI Part 1 and Part 2 where Part 1 has a network configured fixed size and Part 2 size is dynamic, determined by information in Part 1.

Please provide your view below:

| Company | View |
| --- | --- |
| Lenovo | Support |
| Futurewei | We are fine with the proposal. |
| Panasonic | We are fine with the proposal. |
| NTT DOCOMO | Support. |
| Huawei, HiSilicon | OK in principle. |
| Google | Support |
| ZTE | Fine with the proposal. |
| vivo | Support |
| Qualcomm | This aspect needs further study, hence we prefer the following version: In CSI compression using two-sided model use case, for the study of UCI format, consider re-use the legacy CSI reporting principle with CSI Part 1 and Part 2 as a starting point, where Part 1 has a network configured fixed size and Part 2 size is dynamic, determined by information in Part 1. |
| NEC | OK |
| CATT | Fine in general. |
| ETRI | We support Qualcomm’s modification. |
| LG Electronics | Support |
| Xiaomi | Support |

### Proposal 2-3-6 (on hold, pending proposal 2-3-1)

In CSI compression using two-sided model use case, further study the following options to define the information used to ensure UE part model is compatible with the NW part model used by the gNB in CSI configuration and reporting:

- Option 1: The information is indicated through the CSI reconstruction model ID that NW will use.

- Option 2: The information is indicated through the CSI generation model ID that the UE will use.

- Option 3: The information is indicated through the paired CSI generation model and CSI reconstruction model.

- Option 4: The information is indicated though by the dataset ID during training type 3 offline training.

- Option 5: The information is indicated though a reference to a prior training session (e.g. using an API) between NW and UE.

- Other options are not excluded.

Please provide your view below:

| Company | View |
| --- | --- |
| NTT DOCOMO | Support the proposal. |
| Google | Support |
| vivo | Support |

## Performance monitoring, model update, activation/de-activation/switching

Following table summarize company’s proposals related to model performance monitoring, activation/de-activation/switching.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| vivo | - Monitoring inference accuracy is the most direct and reliable performance monitoring method for CSI compression with two-sided models. - Legacy codebook with potential enhancement can be used to report CSI measurement for performance monitoring at NW side in CSI compression. - Study monitoring inference accuracy at NW side as a baseline for performance monitoring in CSI compression. - For NW-side monitoring based on intermediate KPIs, study the necessity and specification impacts of enhancing legacy codebook configurations for CSI measurement reporting. - For UE-side monitoring based on the output of the CSI reconstruction model at NW side, study the feasibility and specification impacts of compressing output CSI indication over-the-air. - For UE-side monitoring based on the output of CSI reconstruction model at UE side, proxy model is a promising solution with satisfying monitoring reliability to avoid the concerns in transferring complicated CSI reconstruction model. - For UE-side monitoring based on the output of CSI reconstruction model at UE side, study the feasibility and specification impacts of using proxy model to generate the output CSI for the purpose of performance monitoring. - Using system KPIs for performance monitoring in CSI compression might have difficulties in judging whether an observed system performance degradation is caused by an outdated CSI compression model or some other reasons. - Monitoring based on data distribution can be viewed as a special case of monitoring based on applicable condition. - There could be accuracy and reliability issues for monitoring methods based on applicable condition. - Design of applicable condition-based performance monitoring methods and development of scenario-/configuration-/site-specific models should be jointly considered in CSI compression. - Study mechanisms for the two sides to jointly select a model among multiple candidate models, including: - Triggering conditions - How to conduct multi-model performance monitoring for purpose of model selection - Sharing of model selection results between NW and UE in CSI compression, where model ID based solution can be considered as a starting point. - Study the potential specification impact of triggering conditions for Model selection, switching/activation/deactivation, fallback. |
| Ericsson | Eventual KPI based monitoring has low complexity and low overhead and may capture network MU-MIMO performance as well. The NW can perform frequent monitoring of eventual KPIs and use it as a first step for detecting potential AI/ML feature/functionality failure. It is necessary to specify NW-side data collection to enable intermediate-KPI-based model monitoring at the NW-side. Intermediate-KPI-based model monitoring is expected to be implemented infrequently (e.g., event triggered or periodically with a large periodicity) at the NW-side, hence, the monitoring data collection overhead for intermediate-KPI-based model monitoring at the NW-side is in general not an issue. - In CSI compression using two-sided model use case, study potential specification impact to enable intermediate-KPI based model monitoring at the NW side. - RRC-message based and L1-fast CSI reporting-based methods to support UE reporting accurate/high-fidelity target CSI (ground truth of output CSI) together with the encoder output for data collection used for monitoring - Signaling and configuration for event triggered and periodical data collection used for monitoring. UE-side based monitoring is problematic as the UE does not have CSI-RS beamforming and cell shaping information nor can it capture the model’s performance in MU-MIMO which is the main motivation for AI/ML based CSI reporting Input/output data distribution-based monitoring method put requirements on computation power and memory at the UE side. Data drifts detected at the UE-part of a two-sided model does not necessarily mean that the two-sided model is not functioning. - In CSI compression using two-sided model use case, for input/output data distribution-based performance monitoring at the UE side, start with studying the feasibility of defining conditions, measurable data statistic KPIs and measurable monitoring results KPIs (e.g., false alarm rate, missed detection rate, latency) to evaluate the performance of this monitoring method. For CSI compression using two-sided model use case, the method of UE-side monitoring based on the output of the CSI reconstruction model indicated/provided by NW does not seem to be feasible in practice, since it may open for disclosing proprietary aspects of the NW-part model. For CSI compression using two-sided model use case, the method of UE-side monitoring based on a proxy model (e.g., Case 3 with a CSI reconstruction model or Case 4 with an intermediate KPI prediction/estimation model) at the UE introduces additional model LCM overhead for training/deploying/monitoring/testing the proxy model. - In CSI compression using two-sided model use case, for intermediate-KPI based performance monitoring at the UE side, study the feasibility of each candidate option first, before studying performance evaluations and potential spec impacts: - Option 1: UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW, or obtained from the network side. - Option 2: UE-side monitoring based on the output of a proxy model at the UE-side, where the proxy model is a proxy CSI reconstruction part. - Option 3: UE-side monitoring based on the output of proxy model at the UE-side, where the proxy model directly outputs intermediate KPIs. |
| ZTE | Proposal 18: In CSI compression using two-sided model use case, for UE-side monitoring, further study the potential specification impact to enable performance monitoring using the proxy model at UE side, at least for - Report monitoring metrics by UE - Model transfer of the proxy model for Type 1 network side training (if applicable); - The dataset required for training the proxy model at UE side for Type 3 NW-first training. Proposal 19: Prioritize to study the specification impacts on at least the following two cases for model performance monitoring, - Case 1: UE-side monitoring based on the output of the proxy CSI reconstruction model at the UE-side - Case 2: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. Proposal 20: For NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, further study a high-resolution CSI based on traditional codebook as ground-truth label. Observation 13: For training type 3, CSI generation model and CSI reconstruction model are actually two separate models. Therefore, if the performance of output CSI is degraded, it cannot be decided whether it’s due to the performance loss of CSI generation model or CSI reconstruction model. Proposal 21: Further study the potential mechanisms and specification impacts on monitoring model performance of the CSI generation model and CSI reconstruction model separately. Observation 14: Whether/how gNB will use the CSI provided by gNB is up to gNB implementation. Even with AI/ML-based CSI, gNB may need to make some adjustment on the AI/ML-based CSI in order to perform MU scheduling. From network perspective, the overall system performance is more important than the performance of a single UE. The precoding on PDSCH is transparent to UE side by nature and the gNB behavior should not be mandated. Observation 15: It is not feasible to support model monitoring based on eventual KPI, e.g., by monitoring the status of data channel to differentiate the performance variations between the reference scheme and AI/ML scheme. Proposal 22: Deprioritize the model performance monitoring based on eventual KPIs. Proposal 23: Prioritize to study the potential specification impact on AI/ML performance monitoring based on intermediate KPI. Proposal 24: Further study the feasibility of input/output-based monitoring methods in Agenda item 9.2.2.1. Proposal 25: The study of performance monitoring should be decoupled with the subsequent decision based on the monitoring results. |
| Spreadtrum Communications | Observation 1: It may be not necessary to do data collection for model monitoring. Observation 2: Codebook subset restriction can be not considered in CSI compression and recovery using two-sided model use case. Observation 3: For UE-side performance monitoring, eventual KPIs and input data based monitoring metric can be considered. Observation 4: For NW-side performance monitoring, eventual KPIs, legacy CSI based monitoring and output data based monitoring metric can be considered. Observation 5: For NW-side performance monitoring, when considering legacy CSI based monitoring, no spec enhancement is needed, and it can be up to gNB implementation. |
| Huawei | Proposal 19: In CSI compression using two-sided model use case, potential specification impact for input distribution-based or output distribution-based monitoring will be further discussed after initial evaluation is performed in 9.2.2.1. Proposal 20: To assist the model monitoring by taking into account the aspect of power consumption, it can be considered to introduce the metric report of power consumption from UE to gNB, e.g., whether the power consumption of the undergoing AI/ML model is higher (and how much higher if so) than the legacy non-AI/ML method. Proposal 21: For Network side monitoring based on intermediate KPI, study the reporting of the target CSI and the associated CSI report by the UE via UCI with higher priority. Proposal 22: For UE side monitoring based on intermediate KPI, study the indication of the recovery CSI with RRC signaling. - The association to the corresponding CSI report fed back at the inference stage can be indicated in together. Proposal 23: For UE side performance monitoring, Network may configure a threshold criterion to facilitate UE to perform model monitoring from the following aspects: - Usage of the threshold criterion, e.g., UE to perform conditional report of monitoring metrics, or to make the conditional monitoring decisions such as deactivation, switching, etc., based on the threshold. - Types of the threshold criterion, e.g., eventual KPI (e.g., ACK/NACK ratio, throughput, RSRP, etc.) and/or intermediate KPI (e.g., SGCS, NMSE, etc.). Proposal 24: For the Network side monitoring of intermediate KPI, to enable the comparison between AI/ML and legacy non-AI/ML, study the differentiated configurations/indications of AI/ML-based report and legacy CSI report, e.g., configuring separate time durations for AI/ML report and legacy report, indicating differentiated measurement resources, for AI/ML report and legacy report, etc. Proposal 25: For the UE side monitoring of eventual KPI, to enable the comparison between AI/ML and legacy non-AI/ML, study configuration/indication of the precoding type applied to the transmission for UE side performance monitoring, e.g., CSI-RS precoded by using AI/ML-based CSI feedback or non-AI/ML-based CSI feedback. Proposal 26: For model monitoring for CSI compression, study the performance comparison with an inactive AI/ML solution to facilitate making decision of switching/selection based on the comparison with the ongoing AI/ML model. |
| Nokia | Proposal 10: For CSI compression, RAN1 shall study the potential specification impact for performance monitoring by considering - Methods of performance monitoring - Option 1 (Fully-NW-sided): Use existing CSI feedback scheme for monitoring performance. - This is mainly for functionality LCM - Option 2 (NW-sided, UE-assisted): UE determine performance metrics and report them based on NW-defined/configured measurement resources, monitoring parameters, and reporting framework. - This is mainly for functionality LCM - Option 3 (UE-sided, NW-assisted): UE determine performance metrics (not report) based on UE-sided assumptions, and require some assistance from the NW for monitoring - This is mainly for model LCM (transparent to the NW) - Consider changes to the reporting framework for Option 1, Option 2 - For Option 1, strive to reuse the legacy CSI reporting framework. - For Option 2, study the enhancements of performance monitoring metrics and thresholds (if any), reporting quantities, reporting timelines, and other spec impacts - Consider changes to the measurement framework for Option 1, option 2, and option 3 - e.g., configure monitoring resources and periodicities - Consider functionality LCM aspects related to the performance monitoring |
| CATT | Proposal 6: In CSI compression using two-sided model use case, for NW side intermediate KPIs based monitoring, further study the signaling and procedures for reporting target CSI, with the following two options considered: - Option 1: The target CSI is reported together with its associated CSI report; - Option 2: The target CSI is reported separately from its associated CSI report. Proposal 7: In CSI compression using two-sided model use case, for NW side intermediate KPIs based monitoring, potential specification impact includes the following: - How to determine the association between target CSI and CSI report by the NW side; - Signaling and procedures for triggering target CSI reporting; - Types of 3GPP signaling takes responsibility on target CSI reporting, e.g., physical signaling, RRC signaling; - Types of target CSI for model monitoring, e.g., precoding matrix, channel matrix etc.; - Formats of target CSI for model monitoring: scaler quantization and/or codebook-based quantization (e.g., e-type II like). Proposal 8: In CSI compression using two-sided model use case, for UE side intermediate KPIs based monitoring, obtaining the output of the CSI reconstruction model based on the CSI reconstruction model by the UE is only supported for AI/ML model trained with training collaboration Type 1 at UE side. Proposal 9: In CSI compression using two-sided model use case, for UE side intermediate KPIs based monitoring, further study the signaling and procedures for transmitting output-CSI-UE from NW side to UE side, with the following options considered: - Option 1: The output-CSI-UE is transmitted to the UE in form of quantization values, e.g., scalar quantization or codebook-based quantization; - Option 2: The output-CSI-UE is transmitted to the UE in form of transmitting precoded CSI-RS that precoded with the output-CSI-UE. Proposal 10: In CSI compression using two-sided model use case, for UE side intermediate KPIs based monitoring, potential specification impact includes the following: - How to determine the association between output-CSI-UE and CSI report by the UE; - Signaling and procedures for indicating output-CSI-UE transmission; - Types of 3GPP signaling takes responsibility on transmitting output-CSI-UE, e.g., physical signaling, RRC signaling; - Types of output-CSI-UE for model monitoring, e.g., precoding matrix, channel matrix etc.; - Formats of output-CSI-UE for model monitoring: scaler quantization and/or codebook-based quantization (e.g., e-type II like). Proposal 11: In CSI compression using two-sided model use case, if eventual KPI is adopted as monitoring metric, how to exclude the impacts of other factors other than AI/ML model performance should be studied. Proposal 12: In CSI compression using two-sided model use case, for NW-side monitoring, it is beneficial to enable performance monitoring using an existing CSI feedback scheme as the reference. Further study potential specification impact on triggering and reporting additional legacy CSI. Proposal 13: In CSI compression using two-sided model use case, for NW-side monitoring with an existing CSI feedback scheme as reference, the following two options on determining the association between AI/ML scheme and existing CSI feedback scheme for monitoring are considered: - Option 1: The AI/ML based CSI is reported together with its associated legacy codebook-based CSI; - Option 2: Associating the associated AI/ML based CSI and legacy codebook-based CSI to a same reference, the reference can be target CSI, CSI-RS, ID, time-domain/frequency-domain resources, etc. Proposal 14: In CSI compression using two-sided model use case, for UE-side monitoring, further study potential specification impact on the following schemes: - Alt 1: UE reports the monitoring metric to NW side to assist the NW side to judge whether the AI/ML model is failed or workable. - Alt 2: UE reports the judgement on whether the AI/ML model is failed or workable to the NW side. Proposal 15: In CSI compression using two-sided model use case, for monitoring metric reporting for UE-side monitoring, study the feasibility and potential specification impact on reusing the legacy CSI reporting framework. |
| Fujitsu | Proposal-7: For the sub use case of CSI compression using two-sided AI/ML models, study the procedures needed for initiating the NW-side AI/ML model performance monitoring. Proposal-8: For the NW-side AI/ML model performance monitoring using existing CSI feedback scheme as the reference, study - the potential specification impact of UE reporting the CSI generated by the reference scheme to the NW, e.g., PMI; and/or, - the potential specification impact of UE reporting the performance of the CSI generated by the reference scheme to the NW, e.g., the SGCS of the recovered CSI from PMI and the ground-truth CSI. Proposal-9: For the CSI compression using two-sided AI/ML models use case, study the procedures and signaling needed for the follow-up mechanism after the AI/ML model performance monitoring, including the falling back to codebook-based CSI report from AI/ML-based CSI report. Proposal-10: For the AI/ML model performance monitoring of the CSI compression using two-sided AI/ML models use case, study the potential specification impacts on monitoring the performance of an AI/ML model in inactivate mode, taking at least the following cases into consideration. - Initial activation of an AI/ML model. - Re-activation of an AI/ML model. |
| Interdigital | Observation 4: It is possible that the AI/ML encoders do not generalize well across all realistic channel conditions. Proposal 8: Study means to configure/reconfigure the UE with the monitoring configuration, including the monitoring metric. Proposal 9: For UE-side monitoring, study both time- and event-based triggers for reporting the monitoring metrics. Proposal 10: For UE-side monitoring, study appropriate monitoring metrics to avoid unnecessary model updating or switching. Proposal 11: For UE-side monitoring, study the UE-side monitoring metrics (including report size, metrics quantization, report frequency) to avoid increasing the feedback overhead. Observation 5: Potential specification impacts of UE-side monitoring based on the output of the (NW-side) CSI reconstruction model include (but may not be limited to): format of the reconstructed CSI, CSI type (full channel matrix or eigenvector), identification of the corresponding CSI report, information on quantization of the reconstructed CSI. Observation 6: UE-side monitoring based on the output of the NW-side CSI reconstruction model may increase the downlink overhead, because the output CSI reconstructed at the NW needs to be indicated by the NW to the UE. Observation 7: Potential specification impacts of UE-side monitoring based on the output of the CSI reconstruction model at the UE-side include (but may not be limited to): UE indication of the ID of the UE-side reconstruction model, and means to adjust the intermediate KPI to account for the difference between the UE-side and NW-side CSI reconstruction models. Observation 8: UE-side monitoring based on the output of the CSI reconstruction model at the UE-side appears to have lower overhead compared to UE-side monitoring based on the output of the NW-side CSI reconstruction model. Observation 9: NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report has the potential to increase the feedback overheads as the target CSI is reported by the UE or obtained from the UE-side. Proposal 12: In case of NW-side monitoring, study monitoring approaches with low signaling overhead. Proposal 13: Study means to mitigate AI/ML encoder model performance degradation. Proposal 14: For CSI compression using two-sided model use case, study AIML model switching or AI/ML model (parameter) update to mitigate AI/ML model performance degradation. Proposal 15: Study mechanisms for fallback to legacy CSI reporting (e.g. for cases when AIML model performance is poor). |
| Intel | Observation 1: - Model performance monitoring based on intermediate KPI or eventual KPI calculated based on one AI-ML model is not giving enough information for proper configuration of AI-ML Model Proposal 1: - Testing of different AI-ML models with the measured channel should be considered for model performance monitoring Proposal 2: - For CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of intermediate KPIs based monitoring for - NW-side monitoring based on the target CSI from SRS and output CSI obtained from SRS measurements using the two-sided model (assuming that CSI generation part of the model is known at the gNB) |
| Google | Proposal 8: Study the AI/ML model adaptation for CSI compression in case of CSI omission, where different AI/ML models may be with different compression ratio. Proposal 9: Do not support to use SGCS as the metric for ML performance monitoring. Proposal 10: Support the hypothetical BLER as the metric for ML performance monitoring. Proposal 11: Support the baseline for model performance monitoring based on the non-ML based CSI, i.e. the CSI based on existing codebook that the UE supports. - ML based CSI compression should not mandate the UE to support eType2 codebook Proposal 12: Do not support the UE to report the Ground-truth CSI for model monitoring. Proposal 13: With regard to the UL overhead and performance, the model monitoring should not require the UE to report a non-ML based CSI. |
| Xiaomi | Observation 5: The feasibility and reliability need to evaluate for the three methods of model performance monitoring. Observation 6: The transmission overhead of target CSI or output CSI is usually larger for model performance. Observation 7: For NW-side model performance monitoring, network calculates the intermediate KPI, which save the calculation complexity of UE-side. Observation 8: The method, container and time behaviour of target CSI or output CSI transmission need to be specified. Proposal 9: For UE-side monitoring using an existing CSI feedback scheme as the reference, the following potential specification should be studied: - The association between AI/ML scheme and existing CSI feedback scheme for monitoring. - The precoding type indication/configuration of the beamformed PDSCH or CSI-RS. - The metric for monitoring and comparison includes intermediate KPI and eventual KPI |
| AT&T | Proposal 3: In CSI compression using two-sided model use case, for UE-side monitoring, further study feasibility and potential specification impact to enable performance monitoring and fallback using an existing CSI feedback scheme as the reference. - Configuration/indication of the precoding type applied to the PDSCH transmission or CSI-RS, i.e., whether precoding is based on reference scheme or AI/ML scheme. - The association between AI/ML scheme and existing CSI feedback scheme for monitoring - Other aspects are not precluded. |
| CAICT | Proposal 5: NW-side monitoring target CSI with realistic channel estimation feedback from UE could be considered as baseline for AI/ML model monitoring. Proposal 6: UE-side monitoring based on the output of the CSI reconstruction model at the UE-side or indicated by the NW from the network side could be considered as assistant. |
| NEC | Proposal 2: For UE-side performance monitoring, study how to report the performance metric(s). Proposal 3: Study to monitor the performances corresponding to multiple different ranks. Proposal 4: Study simultaneous model monitoring for multiple AI/ML models. |
| Lenovo | - Study the specification impact corresponding to AI model performance monitoring, as well as the corresponding scheme adaptation decision - The following four scheme adaptation decisions under AI model performance monitoring are considered as a starting point: (i) No AI model change, (ii) CSI parameters update, (iii) AI model parameter update, (iv) AI model switching, and (v) Fallback to non-AI scheme - Fallback to non-AI CSI feedback scheme is considered a part of the scheme adaptation mechanism - Network-based performance monitoring and model adaptation are supported by default - Further study the specification impact corresponding to the model monitoring schemes: (i) The network configuring the UE to report performance metrics that aid model monitoring, (ii) the network transmitting performance metrics to aid UE-based model monitoring, and (iii) Event-triggered AI model monitoring |
| Qualcomm | Proposal 22: Real-time performance monitoring that incurs high overhead, high complexity, or high latency should be deprioritized. Proposal 23: For model performance monitoring, specification change for reporting the target CSI with high resolution from UE to network requires clear justification as it incurs additional overhead and may not be necessary. Proposal 24: For model performance monitoring, study specification impact of the UE-side monitoring method that directly outputs intermediate KPI at the UE side. Proposal 25: Study specification impact of input-based model monitoring on the UE-side by comparing input samples at inference time to the training samples. |
| Oppo | Proposal 6: Regarding the performance monitoring metrics/methods for AI/ML model monitoring, eventual KPIs(e.g., hypothetical BLER) should be utilized for the performance monitoring, other options can be used to equivalent convert the eventual KPI. Proposal 7: The stability of the performance evaluating and decision-making mechanism should be further studied to avoid the interference of random effects on the evaluation results. - multiple attempts within an evaluation window both in PHY and high layers would be helpful to obtain a relatively stable evaluation result - multi-user involved mechanism should be addressed |
| NTT DOCOMO | Observation 4: NW side monitoring based on the target CSI can maintain the performance accuracy, when the quantization granularity is sufficiently small. Observation 5: The additional model storage and processing is required for UE side monitoring with the reconstruction/proxy model at UE side. However, the additional signalling overhead is not necessary to calculate the monitored performance metric. Observation 6: Empirical system performance does not require the additional signalling and measurement. However, the relevance to the model performance is low compared to the inference accuracy KPI. Proposal 6: UE side monitoring with NW indication of reconstructed CSI can be deprioritized for performance monitoring of CSI compression. Proposal 7: Discuss the feasibility of the performance monitoring based on system performance and the input/output data distribution in CSI compression, before the specification impact discussion related to it. Proposal 8: Study the L1/L2 reporting of the event occurrence or/and calculated performance metrics for real time performance monitoring, assuming UE side monitoring with the reconstruction/proxy model. Proposal 9: Study the L1/L2 signalling for the fallback operation indication to reduce the model/functionality failure duration. Proposal 10: Reuse legacy CSI reporting principle, unless technical issue is observed. Proposal 11: It is unnecessary to explicitly indicate/configure the CSI payload size. Instead, CSI payload size can be implicitly calculated based on the rank and model ID/functionality information. Proposal 12: CQI calculated based on target CSI is sufficient for CSI compression. Proposal 13: For CSI compression, the legacy priority rules for CSI collision handling and CSI omission can be reused except for the priority reporting level within the compressed bits. |
| MediaTek | - Discuss methods and apparatus for monitoring AI/ML models other than the one which is already being used by UE and NW. - For UE-side monitoring, UE can use L1-based signaling for event detection and RRC signal for assisting NW for subsequent actions if needed. - For NW-side monitoring, study spec impact of periodic (non-event-triggered) and aperiodic (event-triggered) monitoring separately. - Prioritize UE-side (Alternative 1) proxy-based model monitoring as the initial monitoring method for tracking intermediate KPI. - System-level indicators cannot be regarded as the single point of decisioning for detection of monitoring events. - Study multi-stage monitoring approach where a low-overhead low-accuracy method triggers a more accurate intermediate-KPI based solution with higher overhead. |
| ETRI | Observation 1: One possible performance improvement of AI models with training datasets from realistic channel estimation, is training denoising function additionally. Observation 2: To train the additional denoising function of the AI model for CSI compression, obtaining a training dataset with pairs can be required. Proposal 4: Consider further studies on potential specification impacts of model changes and fallback operation using the performance monitoring result. |

### Summary:

Some of the proposals are captured in the data collection section, including:

- For NW side intermediate KPI based performance monitoring, the proposal is captured in data collection section including RRC/L1 feedback.

- For UE side intermediate KPI based performance monitoring with a proxy CSI reconstruction model, the dataset to train the proxy model is also captured in data collection section.

- On proposals related to enabling high accuracy ground truth feedback format, since there is ongoing evaluation in 9.2.2.1, proposals will be drafted later based on 9.2.2.1 study.

On input/output data distribution-based monitoring, and applicable condition-based monitoring, it was proposed to discuss input/output distribution as a specifical case of applicable condition. There was also strong concern on the effectiveness of these type of monitoring. FL propose to study in 9.2.2.1 first before potential specification impact is discussed.

Discussion in this section continue the discussion on UE side monitoring.

### Proposal 2-4-1:

In CSI compression using two-sided model use case, for UE-side monitoring, further study the necessity, feasibility, and potential specification impact to enable transmit output CSI from NW to UE including:

- Signaling and procedures for indicating output-CSI transmission

- Method and format of output-CSI to be transmitted from NW to UE

  - Option 1: The output-CSI is transmitted to the UE in form of quantization values, e.g., scalar quantization or codebook-based quantization.

  - Option 2: The output-CSI is transmitted to the UE in form of transmitting precoded CSI-RS with the output-CSI.

- The association between output-CSI at NW and CSI report by the UE

- Other aspects are not precluded.

Please provide your view below:

| Company | View |
| --- | --- |
| Lenovo | Support |
| Futurewei | We are ok with the proposal. |
| Panasonic | UE side monitoring based on NW-side feedback of output-CSI requires much overhead to indicate the output-CSI to the UE. Although we are OK to further study this monitoring solution, we think it should be deprioritized. |
| NTT DOCOMO | Support the proposal. |
| Huawei, HiSilicon | OK with the proposal except Option 2 in the second bullet. If, instead of quantized output-CSI, precoded CSI-RS is sent to the UE, UE has to further estimate output-CSI from the received precoded CSI-RS. This, by its own, is an additional source of estimation error which makes Option 2 always inferior to option 1 which is based direct transmission of output-CSI to UE. We suggest the following: ### Proposal 2-4-1 (modified): In CSI compression using two-sided model use case, for UE-side monitoring, further study the necessity, feasibility, and potential specification impact to enable transmit output CSI from NW to UE including: - Signaling and procedures for indicating output-CSI transmission - Method and format of output-CSI to be transmitted from NW to UE - Option 1: The output-CSI is transmitted to the UE in form of quantization values, e.g., scalar quantization or codebook-based quantization. - Option 2: FFS: The output-CSI is transmitted to the UE in form of transmitting precoded CSI-RS with the output-CSI. - The association between output-CSI at NW and CSI report by the UE - Other aspects are not precluded. |
| Google | OK. We do not agree with HW’s revision. We think option 2 is better than option 1 in terms of overhead. |
| ZTE | For UE side monitoring based on NW-side feedback of output-CSI, the method needs to send back the output of CSI reconstruction part from NW side to UE, which will lead to additional latency. However, the channel condition may already change a lot (e.g., interference) so that PMI and CQI mismatch is unavoidable. In addition, the output CSI should be quantized (e.g., by eType II codebook), which will lead to additional quantization loss. Moreover, sending the output CSI needs enhanced specification to support it. It is also not appropriate for this method to perform UE-side monitoring. Therefore, we propose to prioritize the agreed NW-sided and UE-sided methods to monitor the model performance. |
| vivo | Support |
| OPPO | Support |
| CATT | Support. This is a high-level summary which seems inclusive enough. And also feel that Option 1 and Option 2 should be treated in the same level regarding spec impact, so prefer the original wording. |
| LG Electronics | Agree with ZTE, already agreed monitoring method seems sufficient. |
| Xiaomi | Support |

### Proposal 2-4-2:

In CSI compression using two-sided model use case, for UE-side monitoring, network may configure a threshold criterion to facilitate UE to perform model monitoring including at least:

- Types of the threshold criterion, e.g., eventual KPI (e.g., ACK/NACK ratio, throughput, RSRP, etc.) and/or intermediate KPI (e.g., SGCS, NMSE, etc.).

- Threshold value of each criterion.

- UE report based on the criterion and threshold.

Please provide your view below:

| Company | View |
| --- | --- |
| Lenovo | Support |
| Futurewei | We are ok with the proposal. |
| Panasonic | We are fine with the proposal. |
| NTT DOCOMO | Fine with the principle. However, the feasibility of performance monitoring based on eventual KPI should be confirmed first before studying the potential specification impacts. Hence, we suggest removing the eventual KPI in the example. |
| Huawei, HiSilicon | We think the purpose of the threshold criterion should also be clarified, eg, UE perform conditional report of monitoring metrics, or to make the conditional monitoring decisions such as deactivation, switching, etc., based on the threshold. We suggest the following modification: ### Proposal 2-4-2 (modified): In CSI compression using two-sided model use case, for UE-side monitoring, network may configure a threshold criterion to facilitate UE to perform model monitoring including at least: - Types of the threshold criterion, e.g., eventual KPI (e.g., ACK/NACK ratio, throughput, RSRP, etc.) and/or intermediate KPI (e.g., SGCS, NMSE, etc.). - Threshold value of each criterion. - UE report based on the criterion and threshold. - purpose of the threshold criterion ( eg, conditional report of monitoring metrics, conditional monitoring decisions such as deactivation, switching, etc) |
| Google | Support in principle. We suggest adding “hypothetical BLER” as an example to be aligned with previous agreements. |
| ZTE | We think the feasibility of performance monitoring based on eventual KPI should be proved, since it is hard to identify whether the eventual performance degrades due to the impacts of other factors other than model performance, e.g., varying channel status, varying scheduling/pairing mechanism, etc. So, we suggest rewording the 1st bullet as Types of the threshold criterion, e.g., eventual KPI (e.g., ACK/NACK ratio, throughput, RSRP, etc.) and/or intermediate KPI (e.g., SGCS, NMSE, etc.). |
| vivo | Support |
| OPPO | Support, and based on the identified metrics and threshold criterions, the stability of performance evaluating should be further studied as well. |
| CATT | OK. |
| LG Electronics | Support |
| Xiaomi | We also have concern on eventual KPI. The feasibility of performance monitoring based on eventual KPI should be firstly studied. |

### Proposal 2-4-3:

In CSI compression using two-sided model use case, potential specification impact for applicability condition-based monitoring, input distribution-based and output distribution-based monitoring will be further discussed after initial evaluation is performed in 9.2.2.1.

Please provide your view below:

| Company | View |
| --- | --- |
| Panasonic | We are fine with the proposal. |
| NTT DOCOMO | Potential specification impact of eventual KPI-based monitoring should also be discussed after initial evaluation is performed in 9.2.2.1. |
| Huawei, HiSilicon | Support |
| Google | OK |
| ZTE | We agree with NTT Docomo, the feasibility of eventual KPI-based monitoring should also be evaluated in 9.2.2.1. So, we suggest adding ‘eventual KPI-based’ in the proposal as In CSI compression using two-sided model use case, potential specification impact for applicability condition-based monitoring, input distribution-based and, output distribution-based and eventual KPI- based monitoring will be further discussed after initial evaluation is performed in 9.2.2.1. |
| vivo | Support |
| OPPO | Support |
| CATT | OK. |
| LG Electronics | Support |
| Xiaomi | Support the proposal. For eventual KPI-based performance monitoring, we have similar view with NTT Docomo and ZTE. |

## Framework, UE capability, and other topics

Following table summarize company’s proposals related to framework.

| Company | Key Proposals/Observations/Positions |
| --- | --- |
| ZTE | Proposal 26: In CSI compression using two-sided model use case, further study potential specification impact of the following UE capability options: - How to quantize the UE capability for a CSI report based on the CSI generation model, e.g., CPU occupancy rule and CSI computation time . - UE capability related to LCM procedures |
| Nokia | Proposal 1: For the two-sided CSI feedback compression sub-use case, RAN1 shall define conditions for functionalities to enable functionality-based LCM. Proposal 2: For the two-sided CSI feedback compression sub-use case, RAN1 to study the following conditions for functionalities, • CSI-RS measurement conditions • CSI-RS and CSI reports configuration conditions • CSI calculation conditions (i.e., number of occupied CPUs) • Output CSI conditions • Compression ratio conditions (e.g., CR4, CR8, …) • Quantizer conditions (e.g., SQ1, VQ1, …) • Pairing ID (e.g., model ID, dataset ID) • Generic conditions on supporting ML functionalities Proposal 3: For the two-sided CSI feedback compression sub-use case, identify the additional conditions prior to discussing any reporting framework for that. Proposal 4: For the two-sided CSI feedback compression sub-use case, after NW configures functionalities to the UE, study a reporting framework to report applicable functionalities at the UE side. |
| xiaomi | Proposal 10: UE side model or UE part model for CSI compression feedback can be identified through AI/ML functionality, AI/ML model, or both functionality and model. How to define AI/ML functionality and/or AI/ML model for CSI compression feedback should be firstly studied and discussed. |
| CAICT | Proposal 7: As for the conditions for functionality-based/model-ID-based LCM, MIMO related configurations, application scenario(s), model training type and payload size could be considered as starting point. |
| NVIDIA | Proposal 12: For AI/ML based CSI feedback, study the aspects that should be specified as conditions of a Feature/FG available for functionality. Proposal 13: For AI/ML based CSI feedback, study the aspects that should be considered as additional conditions and how to include them into model description information during model identification. |
| oppo | Proposal 8: To align NW/UE side AI/ML capability(or supported AI/ML feature/FG), follow options could be considered for further study Option1： UE initial, including Option1-1: UE capability report on whether AI/ML based CSI feedback(or CSI generation) is supported Option1-2: UE capability report on whether AI/ML based CSI feedback(or CSI generation) is supported, NW indication on whether AI/ML based CSI feedback(or CSI reconstruction) is supported. Option2：NW initial, including Option2-1: NW indication on whether AI/ML based CSI feedback(or CSI reconstruction) is supported Option2-2: NW indication on whether AI/ML based CSI feedback(or CSI reconstruction) is supported, UE indication on whether AI/ML based CSI feedback(or CSI generation) is supported |
| NTT DOCOMO | Observation 2: Approaches to pair the model at UE side and NW side for CSI compression can be categorized into two types: the compatibility check by UE and the compatibility check by NW. Observation 3: if offline training is applied and compatibility between the encoder and the decoder is ensured, it is not necessary to introduce the quantization alignment mechanism in 3GPP. |

Functionality/model ID based LCM and UE capability report framework is discussed in section 9.2.1. Proposals will be drafted later based on progress in 9.2.1.

# GTW May 22

To facilitate pros/cons discussion/observation of each training collaboration type, we need to align the table structure to move forward. Two main issue: (1) how to capture gradient exchange sequential training (2) how to capture known/unknown model structure (device specific/device agnostic) under NW side training collaboration type 1.  Since this is for observation, not for specification impact, FL suggest taking the option with majority view.

Proposal 2-1-1:

In CSI compression using two-sided model use case, to capture the gradient exchange sequential training methods, choose one of the following options:

- Option 1: Modify type 2 training collaboration to include sequential training.

  - Type 2: Joint or sequential training of the two-sided model at network side and UE side respectively, through gradient exchange between nodes.

- Option 2: Define a new training type:

  - Type 4: Sequential training of the two-sided model at network side and UE side respectively, through gradient exchange between nodes.

- Option 3: Clarify type 2 training collaboration with a note.

  - Type 2: Joint training of the two-sided model at network side and UE side, respectively.

    - Note: Joint training includes both simultaneous training and sequential training.

Proposal 2-1-2:

In CSI compression using two-sided model use case, for discussion of training collaboration type 1 at NW side, choose one of the following options:

- Option 1: Prioritize the pros/cons discussion assuming a known model structure at UE.

- Option 2: Create separate columns to discuss both known model structure at UE, and unknown model structure at UE separately.

- Option 3: Capture both known/unknown model structure in the same column when needed.

Proposal 2-2-1:

In CSI compression using two-sided model use case, further study the necessity, complexity, overhead, latency and potential specification impact on ground truth CSI report for NW side data collection for model performance monitoring, including:

- Scalar quantization for ground-truth CSI

  - FFS: any processing applied to the ground-truth CSI before scalar quantization

- Codebook-based quantization for ground-truth CSI

  - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1

- RRC signaling and/or L1 signaling procedure to enable fast identification of AI/ML model performance.

- gNB-triggered or periodic ground-truth CSI report.

# Appendix: Companies input on training collaboration type comparison table.

FUTUREWEI:

Our views are in bold text and highlighted in green if different from the proposed observation.

Table 2.1-1: Pros and cons analysis across training types

| Training types Characteristics | Type 1 |  | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided |  | NW first | UE first |
| Whether model can be kept proprietary | No | No | Yes | Yes (Note 3) | Yes (Note 3) |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No | No (Note 1) | No (Note 1) | No (Note 1) |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes. With assisted information signaling. Less flexible than Type 1-NW side. | Difficult | Semi-flexible. | Semi-flexible. With assisted information signaling |
| Whether gNB/device specific optimization is allowed | Restricted | Restricted | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible (note 4) | Conditional, flexible with assisted information (note 4) | Not flexible (note 4) | Semi-flexible | Conditional semi-flexible, with assisted information (note 4) |
| Feasibility of allowing UE side and NW side to develop/update models separately (This row can be combined with the “Extendibility” row.) | Feasible but limited (Note 2) | Feasible but limited (Note 2) | Infeasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Conditional, depending on the training dataset and training method. | No | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | Yes | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use (This row can be combined with the “Feasibility” row.) | Feasible but limited (Note 2) | Feasible but limited (Note 2) | Limited | Support | Support |
| Whether training data distribution can match the inference device | Conditional and restricted, with assisted information from UE for device (group) specific model. | Yes | Restricted | Conditional, with assisted information from UE | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Yes, for device specific model. Restricted for device (group) specific model. | Limited Yes | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Note 1: Assume high accuracy PMI is not privacy sensitive data. FFS: other information such as channel matrix and assisted information.

Note 2: For example, after deploying model 1 on the UE side, a new UE model can be obtained by using model 1 as the teacher model and using knowledge distillation method. Model 1 can also refer to a nominal model while the real deployed model can be developed based on the nominal model.

Note 3: Assume information on model structure is not required to be disclosed in training collaboration type 3.

Note 4: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors.

Vivo

- Our modified table for proposed observation 2-1-1 is summarized below, where the modified parts are marked with yellow:

| Training types Characteristics | Type 1 |  | Type 2 | Type 3 |  | Gradient exchange sequential |
| --- | --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided |  | NW first | UE first |  |
| Whether model can be kept proprietary | No | No | Yes | Yes (Note 3) | Yes (Note 3) | Yes |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No | No (Note 1) | No (Note 1) | No (Note 1) | No (Note1) |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes. With assisted information signaling. Less flexible than Type 1-NW side. | Difficult | Semi-flexible. | Semi-flexible. With assisted information signaling | Difficult |
| Whether gNB/device specific optimization is allowed | Restricted | Restricted | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible (note 4) | Conditional, flexible with assisted information (note 4) | Not flexible (note 4) | Semi-flexible | Conditional semi-flexible, with assisted information (note 4) | Not flexible (note 4) |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited (Note 2) | Limited (Note 2) | Infeasible | Feasible | Feasible | Infeasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes No | No | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | Yes | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Limited (Note 2) | Limited (Note 2) | Limited | Support | Support | Support |
| Whether training data distribution can match the inference device | Conditional, with assisted information from UE for device specific model. | Yes | Restricted | Conditional, with assisted information from UE | Yes | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Yes for device specific model. No for device-agnostic model. z4 | Limited Yes | Compatible | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Note 1: Assume high accuracy PMI is not privacy sensitive data. FFS: other information such as channel matrix and assisted information.

Note 2: For example, after deploying model 1 on the UE side, a new UE model can be obtained by using model 1 as the teacher model and using knowledge distillation method. Model 1 can also refer to a nominal model while the real deployed model can be developed based on the nominal model.

Note 3: Assume information on model structure is not required to be disclosed in training collaboration type 3.

Note 4: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors.

Note 5: Yellow highlighted rows are for further discussion.

Ericsson

- Modify the discussed table that summarize the training collaboration types, and add the additional notes plus the columns for training type 4, as below

| Training types Characteristics | Type 1 |  | Type 2 | Type 3 |  | Type 4, frozen decoder with gradient API |
| --- | --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided |  | NW first | UE first |  |
| Whether model can be kept proprietary | No | No | Yes | Yes (Note 3) | Yes (Note 3) | Yes |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No | No (Note 1) | No (Note 1) | No (Note 1) | No (note 1) |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes.Limited, and requiring additional assisted information signaling. Less flexible than Type 1-NW side. (note 8) | Difficult | Semi-flexible. | Semi-flexible. With assisted information signaling | Yes |
| Whether gNB/device specific optimization is allowed | Restricted | Restricted (note 6) | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible (note 4) | Conditional, flexible with assisted information (note 4) | Not flexible (note 4) | Semi-flexible | Conditional semi-flexible, with assisted information (note 4) | Flexible (note 4) |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited (Note 2) | Limited (Note 2) | Infeasible | Feasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes No | No | Pending evaluation in 9.2.2.1 | Yes (note 10)Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 (note 9) | Yes (note 10) |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | Yes (note 6) | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 (note 9) | Yes (note 10)Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 (note 9) |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Limited (Note 2) | Limited (Note 2) | Limited | Support | Support | Support |
| Whether training data distribution can match the inference device | Conditional, with assisted information from UE for device specific model. Yes, to the extent needed. (note 7) | Yes | Restricted | Conditional, with assisted information from UE Yes, to the extent needed. (note 7) | Yes | Yes. |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Yes for device specific model. No for device-agnostic model. | Limited Yes | Compatible | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Note 1: Assume high accuracy PMI is not privacy sensitive data. FFS: other information such as channel matrix and assisted information.

Note 2: For example, after deploying model 1 on the UE side, a new UE model can be obtained by using model 1 as the teacher model and using knowledge distillation method. Model 1 can also refer to a nominal model while the real deployed model can be developed based on the nominal model.

Note 3: Assume information on model structure is not required to be disclosed in training collaboration type 3.

Note 4: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors.

Note 5: Yellow highlighted rows are for further discussion.

Note 6: It seems unlikely that the Type 1 UE-sided training can achieve gNB HW specific optimizations and at the same time allow the UE to run a single unified model. The UE-side may not know what optimizations are needed, different gNBs may have different needs which may also evolve over time as new products reach the market.

Note 7: Simulations from UE/Chipset vendor [R1-2303582] shows that mixing datasets from different Device Types give improved performance. The conclusion that a mixed dataset is generally better is commonplace in AI/ML literature and has been seen in this SI also for other aspects. Hence, the need for matching the inference device in training seems limited.

Note 8: It has been shown in this SI, e.g., [R1-2303475, R1-2302477, R1-2302358], that gNB properties such as, e.g., antenna layout and virtualization may affect the performance of the models. Since these properties can be proprietary and subject to change the UE does not know them. Hence, the UE can neither guarantee that datasets are appropriately mixed at training, nor decide what specialized model is suitable for cell/site/scenario.

Note 9: Additional assistance signaling may be needed. Once the first side has done training, a model defines a mapping between latent space codeword and CSI, i.e., implicitly defining a codebook. If multiple vendors are part in the first-side training, those multiple models may represent multiple codebooks. For the second side to train a unified model, it would require assistance information to ensure that a unified model compresses/decompresses according to the correct codebook.

Note 10: Under the assumption that the vendor training first has engineering freedom to design its own model, the condition follows naturally. To understand the effects of long-term evolution of the AI/ML model in the eco-system, further studies are needed.

ZTE

Table 1. The pros/cons of different offline training collaboration types

| Training types Characteristics | Type 1 |  | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided | Joint/Sequential gradient exchange | NW first | UE first |
| Whether model can be kept proprietary | No | No | Yes | Yes | Yes |
| Whether require privacy-sensitive dataset sharing | No | No | No | No | No |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes. With assisted information signaling. Less flexible than Type 1-NW side. | Difficult | Semi-flexible | Semi-flexible. With assisted information signaling |
| Whether gNB/device specific optimization is allowed | Restricted | Restricted | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible | Flexible. Less flexible than Type 1-NW side. | Not flexible | Semi-flexible | Semi-flexible. Less flexible than Type 3-NW side. |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited (Pending evaluation in 9.2.2.1) | Limited (Pending evaluation in 9.2.2.1) | Infeasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes | No | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | Yes | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Limited | Limited | Limited | Support | Support |
| Whether training data distribution can match the inference device | Conditional, with assisted information from UE | Yes | Restricted | Conditional, with assisted information from UE | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Conditional, with assisted information from UE | Yes | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Spreadtrum Communication

Table 1 Analysis on Training Types

| Training types Characteristics | Type 1 |  |  | Type 2 | Type 3 |  | Gradient exchange sequential |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NW-sided |  | UE-sided |  | NW first | UE first |  |
|  | Device agnostic | Device specific |  |  |  |  |  |
| Whether model can be kept proprietary | No | No | No | Yes | Yes (Note 3) | Yes (Note 3) | Yes |
| Whether require privacy-sensitive dataset sharing | No (Note1) | No | No | No (Note 1) | No (Note 1) | No (Note 1) | No |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes | Yes. With assisted information signaling. Less flexible than Type 1-NW side. | Difficulty | Semi-flexible. | Semi-flexible. With assisted information signaling | Difficulty |
| Whether gNB/device specific optimization is allowed | Restricted | Restricted | Restricted | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible (note 4) | Flexible (note 4) | Conditional flexible, with assisted information (note 4) | Not flexible (note 4) | Semi-flexible | Conditional semi-flexible, with assisted information (note 4) | Not flexible (note 4) |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited (Note 2) | Limited (Note 2) | Limited (Note 2) | Infeasible | Feasible | Feasible | Limited (Note 2) |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes | No | No | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | No | Yes | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Limited (Note 2) | Limited (Note 2) | Limited | Limited | Support | Support | Limited |
| Whether training data distribution can match the inference device | Conditional, with assisted information from UE | Conditional, with assisted information from UE | Yes | Restricted | Conditional, with assisted information from UE | Yes | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No | Yes | Yes | Yes | Yes | Yes | Yes |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Note 1: Assume high accuracy PMI is not privacy sensitive data. FFS: other information such as channel matrix and assisted information. Note 2: For example, after deploying model 1 on the UE side, a new UE model can be obtained by using model 1 as the teacher model and using knowledge distillation method. Model 1 can also refer to a nominal model while the real deployed model can be developed based on the nominal model. Note 3: Assume information on model structure is not required to be disclosed in training collaboration type 3. Note 4: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors. |  |  |  |  |  |  |  |

Huawei

Table 2 Brief comparison of the training types for two-sided model

|  | Type 1 |  | Type 2 (including gradient exchange sequential training) | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided |  | NW first | UE first |
| Whether model can be kept proprietary | No | No | Yes | Yes | Yes |
| Whether require privacy-sensitive dataset sharing | No | No | No | No | No |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | No | No | Yes | No |
| Whether gNB/device specific optimization is allowed | No/Restricted for device agnostic; Yes for device specific; | Restricted | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible | Not flexible | Not flexible | Semi-flexible | Not flexible |
| Engineering isolation (feasibility of allowing UE side and NW side to develop/update models separately) | Isolable/Restricted for device agnostic; Non-isolable for device specific; | Non-isolable | Strongly non-isolable | Isolable | Isolable |
| Model performance | Optimal/Restricted for device agnostic; Suboptimal for device specific; | Suboptimal | Suboptimal | Suboptimal | Suboptimal |
| Whether gNB can maintain/store a single/unified model | Yes/Restricted for device agnostic; No for device specific; | No | Yes | Yes | Restricted |
| Whether UE device can maintain/store a single/unified model | Yes | Yes | Yes | Yes | Yes |
| Extendibility | Support | Support | Support | Support | Support |
| Whether training data distribution can match the inference device | Restricted | Yes | Restricted | Restricted | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No for device agnostic; Yes for device specific; | No | Yes | Yes | Yes |

Nokia

| Training types Characteristics | Type 1 |  |  | Type 2 | Type 3 |  | Gradient exchange sequential |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NW-sided |  | UE-sided |  | NW first | UE first |  |
|  | Device agnostic | Device specific |  |  |  |  |  |
| Whether model can be kept proprietary | No | No | No | Yes | Yes (Note 3) | Yes (Note 3) | Yes |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No (Note 1) | No | No (Note 1) | No (Note 1) | No (Note 1) | No (Note 1) |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes | Semi-flexible. With assisted information signaling. Less flexible than Type 1-NW side. | Difficult | Semi-flexible. | Semi-flexible. With assisted information signaling | Semi-flexible. |
| Whether gNB/device specific optimization is allowed | Restricted | Yes | Restricted | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible (note 4) | Flexible (note 4) | Conditional, flexible with assisted information (note 4) | Not flexible (note 4) | Semi-flexible | Conditional semi-flexible, with assisted information (note 4) | Semi-flexible. |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited (Note 2) | Limited (Note 2) | Limited (Note 2) | Infeasible | Feasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes | Depends on the number of devices; practically may not be feasible | No | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | No | Yes | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Limited (Note 2) | Limited (Note 2) | Limited (Note 2) | Limited | Support | Support | Support |
| Whether training data distribution can match the inference device | Restricted | Conditional, with assisted information from UE | Yes | Restricted | Conditional, with assisted information from UE | Yes | Conditional, with assisted information from UE |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No | Yes | Yes | Compatible | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Note 1: Assume high accuracy PMI is not privacy sensitive data. FFS: other information such as channel matrix and assisted information.

Note 2: For example, after deploying model 1 on the UE side, a new UE model can be obtained by using model 1 as the teacher model and using knowledge distillation method. Model 1 can also refer to a nominal model while the real deployed model can be developed based on the nominal model.

Note 3: Assume information on model structure is not required to be disclosed in training collaboration type 3.

Note 4: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors.

Panasonic

Observation 14: Pros and cons for training collaboration types are summarized as follows.

|  | Type 1-NW | Type 1-UE | Type 2 | Type 3 NW-first | Type 3 UE-first |
| --- | --- | --- | --- | --- | --- |
| Whether model can be kept proprietary | No | No | Yes | Yes | Yes |
| Requirements on privacy-sensitive dataset sharing | May need assistance information | May need assistance information | May need assistance information | May need assistance information | May need assistance information |
| Flexibility to support cell / site / scenario / configuration specific model | Yes | Less flexible | Difficult | Yes | Less flexible |
| gNB / device specific optimization – i.e., whether hardware-specific optimization of the model possible, e.g., compilation for the specific hardware | Device agnostic: Restricted Device specific: Possible | Restricted | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible | Less flexible | Not flexible | Less flexible | Less flexible |
| Feasibility of allowing UE side and NW side to develop / update model separately | Limited | Limited | Infeasible | Feasible | Feasible |
| Whether gNB can maintain / store a single / unified model | Device agnostic: Yes Device specific: No | No | Yes | Yes | No |
| Whether UE device can maintain / store a single / unified model | No | Yes | Yes | Yes | Yes |
| Extendibility: To train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Limited | Limited | Limited | Yes | Yes |
| Whether training data distribution can be matched to the device that will use the model for inference | Device agnostic: No Device specific: Yes | Yes | Restricted | Yes | Yes |
| Whether device capability can be considered for model development | Device agnostic: No Device specific: Yes | Yes | Yes | Yes | Yes |

Xiaomi

Table 2: The pros and cons of joint training of two-sided model at NW sided and NW-first separate training

| Training types Characteristics | Type 1 (Joint training of the two-sided model at NW side with device-specific) | Type 3 (NW-first separate training) |
| --- | --- | --- |
| Whether model can be kept proprietary | No | Yes (Note 3) |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No (Note 1) |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Semi-flexible. |
| Whether gNB/device specific optimization is allowed | Yes | Yes |
| Model update flexibility after deployment | Flexible (note 4) | Semi-flexible (note 4) |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited (Note 2) | Feasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | No | Support |
| Whether training data distribution can match the inference device | Yes | Conditional, with assisted information from UE |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Yes | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Fraunhofer IIS,  Fraunhofer HHI

| Type of training | Type 1- UE side | Type 1 – NW side | Type 1 – 3rd party | Type 2 | Type 3 – NW first | Type 3 – UE first |
| --- | --- | --- | --- | --- | --- | --- |
| UE-processing compatibility | No issues | UE vendor assistance required for UE compatibility | UE and gNB vendors assistance required | No issues (UE-vendor trained) | UE and gNB vendors assistance required | No issues (UE-vendor trained) |
| Encoder-decoder compatibility | Trained at one side | Trained at one side | Trained at one side | Jointly trained | Separately trained | Separately trained |
| Logistics of AI model design | Design by a single vendor, high complexity at UE-side | Easy (training at single entity) | Easy (training at single entity) | Harder (vendor pairings needed for AI model development) | Easy (independent training regardless of encoder-decoder pairing) | Easy (independent training regardless of encoder-decoder pairing) |
| Logistics of AI model training | No issues | Training data exchange is necessary (UE vendors need to exchange training data with | Training data exchange is necessary | Harder (vendor pairs need to exchange training data) | Harder (vendor pairs need to exchange training data) | Harder (vendor pairs need to exchange training data) |
| Revealing proprietary information | NW vendor may need to reveal some implementation info to UE vendor for decoder design | Some proprietary information may be revealed by UE side vendor to NW vendor for designing UE-compatible encoder | Some proprietary information may be revealed by UE | Some proprietary information may be revealed between pairing vendors | No revealing of proprietary information is required | No revealing of proprietary information is required |
| Performance guarantees | Possible | Possible | Possible | Possible | Hard to guarantee due to higher chances of encoder-decoder mismatch | Hard to guarantee due to higher chances of encoder-decoder mismatch |

Qualcomm

|  | Type 1 |  |  |  |  | Type 2 | Type 3 |  | NW-first Gradient exchange sequential |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NW-sided |  |  | UE-sided | Level Y |  | NW first | UE first |  |
|  | Device agnostic | Device specific (z2) | Device specific (z4) |  |  |  |  |  |  |
| Defining assumptions of each training type column |  |  |  |  |  |  |  |  |  |
| Device specificity of encoder (specific / agnostic)? | Agnostic | Specific | Specific | Specific | Specific | Specific | Specific | Specific | Specific |
| Collaboration level | z5 | z2 | z4 | z | y | y | y | y | y |
| Encoder training vendor (NW-vendor / UE-vendor / other)? | NW-vendor | NW-vendor | NW-vendor | UE-vendor | UE-side / NW-side / neutral site | UE-vendor | UE-vendor | UE-vendor | UE-vendor |
| Decoder training vendor (NW-vendor / UE-vendor / other)? | NW-vendor | NW-vendor | NW-vendor | UE-vendor | UE-side / NW-side / neutral site | NW-vendor | NW-vendor | NW-vendor | NW-vendor |
| Model transfer | NW to UE | NW to UE | NW to UE | UE to NW | None | None | None | None | None |
| Multi-vendor joint training assumption | 1 NW and multiple UE vendors |  |  |  |  |  | Not applicable |  |  |
| Data collection for decoder training | At NW | At NW | At NW | Proprietary | Proprietary | At NW or proprietary | At NW or proprietary | Shared by UE-side | At NW or proprietary |
| Data collection for encoder training | At NW | At NW | At NW | Proprietary | Proprietary | At NW or proprietary | Shared by NW-side | Proprietary | Proprietary |
| Properties of each training type |  |  |  |  |  |  |  |  |  |
| Whether model can be kept proprietary | No | No | No | No | No | Yes | Yes | Yes | Yes |
| Whether require privacy-sensitive dataset sharing | No | No | No | No | No | No | No | No | No |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes | Yes | Yes, with assistance info / vendor collaboration | Yes, with assistance info / vendor collaboration | Yes | Yes | Yes, with assistance info / vendor collaboration | Yes |
| Whether gNB specific optimization is allowed | Yes | Yes | Yes | No | Yes | Yes | Yes | Yes | Yes |
| Whether device specific optimization is allowed | No | Yes | No | Yes | Yes | Yes | Yes | Yes | Yes |
| Whether the model is fully tested before deployment | No | Yes | No | Yes | Yes | Yes | Yes | Yes | Yes |
| Whether compilation capability can be avoided | No | Yes | No | Yes | Yes | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment | Yes | Parameter update Yes Structure update No |  | No | No | No | Yes | Yes | Yes |
| Feasibility of allowing UE side and NW side to develop/update models separately | No | No | No | No | No | No | Yes | Yes | Yes |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes | Yes for decoder No for encoder (for model transfer) |  | No | Yes | Yes | Yes | Yes | Yes |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration * | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| Extensibility: to train new UE-side model compatible with NW-side model in use | No | No | No | No | No | No | No | No | Yes |
| Extensibility: to train new NW-side model compatible with UE-side model in use | No | No | No | No | No | No | No | Yes | No |
| Whether training data distribution can match the inference device | No | Yes** | Yes** | Yes | Yes | Yes** | Yes** | Yes | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

* “Whether UE device can maintain/store a single/unified model for a CSI report configuration” : In some cases, a UE may need to switch to a different encoder when the NW vendor changes upon handover; however, since the wording says "for a CSI report configuration", we have assumed this case is not included in the question.

** “Whether training data distribution can match the inference device” : Dataset should be categorized per device type during data collection, otherwise the answer is "No".

CMCC

Observation 11: In CSI compression using two-sided model use case, the following table capture the pros/cons of different offline training collaboration types:

| Training types Characteristics | Type 1 |  | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided |  | NW first | UE first |
| Whether model can be kept proprietary | No | No | Yes | Yes | Yes |
| Whether require privacy-sensitive dataset sharing | No | No | No | No | No |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes. With assisted information signaling. Less flexible than Type 1-NW side. | Difficult | Semi-flexible. | Semi-flexible. With assisted information signaling |
| Whether gNB/device specific optimization is allowed | Restricted | Restricted | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible | Conditional, flexible with assisted information | Not flexible | Semi-flexible | Conditional semi-flexible, with assisted information |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited | Limited | Infeasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes | No | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | Yes | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Limited | Limited | Limited | Support | Support |
| Whether training data distribution can match the inference device | Conditional, with assisted information from UE | Yes | Restricted | Conditional, with assisted information from UE | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Conditional, with assisted information from UE | Yes | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Lenovo

| Training types Characteristics | Type 1 |  | Type 2 | Type 3 |  |
| --- | --- | --- | --- | --- | --- |
|  | NW-sided | UE-sided |  | NW first | UE first |
| Whether model can be kept proprietary | No | No | Yes | Yes | Yes |
| Whether requires privacy-sensitive dataset sharing | No | No | No | No | No |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Difficult | Moderate | Moderate | Moderate |
| Whether gNB/device specific optimization is allowed | Difficult | Difficult | Yes | Yes | Yes |
| Model update flexibility after deployment | Difficult | Difficult | Moderate | Moderate | Moderate |
| Feasibility of allowing UE side and NW side to develop/update models separately | Difficult | Difficult | Difficult | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model | Depends on the generalizability of the designed model |  |  |  |  |
| Whether UE device can maintain/store a single/unified model | Depends on the generalizability of the designed model |  |  |  |  |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Difficult | Difficult | Difficult | Feasible | Feasible |
| Whether training data distribution can match the inference device | Moderate | Yes | Yes | Yes | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No | Compatible | Compatible | Compatible | Compatible |

Table 1: Comparison of different training collaboration types

Samsung

Proposal 2-7: Consider Table 4 for the comparison of two-sided model training types:

Table 4: Challenges for two-sided model development approaches

| Training types Characteristics | Type 1 |  |  | Type 2 | Type 3 |  | Gradient exchange sequential |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NW-sided |  | UE-sided |  | NW first | UE first |  |
|  | Device agnostic | Device specific |  |  |  |  |  |
| Whether model can be kept proprietary | No | No | No | Yes (Note 1) | Yes (Note 1) | Yes (Note 1) | Yes (Note 1) |
| Whether require privacy-sensitive dataset sharing | No | No | No | Needs check | Need check | Needs check | Needs check |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes | Yes. With assisted information signaling. | Difficult | Semi-flexible. | Semi-flexible. With assisted information signaling | Difficult |
| Whether gNB/device specific optimization is allowed | Restricted | Yes | Restricted | Yes (Partially) | Yes | Yes | Yes (Partially) |
| Model update flexibility after deployment | Flexible | Flexible | Conditional, with assisted information (Note 2) | Not flexible | Conditional semi-flexible, with assisted information (Note 2) | Semi-flexible (Note 2) | Not flexible |
| Feasibility of allowing UE side and NW side to develop/update models separately | Does not apply | Does not apply. | Does not apply | Infeasible | Feasible | Feasible | Infeasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes | No | Does not apply | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | Yes | No | Does not apply | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Does not apply | Does not apply | Does not apply | Limited | Support | Support | Support |
| Whether training data distribution can match the inference device | Infeasible | Conditional, with assisted information from UE | Yes | Conditional, with assisted information from UE | Conditional, with assisted information from UE | Yes | Conditional, with assisted information from UE |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No | Yes | Yes | Yes | Yes | Yes | Yes |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Performance dependency for interoperability of multiple UE-side and Network-side models. | Does not apply | Does not apply | Does not apply | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Note 1: Some information pertaining to model structure may have to be exchanged between the UE-side and the network-side for better alignment. For example, for Type 2 training the last layer of the UE-side and the first layer of the network-side shall be aligned.

Note 2: Mode update via Type 3 training may require simultaneous update across multiple vendor, which is practically infeasible.

IIT Kanpur

Observation-1:

Table 1: Pros and cons for different training types

| Training types Characteristics | Type 1 |  |  | Type 2 | Type 3 |  | Gradient exchange sequential |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NW-sided |  | UE-sided |  | NW first | UE first |  |
|  | Device Agnostic | Device Specific |  |  |  |  |  |
| Whether model can be kept proprietary | No | No | No | Yes | Yes (Note 3) | Yes (Note 3) | Yes |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No | No | No (Note 1) | No (Note 1) | No (Note 1) | No |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes | Yes. With assisted information signaling. Less flexible than Type 1-NW side. | Difficult | Semi-flexible. | Semi-flexible. With assisted information signaling | Semi-flexible. |
| Whether gNB/device specific optimization is allowed | Yes (gNB specific), No (device specific) | Yes | Restricted | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible (Should meet our Proposal 1 criteria) | Semi Flexible | Conditional, flexible with assisted information (note 4) | Not flexible (note 4) | Semi-flexible | Conditional semi-flexible, with assisted information (note 4) | Semi-flexible |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited (Note 2) | Limited | Limited (Note 2) | Infeasible | Feasible | Feasible | Infeasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes | Yes for gNB-side decoder | No | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | Yes (device agnostic is anyways a common model, it will be transferred to UE) | No | Yes | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Yes, for NW-side model | Limited (Note 2) | Limited (Note 2) | Limited | Support | Support | Yes, for UE-side model |
| Whether training data distribution can match the inference device | Limited (our Proposal-1 criteria should satisfy) | Conditional, with assisted information from UE | Yes | Yes | Conditional, with assisted information from UE | Yes | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Limited (If our assumption in Proposal-1 is satisfied) | Compatible | Compatible | Compatible | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Note 1: Assume high accuracy PMI is not privacy sensitive data. FFS: other information such as channel matrix and assisted information.

Note 2: For example, after deploying model 1 on the UE side, a new UE model can be obtained by using model 1 as the teacher model and using knowledge distillation method. Model 1 can also refer to a nominal model while the real deployed model can be developed based on the nominal model.

Note 3: Assume information on model structure is not required to be disclosed in training collaboration type 3.

Note 4: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors.

Apple

Table I: Comparison of different training collaboration

.

| Training types Characteristics | Type 1 |  |  | Type 2 | Type 3 |  | Gradient exchange sequential |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NW-sided |  | UE-sided |  | NW first | UE first |  |
|  | Device agnostic | Device specific |  |  |  |  |  |
| Whether model can be kept proprietary | No | No | No | Yes | Yes | Yes | Yes |
| Whether require privacy-sensitive dataset sharing | No (Note 1) | No (Note 1) | No | No (Note 1) | No (Note 1) | No (Note 1) | No (Note 1) |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes | Yes. With assisted information signaling. Less flexible than Type 1-NW side. | Difficult | Semi-flexible. | Semi-flexible. With assisted information signaling | Difficult |
| Whether gNB/device specific optimization is allowed | Restricted | Restricted | Restricted | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment | Flexible (note 4) | Less flexible than device agnostic | Conditional, flexible with assisted information (note 4) | Not flexible (note 4) | Semi-flexible | Conditional semi-flexible, with assisted information (note 4) | Not flexible (note 4) |
| Feasibility of allowing UE side and NW side to develop/update models separately | Limited (Note 2) | Limited (Note 2) | Limited (Note 2) | Infeasible | Feasible | Feasible | Infeasible |
| Whether gNB can maintain/store a single/unified model for a CSI report configuration | Yes | Yes for CSI generation model | No | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Whether UE device can maintain/store a single/unified model for a CSI report configuration | No | No | Yes | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |
| Extendibility: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use | Limited (Note 2) | Limited (Note 2) | Limited (Note 2) | Limited | Support | Support | Support |
| Whether training data distribution can match the inference device | Conditional, with assisted information from UE for device specific model. | Conditional, with assisted information from UE for device specific model. | Yes | Yes | Conditional, with assisted information from UE for device specific model. | Yes | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No | Compatible | Compatible | Compatible | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 | Pending evaluation in 9.2.2.1 |

Note 1: Assume high accuracy PMI is not privacy sensitive data. FFS: other information such as channel matrix and assisted information.

Note 2: For example, after deploying model 1 on the UE side, a new UE model can be obtained by using model 1 as the teacher model and using knowledge distillation method. Model 1 can also refer to a nominal model while the real deployed model can be developed based on the nominal model.

Note 3: Assume information on model structure is not required to be disclosed in training collaboration type 3.

Note 4: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors.

Note 5: Yellow highlighted rows are for further discussion.

# Appendix: Previous meeting agreements

## RAN1 #109e

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

  - Note: Study of other sub use cases is not precluded.

  - Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

Conclusion

- Further discuss temporal-spatial-frequency domain CSI compression using two-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

- Further discuss improving the CSI accuracy based on traditional codebook design using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

- Further discuss CSI prediction using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss CSI-RS configuration and overhead reduction as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss resource allocation and scheduling as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion

- Further discuss joint CSI prediction and compression as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

## RAN1 110

Agreement

In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied:

- Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

- Type 2: Joint training of the two-sided model at network side and UE side, respectively.

- Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

- Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes).

- Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW

- Other collaboration types are not excluded.

Conclusion

CSI-RS configuration and overhead reduction is NOT selected as one representative sub-use case for CSI feedback enhancement use case.

Conclusion

Resource allocation and scheduling is NOT selected as one representative sub-use case for CSI feedback enhancement use case.

Agreement

In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least

- CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input.

- CQI determination

- RI determination

Agreement

In CSI compression using two-sided model use case, further study potential specification impact on output CSI, including at least

- Model output type/dimension/configuration and potential post processing

Agreement

In CSI compression using two-sided model use case, further discuss at least the following aspects, including their necessity/feasibility/potential specification impact,  for data collection for AI/ML model training/inference/update/monitoring:

- Assistance signaling for UE’s data collection

- Assistance signaling for gNB’s data collection

- Delivery of the datasets.

## RAN1 #110bis-e

Conclusion

Joint CSI prediction and CSI compression is NOT selected as one representative sub-use case for CSI feedback enhancement use case.

Conclusion

CSI accuracy enhancement based on traditional codebook design is NOT selected as one representative sub-use case for CSI feedback enhancement use case.

Conclusion

Temporal-spatial-frequency domain CSI compression using two-sided model is NOT selected as one representative sub-use case for CSI enhancement use case.

• 	Up to each company to report whether past CSI is used as model input for spatial-frequency domain CSI compression

Agreement

In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including:

- NW-side performance monitoring:  NW monitors the performance and make decisions of model activation/ deactivation/updating/switching

- UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/ deactivation/updating/switching

Agreement

In CSI compression using two-sided model use case, further study potential specification impact related to assistance signaling and procedure for model performance monitoring.

Agreement

In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

Agreement

In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods:

- Intermediate KPIs as monitoring metrics (e.g., SGCS)

- Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK).

- Legacy CSI based monitoring: schemes using additional legacy CSI reporting

- Other monitoring solutions, at least including the following option:

  - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

Agreement

In CSI compression using two-sided model use case, further study at least use cases of the following potential specification impact on quantization method alignment between CSI generation part at UE and CSI reconstruction part at gNB:

- Alignment of the quantization/dequantization method and the feedback message size between Network and UE

## RAN1 #111

Agreement

Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement.

Note: Continue evaluation discussion in 9.2.2.1.

Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer futher till the end of R18 AI/ML SI.

Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

Conclusion

In CSI compression using two-sided model use case, training collaboration type 2 over the air interface for model training (not including model update) is deprioritized in R18 SI.

Note:

- To align terminology, output CSI assumed at UE in previous agreement will be referred as output-CSI-UE.

- To align terminology, input-CSI-NW is the input CSI assumed at NW

## RAN1 #112

Agreement

In CSI compression using two-sided model use case, further study potential specification impact of the following output-CSI-UE and input-CSI-NW at least for Option 1:

- Option 1: Precoding matrix

  - 1a: The precoding matrix in spatial-frequency domain

  - 1b: The precoding matrix represented using angular-delay domain projection

- Option 2: Explicit channel matrix (i.e., full Tx * Rx MIMO channel)

  - 2a: raw channel is in spatial-frequency domain

  - 2b: raw channel is in angular-delay domain

- Note: Whether Option 2 is also studied depends on the performance evaluations in 9.2.2.1.

- Note: RI and CQI will be discussed separately

Agreement

In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured.

- Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including

  - Option 1a: CQI is calculated based on target CSI with realistic channel measurement

  - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

  - Option 1c: CQI is calculated based on legacy codebook

- Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including

  - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment

    - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW.

  - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder.

- Other options are not precluded

- Note1: feasibility of different options should be evaluated

- Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated

- Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

Conclusion

In CSI compression using two-sided model use case, further discuss the pros/cons of different offline training collaboration types including at least the following aspects:

- Whether model can be kept proprietary

- Requirements on privacy-sensitive dataset sharing

- Flexibility to support cell/site/scenario/configuration specific model

- gNB/device specific optimization – i.e., whether hardware-specific optimization of the model is possible, e.g. compilation for the specific hardware

- Model update flexibility after deployment

- feasibility of allowing UE side and NW side to develop/update models separately

- Model performance based on evaluation in 9.2.2.1

- Whether gNB can maintain/store a single/unified model

- Whether UE device can maintain/store a single/unified model

- Extendability: to train new UE-side model compatible with NW-side model in use; Or to train new NW-side model compatible with UE-side model in use

- Whether training data distribution can be matched to the device that will use the model for inference

- Whether device capability can be considered for model development

- Other aspects are not precluded

- Note: training data collection and dataset/model delivery will be discussed separately

Agreement

- In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least

- Enhancement of CSI-RS configuration to enable higher accuracy measurement.

- Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc.

  - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side.

- Signaling for triggering the data collection

- In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least:

- Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement.

- Contents of the ground-truth CSI including:

  - Data sample type, e.g., precoding matrix, channel matrix etc.

  - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like).

  - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator)

- Latency requirement for data collection

- Signaling for triggering the data collection

Agreement

In CSI compression using two-sided model use case, further study the following aspects for CSI configuration and report:

- NW configuration to determine CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration.

- How UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network.

Agreement

In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least:

- The priority rule regarding CSI collision handling and CSI omission

- Codebook subset restriction

- CSI processing Unit

Agreement

In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least:

- NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side.

- UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side.

  - Network may configure a threshold criterion to facilitate UE to perform model monitoring.

- UE-side monitoring based on the output of the CSI reconstruction model at the UE-side

  - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side.

  - Network may configure a threshold criterion to facilitate UE to perform model monitoring.

- FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE.

Note: Monitoring approaches not based on intermediate KPI are not precluded

Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

## RAN1 #112bis-e

Agreement

The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study potential specification enhancement on

- CSI-RS configurations (No discussion on CSI-RS pattern design enhancements)

- CSI reporting configurations

- CSI report UCI mapping/priority/omission

- CSI processing procedures.

- Other aspects are not precluded.

Agreement

In CSI compression using two-sided model use case, for UE-side monitoring, further study potential specification impact on triggering and means for reporting the monitoring metrics, including periodic/semi-persistent and aperiodic reporting, and other reporting initiated from UE.

Agreement

In CSI prediction using UE-side model use case, whether to address the potential spec impact of CSI prediction depends on RAN#100 final conclusion, focusing on the following

- data collection procedure, mainly including RS configuration, measurement and report configuration, resusing as much as possible what is defined for UE side use cases

- monitoring procedure and metric for AI-based CSI prediction.

- Model/functionality selection/switching and finetuning procedure.

- Note: Discussion on potential specification impact is limited to aspects which would NOT duplicate the work in Rel-18 MIMO WI.

- Note: Minimize LCM related potential specification impact discussion that follow the high-level principle of other one-sided model sub-cases.

Agreement

In CSI compression using two-sided model use case, for NW-side monitoring, further study the necessity, feasibility and potential specification impact to enable performance monitoring using an existing CSI feedback scheme as the reference.

- The association between AI/ML scheme and existing CSI feedback scheme for monitoring

- Note: The metric for monitoring and comparison includes intermediate KPI and eventual KPI.

- Other aspects are not precluded.

Conclusion

In CSI compression using two-sided model use case, gradient-exchange based sequential training over the air interface is deprioritized in R18 SI.

Agreement

In CSI compression using two-sided model use case, further study the necessity and potential specification impact of the following aspects related to the ground truth CSI format for NW side data collection for model training:

- Scalar quantization for ground-truth CSI

  - FFS: any processing applied to the ground-truth CSI before scalar quantization, based on evaluation results in 9.2.2.1

- Codebook-based quantization for ground-truth CSI

  - FFS: Parameter set enhancement of existing eType II codebook, based on evaluation results in 9.2.2.1

- Number of layers for which the ground truth data is collected. And whether UE or NW determine the number of layers for ground-truth CSI data collection.

Agreement

In CSI compression using two-sided model use case, further study the necessity and potential specification impact on quantization alignment, including at least:

- For vector quantization scheme,

  - The format and size of the VQ codebook

- Size and segmentation method of the CSI generation model output

  - For scalar quantization scheme,

- Uniform and non-uniform quantization

  - The format, e.g., quantization granularity, the distribution of bits assigned to each float.

- Quantization alignment using 3GPP aware mechanism.

# Reference

R1-2304372	Discussion on other aspects of AI/ML for CSI feedback enhancement	FUTUREWEI

R1-2304472	Other aspects on AI/ML for CSI feedback enhancement	vivo

R1-2304522	Discussion on AI-CSI	Ericsson

R1-2304535	Discussion on other aspects for AI CSI feedback enhancement	ZTE

R1-2304551	Discussion on other aspects on AIML for CSI feedback	Spreadtrum Communications

R1-2304654	Discussion on AI/ML for CSI feedback enhancement	Huawei, HiSilicon

R1-2304682	Other aspects on ML for CSI feedback enhancement	Nokia, Nokia Shanghai Bell

R1-2304723	Discussion on AI/ML for CSI feedback enhancement	CATT

R1-2304765	Views on specification impact for CSI feedback enhancement	Fujitsu

R1-2304780	Discussion on AI/ML for CSI feedback enhancement	InterDigital, Inc.

R1-2304814	Discussion on AI/ML for CSI feedback	Intel Corporation

R1-2304843	On Enhancement of AI/ML based CSI	Google

R1-2304855	Discussion on AI/ML for CSI feedback enhancement	China Telecom

R1-2304869	Discussion on AI/ML for CSI feedback enhancement	Panasonic

R1-2304894	Further discussion on specification impact for CSI feedback based on AI/ML	xiaomi

R1-2304949	Discussion on AI/ML for CSI feedback enhancement	AT&T

R1-2305016	Discussions on AI-ML for CSI feedback	CAICT

R1-2305032	Considerations on CSI measurement enhancements via AI/ML	Sony

R1-2305058	Discussion on AI/ML based methods for CSI feedback enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2305070	Discussion on AI/ML for CSI feedback enhancement	NEC

R1-2305086	Discussion on other aspects on AI/ML for CSI feedback enhancement	CMCC

R1-2305161	AI and ML for CSI feedback enhancement	NVIDIA

R1-2305203	Further aspects of AI/ML for CSI feedback	Lenovo

R1-2305235	Discussion on other aspects of AI/ML for CSI enhancement	Apple

R1-2305297	Other aspects on AI/ML for CSI feedback enhancement	LG Electronics

R1-2305329	Other aspects on AI/ML for CSI feedback enhancement	Qualcomm Incorporated

R1-2305460	On sub use cases and other aspects of AI/ML for CSI feedback enhancement	OPPO

R1-2305506	Discussion on potential specification impact for CSI feedback enhancement	Samsung

R1-2305592	Discussion on other aspects on AI/ML for CSI feedback enhancement	NTT DOCOMO, INC.

R1-2305656	Other aspects on AI/ML for CSI feedback enhancement	MediaTek Inc.

R1-2305763	Discussion on AI/ML for CSI feedback enhancement	ITL

R1-2305790	Discussion on other aspects on AI/ML for CSI feedback enhancement	ETRI

R1-2305873	Other aspects on AI/ML for CSI feedback enhancement	IIT Kanpur