3GPP TSG-RAN WG1 #113	Tdoc R1-2306054

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.2.4.1

Source:	Moderator (Ericsson)

Title:	Summary #1 of Evaluation on AI/ML for positioning accuracy enhancement

Document for:	Discussion, Decision

# Introduction

This document summarizes the discussions during RAN1#113 for the agenda item 9.2.4.1, Evaluation on AI/ML for positioning accuracy enhancement.

This discussion corresponds to the objectives related to the positioning use case described in RP-213599 (SID) below.

RP-213599 (SID): Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. … Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

The list of contact of companies' delegates is maintained below.

| Company | Name | Email |
| --- | --- | --- |
| Apple | Kome Oteri | ooteri@apple.com |
| CATT | Yongqiang Fei | feiyongqiang@catt.cn |
| CMCC | Liu Yongchang | liuyongchang@chinamobile.com |
| China Telecom | Bei YANG | yangbei1@chinatelecom.cn |
| Ericsson | Yufei Blankenship | yufei.blankenship@ericsson.com |
| Ericsson | Henrik Ryden | henrik.a.ryden@ericsson.com |
| Fujitsu | Wang, Xin | wangxin@fujitsu.com |
| Fujitsu | Shan, Yujia | shanyujia@fujitsu.com |
| Huawei, HiSilicon | Thorsten Schier | thorsten.schier@huawei.com |
| Intel | Javad Abdoli | Javad.abdoli@intel.com |
| InterDigital | Fumihiro Hasegawa | Fumihiro.Hasegawa@InterDigital.com |
| LG Electronics | Jaehoon Chung | jhoon.chung@lge.com |
| MediaTek | Harrison Chuang | harrison.chuang@mediatek.com |
| Nokia | Dick Carrillo Melgarejo | dick.carrillo_melgarejo@nokia.com |
| Nokia | Venkatraman, Ganesh | ganesh.venkatraman@nokia.com |
| OPPO | Zhe Liu | zhe.liu@oppo.com |
| QC | Mohammed Hirzallah (Ali) | mhirzall@qti.qualcomm.com |
| Samsung | Qi (Mark) Xiong | q1005.xiong@SAMSUNG.COM |
| NEC | Wei Chen | chen_wei@nec.cn |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| vivo | Huaming Wu | huaming.wu@vivo.com |

# Dataset, model complexity

## Dataset - general

- Nokia (R1-2304685) Observation 1: The final technical report should highlight the usability of data augmentation to enhance the model performance used in AI/ML positioning. Observation 2: One key challenge in AI/ML-based positioning is the availability of good quality data with sufficient diversity of positioning ground truth labels and samples with accurate information for fine tuning, updating, and monitoring. Observation 3: For data collection, the required sample density (e.g., #samples/m2) as indicator to achieve a given positioning accuracy target is limited to simulated scenarios using UE’s uniform distribution. Thus, it is important to determine a complementary indicator for quality data collection in addition to sample density. Proposal 2: RAN1 to study and evaluate for AI/M-based positioning additional quality indicators for data collection (e.g., IPD) in addition to sample/density (# samples/m2) for non-uniform UEs distribution. Observation 5: For data collection, RAN1 to consider additional metrics (e.g., IPD) to sample density (e.g., #samples/m2) to identify potential non-uniform UE distribution. Consequently, to trigger methodologies (e.g., data augmentation) to enhance the data collection and model performance.

- MediaTek (R1-2305659) Proposal 11 : Further evaluate performance of AI/ML positioning for non-uniform UE distribution.

## Impact of user density/size of the training dataset

### Direct AI/ML positioning

- MediaTek Inc. (R1-2305659) Table 32. Evaluation results for AI/ML model deployed on UE or network-side without generalization, CNN, UE distribution area = [120x60 m], different UE density Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size UE density; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Model complexity; Computation complexity; AI/ML. CIR [18,2,256]; UE pos [x,y]; 0%; InF-DH{60%,6m,2m}; InF-DH{60%,6m,2m}; 4.5UEs/m2; 464.24K; 0.266G; 0.940. 2.25UEs/m2; 1.273. 1.125UEs/m2; 1.839. 0.5625UEs/m2; 3.250. PDP [18,2,256]; UE pos [x,y]; 0%; 4.5UEs/m2; 463.95K; 0.264G; 0.821. 2.25UEs/m2; 1.357. 1.125UEs/m2; 2.039. 0.5625UEs/m2; 2.758. Observation 42: Performance of direct AI positioning decreases as the UE density decreases. Table 33. Evaluation results for AI/ML model deployed on UE or network-side without generalization, CNN, UE distribution area = [120x60 m], non-uniform UE distribution Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. PDP [18,2,256]; UE pos [x,y]; 0%; InF-DH{60%,6m,2m}; Left InF-DH; 3600 (left) +14400 (right)=18000; 3600; 463.95K; 0.264G; 1.518. Right InF-DH; 3600; 1.021. Proposal 11 : Further evaluate performance of AI/ML positioning for non-uniform UE distribution.

- China Telecom (R1-2304857) Observation 2: The performance of AI/ML based positioning enhancement decreases when the training dataset size decreases, but it still significantly outperforms the traditional method.

- NVIDIA (R1-2305164) Figure 9: Positioning accuracy of AI/ML based method under different user densities. Observation 9: Increasing user density can improve the positioning accuracy of the AI/ML model for RF fingerprinting. For example, increasing the user density by 64x can reduce the 90th percentile of the positioning error from 2.3 m to 74 cm. Observation 10: Increasing user density to improve the positioning accuracy of the AI/ML model for RF fingerprinting comes at the cost of higher requirement on the training dataset for supervised training. Proposal 4: Study data collection methods (e.g., utilizing digital twin technology) for obtaining training data set with high user density.

- xiaomi (R1-2304897) Figure 3 Relationship between size of training data set and positioning accuracy for direct AI-based positioning

- Huawei (R1-2304657) Observation 20 : For direct AI/ML positioning, when the model input is PDP or CIR, over different dataset sizes for training, the performance of AI/ML-based fingerprint positioning decreases when the training dataset becomes smaller. In general, less complex models converge faster and need less labels to achieve a given accuracy, e.g., • 4 TRPs with length-128 PDP-based fingerprinting can provide sub-meter accuracy with training 15,000 samples, whereas the more complex CIR based model requires up to 25,000 samples for the same input dimensions.

- Ericsson (R1-2304339) Table 7 90%tile 2D positioning accuracy using PDP inputs for different model size classes and different training dataset sizes in the {60%, 6m, 2m} InF-DH scenario. Nt=Nt'=256. Model class; Positioning approach; Model complexity [# parameters]; Computational complexity [FLOPs]; 90%tile 2D positioning error [m] with different train set sizes in {60%, 6m, 2m} InF-DH. 80,000; 40,000; 20,000; 10,000. Small models PDP; Dist. Assist.; 0.43 M; 11.5 M; 0.531; 0.680; 0.902; 1.249. Cent. Assist.; 0.36 M; 9 M; 0.426; 0.524; 0.658; 0.873. Cent. Direct; 0.36 M; 9 M; 0.426; 0.510; 0.656; 0.863. Medium-size models PDP; Dist. Assist.; 1.69 M; 43 M; 0.351; 0.476; 0.675; 1.004. Cent. Assist.; 1.4 M; 34 M; 0.282; 0.360; 0.474; 0.707. Cent. Direct; 1.4 M; 34 M; 0.269; 0.349; 0.496; 0.735. Large models PDP; Dist. Assist.; 5.6 M; 140 M; 0.273; 0.403; 0.596; 0.933. Cent. Assist.; 5.6 M; 132 M; 0.202; 0.271; 0.397; 0.629. Cent. Direct; 5.6 M; 132 M; 0.214; 0.288; 0.425; 0.653. - Proposal 4 To further 3GPP discussion and preparation of observations/conclusions for the technical report, four train dataset size classes are defined as follows: - Small datasets: density ~1.39 UE/m2 - Medium-size datasets: density ~2.79 UE/m2 - Large datasets: density ~5.56 UE/m2 - Very large datasets: >6.94 UE/m2

### AI/ML-assisted positioning

- MediaTek Inc. (R1-2305659) Table 2. Evaluation results for AI/ML model deployed on UE or network side, without model generalization, CNN, UE distribution area = 120x60 m, different user density Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; LOS/NLOS identification accuracy. Training; test; Training & validation; test; Model complexity; Computational complexity; AI/ML. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; {40%, 2m, 2m}; {40%, 2m, 2m}; 6480; 720; 185.7k; 29.4M*18; 92.8%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 185.7k; 29.4M*18; 95.2%. Observation 2: High user density of training dataset provides an improvement in LOS/NLOS identification accuracy over the low user density. Table 10. Evaluation results for AI/ML model deployed on UE or network side, without model generalization, CNN, UE distribution area = 120x60 m, different user density Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training & validation; test; Model complexity; Computational complexity; AI/ML. non-normalized PDP (18*8*256); 18TOA; Ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 6480; 720; 205k; 77M; 2.4. non-normalized PDP (18*8*256); 18TOA; Ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 1.05. non-normalized PDP (18*8*256); 18TOA; Ideal; {40%, 2m, 2m}; {40%, 2m, 2m}; 6480; 720; 205k; 77M; 3.1. non-normalized PDP (18*8*256); 18TOA; Ideal; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 205k; 77M; 1.56. Observation 13: High user density of training dataset provides an improvement in AI/ML assisted TOA estimation positioning over the low user density. Proposal 4 : Support different user density of training dataset for different requirement on AI/ML positioning.

- Apple (R1-2305973) Table 1: AI/ML-assisted Positioning (TOA estimation): Evaluation results for AI/ML model deployed on UE or Network-side, without model generalization, with a CNN, UE distribution area = 100x40 m Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR [18,1,256,2]; Direct path ToA [1x1]; Ideal TOA 100% labeled; Drop 1; Drop 1; 47500; 2500; 1.6 M; 3.1 G; 1.138m. CIR [18,1,256,2]; Direct path ToA [1x1]; Ideal TOA 100% labeled; Drop 1; Drop 1; 23750; 1250; 1.6 M; 3.1 G; 1.396m. CIR [18,1,256,2]; Direct path ToA [1x1]; Ideal TOA 100% labeled; Drop 1; Drop 1; 9500; 500; 1.6 M; 3.1 G; 1.748m.

- xiaomi (R1-2304897) Figure 4 Relationship between size of training data set and positioning accuracy for ToA-based prediction

- Ericsson (R1-2304339) Table 7 90%tile 2D positioning accuracy using PDP inputs for different model size classes and different training dataset sizes in the {60%, 6m, 2m} InF-DH scenario. Nt=Nt'=256. Model class; Positioning approach; Model complexity [# parameters]; Computational complexity [FLOPs]; 90%tile 2D positioning error [m] with different train set sizes in {60%, 6m, 2m} InF-DH. 80,000; 40,000; 20,000; 10,000. Small models PDP; Dist. Assist.; 0.43 M; 11.5 M; 0.531; 0.680; 0.902; 1.249. Cent. Assist.; 0.36 M; 9 M; 0.426; 0.524; 0.658; 0.873. Cent. Direct; 0.36 M; 9 M; 0.426; 0.510; 0.656; 0.863. Medium-size models PDP; Dist. Assist.; 1.69 M; 43 M; 0.351; 0.476; 0.675; 1.004. Cent. Assist.; 1.4 M; 34 M; 0.282; 0.360; 0.474; 0.707. Cent. Direct; 1.4 M; 34 M; 0.269; 0.349; 0.496; 0.735. Large models PDP; Dist. Assist.; 5.6 M; 140 M; 0.273; 0.403; 0.596; 0.933. Cent. Assist.; 5.6 M; 132 M; 0.202; 0.271; 0.397; 0.629. Cent. Direct; 5.6 M; 132 M; 0.214; 0.288; 0.425; 0.653.

## Model complexity

- Lenovo (R1-2305206) Observation 1: AI/ML models for positioning require a careful balance between performance and complexity depending on the type of positioning mode (UE-assisted or UE-based). Proposal 2: Await further discussions and progress on the definition “Nominal Computational Complexity” in AI 9.2.1 to establish a fair complexity comparison of performance evaluations among different companies

- CMCC (R1-2305089) Observation 5: As the complexity of the model increases, the positioning accuracy improves.

- Ericsson (R1-2304339) Proposal 1 For 3GPP AI/ML for PHY SI discussion, companies shall report nominal computational complexity values based on HLO representations (and not accelerator-optimized computational complexity values). Otherwise, the reported computation complexity value cannot be included for a fair cross-company comparison. Proposal 2 To further 3GPP discussion and preparation of observations/conclusions for the technical report, three model size classes are defined for the positioning use case as follows: - Small models: < 1 M model parameters - Medium-size models: 1 – 8 M model parameters - Large models: > 8 M model parameters

## 1st round discussion

Based on the evaluation results submitted by companies (e.g., see section 2.2), the following observations are drawn on training dataset size. Recall that this is a topic that RAN1 has agreed to study.

Observation 2.4-1

For AI/ML based positioning, based on evaluation results using uniform or grid UE distribution in training data collection, the positioning accuracy is affected by the training dataset size for a given UE distribution area (or equivalently, sample density in #samples/m2), when the UE is distributed uniformly in training data collection.

- There exists a tradeoff between the training dataset size and the achievable positioning accuracy. The larger the training dataset size (i.e., higher sample density), the smaller the positioning error (in meters), until a saturation point is reached where additional training data does not bring further improvement to the positioning accuracy.

- Note: here a sample refers to the training data collected of one UE at one location. Sample density is equivalent to the density of UEs with data collected in the training dataset.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Support |
| LG | Fine |
| mtk | okay |
| ZTE | Agree |
| OPPO | Fine |

Observation 2.4-2

For data collection of training dataset for AI/ML based positioning, for a given deployment scenario (e.g., InF-scenario, clutter parameter, drop) and with uniform UE distribution, the required sample density (e.g., #samples/m2) for achieving a given positioning accuracy target varies with AI/ML design choices including:

- different positioning approach (direct AI/ML, AI/ML-assisted),

- different type of model input (e.g., CIR, PDP, DP),

- the size of model input (e.g., Nt, N't, N'TRP),

- AI/ML model size (model complexity and computational complexity).

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | [Fine in principle from our own observation], but maybe some more presentation/discussion of the underlying results within the group would be helpful, so that we get all the same understanding of this observation. |
| ZTE | Our thinking is this observation should be based on the assumption that model generalization is not evaluated. We want to check if some further details will be summarized beyond this proposal. For example, the size of model input to achieve sub-meter level positioning accuracy. Is it any chance to incorporate detailed results from companies? |

For the topic of non-uniform UE distribution, Nokia (R1-2304685) and MediaTek (R1-2305659) suggest that RAN1 study the impact of non-uniform UE distribution. Moderator's understanding is, this refers to the non-uniform distribution of samples during training data collection. Essentially, this means the agreement below is to be expanded to include an Option 3 (non-uniform distribution). Furthermore, Nokia (R1-2304685) suggest to consider the inter-point distance metric (IPD) in addition to sample density (#samples/m2).

Agreement For evaluation of AI/ML based positioning, details of the training dataset generation are to be reported by proponent company. The report may include (in addition to other selected settings, if applicable): - The size of training dataset, for example, the total number of UEs in the evaluation area for generating training dataset; - The distribution of UE location for generating the training dataset may be one of the following: - Option 1: grid distribution, i.e., one training data is collected at the center of one small square grid, where, for example, the width of the square grid can be 0.25/0.5/1.0 m. - Option 2: uniform distribution, i.e., the UE location is randomly and uniformly distributed in the evaluation area.

Moderator's understanding is, the UE distribution for testing should follow uniform distribution (see " UE horizontal drop procedure" below) regardless of uniform or non-uniform training data collection.

| UE horizontal drop procedure | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - (baseline) the whole hall area, and the CDF values for positioning accuracy is obtained from whole hall area. - (optional) the convex hull of the horizontal BS deployment, and the CDF values for positioning accuracy is obtained from the convex hull. |
| --- | --- |

Thus, the following proposal is presented accordingly.

Proposal 2.4-3

For AI/M-based positioning, additionally evaluate non-uniform sample distribution in training data collection.

- For non-uniform sample distribution, companies report inter-point distance metric (IPD) in addition to sample density (#samples/m2)

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Should be optional |
| LG | Similar view with HW that it can be evaluated ‘optionally’ among the companies who are interested in |
| ZTE | It’s optional at this late stage. |
| OPPO | Agree with companies, this should be optional. |

# Model input

## Measurement size / signaling overhead for model input

## Qualcomm (R1-2305332) Table 1 Reporting/signalling overhead computation for different cases and measurements Case; Reporting; Reporting/signaling overhead (bits); Notes. Case1; UE location; 2*Bloc; 2D UE location (horizontal UE location). Case2a; Existing DL-TDoA signal measurements with first path (i.e., N’t =1); N’TRP * 1 * Bt; measurements include timing for first path (single port). Case2a/Case2b (existing or enhanced measurements); Existing DL-TDoA signal measurements with N’t path timing information (i.e., firth path and N’t – 1 additional paths); N’TRP * N’t * Bt; measurements include timing for N’t paths (single port). Case2b (new measurements); New CIR measurements; N’TRP * Nport * N’t * 2 * Breal,CIR + N’TRP * Nport * Nt; measurements include magnitude, phase, and timing for reported samples. Proposal 1: For the study on accuracy and reporting overhead in AI/ML positioning, companies evaluate accuracy, model complexity, and reporting overhead for existing measurements (e.g., timing of first and/or additional multipath measurement reporting) for LMF-sided models.

- InterDigital (R1-2305123) Table 10. Comparison between horizontal accuracy, measurement size and signaling overhead (direct AI/ML positioning) Model input; Measurement Size; 90% horizontal positioning accuracy. UE position (2D horizontal); 2; NA. RSRP (Per beam RSRP from multiple TRPs, 18 TRPs, 6 beams per TRP); 108; 3.35. Per beam RSRP from multiple TRPs (18 TRPs, 6 beams per TRP) and per TRP RSTD value (18 RSTD values); 126; 1.69. CIR (18 TRPs, 256 taps per TRP, Complex Number=2); 9216; 0.98. PDP (18 TRPs, 256 taps per TRP); 4608; 1.59. CIR (6 TRPs, 256 taps per TRP, Complex Number=2); 3072; 1.70. CIR (18 TRPs, 32 taps per TRP, Complex Number=2); 1152; 1.85. Proposal 2: For UE assisted AI/ML based positioning, study RSRP+RSTD measurement input as one of the options due to lower signalling overhead.

- vivo (R1-2304475) - Table 78 Contained information for different model input types - Model input types; - Contained multipath information related to positioning. - Delay; - Power; - Phase. - CIR; - Y; - Y; - Y. - PDP; - Y; - Y; - N. - DP; - Y; - N; - N. - RSRPP; - N; - Y; - N. Observation 57: The bit overhead of CIR reporting is acceptable when the number of TRPs used for positioning is relatively small, such as 4 TRPs.

- CATT (R1-2304726) In most of our simulations, for the evaluation of AI/ML based positioning method, CIR of all (NTRP = 18) TRPs with all (NT = 256) taps are used as the model input, and single port (Nport = 1) is assumed. Each sample is represented by double float type (8 Byte, i.e. 64 bits). Thus, the measurement size is NTRP*Nport*NT*2*64 = 294912bit per UE, unless otherwise stated, e.g. in Section Error! Reference source not found., Error! Reference source not found. and Error! Reference source not found.. We do not optimize the signaling overhead for AI/ML model at LMF side, and thus it can be viewed as the same with the measurement size. 3.1.6.2. Different CIR taps number as model input In this section, we evaluate the positioning performance and the computation complexity of the AI/ML model by reducing the number of CIR taps. In the simulation, only the first 128, 64, and 32 CIR taps from the 256 CIR taps in time domain are used as the model input respectively. So the values of measurement size are 147456bits per UE (NT=128), 73728bits per UE (NT=64), 36864bits per UE (NT=32).

- Ericsson (R1-2304339) - Table 10 Dataset storage sizes [bytes] for 40,000 samples with different sub-sampled length N't. Dataset storage sizes for 40,000 samples [MB] with different sub-sampled length N't. Input type; Nt'=256; Nt'=128; Nt'=64; Nt'=32; Nt'=16; Nt'=9. CIR; 2949.1; 1497.6; 760.32; 391.68; 207.36; 126.72. PDP; 737.3; 391.68; 207.36; 115.2; 69.12; 48.96. - Table 12 Dataset storage sizes [bytes] for 40,000 samples with different truncated time domain lengths Nt. Dataset storage sizes for 40,000 samples [MB] with different truncated time domain lengths Nt. Input type; Nt =256; Nt =128; Nt =64; Nt =32. CIR; 2949.1; 1474.6; 737.3; 368.6. PDP; 737.3; 368.6; 184.3; 92.2. DP; 23.0; 11.5; 5.8; -. Measurement size and signalling overhead for DP For the evaluation of AI/ML based positioning method, model input size for one sample of DP is calculated as the following. - If using bitmap to indicate the timing of the N't paths: (N'TRP * Nt) bits, where a length Nt bitmap is used to report DP for a link. - If directly indicating path timing of the N't paths: (N'TRP * Nt' * Bt ) bits, where Bt is the number bits to represent the timing value of a detected path. Measurement size and signalling overhead for CIR and PDP when not applying sub-sampling For the evaluation of AI/ML based positioning method, model input size for one sample of CIR or PDP is calculated as the following, if sub-sampling is not applied. - For CIR: (N'TRP * Nport * Nt * 2 * Breal,CIR ) bits, where Breal,CIR is the number of bits to represent a real value for CIR. - For PDP: (N'TRP * 1 * Nt * Breal,PDP ) bits, where Breal,PDP is the number of bits to represent a real value for path power. Measurement size and signalling overhead for CIR and PDP when sub-sampling is applied For the evaluation of AI/ML based positioning method, model input size for one sample of CIR or PDP is calculated as the following, if sub-sampling is applied. - If using bitmap to indicate the timing of the N't paths: For CIR: (N'TRP * Nt + N'TRP * Nport * N't * 2 * Breal,CIR) bits, where Breal,CIR is the number of bits to represent a real value for CIR. For PDP: (N'TRP * Nt + N'TRP * 1 * N't * Breal,PDP ) bits, where Breal,PDP is the number of bits to represent a real value for path power. - If directly indicating path timing of the N't paths: For CIR: (N'TRP * Nport * N't * (Bt + 2 * Breal,CIR )) bits, where Breal,CIR is the number of bits to represent a real value for CIR. For PDP: (N'TRP * 1 * N't * (Bt + Breal,PDP )) bits, where Breal,PDP is the number of bits to represent a real value for path power.

### 1st round discussion

As agreed in RAN1#112bis, several companies have reported the measurement size for their model input. When measurements for model input need to be sent to LMF (Case 2a/2b), this has impact to signaling overhead as well.

Agreement For the evaluation of AI/ML based positioning method, the measurement size and signalling overhead for the model input is reported.

Based on the analysis and proposals submitted by companies (see the cited contribution texts in section above), the following are recommended for discussion.

- Observation 3.2.1-1 is inspired by vivo (R1-2304475) description. As CIR/PDP/DP carries different amount of information about the radio link, correspondingly the measurement size and signalling overhead is different, regardless of how the signalling is defined in the end.

- Observation 3.2.1-2 is based on descriptions in Qualcomm (R1-2305332), InterDigital (R1-2305123), vivo (R1-2304475), CATT (R1-2304726), Ericsson (R1-2304339). Several parameter values directly and (approximately) linearly affect the measurement size.

Observation 3.1.1-1

For AI/ML based positioning with multipath measurement for model input,

- For a given set of parameters (N'TRP, Nt, N't, Nport)

  - CIR has the largest measurement size, where CIR is composed of a list of per-path measurements of (a) path delay, (b) path power and (c) path phase.

  - PDP has smaller measurement size than CIR, where PDP is composed of a list of per-path measurements of (a) path delay and (b) path power.

  - DP has the smallest measurement size, where DP is composed of a list of selected path delays.

    - Note: for DP, Nport =1

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | The observation does not seem to be needed in our understanding and at least some more clarifications could be helpful firstly: Does the measurement size also includes the #bits that are needed to represent the values? DP can be considered as a special case of PDP where 1bit is used for quantization. Thus, we do not think it is needed to explicitly mention DP, it is just one form of the PDP. And if quantization is not considered, wouldn’t then anyway have PDP and DP the same size, or am I missing something here? |
| LG | The motivation of this observation is unclear. It is based on the measurement size but the corresponding tradeoff is also to be considered in terms of the accuracy performance |
| mtk | 1, DP may be limited to timing based measurement. The angle based measurement method, such as DL-AOD and UL-AOA may not be applicable. So, this is a trade off. Maybe it should also be captured within the observation |
| ZTE | OK with the general conclusion. The note is not necessary. Nport =1 can be applied to all measurement types. |

For reference, the observation above is about measurement size and signalling overhead. The related tradeoff of positioning performance is captured in the observation below.

Observation 6.1.1.1-1 (CIR vs PDP vs DP) For direct AI/ML positioning, the positioning accuracy at model inference is affected by the type of model input. Evaluation results submitted to RAN1#113 show that if changing model input type while holding other parameters (e.g., Nt, N't, Nport, N'TRP) constant, - The positioning error of PDP as model input is 0.82 ~ 1.42 times the positioning error of CIR as model input. - The positioning error of DP as model input is 0.84 ~ 1.96 times the positioning error of CIR as model input. - Note: DP uses Nport=1 only.

Observation 3.1.1-2

For AI/ML based positioning with multipath measurement for model input,

- For each model input type (CIR, PDP, DP)

  - The measurement size increases linearly as N'TRP increases, where N'TRP is the number of TRPs that provide measurements for the positioning.

- For model input type CIR and PDP,

  - The measurement size increases linearly as Nport increases, where Nport is the number of transmit/receive antenna port pairs that provide measurements for the positioning.

  - If N’t (N’t < Nt) samples with the strongest power are selected as model input, measurement size for model input increases (approximately) linearly with N’t; otherwise (i.e., full set of Nt samples in time domain is used as model input), measurement size for model input increases linearly with Nt;

- Note: for Case 2b and 3b, measurement size of model input has impact to signaling overhead for model inference.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | This observation does not seem to be needed. It is already clear from the formula that that measurement size increases linearly. |
| LG | To our understanding, the main content of the observation on MTRP construction is to adopt N_t’ instead of N_t that the size/dimension of the model input is reduced with respect to N_t’ but the degradation of the accuracy performance is reasonable (i.e. good tradeoff between input signaling overhead and the accuracy performance) |
| mtk | 1, just think that single port may not be un-common for CIR and PDP, since DL-PRS is only single port from spec point of view 2, the motivation to have this observation is not quite clear |
| ZTE | Not necessary. Observation 3.1.1-1 is clear enough. |
| OPPO | This observation seems not needed. |

## TRP index for model input

- vivo (R1-2304475) Table 77 Evaluation results for AI/ML model deployed on UE or Network side, UE distribution area = [120x60 m] Model input; Model output; Selected TRP number; Total TRP number; TRP pattern; Approach; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Model complexity; Computation complexity; AI/ML. CIR; Pos.; 9; 18; Fixed 0:2:17; 1-A; 1.65M; 22.30M; 1.00. CIR; Pos.; 9; 18; Random; 1-B; 1.65M; 22.30M; 1.76. CIR; Pos.; 9; 18; Fixed 0:2:17; 2-A; 1.65M; 22.30M; 1.01. CIR; Pos.; 9; 18; Random; 2-B w/ TRP coding; 1.65M; 22.30M; 1.54. CIR; Pos.; 9; 18; Random; 2-B w/o TRP coding; 1.65M; 22.30M; >10. Proposal 35: Approach 2 should be divided into two branches in the same way as Approach 1, i.e., Approach 2-A and Approach 2-B. - Approach 2-A：The set of TRPs (N’TRP) that provide measurements is fixed. - Approach 2-B：The set of TRPs (N’TRP) that provide measurements can change dynamically. Observation 54: Approach 1-a and Approach 2-a with a fixed TRP pattern can achieve similar performance. Observation 55: Approach 1-b and Approach 2-b with random (dynamic) TRP patterns can still achieve great positioning performance (1.54m@90%), indicating that it is feasible to deploy a single AI/ML model to cover dynamic TRP patterns in practice. Observation 56: The performance of Approach 2-b is slightly better than that of Approach 1-b when TRP coding is included into the model input, while Approach 2-b suffers from poor positioning performance when TRP coding is not included into the model input. Proposal 36: The TRP information, which implicitly or explicitly indicates the TRP identification/index of measurement should be included into the model input for training of Approach 1-b and Approach 2-b with random (dynamic) TRP patterns.

### 1st round discussion

For the evaluation of TRP reduction, vivo suggested that Approach 2 can be updated to include 2-A and 2-B, so that the set of TRPs providing measurements can change dynamically, similar to Approach 1-B. For Approach 2-B, vivo suggest to develop a robust AI/ML model that can effectively handle various patterns of active TRPs, and the pattern can change dynamically. To achieve this, TRP indices are provided as an input to the AI/ML model.

To reflect vivo suggestion, Proposal 3.2-1 is presented below to include the Approach 2-B.

Proposal 3.2.1-1

The agreement made in RAN1#112bis is updated to the following to include Approach 2-B.

Agreement

For the evaluation of AI/ML based positioning, the study of model input due to different number of TRPs include the following approaches. Proponent of each approach provide analysis for model performance, signaling overhead (including training data collection and model inference), model complexity and computational complexity.

- Approach 1: Model input size stays constant as NTRP=18. The number of TRPs (N’TRP) that provide measurements to model input varies. When N’TRP < NTRP, the remaining (NTRP  N’TRP) TRPs do not provide measurements to model input, i.e., measurement value is set to 0 such that the (NTRP  N’TRP) TRPs do not affect model output.

  - Approach 1-A. The set of TRPs (N’TRP) that provide measurements is fixed.

  - Approach 1-B. The set of TRPs (N’TRP) that provide measurements can change dynamically.

  - Note: for Approach 1, one model is provided to cover the entire evaluation area.

- Approach 2: The TRP dimension of model input is equal to the number of TRPs (N’TRP) that provide measurements as model input. When N’TRP < NTRP, the remaining (NTRP  N’TRP) TRPs are ignored by the given model. For a given AI/ML model, the set of TRPs (N’TRP) that provide measurements is fixed.

  - Approach 2-A. The set of active TRPs (N’TRP) that provide measurements is fixed.

    - For Approach 2-A: one model can be provided to cover the entire evaluation area, which is equivalent to deploying N’TRP TRPs in the evaluation area for positioning if ignoring the potential inference from the remaining (18  N’TRP) TRPs.

  - Approach 2-B. The set of active TRPs (N’TRP) that provide measurements can change dynamically.

    - For Approach 2-B, one model is developed to handle various patterns of active TRPs.

  - For Approach 2, if Nmodel (Nmodel >1) models are provided to cover the entire evaluation area, the total complexity (model complexity is the summation of the Nmodel models.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Ok with the update. |
| LG | Fine |
| mtk | The change seems to become better to align between two approaches. We support |
| ZTE | Support. |

## Other

In the following, selected inputs from companies’ contributions are provided.

- CMCC (R1-2305089) Observation 1: If RSRP is taken as an additional model input to CIR, the positioning accuracy can be improved.

- China Telecom (R1-2304857) Observations 1: The positioning accuracy can be further improved when the measurement information is combined as the input of AI/ML model.

- Indian Institute of Tech (M), IIT Kanpur (R1-2305689) Observation 1: Combination of CIR and RSRP across different dimensions provides different positioning error.

- Samsung (R1-2305509) Fig.6 CDF of positioning errors with normalized or non-normalized CIR Observation 6: the normalized CIR may degrade the inference performance. Observation 9: SIG based input could adopt better (e.g., 7~8 times better in pos accuracy) with simple AI/ML model (e.g., MLP) than CIR based input. Observation 10: the SIG-based input could drastically reduce the input data size and the complexity (e.g., with 98% reduction) without accuracy loss (e.g., even with 1% improvement).

- vivo (R1-2304475) - Figure 4 CDF of positioning accuracy (m) of different measurements Observation 2: Different inputs of AI/ML model will affect the positioning performance for AI/ML based positioning. Time domain channel CIR as the input of AI/ML model obtains the best positioning accuracy. Proposal 1: Capture in the TR that time domain CIR as the model input for direct AI/ML positioning obtains the best performance compared to other model inputs. Proposal 2: Support time domain CIR as the model input at least for direct AI/ML positioning.

- Google (R1-2304846) Proposal 1: For CIR/PDP based model input, study the impact from the following Rx schemes - Rx Scheme 1: The CIR/PDP is measured from the Rx port with the strongest RSRP - Rx Scheme 2: The CIR/PDP is averaged over all the Rx ports Proposal 2: For CIR/PDP based model input, study at least the following options for CIR/PDP quantization: - Option 1: The CIR/PDP is quantized based on several DFT bases - Option 2: The CIR/PDP is quantized based on several DCT bases Proposal 3: Study to use L1-SINR from each cell in addition to the CIR/PDP as the input to reflect the potential channel estimation accuracy for the CIR/PDP.

- Ericsson (R1-2304339) Observation 9 When compared to the averaged PDP input type, the 2-port PDP input type (1) doubles the dataset sizes; (2) requires higher computational complexity; and (3) achieves marginal performance improvements. - Proposal 4 For AI/ML based positioning, do not support multi-port PDP as model input. Single-port PDP as in existing specification is sufficient.

# Model output

## Hard decision and soft decision output

- Qualcomm (R1-2305332) Observation 21: In AI/ML assisted positioning, evaluations show that the ML-based soft-decision algorithm outperforms the hard-decision approach for AI-ML-assisted positioning. - The 90th percentile positioning error improves from 25.0 m to 4.7 m for the {60%, 6m, 2m} clutter setting - The 90th percentile positioning error improves from 14.8 m to 0.5 m for the {40%, 4m, 2m} setting.

## Label error

- CEWiT (R1-2305896) Proposal-5: For AI/ML assisted positioning with AOA/AOD as model output, study the im-pact of the labelling error to Angle estimation accuracy and /or positioning accuracy con-sidering the location error to be modeled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range.

# Evaluation of generalization issues of direct AI/ML positioning

In this meeting, a large amount of evaluation work has been performed by companies for direct AI/ML positioning. These valuable results are very important to help RAN1 to make progress.

Selected results submitted by companies are copied below.

## Evaluation without generalization considerations (same setting for training and testing)

- MediaTek Inc. (R1-2305659) Table 21. Evaluation results for AI/ML model deployed on UE or network-side without generalization, CNN, UE distribution area = [120x60 m] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. 1-port CIR [18,1,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 202.10K; 73.18M; 1.32. CIR [18,2,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 464.24K; 0.266G; 0.940. PDP [18,2,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 463.95K; 0.264G; 0.821. Observation 26: Direct AI positioning can significantly improve the positioning performance in heavy-NLOS scenarios compared to conventional methods. Observation 27: The evaluation results shown that the positioning performance of 2 transmit antenna ports (by different polarization) is better than the existing 1 antenna port in the spec. Proposal 8 :At least support PDP as model input for direct AI/ML positioning, and further study CIR to check whether the phase part in CIR is useful. Proposal 9 : Study and evaluate the performance of direct AI/ML positioning with multiple transmit/receive antenna port pairs (for example, 2 ports with different polarization).

- InterDigital (R1-2305123) Table 1. Evaluation results for AI/ML model deployed on UE-side, different model inputs, without model generalization, UE distribution area = [120x60 m] Model input; Model output; (Percentage of training data set without) Label; Clutter param; Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Model complexity; Comp. complexity; AI/ML. RSRP; UE position; 0% (default); 60%, 6m, 2m; 16000; 4000; 332k; 11.37 M; 3.35. RSRP +RSTD; UE position; 0% (default); 60%, 6m, 2m; 16000; 4000; 334k; 11.41 M; 1.69. CIR (NTRP =18* Nt = 256*Complex Number=2); UE position; 0% (default); 60%, 6m, 2m; 16000; 4000; 37 M; 843 M; 0.98. PDP (NTRP =18* Nt = 256*Real Number=1); UE position; 0% (default); 60%, 6m, 2m; 16000; 4000; 37 M; 839 M; 1.59. Observation 1: Direct AI/ML positioning technique based on PDP measurements as model input, achieves ~1.59 m horizontal positioning accuracy for 90% UEs. Observation 2: Direct AI/ML positioning technique based on PDP measurements as model input results ~0.61 m worse horizontal accuracy than CIR measurements as model input for 90%ile UEs. Observation 3: Direct AI/ML positioning technique based on PDP measurements as model input achieves ~0.1 m better horizontal accuracy than RSRP+RSTD measurements with significantly higher model complexity (~112 times) and computational complexity(~76 times) compared to RSRP+RSTD measurements as model input.

- Apple (R1-2305973) Observation 1: For Direct AI/ML positioning - As the complexity of the model increases, the positioning error performance improves. - The performance of AI/ML positioning with the evaluation area as the convex hull of the horizontal BS deployment shows better performance than that with the entire factory as evaluation area. - Use of the PDP and DP as input shows improved performance compared with using the CIR as input for the default model - NOTE: This may be model specific and should be investigated. - In the case of a uniform distribution of the UEs in the factory, as the user density increase, the positioning performance improves. Proposal 1: Investigate the effect with a non-uniform distribution of users.

- China Telecom (R1-2304857) Observations 1: The positioning accuracy can be further improved when the measurement information is combined as the input of AI/ML model. Table 2: Positioning accuracy for different schemes Methods; 50%; 67%; 80%; 90%. Traditional method; 11.89; 13.62; 14.78; 16.36. AI + TOA; 0.37; 0.46; 0.57; 0.69. AI + DL-TDOA; 0.38; 0.49; 0.59; 0.73. AI + CIR; 0.27; 0.36; 0.43; 0.54. AI + RSRP +TOA; 0.22; 0.29; 0.36; 0.43. AI + RSRP + DL-TDOA; 0.19; 0.25; 0.31; 0.38.

- CAICT (R1-2305019) Observation 1: The horizontal positioning accuracy of direct AI/ML positioning with complex model (CNN+ CR_CBAM_Block) could be 0.046m at CDF=90% when all samples in training dataset have ground truth label without model generalization. Observation 2: For AI/ML based directly positioning method, a lower complexity (CNN based) model can still achieve acceptable positioning accuracy (e.g., <1m), when compared to a higher complexity model. Observation 3: The horizontal positioning accuracy of direct AI/ML positioning could be 0.493m at CDF=90% when partial samples (1000 samples out of 150000 samples) in training dataset have ground truth label and semi-supervised learning is used.

- Indian Institute of Tech (M), IIT Kanpur (R1-2305689) Table 1: Horizontal Position Error (m) and Model Complexity for 3 cases of combining CIR and RSRP Approach; 10 %; 50 %; 90%; Model Complexity. Case 1; 1.15; 2.89; 5.39; 2.66M parameters. Case 2; 1.09; 2.92; 5.45; 2.66M parameters. Case 3; 1.07; 2.63; 5.04; 1.48M parameters.

- Huawei (R1-2304657) Observation 7 : For direct AI/ML positioning, when the model input type is CIR, the positioning accuracy for the scenario setting {60%, 6m, 2m} decreases by 0.22 meters when the number of input time domain points is reduced from 256 to 32. While the positioning accuracy for the scenario setting {40%, 2m, 2m} decreases by 0.64 meters when the number of input time domain points is reduced from 256 to 32. Observation 8 : For direct AI/ML positioning, when the model input type is PDP, the positioning accuracy for the scenario setting {60%, 6m, 2m} decreases by 0.08 meters when the number of input time domain points is reduced from 256 to 32. While the positioning accuracy for the scenario setting {40%, 2m, 2m} decreases by 0.64 meters when the number of input time domain points is reduced from 256 to 32. Proposal 1 : At least for direct positioning, since the required measurement payload size to achieve a given accuracy target varies depending on deployment scenario and channel conditions, measurement reporting with flexible payload size should be supported.

- CATT (R1-2304726) Observation 1: For directly estimating UE’s positioning with perfect network synchronization, the horizontal positioning accuracy is 0.58m@90% of CDF percentile. Observation 2: For directly estimating UE’s positioning with network synchronization error (truncated Gaussian distribution of 50 ns), the horizontal positioning accuracy is 0.84m@90% of CDF percentile.

## Evaluation of generalization aspects (different setting for training and testing)

### Different drops

- NVIDIA (R1-2305164) Observation 3: RF fingerprinting is site specific, i.e., the AI/ML model learns the mapping between channel realizations and the corresponding UE’s positions. Observation 4: The AI/ML model for RF fingerprinting trained on Drop 1 does not generalize well to a different Drop 2. Proposal 1: If the AI/ML model for RF fingerprinting trained on Drop 1 is directly applied to a different Drop 2, it should not be expected that the model can generalize well.

- ZTE (R1-2304538) Observation 9: For model generalization evaluation in various simulation drops, when datasets for training and test are from different simulation drops, direct AI/ML positioning based on CIR cannot generalize its performance in the test dataset. Observation 10: For model generalization evaluation in various simulation drops, when model update (or fine-tuned) is performed based on dataset in a simulation drop that is different from the simulation drop used for model training, • Positioning performance of direct AI/ML positioning based on CIR can be improved in a test dataset that is in the same simulation drop as the dataset for model update; • Positioning performance of direct AI/ML positioning based on CIR is increased along with the number of data samples used for model update. Observation 11: For model generalization evaluation in various simulation drops, when training dataset includes mixed data from two simulation drops, direct AI/ML positioning based on CIR has a good model generalization capability in a test dataset from a simulation drop included in the mixed simulation drops.

- Huawei (R1-2304657) Observation 9 : When the inference dataset and the training dataset are from different drops, direct AI/ML positioning model provides poor generalization performance. But when the mixed training dataset consists of samples from the same drop as the inference dataset, the generalization performance is improved. Observation 10 : For direct AI/ML positioning, enriching the composition of the mixed training dataset can improve unseen drop’s positioning accuracy. Observation 11 : For direct AI/ML positioning, the positioning performance for a seen drop improves when the amount/ratio of data samples from that drop in the mixed training dataset increases.

#### Model fine-tuning for different drops

- MediaTek Inc. (R1-2305659) Table 23. Evaluation results for AI/ML model deployed on UE or network-side with generalization, CNN, UE distribution area = [120x60 m], fine-tuning for different drop Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. 1-port CIR [18,1,256]; UE pos [x,y]; 0%; Drop1; Drop2; Drop2; 32400; 0; 3600; 202.10K; 73.18M; 9.15. 1200; 3.05. 2400; 2.52. 3600; 2.25. CIR [18,2,256]; UE pos [x,y]; 0%; Drop1; Drop2; Drop2; 32400; 0; 3600; 464.24K; 0.266G; 9.36. 1200; 3.08. 2400; 2.54. 3600; 2.17. PDP [18,2,256]; UE pos [x,y]; 0%; Drop1; Drop2; Drop2; 32400; 0; 3600; 463.95K; 0.264G; 8.57. 1200; 3.23. 2400; 3.01. 3600; 2.37. Observation 30: Fine-tuning the model with samples from a drop can achieve positioning accuracy improvement when the pre-trained model is transferred to a new drop for direct AI/ML positioning.

- Qualcomm (R1-2305332) Table 12 Evaluation results for AI/ML model deployed on UE-side, with model finetuning generalization (Type 2 – different drops), CNN, UE distribution area = 120x60 m Model input; Model output; Label; Settings ({60%, 6m, 2m}); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). train; finetune; Test; Train; Finetune; Test; Model complexity [parameters]; Computational complexity FLOPs; AI/ML. CIR (18,4,400); 2D; 0%; drop A (with UE clock drift); --; drop B (with UE clock drift); 15k; 0; 2K; 1.5M; 1.54G; 12.33. CIR (18,4,400); 2D; 0%; drop A (with UE clock drift); drop B (with UE clock drift); drop B (with UE clock drift); 15k; 100; 2K; 1.5M; 1.54G; 10.47. CIR (18,4,400); 2D; 0%; drop A (with UE clock drift); drop B (with UE clock drift); drop B (with UE clock drift); 15k; 240; 2K; 1.5M; 1.54G; 6.92. CIR (18,4,400); 2D; 0%; drop A (with UE clock drift); drop B (with UE clock drift); drop B (with UE clock drift); 15k; 500; 2K; 1.5M; 1.54G; 6.07. Observation 1: Positioning enhancement gains of AI/ML model fine-tuning depends on the size of fine-tuning data.

- CMCC (R1-2305089) Table IV. Evaluation results for AI/ML model deployed on UE side, CNN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; Test; Model complexity; Computation complexity; AI/ML. CIR size:18*1*256; UE coordinates; ideal UE coordinates; {60%, 6m, 2m}, Drop1; {60%, 6m, 2m}, Drop 2; {60%, 6m, 2m}, Drop 2; 25000; 500; 2500; 3.71M; 7.42M; 4.14. 1000; 3.35. 2000; 2.74. 3000; 1.95. Observation 4: Model updating with a small amount of fine-tuning data can obviously improve the generalization performance of different drops.

- NVIDIA (R1-2305164) Table 4: Summary of CDF percentiles of horizontal positioning accuracy with model finetuning. Training; Testing; Finetuning; 50%; 67%; 80%; 90%. Drop 1; Drop 1; N/A; 1.1 m; 1.5 m; 1.8 m; 2.3 m. Drop 1; Drop 2; No finetuning; 7.1 m; 9.3 m; 11.6 m; 14.5 m. Drop 1; Drop 2; Finetuning with 1k samples; 2.5 m; 3.3 m; 4.2 m; 5.3 m. Drop 1; Drop 2; Finetuning with 2k samples; 2.1 m; 2.7 m; 3.5 m; 4.3 m. Note: The original model was trained with 16k samples. Thus, 1k (resp. 2k) finetuning samples corresponds to 6.25% (resp. 12.5%) of the total 16k samples.. Observation 5: When the AI/ML model for RF fingerprinting trained on Drop 1 is finetuned with a small number of samples from Drop 2, the positioning accuracy of the finetuned AI/ML model is much improved compared to the performance of the AI/ML model without finetuning. Proposal 2: When an AI/ML model for RF fingerprinting trained in a first scenario is transferred to a second scenario, the AI/ML model should be finetuned/retrained with new data from the second scenario.

- Huawei (R1-2304657) Observation 21 : From the model updating evaluation results of the above aspect - Different Drops, on top of the dataset with large amount of samples from a different drop, fine-tuned with a relatively small amount of samples from the same drop as the inference dataset will be helpful to improve the generalization performance.

#### Mixed dataset for different drops

- MediaTek Inc. (R1-2305659) Table 22. Evaluation results for AI/ML model deployed on UE or network-side with generalization, CNN, UE distribution area = [120x60 m], mixed training for different drop Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. 1-port CIR [18,1,256]; UE pos [x,y]; 0%; Drop1; Drop2; 32400; 3600; 202.10K; 73.18M; 9.15. Drop1+Drop2; Drop2; 16200+16200; 3600; 3.63. CIR [18,2,256]; UE pos [x,y]; 0%; Drop1; Drop2; 32400; 3600; 464.24K; 0.266G; 9.36. Drop1+Drop2; Drop2; 16200+16200; 3600; 1.52. PDP [18,2,256]; UE pos [x,y]; 0%; Drop1; Drop2; 32400; 3600; 463.95K; 0.264G; 8.57. Drop1+Drop2; Drop2; 16200+16200; 3600; 1.72. Observation 28: Performance of direct AI/ML positioning degrades when the model trained with dataset of one drop is tested with dataset of other drops since there is no correlation between multipath realization of different drops. Observation 29: Training the AI model with mixed dataset can be an effective way to improve the performance of direct AI positioning when the model is deployed at different drops.

- Ericsson (R1-2304339) Table 38 90%tile 2D positioning accuracy using CIR inputs for different environmental random seeds in the {60%, 6m, 2m} InF-DH scenario for small model (Model I) trained with different datasets. Model details; Positioning approach; 90%tile 2D positioning error [m] with different train set sizes in {60%, 6m, 2m} InF-DH. Drop 1; Drop 2. CIR trained with drop 1 dataset; Cent. Assist.; 0.371; 17.332. Cent. Direct; 0.373; 16.802. CIR trained with drop 1 and drop 2 datasets; Cent. Assist.; 0.435; 0.446. Cent. Direct; 0.437; 0.439. - Table 40 90%tile 2D positioning accuracy using DP inputs for different environmental random seeds in the {60%, 6m, 2m} InF-DH scenario for small model (Model I) trained with different datasets. Model details; Positioning approach; 90%tile 2D positioning error [m] with different train set sizes in {60%, 6m, 2m} InF-DH. Drop 1; Drop 2. DP trained with drop 1 dataset; Cent. Assist.; 0.653; 21.488. Cent. Direct; 0.658; 20.374. DP trained with drop 1 and drop 2 datasets; Cent. Assist.; 0.826; 0.811. Cent. Direct; 0.784; 0.807.

### Different clutter parameters

- ZTE (R1-2304538) Observation 12: For model generalization evaluation in various clutter settings, when datasets for training and test are from different clutter settings, direct AI/ML positioning based on CIR cannot achieve cannot generalize its performance in the test dataset. Observation 13: For model generalization evaluation in various clutter settings, when model update is performed based on dataset in a clutter setting that is different from the clutter setting used for model training, • Positioning performance of direct AI/ML positioning based on CIR can be improved in a test dataset that is in the same clutter setting as the dataset for model update; • Positioning performance of direct AI/ML positioning based on CIR is increased along with the number of data samples used for model update. Observation 14: For model generalization evaluation in various clutter settings, when a dataset for training includes data from mixed clutter settings, direct AI/ML positioning based on CIR has a good model generalization capability in a test dataset from a simulation drop included in the mixed clutter settings.

- Huawei (R1-2304657) Observation 12 : For direct AI/ML positioning, when the channel parameters of the inference dataset and the training dataset are different, direct AI/ML positioning model provides poor generalization performance. Observation 13 : For direct AI/ML positioning, when the mixed training dataset consists of samples with the same channel parameters as the inference dataset, the positioning performance is improved and reaches the sub-meter level.

#### Model fine-tuning for different clutter parameters

- MediaTek Inc. (R1-2305659) Table 25. Evaluation results for AI/ML model deployed on UE or network-side with generalization, CNN, UE distribution area = [120x60 m], fine-tuning for different clutter setting Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. 1port-CIR [18,1,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 0; 3600; 202.10K; 73.18M; 9.10. 1200; 4.49. 2400; 3.86. 3600; 3.50. CIR [18,2,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 0; 3600; 464.24K; 0.266G; 7.43. 1200; 3.80. 2400; 3.42. 3600; 3.09. PDP [18,2,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 0; 3600; 463.95K; 0.264G; 7.72. 1200; 3.74. 2400; 3.38. 3600; 3.10. Observation 33: Fine-tuning a model with samples of clutter setting ({40%,2m,2m}) can significantly improve the performance of direct AI/ML positioning when the model pre-trained in ({60%,6m,2m}) is transferred to clutter setting({40%,2m,2m}).

- NVIDIA (R1-2305164) Figure 8: Positioning accuracy of AI/ML based method under different drops. Observation 7: When the AI/ML model for RF fingerprinting trained on clutter parameter set 1 is finetuned with a small number of samples from clutter parameter set 2, the positioning accuracy of the finetuned AI/ML model is much improved compared to the performance of the AI/ML model without finetuning. Observation 8: Compared to finetuning for different drops, finetuning for different clutter parameter sets provides less positioning accuracy improvement with the same finetuning samples. This shows that finetuning is more effective when the first scenario (for which the AI/ML model is originally trained) and the second scenario (that finetuning targets) are more similar.

- xiaomi (R1-2304897) Observation 5: - For fine-tuning with different clutter parameters o The positioning accuracy is improved compared with the situation of different clutter parameter between training data set and test data set o The improvement of positioning accuracy in the fine-tuning solution is less than that in the solution of mixed training data set with different clutter parameter

- Huawei (R1-2304657) Observation 22 : From the model updating evaluation results of the above aspect - Clutter parameters, on top of the dataset with large amount of samples from a different clutter setting, fine-tuned with a relatively small amount of samples from the same clutter setting as the inference dataset will be helpful to improve the generalization performance.

- CATT (R1-2304726) Observation 7: For direct AI/ML positioning, by training the AI/ML model using the dataset that assumes a clutter parameter of {60%, 6m, 2m}, and subsequently fine-tuning it using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

#### Mixed dataset for different clutter parameters

- MediaTek Inc. (R1-2305659) Table 24. Evaluation results for AI/ML model deployed on UE or network-side with generalization, CNN, UE distribution area = [120x60 m], mixed trainig for different clutter setting Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. 1port-CIR [18,1,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {40%, 2m, 2m}; 32400; 3600; 202.10K; 73.18M; 9.10. {60%, 6m, 2m}+{40%, 2m, 2m}; {40%, 2m, 2m}; 16200+16200; 3600; 5.49. CIR [18,2,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {40%, 2m, 2m}; 32400; 3600; 464.24K; 0.266G; 7.43. {60%, 6m, 2m}+{40%, 2m, 2m}; {40%, 2m, 2m}; 16200+16200; 3600; 2.30. PDP [18,2,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {40%, 2m, 2m}; 32400; 3600; 463.95K; 0.264G; 7.72. {60%, 6m, 2m}+{40%, 2m, 2m}; {40%, 2m, 2m}; 16200+16200; 3600; 2.43. Observation 31: Performance of direct AI/ML positioning degrades when the model trained with clutter setting ({60%,6m,2m}) is tested with dataset of another clutter setting ({40%,2m,2m}). Observation 32: Training the AI model with mixed dataset can be an effective way to improve the performance of direct AI positioning when the model is deployed at different clutter settings.

- xiaomi (R1-2304897) Observation 4: - Generating the training data set with different cluster parameters could relax the problem of inferior generalization capability

- CATT (R1-2304726) Observation 4: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of clutter parameter {60%, 6m, 2m} with a small dataset of clutter parameter {40%, 2m, 2m}, the AI/ML model achieved a horizontal positioning accuracy of 1.77m, which represents an improvement compared to the performance achieved without mix-training and with a positioning accuracy of 2.64m.

- vivo (R1-2304475) Observation 9: Positioning performance of direct AI/ML positioning degrades when the training and testing datasets are of different clutter parameters in an InF-DH scenario. Observation 10: Training AI/ML model with a mixed dataset is an effective way to improve model generalization performance. Proposal 5: Capture in the TR the benefits of training dataset with mixed/different configurations for AI/ML based positioning in terms of AI model generalization capability.

- Ericsson (R1-2304339) Table 33. 90%tile UE 2D positioning errors for small Model I trained with 40,000 PDP samples from {60%, 6m, 2m} and/or {40%, 2m, 2m} and tested on four different environments. Positioning approach; Model complexity [# of parameters]; Training dataset; 90%tile UE 2D positioning errors [m] – small Model I on different test sets. {40%, 2m, 2m}; {60%, 2m, 2m}; {40%, 6m, 2m}; {60%, 6m, 2m}. Dist. Assisted; 0.43 M; {60%, 6m, 2m}; 8.176; 4.134; 0.728; 0.684. {40%, 2m, 2m}; 0.596; 0.891; 2.555; 2.841. {40%, 2m, 2m} {60%, 6m, 2m}; 0.601; 0.701; 0.792; 0.784. Cent. Assisted; 0.36 M; {60%, 6m, 2m}; 6.904; 2.453; 0.549; 0.524. {40%, 2m, 2m}; 0.854; 0.900; 1.408; 1.518. {40%, 2m, 2m} {60%, 6m, 2m}; 0.816; 0.707; 0.592; 0.586. Cent. Direct; 0.36 M; {60%, 6m, 2m}; 6.542; 2.463; 0.544; 0.510. {40%, 2m, 2m}; 0.810; 0.883; 1.413; 1.521. {40%, 2m, 2m} {60%, 6m, 2m}; 0.807; 0.701; 0.587; 0.579.

### Network synchronization error

- ZTE (R1-2304538) Observation 15: For model generalization evaluation in various network synchronization errors, when a dataset for training has ideal network synchronization errors and a dataset for test has network synchronization errors following a truncated Gaussian distribution between [-2T1, 2T1] (T1 is a rms value), the positioning performance degrades seriously compared to ideal network synchronization. Observation 16: For model generalization evaluation in various network synchronization errors, when both training dataset and test dataset have network synchronization errors following a truncated Gaussian distribution between [-2T1, 2T1] (T1 is a rms value), positioning performance is comparable to ideal network synchronization. This may be explained by: • CNN model is translation-invariant to the time shift of channel information due to network synchronization error; • Add random network synchronization errors on training dataset are analogous to data augmentation on the training dataset, thus increase the model robustness to various network synchronization errors.

- xiaomi (R1-2304897) Observation 6: - If the AI/ML model is trained with data set of ideal network synchronization and the tested by data set is with network synchronization error, poor generalization performance is observed

- Huawei (R1-2304657) Observation 14 : For direct AI/ML positioning, when the model is trained without network synchronization error but inferred with the added network synchronization error randomly distributed with the standard deviation value T_1 = 50ns, direct AI/ML positioning model provides poor generalization performance. Observation 15 : For direct AI/ML positioning, when the model is both trained and inferred with the added network synchronization error randomly distributed with the standard deviation value T_1 = 50ns, the positioning performance is improved compared with trained without error. Observation 16 : For direct AI/ML positioning, when the model is both trained and inferred with the added network synchronization error randomly distributed with mixed T_1 = 0&30&40&50ns, the positioning performance is improved compared with trained without error. The larger synchronization error the network have, the poorer positioning performance they will have.

- Ericsson (R1-2304339)

#### Model fine-tuning for network synchronization error

- MediaTek Inc. (R1-2305659) Table 27. Evaluation results for AI/ML model deployed on UE or network-side with generalization, CNN, UE distribution area = [120x60 m], fine-tuning for timing error Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML 10ns; AI/ML 50ns. CIR [18,2,256]; UE pos [x,y]; 0%; 50ns; 10ns; 10ns/50ns; 32400; 0; 3600; 464.24K; 0.266G; 2.714; 3.275. 1200; 2.124; 3.496. 3600; 2.098; 4.381. 7200; 1.869; 4.571. PDP [18,2,256]; UE pos [x,y]; 0%; 50ns; 10ns; 10ns/50ns; 32400; 0; 3600; 463.95K; 0.264G; 2.705; 3.366. 1200; 2.361; 4.055. 3600; 2.154; 4.223. 7200; 1.910; 4.594.

- xiaomi (R1-2304897) Observation 8: - From the aspect that AI model is trained by data set with ideal network synchronization error and fine-tuned by data set with 50ns network synchronization error, improvement in the positioning accuracy is observed. But the improved positioning accuracy is still poor

- Huawei (R1-2304657) Observation 23 : From the model updating evaluation results of the above aspect - Network synchronization error, on top of the dataset with large amount of samples without network synchronization error, fine-tuned with a relatively small amount of samples with the same added network synchronization error randomly distributed as the inference dataset will be helpful to improve the generalization performance but the performance is still poor. Fine-tuning helps less in solving the network synchronization error.

- CATT (R1-2304726) Observation 8: For direct AI/ML positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

#### Mixed dataset for network synchronization error

- MediaTek Inc. (R1-2305659) Observation 35: The positioning performance of direct AI/ML can be improved by mixing dataset with different timing errors at the cost of the training complexity.

- xiaomi (R1-2304897) Observation 7: - Generating the training data set with mixed network synchronization error could relax the problem of inferior generalization capability

- CATT (R1-2304726) Observation 5: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of ideal network synchronization with a small dataset of network synchronization error of 50ns, the AI/ML model achieved a horizontal positioning accuracy of 3.04m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 12.6m.

- vivo (R1-2304475) Figure 38 Evaluation of the impact of synchronization error on positioning accuracy Observation 19: The positioning accuracy of direct AI/ML positioning significantly degrades with the increase of network synchronization error. Observation 20: The positioning accuracy of AI/ML model is significantly improved from 10.18m@90% to 1.52m@90% by mix-training with samples of synchronization error.

### UE/gNB RX and TX timing error

| - ZTE (R1-2304538) Observation 17: For model generalization evaluation in various UE Rx timing errors, the positioning performance in a test dataset with UE Rx timing errors degrades slightly compared to the dateset without UE Rx timing errors. Observation 18: For model generalization evaluation in various UE Rx timing errors, when both training dataset and test dataset have UE Rx timing errors following a truncated Gaussian distribution between [-2T1, 2T1] (T1 is a rms value), positioning performance is comparable to the case without UE Rx timing error. This may be explained by: • CNN model is translation-invariant to the time shift of channel information due to UE Rx timing error; • Add random UE Rx timing errors on training dataset are analogous to data augmentation on the training dataset, thus increase the model robustness to various UE Rx timing errors. |
| --- |
| - xiaomi (R1-2304897) Observation 9: - If the AI/ML model is trained with data set of ideal Rx timing and the tested by data set is with Rx timing error, there is some positioning accuracy degradation Observation 10: - Generating the training data set with mixed Rx timing error could relax the problem of inferior generalization capability Observation 11: - From the aspect that AI model is trained by data set with ideal timing error and fine-tuned by data set with 10ns Rx timing error, improvement in the positioning accuracy is observed. But the improved positioning accuracy is still poor |  |
| - Huawei (R1-2304657) Observation 17 : For direct AI/ML positioning, when the model is trained without UE timing error but inferred with the added UE timing error randomly distributed with the standard deviation value T_1 = 10ns, the direct AI/ML positioning model provides poor generalization performance. Observation 18 : For direct AI/ML positioning, when the model is both trained and inferred with the added UE timing error randomly distributed with T_1 = 10ns, the positioning performance is improved compared with trained without error. Observation 19 : For direct AI/ML positioning, when the model is both trained and inferred with the added UE timing error randomly distributed with mixed T_1 = 0&10&20&30ns, the positioning performance is improved compared to when trained without error. |  |
| - Ericsson (R1-2304339) |  |

#### Model fine-tuning for UE/gNB RX and TX timing error

- Huawei (R1-2304657) Observation 24 : From the model updating evaluation results of the above aspect - UE timing error, on top of the dataset with large amount of samples without UE timing error, fine-tuned with a relatively small amount of samples with the same added UE timing error randomly distributed as the inference dataset will be helpful a lot to improve the generalization performance. Proposal 2 : Model updating should be supported to improve the performance under the presence of UE timing errors and for the occurrence of the unlearned channel characteristics, including unseen drops and clutter settings.

#### Mixed dataset for UE/gNB RX and TX timing error

- vivo (R1-2304475) Observation 24: The positioning accuracy of AI/ML model is significantly improved from 3.45m@90% to 2.04m@90% by mix-training with samples of UE timing error for direct AI/ML positioning.

### Different InF scenarios

- MediaTek Inc. (R1-2305659) Table 30. Evaluation results for AI/ML model deployed on UE or network-side with generalization, CNN, InF-DH area = [120x60 m], InF-SH/HH area == [300x150 m] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML InF-SH; AI/ML InF-HH. CIR [18,2,256]; UE pos [x,y]; 0%; InF-DH({60%, 6m, 2m}),small hall; InF-SH/HH, large hall; InF-SH/HH, large hall; 32400; 0; 3600; 464.24K; 0.266G; >100; >100. 1200; 9.806; 14.073. 3600; 5.391; 5.972. 7200; 3.815; 5.104. PDP [18,2,256]; UE pos [x,y]; 0%; InF-DH({60%, 6m, 2m}); InF-SH/HH, large hall; InF-SH/HH, large hall; 32400; 0; 3600; 463.95K; 0.264G; >100; .>100. 1200; 8.964; 21.755. 3600; 5.186; 14.537. 7200; 4.162; 9.581. Observation 40: Performance of direct AI/ML positioning degrades when the model trained with dataset of InF-DH is tested with dataset of InF-SH/HH with different hall size, and fine-tuning can improve the performance.

- vivo (R1-2304475) CDF of positioning accuracy when training dataset and test dataset are not matched Observation 11: The positioning accuracy of direct AI/ML positioning trained with dataset from one InF scenario is seriously degraded when tested on dataset from a different InF scenario.

#### Model fine-tuning for different InF scenarios

- CATT (R1-2304726) Observation 9: For direct AI/ML positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

#### Mixed dataset for different InF scenarios

- CATT (R1-2304726) Observation 6: For direct AI/ML positioning, by employing the mix-train method and combining the dataset of InF-DH {60%, 6m, 2m} with a small dataset of InF-SH {20%, 2m, 10m}, the AI/ML model achieved a horizontal positioning accuracy of 1.81m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 6.48m.

### SNR mismatch

- Apple (R1-2305973) Observation 2 - Synchronization and SNR levels have the largest negative effect on the generalization performance of the direct AI-ML model - An SNR mismatch between the training data and inference input shows increasing performance loss with increasing mismatch as expected. - Training with a lower SNR data set seems to provide some benefits in the case there is an SNR mismatch between the training and test data.

- Ericsson (R1-2304339) Table 35 90%tile 2D positioning accuracy using CIR inputs for different UE transmit powers in the {60%, 6m, 2m} InF-DH scenario for small model (Model I) trained with different UE transmit powers. Model details; Positioning approach; 90%tile 2D positioning error [m] with different UE transmit powers in {60%, 6m, 2m} InF-DH. 23 dBm UE; 15.5 dBm UE; 8.5 dBm UE; 0.5 dBm UE; -7 dBm UE. CIR trained with 23 dBm UE; Cent. Assist.; 0.368; 0.642; 2.284; 6.200; 53.119. Cent. Direct; 0.373; 0.614; 2.029; 5.520; 29.281. CIR trained with -7 dBm UE; Cent. Assist.; 7.721; 7.297; 4.990; 1.236; 0.513. Cent. Direct; 7.441; 7.165; 5.401; 1.324; 0.532. CIR trained with 23, -7 dBm UE; Cent. Assist.; 0.467; 0.464; 0.467; 0.475; 0.507. Cent. Direct; 0.446; 0.449; 0.446; 0.456; 0.487. CIR trained with 23, 8, -7 dBm UE; Cent. Assist.; 0.419; 0.414; 0.421; 0.436; 0.505. Cent. Direct; 0.425; 0.424; 0.427; 0.448; 0.492.

### Time varying changes

- MediaTek Inc. (R1-2305659) Table 31. Evaluation results for AI/ML model deployed on UE or network-side with generalization, CNN, UE distribution area = [120x60 m], fine-tuning of time varying Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML. CIR [18,2,256]; UE pos [x,y]; 0%; without time varying; with time varying; with time varying; 32400; 0; 3600; 464.24K; 0.266G; >100. 1200; 11.206. 3600; 6.712. 7200; 5.076. PDP [18,2,256]; UE pos [x,y]; 0%; without time varying; with time varying; with time varying; 32400; 0; 3600; 463.95K; 0.264G; >100. 1200; 9.950. 3600; 6.146. 7200; 4.796. Observation 41: Performance of direct AI/ML positioning degrades when there is time varying change between the training data and test data, and fine-tuning can improve the performance.

### Channel estimation error

- vivo (R1-2304475) Figure 36 Evaluation of the impact of CIR estimation error on positioning accuracy Observation 18: The interference from TPRs can dramatically impair the positioning performance of AI/ML model.

#### Model fine-tuning for channel estimation error

- MediaTek Inc. (R1-2305659) Table 29. Evaluation results for AI/ML model deployed on UE or network-side with generalization, CNN, UE distribution area = [120x60 m], fine-tuning of channel estimation error Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML 20dB; AI/ML 0dB. CIR [18,2,256]; UE pos [x,y]; 0%; 0dB; 20dB; 20dB; 32400; 0; 3600; 464.24K; 0.266G; 3.956; 2.645. 1200; 2.024; 7.420. 3600; 1.944; 14.220. 7200; 1.778; 45.923. PDP [18,2,256]; UE pos [x,y]; 0%; 0dB; 20dB; 20dB; 32400; 0; 3600; 463.95K; 0.264G; 2.405; 2.255. 1200; 1.875; 3.755. 3600; 1.733; 13.873. 7200; 1.607; 16.828. Observation 39: Fine-tuning a model with samples of new parameter setting (e.g., drop, clutter setting, channel estimation error, timing error, scenario) can achieve positioning accuracy improvement when the pre-trained model is transferred to a new parameter setting for direct AI/ML positioning (the more fine-tuning data the better performance), but performance degrades for pre-trained parameter setting.

#### Mixed dataset for channel estimation error

- MediaTek Inc. (R1-2305659) Table 28. Evaluation results for AI/ML model deployed on UE or network-side with generalization, CNN, UE distribution area = [120x60 m], different channel estimation error Model input; Model output; Dataset size; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Test Train; SNR=0dB; SNR=10dB; SNR=20dB; Ideal. CIR [18,2,256]; UE pos [x,y]; 32400; 3600; SNR=0dB; 2.575; 4.697; 4.878; 4.887. SNR=10dB; 53.064; 1.541; 1.520; 1.516. SNR=20dB; 60.615; 46.2; 1.168; 1.220. Ideal; 53.918; 59.543; 43.803; 0.940. 32400+32400; 3600; 0dB & 10dB; 2.211; 1.456; 1.453; 1. 452. PDP [18,2,256]; UE pos [x,y]; 32400; 3600; SNR=0dB; 2.252; 3.036; 3.108; 3.102. SNR=10dB; 43.713; 1.418; 1.455; 1.457. SNR=20dB; 57.18; 37.334; 1.034; 1.00. Ideal; 64.88; 37.01; 1.561; 0.821. 32400+32400; 3600; 0dB & 10dB; 1.825; 1.253; 1.259; 1.260. Observation 36: The direct AI Model trained by dataset with large channel estimation error can be generalized to dataset with small channel estimation error. Observation 37: The positioning performance can be improved by mixing dataset with different channel estimation errors at the cost of the training complexity. Observation 38: In fact, channel estimation errors can be used as a type of data augmentation to enhance trained model performance and to increase the model robustness to various channel estimation errors.

- CEWiT (R1-2305896) Observation-8: From the evaluation result, it is observes that the performance of AI/ML model for inference dataset with channel estimation error is better than the inference da-taset without channel estimation error.

### Hall size

- Samsung (R1-2305509) Fig.7 CDF of positioning errors with different deployment (hall size) Observation 4: When the training dataset is from DH662 small hall size, and the testing/inference dataset generated from different scenario (InF-SH), or different clutter parameter (DH422), or different deployment (large hall size), the performance of all three genralzation cases are degraded severely. Observation 5: When the training dataset is from DH662 small hall size, and 1k training data update and the testing/inference dataset generated from different scenario (InF-SH), or different clutter parameter (DH422), or different deployment (large hall size), the performance of all three generalizations cases are recovered significantly. The larger FH size, the better performance could be achieved.

## High-level summary of methods to address generalization problem

### Model fine-tuning

- vivo (R1-2304475) Table 61 Evaluation of model fine-tuning for different cases Cases; Training; Fine-tuning; Testing; Positioning accuracy @90%. Direct AI/ML positioning; {0.6, 6, 2}; {0.4, 2, 2}; {0.4, 2, 2}; 4.40. {0.4, 2, 2}; {0.6, 6, 2}; {0.6, 6, 2}; 3.23. Drop1; Drop2; Drop2; 3.97. InF-DH; InF-HH; InF-HH; 8.78. Sync 0ns; 50ns; 50ns; 2.39. Sync 0ns; 10ns; 10ns; 1.28. Sync 0ns; 2ns; 2ns; 1.11. AI/ML assisted positioning; {0.6, 6, 2}; {0.4, 2, 2}; {0.4, 2, 2}; 0.63. Drop1; Drop2; Drop2; 5.50. InF-DH; InF-HH; InF-HH; 0.17. InF-DH; InF-SH; InF-SH; 0.17. Sync 0ns; 50ns; 50ns; 3.40. Sync 0ns; 10ns; 10ns; 1.78. Sync 0ns; 2ns; 2ns; 1.30. Observation 38: Model fine-tuning is suitable for the following tasks: • The source domain and the target domain are greatly similar, such as with different synchronization error. • The target domain is easy to fit, such as TOA estimation of LOS path. Figure 72 The curve of positioning error reduction with increasing number of sample size (data efficiency) Proposal 19: Model fine-tuning can flexibly adapt AI/ML model to various dynamic changes in environment, while mix-training is more suitable for these static changes. Proposal 20: It is benefit to integrate model fine-tuning and mix-training so as to fully leverage their advantages.

- Apple (R1-2305973) Observation 3 - Finetuning improves the performance with more improvement as the data size increases. - Note: Results may improve with optimization - Study the relative performance between model finetuning and model training with the same amount of data.

- Huawei (R1-2304657) Observation 25 : For direct AI/ML positioning, when the model input is CIR, over different dataset sizes for fine-tuning, the positioning accuracy increases when the size of fine-tuning dataset increases. At least 5000 samples are needed for fine-tuning to maintain sub-meter level accuracy. Observation 26 : For direct AI/ML positioning, when the model input is PDP, over different dataset sizes for fine-tuning, the positioning accuracy increases when the size of fine-tuning dataset increases. At least 5000 samples are needed for fine-tuning to maintain meter level accuracy.

- Qualcomm (R1-2305332) - Observation 16: In direct AI/ML positioning, evaluations show that model fine-tuning with small dataset can only offer slight to moderate enhancement to positioning performance of AI/ML positioning when tested with different drops (i.e., inter-site generalization).

- Ericsson (R1-2304339) Table 23. 90%tile UE 2D positioning errors for small Model I trained with 40,000 CIR samples from {60%, 6m, 2m} and fine-tuned with different number of CIR samples from {40%, 2m, 2m}. Tested on {40%, 2m, 2m} test dataset. Positioning approach; Model complexity [# of parameters]; 90%tile UE 2D positioning errors [m] – small Model I. Trained with samples from {40%, 2m, 2m}; Originally trained with 40,000 samples from {60%, 6m, 2m} and fine-tuned with different number of samples from {40%, 2m, 2m}; Trained with samples from {60%, 6m, 2m}. 40,000; 40,000; 20,000; 10,000; 4,000; 2,000; 1,000; 40,000. Dist. Assist.; 0.86 M; 0.409; 0.420; 0.543; 0.782; 1.285; 1.808; 2.404; 8.720. Cent. Assist.; 0.73 M; 0.697; 0.621; 0.818; 1.141; 1.806; 2.284; 2.885; 6.913. Cent. Direct; 0.73 M; 0.674; 0.621; 0.841; 1.177; 1.801; 2.329; 3.030; 7.354. Table 24. 90%tile UE 2D positioning errors for small Model I trained with 40,000 PDP samples from {60%, 6m, 2m} and fine-tuned with different number of PDP samples from {40%, 2m, 2m}. Tested on {40%, 2m, 2m} test dataset. Positioning approach; Model complexity [# of parameters]; 90%tile UE 2D positioning errors [m] – small Model I. Trained with samples from {40%, 2m, 2m}; Originally trained with 40,000 samples from {60%, 6m, 2m} and fine-tuned with different number of samples from {40%, 2m, 2m}; Trained with samples from {60%, 6m, 2m}. 40,000; 40,000; 20,000; 10,000; 4,000; 2,000; 1,000; 40,000. Dist. Assist.; 0.43 M; 0.596; 0.552; 0.738; 0.989; 1.408; 1.882; 2.486; 8.176. Cent. Assist.; 0.36 M; 0.854; 0.782; 0.983; 1.269; 1.879; 2.378; 2.843; 6.904. Cent. Direct; 0.36 M; 0.810; 0.791; 0.991; 1.267; 1.876; 2.418; 2.880; 6.542. Table 25. 90%tile UE 2D positioning errors for small Model I trained with 40,000 DP samples from {60%, 6m, 2m} and fine-tuned with different number of DP samples from {40%, 2m, 2m}. Tested on {40%, 2m, 2m} test dataset. Positioning approach; Model complexity [# of parameters]; 90%tile UE 2D positioning errors [m] – small Model I. Trained with samples from {40%, 2m, 2m}; Originally trained with 40,000 samples from {60%, 6m, 2m} and fine-tuned with different number of samples from {40%, 2m, 2m}; Trained with samples from {60%, 6m, 2m}. 40,000; 40,000; 20,000; 10,000; 4,000; 2,000; 1,000; 40,000. Cent. Assist.; 0.36 M; 1.071; 1.025; 1.281; 1.614; 2.455; 3.164; 3.972; 11.940. Cent. Direct; 0.36 M; 1.033; 1.036; 1.294; 1.618; 2.441; 3.230; 3.902; 11.554. Observation 32 Once the models are fine-tuned to operate for a substantially different, new, environment, they no long perform adequately for the original environment. That is, if operation at state-of-the-art performance in multiple environments is needed, multiple sets of model weights need to be stored. Table 26. 90%tile UE 2D positioning errors for small Model I trained with 40,000 CIR samples from {60%, 6m, 2m} and fine-tuned with different number of CIR samples from {40%, 2m, 2m} and test on {60%, 6m, 2m} test dataset. Positioning approach; Model complexity [# of parameters]; 90%tile UE 2D positioning errors [m] – small Model I. Trained with samples from {60%, 2m, 2m}; Originally trained with 40,000 samples from {60%, 6m, 2m} and fine-tuned with different number of samples from {40%, 2m, 2m}. 40,000; 40,000; 20,000; 10,000; 4,000; 2,000; 1,000. Dist. Assist.; 0.86 M; 0.451; 2.111; 2.653; 3.465; 4.293; 5.267; 5.863. Cent. Assist.; 0.73 M; 0.371; 1.358; 1.549; 1.865; 2.473; 2.911; 3.258. Cent. Direct; 0.73 M; 0.373; 1.351; 1.668; 1.993; 2.632; 3.453; 3.352. Table 27. 90%tile UE 2D positioning errors for small Model I trained with 40,000 PDP samples from {60%, 6m, 2m} and fine-tuned with different number of PDP samples from {40%, 2m, 2m} and test on {60%, 6m, 2m} test dataset. Positioning approach; Model complexity [# of parameters]; 90%tile UE 2D positioning errors [m] – small Model I. Trained with samples from {60%, 2m, 2m}; Originally trained with 40,000 samples from {60%, 6m, 2m} and fine-tuned with different number of samples from {40%, 2m, 2m}. 40,000; 40,000; 20,000; 10,000; 4,000; 2,000; 1,000. Dist. Assist.; 0.43 M; 0.684; 2.424; 2.813; 3.417; 4.123; 4.962; 5.110. Cent. Assist.; 0.36 M; 0.524; 1.383; 1.515; 1.803; 2.352; 2.917; 2.911. Cent. Direct; 0.36 M; 0.510; 1.379; 1.591; 1.795; 2.455; 2.794; 3.013. Table 28. 90%tile UE 2D positioning errors for small Model I trained with 40,000 DP samples from {60%, 6m, 2m} and fine-tuned with different number of DP samples from {40%, 2m, 2m} and test on {60%, 6m, 2m} test dataset. Positioning approach; Model complexity [# of parameters]; 90%tile UE 2D positioning errors [m] – small Model I. Trained with samples from {60%, 2m, 2m}; Originally trained with 40,000 samples from {60%, 6m, 2m} and fine-tuned with different number of samples from {40%, 2m, 2m}. 40,000; 40,000; 20,000; 10,000; 4,000; 2,000; 1,000. Cent. Assist.; 0.36 M; 0.653; 1.880; 2.143; 2.430; 3.264; 3.580; 4.047. Cent. Direct; 0.36 M; 0.658; 1.957; 2.242; 2.436; 3.346; 3.715; 3.967.

### Mixed dataset

- CMCC (R1-2305089) Observation 3: If the mixed training dataset comprises the samples of the drop as the test dataset, the positioning accuracy can be improved obviously.

- Ericsson (R1-2304339) - Observation 37 Even with small model sizes, models trained with mixed datasets can operate at state-of-the-art performance in multiple substantially different environments with no need of explicit environment identification and model switching. - Observation 38 For the alternative approach of keeping multiple models for multiple operating environments, reliability of environment identification becomes the critical point of failure. - Observation 39 To operate at state-of-the-art performance in multiple substantially different environments, mixed dataset training is superior to either training multiple models from scratch or storing multiple fine-tuned models with explicit environment identification and model switching.

## 1st round discussion

In RAN1#112, the following observation was made for direct AI/ML positioning:

Observation (RAN1#112) Evaluation of the following generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario. - The generalization aspects include: - Different drops - Different clutter parameters - Different InF scenarios - Network synchronization error - Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training. - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset. - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset. Note: ideal model training and switching may provide the upper bound of achievable performance when the AI/ML model needs to handle different deployment scenarios.

The generalization aspects "UE/gNB RX and TX timing error", "SNR mismatch", and "Channel estimation error" were not included due to insufficient evaluation results. Based on the evaluation results to this meeting, the following observations are proposed to address this. The intention of the Observation 5.1-1 is to clarify, model fine-tuning/re-training and mixed dataset are also effective for addressing the generalization issues, based on the evaluation results from companies. It is noted that "time varying changes" is not included, since there is no evaluation results of fine-tuning or mixed dataset for it.

- For UE/gNB RX and TX timing error: please see exemplary evaluation results in section 5.2.4 and the references therein.

- For SNR mismatch: please see exemplary evaluation results in section 5.2.6 and the references therein.

- For channel estimation error: please see exemplary evaluation results in section 5.2.8 and the references therein.

Observation 5.4-1

For direct AI/ML positioning, evaluation results demonstrate that for the generalization aspects of: UE/gNB RX and TX timing error, SNR mismatch, and channel estimation error,

- the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training.

  - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset.

  - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset.

The observation 5.4-2 below on fine-tuning is based on evaluation results submitted by companies. For example,

- Huawei (R1-2304657) showed that at least 5000 samples are needed for fine-tuning to maintain sub-meter level accuracy.

- Qualcomm (R1-2305332) showed that with 500 samples for fine-tuning, a model trained for Drop A and tested for Drop B, have positioning error=6.07 at CDF=90%, i.e., still poor performance, and fine-tuning with a small dataset is not effective.

- NVIDIA (R1-2305164) showed that when fine-tuning a model of clutter parameter 1 to clutter parameter 2, fine-tuning with 4000 samples achieves 6.5m at CDF=90%, which is still far from the 2.3m accuracy expected.

- vivo (R1-2304475) showed that with CIR as model input, when using 1000 samples for fine-tuning,

  - a model trained with synchronization error=0ns can be updated for the new scenario of synchronization error=10ns, and have positioning error = 1.28m at CDF=90%;

  - however, a model trained in InF-DH can be updated for the new scenario of InF-HH, and have positioning error = 8.78m at CDF=90%;

- Ericsson (R1-2304339) showed that for CIR, PDP, or DP as model input, a model trained with 40,000 samples from {60%, 6m, 2m} requires 40,000 samples to retrain the model for {40%, 2m, 2m}, in order to achieve the same performance as a model trained from scratch for {40%, 2m, 2m}.

Observation 5.4-2

For direct AI/ML positioning,

- Model fine-tuning with a small dataset size is most useful for enhancing positioning accuracy for small unseen changes (e.g., synchronization error).

- If the new deployment scenario is significantly different from the deployment scenario the model was trained for (e.g., different drops, different clutter parameter, different InF scenarios), fine-tuning is less effective. Re-training an existing model requires similarly large training dataset size as training the model from scratch, in order to achieve the converged performance for the new deployment scenario.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Do not agree. The observation seems vague in our opinion, how to quantify a small unseen change and what a new scenario? It seems it would be difficult to draw a borderline which could lead to wrong conclusions or different understandings across companies. In table 16 in our paper (R1-2304657), we have e.g. evaluated different clutter parameters and used fine-tuning with 3000 and 5000 samples (compared to training 25000) and achieved very good accuracy. Thus, according to our simulations, fine-tuning with small dataset, can be useful for new scenarios as well. |
| LG | It is somewhat vague on Observation 5.4-2 that what is the granularity of the deploy scenario is different? |
| ZTE | Agree with Huawei. It’s unclear about the unseen change. |
| OPPO | Similar view with Huawei. It’s unclear about the definition of small unseen changes and small dataset size. |

There has been a question whether a model fine-tuned for a new scenario could still works well for the old scenario. Ericsson (R1-2304339, Table 26-28) showed that when a model previously trained for {60%, 6m, 2m} is fine-tuned for {40%, 2m, 2m}, the new model does not work well for {60%, 6m, 2m} any more. Accordingly, the following observation is suggested.

Observation 5.4-3

For direct AI/ML positioning, evaluation results show that fine-tuning/re-training an old model with dataset of the new deployment scenario (e.g., new clutter parameter setting) can improve positioning accuracy when the updated model is deployed in a new deployment scenario. On the other hand, the performance of the updated model degrades for the previous deployment scenario (e.g., previous clutter parameter setting) that the old model was trained for.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | We think that this observation is not relevant. The dataset for the old model is for the old scenario but here we should look at how we can adapt from a generically trained model to a given scenario. |
| LG | Generally fine. But, as similar comment as the above, it is based on the amount of difference on a new deployment scenario compared to that of old model. If the difference is relatively small, the old model can still work. |

Apple (R1-2305973) provided simulation results to show that the AI/ML model delivers better performance if the evaluation is the convex hull rather than the entire area. This

Observation 5.4-4

The performance of AI/ML positioning with the evaluation area as the convex hull of the horizontal BS deployment shows better performance than that with the whole hall area as evaluation area.

- For Convex hull: UE distribution area = 100x40 m;

- For whole hall area: UE distribution area = 120x60 m

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |

# Impact of direct AI/ML positioning performance by model input type and size

## Evaluation of input size reduction

### Different type of model input

- vivo (R1-2304475) Table 79 Evaluation results for AI/ML model deployed on UE or Network side, UE distribution area = [120x60 m] Model input; Model output; Number of TRP; Length; Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m); Average bits overhead. Train; test; Model complexity; Computation complexity; AI/ML. Delay; Pos.; 6; 64; 25k; 1k; 1.65M; 22.30M; 0.86; 455. Delay + Power (PDP); Pos.; 6; 64; 25k; 1k; 1.65M; 22.30M; 0.68; 1174. Delay + Power+ Phase (CIR); Pos.; 6; 64; 25k; 1k; 1.65M; 22.30M; 0.62; 1842. RSRPP; Pos.; 6; 9; 25k; 1k; 1.65M; 22.30M; 2.00; 432.

- Qualcomm (R1-2305332) Observation 4: We observe the following regarding the trade-off between positioning accuracy and reporting overhead for AI/ML positioning: Case1 produces the best performance with least reporting overhead (e.g., 2.24m with 24 bits). Case2a with N’t =1 (i.e., one path) produces higher positioning accuracy (e.g., 2.93m vs. 3.17m) and has smaller reporting overhead (e.g., 272 vs. 13824 bits) than Case2b with CIR of N’t =8 samples. Case2b with multipath measurements (up to N’t =8 multipaths) produces higher positioning accuracy (e.g., 2.52m vs. 3.17m) and has smaller reporting overhead (e.g., 1152 vs. 13824 bits) than Case2b with CIR of N’t =8 samples. Case2b with multipath measurements (up to N’t =8 multipaths) produces comparable positioning accuracy (e.g., 2.52m vs. 2.51m) and has smaller reporting overhead (e.g., 1152 vs. 23040 bits) than Case2b with CIR of N’t =16 samples. Case2b with multipath measurements (up to N’t =16 multipaths) produces higher positioning accuracy (e.g., 2.31m vs. 2.51m) and has smaller reporting overhead (e.g., 2304 vs. 23040 bits) than Case2b with CIR of N’t =16 samples. Case2b with multipath measurements (up to N’t =16 multipaths) produces comparable positioning accuracy (e.g., 2.31m vs. 2.29m) and has much smaller reporting overhead (e.g., 2304 vs. 78336 bits) than Case2b with CIR of N’t =64 samples. Observation 5: For AI/ML positioning evaluation of reporting overhead (Case2b), evaluations show that reporting of CIR measurements with N’t > 64 incurs huge reporting overhead and the expected performance gain is minimal (e.g., enhancing accuracy from 2.29m to 2.24m). Observation 6: For AI/ML positioning evaluation of reporting overhead (Case2b), evaluations show that reporting of CIR measurements with N’t ≤ 64 can have less or comparable performance gain to multipath reporting and has higher reporting overhead. Observation 7: For AI/ML positioning evaluation of reporting overhead (Case2b), when considering same reporting overhead for N’t ≤ 64, evaluations show that Case2b achieves higher positioning accuracy with multipath reporting than CIR measurement reporting. Proposal 9: For specifying model input in Case2b in AI/ML positioning, deprioritize CIR measurement reporting due to its the huge reporting overhead and comparable or minimal performance gain when compared to existing multipath reporting.

- Apple (R1-2305973) Table 5: Direct AI/ML Positioning: Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, with a CNN, UE distribution area = 100x40 m, Model Input Comparison Study (CIR, PDP, DP) Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR [18,1,256,2]; UE coordinates [1x2]; 2-D UE position 100% labeled; Drop 1; Drop 1; 47500; 2500; 1.48 M (default); 2.75G (default); 0.884m. Delay Profile [18,1,256,1] DP; UE coordinates [1x2]; 2-D UE position 100% labeled; Drop 1; Drop 1; 47500; 2500; 1.48 M; 2.75G; 0.7289m. Power Delay Profile [18,1,256,1]; UE coordinates [1x2]; 2-D UE position 100% labeled; Drop 1; Drop 1; 47500; 2500; 1.48 M; 2.75G; 0.7441m.

- Ericsson (R1-2304339)

- Samsung (R1-2305509) A new CIR post processing method on the model input, which called “signature transform” or “log signature transform” over the CIR. The target is to keep the useful information from the CIR and extract them as “signatures” to be used in the model training. The log signature transform is the mapping from a path to the logarithm of its signature. The signature of a path, which was originally introduced [4], can be seen as a collection of features extracted from the path by iterated integrals. The signature provides a well summary of the path and has meaningful interpretations from the geometrical or statistical point of view. The log signature, i.e., the logarithm of the signature, is a lossless compression of the signature, providing the same amount of information with fewer features. The key points to using log signature transform as a feature extraction are constructing a suitable path (continuous mapping) and doing necessary augmentation from the collected raw discrete time series. For the purpose of positioning, the path constructed by cumulative sum of energy (CSE) of the CIR and time augmentation are proposed, with the benefit of keeping the vital information for positioning. The log signature transform is applied to the constructed path to obtain features for subsequent AI/ML model training and inference. The basic procedure is shown in following figure: Fig. 10 illustration of log-signature transform based processing over CIR Among the procedure, most importantly, the specifically constructed time-augmented CSE path of each CIR keeps the critical information (power and time) for the positioning purpose and becomes suitable for feature derivation. Then the features of the path signature and log signature are obtained. With closed-form expressions of these features, the superior knowledge of filtering the necessary features can be derived before feeding into an AI model for learning and inference. Experiments in show that the proposed method can drastically reduce the input data size and model complexity and without accuracy reduction or achieve better positioning accuracy with a simple AI/ML model (like MLP) compared with CIR-based AI/ML models. ### Using simple AI/ML model, MLP Fig.11 comparison of CIR-based and SIG based in MLP It’s clear that the SIG based positioning achieves better performance than the CIR based one when using the MLP (in case of 2 layers, 4 layers and 6 layers). Observation 9: SIG based input could adopt better (e.g., 7~8 times better in pos accuracy) with simple AI/ML model (e.g., MLP) than CIR based input. ### Using ResNet Fig.12 comparison of CIR-based and SIG based in ResNet Fig.13 illustration of complexity reduction and Data size reduction It shows that the pos accuracy when using ResNet for both input type are comparable but the used input data size and the complexity are quite different, in which the SIG based input type reduced with 98% in both aspects. Observation 10: the SIG-based input could drastically reduce the input data size and the complexity (e.g., with 98% reduction) without accuracy loss (e.g., even with 1% improvement). Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. SIG With 18*6; 2D-coordinates; 2D-coordinates; DH662(normalized CIR); DH662(normalized CIR); 18000; 2000; 74K; 0.21M; 3.13. CIR with 18x256x2; 2D-coordinates; 2D-coordinates; DH662(normalized CIR); DH662(normalized CIR); 18000; 2000; 76K; 9.5M; 3.18. Table X. Evaluation results for AI/ML model deployed on UE-side or network-side, MLP Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR with 18x256x2; 2D-coordinates; 2D-coordinates; DH662; DH662; 18000; 2000; 932K; 1.86M; 36.23(2 layer). CIR with 18x256x2; 2D-coordinates; 2D-coordinates; DH662; DH662; 18000; 2000; 952K; 1.91M; 35.35 (4 layer). CIR with 18x256x2; 2D-coordinates; 2D-coordinates; DH662; DH662; 18000; 2000; 972K; 1.94M; 34.23 (6 layer). SIG With 18*6; 2D-coordinates; 2D-coordinates; DH662; DH662; 18000; 2000; 21.2K; 42.2K; 5.24(2 layer). SIG With 18*6; 2D-coordinates; 2D-coordinates; DH662; DH662; 18000; 2000; 41..2K; 82.3K; 4.20 (4 layer). SIG With 18*6; 2D-coordinates; 2D-coordinates; DH662; DH662; 18000; 2000; 61.2K; 0.123M; 4.17 (6 layer).

#### 1st round discussion

Evaluation results submitted by companies are summarized below in Table 2 for different types of model input. Please add/correct if anything is missing or wrong.

Table 2. Positioning error of different types of model input

|  | E_CIR | E_PDP | E_DP | E_PDP/E_CIR | E_DP/E_CIR |
| --- | --- | --- | --- | --- | --- |
| MediaTek R1-2305659, Table 35 | 0.94 | 0.821 |  | 0.87 |  |
| MediaTek R1-2305659, Table 35 | 0.915 | 1.089 |  | 1.19 |  |
| MediaTek R1-2305659, Table 35 | 1.017 | 1.133 |  | 1.11 |  |
| MediaTek R1-2305659, Table 35 | 0.984 | 0.942 |  | 0.96 |  |
| MediaTek R1-2305659, Table 35 | 0.851 | 0.965 |  | 1.13 |  |
| MediaTek R1-2305659, Table 35 | 1.097 | 1.057 |  | 0.96 |  |
| vivo R1-2304475, Table 79 | 0.62 | 0.68 | 0.86 | 1.10 | 1.39 |
| Apple R1-2305973, Table 5 | 0.884 | 0.7289 | 0.7441 | 0.82 | 0.84 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.3 | 0.426 | 0.558 | 1.42 | 1.86 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.373 | 0.51 | 0.658 | 1.37 | 1.76 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.498 | 0.656 | 0.789 | 1.32 | 1.58 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.718 | 0.863 | 1.014 | 1.20 | 1.41 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.199 | 0.269 | 0.391 | 1.35 | 1.96 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.268 | 0.349 | 0.465 | 1.30 | 1.74 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.385 | 0.496 | 0.6 | 1.29 | 1.56 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.597 | 0.735 | 0.823 | 1.23 | 1.38 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.155 | 0.214 | 0.298 | 1.38 | 1.92 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.233 | 0.288 | 0.379 | 1.24 | 1.63 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.354 | 0.425 | 0.522 | 1.20 | 1.47 |
| Ericsson R1-2304339, Table 4 & Table 7 | 0.556 | 0.653 | 0.758 | 1.17 | 1.36 |
|  |  |  | max | 1.42 | 1.96 |
|  |  |  | min | 0.82 | 0.84 |

Based on the summary table above, the following observation is drawn.

Observation 6.1.1.1-1 (CIR vs PDP vs DP)

For direct AI/ML positioning, the positioning accuracy at model inference is affected by the type of model input.  Evaluation results submitted to RAN1#113 show that if changing model input type while holding other parameters (e.g., Nt, N't, Nport, N'TRP) the same,

  - Three sources (MediaTek R1-2305659, vivo R1-2304475, Ericsson R1-2304339) showed The positioning error of PDP as model input is 1.1 ~ 1.42 times the positioning error of CIR as model input.

  - Two sources (MediaTek R1-2305659, Apple R1-2305973) showed that the positioning error of PDP as model input is 0.82 ~ 0.96 times the positioning error of CIR as model input.

- When comparing PDP and CIR as model input, The positioning error of PDP as model input is 0.82 ~ 1.42 times the positioning error of CIR as model input

  - Three sources (vivo R1-2304475, Ericsson R1-2304339) showed The positioning error of PDP as model input is 1.36 ~ 1.96 times the positioning error of CIR as model input.

  - One source (Apple R1-2305973) showed that the positioning error of PDP as model input is 0.84 times the positioning error of CIR as model input.

- Note: DP uses Nport=1 only.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Not agree. We do not think that DP needs to be treated separately. In our view the DP is just one quantization realization of the PDP. And the quantization of the PDP itself has not be discussed yet. Therefore. We could say that depending on the quantization level of the PDP, its accuracy is between 0.82 and 1.96 times CIR. |
| ZTE | Not clear about the assumptions on Nt, N't, Nport, N'TRP. It’s not appropriate to take CIR as baseline to compare the results. Our suggestion is to take PDP as baseline since it’s alrady defined in the spec. For example, PDP { Nt, N't, Nport, N'TRP}= {256, 256, 1, 18} can be the baseline for comparison. Companies have different numbers of training data, which is not reflected in this observation. In addition, ZTE also provides the evaluation results for PDP and CIR. It’s not captured in here. For example, when the training samples are 28800: E_CIR; E_PDP; E_DP; E_PDP/E_CIR; E_DP/E_CIR. ZTE R1-2302538 Excel {Nport, NTRP, Nt, Nt'} = {2, 18, 256, 128}; 0.28; 0.45; 1.60. |

### Model input truncated in time domain

- MediaTek Inc. (R1-2305659) Table 35. Evaluation results for AI/ML model deployed on UE or network-side without generalization, CNN, UE distribution area = [120x60 m], different N’t and Nt Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Nt; N’t; Train; Test; Model complexity; Computation complexity; AI/ML. CIR [18,2,256]; UE pos [x,y]; 0%; 256; 25; 32400; 3600; 464.24K; 0.266G; 0.940. 20; 0.915. 15; 1.017. 64; 25; 243.058K; 0.066G; 0.984. 20; 0.851. 15; 1.097. PDP [18,2,256]; UE pos [x,y]; 0%; 256; 25; 32400; 3600; 463.95K; 0.264G; 0.821. 20; 1.089. 15; 1.133. 64; 25; 242.770K; 0.066G; 0.942. 20; 0.965. 15; 1.057. Observation 44: By selecting appropriate Nt and N’t, the computational complexity, model complexity and signaling overhead can be reduced without significant performance loss.

- Apple (R1-2305973) Table 10: Direct AI/ML Positioning: Evaluation results for AI/ML model deployed on UE or Network side, using a different # of consecutive taps , with a CNN, UE distribution area = 100x40 m Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR [18,1,256,2]; UE coordinates [1x2]; 2-D UE position 100% labeled; Drop 1 18 TRP; Drop 1 18 TRP; 47500; 2500; 1,480,140; 2.75G; 0.884m. CIR [18,1,128,2]; UE coordinates [1x2]; 2-D UE position 100% labeled; Drop 1 9 TRP; Drop 1 9 TRP; 47500; 2500; 1,480,140; 2.75G; 0.884. CIR [18,1,64,2]; UE coordinates [1x2]; 2-D UE position 100% labeled; Drop 1 4 TRP; Drop 1 4 TRP; 47500; 2500; 1,480,140; 2.75G; 1.1206. Observation 5: For direct AI/ML based positioning - Reducing the number of taps from 256 to 128 does not affect the performance - Reducing the number of taps to 64 starts affecting the performance negatively - Reducing the number of taps ultimately reduces the overhead by the same factor

- Huawei (R1-2304657) Observation 2 : For direct AI/ML positioning, when the model input is CIR and consists of different numbers of time domain samples, sub-meter level accuracy can be maintained with short CIR lengths while the computational complexity can be reduced significantly. • For 18 CIRs as model input, sub-meter level accuracy is still kept when the number of CIR samples is reduced from 256 to 32. The computational complexity can be reduced from 25.81M to 3.36M accordingly. • For 4 CIRs as model input, sub-meter level accuracy is still kept when the number of CIR samples is reduced from 256 to 64. The computational complexity can be reduced from 6.52M to 1.68M accordingly. Observation 3 : For direct AI/ML positioning, when the model input is CIR, compared to the initial assumptions of 18 TRPs and 256 samples per CIR as model input, the signaling payload could be reduced to 1/18 when going down to 4 TRPs and 64 samples per CIR, while still maintaining sub-meter level accuracy. Observation 4 : For direct AI/ML positioning, when the AI/ML model input is PDP (one PDP per TRP) and different numbers of TRPs are evaluated, • The positioning accuracy decreases slightly when the number of TRPs is reduced from 18 to 4. But to maintain sub-meter level accuracy, the number of TRPs should be at least 4. • Model and computational complexity decrease significantly with a smaller number of TRPs. The model complexity and computation complexity are both reduced by more than 70% when reducing the number of TRPs from 18 to 4. Observation 5 : For direct AI/ML positioning, when the model input is PDP and consists of different numbers of time domain samples, sub-meter level accuracy can be maintained with short PDP lengths while the computational complexity can be reduced significantly. • For 4 PDPs as model input, sub-meter level accuracy is still kept when the number of PDP samples is reduced from 256 to 32. The computational complexity can be reduced from 4.11M to 0.623M accordingly.

- vivo (R1-2304475) Table 71 Evaluation results for AI/ML model deployed on UE or Network side, UE distribution area = [120x60 m] Model input; Model output; Truncated CIR; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; Pos.; 1-30; DH; DH; 25k; 1k; 1.65M; 22.30M; 1.04. CIR; Pos.; 1-100; DH; DH; 25k; 1k; 1.65M; 22.30M; 1.04. CIR; Pos.; 30-100; DH; DH; 25k; 1k; 1.65M; 22.30M; 2.99.

- CATT (R1-2304726) Observation 14: For direct AI/ML positioning, reducing the number of CIR taps can reduce the computation complexity of a model while keeping similar inference accuracy.

- Ericsson (R1-2304339) Table 13 UE 2D positioning errors for {60%, 6m, 2m} test dataset at different percentiles and different numbers of time domain window size Nt (training dataset size = 40,000 samples, Nt=Nt'). Input type; Time domain window size Nt; Model complexity [# parameters]; Computational complexity [FLOPs]; 90%tile 2D positioning error [m] in {60%, 6m, 2m} InF-DH. Cent. Assist.; Cent. Direct. CIR; 256; 0.73 M; 32 M; 0.371; 0.373. CIR; 128; 0.73 M; 15 M; 0.463; 0.443. CIR; 64; 0.73 M; 7 M; 0.704; 0.668. CIR; 32; 0.73 M; 3 M; 1.627; 1.602. PDP; 256; 0.36 M; 9 M; 0.520; 0.510. PDP; 128; 0.36 M; 4 M; 0.596; 0.563. PDP; 64; 0.36 M; 2 M; 0.713; 0.711. PDP; 32; 0.36 M; 0.8 M; 1.261; 1.165. DP; 256; 0.36 M; 9 M; 0.653; 0.658. DP; 128; 0.36 M; 4 M; 0.679; 0.658. DP; 64; 0.36 M; 2 M; 1.970; 1.976.

#### 1st round discussion

Evaluation results submitted by companies are summarized in Table 3 below for truncation in time domain (i.e. vary Nt; no sub-sampling). Model input CIR and PDP are considered. DP is not included since DP always select a subset of taps from timing window Nt.

Please add/correct if anything in Table 3 is missing or wrong.

Table 3. Positioning error E_* (meter) when time window size Nt varies. Model input is CIR or PDP. Full set of Nt samples are used as model input (i.e., N't=Nt)

|  | Nt=256 | Nt=128 | Nt=64 | Nt=32 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | E_256 | E_128 | E_64 | E_32 | E_128/E_256 | E_64/E_256 | E_32/E_256 |
| MediaTek R1-2305659, Table 35 | 0.94 |  | 0.984 |  |  | 1.05 |  |
| MediaTek R1-2305659, Table 35 | 0.915 |  | 0.851 |  |  | 0.93 |  |
| MediaTek R1-2305659, Table 35 | 1.017 |  | 1.097 |  |  | 1.08 |  |
| MediaTek R1-2305659, Table 35 | 0.821 |  | 0.942 |  |  | 1.15 |  |
| MediaTek R1-2305659, Table 35 | 1.089 |  | 0.965 |  |  | 0.89 |  |
| MediaTek R1-2305659, Table 35 | 1.133 |  | 1.057 |  |  | 0.93 |  |
| vivo R1-2304475, Table 71 | 0.99 |  | 1.04 | 1.04 |  | 1.05 | 1.05 |
| Apple R1-2305973, Table 10 | 0.884 | 0.884 | 1.1206 |  | 1.00 | 1.27 |  |
| Huawei R1-2304657, Table 4 | 0.62 | 0.5 | 0.64 | 0.71 | 0.81 | 1.03 | 1.15 |
| Huawei R1-2304657, Table 5 | 0.97 | 0.85 | 0.85 | 1.19 | 0.88 | 0.88 | 1.23 |
| Ericsson R1-2304339, Table 13 | 0.373 | 0.443 | 0.668 | 1.602 | 1.19 | 1.79 | 4.29 |
| Ericsson R1-2304339, Table 13 | 0.51 | 0.563 | 0.711 | 1.165 | 1.10 | 1.39 | 2.28 |
| CATT R1-2304726, Table 1 | 0.58m |  |  |  |  |  |  |
| CATT R1-2304726, Table 14 |  | 0.57m | 0.58m | 0.62m | 0.98 | 1 | 1.07 |
|  |  |  |  | max | 1.19 | 1.79 | 4.29 |
|  |  |  |  | min | 0.81 | 0.88 | 1.05 |

Observation 6.1.2.1-1 (time domain truncation of Nt)

For direct AI/ML positioning, the positioning accuracy at model inference tends to degrade as the time domain window size Nt decreases, where Nt consecutive time domain samples are used as model input. Evaluation results submitted to RAN1#113 show that show that when CIR or PDP are used as model input, using different Nt while holding other parameters constant,

- Positioning error of Nt=128 is 0.81 ~ 1.19 times the positioning error of Nt=256;

  - Thus: Reducing Nt from 256 to 128 does not appreciably degrade the positioning accuracy, while the measurement size and signaling overhead shrink to ½ of Nt=256.

- Positioning error of Nt=64 is 0.88 ~ 1.79 times the positioning error of Nt=256;

- Positioning error of Nt=32 is 1.05 ~ 4.29 times the positioning error of Nt=256;

  - Thus: Reducing Nt from 256 to 64~32 may degrade the positioning accuracy, while the measurement size and signaling overhead shrink to 1/4 ~1/8 of Nt=256, respectively.

  - In terms of achievable positioning accuracy, Nt can be reduced to 64~32 while maintaining positioning accuracy around or below 1 meter.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Not sure if we can capture these result so straight forwardly. Should we really only take the extreme cases to set the range? For example for E_32/E_256, the 4.29 seems to stick out from the rest of the numbers. |
| ZTE | Better to have different observations for CIR and PDP. The following comment may be applied to all the observations in the paper. Otherwise, it may be confused when reading the TR. Each observation should accompanied by the detailed assumptions: - Dataset generation for training - With/without generalization - Model input types - The values of {Nport, NTRP, Nt, Nt'} - Other details if necessary |
| CATT | CATT’s result is missing for this case (see Table 1 and Table 14 in R1-2304726). Add it back to the table. Though it seems the min and max is not changed. |

### Reduced number of taps as model input

- ZTE (R1-2304538) Observation 2. For direct AI/ML positioning by using PDP as model input, when the width of square grid is 1.0 m in grid distribution used for training dataset generation(InF-DH, Clutter parameters {60%, 6m,2m}), the N_t^' value should be larger than 32 in order to have sub-meter level positioning accuracy at 90% UEs. Observation 3. For direct AI/ML positioning by using PDP as model input, when the width of square grid is 1.0 m in grid distribution used for training dataset generation (InF-DH, Clutter parameters {60%, 6m,2m}), the N_t^' value should be larger than 8 in order to have sub-meter level positioning accuracy at 90% UEs. Observation 4. With the decrease in N_t^' value, positioning performances degrade significantly. In the following exemplary results, the positioning accuracy at 90% UEs is even smaller than the width of square grid in grid distribution used for training dataset generation (InF-DH, Clutter parameters {60%, 6m,2m}): - N_t^'=8,16,32,64 when the width of square grid is 1.0 m; - N_t^'=8 when the width of square grid is 0.5 m. Observation 5. AI/ML positioning accuracy is affected by user density/size of the training dataset (InF-DH, Clutter parameters {60%, 6m,2m}). When the training dataset generation follows the grid distribution, the positioning performance is increased in a smaller grid width. Exemplary results are: The sub-meter level positioning accuracy (i.e., 0.98 m) can be reached at N_t^'=16 when the width of square grid is 0.5 m. However, the positioning accuracy is 1.40m in the same settings when the width of square grid is 1.0 m.

- MediaTek Inc. (R1-2305659) Table 35. Evaluation results for AI/ML model deployed on UE or network-side without generalization, CNN, UE distribution area = [120x60 m], different N’t and Nt Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Nt; N’t; Train; Test; Model complexity; Computation complexity; AI/ML. CIR [18,2,256]; UE pos [x,y]; 0%; 256; 25; 32400; 3600; 464.24K; 0.266G; 0.940. 20; 0.915. 15; 1.017. 64; 25; 243.058K; 0.066G; 0.984. 20; 0.851. 15; 1.097. PDP [18,2,256]; UE pos [x,y]; 0%; 256; 25; 32400; 3600; 463.95K; 0.264G; 0.821. 20; 1.089. 15; 1.133. 64; 25; 242.770K; 0.066G; 0.942. 20; 0.965. 15; 1.057. Observation 44: By selecting appropriate Nt and N’t, the computational complexity, model complexity and signaling overhead can be reduced without significant performance loss.

- Ericsson (R1-2304339) Table 11 UE 2D positioning errors for {60%, 6m, 2m} test dataset at different percentiles and different numbers of down sampled taps (training dataset size = 40,000 samples, Nt=256). Model class; Positioning approach; Model complexity [# paras]; 90%tile 2D positioning error [m] with numbers of down sampled taps in {60%, 6m, 2m} InF-DH. Nt'=256; Nt'=128; Nt'=64; Nt'=32; Nt'=16; Nt'=9. Small models CIR; Dist. Assist.; 0.86 M; 0.453; 0.478; 0.558; 0.764; 1.208; 1.995. Cent. Assist.; 0.73 M; 0.371; 0.372; 0.434; 0.545; 0.678; 0.805. Cent. Direct; 0.73 M; 0.373; 0.382; 0.425; 0.540; 0.678; 0.824. Small models PDP; Dist. Assist.; 0.43 M; 0.680; 0.687; 0.732; 0.856; 1.001; 1.452. Cent. Assist.; 0.36 M; 0.524; 0.534; 0.514; 0.604; 0.698; 0.868. Cent. Direct; 0.36 M; 0.510; 0.522; 0.522; 0.580; 0.689; 0.824.

#### 1st round discussion

Evaluation results submitted by companies are summarized in Table 4 below for reduced number of taps (N't), where the full set of taps is Nt = 256. Please add/correct if anything in Table 4  is missing or wrong.

Table 4. Positioning error when Nt = 256 and N't varies

| N't | 256 | 128 | 64 | 32 | 25 | 20 | 16 | 15 | 9 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MediaTek R1-2305659, Table 35 |  |  |  |  | 0.94 | 0.915 |  | 1.017 |  |  |
| MediaTek R1-2305659, Table 35 |  |  |  |  | 0.821 | 1.089 |  | 1.133 |  |  |
| ZTE R1-2302538 Excel | 0.69 | 0.71 | 0.83 | 1.4 |  |  | 1.75 |  |  | 1.93 |
| ZTE R1-2302538 Excel | 0.42 | 0.45 | 0.51 | 0.63 |  |  | 0.98 |  |  | 1.38 |
| Ericsson R1-2304339, Table 11 | 0.373 | 0.382 | 0.425 | 0.54 |  |  | 0.678 |  | 0.824 |  |
| Ericsson R1-2304339, Table 11 | 0.51 | 0.522 | 0.522 | 0.58 |  |  | 0.689 |  | 0.824 |  |

Based on Table 4, Table 5 calculates the positioning error with a reduced N't over the positioning error with the full set of 256 taps.

Table 5. Positioning error of N't over positioning error of Nt = 256

| N't |  | E_128 /E_256 | E_64 /E_256 | E_32 /E_256 | E_25 /E_256 | E_20 /E_256 | E_16 /E_256 | E_15 /E_256 | E_9 /E_256 | E_8 /E_256 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MediaTek R1-2305659, Table 35 |  |  |  |  |  |  |  |  |  |  |
| MediaTek R1-2305659, Table 35 |  |  |  |  |  |  |  |  |  |  |
| ZTE R1-2302538 Excel |  | 1.03 | 1.20 | 2.03 |  |  | 2.54 |  |  | 2.80 |
| ZTE R1-2302538 Excel |  | 1.07 | 1.21 | 1.50 |  |  | 2.33 |  |  | 3.29 |
| Ericsson R1-2304339, Table 11 |  | 1.02 | 1.14 | 1.45 |  |  | 1.82 |  | 2.21 |  |
| Ericsson R1-2304339, Table 11 |  | 1.02 | 1.02 | 1.14 |  |  | 1.35 |  | 1.62 |  |
| max |  | 1.07 | 1.21 | 2.03 |  |  | 2.54 |  | 2.21 | 3.29 |
| min |  | 1.02 | 1.02 | 1.14 |  |  | 1.35 |  | 1.62 | 2.80 |

Based on Table 4 and Table 5, the following observation is drawn.

Observation 6.1.3.1-1 (size of sub-sampled taps N't)

For direct AI/ML positioning, the positioning accuracy at model inference tends to degrade as N't decreases, where N't time domain samples (i.e., paths) with the strongest power are selected as model input. Evaluation results submitted to RAN1#113 show that for model input of CIR or PDP and Nt=256, using different N't while holding other parameters constant,

- Positioning error of N't=64 is 1.02 ~ 1.21 times the positioning error of Nt=N't=256;

  - Thus: Reducing N't from 256 to 64 does not appreciably degrade the positioning accuracy, while the measurement size and signaling overhead shrink to (approximately) ¼ of Nt=N't=256.

- Positioning error of N't=32 is 1.14 ~ 2.03 times the positioning error of Nt=N't=256;

- Positioning error of N't=16 is 1.35 ~ 2.54 times the positioning error of Nt=N't=256;

  - Thus: Reducing N't from 256 to 32~16 may degrade the positioning accuracy, while the measurement size and signaling overhead shrink to (approximately) 1/8 ~ 1/16 of Nt=N't=256.

  - In terms of achievable positioning accuracy, N't can be reduced to 32~16 while maintaining positioning accuracy around or below 1 meter.

- Positioning error of N't=9~8 is 1.62 ~ 3.29 times the positioning error of Nt=N't=256;

  - Thus: Reducing N't from 256 to 9~8 degrade the positioning accuracy, while the measurement size and signaling overhead shrink to (approximately) 1/32 of Nt=N't=256.

  - In terms of achievable positioning accuracy, N't can be reduced to 9~8 while maintaining positioning accuracy in the range of 1 to 2 meters.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| ZTE | Similar comments as Observation 6.1.2.1-1. |

### Reduced number of TRPs

| - Qualcomm (R1-2305332) Table 3 Evaluation results for AI/ML model complexity and reporting overhead when deployed on NW-side for different TRP measurement optimizations (Bt=8 bits, Nt=64, Bcir_real=8bits, Bloc=12 bits), without model generalization, CNN, UE distribution area = 120x60 m Model input (N’TRP , Nport , N’t); Model output; Label; Setting ({60%, 6, 2}); Dataset size; AI/ML complexity; Reporting/signalling complexity (bits); Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Training; test; Model complexity [parameters]; Computational complexity; AI/ML. Case2b: CIR (18,4, 8); 2D; 0%; Drop A; Drop A; 15k; 2k; 1.5M; 1.54G FLOPs; 13824 bits; 3.17. Case2b: CIR (8,4, 8) [dynamic TRP selection]; 2D; 0%; Drop A; Drop A; 15k; 2k; 1.5M; 1.54G FLOPs; 6144 bits; 3.89. Case2b: CIR (4,4, 8) [dynamic TRP selection]; 2D; 0%; Drop A; Drop A; 15k; 2k; 1.5M; 1.54G FLOPs; 3072 bits; 5.32. Case2b: CIR (8,4, 8) [Fixed TRP selection: TRPs= {0,2,6,8,9, 11, 15,17}]; 2D; 0%; Drop A; Drop A; 15k; 2k; 1.5M; 1.54G FLOPs; 6144 bits; 3.34. Case2b: CIR (4,4, 8) [Fixed TRP selection: TRPs= {0,2,15,17}]; 2D; 0%; Drop A; Drop A; 15k; 2k; 1.5M; 1.54G FLOPs; 3072 bits; 12.02. Case2b: CIR (18,4, 400); 2D; 0%; Drop A; Drop A; 15k; 2k; 1.5M; 1.54G FLOPs; 489600 bits; 2.24. Observation 8: For Case2b in AI/ML positioning, reducing the number of reported TRPs degrades the positioning accuracy; in dynamic TRP selection with CIR measurement type, the positioning accuracy drops from 3.17m to 3.89m and 5.32m, when the number of TRPs reduces from 18 to 8 and 4, respectively; in fixed TRP selection with CIR measurement type, the positioning accuracy drops from 3.17m to 3.34m and 12.02m, when the number of TRPs reduces from 18 to 8 and 4, respectively. |
| --- |
| - MediaTek Inc. (R1-2305659) Observation 45: The performance of direct AI/ML positioning decreases with the decrease of NTRP. Observation 46: For reduced number of TRP evaluation, the direct AI/ML performance of Approach 2 (one model) is better than that of Approach 1-B. Proposal 12 : For reduced number of TRP evaluation, Approach 2 is recommended because in addition to better performance than Approach 1, it is also superior to Approach 1 in terms of reducing model complexity and computational complexity. |  |
| - vivo (R1-2304475) Approach 2 Observation 50: The loss of positioning accuracy is negligible when reducing the number of TRPs from 18 to 4, indicating that the information contained in the CIRs of 4 TRPs is sufficient to determine the UEs’ location under the given simulation assumption. Observation 51: Reducing the number of TRPs from 4 to 2 results in a relatively large performance loss, indicating that the information contained in the CIRs of 2 TRPs is insufficient to achieve a high-accuracy positioning. |
| - Indian Institute of Tech (M), IIT Kanpur (R1-2305689) Observation 2: If (NTRP - N’TRP) is small, there is only a small difference between approaches 1A and 2. Observation 3: If (NTRP - N’TRP) is large, then Approach 2 is better than Approach 1A. Observation 4: If (NTRP - N’TRP) is random, then Approach 1B is better than Approach 2. |  |
| - Nokia (R1-2304685) Figure 10 - Positioning accuracy vs. IPD for 18, 9, and 6 TRPs cases. Table 18 – For approach 1 - Evaluation for different number of TRPs (N_TRPs) considering an arbitrarily TRPs selection and compared with their respective model complexity and computational complexity. The input parameter is PDP, the N’t =128, UE distribution area = 120x60 m and using a highest computational complexity than Table 17. N_TRPs (input parameter fixed = 18); 4 TRPs with real values and 14 TRPs set zero; 8 TRPs with real values and 10 TRPs set zero; 12 TRPs with real values and 6 TRPs set zero; 16 TRPs with real values and 2 TRPs set zero; 18 TRPs with real values and 0 TRPs set Zero. Model complexity (parameters); 466.9K; 466.9K; 466.9K; 466.9K; 466.9K. computational complexity (flops); 6390M; 6390M; 6390M; 6390M; 6390M. Horizontal 2D error at CDF 90% (meters); 53.17; 26.94; 17.3; 1.39; 0.732. Table 20 – For approach 2 - Evaluation for different number of TRPs (N_TRPs) considering an arbitrarily TRPs selection and compared with their respective model complexity and computational complexity. The input parameter is PDP, the N’t =128, UE distribution area = 120x60 m and using a highest computational complexity than Table 19. N_TRPs; 4; 8; 12; 16; 18. Model complexity (parameters); 462.8K; 464K; 465.2K; 466.3K; 466.9K. computational complexity (flops); 2490M; 3600M; 4720M; 5830M; 6390M. Horizontal 2D error at CDF 90% (meters); 1.3106; 1.13219; 1.0132; 0.9266; 0.6838. Observation 18: The horizontal performance is enhanced when the number of TRPs is increased at the expense of the computational complexity. |
| - Apple (R1-2305973) Observation 4: For direct AI-ML positioning, as the # of TRPs reduces, the positioning performance suffers. It may be possible to regain some of those losses by mixing different input types e.g. CIR for X TRPs and L1-RSRP for Y TRPs Proposal 2: Study the performance using mixed inputs |  |
| - Huawei (R1-2304657) - : For direct AI/ML positioning, when the AI/ML model input is CIR (one CIR per TRP) and different numbers of TRPs are evaluated, - The positioning accuracy decreases slightly when the number of TRPs is reduced from 18 to 4. But to maintain sub-meter level accuracy, the number of TRPs should be at least 4. - Model and computational complexity decrease significantly with a smaller number of TRPs. The model complexity is reduced by more than 50% and computational complexity about 75% when reducing the number of TRPs from 18 to 4. - When Approach 2 is applied, using different sets of TRPs for model input may lead to slightly different but still comparable positioning performance. |  |
| - CATT (R1-2304726) - 3.1.6.1.1. One model covers entire area The first case is to select 9 fixed TRPs and train a model to cover the entire simulation area. In inference phase, this model is used to cover the entire evaluation area. In this simulation, since NTRP = 9, the measurement size is NTRP*Nport*NT*2*64 = 147456bit per UE. In our first design, Approach 2 is applied, i.e. the TRP dimension of model input is equal to the number of TRPs (N’TRP) that provides measurements as model input. In our second design, Approach 1-A is applied, i.e. 9 fixed TRPs provides the effective input value to the model and the input of the other 9 TRPs are zero-padded. Observation 11: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the AI/ML model performance will not significantly degrade. Observation 12: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the FLOPs can be significantly reduced. 3.1.6.1.2. Two models, each covers half the area The second case is to divide the original area into two regions with equal size, and two models are trained based on the region-specific dataset separately. In inference phase, each model is used to cover the corresponding region, and the whole evaluation area can be covered by these two models. In our design, Approach 2 is applied, Observation 13: For direct AI/ML positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs and the total dataset size is unchanged, the horizontal position accuracy of using two models to cover the entire area is slightly degraded compared to the performance of using single model to cover the entire area. |  |
| - Ericsson (R1-2304339) Figure 13: Positioning accuracy vs number of active TRP (N'TRP) for centralized ML assisted positioning and centralized ML direct positioning. |  |

### Other

- vivo (R1-2304475) Figure 77 CDF of positioning accuracy of multi-port positioning and single-port positioning. Observation 40: Positioning with multi-port data can achieve a more accurate position estimation as compared to single-port positioning.

- MediaTek Inc. (R1-2305659) Observation 27: The evaluation results shown that the positioning performance of 2 transmit antenna ports (by different polarization) is better than the existing 1 antenna port in the spec. Proposal 9 : Study and evaluate the performance of direct AI/ML positioning with multiple transmit/receive antenna port pairs (for example, 2 ports with different polarization).

- ZTE (R1-2304538) Observation 7. When the model input includes channel observation from multi-port PRS, better positioning performance can be observed when compared to single port PRS. Exemplary results are: - When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of single port PRS is about 1.27 times that of multi-port PRS at N_t^'=256 for PDP based positioning; - When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of single port PRS is about 1.58 times that of multi-port PRS at N_t^'=256 for CIR based positioning; Observation 8. Due to the enriched channel observations from multi-port PRS, positioning performance is improved under a given number of data samples for model training, which can relief the efforts to collect training data in reality. Proposal 2. Study and identify the benefits of AI/ML based positioning when the model input includes channel observations from multi-port PRS. Proposal 3. When multi-port PRS is evaluated for AI/ML based positioning, companies should report its assumption on the number of ports for a single data sample used in model input.

- Samsung (R1-2305509) Fig.13 illustration of complexity reduction and Data size reduction Observation 9: SIG based input could adopt better (e.g., 7~8 times better in pos accuracy) with simple AI/ML model (e.g., MLP) than CIR based input. Observation 10: the SIG-based input could drastically reduce the input data size and the complexity (e.g., with 98% reduction) without accuracy loss (e.g., even with 1% improvement).

# Impact of direct AI/ML positioning performance by model output

## Evaluation of noisy ground truth labels

| - vivo (R1-2304475) 9.1. Data cleaning Table 81 Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, ViT, UE distribution area = [120x60 m] Model input; Model output; Threshold T m; Training samples; Settings (e.g., drops, clutter param, mix); Horizontal pos. accuracy at CDF=90% (m). Total samples before data cleaning; Retained samples for training; Train; Test; AI/ML. CIR; Pos.; T = 1m; 25k; 2.4k; L = 4; L = 0; 5.58. CIR; Pos.; T = 2m; 25k; 5.0k; L = 4; L = 0; 3.58. CIR; Pos.; T = 3m; 25k; 7.5k; L = 4; L = 0; 3.30. CIR; Pos.; T = 4m; 25k; 10k; L = 4; L = 0; 3.58. CIR; Pos.; T = 8m, i.e., w/o data cleaning; 25k; 25k; L = 4; L = 0; 5.90. 9.2. Sample-wise weighting for model training Table 82 Evaluation results for AI/ML model deployed on UE or Network side, without model generalization, ViT, UE distribution area = [120x60 m] Model input; Model output; Hyper-parameter a; Dataset; Settings (e.g., drops, clutter param, mix); Horizontal pos. accuracy at CDF=90% (m). Training dataset; Testing dataset; Train; Test; AI/ML. CIR; Pos.; a = 0.1; 25k; 1k; L = 4; L = 0; 3.53. CIR; Pos.; a = 0.5; 25k; 1k; L = 4; L = 0; 3.39. CIR; Pos.; a = 1.0; 25k; 1k; L = 4; L = 0; 3.18. CIR; Pos.; a = 1.5; 25k; 1k; L = 4; L = 0; 3.12. CIR; Pos.; a = 2.0; 25k; 1k; L = 4; L = 0; 3.19. CIR; Pos.; a = 3.0; 25k; 1k; L = 4; L = 0; 3.77. CIR; Pos.; a = 0.0 (no weighting); 25k; 1k; L = 4; L = 0; 5.90. 9.3. Model training with estimated labels Table 83 Evaluation results for AI/ML model deployed on UE or Network side, UE distribution area = [120x60 m] Model input; Model output; Number of samples with ground truth labels; Number of samples with noisy labels; Usage of label quality; Legacy positioning method to generate noisy labels; Horizontal pos. accuracy at CDF=90% (m). CIR; Pos.; 25k; 0; /; /; 0.89. CIR; Pos.; 5k; 0; /; /; 3.73. CIR; Pos.; 5k; 20k; Sample-wise weighting; RSTD based; 2.23. CIR; Pos.; 5k; 20k; Sample-wise weighting; TOA based; 1.72. CIR; Pos.; 5k; 0; /; Semi-supervised learning; 2.78. |
| --- |
| - OPPO (R1-2305463) Impact of noisy labels based on NR positioning Observation 10: For the InF-DH scenario, if the label for the training data set of AI model is obtained by traditional NR DL-TDOA scheme, the AI model inference performance for the case will suffer larger performance loss - Compared to the traditional NR DL-TDOA scheme, the AI model doesn’t show obvious performance gain in this case. Observation 11: For the InF-DH scenario, if the number of total training data is fixed, the AI model inference performance will degrade as the ratio of the labels that are obtained by traditional NR DL-TDOA scheme increases Observation 12: For the InF-DH scenario, when the number of training data with ideal label is fixed, the AI model inference performance will degrade if training data with the labels that are obtained by traditional NR DL-TDOA scheme are added to the training data set. Impact of noisy labels noisy labels based on truncated Gaussian modeling Observation 14: For the InF-DH scenario, if the label for the training data set of AI model is obtained by truncated Gaussian modeling, the AI model inference performance cannot reflect that of a practical AI model (e.g., an AI model trained by labels based on NR positioning - The estimation errors of NR positioning are correlated for the close UEs due to the spatial consistency, whereas the error of truncated Gaussian modelling are independent even for collocated UEs. |  |
| ## Qualcomm (R1-2305332) Observation 9: For data labelling in AI/ML positioning, evaluations show it is feasible to consider data labelling using NR RAT-positioning methods when the number of clean labelled data is limited. The performance of direct AI/ML positioning with 1k clean labelled samples improves from 13.76m to 8.72m when considering additional 350 samples that are labelled using NR-RAT positioning method. Table 4 Evaluation results for feasibility of labelling using NR RAT positioning methods when deployed on UE- or NW-side, without model generalization, CNN, UE distribution area = 120x60 m Model input (N’TRP , Nport , N’t); Model output; Label; Setting ({60%, 6, 2}); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Train; Test; Training; test; Model complexity [parameters]; Computational complexity; AI/ML. CIR (18,4, 400); 2D; 0%; Drop A; Drop A; 1k clean labels; 2k; 1.5M; 1.54G FLOPs; 13.76. CIR (18,4, 400); 2D; 25.9% noisy label; Drop A; Drop A; 1k clean+ 350 noisy labels (NR RAT positioning); 2k; 1.5M; 1.54G FLOPs; 8.72. |
| - MediaTek Inc. (R1-2305659) Observation 47: Performance of direct AI/ML positioning degrades with increasement the of labelling error. Observation 48: When the standard deviation of label error is less than 0.2, the positioning accuracy can reach the sub-meter level with the UE density 4.5UEs/m^2 in the scenario InF-DH({60%, 6m, 2m}). |
| - vivo (R1-2304475) - Figure 40 Evaluation of the impact of labeling error on positioning accuracy - Observation 21: The positioning accuracy gradually degrades with the increase of labeling error, but is still acceptable until standard deviation is 1 m. The maximum acceptable labeling errors (standard deviation) in the horizontal direction should be less than 1m to achieve 2m@90% positioning accuracy. - Observation 22: AI/ML based positioning is robust to label noise to some extent. - Proposal 8: According to the requirement of positioning accuracy, the maximum acceptable labeling error should be identified firstly before data collection |  |
| - CMCC (R1-2305089) Observation 6: The positioning error may not increase approximately in proportion to L, if the model of the label error generation changed |
| - CEWiT (R1-2305896) Observation-9: It is observed that AI/ML based positioning is sensitive to labelling noise. Therefore it is important to consider and identify the labelling error before the data collec-tion. |
| - NVIDIA (R1-2305164) Figure 10: Positioning accuracy of AI/ML based method under different degrees of label errors. |
| - Nokia (R1-2304685) Table 10 - Evaluation of ground truth labelling error with different noise standard deviation and their impact on the horizontal 2D error at CDF 90% for N_TRP=18. The error is indicated using the mean and the variance between 20 different realizations of the same experiment (UE distribution area = 120x60 m). Standard deviation of L meters.; 0.0; 0.5; 1.0; 1.5; 2.0; 2.5; 3.0. Horizontal 2D error at CDF 90% (meters); 0.7; 0.8617; 1.0268; 1.4153; 1.7279; 1.8259; 1.9751. Observation 12: In the evaluation, Direct AI/ML positioning is robust to label noise with Gaussian distribution. However, modeling this error type in real-world scenarios could be a challenge. |
| - InterDigital (R1-2305123) Observation 4: For direct AI/ML positioning with RSRP measurements as model input, when different labelling errors(L) are evaluated, • For labelling error values less than or equal to 0.5 m, the horizontal positioning accuracy degrades slightly (less than ~ 0.03 m) for 90% of the UEs. • For labelling error values greater equal to 1 m, the horizontal accuracy degrades in proportion to labelling error (L) for 90% of the UEs. Observation 5: For direct AI/ML positioning with RSRP+ RSTD measurements as model input, when different labelling errors(L) are evaluated, • For labelling error values less than or equal to 0.25 m, the horizontal positioning accuracy degrades slightly (less than ~ 0.05 m) for 90% of the UEs. • For labelling error values greater than or equal to 0.5 m, the horizontal accuracy degrades in proportion to labelling error (L) for 90% of the UEs. Observation 6: For direct AI/ML positioning with CIR measurements as model input (larger model in table 4), when different labelling errors(L) are evaluated, • For labelling error values equal to 0.05 m, the horizontal positioning accuracy degrades slightly (~ 0.12 m) for 90% of the UEs. • For labelling error values greater than or equal to 0.5 m, the horizontal accuracy degrades in proportion to labelling error (L) for 90% of the UEs. Observation 7: For direct AI/ML positioning with CIR measurements as model input (less complex model in table 5), when different labelling errors(L) are evaluated, • For labelling error values equal to 0.05 m, the horizontal positioning accuracy degrades ~ 0.31 m for 90% of the UEs. • For labelling error values greater than or equal to 0.5 m, the horizontal accuracy degrades significantly (greater than ~0.39 m) which is in proportion to labelling error(L) for 90% of the UEs. Observation 8: For direct AI/ML positioning, for L in the range of 0.25 m to 2 m, degradation in positioning accuracy (due to labelling error) increase in proportion to L. Observation 9: For direct AI/ML positioning, for different model inputs (RSRP, RSRP+RSTD and CIR) while keeping the same amount of labelling error, we observe different values of positioning accuracy degradation. Observation 10: For direct AI/ML positioning, for different model complexity (more complex and lesser complex model), while keeping model inputs (CIR) and labelling error same, we observe different values of positioning accuracy degradation. |
| - Apple (R1-2305973) Figure 4: Performance Accuracy vs label accuracy Observation 6: • label error that is on the order of (less than or equal to) the positioning accuracy without label errors does not hurt (or even helps) the overall performance. • Label error larger than the order of the performance error hurts overall performance as expected. Proposal 4: • The expected label error is needed during data collection for training and for performance monitoring. |
| - CATT (R1-2304726) Observation 10: When AI/ML model is trained with different value of label error, as value of L increases, direct AI/ML positioning error increases approximately in proportion to L. |
| - Ericsson (R1-2304339) Figure 14: Positioning accuracy vs label error standard deviation L (meters) for three positioning approaches and model input type of CIR, PDP, and DP. |

## Semi-supervised learning

- MediaTek Inc. (R1-2305659) Table 34. Evaluation results for AI/ML model deployed on UE or network-side without generalization, CNN, UE distribution area = [120x60 m], semi-supervised learning Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR [18,2,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400 labelled; 3600; 464.24K; 0.266G; 0.940. 0%; 7200 labelled; 7200; 2.018. 75%; 7200 labelled + 21600 unlabelled; 7200; 1.722. PDP [18,2,256]; UE pos [x,y]; 0%; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400 labelled; 3600; 463.95K; 0.264G; 0.821. 0%; 7200 labelled; 7200; 2.213. 75%; 7200 labelled + 21600 unlabelled; 7200; 2.070.

- vivo (R1-2304475) Figure 74 Positioning accuracy comparison of semi-supervised learning and supervised learning with different numbers of labeled samples Observation 39: Semi-supervised learning can achieve a more accurate position estimation as compared to supervised learning with less amount of labeled data.

- ZTE (R1-2304538) Observation 19: Semi-supervised learning (i.e., using a large number of unlabeled samples to pretrain a model and the model is further fine-tuned by a small number of labeled samples) outperforms supervised learning (e.g., training from scratch) under the same am of labeled samples for model training. Exemplary results are: • The positioning error of supervised learning based on 2K labeled samples is about 2.29 times that of semi-supervised learning based on 100K unlabeled samples and 2K labeled samples (i.e., 98% data samples are unlabeled); • The positioning error of supervised learning based on 4K labeled samples is about 1.96 times that of semi-supervised learning based on 100K unlabeled samples and 4K labeled samples (i.e., 96% data samples are unlabeled); • The positioning error of supervised learning based on 8K labeled samples is about 1.37 times that of semi-supervised learning based on 100K unlabeled samples and 8K labeled samples (i.e., 93% data samples are unlabeled); Observation 20: As the increase of labeled samples used for model training, semi-supervised learning has a similar performance as supervised learning. Exemplary results are: • The positioning error of supervised learning based on 16K labeled samples is similar as that of semi-supervised learning based on 100K unlabeled samples and 16K labeled samples (i.e., 86% data samples are unlabeled). Proposal 4: Study and identify the benefits of semi-supervised learning for AI/ML based positioning.

- Ericsson (R1-2304339) Observation 27 For a dataset with both labeled and unlabeled samples, semi-supervised learning is a viable solution to utilize both labeled and unlabeled samples for training the AI/ML models. However, semi-supervised learning training requires more computational complexity than supervised learning training. Observation 28 It is not clear that semi-supervised learning training using both labeled and unlabeled samples brings clear improvement over well-designed supervised learning training using only the limited labeled samples. Table 17 90%tile 2D positioning accuracy using PDP inputs for AI/ML direct positioning approach and different unlabeled sample percentages in the {60%, 6m, 2m} InF-DH dataset. Input; Unlabeled %; Labeled; Unlabeled; Total; M paras; M FLOPs; 90%tile [m]. PDP; 97.50%; 1,000; 39,000; 40,000; 1.4; 34; 3.206. PDP; 97.50%; 2,000; 78,000; 80,000; 1.4; 34; 2.038. PDP; 95.00%; 1,000; 19,000; 20,000; 1.4; 34; 3.261. PDP; 95.00%; 2,000; 38,000; 40,000; 1.4; 34; 2.059. PDP; 95.00%; 4,000; 76,000; 80,000; 1.4; 34; 1.309. PDP; 90.00%; 1,000; 9,000; 10,000; 1.4; 34; 3.324. PDP; 90.00%; 2,000; 18,000; 20,000; 1.4; 34; 2.164. PDP; 90.00%; 4,000; 36,000; 40,000; 1.4; 34; 1.348. PDP; 90.00%; 8,000; 72,000; 80,000; 1.4; 34; 0.863. PDP; 80.00%; 4,000; 16,000; 20,000; 1.4; 34; 1.357. PDP; 80.00%; 8,000; 32,000; 40,000; 1.4; 34; 0.906. PDP; 80.00%; 16,000; 64,000; 80,000; 1.4; 34; 0.626.

## 1st round discussion

On label error, observation can be drawn based on evaluation results to RAN1#113 together with the agreement from RAN1#112bis.

Observation (RAN1#112bis) For direct AI/ML positioning, for L in the range of 0.25m to 5m, the positioning error increases approximately in proportion to L, where L (in meters) is the standard deviation of truncated Gaussian Distribution of the ground truth label error.

Observation 7.3-1

Evaluation shows that direct AI/ML positioning is robust to small label error. The exact range of label error that can be tolerated depends on the positioning accuracy requirement,  where tighter positioning accuracy requirement demands smaller label error.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| LG | Fine with the observation |
| ZTE | Ok with the general observation. It would be better to have more detailed results. In addition, do we have a separate proposal for semi-supervised learning. Our evaluation results are not captured. |
| OPPO | Generally fine with the observation. But the description in the observation is very general, we think it would be better to add evaluation results to show the relation between output accuracy and label error. |

# Evaluation of generalization issues of AI/ML-assisted positioning

In this meeting, a large amount of evaluation work has been performed by companies for AI/ML-assisted positioning. These valuable results are crucial to help RAN1 to make progress.

Representative results submitted by companies are copied below.

## Evaluation of single-TRP construction with same model for N TRP

### Evaluation without generalization considerations (same setting for training and testing)

- MediaTek Inc. (R1-2305659) Table 8. Evaluation results for AI/ML model deployed on UE or network side, without model generalization, CNN, UE distribution area = 120x60 m Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training & validation; test; Model complexity; Computational complexity; AI/ML. non-normalized PDP (1*8*256); 1TOA; Ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 187k; 15M*18; 20.1. non-normalized PDP (18*8*256); 18TOA; Ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 1.05. non-normalized PDP (1*8*256); 1TOA; Ideal; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 187k; 15M*18; 14.9. non-normalized PDP (18*8*256); 18TOA; Ideal; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 205k; 77M; 1.56. Observation 11: Multi-TRP approach outperforms single-TRP for AI/ML assisted TOA estimation positioning.

- ZTE (R1-2304538) Observation 23: The AI/ML assisted positioning method can achieve a good accuracy rate in LOS/NLOS identification. Exemplary results are: • When the training samples are 60K (InF-DH, Clutter parameters: {40%, 2m, 2m}), Los/NLos accuracy rate by using CIR from single TRP as model input is about 94.7%. Proposal 7: Study and evaluate the performance of AI/ML assisted positioning where the model output includes confidence level of LOS/NLOS identification.

- OPPO (R1-2305463) Observation 3: For AI/ML assisted positioning • The performance of “Indirect: Normalized CIR of single TRP” is much worse than “Indirect: Normalized CIR of all TRPs” • The performance of “Indirect: Normalized CIR of single TRP” is comparable to the traditional non-AI scheme Proposal 1: For AI/ML assisted positioning, if the output of AI model is timing-based result (e.g., TOA), prioritize the scheme where the measurement results corresponding to all TRPs are used as the input of AI model • the scheme where the measurement results corresponding to single TRP are used as the input of AI model is deprioritized.

- Huawei (R1-2304657) Observation 27 : From the evaluation results, it is observed that for a small number of gNB antennas, AI/ML-based LOS/NLOS identification could significantly improve the positioning accuracy. Observation 28 : For AI/ML-based LOS/NLOS identification evaluation, the applied model only needs very small number of parameters and does not require tremendous FLOPs. Observation 32 : From the evaluation results, it is observed that AI/ML-based TOA estimation could significantly improve the positioning accuracy in NLOS environments both in Multi-TRP and Single-TRP construction.

- vivo (R1-2304475) Table 5 Evaluation results for AI/ML model deployed on UE or Network side, UE distribution area = [120x60 m] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR w. BS info.; TOA; 0; Drop1; Drop1; 25k*18; 1k*18; 11.92M*1; 23.79M*1; 0.83. CIR w/o. BS info.; TOA; 0; Drop1; Drop1; 25k*18; 1k*18; 11.92M*1; 23.79M *1; 2.57. Observation 4: For Construction 2 (Single-TRP, same model for N TRPs), it is beneficial to incorporate TRP’s information into model input so as to improve the positioning accuracy.

### Generalization aspects (different setting for training and testing)

#### Different drops

- Qualcomm (R1-2305332) Observation 23: For AI/ML assisted positioning, evaluations show that the ML-based soft information reporting method using single-TRP approach generalizes well across inter-site changes with homogeneous clutter settings. Figure 21 CDF of horizontal positioning error for ML-based soft information reporting across drops

- ZTE (R1-2304538) Observation 26: For model generalization evaluation on AI/ML assisted LOS/NLOS identification, • The model performance can generalize well to a test dataset that is in a different simulation drop of training dataset but in the same InF-DH scenario (clutter parameters {40%, 2m, 2m}); • The model performance degrades slightly when the training data is in InF-DH scenario (clutter parameters {40%, 2m, 2m}) and test data is in InF-SH scenario (clutter parameters {30%, 2m, 2m}). Proposal 8: To evaluate model performance on AI/ML assisted LOS/NLOS identification, the evaluation assumptions for dataset generation should have a moderate LOS probability (e.g., Clutter parameters {40%, 2m, 2m} for InF-DH scenario and clutter parameters {30%, 2m, 2m} for InF-SH scenario)

- OPPO (R1-2305463) Observation 4: For the InF-DH scenario, if the training and testing data sets for AI model training and testing are generated from different drops, there will be large performance degradation for AI-based positioning.

- Huawei (R1-2304657) Observation 33 : When the inference dataset and the training dataset are from different drops, AI/ML-based TOA estimation model provides poor generalization performance.

#### Different clutter parameters

- Qualcomm (R1-2305332) Observation 24: In AI/ML assisted positioning, evaluations show that training on a mix of clutter settings achieves good accuracy in each setting without the overhead of model switching, while training a separate model for each setting provides better accuracy. - The 90th percentile error increases from 4.74 m to 7.34 m when testing on (60%, 6m, 2m) clutter, and - from 0.53 m to 0.91 m when testing on (40%, 2m, 2m) clutter

- OPPO (R1-2305463) Observation 5: For the InF-DH scenario, if the training and testing data sets for AI model training and testing are generated with different clutter settings, there will be large performance degradation for AI-based positioning. Observation 6: For the InF-DH scenario, by training AI model based on the mixed data set with different clutter settings, the performance of AI model inference for the data set with one of these clutter settings can be improved.

- Huawei (R1-2304657) Observation 29 : When the channel parameters of the inference dataset and the training dataset are different, AI/ML-based LOS/NLOS identification model provides great generalization performance of the intermediate LOS/NLOS identification rate. Observation 34 : When the channel parameters of the inference dataset and the training dataset are different, AI/ML-based TOA estimation model provides poor generalization performance.

- CATT (R1-2304726) Observation 30: For the AI/ML-assisted positioning with LOS/NLOS identification, the generalization performance with different clutter parameters and different scenarios is good.

#### Network synchronization error

- MediaTek Inc. (R1-2305659) Table 5. Evaluation results for AI/ML model deployed on UE or network side, CNN, UE distribution area = 120x60 m Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; LOS/NLOS identification accuracy. Training {40%, 2m, 2m}; Test {40%, 2m, 2m}; Training & validation; Test; Model complexity; Computational complexity; AI/ML. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; 50ns; 50ns; 32400; 3600; 185.7k; 29.4M*18; 93.3%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; 20ns; 32400; 3600; 185.7k; 29.4M*18; 93.9%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; 0ns; 32400; 3600; 185.7k; 29.4M*18; 93.9%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; 20ns; 50ns; 32400; 3600; 185.7k; 29.4M*18; 93.2%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; 20ns; 32400; 3600; 185.7k; 29.4M*18; 94.2%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; 0ns; 32400; 3600; 185.7k; 29.4M*18; 94.4%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; 0ns; 50ns; 32400; 3600; 185.7k; 29.4M*18; 74.2%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; 20ns; 32400; 3600; 185.7k; 29.4M*18; 75.4%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; 0ns; 32400; 3600; 185.7k; 29.4M*18; 95.2%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; Mix 50ns and 20ns; 50ns; 32400; 3600; 185.7k; 29.4M*18; 93.1%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; 20ns; 32400; 3600; 185.7k; 29.4M*18; 93.8%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; 0ns; 32400; 3600; 185.7k; 29.4M*18; 93.8%. Observation 6: Performance of AI/ML assisted LOS/NLOS identification positioning significantly degrades when the model is trained with small timing error and tested with large timing error. Observation 7: AI/ML model in AI/ML assisted LOS/NLOS identification positioning has robust generalization capability when the model is trained with large timing error and tested with small timing error.

- OPPO (R1-2305463) Observation 7: For the InF-DH scenario, if the training data set of AI model is generated without NW synchronization error, the AI model performance for the case with NW synchronization error will suffer larger performance loss • In this case, the AI/ML assisted positioning scheme “Indirect: Normalized CIR for all TRPs” achieves similar performance as the traditional non-AI based scheme.

#### UE/gNB RX and TX timing error

- OPPO (R1-2305463) Observation 8: For the InF-DH scenario, if the training data set of AI model is generated without UE timing error, the AI model inference performance for the case with UE timing error will suffer larger performance loss • In this case, the AI/ML assisted positioning scheme “Indirect: Normalized CIR for all TRPs” is more sensitive to the UE timing error, e.g., its performance is even worse than the traditional non-AI based scheme

#### Different InF scenarios

- MediaTek Inc. (R1-2305659) Table 3. Evaluation results for AI/ML model deployed on UE or network side, CNN, UE distribution area = [DH 120x60 m, SH 300x150 m] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; LOS/NLOS identification accuracy. Training; test; Training & validation; test; Model complexity; Computational complexity; AI/ML. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; InF-DH {40%, 2m, 2m}; InF-DH {40%, 2m, 2m} same drop; 32400; 3600; 185.7k; 29.4M*18; 95.2%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; InF-DH {40%, 2m, 2m} new drop; 32400; 3600; 185.7k; 29.4M*18; 93.0%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; InF-DH {60%,6m, 2m}; 32400; 3600; 185.7k; 29.4M*18; 97.2%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; InF-SH hall size 300x150; 32400; 3600; 185.7k; 29.4M*18; 60.3%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; Mix InF-DH {40%, 2m, 2m} and InF-SH; InF-DH {40%, 2m, 2m}; 32400; 3600; 185.7k; 29.4M*18; 93.7%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; InF-SH hall size 300x150; 32400; 3600; 185.7k; 29.4M*18; 92.9%. Observation 3: Performance of AI/ML assisted LOS/NLOS identification positioning degrades when the model is trained with dataset of InF-DH scenario and tested with dataset of InF-SH large hall size scenario. And the performance is improved when mix InF-DH and InF-SH training data.

- Huawei (R1-2304657) Observation 30 : When the model is trained under the InF-DH scenario but inferred under the InF-SH scenario, the performance deteriorates significantly. But when trained under the InF-SH scenario and inferred under the InF-DH scenario, the AI/ML-based LOS/NLOS identification model provides great generalization performance of the intermediate LOS/NLOS identification rate. Observation 31 : When the mixed training dataset of InF-DH and InF-SH consists of samples with the same scenarios as the inference dataset, the performance of the intermediate LOS/NLOS identification rate is improved under both scenarios’ inference.

#### Time varying changes

- Qualcomm (R1-2305332) Observation 25: In AI/ML assisted positioning, evaluations show that the ML-based soft information reporting using single-TRP approach has good robustness to zone-specific time varying changes. Figure 23. CDF of horizontal positioning error for ML-based soft information reporting across time varying changes in a specific zone.

#### Channel estimation error

- MediaTek Inc. (R1-2305659) Table 4. Evaluation results for AI/ML model deployed on UE or network side, CNN, UE distribution area = 120x60 m, channel estimation error Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; LOS/NLOS identification accuracy. Training {40%, 2m, 2m}; Test {40%, 2m, 2m}; Training & validation; test; Model complexity; Computational complexity; AI/ML. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR=0dB; SNR=0dB; 32400; 3600; 185.7k; 29.4M*18; 95.9%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR =10dB; 32400; 3600; 185.7k; 29.4M*18; 91.1%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR =20dB; 32400; 3600; 185.7k; 29.4M*18; 90.9%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; Without noise; 32400; 3600; 185.7k; 29.4M*18; 90.9%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR=10dB; SNR=0dB; 32400; 3600; 185.7k; 29.4M*18; 75%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR =10dB; 32400; 3600; 185.7k; 29.4M*18; 94.3%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR =20dB; 32400; 3600; 185.7k; 29.4M*18; 93.9%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; Without noise; 32400; 3600; 185.7k; 29.4M*18; 93.8%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR=20dB; SNR=0dB; 32400; 3600; 185.7k; 29.4M*18; 56.9%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR =10dB; 32400; 3600; 185.7k; 29.4M*18; 84.3%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR =20dB; 32400; 3600; 185.7k; 29.4M*18; 95.1%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; Without noise; 32400; 3600; 185.7k; 29.4M*18; 95.1%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; Without noise; SNR=0dB; 32400; 3600; 185.7k; 29.4M*18; 56.3%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR =10dB; 32400; 3600; 185.7k; 29.4M*18; 65.6%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR =20dB; 32400; 3600; 185.7k; 29.4M*18; 90.3%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; Without noise; 32400; 3600; 185.7k; 29.4M*18; 95.2%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; mix SNR=0dB and SNR =20dB; SNR=0dB; 32400; 3600; 185.7k; 29.4M*18; 91.7%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR =10dB; 32400; 3600; 185.7k; 29.4M*18; 93.1%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; SNR =20dB; 32400; 3600; 185.7k; 29.4M*18; 93.3%. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; Without noise; 32400; 3600; 185.7k; 29.4M*18; 93.3%. Observation 4: Performance of AI/ML assisted LOS/NLOS identification positioning significantly degrades when the model is trained with small channel estimation error and tested with large channel estimation error. Observation 5: AI/ML model in AI/ML assisted LOS/NLOS identification positioning has robust generalization capability when the model is trained with large channel estimation error and tested with small channel estimation error.

## Evaluation of single-TRP construction with N models for N TRP

### Evaluation without generalization considerations (same setting for training and testing)

- vivo (R1-2304475) Table 3 Evaluation results for AI/ML model deployed on UE or Network side, UE distribution area = [120x60 m] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; Pos.; 0; Drop1; Drop1; 25k; 1k; 1.65M; 22.30M; 0.99. CIR; TOA; 0; Drop1; Drop1; 25k; 1k; 4.26M*18; 8.50M*18; 0.73. Observation 3: For Construction1 (Single-TRP, N models for N TRPs), the AI/ML based TOA estimation positioning method achieves remarkable performance gain compared to direct AI/ML positioning method.

### Generalization aspects (different setting for training and testing)

#### Different drops

- vivo (R1-2304475) Figure 26 CDF of positioning accuracy when AI model is tested on other drops

#### Different clutter parameters

- vivo (R1-2304475) Figure 28 CDF of positioning accuracy when AI model is tested on other clutter Observation 13: Positioning performance of AI/ML assisted positioning is slightly degraded but still acceptable when the model trained with dataset of one clutter parameter is tested with dataset of another clutter parameter. Observation 14: AI/ML assisted positioning enjoys better generalization performance than direct AI/ML positioning across clutter parameters.

#### Network synchronization error

- vivo (R1-2304475) Observation 26: The positioning accuracy of AI/ML assisted positioning significantly degrades with the increase of network synchronization error, but it is still better than direct AI/ML positioning.

#### UE/gNB RX and TX timing error

- vivo (R1-2304475) Table 39 Evaluation results for AI/ML model deployed on UE or Network side, UE distribution area = [120x60 m] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; TOA; 0; 0ns; 0ns; 25k; 1k; 4.26M*18; 8.50M*18; 0.73. CIR; TOA; 0; 0ns; 2ns; 25k; 1k; 4.26M*18; 8.50M*18; 0.75. CIR; TOA; 0; 0ns; 10ns; 25k; 1k; 4.26M*18; 8.50M*18; 1.51. CIR; TOA; 0; 0ns; 50ns; 25k; 1k; 4.26M*18; 8.50M*18; 10.18.

#### Different InF scenarios

- vivo (R1-2304475) Observation 15: Positioning performance of AI/ML assisted positioning is degraded when the model trained with dataset of DH is tested with datasets of SH and HH. Observation 16: For those scenarios whose positioning does not rely on fingerprint features, AI/ML based TOA estimation has better generalization ability than direct AI/ML positioning. Observation 17: AI/ML based TOA estimation has great advantages in positioning performance, deployment flexibility, compatibility with existing positioning protocol framework, and generalization capability.

#### Channel estimation error

- vivo (R1-2304475) Observation 25: CIR estimation error can dramatically degrade the positioning performance of AI/ML assisted positioning, while is more robust to small CIR estimation error compared to direct AI/ML positioning.

## Evaluation of multi-TRP construction

### Evaluation without generalization considerations (same setting for training and testing)

- MediaTek Inc. (R1-2305659) Table 8. Evaluation results for AI/ML model deployed on UE or network side, without model generalization, CNN, UE distribution area = 120x60 m Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training & validation; test; Model complexity; Computational complexity; AI/ML. non-normalized PDP (1*8*256); 1TOA; Ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 187k; 15M*18; 20.1. non-normalized PDP (18*8*256); 18TOA; Ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 1.05. non-normalized PDP (1*8*256); 1TOA; Ideal; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 187k; 15M*18; 14.9. non-normalized PDP (18*8*256); 18TOA; Ideal; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 205k; 77M; 1.56. Observation 11: Multi-TRP approach outperforms single-TRP for AI/ML assisted TOA estimation positioning.

- vivo (R1-2304475) Table 7 Evaluation results for AI/ML model deployed on UE or Network side, UE distribution area = [120x60 m] Construction; Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Construction 1; CIR; TOA; 0; Drop1; Drop1; 25k; 1k; 4.26M*18; 8.50M*18; 0.73. Construction 2; CIR; TOA; 0; Drop1; Drop1; 25k*18; 1k*18; 11.92M*1; 23.79M *1; 0.83. Construction 3; CIR; TOA; 0; Drop1; Drop1; 25k; 1k; 1.65M; 22.30M; 1.08.

- CATT (R1-2304726) Observation 15: For AI/ML-assisted positioning with perfect network synchronization, the intermediate result of ToA estimating is 1.59ns@90% and the horizontal position accuracy is 0.655m@90% of CDF percentile. Observation 16: For AI/ML-assisted positioning with network synchronization error, the intermediate result of ToA estimating is 1.74ns@90% and the eventual result is 0.7m@90% of CDF percentile of horizontal accuracy.

### Evaluation of generalization aspects

#### Different drops

- ZTE (R1-2304538) Observation 24: For model generalization evaluation in various simulation drops, when datasets for training and test are from different simulation drops, AI/ML assisted positioning using DL RSTD value as model output cannot generalize its performance in the test dataset. Observation 25: For model generalization evaluation in various simulation drops, when model update (or fine-tuned) is performed based on dataset in a simulation drop that is different from the simulation drop used for model training, • Positioning performance of AI/ML assisted positioning using DL RSTD value as model output can be improved in a test dataset that is in the same simulation drop as the dataset for model update; • Positioning performance of AI/ML assisted positioning using DL RSTD value as model output is increased along with the number of data samples used for model update.

#### Different clutter parameters

- CATT (R1-2304726) Observation 18: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of clutter parameter {60%, 6m, 2m} with a small dataset of clutter parameter {40%, 2m, 2m}, the AI/ML model achieved a horizontal positioning accuracy of 1.51m, which represents an improvement compared to the performance achieved without mix-training and with a positioning accuracy of 3.11m. Observation 21: For AI/ML-assisted positioning, by training the AI/ML model using the dataset that assumes a clutter parameter of {60%, 6m, 2m}, and subsequently fine-tuning it using a small dataset that assumes a clutter parameter of {40%, 2m, 2m}, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

#### Network synchronization error

- CATT (R1-2304726) Observation 19: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of ideal network synchronization with a small dataset of network synchronization error of 50ns, the AI/ML model achieved a horizontal positioning accuracy of 1.81m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 12.8m. Observation 22: For AI/ML-assisted positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

#### UE/gNB RX and TX timing error

- MediaTek Inc. (R1-2305659) Observation 17: Performance of AI/ML assisted TOA estimation positioning significantly degrades when the model is trained with small timing error and tested with large timing error. Observation 18: AI/ML model in AI/ML assisted TOA estimation positioning has robust generalization capability when the model is trained with large timing error and tested with small timing error.

#### Different InF scenarios

- MediaTek Inc. (R1-2305659) Table 11. Evaluation results for AI/ML model deployed on UE or network side, CNN, UE distribution area = [DH 120x60 m, SH 300x150 m], different scenario Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; Test; Training & validation; Test; Model complexity; Computational complexity; AI/ML. non-normalized PDP (18*8*256); 18TOA; Ideal; InF-DH {40%, 2m, 2m}; InF-DH {40%, 2m, 2m} same drop; 32400; 3600; 205k; 77M; 1.56. non-normalized PDP (18*8*256); 18TOA; Ideal; InF-DH {40%, 2m, 2m} new drop; 32400; 3600; 205k; 77M; 4.50. non-normalized PDP (18*8*256); 18TOA; Ideal; InF-DH {60%,6m, 2m}; 32400; 3600; 205k; 77M; 4.36. non-normalized PDP (18*8*256); 18TOA; Ideal; InF-SH Large hall size; 32400; 3600; 205k; 77M; >100. non-normalized PDP (18*8*256); 18TOA; Ideal; InF-DH {60%,6m, 2m}; InF-DH {60%,6m, 2m} same drop; 32400; 3600; 205k; 77M; 1.05. non-normalized PDP (18*8*256); 18TOA; Ideal; InF-DH {60%,6m, 2m} new drop; 32400; 3600; 205k; 77M; 7.42. non-normalized PDP (18*8*256); 18TOA; Ideal; InF-DH {40%, 2m, 2m}; 32400; 3600; 205k; 77M; 8.14. non-normalized PDP (18*8*256); 18TOA; Ideal; InF-SH Large hall size; 32400; 3600; 205k; 77M; >100. non-normalized PDP (18*8*256); 18TOA; Ideal; Mix InF-DH {40%, 2m, 2m} and InF-SH; InF-DH {40%, 2m, 2m}; 32400; 3600; 205k; 77M; 2.42. non-normalized PDP (18*8*256); 18TOA; Ideal; InF-SH Large hall size; 32400; 3600; 205k; 77M; 3.6. Observation 14: Performance of AI/ML assisted TOA estimation positioning degrades when the model is trained with dataset of one scenario and tested with dataset of another scenario, especially for different hall size scenario. Proposal 5 : Further study the monitor mechanism of different hall size scenario for AI/ML positioning.

- CATT (R1-2304726) Observation 20: For AI/ML-assisted positioning, by employing the mix-train method and combining the dataset of InF-DH {60%, 6m, 2m} with a small dataset of InF-SH {20%, 2m, 10m}, the AI/ML model achieved a horizontal positioning accuracy of 1.467m, which represents a significant improvement compared to the performance achieved without mix-training and with a positioning accuracy of 6.894m. Observation 23: For AI/ML-assisted positioning, by training the AI/ML model using the dataset with perfect network synchronization, and subsequently fine-tuning it using a small dataset with network synchronization error, the horizontal positioning accuracy is significantly improved compared to the performance achieved without fine-tuning. The degree of improvement in the positioning accuracy is greater when the size of the fine-tuning dataset is larger.

#### Channel estimation error

- MediaTek Inc. (R1-2305659) Observation 15: Performance of AI/ML assisted TOA estimation positioning significantly degrades when the model is trained with small channel estimation error and tested with large channel estimation error. Observation 16: AI/ML model in AI/ML assisted TOA estimation positioning has robust generalization capability when the model is trained with large channel estimation error and tested with small channel estimation error.

## High-level summary of methods to address generalization problem

- MediaTek Inc. (R1-2305659) Observation 19: Fine-tuning the model with samples from a scenario can achieve positioning accuracy improvement when the pre-trained model is transferred to a new scenario for AI/ML assisted TOA estimation positioning (the more fine-tuning data the better performance), but performance degrades for pre-trained scenario.

- vivo (R1-2304475) Table 61 Evaluation of model fine-tuning for different cases Cases; Training; Fine-tuning; Testing; Positioning accuracy @90%. Direct AI/ML positioning; {0.6, 6, 2}; {0.4, 2, 2}; {0.4, 2, 2}; 4.40. {0.4, 2, 2}; {0.6, 6, 2}; {0.6, 6, 2}; 3.23. Drop1; Drop2; Drop2; 3.97. InF-DH; InF-HH; InF-HH; 8.78. Sync 0ns; 50ns; 50ns; 2.39. Sync 0ns; 10ns; 10ns; 1.28. Sync 0ns; 2ns; 2ns; 1.11. AI/ML assisted positioning; {0.6, 6, 2}; {0.4, 2, 2}; {0.4, 2, 2}; 0.63. Drop1; Drop2; Drop2; 5.50. InF-DH; InF-HH; InF-HH; 0.17. InF-DH; InF-SH; InF-SH; 0.17. Sync 0ns; 50ns; 50ns; 3.40. Sync 0ns; 10ns; 10ns; 1.78. Sync 0ns; 2ns; 2ns; 1.30.

## Observations on generalization

### High-level observation

#### 1st round discussion

In RAN1#112, the following observation was made:

Observation (RAN1#112) Evaluation of the following generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario. - The generalization aspects include: - Different drops - Different clutter parameters - Different InF scenarios - Network synchronization error - Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training. - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset. - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset. Note: ideal model training and switching may provide the upper bound of achievable performance when the AI/ML model needs to handle different deployment scenarios.

Correspondingly, similar observation can be made for AI/ML assisted positioning.

- For Observation 8.5.1-1, " Network synchronization error" is not included here. Observation in section 8.5.2 address it in details.

- For Observation 8.5.1-2: based on evaluation results submitted thus far, regarding fine-tuning and mixed dataset, all generalization aspects are included except "time varying changes". See also discussion in section 5.4 for direct AI/ML positioning.

Observation 8.5.1.1-1:

Evaluation of the following generalization aspects show that the positioning accuracy of AI/ML assisted positioning with ToA as model output deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario.

  - Different drops

  - Different clutter parameters

  - Different InF scenarios

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSI | Not support. For assisted positioning, it would be better to look at the intermediate KPI, and not at the final KPI. For LOS identification, for example, it depends on in which scenarios the model has been trained and tested. We have e.g. Observation 30 which indicates that there can be a great generalization performance. Observation 30: When the model is trained under the InF-DH scenario but inferred under the InF-SH scenario, the performance deteriorates significantly. But when trained under the InF-SH scenario and inferred under the InF-DH scenario, the AI/ML-based LOS/NLOS identification model provides great generalization performance of the intermediate LOS/NLOS identification rate. |

Observation 8.5.1.1-2:

For AI/ML assisted positioning, evaluation results demonstrate that for the generalization aspects of:

  - Different drops

  - Different clutter parameters

  - Different InF scenarios

  - Network synchronization error

  - UE/gNB RX and TX timing error

  - SNR mismatch

  - channel estimation error

the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training.

- Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset.

- Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Support |

### Observation on network synchronization error

#### 1st round discussion

For the generalization aspect of network synchronization error, evaluation results from companies' contributions are collected in Table 6 and Table 7 below. Please add/correct if anything in Table 6 and Table 7 is missing or wrong.

Analysis in Table 6 and Table 7 are used to derive the observation on network synchronization error.

Table 6. Network synchronization error: evaluation results when t2 (test) <= t1 (train).

| Source | Selected simulation parameter | E1= positioning error (meters) @90% CDF with (train, test) = (50ns, 50ns) | E2= positioning error (meters) @90% CDF with (train, test) = (50ns, 20-25 ns) | E3= positioning error (meters) @90% CDF with (train, test) = (50ns, 0ns) | E2/E1 | E3/E1 |
| --- | --- | --- | --- | --- | --- | --- |
| MediaTek R1-2305659 | PDP, {40%, 2m, 2m} | 6.78 | (t2=20ns) 5.11 | 5.14 | 0.75 | 0.76 |
| MediaTek R1-2305659 | PDP, {60%, 6m, 2m} | 4.09 | (t2=20ns) 3.06 | 3.28 | 0.75 | 0.80 |
| Ericsson R1-2304339 | CIR | 0.522 | (t2=25ns) 0.436 | 0.410 | 0.84 | 0.79 |
| Ericsson R1-2304339 | PDP | 0.675 | (t2=25ns) 0.574 | 0.541 | 0.85 | 0.80 |
| Ericsson R1-2304339 | DP | 1.023 | (t2=25ns) 0.851 | 0.778 | 0.83 | 0.76 |
|  |  |  |  | max | 0.85 | 0.80 |
|  |  |  |  | min | 0.75 | 0.76 |

Table 7. Network synchronization error: evaluation results when t2 (test) > t1 (train).

| Source | Selected simulation parameter | E1= positioning error (meters) @90% CDF with (train, test) = (0ns, 0ns) | E2= positioning error (meters) @90% CDF with (train, test) = (0ns, 10ns) | E3= positioning error (meters) @90% CDF with (train, test) = (0ns, 20-25 ns) | E4= positioning error (meters) @90% CDF with (train, test) = (0ns, 50ns) | E2/E1 | E3/E1 | E4/E1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo R1-2304475 | CIR | 0.73 | 2.05 |  | 8.45 | 2.07 |  | 13.95 |
| MediaTek R1-2305659 | PDP, {40%, 2m, 2m} | 1.56 |  | (t2=20ns) 10.9 | 16.57 |  | 6.99 | 10.62 |
| MediaTek R1-2305659 | PDP, {60%, 6m, 2m} | 1.05 |  | (t2=20ns) 10.62 | 16.59 |  | 10.11 | 15.80 |
| Apple-R12305238 | CIR | 1.138 |  |  | 11.0172 |  |  | 51.52 |
| Xiaomi R1-2304897 | CIR, {0.6,6,2} | 0.6778 |  |  | 12.7748 |  |  | 38.57 |
| Xiaomi R1-2304897 | CIR, {0.4,2,2} | 0.8533 |  |  | 15.4699 | 2.30 | 13.00 | 43.03 |
| Ericsson R1-2304339 | CIR | 0.423 | 0.77 | (t2=25ns) 2.553 | 13.514 | 2.19 | 11.54 | 34.27 |
| Ericsson R1-2304339 | PDP | 0.589 | 0.876 | (t2=25ns) 2.942 | 16.501 | 1.34 | 5.66 | 26.25 |
| Ericsson R1-2304339 | DP | 0.740 | 0.860 | (t2=25ns) 1.622 | 10.301 | 2.07 |  | 13.95 |
| CATT R1-2304726 | CIR | 0.655 |  |  | 12.8 |  |  | 19.54 |
|  |  |  |  |  | max | 2.81 | 10.11 | 31.95 |
|  |  |  |  |  | min | 1.16 | 2.19 | 9.68 |

Observation 8.5.2.1-1

For AI/ML assisted positioning with TOA as model output, based on evaluation results of network synchronization error in the range of 0-50 ns, when the model is trained by a dataset with network synchronization error t1 (ns) and tested in a deployment scenario with network synchronization error t2 (ns), for a given t1,

- For a case evaluated by a given source, the positioning accuracy of cases with t2 smaller than t1 is better than the cases with t2 equal to t1. For example,

  - For the case of (t1, t2)=(50ns, 20~25ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(50ns, 20~25ns) is 0.75~0.85 times that of (t1, t2)=(50ns, 50ns).

  - For the case of (t1, t2)=(50ns, 0ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(50ns, 0ns) is 0.76~0.80 times that of (t1, t2)=(50ns, 50ns).

- For a case evaluated by a given source, the positioning accuracy of cases with t2 greater than t1 is worse than the cases with t2 equal to t1. The larger the difference between t1 and t2, the more the degradation. For example,

  - For the case of (t1, t2)=(0ns, 10ns), evaluation results submitted to RAN1#113 show the positioning error of (0ns, 10ns) is 1.16~2.81 times that of (0ns, 0ns).

  - For the case of (t1, t2)=(0ns, 20~25ns), evaluation results submitted to RAN1#113 show the positioning error of (0ns, 50ns) is 2.19~10.11 times that of (0ns, 0ns).

  - For the case of (t1, t2)=(0ns, 50ns), evaluation results submitted to RAN1#113 show the positioning error of (0ns, 50ns) is 9.68~31.95 times that of (0ns, 0ns).

Note: here the positioning error is the horizonal positioning error (meters) at CDF=90%.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | For assisted positioning, we should distinguish whether the output is TOA or LOS, which can have different performance. For LOS, both the NW and RX/TX timing errors should not matter. |
| CATT | Seems CATT‘s E4/E1 is missing. Calculate it to the table. Though the min and max is not changed. |

### Observation on UE/gNB RX and TX timing error

#### 1st round discussion

For the generalization aspect of UE/gNB RX and TX timing error, evaluation results from companies' contributions are collected in Table 8 and Table 9 below. Please add/correct if anything in Table 8 and Table 9 is missing or wrong.

Analysis in Table 8 and Table 9 are used to derive the observation on UE/gNB RX and TX timing error.

Table 8. UE/gNB RX and TX timing error: evaluation results when t2 (test) <= t1 (train).

| Source | Selected simulation parameter | E1= positioning error (meters) @90% CDF with (train, test) = (50ns, 50ns) | E2= positioning error (meters) @90% CDF with (train, test) = (50ns, 20-25 ns) | E3= positioning error (meters) @90% CDF with (train, test) = (50ns, 0ns) | E2/E1 | E3/E1 |
| --- | --- | --- | --- | --- | --- | --- |
| MediaTek R1-2305659 | PDP, {40%, 2m, 2m} | 6.78 | (t2=20ns) 5.11 | 5.14 | 0.75 | 0.76 |
| MediaTek R1-2305659 | PDP, {60%, 6m, 2m} | 4.09 | (t2=20ns) 3.06 | 3.28 | 0.75 | 0.80 |
| Ericsson R1-2304339 | CIR | 0.581 | (t2=25ns) 0.557 | 0.544 | 0.96 | 0.94 |
| Ericsson R1-2304339 | PDP | 0.810 | (t2=25ns) 0.776 | 0.762 | 0.96 | 0.94 |
| Ericsson R1-2304339 | DP | 0.503 | (t2=25ns) 0.483 | 0.477 | 0.96 | 0.95 |
|  |  |  |  | max | 0.96 | 0.95 |
|  |  |  |  | min | 0.75 | 0.76 |

Table 9. UE/gNB RX and TX timing error: evaluation results when t2 (test) > t1 (train).

| Source | Selected simulation parameter | E1= positioning error (meters) @90% CDF with (train, test) = (0ns, 0ns) | E2= positioning error (meters) @90% CDF with (train, test) = (0ns, 10ns) | E3= positioning error (meters) @90% CDF with (train, test) = (0ns, 20~25ns) | E4= positioning error (meters) @90% CDF with (train, test) = (0ns, 50ns) | E2/E1 | E3/E1 | E4/E1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vivo R1-2304475 | CIR | 0.73 | 1.51 |  | 10.18 | 2.07 |  | 13.95 |
| MediaTek R1-2305659 | CIR | 1.56 |  | (t2=20ns) 10.9 | 16.57 |  | 6.99 | 10.62 |
| MediaTek R1-2305659 | PDP | 1.05 |  | (t2=20ns) 10.62 | 16.59 |  | 10.11 | 15.80 |
| OPPO R1-2305463 | CIR, 1 drop | 0.52 |  |  | 26.79 |  |  | 51.52 |
| OPPO R1-2305463 | CIR, 10 drops | 1.03 |  |  | 39.73 |  |  | 38.57 |
| Ericsson R1-2304339 | CIR | 0.427 | 0.98 | (t2=25ns) 5.553 | 18.375 | 2.30 | 13.00 | 43.03 |
| Ericsson R1-2304339 | PDP | 0.586 | 1.281 | (t2=25ns) 6.76 | 20.081 | 2.19 | 11.54 | 34.27 |
| Ericsson R1-2304339 | DP | 0.732 | 0.981 | (t2=25ns) 4.142 | 19.213 | 1.34 | 5.66 | 26.25 |
|  |  |  |  |  | max | 2.30 | 13.00 | 51.52 |
|  |  |  |  |  | min | 1.34 | 5.66 | 10.62 |

Observation 8.5.3.1-1

For AI/ML assisted positioning with TOA as model output, based on evaluation results of timing error in the range of 0-50 ns, when the model is trained by a dataset with UE/gNB RX and TX timing error t1 (ns) and tested in a deployment scenario with UE/gNB RX and TX timing error t2 (ns), for a given t1,

- For a case evaluated by a given source, the positioning accuracy of cases with t2 smaller than t1 is better than the cases with t2 equal to t1. For example,

  - For the case of (t1, t2)=(50ns, 20~25ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(50ns, 20~25ns) is 0.75~0.96 times that of (t1, t2)=(50ns, 50ns).

  - For the case of (t1, t2)=(50ns, 0ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(50ns, 0ns) is 0.76~0.95 times that of (t1, t2)=(50ns, 50ns).

- For a case evaluated by a given source, the positioning accuracy of cases with t2 greater than t1 is worse than the cases with t2 equal to t1. The larger the difference between t1 and t2, the more the degradation. For example,

  - For the case of (t1, t2)=(0ns, 10ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(0ns, 10ns) is 1.34~2.30 times that of (t1, t2)=(0ns, 0ns).

  - For the case of (t1, t2)=(0ns, 20~25ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(0ns, 20~25ns) is 5.66~13.0 times that of (t1, t2)=(0ns, 0ns).

  - For the case of (t1, t2)=(0ns, 50ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(0ns, 50ns) is 10.62~51.52 times that of (t1, t2)=(0ns, 0ns).

Note: here the positioning error is the horizonal positioning error (meters) at CDF=90%.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | Same comment as above, we should specify whether it is TOA or LOS. |

# Impact of AI/ML assisted positioning performance by model input type and size

## Evaluation of input size reduction

### Different type of model input

- Ericsson (R1-2304339)

### Model input truncated in time domain

- Apple (R1-2305973) Table 10: AI/ML-assisted Positioning: Evaluation results for AI/ML model deployed on UE or Network side, using a different # of consecutive taps , with a CNN, UE distribution area = 100x40 m Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR [18,1,256,2]; Direct path ToA [1x1]; Ideal TOA 100% labeled; Drop 1; Drop 1; 47500; 2500; 1.6 M; 3.1 G; 1.138m. CIR [128,1,128,2]; Direct path ToA [1x1]; Ideal TOA 100% labeled; Drop 1; Drop 1; 23750; 1250; 1.6 M; 3.1 G; 1.1362m. CIR [18,1,64,2]; Direct path ToA [1x1]; Ideal TOA 100% labeled; Drop 1; Drop 1; 9500; 500; 1.6 M; 3.1 G; 1.2410m. Observation 10: For AI/ML assisted positioning, when optimizing the data size by reducing the number of consecutive taps into the model - Reducing the number of taps even to 54 does not does not affect the performance materially - This seems to be more robust than direct AI-ML based positioning. - Reducing the number of taps ultimately reduces the overhead by the same factor

- vivo (R1-2304475) Table 72 Evaluation results for AI/ML model deployed on UE or Network side, UE distribution area = [120x60 m] Model input; Model output; Truncated CIR; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; TOA; 1-256; DH; DH; 25k; 1k; 4.26M*18; 8.50M*18; 0.73. CIR; TOA; 1-30; DH; DH; 25k; 1k; 4.26M*18; 8.50M*18; 3.72. CIR; TOA; 1-100; DH; DH; 25k; 1k; 4.26M*18; 8.50M*18; 1.04. CIR; TOA; 30-100; DH; DH; 25k; 1k; 4.26M*18; 8.50M*18; 4.17.

- CATT (R1-2304726) Observation 28: For AI/ML-assisted positioning, reducing the number of CIR taps can reduce the computation complexity of a model while keeping similar inference accuracy.

- Ericsson (R1-2304339) Table 13 UE 2D positioning errors for {60%, 6m, 2m} test dataset at different percentiles and different numbers of time domain window size Nt (training dataset size = 40,000 samples, Nt=Nt'). Input type; Time domain window size Nt; Model complexity [# parameters]; Computational complexity [FLOPs]; 90%tile 2D positioning error [m] in {60%, 6m, 2m} InF-DH. Cent. Assist.; Cent. Direct. CIR; 256; 0.73 M; 32 M; 0.371; 0.373. CIR; 128; 0.73 M; 15 M; 0.463; 0.443. CIR; 64; 0.73 M; 7 M; 0.704; 0.668. CIR; 32; 0.73 M; 3 M; 1.627; 1.602. PDP; 256; 0.36 M; 9 M; 0.520; 0.510. PDP; 128; 0.36 M; 4 M; 0.596; 0.563. PDP; 64; 0.36 M; 2 M; 0.713; 0.711. PDP; 32; 0.36 M; 0.8 M; 1.261; 1.165. DP; 256; 0.36 M; 9 M; 0.653; 0.658. DP; 128; 0.36 M; 4 M; 0.679; 0.658. DP; 64; 0.36 M; 2 M; 1.970; 1.976.

### Reduced number of taps as model input

- MediaTek Inc. (R1-2305659) Table 6. Evaluation results for AI/ML model deployed on UE or network side, CNN, UE distribution area = 120x60 m Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; LOS/NLOS identification accuracy. Nt; N’t; Train; AI/ML; Model complexity; Computation complexity; AI/ML. non-normalized CIR (8*Nt*2); LOS/NLOS; Ideal; 256; 256; 32400; 3600; 185.7k; 29.4M*18; 95.2%. 25; 95.3%. 15; 95.0%. 64; 64; 32400; 3600; 173.4k; 7.39M*18; 94.7%. 25; 94.6%. 15; 94.8%. Observation 8: For AI/ML assisted LOS/NLOS identification positioning, by selecting appropriate Nt and N’t, the computational complexity, model complexity and signaling overhead can be reduced without significant performance loss. Table 18. Evaluation results for AI/ML model deployed on UE or network side, CNN, UE distribution area = 120x60 m Model input; Model output; Label; Settings (e.g., drops, clutter param, mix) {40%, 2m, 2m}; Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Nt; N’t; Train; AI/ML; Model complexity; Computation complexity; AI/ML. non-normalized PDP (18* Nt*8); 18TOA; Ideal; 256; 256; 32400; 3600; 205k; 77M; 1.56. 25; 1.65. 15; 1.63. 64; 64; 32400; 3600; 181.3k; 19.3M; 1.7. 25; 1.73. 15; 1.74. Observation 21: For AI/ML assisted TOA estimation positioning, by selecting appropriate Nt and N’t, the computational complexity, model complexity and signaling overhead can be reduced without significant performance loss.

- ZTE (R1-2304538) Observation 21. The AI/ML assisted positioning has an excellent accuracy on the estimation of DL PRS RSTD values even in heavy NLOS conditions (e.g., InF-DH {60%, 6m, 2m}). Exemplary results are: - When the width of square grid is 0.5 m in grid distribution used for training dataset generation, positioning error of CIR based positioning is about 0.28 m at N_t^'=256;

- xiaomi (R1-2304897) Observation 14: - Reducing the number of CIR taps per TRP to 24 could reduce the computation complexity and signaling overhead significantly while still maintain less 1m positioning error

- Ericsson (R1-2304339) Table 11 UE 2D positioning errors for {60%, 6m, 2m} test dataset at different percentiles and different numbers of down sampled taps (training dataset size = 40,000 samples, Nt=256). Model class; Positioning approach; Model complexity [# paras]; 90%tile 2D positioning error [m] with numbers of down sampled taps in {60%, 6m, 2m} InF-DH. Nt'=256; Nt'=128; Nt'=64; Nt'=32; Nt'=16; Nt'=9. Small models CIR; Dist. Assist.; 0.86 M; 0.453; 0.478; 0.558; 0.764; 1.208; 1.995. Cent. Assist.; 0.73 M; 0.371; 0.372; 0.434; 0.545; 0.678; 0.805. Cent. Direct; 0.73 M; 0.373; 0.382; 0.425; 0.540; 0.678; 0.824. Small models PDP; Dist. Assist.; 0.43 M; 0.680; 0.687; 0.732; 0.856; 1.001; 1.452. Cent. Assist.; 0.36 M; 0.524; 0.534; 0.514; 0.604; 0.698; 0.868. Cent. Direct; 0.36 M; 0.510; 0.522; 0.522; 0.580; 0.689; 0.824.

### Reduced number of TRPs

- MediaTek Inc. (R1-2305659) Table 20. Evaluation results for AI/ML model deployed on UE or network side, without model generalization, CNN, UE distribution area = 120x60 m, N’TRP Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training & validation; test; Model complexity; Computational complexity; AI/ML. non-normalized PDP (18*8*256) N’TRP =18; 18TOA; ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 1.05. non-normalized PDP (18*8*256) Approach 1-A N’TRP =9; 18TOA; ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 1.45. non-normalized PDP (18*8*256) Approach 1-B N’TRP =9; 18TOA; ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 7.92. non-normalized PDP (9*8*256) Approach 2 N’TRP =9; 18TOA; ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 188k; 45.9M; 1.50. non-normalized PDP (18*8*256) Approach 1-A N’TRP =4; 18TOA; ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 5.0. non-normalized PDP (18*8*256) Approach 1-B N’TRP =4; 18TOA; ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 44.9. non-normalized PDP (4*8*256) Approach 2 N’TRP =4; 18TOA; ideal; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 188k; 18.2M; 7.3. Observation 23: For reduced number of TRP evaluation approach 1-A and approach 2, performance of AI/ML assisted TOA estimation positioning will not significantly degrade when number of TRPs (N’TRP) is 9. Observation 24: For reduced number of TRP evaluation approach 1-B, performance of AI/ML assisted TOA estimation positioning degrade with the decrease of number of TRPs (N’TRP). Observation 25: For reduced number of TRP evaluation approach 2, computational complexity can be significantly reduced with the decrease of number of TRPs (N’TRP).

- InterDigital (R1-2305123) Observation 11: For direct AI/ML positioning, for different number of TRPs [N’TRP=12,9,6,4] when measurements are collected from fixed set of TRPs, horizontal positioning accuracy degrades as we reduce number of TRPs (N’TRP) for 90% UEs. (Table 7) Observation 12: For direct AI/ML positioning, for different number of TRPs [N’TRP=12,9,6,4] when measurements are collected from TRPs with top N’TRP RSRP values, horizontal positioning accuracy degrades as we reduce number of TRPs (N’TRP) for 90% UEs. (Table 8) Observation 13: For direct AI/ML positioning, for different number of TRPs [N’TRP=12,9,6,4] when measurements are collected from TRPs with top N’TRP RSRP values (Approach B) results better 90% horizontal positioning accuracy compared to fixed TRP selection approach (Approach A). Observation 14: For direct AI/ML positioning, for different number of CIR taps when model input is CIR measurements: • For Nt= 128 and 64, we observe similar (~ < 1m) horizontal positioning accuracy as Nt=256. • For Nt =32 and 16, 90% horizontal accuracy degrades compared to Nt=256.

- xiaomi (R1-2304897) Observation 14: - Reducing the number of CIR taps per TRP to 24 could reduce the computation complexity and signaling overhead significantly while still maintain less 1m positioning error - Reducing the number of TRP to 9 could reduce the computation complexity and signaling overhead while maintain around 1m positioning error

- CATT (R1-2304726) In this simulation, since NTRP = 9, the measurement size is NTRP*Nport*NT*2*64 = 147456bit per UE. 3.2.3.1.1. One model covers entire area The first case is to select 9 fixed TRPs and train a model to cover the entire simulation area. In our first design, Approach 2 is applied, i.e. the TRP dimension of model input is equal to the number of TRPs (N’TRP) that provides measurements as model input. In our second design, Approach 1-A is applied, i.e. 9 fixed TRPs provides the effective input value to the model and the input of the other 9 TRPs are zero-padded. Observation 25: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the AI/ML model performance will not significantly degrade. Observation 26: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs, the FLOPs can be significantly reduced. 3.2.3.1.2. Two models each covers half the area In this case, we divide the entire simulation area into two regions, named as region A and region B, by equal proportion. The TRP ID of region A is {0, 1, 2, 3, 4, 5, 6, 7, 8}, and the TRP ID of region B is {9, 10, 11, 12, 13, 14, 15, 16, 17}. Approach 2 is applied, Observation 27: For AI/ML-assisted positioning, when the input dimension is reduced from 18 TRPs to 9 TRPs and the total dataset size is unchanged, the horizontal position accuracy of using two models to cover the entire area is slightly degraded compared to the performance of using single model to cover the entire area.

- vivo (R1-2304475) Observation 52: AI//ML assisted positioning is more sensitive to the number of TRPs used for positioning than direct AI/ML positioning Proposal 33: The minimal number of TRPs used for positioning should be identified to reach the target positioning accuracy for AI/ML assisted positioning. Observation 53: Direct AI/ML positioning has greater potential for compressing the dimensions of the CIR/PDP compared to AI/ML assisted positioning.

- Ericsson (R1-2304339) Figure 13: Positioning accuracy vs number of active TRP (N'TRP) for centralized ML assisted positioning and centralized ML direct positioning.

# Impact of AI/ML-assisted positioning performance by model output

## Evaluation of noisy ground truth labels

- MediaTek Inc. (R1-2305659) Table 7. Evaluation results for AI/ML model deployed on UE or network side, without model generalization, CNN, UE distribution area = 120x60 m, labelling error Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; LOS/NLOS identification accuracy. Training; test; Training & validation; test; Model complexity; Computational complexity; AI/ML. non-normalized CIR (8*256*2); LOS/NLOS; Ideal; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 185.7k; 29.4M*18; 95.2%. non-normalized CIR (8*256*2); LOS/NLOS; 10% LOS labelling error 10% NLOS labelling error; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 185.7k; 29.4M*18; 93.1%. non-normalized CIR (8*256*2); LOS/NLOS; 20% LOS labelling error 20% NLOS labelling error; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 185.7k; 29.4M*18; 92.1%. Table 19. Evaluation results for AI/ML model deployed on UE or network side, without model generalization, CNN, UE distribution area = 120x60 m, labelling error Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal positioning accuracy at CDF=90% (meters). Training; test; Training & validation; test; Model complexity; Computational complexity; AI/ML. non-normalized PDP (18*8*256); 18TOA; L=0; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 205k; 77M; 1.56. non-normalized PDP (18*8*256); 18TOA; L=0.5; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 205k; 77M; 2.20. non-normalized PDP (18*8*256); 18TOA; L=1; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 205k; 77M; 2.37. non-normalized PDP (18*8*256); 18TOA; L=2; {40%, 2m, 2m}; {40%, 2m, 2m}; 32400; 3600; 205k; 77M; 3.32. non-normalized PDP (18*8*256); 18TOA; L=0; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 1.05. non-normalized PDP (18*8*256); 18TOA; L=0.5; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 1.86. non-normalized PDP (18*8*256); 18TOA; L=1; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 2.21. non-normalized PDP (18*8*256); 18TOA; L=2; {60%, 6m, 2m}; {60%, 6m, 2m}; 32400; 3600; 205k; 77M; 3.06. Observation 22: Performance of AI/ML assisted TOA estimation positioning degrades with increasement the of labelling error.

- CMCC (R1-2305089) Observation 7: Traditional positioning algorithms can not work properly in inF scenario. Table X. Evaluation results for AI/ML assisted positioning with CHAN Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; Test; Model complexity; Computation complexity; AI/ML. CIR size：18*1*256; TOA; TOA; {60%, 6m, 2m}, 10 Drops mixed; {60%, 6m, 2m}, 10 Drops mixed; 78400; 1600; 3.71M; 7.42M; 45.36. TDOA; TDOA; 39.58. TOA; Ideal-TOA; 0.307. Observation 8: AI/ML assisted positioning can not work properly in inF scenario due to the lack of LOS path.

- Nokia (R1-2304685) Figure 12 - ToA estimation performance evaluation using mean-square error (MSE) for different L and number of TRPs values. Observation 13: The impact of labelling error on TOA accuracy (in MSE) is less than 1.5 for low/moderate standard deviation L=1, 2, and 3. Observation 14: Using many TRPs do not enhance the performance of the TOA indication significantly in the presence of labelling error on TOA accuracy (in MSE), except for large L values (e.g., L=10). Table 15 – [NTRP=18] Evaluation of ground truth labelling error with different m% LOS label error and n% NLOS label error. The scenario is InF-DH with clutter density of 40% (dataset 4). The UE distribution area is 120x60 m. m (%); 0; 5; 10; 20; 30; 50; 0; 50; 100. n (%); 0; 10; 20; 30; 20; 0; 50; 50; 100. Accuracy; 98.1%; 99.32%; 97.18%; 90.75%; 92.36%; 89.12%; 79.2%; 70.35%; 24.98%. F1-score; 0.975; 0.9914; 0.9651; 0.8902; 0.9014; 0.8405; 0.5639; 0.6539; 0.2132. Observation 15: When the number of TRPs is more than one, adding a labelling error of less than 10% for LOS/NLOS indication the performance is not impacted, in some cases the performance is enhanced when compared to the benchmark (m%=0, and n%=0). Observation 16: For AI/ML assisted positioning with LOS/NLOS indicator as model output, an acceptable LOS/NLOS indication is obtained until 20% of error for both, n% and m% error. Observation 17: The impact of labeling error for LOS/NLOS indication is independent to the number of TRPs used in the model construction. Table 21 - LOS/NLOS indication performance for assisted AI/ML positioning when a semi-supervised learning is applied on scenarios with low labeled samples. Training dataset; Testing dataset; Model complexity; Computational complexity; LOS/NLOS classification accuracy. Labeled; Unlabeled. 1k; 0; 3k; 129k; 141k; 82.40%. 2k; 0; 3k; 129k; 141k; 84.37%. 3k; 0; 3k; 129k; 141k; 85.93%. 1k; 100k; 3k; 129k; 141k; 83.83%. 2k; 100k; 3k; 129k; 141k; 85.07%. 3k; 100k; 3k; 129k; 141k; 86.37%. Observation 19: The semi-supervised learning enhances the performance of LOS/NLOS classification accuracy as compared to supervised learning when there is a small number of labeled samples are available.

- Apple (R1-2305973) Observation 11: AI-ML assisted positioning seems to be robust to label error derived from an error in the GT location.

- CATT (R1-2304726) Observation 24: When AI/ML model is trained with different value of label error, as value of L increases, AI/ML–assisted positioning error increases approximately in proportion to L.

- vivo (R1-2304475) Table 37 Evaluation results for AI/ML model deployed on UE or Network side, UE distribution area = [120x60 m] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML. CIR; TOA; 0; Std = 0; 0; 25k; 1k; 4.26M*18; 8.50M*18; 0.73. CIR; TOA; 0; Std = 0.5; 0; 25k; 1k; 4.26M*18; 8.50M*18; 0.90. CIR; TOA; 0; Std = 1; 0; 25k; 1k; 4.26M*18; 8.50M*18; 1.73. CIR; TOA; 0; Std = 2; 0; 25k; 1k; 4.26M*18; 8.50M*18; 2.24. Observation 27: Labeling error can slightly impair the positioning performance for AI/ML assisted positioning, and AI/ML assisted positioning is more robust to labeling error compared with direct AI/ML positioning.

- Ericsson (R1-2304339) Figure 14: Positioning accuracy vs label error standard deviation L (meters) for three positioning approaches and model input type of CIR, PDP, and DP.

## Semi-supervised learning

- MediaTek Inc. (R1-2305659) Observation 20: With less amount of labelled data, semi-supervised learning with more un-labelled data provides a more accurate position accuracy than supervised learning for AI/ML assisted TOA estimation positioning. Proposal 6 : Support semi-supervised learning for AI/ML positioning when limited labelled data are collected for training.

- Ericsson (R1-2304339) Table 16 90%tile 2D positioning accuracy using PDP inputs for centralized AI/ML assisted positioning approach and different unlabeled sample percentages in the {60%, 6m, 2m} InF-DH dataset. Input; Unlabeled %; Labeled; Unlabeled; Total; M paras; M FLOPs; 90%tile [m]. PDP; 97.50%; 1,000; 39,000; 40,000; 1.4; 34; 2.985. PDP; 97.50%; 2,000; 78,000; 80,000; 1.4; 34; 1.898. PDP; 95.00%; 1,000; 19,000; 20,000; 1.4; 34; 3.081. PDP; 95.00%; 2,000; 38,000; 40,000; 1.4; 34; 1.975. PDP; 95.00%; 4,000; 76,000; 80,000; 1.4; 34; 1.188. PDP; 90.00%; 1,000; 9,000; 10,000; 1.4; 34; 3.212. PDP; 90.00%; 2,000; 18,000; 20,000; 1.4; 34; 2.040. PDP; 90.00%; 4,000; 36,000; 40,000; 1.4; 34; 1.247. PDP; 90.00%; 8,000; 72,000; 80,000; 1.4; 34; 0.823. PDP; 80.00%; 4,000; 16,000; 20,000; 1.4; 34; 1.294. PDP; 80.00%; 8,000; 32,000; 40,000; 1.4; 34; 0.872. PDP; 80.00%; 16,000; 64,000; 80,000; 1.4; 34; 0.598.

## Other

- MediaTek Inc. (R1-2305659) Observation 12: The soft-decision approach outperforms the hard-decision approach for AI/ML assisted TOA estimation positioning. Proposal 3 : Support means and variance of TOA as model output to report for AI/ML assisted TOA estimation positioning.

- Qualcomm (R1-2305332) Observation 22: In AI/ML assisted positioning, evaluations show that ML-based soft information reporting method provides a significant improvement in positioning accuracy over the classical scheme. The 90th percentile of the horizontal positioning error reduces from >20 m for the classical scheme to 4.74 m.

## 1st round discussion

In RAN1#112bis, the following observation was made.

Observation (RAN1#112bis) For direct AI/ML positioning, for L in the range of 0.25m to 5m, the positioning error increases approximately in proportion to L, where L (in meters) is the standard deviation of truncated Gaussian Distribution of the ground truth label error.

Based on the evaluation results submitted for AI/ML assisted positioning with TOA as model output, similar observation can be made for AI/ML assisted positioning as well.

Observation 10.4-1

For AI/ML assisted positioning with TOA as model output, for L in the range of 0.25m to 5m, the positioning error increases approximately in proportion to L, where L (in meters) is the standard deviation of truncated Gaussian distribution of the ground truth label error.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |

Observation 10.4-2

Evaluation shows that AI/ML assisted positioning with ToA as model output is robust to small label error. The exact range of label error that can be tolerated depends on the positioning accuracy requirement,  where tighter positioning accuracy requirement demands smaller label error.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |

Observation 10.4-3

For AI/ML based positioning, evaluation results show that semi-supervised learning is helpful for improving the positioning accuracy when the same amount of data is used for supervised learning, and the number of labelled data is limited.

# Evaluation of model monitoring methods

- Nokia (R1-2304685) Observation 7: The performance monitoring based on the distribution comparison of different datasets is impacted by the feature that is selected to monitor. For instance, in evaluating the similarity between two distributions with Kolmogorov-Smirnov Test (Kstest) method, the RSRP measurement provides an accurate distribution comparison compared to CIR and PDP. Proposal 4: RAN1 further evaluate the impact of at least RSRP measurement as dominant feature for performance monitoring. Figure 9 - ROC curve of Binary classification performance of two datasets with different statistical distribution of the RSRP with AUC score of 0.743. Observation 8: For Performance monitoring purposes, using ML models as binary classification provides an easily discriminatory criteria between two different dataset distributions. However, in real scenarios it is expected to get a diversity set of distributions. Thus, an extension of a binary classification to a multi-class classification could be a potential alternative, in addition an extra expense of model and computational complexity could be expected. Proposal 5: To RAN1 further study and evaluate the monitoring performance based on AI/ML models considering the model and computational complexity of the AI/ML model used for monitoring.

- vivo (R1-2304475) Observation 41: Adopting SINR as a dominant feature of CIR is valid for model monitoring Observation 42: The proposed adversarial validation can achieve accurate model monitoring at the cost of acceptable hardware resource consumption for online training. Observation 43: The proposed AI/ML based out-of-distribution detection can achieve accurate and flexible model monitoring without need of online training and large-scale data collection. Observation 44: Motion state information can be used to assist model monitoring. Observation 45: The proposed self-monitoring method can achieve model monitoring for AI/ML assisted positioning. Observation 46: Ranging model assisted model monitoring is valid for direct AI/ML positioning without need of ground truth label.

- Apple (R1-2305973) Observation 12: For direct AI-positioning, any of the following is suitable for performance monitoring: • Model monitoring using statistics of measurement(s) without ground truth label • Model monitoring based on provided ground truth label (or its approximation) • Model monitoring using statistics of output without ground truth label.

- MediaTek Inc. (R1-2305659) Observation 49: Soft information can be used to monitor the performance of the direct AI/ML positioning. Observation 50: Based on the soft information, the performance of the direct AI/ML model can be monitored whether the data is labeled or not. Proposal 13 ：For model monitoring of direct AI/ML positioning, support the soft information associated to the estimated location as model output.

- Fujitsu (R1-2304768) Proposal 1 Before evaluating the performance of the model monitoring, the evaluation methodology and the KPI have to be discussed and aligned among companies, especially for introducing new metrics for label-free model monitoring. Proposal 2 Study the baseline method for model monitoring performance evaluation in order to achieve comparable results among companies. Proposal 3 For the label-free model monitoring for positioning method, study at least the following methods: • AI/ML model applicability condition information. • Historical model monitoring results.

- LG (R1-2305300) Proposal #2: Consider model monitoring metric based on model input depending on the amount of similarity for output distribution between full-set and sub-set of model input on MTRP construction. Proposal #3: For model monitoring evaluation, consider to align the procedure (e.g. model fine-tuning/transfer) depending on model monitoring metric/performance among companies. Proposal #4: Consider model monitoring metric based on model input depending on the amount of similarity between input and training data distribution. Proposal #5: For LOS/NLOS classification of AI/ML assisted positioning, consider also to utilize a soft value of the ML output as a LOS classification accuracy and model monitoring metric.

- CATT (R1-2304726) Observation 31: For direct AI/ML positioning, the relative displacement method can monitor the model performance. Observation 32: For AI/ML-assisted positioning, the relative displacement method can monitor the model performance.

# Proposals for online sessions

## Proposals for Monday online session

Proposal 3.2.1-1

The agreement made in RAN1#112bis is updated to the following.

Agreement

For the evaluation of AI/ML based positioning, the study of model input due to different number of TRPs include the following approaches. Proponent of each approach provide analysis for model performance, signaling overhead (including training data collection and model inference), model complexity and computational complexity.

- Approach 1: Model input size stays constant as NTRP=18. The number of TRPs (N’TRP) that provide measurements to model input varies. When N’TRP < NTRP, the remaining (NTRP  N’TRP) TRPs do not provide measurements to model input, i.e., measurement value is set to 0 such that the (NTRP  N’TRP) TRPs do not affect model output.

  - Approach 1-A. The set of TRPs (N’TRP) that provide measurements is fixed.

  - Approach 1-B. The set of TRPs (N’TRP) that provide measurements can change dynamically.

  - Note: for Approach 1, one model is provided to cover the entire evaluation area.

- Approach 2: The TRP dimension of model input is equal to the number of TRPs (N’TRP) that provide measurements as model input. When N’TRP < NTRP, the remaining (NTRP  N’TRP) TRPs are ignored by the given model. For a given AI/ML model, the set of TRPs (N’TRP) that provide measurements is fixed.

  - Approach 2-A. The set of active TRPs (N’TRP) that provide measurements is fixed.

    - For Approach 2-A: one model can be provided to cover the entire evaluation area, which is equivalent to deploying N’TRP TRPs in the evaluation area for positioning if ignoring the potential inference from the remaining (18  N’TRP) TRPs.

  - Approach 2-B. The set of active TRPs (N’TRP) that provide measurements can change dynamically.

    - For Approach 2-B, one model is developed to handle various patterns of active TRPs.

  - For Approach 2, if Nmodel (Nmodel >1) models are provided to cover the entire evaluation area, the total complexity (model complexity is the summation of the Nmodel models.

Observation 2.4-1

For AI/ML based positioning, based on evaluation results using uniform or grid UE distribution in training data collection, the positioning accuracy is affected by the training dataset size for a given UE distribution area (or equivalently, sample density in #samples/m2), when the UE is distributed uniformly in training data collection.

- There exists a tradeoff between the training dataset size and the achievable positioning accuracy. The larger the training dataset size (i.e., higher sample density), the smaller the positioning error (in meters), until a saturation point is reached where additional training data does not bring further improvement to the positioning accuracy.

- Note: here a sample refers to the training data collected of one UE at one location. Sample density is equivalent to the density of UEs with data collected in the training dataset.

AI/ML assisted positioning

Observation 8.5.1.1-1:

Evaluation of the following generalization aspects show that the positioning accuracy of AI/ML assisted positioning with ToA as model output deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario.

  - Different drops

  - Different clutter parameters

  - Different InF scenarios

Observation 8.5.1.1-2:

For AI/ML assisted positioning, evaluation results demonstrate that for the generalization aspects of:

  - Different drops

  - Different clutter parameters

  - Different InF scenarios

  - Network synchronization error

  - UE/gNB RX and TX timing error

  - SNR mismatch

  - channel estimation error

the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training.

- Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset.

- Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset.

Observation 8.5.2.1-1

For AI/ML assisted positioning with TOA as model output, based on evaluation results of network synchronization error in the range of 0-50 ns, when the model is trained by a dataset with network synchronization error t1 (ns) and tested in a deployment scenario with network synchronization error t2 (ns), for a given t1,

- For a case evaluated by a given source, the positioning accuracy of cases with t2 smaller than t1 is better than the cases with t2 equal to t1. For example,

  - For the case of (t1, t2)=(50ns, 20~25ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(50ns, 20~25ns) is 0.75~0.85 times that of (t1, t2)=(50ns, 50ns).

  - For the case of (t1, t2)=(50ns, 0ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(50ns, 0ns) is 0.76~0.80 times that of (t1, t2)=(50ns, 50ns).

- For a case evaluated by a given source, the positioning accuracy of cases with t2 greater than t1 is worse than the cases with t2 equal to t1. The larger the difference between t1 and t2, the more the degradation. For example,

  - For the case of (t1, t2)=(0ns, 10ns), evaluation results submitted to RAN1#113 show the positioning error of (0ns, 10ns) is 1.16~2.81 times that of (0ns, 0ns).

  - For the case of (t1, t2)=(0ns, 20~25ns), evaluation results submitted to RAN1#113 show the positioning error of (0ns, 50ns) is 2.19~10.11 times that of (0ns, 0ns).

  - For the case of (t1, t2)=(0ns, 50ns), evaluation results submitted to RAN1#113 show the positioning error of (0ns, 50ns) is 9.68~31.95 times that of (0ns, 0ns).

Note: here the positioning error is the horizonal positioning error (meters) at CDF=90%.

Observation 8.5.3.1-1

For AI/ML assisted positioning with TOA as model output, based on evaluation results of timing error in the range of 0-50 ns, when the model is trained by a dataset with UE/gNB RX and TX timing error t1 (ns) and tested in a deployment scenario with UE/gNB RX and TX timing error t2 (ns), for a given t1,

- For a case evaluated by a given source, the positioning accuracy of cases with t2 smaller than t1 is better than the cases with t2 equal to t1. For example,

  - For the case of (t1, t2)=(50ns, 20~25ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(50ns, 20~25ns) is 0.75~0.96 times that of (t1, t2)=(50ns, 50ns).

  - For the case of (t1, t2)=(50ns, 0ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(50ns, 0ns) is 0.76~0.95 times that of (t1, t2)=(50ns, 50ns).

- For a case evaluated by a given source, the positioning accuracy of cases with t2 greater than t1 is worse than the cases with t2 equal to t1. The larger the difference between t1 and t2, the more the degradation. For example,

  - For the case of (t1, t2)=(0ns, 10ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(0ns, 10ns) is 1.34~2.30 times that of (t1, t2)=(0ns, 0ns).

  - For the case of (t1, t2)=(0ns, 20~25ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(0ns, 20~25ns) is 5.66~13.0 times that of (t1, t2)=(0ns, 0ns).

  - For the case of (t1, t2)=(0ns, 50ns), evaluation results submitted to RAN1#113 show the positioning error of (t1, t2)=(0ns, 50ns) is 10.62~51.52 times that of (t1, t2)=(0ns, 0ns).

Note: here the positioning error is the horizonal positioning error (meters) at CDF=90%.

Observation 10.4-1

For AI/ML assisted positioning with TOA as model output, for L in the range of 0.25m to 5m, the positioning error increases approximately in proportion to L, where L (in meters) is the standard deviation of truncated Gaussian distribution of the ground truth label error.

Observation 10.4-2

Evaluation shows that AI/ML assisted positioning with ToA as model output is robust to small label error. The exact range of label error that can be tolerated depends on the positioning accuracy requirement,  where tighter positioning accuracy requirement demands smaller label error.

Direct AI/ML positioning

Observation 6.1.2.1-1 (time domain truncation of Nt)

For direct AI/ML positioning, the positioning accuracy at model inference tends to degrade as the time domain window size Nt decreases, where Nt consecutive time domain samples are used as model input. Evaluation results submitted to RAN1#113 show that show that when CIR or PDP are used as model input, using different Nt while holding other parameters constant,

- Positioning error of Nt=128 is 0.81 ~ 1.19 times the positioning error of Nt=256;

  - Thus: Reducing Nt from 256 to 128 does not appreciably degrade the positioning accuracy, while the measurement size and signaling overhead shrink to ½ of Nt=256.

- Positioning error of Nt=64 is 0.88 ~ 1.79 times the positioning error of Nt=256;

- Positioning error of Nt=32 is 1.05 ~ 4.29 times the positioning error of Nt=256;

  - Thus: Reducing Nt from 256 to 64~32 may degrade the positioning accuracy, while the measurement size and signaling overhead shrink to 1/4 ~1/8 of Nt=256, respectively.

  - In terms of achievable positioning accuracy, Nt can be reduced to 64~32 while maintaining positioning accuracy around or below 1 meter.

Observation 6.1.3.1-1 (size of sub-sampled taps N't)

For direct AI/ML positioning, the positioning accuracy at model inference tends to degrade as N't decreases, where N't time domain samples (i.e., paths) with the strongest power are selected as model input. Evaluation results submitted to RAN1#113 show that for model input of CIR or PDP and Nt=256, using different N't while holding other parameters constant,

- Positioning error of N't=64 is 1.02 ~ 1.21 times the positioning error of Nt=N't=256;

  - Thus: Reducing N't from 256 to 64 does not appreciably degrade the positioning accuracy, while the measurement size and signaling overhead shrink to (approximately) ¼ of Nt=N't=256.

- Positioning error of N't=32 is 1.14 ~ 2.03 times the positioning error of Nt=N't=256;

- Positioning error of N't=16 is 1.35 ~ 2.54 times the positioning error of Nt=N't=256;

  - Thus: Reducing N't from 256 to 32~16 may degrade the positioning accuracy, while the measurement size and signaling overhead shrink to (approximately) 1/8 ~ 1/16 of Nt=N't=256.

  - In terms of achievable positioning accuracy, N't can be reduced to 32~16 while maintaining positioning accuracy around or below 1 meter.

- Positioning error of N't=9~8 is 1.62 ~ 3.29 times the positioning error of Nt=N't=256;

  - Thus: Reducing N't from 256 to 9~8 degrade the positioning accuracy, while the measurement size and signaling overhead shrink to (approximately) 1/32 of Nt=N't=256.

  - In terms of achievable positioning accuracy, N't can be reduced to 9~8 while maintaining positioning accuracy in the range of 1 to 2 meters.

Observation 3.1.1-1

For AI/ML based positioning with multipath measurement for model input,

- For a given set of parameters (N'TRP, Nt, N't, Nport)

  - CIR has the largest measurement size, where CIR is composed of a list of per-path measurements of (a) path delay, (b) path power and (c) path phase.

  - PDP has smaller measurement size than CIR, where PDP is composed of a list of per-path measurements of (a) path delay and (b) path power.

  - DP has the smallest measurement size, where DP is composed of a list of selected path delays.

    - Note: for DP, Nport =1

Observation 3.1.1-2

For AI/ML based positioning with multipath measurement for model input,

- For each model input type (CIR, PDP, DP)

  - The measurement size increases (approximately) linearly as N'TRP increases, where N'TRP is the number of TRPs that provide measurements for the positioning.

- For model input type CIR and PDP,

  - The measurement size increases (approximately) linearly as Nport increases, where Nport is the number of transmit/receive antenna port pairs that provide measurements for the positioning.

  - If N’t (N’t < Nt) samples with the strongest power are selected as model input, measurement size for model input increases (approximately) linearly with N’t;

  - If full set of Nt samples in time domain is used as model input, measurement size for model input (approximately) increases linearly with Nt;

- Note: for Case 2b and 3b, measurement size of model input has impact to signaling overhead for model inference.

Observation 7.3-1

Evaluation shows that direct AI/ML positioning is robust to small label error. The exact range of label error that can be tolerated depends on the positioning accuracy requirement,  where tighter positioning accuracy requirement demands smaller label error.

# Conclusion

TBD

# References

RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP RAN#96, June 2022.

R1-2304339	Evaluation of AI/ML for Positioning Accuracy Enhancement	Ericsson

R1-2304475	Evaluation on AI/ML for positioning accuracy enhancement	vivo

R1-2304538	Evaluation on AI positioning enhancement	ZTE

R1-2304657	Evaluation on AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2304685	Evaluation of ML for positioning accuracy enhancement	Nokia, Nokia Shanghai Bell

R1-2304726	Evaluation on AI/ML for positioning enhancement	CATT

R1-2304768	Discussions on evaluation results of AIML positioning accuracy enhancement	Fujitsu

R1-2304846	On Evaluation of AI/ML based Positioning	Google

R1-2304857	Evaluation on AI/ML for positioning accuracy enhancement	China Telecom

R1-2304897	Evaluation on AI/ML for positioning accuracy enhancement	xiaomi

R1-2305019	Some discussions on evaluation on AI-ML for positioning accuracy enhancement	CAICT

R1-2305089	Discussion on evaluation on AI/ML for positioning accuracy enhancement	CMCC

R1-2305123	Evaluation on AI/ML for positioning accuracy enhancement	InterDigital, Inc.

R1-2305164	Evaluation of AI and ML for positioning enhancement	NVIDIA

R1-2305206	Discussion on AI/ML Positioning Evaluations	Lenovo

R1-2305238 (updated to R1-2305973)	Evaluation on AI/ML for positioning accuracy enhancement	Apple

R1-2305300	Evaluation on AI/ML for positioning accuracy enhancement	LG Electronics

R1-2305332	Evaluation on AI/ML for positioning accuracy enhancement	Qualcomm Incorporated

R1-2305463	Evaluation methodology and results on AI/ML for positioning accuracy enhancement	OPPO

R1-2305509	Evaluation on AI/ML for Positioning	Samsung

R1-2305659	Evaluation of AIML for Positioning Accuracy Enhancement	MediaTek Inc.

R1-2305689	Evaluation of AI/ML for Positioning Accuracy Enhancement	Indian Institute of Tech (M), IIT Kanpur

R1-2305896	Evaluation on AI/ML for Positioning Accuracy Enhancement	CEWiT