3GPP TSG RAN WG1 #113	R1-2306113

Incheon, Korea, May 22-26, 2023

Agenda item:	9.17

Source:	Moderator (Ericsson)

Title:	Summary of higher layer parameters for R18 DSS Enhancements

WI:	NR_DSS_enh

Document for:	Discussion and Decision

# 1	Introduction

This document summarizes discussions in RAN1#113 on higher layer parameters for Rel18 DSS Enhancements WI considering below documents [1-6] submitted for A.I. 9.17.

| Ref# | TDoc | Title | Source | Agenda item |
| --- | --- | --- | --- | --- |
| 1 | R1-2304517 | Discussion on Rel-18 Multi-carrier enhancements, eDSS and NCR | vivo | 9.17 |
| 2 | R1-2304586 | Discussion on remaining issues in RRC signaling for Rel-18 multi-carrier enhancement and eDSS | Spreadtrum Communications | 9.17 |
| 3 | R1-2305475 | Discussion on remaining issues in Rel-18 eDSS | OPPO | 9.17 |
| 4 | R1-2305627 | Discussion on higher layer parameters and remaining issues on Rel-18 WIs | NTT DOCOMO, INC. | 9.17 |
| 5 | R1-2305716 | Higher layer signaling for NR Rel-18 | Ericsson | 9.17 |
| 6 | R1-2305932 | Discussions on draft CRs and higher layer signalling for other Rel-18 items | Huawei, HiSilicon | 9.17 |

# 2	Discussion

Initial version of RRC parameter spreadsheet for RAN1#113 is in drafts folder  –  v001. 
Note – contents in this version are same as eDSS RRC parameters spreadsheet from RAN1#112b-e in R1-2304221 [7].

Also, in RAN1#112b-e, following related draft CRs for TS 38.211 [8] and 38.213 [9] were in principle endorsed and initial list of UE features related to the WI were agreed [10].

Agreement

The draft CR in R1-2304277 for Rel-18 TS38.211 eDSS is endorsed in principle.

Agreement

The draft CR in R1-2304195 for Rel-18 TS38.213 eDSS is endorsed in principle.

The endorsed CRs included an explicit RRC parameter for enabling/disabling ‘Reception of NR PDCCH candidates overlapped with LTE CRS REs’ and details of such parameter were discussed in [112bis-e-R18-RRC-01] email thread (row 4 of the eDSS RRC parameters spreadsheet in [7] which is currently FFS).

In [1-6] higher layer signalling for enabling/disabling ‘Reception of NR PDCCH candidates overlapped with LTE CRS REs’ and potential higher layer signalling to explicitly indicate the UE which channel estimation (CE) mechanism to use (i.e., the ‘legacy CE’, ‘clean symbol CE’ from RAN1#110 agreements) are discussed.

### Question 1

Based on discussion in the tdocs, following options for higher layer signalling related to ‘Reception of NR PDCCH candidates overlapped with LTE CRS REs’ can be considered.

- Option 1 (based on input in [1])

  - The RRC parameter pdcchCandidateReception-WithCRSOverlap is supported to indicate a channel estimation (CE) option to apply.

    - if the RRC indicator is provided, R18 DSS is enabled;

    - if the RRC indicator is not provided, R18 DSS is not enabled

- Option 2 (based on input in [2])

  - Agree to the contents proposed in row 4 of eDSS RRC parameter list (v001) for the parameter to enable/disable ‘Reception of NR PDCCH candidates overlapped with LTE CRS REs’

  - Agree to separate RRC parameter (e.g., PDCCH-DMRS-ChannelEstimation as proposed in [2]) to indicate the recommendation on NR-PDCCH channel estimation method

- Option 3 (based on input in [5])

  - Agree to the contents proposed in row 4 of eDSS RRC parameter list (v001) for the parameter to enable/disable ‘Reception of NR PDCCH candidates overlapped with LTE CRS REs’

  - Note: whether/how to introduce additional RRC parameter related to channel estimation (CE) mechanisms is to be discussed after conclusion on related UE capability details.

- Option 4 (based on input in [6])

  - The “Value range” in row 4 of eDSS RRC parameter list (v001) is updated as “ENUMERATED {‘PDCCH candidate with at least one non-overlapping symbol’, ‘[PDCCH candidate with or without non-overlapping symbol]’}”

Please provide your view on above options for higher layer signalling

| Company Name | Comments |
| --- | --- |
| Vivo | Thanks moderator for the summary. We are ok with either option 1(reuse the same IE of the enabling/disabling of eDSS) or option 2(separate IE) to indicate the CE option. We are also open to option4, but if ‘PDCCH candidate with or without non-overlapping symbol’ indicates the UE to receive the PDCCH overlapped with the LTE CRS, regardless of whether there is a clean symbol; and ‘PDCCH candidate with at least one non-overlapping symbol’ indicates the UE to receive the PDCCH overlapped with the LTE CRS only when there is a clean symbol, we prefer to directly say ‘receive PDCCH candidate with at least one non-overlapping symbol’, ‘[receive PDCCH candidate with or without non-overlapping symbol]’ to make it clear |
| NTT DOCOMO | We support Option 3 at this moment. If RAN4 confirms legacy CE Sumption also works, we prefer to have some indication as assistance information for UE to decide applied CE e.g., either legacy CE assumption or CE on clean symbol(s) only. |
| Qualcomm | We think RAN1 can simply agree to the contents proposed in row 4 of eDSS RRC parameter list (v001) for the parameter to enable/disable ‘Reception of NR PDCCH candidates overlapped with LTE CRS REs’. |
| ZTE | Agree with Qualcomm that it is sufficient to introduce an enabling/disabling RRC parameter. |
| Spreadtrum | We support Option 2. A separate RRC parameter can be introduced to indicate the recommendation on NR-PDCCH channel estimation method. |

### Question 2

Please provide your view on Proposals 1,3,4 in [3] (Note: these are not directly related to higher layer signalling)

| Company Name | Comments |
| --- | --- |
| vivo | Proposal 1, open to have this clarification Proposal 4, these changes are already captured in the CR R1-2304277 in the last meeting |
| Qualcomm | We agree with Proposal 1. Proposal 3 is related to FG52-3. We first need to agree to support FG52-3. On Proposal 4, agree with vivo. |
| ZTE | Proposal 1: The current spec is already clear based on the CR in R1-2304195. Proposal 3: OK to discuss. Proporal 4: Agree with vivo |
| Spreadtrum | We are fine with Proposal 1. Proposal 3 is related to FG52-3. We don’t support FG52-3. For Proposal 4, we agree with vivo. As mentioned in [2], for FG 52-1a, it should be clarified the definition of clean symbol in the case of PDCCH candidate overlap with multiple LTE CRS patterns and LTE CRS pattern corresponds different antenna ports. For example, whether “clean symbol” applies to a PDCCH candidate or PDCCH CPRESET should be clarified. |

### Offline Session Tuesday PM

Proposal - offline stable

- Agree to the contents proposed in row 4 of eDSS RRC parameter list (v001) for the parameter to enable/disable ‘Reception of NR PDCCH candidates overlapped with LTE CRS REs’

- Note: whether/how to introduce additional RRC parameter related to channel estimation (CE) mechanisms is to be discussed after conclusion on related UE capability details.

# 3	Conclusion

Offline Session Tuesday PM

Proposal - offline stable

- Agree to the contents proposed in row 4 of eDSS RRC parameter list (v001) for the parameter to enable/disable ‘Reception of NR PDCCH candidates overlapped with LTE CRS REs’

- Note: whether/how to introduce additional RRC parameter related to channel estimation (CE) mechanisms is to be discussed after conclusion on related UE capability details.

# 4	References

[1] R1-2304517 Discussion on Rel-18 Multi-carrier enhancements, eDSS and NCR	vivo

[2] R1-2304586 Discussion on remaining issues in RRC signaling for Rel-18 multi-carrier enhancement and eDSS

[3] R1-2305475 Discussion on remaining issues in Rel-18 eDSS	OPPO

[4] R1-2305627 Discussion on higher layer parameters and remaining issues on Rel-18 WIs	NTT DOCOMO, INC.

[5] R1-2305716 Higher layer signaling for NR Rel-18	Ericsson

[6] R1-2305932 Discussions on draft CRs and higher layer signalling for other Rel-18 items	Huawei, HiSilicon

[7] R1-2304238, “Consolidated higher layers parameter list for Rel-18 NR”, Moderator (Ericsson), RAN1 #112b-e, eMeeting, April 2023.

[8] R1-2304277, Draft CR to 38.211 on “Introduction of dynamic spectrum sharing enhancements”, Ericsson, RAN1#112b-e, eMeeting, April 2023.

[9] R1-2304195, Draft CR to 38.213 on “Introduction of dynamic spectrum sharing enhancements”, Samsung, RAN1#112b-e, eMeeting, April 2023.

[10] R1-2304221, “Collection of updated higher layers parameter list for Rel-18 NR”, Moderator (Ericsson), RAN1#112b-e, eMeeting, April 2023.

# Annex – Agreements

##### NR PDCCH reception in symbols with LTE CRS REs

###### RAN1#110 (August 2022)

Agreement

If the feature “Reception of NR PDCCH candidates that overlap with LTE CRS REs “is supported, it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation.

Agreement

Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs

PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side.

Note: depends on UE capability

Following options can be used for PDCCH-DMRS channel estimation

- legacy CE assumption

  - RAN1 consider support this, if no RAN4 performance requirements are defined

- CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET)

Note: Restriction on the symbols and/or LTE CRS patterns applicable for above agreements can be considered during UE capability session.

Conclusion

RAN1 understands above agreements applies to 15kHz SCS only.

###### RAN1#109-e (May 2022)

Agreement

To evaluate the following options:

  - Option-1-1: No NR-PDCCH-DMRS is transmitted for only the REs overlapping with LTE-CRS of the OFDM symbol, NR-PDCCH is punctured on REs colliding with LTE-CRS, NR-PDCCH must span at least 2 consecutive symbols with at least 1 symbol not overlapping with LTE-CRS

  - Option-1-2: No NR-PDCCH-DMRS is transmitted in any RE of the OFDM symbol, NR-PDCCH is transmitted on REs not colliding with LTE-CRS including the original DMRS, NR-PDCCH is punctured on REs colliding with LTE-CRS, NR-PDCCH must span at least 2 consecutive symbols with at least 1 symbol not overlapping with LTE-CRS

  - Option-2: NR-PDCCH or NR-PDCCH-DMRS is transmitted on REs not colliding with LTE-CRS, NR-PDCCH and NR-PDCCH-DMRS may or may not be punctured on REs colliding with LTE-CRS

    - No puncture is baseline (UE side)

Observation

For evaluations consider the following options:

Option 1-1:

- PDCCH and PDCCH DMRS mapping to REs: Legacy

- PDCCH REs overlapping with LTE CRS: Receiver punctures

- PDCCH DMRS REs overlapping with LTE CRS: All DMRS REs on overlapping symbol Not used for CE, or legacy pattern is assumed

- gNB transmits: Irrelevant what the gNB transmits on REs overlapping with the LTE CRS REs as indicated in the CRS RM pattern.

- Channel estimator: operate on clean symbol DMRS only, Legacy

Option 1-2:

- PDCCH and PDCCH DMRS mapping to REs: New PDCCH rate-matching

  - No PDCCH DMRS on the symbol overlapping with LTE CRS

- PDCCH REs overlapping with LTE CRS: Receiver punctures

- PDCCH DMRS REs overlapping with LTE CRS: Not expected

- Channel estimator (UE assumption): Operate on clean symbol DMRS only

- gNB transmits: Irrelevant what the gNB transmits on REs overlapping with the LTE CRS REs as indicated in the CRS RM pattern

Option 2:

- PDCCH and PDCCH DMRS mapping to REs: Legacy

- PDCCH REs overlapping with LTE CRS: Baseline: Process as legacy

- PDCCH DMRS REs overlapping with LTE CRS: Aware or unaware

- Channel estimator: Baseline: Process as legacy (Receiver does not puncture DMRS), Optional: Advanced receiver (Use the DMRS other than legacy behavior)

- gNB transmits:

  - Baseline: May puncture the PDCCH/PDCCH DMRS, or may superposition the two.

  - Optional: may puncture LTE CRS of Port#2&3.

  - Impact to LTE UEs should be considered if superposition is used.

Agreement

For evaluations consider the following list of scenarios:

Scenario#1A: 1 symbol CORESET, overlapped with CRS – Option 2 only

Scenario#2: 2 symbols CORESET, including 1 overlapping symbol and 1 clean symbol – Option 1-1/1-2/2

Scenario#3: 3 symbols CORESET, including 1 overlapping symbol and 2 clean symbols – Option 1-1/1-2/2

Agreement

LLS simulations assumptions, [] are optional:

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Bandwidth | 20 MHz [5, 10 MHz], LTE bandwidth equal or smaller than NR |
| Channel model | TDL-C 300, [TDL-A 300] |
| Correlation | Low |
| Number of BS antennas | 4 Tx, (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1), [2 Tx, (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1).] |
| Number of UE antennas | 2 Rx (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) |
| DCI payload (excluding CRC) | 60 bits [50bits] |
| Interleaving | Non-Interleaved, [Interleaved] |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBs |
| CRS | single 4 port CRS pattern, [additional 4 port CRS pattern] |
| Channel estimation | practical – companies to report details |
| UE speed | 30 kmph [3kmph, 120 kmph, 350 kmph] |
| Power ratio of LTE-CRS RE/NR PDCCH RE, Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE | Companies to report (if applicable) |

Agreement

SLS simulations assumptions, [] are optional:

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2.1 GHz |
| SCS | 15 kHz |
| Simulation bandwidth | 20 MHz [5, 10 MHz]，same as that in LLS |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 49 dBm 20 MHz |
| Scenario | Urban Macro (500m ISD), [Rma (1732m ISD)] |
| Device deployment | 80% indoor, 20% outdoor (Uma) [50% indoor,50% in-car (Rma)] |
| UE speeds | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic geometry | Full Buffer |
| Macro sites | 19 |
| Downtilt | 102° or according to Scenario |
| Minimum BS to UE distance | 35m |
| KPI | Companies to report (e.g., total PDCCH capacity, PDCCH coverage/outage, Potential degradation of LTE, whether and how to achieve coexistence with legacy UEs) |
| Others | Companies to report (e.g., fraction of LTE UEs, fraction of Rel-18 DSS NR UEs, etc.). Companies to report considered baseline(s). Baseline(s) aim(s) to be comparable to the evaluated option(s) |

##### UE support for two overlapping CRS rate matching patterns

###### RAN1#110 (August 2022)

Agreement

  - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS.

    - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured only if lte-CRS-PatternList3-r18 is configured in ServingCellConfig.

    - For case when UE is not configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied

    - For case when UE is configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18

      - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’

      - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied.

    - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e.,

      - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on.”

The working Assumption made in RAN1#109e does not need to be confirmed.

Working Assumption(RAN1#109e)

  - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS.

    - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured if lte-CRS-PatternList3-r18 is configured in ServingCellConfig.

    - For case when UE is not configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied

    - For case when UE is configured with two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18

      - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’

      - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied.

    - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e.,

      - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on.”

Agreement

lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applicable to 15 kHz SCS PDSCH only.

Agreement

The following TP for sub-clause 5.1.4.2 of TS 38.214 h20 is endorsed in principle.

## 5.1.4.2 PDSCH resource mapping with RE level granularity <omitted text> A UE may be configured with any of the following higher layer parameters: - REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 or lte-CRS-PatternList3-r18 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - For the UE for broadcast reception, REs indicated by 'RateMatchPatternLTE-CRS' in PDSCH-Config-MCCH or PDSCH-Config-MCCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to ‘0’, or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to ‘1’; - otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig. - If the UE is not configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet, and if the UE is configured by higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are declared as not available for PDSCH. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList3-r18 if the PDSCH is associated with coresetPoolIndex set to ‘0’, or the CRS pattern(s) in lte-CRS-PatternList4-r18 if PDSCH is associated with coresetPoolIndex set to ‘1’; - otherwise, REs indicated by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, in ServingCellConfig. <omitted text>

Agreement

- Agree the following RRC parameters for support of two overlapping LTE-CRS patterns.

- Send an LS to RAN2 about the agreed RRC parameters together with all RAN1 agreements in AI 9.9.2.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList3 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The network does not configure this field and lte-CRS-ToMatchAround, or this field and lte-CRS-PatternList1, or this field and lte-CRS-PatternList2 simultaneously. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |
| NR_DSS_enh | Support for two overlapping CRS rate matching patterns | lte-CRS-PatternList4 | New | A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. This list can be configured if [Rel-18 DSS capability(ies)] specified in TS 38.306 [26] is indicated. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3. The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList3, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and the field lte-CRS-PatternList3 is configured. | LTE-CRS-PatternList-r16 | ServingCellConfig | UE-specific | 38.331 |

Agreement

The draft LS R1-2208193 is endorsed in principle by replacing

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16.

With

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16 in Rel-18 ASN.1.

Agreement

Final LS R1-2208194 is endorsed.

###### RAN1#109-e (May 2022)

Working Assumption

Introduce two new RRC parameters (i.e., list3 and list4). gNB cannot configure legacy list1/2 and new list3/4 simultaneously. If new list3/4 are configured, UE applies list3/4 regardless of whether CORESETPoolIndex= 1 is configured for any of DL BWPs of the serving cell.

Working Assumption

  - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS.

    - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured if lte-CRS-PatternList3-r18 is configured in ServingCellConfig.

    - For case when UE is not configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied

    - For case when UE is configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18

      - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’

      - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied.

    - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e.,

      - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList31, and so on.”

Agreement

- Introduce new UE capability(ies) for support of two overlapping LTE CRS patterns in Rel-18 DSS if the UE is NOT configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet.

  - NW can configure two LTE-CRS overlapping rate matching patterns without coresetPoolIndex only if the UE indicates support of the new capability(ies).

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16.

- Maximum number of LTE-CRS rate matching patterns for PDSCH supported by a UE (i.e., maxNumberPatterns-r16 and maxNumberNon-OverlapPatterns-r16) is kept unchanged.