3GPP TSG RAN WG1 #112bis-e		R1- 2306155

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.1.1.1

Source: 	Moderator (MediaTek Inc.)

Title: 	Moderator summary on extension of unified TCI framework (Final)

Document for:	Discussion and Decision

- Introduction

In RAN#94e, the Rel-18 WID of MIMO evolution for downlink and uplink is approved. In the approved WID, extension of unified TCI framework is a part of the RAN1 objectives, and the detailed scope of this agenda item (AI 9.1.1.1) includes the following highlighted objectives:

RAN1: - Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

- Plan

Based on the contributions from companies [1]-[32], the followings are provided in this document:

- Summary of companies’ views on each of open issues raised by interested companies, where the open issues are categorized as follow:

  - Issue 1 – General issue for unified TCI extension

  - Issue 2 – TCI state update and activation

  - Issue 3 – How to inform UE which indicated TCI state(s) that UE shall apply to target channel/signal

  - Issue 4 – UL power control for UL MTRP operation

  - Issue 5 – PDSCH-CJT Tx scheme

  - Issue 6 – Beam failure recovery

- Observations and recommended proposals based on the summary of companies’ views

- Contact Person

For potential offline discussion, companies/delegates are encouraged to enter the contact information in the table below:

Table 0 Contact Information

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| Apple | Hong | hhe5@apple.com |
| CEWiT | Vishakha Singh | Vish@cewit.org.in |
| CMCC | Yan | liyanwx@chinamobile.com |
| Ericsson | Claes | Claes.tidestav@ericsson.com |
| FGI | Cubie | wanchen.lin@fginnov.com |
| Fraunhofer IIS/HHI | Sutharshun | sutharshun.varatharaajan@iis.fraunhofer.de |
| Fujitsu | Jian | zhangjian1288@fujitsu.com |
| Futurewei | Zhigang | zrong@futurewei.com |
| Google | Alex | alexliou@google.com |
| Huawei, HiSilicon | Keyvan | Keyvan.zarifi@huawei.com |
| Hyundai | Jeongsu Lee | Jeongsu.lee@hyundai.com |
| Intel | Avik | avik.sengupta@intel.com |
| InterDigital | Jonghyun | jonghyun.park@interdigital.com |
| Lenovo | Bingchao Liu | liubc2@lenovo.com |
| LG | Jaehoon | jhoon.chung@lge.com |
| MediaTek | Darcy | darcy.tsai@mediatek.com |
| MediaTek | Rebecca | rebecca.chen@mediatek.com |
| NEC | Peng | guan_peng@nec.cn |
| NTT DOCOMO | Yuki | yuuki.matsumura.vz@nttdocomo.com |
| NTT DOCOMO | Weiqi | sunwq@docomolabs-beijing.com.cn |
| OPPO | Jeffrey | caojianfei@oppo.com |
| Panasonic | Khalid | khalid.zeineddine@eu.panasonic.com |
| Qualcomm | Yan | yanzhou@qti.qualcomm.com |
| Samsung | Dalin | dalin.zhu@samsung.com |
| Sharp | Taka | fukui.takahisa@sharp.co.jp |
| Spreadtrum | Qiyishu Li | qiyishu.li@unisoc.com |
| vivo | Yang | songyang@vivo.com |
| Xiaomi | Mingju LI | limingju@xiaomi.com |
| ZTE | Bo | gao.bo1@ZTE.com.cn |

- Proposal to be discussed in the online session

Agreement

On unified TCI framework extension for M-DCI based MTRP,  An RRC configuration can be provided to an aperiodic CSI-RS resource set or a CSI-RS resource in an aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set or to the CSI-RS resource in the aperiodic CSI-RS resource set, if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state

- The first and the second indicated joint/DL TCI states correspond to the indicated joint/UDL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

- (Working Assumption) Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed)

- Support of ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ RRC configuration is up to UE capability

Proposal 3.13 (offline consensus)

On unified TCI framework extension for M-DCI based MTRP, support of ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ RRC configuration that informs the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set or to the CSI-RS resource in the aperiodic CSI-RS resource set is up to UE capability

Proposal 3.12 (offline consensus)

On unified TCI framework extension for S-DCI based MTRP, if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2:

- When the offset between the reception of the scheduling/activation DCI format 1_0/1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2, the UE shall apply the first indicated joint/DL TCI state to the scheduled/activated PDSCH reception

Proposal 2.3

On unified TCI framework extension, the following cases for CA operation are supported:

- A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in S-DCI based MTRP

  - FFS: How to support common TCI state ID activation/update for this case

  - For the CCs in above set of CCs, TCI state ID activation/update MAC-CE can only be sent to a S-DCI based MTRP CC

- A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in M-DCI based MTRP

  - FFS: How to support common TCI state ID activation/update for this case

- For the CCs in above set of CCs, TCI state ID activation/update MAC-CE can only be sent to a M-DCI based MTRP CC

- a CC in the set of CCs operating in S-DCI/M-DCI based MTRP can be configured as the reference CC.

- For each CC in the above set of CCs, an RRC parameter is configured to the CC to indicate that the first, the second or both joint/DL/UL TCI states are applied to the CC.

Note: “A CC operates in STRP” for above means a CC in which only one joint/UL/DL TCI state is applied

Note: “A CC operates in S/M-DCI based MTRP” for above means a CC operates in Rel-18 unified TCI framework extension for S/M-DCI based MTRP operation

Proposal 4.1.B

On unified TCI framework extension for S-DCI based PUSCH/PUCCH STxMP:

- The UE shall determine a first Tx power for PUSCH/PUCCH transmission occasion i based on the UL PC parameter settings for PUSCH/PUCCH, if any, and the PL-RS included in the first indicated joint/UL TCI state, and a second Tx power for the same PUSCH/PUCCH transmission occasion i based on the UL PC parameter settings for PUSCH/PUCCH, if any, and the PL-RS included in the second indicated joint/UL TCI state

  - The UE shall apply the first Tx power to the PUSCH antenna port(s) applying the first indicated joint/UL TCI state in PUSCH transmission occasion i associated with the first SRS resource set, and the second Tx power to the PUSCH antenna port(s) applying the second indicated joint/UL TCI state in the same PUSCH transmission occasion i associated with the second SRS resource set

  - The UE shall apply the first and second Tx power to PUCCH transmission occasion i

- FFS: UE configured maximum output power when determine the first and second Tx power

- RAN1 should ensure that the sum of the first Tx power and the second Tx power is less than EIRP_max

- Discussion

- Issue 1 – General issue for unified TCI extension

- Issue 2 – TCI state update and activation

Table 2-1 Summary for Issue 2

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 2.1 | (S-DCI) How to configure/determine that a CC is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP? | Question: How to configure/determine that a CC is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP: - Alt1: A CC is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP if an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation that activates at least one TCI codepoint mapped with more than one join TCI states, more than one DL TCI states, or more than one UL TCI states is received and applied in the CC - If all joint/DL/UL TCI states activated by the TCI state activation command are the first joint/DL/UL TCI states or the second joint/DL/UL TCI states, the UE shall fallback to Rel-17 unified TCI framework in the CC - Alt2: A CC is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP if an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation (which is different from the TCI state activation command (MAC-CE) used for Rel-17 unified TCI framework) is received and applied in the CC Support Alt1: ZTE, Intel, Huawei/HiSilicon, Lenovo, Xiaomi, Futurewei, Fujitsu, Docomo, Sharp, QC, CEWiT Support Alt2: vivo, FGI, OPPO, Apple, IDC FL note: Given the majority view on this issue, the following proposal is recommended: Proposal 2.1: On unified TCI framework extension for S-DCI based MTRP, if the UE receives and applies an Rel-18 TCI state activation command (MAC-CE) for S-DCI based MTRP operation to a CC/BWP: - If all activated joint/DL/UL TCI states in the TCI state activation command are the first joint/DL/UL TCI states or the second joint/DL/UL TCI states, the CC/BWP is operated in Rel-17 unified TCI framework - Otherwise, the CC/BWP is operated in Rel-18 unified TCI framework for S-DCI based MTRP operation Support: Nokia, QC, Futurewei, Xiaomi, Docomo, CMCC, ZTE, Fujitsu, Docomo, Apple, Samsung, CATT, Google, Fraunhofer, Spreadtrum, CEWiT Concern: vivo, NEC (2nd bullet), OPPO, IDC, Panasonic, LG (prefer RRC configured), Ericsson, FGI |
| 2.3 | (CA) Common TCI state ID activation/update for a CC list comprised of a mix of STRP CC(s) and MTRP CC(s) | Question: Whether to support each of the following cases for common TCI state ID activation/update: - Case 1: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in S-DCI based MTRP - Case 2: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in M-DCI based MTRP - Case 3: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in S-DCI based MTRP and CC(s) operating in M-DCI based MTRP - Case 4: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP, CC(s) operating in S-DCI based MTRP, and CC(s) operating in M-DCI based MTRP Note: A CC operates in STRP means a CC operates in Rel-17 unified TCI framework Note: A CC operates in S/M-DCI based MTRP means a CC operates in Rel-18 unified TCI framework extension for S/M-DCI based MTRP Support Case 1: ZTE, Spreadtrum, CATT, Fujitsu, Xiaomi, Huawei/HiSilicon, Lenovo, Google, Apple, OPPO, IDC Support Case 2: ZTE, vivo, MTK, Spreadtrum, CATT, Xiaomi, Huawei/HiSilicon, Lenovo, Google, Apple, OPPO, IDC Support Case 3: CATT, Xiaomi, Lenovo, Google, OPPO Support Case 4: CATT, Lenovo, LG Not support all of them: Ericsson, Qualcomm, Panasonic FL note: Given that Case 1 and Case 2 are supported by many companies, the following proposal is recommended. For proponents of Case 1 and/or Case 2, it would be appreciated if you can provide the corresponding solutions for the FFS on how to support Case 1 and/or Case 2. Proposal 2.3: On unified TCI framework extension, the following cases for CA operation are supported: - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in S-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - For above two cases, a CC in the set of CCs operating in S-DCI/M-DCI based MTRP can be configured as the reference CC. - For each CC in the set of CCs not configured as the reference CC, an RRC parameter is configured to the CC to indicate that the first, the second or both joint/DL/UL TCI states are applied to the CC. Note: “A CC operates in STRP” for above means a CC operates in Rel-17 unified TCI framework Note: “A CC operates in S/M-DCI based MTRP” for above means a CC operates in Rel-18 unified TCI framework extension for S/M-DCI based MTRP operation Support: IDC, Xiaomi, OPPO, vivo, Futurewei, Nokia, Docomo, ZTE, Fujitsu, Docomo, Apple, CATT, Huawei/HiSilicon, FGI Concern: QC, CMCC, Samsung, Panasonic, Ericsson, Fraunhofer, NEC (Case 1) |

Table 2-2 Company input for Issue 2

| Company | Input to summary |
| --- | --- |
| Mod V00 | Please check the updated Proposal 2.1 |
| Huawei, HiSilicon | After discussion with a few delegates, we suggest the following change to address some companies’ concern: Proposal 2.3: On unified TCI framework extension, the following cases for CA operation are supported: - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in S-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - For the CCs in above set of CCs, TCI state ID activation/update MAC-CE can only be sent to a S-DCI based MTRP CC - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - For the CCs in above set of CCs, TCI state ID activation/update MAC-CE can only be sent to a M-DCI based MTRP CC - a CC in the set of CCs operating in S-DCI/M-DCI based MTRP can be configured as the reference CC. - For each CC in the above set of CCs, an RRC parameter is configured to the CC to indicate that the first, the second or both joint/DL/UL TCI states are applied to the CC. Note: “A CC operates in STRP” for above means a CC in which only one joint/UL/DL TCI state is applied Note: “A CC operates in S/M-DCI based MTRP” for above means a CC operates in Rel-18 unified TCI framework extension for S/M-DCI based MTRP operation |

- Issue 3 – How to inform UE which indicated TCI state(s) that UE shall apply to target channel/signal

A plan for discussion in this meeting on the TCI selection scheme for each target channel/signal and remaining issues is provided in the following table, including both S-DCI and M-DCI based MTRP operation:

Table 3-0 Summary of TCI selection scheme for each target channel/signal in S-DCI/M-DCI based MTRP operation

| S-DCI based MTRP operation |  |  |
| --- | --- | --- |
| Channel/signal | Conclusion | TCI selection scheme |
| PDCCH | Yes | RRC configuration (first/second/both/none) per CORESET |
| PDSCH scheduled/activated by DCI format 1_1/1_2 if the [TCI selection field] is present in DCI format 1_1/1_2 | Yes | [TCI selection field] in DCI format 1_1/1_2 |
| PDSCH scheduled/activated by DCI format 1_1/1_2 if the [TCI selection field] is not present in DCI format 1_1/1_2 | Yes | Apply both indicated joint/DL TCI states |
| PDSCH scheduled/activated by DCI format 1_0 (including DG and SPS) | Yes | RRC configuration (first/second/both) |
| PDSCH scheduled/activated by DCI format 1_0/1_1/1_2 before threshold if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2 | No | FFS: Detail of the capability of two default beams for S-DCI based MTRP and the threshold value, discussed in Issue 3.12 |
| PUSCH scheduled/activated by DCI format 0_1/0_2 (including DG and Type2 CG) | Yes | The existing SRS resource set indicator in DCI format 0_1/0_2 |
| PUSCH scheduled/activated by DCI format 0_0 (including DG and Type2 CG) | Yes | Apply the first indicated joint/UL TCI state |
| Type1 CG-PUSCH | Yes | RRC configuration (first/second/both) per Type1 CG configuration |
| PUCCH | Yes | RRC configuration (first/second/both) per PUCCH resource/resource group |
| AP CSI-RS for CSI/BM | Yes | RRC configuration (first/second) per resource or per resource set (FFS: whether to have a fixed rule for NCJT CSI discussed in Issue 3.11, AP CSI-RS triggered before threshold) |
| SRS for CB/NCB/AS and AP SRS for BM | Yes | RRC configuration (first/second) per SRS resource set |
| M-DCI based MTRP operation |  |  |
| Channel/signal | Conclusion | TCI selection scheme |
| PDCCH | Yes | According to coresetPoolIndex value |
| PDSCH scheduled/activated by DCI format 1_0/1_1/1_2 | Yes | According to coresetPoolIndex value corresponding to scheduling PDCCH |
| PUSCH scheduled/activated by DCI format 0_0/0_1/0_2 (including DG and Type2 CG) | Yes | According to coresetPoolIndex value corresponding to scheduling PDCCH |
| PUCCH | No | RRC configuration (first/second) per PUCCH resource/resource group (FFS: whether to support Opt3 and/or Opt4, discussed in Issue 3.9) |
| Type1 CG-PUSCH | Yes | RRC configuration (first/second) per Type1 CG configuration |
| AP CSI-RS for CSI/BM | Yes | RRC configuration (first/second) per resource or per resource set |
| SRS for CB/NCB/AS and AP SRS for BM | Yes | RRC configuration (first/second) per SRS resource set. If not configured, according to coresetPoolIndex value corresponding to triggering PDCCH |

Table 3-2 Summary for Issue 3

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 3.8 | (M-DCI) AP CSI-RS for CSI/BM | Agreement On unified TCI framework extension for M-DCI based MTRP, An RRC configuration can be provided to an aperiodic CSI-RS resource set or a CSI-RS resource in an aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set or to the CSI-RS resource in the aperiodic CSI-RS resource set, if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - The first and the second indicated joint/DL TCI states correspond to the indicated joint/UDL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - (Working Assumption) Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) Proposal 3.13 On unified TCI framework extension for M-DCI based MTRP, support of ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ RRC configuration that informs the UE shall apply the first or the second indicated joint/DL TCI state to the aperiodic CSI-RS resource set or to the CSI-RS resource in the aperiodic CSI-RS resource set is up to UE capability |
| 3.9 | (M-DCI) PUCCH | Question: Whether to support Opt3 and/or Opt4? Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Support Opt3: Futurewei, vivo, Fraunhofer, Apple, OPPO, Sharp, CATT Support Opt4: Futurewei, vivo, Huawei/HiSilicon, CATT Support neither: Panasonic, Spreadtrum, CATT, Ericsson, Google, Nokia, QC, Docomo, ZTE, LG, Samsung FL note: It seems there is no consensus to support Opt3 and Opt4. If the situation is not changed, I will not provide proposal for this issue. |
| 3.10 | How to handle the error case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission | FL note: Most companies think if we strictly follow current RAN1 agreements, the UE should follow the indicated joint/UL TCI state(s) applied to the PUSCH transmission, regardless of whether the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) is the same or different from the spatial Tx filter(s) used for the corresponding SRS transmission(s). Majority also prefer no handling in specification for the error case that the spatial Tx filters are not aligned between PUSCH and SRS, i.e., it should/could be avoided by NW implementation. Based on above observations, I think it is beneficial to have the following conclusion to clarify the UE behavior. Conclusion 3.10: On unified TCI framework extension, for a PUSCH transmission, the UE shall apply the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applying to the PUSCH transmission regardless of whether the determined spatial Tx filter(s) is the same or different from the spatial Tx filter(s) used for the SRS transmission(s) corresponding to the SRS resource(s) indicated to the PUSCH transmission - Note: This conclusion does not introduce any specification impact |
| 3.11 | (S-DCI) Whether to introduce a fixed rule for NCJT CSI | Question: Whether to support the FFS in the following agreement for NCJT CSI? Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State for each CSI-RS resource set or for each CSI-RS resource in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - FFS: If the UE is configured for CSI-RS resource set, for an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if above RRC configuration is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. - ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ is up to UE capability Support: Huawei/HiSilicon, Panasonic Not support: vivo |
| 3.12 | (S-DCI) PDSCH scheduled/activated by DCI format 1_0/1_1/1_2 before threshold if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2 | FL note: Based on contributions to this meeting, most companies prefer a simple behavior for this case (i.e., applying the first indicated joint/DL TCI state), thus the following proposal is recommended to this issue: Proposal 3.12 On unified TCI framework extension for S-DCI based MTRP, if the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2: - When the offset between the reception of the scheduling/activation DCI format 1_0/1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2, the UE shall apply the first indicated joint/DL TCI state to the scheduled/activated PDSCH reception |

Table 3-3 Company input for Issue 3

| Company | Comment |
| --- | --- |
| Mod V00 | - Please share your answer to the question in Issue 3.11 - Please share your comment to Proposal 3.12, if any |
| Xiaomi | Issue 3.11 Prefer use RRC for simple. Proposal 3.12 Support and AP CSI-RS can be discussed together. |
| Huawei, HiSilicon | Issue 3.11: Support. And please note that, based on the agreement in the last meeting, this is not an optimization anymore: Since ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ indicated TCI selection configuration is up to UE capability, if UE only supports ‘per CSI-RS resource set’ configuration and is further configured with aperiodic CSI-RS resource set with two Resource Groups for NCJT CSI, based on the current agreement, UE would have to apply the same indicated TCI to the two resource groups. This would be a misbehaviour from UE since the two resource groups are transmitted from two different TRPs. Therefore, we do believe that, at least for the case that UE does not support ‘per CSI-RS resource’ configuration, the rule in the FFS of the above agreement should apply: For an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if the configuration of the indicated TCI selection is not provided per CSI-RS resource, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. Proposal 3.12: OK. |

- Issue 4 – UL power control for UL MTRP operation

Table 4-1 Summary for Issue 4

| # | Issue | Companies’ view and Recommended Proposal |
| --- | --- | --- |
| 4.1 | UL PC for S-DCI based STxMP (including SDM/SFN based PUSCH Tx and SFN based PUCCH Tx) | Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS FL note: Based on above RAN1 agreement, for S-DCI based PUSCH/PUCCH STxMP, the UE needs to determine two UL Tx power according to the first and second indicated joint/UL TCI states for a same PUSCH/PUCCH Tx occasion, where each determined UL Tx power is applied to the corresponding antenna port(s). However, in current UL PC procedure of TS 38.213, only one UL Tx power is determined per PUSCH/PUCCH Tx occasion. Thus, to support S-DCI based PUSCH/PUCCH STxMP, some companies propose to introduce a new parameter (t) in current UL PC determination for PUSCH/PUSCH as follows: PPUSCH,b,f,c,ti,j,qd,l= minPCMAX,f,c,tiPO_PUSCH,b,f,c,tj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,c,tj∙PLb,f,cqd,t+∆TF,b,f,ci+fb,f,ci,lt [dBm], where t=0,1 corresponds to the first and second indicated joint/UL TCI states, respectively. PPUCCH,b,f,c,ti,qu,qd,l= minPCMAX,f,c,tiPO_PUCCH,b,f,c,t(qu)+10log102μ⋅MRB,b,f,cPUCCH(i)+PLb,f,c(qd,t)+∆F_PUCCH(F)+∆TF,b,f,ci+gb,f,c(i,lt) [dBm], where t=0,1 corresponds to the first and second indicated joint/UL TCI states, respectively. Question: Whether to support above modifications to current UL PC procedure of TS 38.213 for S-DCI based PUSCH/PUCCH STxMP? Yes: QC, vivo, NEC, OPPO, Xiaomi, LG, Docomo, CMCC, CATT, Ericsson, FGI No: Samsung, Huawei/HiSilicon Proposal 4.1 (PUSCH): On unified TCI framework extension for S-DCI based MTRP, support the following Tx power determination for S-DCI based PUSCH STxMP (including SDM and SFN based Tx schemes) in TS 38.213: PPUSCH,b,f,c,ti,j,qd,l= minFFS: PCMAX,f,c,ti or PCMAX,f,ciPO_PUSCH,b,f,c,tj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,c,tj∙PLb,f,c, tqd+∆TF,b,f,ci+fb,f,c,ti,l [dBm], where t=0 corresponds to the PUSCH antenna port(s) in PUSCH transmission occasion i applying the first indicated joint/UL TCI state and t=1 corresponds to the PUSCH antenna port(s) in PUSCH transmission occasion i applying the second indicated joint/UL TCI state. Proposal 4.1.B On unified TCI framework extension for S-DCI based PUSCH/PUCCH STxMP: - The UE shall determine a first Tx power for PUSCH/PUCCH transmission occasion i based on the UL PC parameter settings for PUSCH/PUCCH, if any, and the PL-RS included in the first indicated joint/UL TCI state, and a second Tx power for the same PUSCH/PUCCH transmission occasion i based on the UL PC parameter settings for PUSCH/PUCCH, if any, and the PL-RS included in the second indicated joint/UL TCI state - The UE shall apply the first Tx power to the PUSCH/PUCCH antenna port(s) applying the first indicated joint/UL TCI state in PUSCH transmission occasion i associated with the first SRS resource set, and the second Tx power to the PUSCH/PUCCH antenna port(s) applying the second indicated joint/UL TCI state in the same PUSCH transmission occasion i associated with the second SRS resource set - Note: How to capture above in TS 38.213 is up to the spec editor - FFS: UE configured maximum output power when determine the first and second Tx power - FFS: How to ensure that the sum of the first Tx power and the second Tx power is less than EIRP_max |

Table 4-2 Company input for Issue 4

| Company | Input to summary |
| --- | --- |
| Mod V00 | Please check Proposal 4.1.B |
| Nokia/NSB | On the earlier proposal that discussed power control formula, it seems that the proposed formulas consider more power limitation per panel for STxMP – which we think is fine to support. Overall, it would be good to clarify which approach/assumption is considered regarding power limitation. - However, the power adjustment component ∆TF,b,f,ci should also be discussed (for PUSCH power control), as it’s defined as follows based on the existing specifications: ∆TF,b,f,ci=10log102BPRE⋅Ks-1⋅βoffsetPUSCH, where BPRE=r=0C-1KrNRE, and Kr is a size for code block r. Hence, the impact on ∆TF,b,f,ci, and specifically BPRE (bits per resource element), would need to be discussed for the two PUSCH transmission occasions in case of SDM scheme since the layers and thus code-block are split between the two PUSCH occasions. So similar to other parameters, and using the indexing suggested by FL, we would need to consider ∆TF,b,f,c,ti. Then this ∆TF,b,f,c,ti should be further discussed. - Finally, on the closed-loop power control parameter, shouldn’t we first discuss/ agree on the following (as we previously suggested)? - For Rel-18 multi-TRP PUSCH/PUCCH operations under unified TCI framework, reuse the Rel-17 multi-TRP power control enhancement as follows: - a second TPC field can be configured in DCI formats 1_1/1_2 and 0_1/0_2 to support per-TRP closed-loop power control. Specifically, first and second indicated TPC commands would apply to UL transmissions/repetitions mapped to first and second indicated TCI states, respectively; - if the second TPC field is not configured, indicated TPC command applies to UL transmissions/repetitions mapped to both first and second indicated TCI states. |
| Xiaomi | First, we are OK with the association between PUSCH/PUCCH and the open loop power control parameters described in the first sub-bullet of proposal 4.1B. For the sub-bullet of the first sub-bullet, from our understanding, it needs to be modified as: - The UE shall apply the first Tx power to the PUSCH antenna port(s) in PUSCH transmission occasion i associated with the first SRS resource set indicated joint/UL TCI state, and the second Tx power to the PUSCH antenna port(s) in the same PUSCH transmission occasion i associated with the second SRS resource set indicated joint/UL TCI state. In addition, agree with Nokia, the closed-loop power control parameter indication also needs to be discussed. For example, the closed-loop power control parameter indication method for single-DCI based MTRP UL TDM transmission, which are indicated via two TPC field, can be reused. |
| Huawei, HiSilicon | Proposal 4.1B: Proposal is quite vague. It doesn’t address the issue of max Power per panel and the mechanism to ensure that the total UE power is less than EIRP_max. Also, the proposal is just a general description of the parameters involving PC. How we can leave the exact specification to the Editor based on such general description? There are multiple quite different ways to capture such an agreement. |

- Issue 5 – PDSCH-CJT Tx scheme

- Issue 6 – Beam failure recovery

- Other issue

If there is any important issue not captured in the discussion of previous meetings, company can input to Table 7-1.

Table 7-1 Company inputs for other issue

| Company | Input |
| --- | --- |
| FGI | We’ve already had an agreement on the behavior when the TCI field is present, and the TCI selection field is absent. However, the case that the TCI field is absent, but the TCI selection field is present still needs to be clarified. In such case, it is unclear that the codepoint of the TCI selection field corresponds to which set of indicated TCI states. If the UE is not expected to be configured with such situation, we might need to specify it in the meeting. |
| Huawei, HiSilicon | We think the agreement for aperiodic CSI-RS of sDCI based MTRP is incomplete. In the last meeting, we had the following agreement: Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State for each CSI-RS resource set or for each CSI-RS resource in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - FFS: If the UE is configured for CSI-RS resource set, for an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if above RRC configuration is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. - ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ is up to UE capability. Since ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ indicated TCI selection configuration is up to UE capability, if UE only supports ‘per CSI-RS resource set’ configuration and is further configured with aperiodic CSI-RS resource set with two Resource Groups for NCJT CSI, based on the current agreement, UE would have to apply the same indicated TCI to the two resource groups. This would be a misbehaviour from UE since the two resource groups are transmitted from two different TRPs. Therefore, we do believe that, at least for the case that UE does not support ‘per CSI-RS resource’ configuration, the rule in the FFS of the above agreement should apply: For an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if the configuration of the indicated TCI selection is not provided per CSI-RS resource, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. |
| Fraunhofer IIS/HHI | The default TCI states in the following scenarios, if applicable, should be discussed: - PDSCH configured for STRP while PDCCH configured with MTRP repetition or SFN. - PUSCH configured for STRP while PUCCH configured for MTRP. - Other hybrid STRP-MTRP configurations across various channels. |

- Appendix: Agreements/conclusions before/in RAN1#113

RAN1#113

Hope we can have similar progress as we had in the previous meeting

RAN1#112b-e

Conclusion On unified TCI framework extension for S-DCI based MTRP operation, there is no consensus to support dynamic switching between single-TRP operation and multi-TRP operation for channels/signals based on the number of TCI states mapped to the received TCI codepoint in DCI format 1_1/1_2 - FFS: How to switch between Rel-17 sTRP operation and Rel-18 mTRP operation Agreement On unified TCI framework extension, the Rel-17 timeline for updating the indicated joint/DL/UL TCI state(s) is retained, i.e., the indicated joint/DL/UL TCI state(s) applied to the DL reception or UL transmission in each slot is updated based on the Rel-17 beam application time Agreement On unified TCI framework extension for S-DCI based MTRP, the UE shall apply the first indicated joint/UL TCI state to PUSCH transmission(s) scheduled/activated by DCI format 0_0 (including DG and Type2 CG) Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration is provided to a Type1 CG configuration to inform that the UE shall apply the first, the second, or both indicated joint/UL TCI states to the corresponding CG-PUSCH transmission - If the first or the second indicated joint/UL TCI state is applied, the UE shall apply the first or the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If both indicated joint/UL TCI states are applied: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB - FFS: SDM and SFN based PUSCH Tx schemes Agreement On unified TCI framework extension for S-DCI based MTRP, PDSCH-CJT Tx scheme is RRC-configured, and dynamic switching between PDSCH-CJT and other S-DCI based PDSCH Tx schemes is not supported Agreement If the UE is configured with SSB-MTC-AdditionalPCI and receives TCI state activation command (MAC-CE) that activates a set of joint/DL /UL TCI state(s) specific to each coresetPoolIndex value for M-DCI based MTRP in unified TCI framework extension, the activated joint/DL /UL TCI state(s) specific to one coresetPoolIndex value is associated with the serving cell PCI and the activated joint/DL /UL TCI state(s) specific to another coresetPoolIndex value can be associated with a PCI other than the serving cell PCI . - Note: How to implement above in specification is up to spec editor Agreement On unified TCI framework extension for M-DCI based MTRP, after NW response to TRP-specific BFR request to a BFD-RS set associated with a coresetPoolIndex value, QCL assumption/spatial Tx filter/PL-RS for channel(s)/signal(s) that applies the indicated joint/DL /UL TCI state specific to the coresetPoolIndex value are updated according to the new beam (q new ) corresponding to the BFD-RS set. Agreement On unified TCI framework extension for S-DCI based MTRP, the presence of the [TCI selection field] can be RRC-configured per DL BWP - FFS: Whether the presence of the [TCI selection field] can be configured individually for DCI format 1_1 and DCI format 1_2 in the same DL BWP Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint Agreement On unified TCI framework extension for M-DCI based MTRP, support at least Opt2 for PUCCH transmission, and Opt1 is not supported - Note: Opt3 and Opt4 are not precluded Conclusion On unified TCI framework extension for S-DCI based MTRP, there is no consensus in RAN1 on whether to reuse the Rel-17 RRC parameter followUnifiedTCIstate as a part of the RRC configuration that informs the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to a CORESET - Above does not impact how RAN2 writes their specifications Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration can be provided in CSI-AssociatedReportConfigInfo of CSI-AperiodicTrigger State for each CSI-RS resource set or for each CSI-RS resource in each aperiodic CSI-RS resource set to inform that the UE shall apply the first or the second indicated joint/DL TCI state to the CSI-RS resource if the aperiodic CSI-RS resource set for CSI/BM is configured to follow unified TCI state - Above applies at least if the offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in the aperiodic CSI-RS resource set is equal to or larger than a threshold (if the threshold is needed) - FFS: If the UE is configured for CSI-RS resource set, for an aperiodic CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if above RRC configuration is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2. - ‘per CSI-RS resource set’ or ‘per CSI-RS resource’ is up to UE capability Agreement On unified TCI framework extension, support the following cases for CA operation: - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in S-DCI based MTRP - A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in M-DCI based MTRP - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in S-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in S-DCI based MTRP and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case - FFS: A set of CCs configured for common TCI state ID activation/update can include CC(s) operating in STRP, CC(s) operating in S-DCI based MTRP, and CC(s) operating in M-DCI based MTRP - FFS: How to support common TCI state ID activation/update for this case Agreement On unified TCI framework extension for M-DCI based MTRP, an RRC configuration is provided to a Type1 CG configuration to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding CG-PUSCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

RAN1#112

Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value Agreement On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported Agreement On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed)

RAN1#111

Agreement On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Increase on the size of the TCI field - Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design Agreement On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework Agreement On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 Agreement On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT): - Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA - Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state - Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB Agreement On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH. Agreement On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - FFS: Detail design of the new indicator field

RAN1#110b-e

Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support simultaneous configuration of both joint and separate DL/UL TCI modes in a serving cell Conclusion On unified TCI framework extension in Rel-18, there is no consensus to support separate RRC-configured TCI state list(s) for each of TRPs Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - FFS: PL-RS(s), and UL PC parameter setting(s) (including P0, alpha, and closed loop index) for the PUSCH Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUCCH transmission: - Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group - Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group associated with the same CORESET group - Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group - Note: the association indicates whether the UE shall apply the first one, the second one, or both of the joint/UL TCI states indicated by DCI/MAC-CE to a PUCCH resource/group Agreement On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode - Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability - FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT - Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1 Agreement On unified TCI framework extension for M-DCI based MTRP: - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value - The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value - FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value - FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request Agreement On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111: - Alt1: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) for one of the two TRPs or both TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Alt2: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) only specific to one of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list - Note: According to the agreement in RAN1#109-e, support of one additional TCI field or a field associating the TCI field to the TRP(s) is not precluded Note: It has been agreed to use the existing TCI field for TCI state indication for S-DCI based MTRP in RAN1#109e Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS FFS: The behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP

RAN1#110

Agreement On unified TCI framework extension, for the target use cases agreed in RAN1#109-e in AI 9.1.1.1, up to 4 TCI states can be indicated in a CC/BWP or a set of CCs/BWPs in a CC list to DL receptions and/or UL transmissions, where these TCI states are indicated/updated by MAC-CE/DCI with the necessary MAC-CE based TCI state activation - FFS: The possible combination(s) of joint/DL/UL TCI states that can be indicated to DL receptions and/or UL transmissions in a BWP/CC/TRP - Note: This agreement does not imply that there will be more than 2 DL or UL or joint TCI states indicated in a CC/BWP for the target use cases agreed in RAN1#109-e in AI 9.1.1.1 - Note: The maximum number of TCI states that can be indicated to each of the target use cases agreed in RAN1#109-e in AI 9.1.1.1 is remained the same as in Rel-16/17 Note: The maximum number of TCI states that can be indicated simultaneously to CJT-based PDSCH reception and the required type(s) of TCI states (i.e., DL /UL/joint) are independently discussed in this AI Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, down-selection at least one alternative from the followings: - Alt1-1: Use RRC parameter(s) in a CORESET configuration to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on the CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE - Alt1-2: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/DL TCI state(s) is associated with each CORESET group - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the CORESET group(s) - FFS: How to associate the indicated joint/DL TCI state(s) with each CORESET group - FFS: The UE applies the indicated joint/DL TCI state(s) to a CORESET according to the CORESET group(s) the CORESET belongs to, or the UE applies the indicated joint/DL TCI state(s) associated with the CORESET group(s) in which the beam indication DCI is received to all PDCCH receptions - Alt2: The association between a CORESET and the indicated joint/DL TCI state(s) is determined based on a fixed rule, and the UE shall apply the indicated joint/DL TCI state(s) to the corresponding PDCCH receptions on the CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE - Alt3: Use MAC-CE to inform the UE whether and/or which indicated joint/DL TCI state(s) shall be applied to the corresponding PDCCH receptions on a CORESET - FFS: Whether only the CORESET(s) that always/can share the unified TCI state as defined in Rel-17 unified TCI framework can be associated with the joint/DL TCI state(s) indicated by DCI/MAC-CE Switching between multi-TRP and single TRP operation is not precluded Agreement On unified TCI framework extension for S-DCI based MTRP, for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2, down-selection one alternative from the followings: - Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in a DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - Alt2: PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2 - Alt3: Use an RRC parameter in a CORESET configuration to inform that the CORESET belongs to which CORESET group(s), and the indicated joint/UL TCI state(s) is associated with each CORESET group. When a scheduling/activation DCI format 0_1/0_2 is received in a CORESET group, the indicated joint/UL TCI state(s) associated with the CORESET group is applied to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 - FFS: Details of CORESET group(s) FFS: PUSCH transmission scheduled/activated by a DCI format 0_0 and Type-1 CG-PUSCH Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with joint/UL TCI state(s) indicated by DCI/MAC-CE for PUCCH transmission, down-selection at least one alternative from the followings: - Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group - Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group - Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group - Alt4: Use DCI to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

RAN1#109e

Agreement On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17 - Consider, if STxMP is supported, Rel-18 MTRP scheme(s) with STxMP Agreement On unified TCI framework extension at least for single-DCI based MTRP, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Detail of mapping joint/DL/UL TCI state ID(s) to a TCI codepoint, e.g., possible combinations of joint, DL, and/or UL TCI state IDs that can be mapped to a TCI codepoint - FFS: Whether to increase the max number of MAC CE activated TCI codepoints, i.e., more than 8 codepoints - FFS: Whether to increase the max number of TCI field bits, i.e., more than 3 bits - Note: This doesn't imply that support of one additional TCI field or a field associating the TCI field to the TRP(s) is precluded Note: The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS Agreement On unified TCI framework extension for M-DCI based MTRP, consider the following alternatives for TCI state update: - Alt1: Reuse the same TCI state update scheme for S-DCI based MTRP - Atl2: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate the joint/DL/UL TCI state(s) corresponding to the same CORESETPoolIndex value - Alt3: Use the existing TCI field in any DCI format 1_1/1_2 (with or without DL assignment) to indicate all joint/DL/UL TCI states corresponding to both CORESETPoolIndex values - Study the association between the indicated joint/DL/UL TCI state(s) and a CORESETPoolIndex value - Alt4: Use the existing TCI field in the DCI format 1_1/1_2 (with or without DL assignment) associated with one of CORESETPoolIndex values to indicate joint/DL/UL TCI state(s) corresponding to the same or different CORESETPoolIndex value. - Study whether the indicated joint/DL/UL TCI state(s) applies to the channels/signals associated with the same CORESETPoolIndex value or different CORESETPoolIndex value is indicated by DCI Agreement On unified TCI framework extension for S-DCI based MTRP, consider at least the following alternatives to map/associate a joint/DL TCI state to PDCCH reception(s) - Atl1: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a CORESET or a CORESET group - Alt2: Use RRC configuration to inform the mapping/association between a configured or indicated joint/DL TCI state and a search space set - Alt3: Use MAC-CE to inform the mapping/association between an activated or indicated joint/DL TCI state and a CORESET or a CORESET group - Alt4: Use DCI to inform the mapping/association between an indicated joint/DL TCI state and a CORESET or a CORESET group - Alt5: Based on a fixed mapping/association rule, e.g., the first indicated joint/DL TCI state always applies to PDCCH receptions Consider above alternatives for PDCCH repetition, PDCCH-SFN, PDCCH w/o repetition/SFN, and potential support of dynamic switching between S-TRP and M-TRP for PDCCH. It is not precluded to adopt one single alternative or multiple alternatives to support these cases. Agreement On unified TCI framework extension, if an indicated joint or UL TCI state applies to a PUSCH/PUCCH transmission occasion at least for S-DCI based PUSCH/PUCCH repetition with TDM and the indicated joint or UL TCI state is associated with an UL PC parameter setting for PUSCH/PUCCH (including P0, alpha for PUSCH, and closed loop index) and a PL-RS, the UE should apply the UL PC parameter setting and the PL-RS for the PUSCH /PUCCH transmission occasion. - FFS: How to extend to other Rel-18 MTRP scheme(s) with STxMP, if supported - FFS: UL PC enhancement for CB and non-CB SRS in above case FFS: The applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 is endorsed in R1-2205639.

- References

| 1 | R1-2305495 | Views on unified TCI extension focusing on m-TRP | Samsung |
| --- | --- | --- | --- |
| 2 | R1-2304963 | Discussion on unified TCI framework extension for multi-TRP | Hyundai Motor Company |
| 3 | R1-2304950 | Discussion of unified TCI framework for multi-TRP | Lenovo |
| 4 | R1-2305583 | Discussion on unified TCI framework extension for multi-TRP | NTT DOCOMO, INC. |
| 5 | R1-2305077 | Discussion on unified TCI framework extension for multi-TRP | CMCC |
| 6 | R1-2305060 | Multi-TRP enhancements for the unified TCI framework | Fraunhofer IIS, Fraunhofer HHI |
| 7 | R1-2304988 | Discussion on unified TCI framework extension for multi-TRP | NEC |
| 8 | R1-2305008 | Discussion on unified TCI framework extension for multi-TRP | Google |
| 9 | R1-2305401 | Unified TCI framework extension for multi-TRP | OPPO |
| 10 | R1-2305288 | Unified TCI framework extension for multi-TRP/panel | LG Electronics |
| 11 | R1-2305318 | Extension of unified TCI framework for mTRP | Qualcomm Incorporated |
| 12 | R1-2305226 | Unified TCI framework extension for multi-TRP | Apple |
| 13 | R1-2304817 | On Unified TCI Framework for multi-TRP | Intel Corporation |
| 14 | R1-2304873 | Unified TCI framework extension for multi-TRP | xiaomi |
| 15 | R1-2304783 | Unified TCI framework extension for multi-TRP | Ericsson |
| 16 | R1-2304760 | Discussion on unified TCI framework extension for multi-TRP | Fujitsu |
| 17 | R1-2304705 | Discussion on unified TCI framework extension for multi-TRP operation | CATT |
| 18 | R1-2304636 | Discussion on unified TCI framework extension for multi-TRP | Huawei, HiSilicon |
| 19 | R1-2304542 | Discussion on unified TCI framework extension for multi-TRP | Spreadtrum Communications |
| 20 | R1-2304421 | Enhanced Unified TCI for mTRP | InterDigital, Inc. |
| 21 | R1-2304375 | Unified TCI framework extension for multi-TRP | Panasonic |
| 22 | R1-2304392 | Enhancements on unified TCI framework extension for multi-TRP | ZTE |
| 23 | R1-2304463 | Further discussion on unified TCI framework extension for multi-TRP | vivo |
| 24 | R1-2304348 | Unified TCI framework extension for multi-TRP | FUTUREWEI |
| 25 | R1-2305892 | Discussion on Unified TCI framework extension for multi-TRP | CEWiT |
| 26 | R1-2305771 | Discussion on unified TCI framework extension for multi-TRP | ITRI |
| 27 | R1-2305776 | Discussion on unified TCI framework extension for multi-TRP | FGI |
| 28 | R1-2305748 | Unified TCI framework extension for multi-TRP | Nokia, Nokia Shanghai Bell |
| 29 | R1-2305733 | Unified TCI framework extension for multi-TRP | Sharp |
| 30 | R1-2305704 | Discussion on unified TCI framework extension for multi-TRP | Transsion Holdings |
| 31 | R1-2305642 | Unified TCI framework extension for multi-TRP | MediaTek Inc. |