3GPP TSG-RAN WG1 Meeting #113 	R1-2306262

Incheon, Korea, May 22nd – 26th, 2023

Agenda Item:	9.7

Source:	Rapporteur (Huawei)

Title:	Sorted RAN1 agreements for Rel-18 network energy savings post 113

Document for:	Information

- Introduction

This document contains RAN1 agreements made till RAN1#113 meeting, for Rel-18 WI network energy savings for NR.

- Agreements per meeting and agreements per topic

NZP CSI-RS resource configuration for channel measurement

Agreement@112

For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases

- Type 1: all antenna elements associated to a logical antenna port is disabled/enabled

- Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

Agreement @112bis-e

Define necessary enhancements to support both types of spatial adaptation cases (as defined in RAN1#112) in Rel-18.

- Note: This does not imply explicit definition in specifications for adaptation types.

- Note: This does not imply explicit specification changes are made for both cases

Agreement@112

For spatial element adaptation, further study the following

- A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern

  - FFS: Details on how the association is done

- A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns

  - FFS: Details on how the association is done

- FFS: Details on the definition of “spatial adaptation patterns”

Agreement@112bis-e

Support configurability of NZP CSI-RS resource(s) for channel measurement within one resource setting corresponding to more than one spatial adaptation patterns with at least one of the following

- A1-1-revised: a resource set with multiple resources is configured within a resource setting, where each resource is associated with only one spatial adaptation pattern

- A1-2-revised: For a resource configured in a resource set within a resource setting, the resource can be associated with more than one spatial adaptation patterns

  - One or more resources can be configured in the resource set for channel measurement.

Agreement@112bis-e

    - For R18 NES, only legacy port configuration values (N1, N2) or (Ng, N1, N2) are supported.

    - FFS: Whether/what restriction for A1-1-revised and A-1-2-revised w.r.t number of ports

Agreement@113

- For A1-1-revised for Type 2, one or more CSI-RS resources from a CSI-RS resource set for channel measurement can be associated with the same sub-configuration provided in a CSI report configuration

  - Resources in the resource set for channel measurement have the same number of antenna ports

- For A1-2-revised for Type 1, all CSI-RS resource(s) (which can be one or more) in the CSI-RS resource set for channel measurement are associated with each sub-configuration provided in a CSI report configuration

  - i.e. each CSI-RS resource is associated with all the sub-configurations

  - Resources in the resource set for channel measurement have the same number of antenna ports

- FFS: restriction on total number of CSI-RS resources for channel measurement in a CSI-ReportConfig and/or sub-configuration.

Working Assumption@112bis-e

Al-1-revised and A1-2-revised are supported

    - FFS: Which Type of SD adaptation A1-1-revised and A1-2-revised are applicable for

Agreement@113

Confirm the working assumption with the following update (in blue)

    - Al-1-revised and A1-2-revised are supported

      - For Type 1 SD adaptation

        - A1-2-revised is supported

      - For Type 2 SD adaptation

        - A1-1-revised is supported.

Conclusion@112bis-e

New CSI-RS resource (RE mapping) pattern is not introduced for R18 network energy savings purpose.

- Note: CSI-RS resource (RE mapping) pattern above refers to a row in TS 38.211 Table 7.4.1.5.3-1 determining CSI-RS locations within a slot.

CSI report configuration including the sub-configurations

Agreement@112

For spatial element adaptation, further study the following

- A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern

- A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern

  - FFS: Details of sub-configuration

Agreement@112

For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g.

- FFS: gNB indicates to UE which CSI(s) the UE shall report

- FFS: the UE selects which CSI(s) are reported

- FFS: multiple CSI(s) are reported in a joint CSI report

- FFS: Overhead reduction for multiple CSI(s)

Note: UE complexity needs to be taken into account.

Agreement@112bis-e

At least support A2-2, i.e. one CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern.

- FFS: impact on CSI processing requirement

Agreement@112bis-e

For CSI report configuration, if L>1 in a CSI report configuration, at least the following can be included for each sub-configuration for Type 1 SD adaptation

- N1, N2 for single-panel and N1, N2, Ng for multi-panel

  - FFS: details on explicit indication or implicit derivation

- Port subset indication when A1-2 is used (if A1-2 is supported)

  - FFS: details on explicit indication or implicit derivation

- FFS: rank restriction

- FFS: codebook subset restriction

- FFS: supported codebook types for PMI, e.g., Type-I or Type-II

- FFS: report quantity

- FFS: reportFreqConfiguration

- FFS: Group identity of NZP CSI-RS resource(s) in a resource set for channel measurement when A1-1 is used

For CSI report configuration for type 2 SD adaptation, further study under which cases sub-configurations may or may not be needed including sub-configuration content

Agreement@113

For a CSI report configuration with L>1, for Type 1 SD, at least when A1-2-revised is used for the associated codebook configuration,

- Only common codebook type for PMI across sub-configurations is supported

  - Codebook type-1 for PMI is supported

Agreement@113

For Type 1 adaptation, for each sub-configuration,

- Port subset indication is based bitmap is supported

    - One bit per port for single panel case (i.e. turning off in a port level)

    - FFS: One bit per panel for multi-panel case (i.e. turning off in panel level)

    - Note: It is up to the gNB to ensure the mapping of the bit to a uniform x-pol rectangular array

Agreement@113

For Type 1 adaptation, for each sub-configuration, for multi-panel case,

- One bit per port based on bitmap is supported

- Note: It is up to the gNB to ensure the mapping of the bit to a uniform x-pol rectangular array for each of the activated panel(s). Additionally, if more than one panel is activated, uniformity across panels is ensured by the gNB (i.e., the same N1, N2 across multiple activated panels)

Agreement@113

For the sub-configuration(s) in a CSI report configuration with L>1,

- for Type 1 SD with A1-2-revised, the following is configured in each sub-configuration

    - codebook subset restriction,

    - rank restriction

    - N1, N2 and Ng

    - FFS: the case when the number of ports is less than 4

- for Type 2 SD adaptation with A1-1-revised, for each sub-configuration

    - a list of CSI-RS resource ID

    - FFS: codebookConfig (including codebookSubsetRestriction/ ri-Restriction)

    - FFS: CQI table indication

    - FFS: reportFreqConfiguration

    - FFS: report quantity

Above is agreed in addition to what was agreed in previous RAN1 agreements

CSI reporting procedures

(Rapporteur note: CSI reporting framework)

Agreement@112

For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g.

- FFS: gNB indicates to UE which CSI(s) the UE shall report

- FFS: the UE selects which CSI(s) are reported

- FFS: multiple CSI(s) are reported in a joint CSI report

- FFS: Overhead reduction for multiple CSI(s)

Note: UE complexity needs to be taken into account.

Agreement@112bis-e

For a CSI report config with L sub-configuration(s), support a framework that enables a UE to report N CSI(s) in one reporting instance where the N CSI(s) are associated with N sub-configuration(s) from L (where 1≤N≤L) and each CSI corresponds to one sub-configuration.

- For discussion purpose, N=1 refers to single-CSI while N>1 refers to multi-CSI.

- For Semi-persistent/Aperiodic CSI reporting, support gNB trigger/indicate/activate report of N≤L CSIs where N>=1

- The maximum value of N and L are subject to UE capability

- Further study how to address/minimize additional UE complexity

The following bullet not agreed due to objection from Apple and vivo

- For Periodic CSI reporting, at least the case of N=L is supported where N>=1

(Rapporteur note: CSI payload/reportQuantity)

Agreement@112bis-e

- For CSI feedback with CSI overhead/report payload reduction, further study whether/how to report a common value and/or a differential and/or joint coded value across same CSI quantity of different sub-configurations/adaptation patterns, at least for the following

  - CRI

  - RI

  - PMI

  - CQI

  - FFS: L1-RSRP

  - Other (new) report quantity, if any

- Further study whether/how it is feasible/possible for the UE to skip the evaluations of some sub-configurations/adaptation patterns to reduce the burden at the UE

Agreement@113

For both spatial domain NES, when UE reports CSIs corresponding to one or more sub-configurations provided in a CSI report configuration,

- At least support baseline: Report CSI for each indicated sub-configuration, according to reportQuantity configuration

  - FFS: details on how to map CSI(s) in a CSI report

- Further enhancement on CSI payload reduction is not precluded

(Rapporteur note: CPU counting)

Agreement@113

For spatial domain adaptation or power domain adaptation, for CSIs reporting corresponding to N indicated sub-configurations from L sub-configurations in a CSI report, for the case without CSI payload reduction

- OCPU=i=1[N or L]Ksi, where Ksi is the total number of CSI-RS resources corresponding to i-th sub-configuration in the CSI-RS resource set for channel measurement.

  - the summation is over N for A-CSI RS

  - This is for CSI processing criteria for NES in Clause 5.2.1.6 of TS 38.214

Agreement@113

Alt 2: For P-CSI reporting from L configured sub-configurations, support:

- All L configured sub-configurations are reported in every periodic occasion.

- The maximum value of L can be different for A-CSI, SP-CSI, and P-CSI.

- OCPU=i=1[N or L]Ksi, where Ksi is the total number of CSI-RS resources corresponding to i-th sub-configuration in the CSI-RS resource set for channel measurement. (N=L in the equation)

- FFS: Details on active CSI-RS resource / port counting

L1 signaling aspects

Agreement@112

Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that

- Whether there is a need for transition time per adaptation (for UE)

- Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

Agreement@112bis-e

For Semi-persistent/Aperiodic CSI reporting with 1≤N≤L, study what enhancements to the current DCI and MAC-CE mechanisms are needed for gNB triggering/indication/activation of the N CSI(s) in a reporting instance, where the N CSI(s) are associated with N sub-configuration(s) from L in a report config.

Agreement@113

For N>=1 CSI reporting corresponding to N out of L sub-configurations in one reportConfig where each sub-configuration corresponding to an SD adaptation pattern or/[and] a powerControlOffset value,

- For A-CSI and SP-CSI on PUSCH report, support DCI-based triggering

  - For A-CSI-RS, CPU and CSI-RS resource/port counting depend on N indicated sub-configurations

    - FFS: How to do the counting

  - FFS: For P-CSI-RS/SP-CSI-RS, CPU and CSI-RS resource/port counting depend on L or N sub-configurations

- For SP-CSI on PUCCH report, support MAC-CE-based triggering

  - FFS: For P-CSI-RS/SP-CSI-RS, CPU and CSI-RS resource/port counting depend on L or N sub-configurations

Note: UE complexity reduction is not precluded

- For DCI-based triggering,

  - Alt 1: A triggering state corresponding to N sub-configurations is indicated via the existing CSI request field in DCI. Different triggering states could represent different subsets of L sub-configurations.

    - The DCI is UE specific (in this case, legacy DCI format applies)

- For MAC-CE based triggering

  - Opt 2: An indication to select to N sub-configurations in a MAC-CE is supported

    - It is up to RAN2 to decide the signaling designs of the MAC-CE (including whether it is a new MAC CE or an existing MAC CE)

    - Only one MAC CE is used for this triggering

Power domain (PD) adaptation techniques

Agreement@112

For adaptation of power offset values between PDSCH and CSI-RS, further study the following

- Where/how to configure multiple power offset values

  - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception

  - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS

  - Whether other UE report content can be included

Agreement@112bis-e

For power domain adaptation, for CSI(s) reporting, support configuration of more than one power offset values for PDSCH relative to CSI-RS

- FFS: impact on CSI processing requirement

- FFS: details on configuration/indication of the power offset values

- FFS: whether/how to additionally consider the case where CSI-RS power is changed

Agreement@112bis-e

For power domain adaptation, support the following configuration(s) for CSI-RS resource configuration,

- A1-2-power: one or more resources can be configured in a resource set within a resource setting and each resource can be associated with one or more power offset values

- FFS: A1-1-power: a resource set with multiple resources is configured within a resource setting, where resources can have different power offset values

- FFS: Details of how the different power offset values(s) are configured/indicated.

Agreement@113

Joint operation of SD and PD adaptation is supported.

BM/TCI states related aspects

Agreement@113

- Downselect one of the following for BM enhancements in RAN1#114

    - Case 1: Support scaling the threshold of beam failure detection and threshold of candidate beam identification for power domain network energy saving

    - Case 2: Support UE to send hypothetical beam failure and/or radio link failure (RLF) reports for the indicated hypothetical power offset values.

    - Case 3: No further work on BM enhancements

- Downselect one of the following for TCI configuration enhancement in RAN1#114

    - Method 1: Configure multiple candidate CSI-RS resources as reference signal for QCL information or for spatial relation information, and switch one of them based on L1/L2 signaling

    - Method 2: Configure multiple candidate sets of TCI state(s) associated with DL/UL signal/channel and switch one of them based on L1/L2 signaling

    - Method 3: No further work on TCI configuration enhancement

Other logistics for SD/PD adaptation

Agreement@112

For spatial and power domain adaptation, solution(s) based on adaptation within an active BWP is considered as baseline

Cell DTX/DRX: affected channel/signals

Agreement@112

- RAN1 continues discussion on the at least following physical layer related aspects of cell DTX/DRX aspects

  - physical layer signals/channels and procedures expected to be impacted during non-active periods of cell DTX/DRX

    - consider impact to at least KPIs from the SI when physical layers/signals/channels are impacted by cell DTX/DRX

- Further discussions on other aspects are not precluded

Agreement@112

At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS.

- DL

  - Periodic/Semi-persistent CSI-RS (including TRS)

  - PRS

  - PDCCH scrambled with UE specific RNTI

  - PDCCH in Type-3 CSS

  - SPS-PDSCH

- UL

  - SR

  - Periodic/Semi-persistent CSI report

  - Periodic/Semi-persistent SRS

  - CG-PUSCH

Other signals/channels are not precluded

Agreement@112bis-e

From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions.

- Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting)

- FFS:

  - PDCCH in USS

    - UE behaviour for retransmission

    - if some specific RNTI scrambled PDCCH in USS will be excluded from cell DTX operation

  - PDCCH in Type-3 CSS

    - UE behaviour for retransmission

    - if some specific RNTI scrambled PDCCH in Type-3 CSS will be excluded from cell DTX operation

  - PRS

  - CSI-RS configured by measObjectNR (for RRM)

  - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD)

  - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking)

  - Periodic/Semi-persistent CSI-RS (for BM)

    - FFS on how to differentiate (if needed) with other CSI-RS used for CSI reports for BM

- FFS: Whether the same or different UE behaviour is applicable with or without C-DRX

- FFS: Whether the list of impacted signals/channels can be configurable

- FFS: Whether there will be exception case(s) for UE receiving and/or processing listed signals/channels during non-active periods of DTX

- FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period

Agreement@112bis-e

From RAN1 point of view, Rel-18 UE supporting cell DRX is not expected to transmit the following signals/channels to the gNB during non-active periods of cell DRX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions.

- Periodic/Semi-persistent CSI report

- Periodic/Semi-persistent SRS

  - FFS: SRS for positioning

- FFS:

  - HARQ feedback for SPS PDSCH

- FFS whether there will be exception case(s) for UE transmitting listed signals/channels during non-active periods of DRX

- FFS Whether the listed signals/channels can be configurable by gNB

- FFS: Whether the same or different UE behavior is applicable with or without C-DRX

FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period

Agreement@112bis-e

For PDDCH monitoring, further work on Rel-18 NES in RAN1 is to follow the RAN2 agreement below:

10.	The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

Cell DTX/DRX: L1 signaling for (de)activation

Agreement@112bis-e

Study L1 signalling for enhancing cell DTX/DRX including activation/deactivation for a single configuration which will have the following characteristics:

- PDCCH based signaling

  - FFS: Whether enhancing legacy DCI or introducing new DCI

  - FFS: DCI content

  - FFS: Whether L1 signaling is UE specific DCI or group common DCI

  - FFS: Timer or validity duration based activation/deactivation of cell DTX/DRX

  - FFS: whether to specify a reference time for activation/deactivation of cell DTX/DRX

  - FFS: If multiple Cell DTX/DRX patterns are to be supported

- FFS on detailed UE behavior upon reception of L1 signaling at least including application delay

- FFS how to guarantee reliability of the L1 signaling

- FFS whether the L1 signal can be monitored in non-active periods.

Working Assumption@112bis-e

Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disenabling the feature) from RAN1 perspective.

- This does not imply that L1 activation/deactivation is supported in Rel-18\

- Note: Reliability, overhead, and benefits are FFS

Agreement@113

RAN1 supports the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback

- Send an LS to RAN2 to consider the additional support of a MAC CE based indication

- Subject to UE capability

Agreement@113

Confirmation of WA from previous meeting with removal of the two sub-bullets.

Working Assumption

  - Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disenabling the feature) from RAN1 perspective.

    - This does not imply that L1 activation/deactivation is supported in Rel-18\

    - Note: Reliability, overhead, and benefits are FFS

Agreement@113

DCI format for group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation (downselect just one among alternatives)

- Alt 1) DCI Format 2_6 (power saving information outside DRX Active Time)

  - FFS: Monitoring within DRX active time

  - FFS: Field content

- Alt 2) Based on new DCI format 2_X

  - Field content format

    - Block number 1, block number 2, …, block number N

    - For each block should at least support the following:

      - DTX configuration activation/deactivation

      - DRX configuration activation/deactivation

    - FFS: other field details, mapping of UE and each blocks

  - DCI size indicated by higher layers

  - FFS: RNTI

- FFS: application delay, timers for activation/deactivation

- FFS: handling of multiple cells including when UE supports different number of cells

- FFS: details on PDCCH monitoring aspects, including but not limited to:

  - Search Space

  - PDCCH monitoring occasion

  - slots to monitor (during cell DTX/DRX non-active periods, and active periods)

  - BD/CE aspects

- FFS: UE behavior upon reception of the group common PDCCH (during cell DTX/DRX non-active periods, and active periods), including fallback behavior (if any)

Agreement@113

For the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation

- Alt 2) Based on new DCI format 2_X

  - DCI size budget is not increased

  - Number of required BDs is not increased

  - FFS: PDCCH monitoring configuration for the new DCI format is identical to PDCCH monitoring configuration for DCI format 2_6 if the UE monitors both DCI formats

    - FFS: New RNTI is used

Other logistics for cell DTX/DRX

Further study the following in RAN1:

- Handling of HARQ-ACK codebook generation when configured with cell DTX/DRX

- Handling of PUCCH deferral operation during non-active periods of cell DRX

- Handling of overlapping channels where a least a channel overlaps with non-active periods of cell DTX/DRX

- Handling of signals/channels that can be received/transmitted repeatedly during non-active periods of cell DTX/DRX

- Handling of PUCCH switching during non-active period to an active cell

- Other enhancements are not precluded.

Draft LS in R1-2306169 is endorsed in principle with the following revision. Further RAN1 agreements can be added in final LS if necessary.

“With respect to questions asked by RAN2, RAN1 has made the following agreements:”

Final LS to include the agreement on the introduction of new DCI format. Final LS is in R1-2306246.