3GPP TSG RAN WG1 Meeting #114																   R1-2308024

Toulouse, France, August 21st – August 25th, 2023

Agenda item:		9.4.1.2

Title:						NR Sidelink Unlicensed Physical Channel Design

Source:				Fraunhofer HHI, Fraunhofer IIS

Document for:		Discussion

# Introduction

The work item for NR sidelink evolution was approved in RAN#94e and revised in RAN#99e [1], and the following agreements [2] were made during the previous meeting in relation to the physical channel design framework for supporting sidelink on the unlicensed spectrum:

Agreement Regarding PSFCH transmission with 15 kHz and 30 kHz SCS: - One of the following alternatives is (pre-)configured: - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - K3 is (pre-)configured - Value range for K3 at least includes {1, 2, 5} - K3 dedicated PRB(s) are on the same interlace - There can be some guardband PRB(s) between common PRB and dedicated PRB - FFS details, e.g., whether/how to derive the number of guardband PRB(s), whether to additionally introduce a (pre-)configured gap (including 0), or whether this can be satisfied by (pre-)configuration and there is no additional specification impact (e.g., setting proper bit values in bitmap for PSFCH PRB allocation), etc. - FFS whether to additionally introduce guardband RE between common PRB and dedicated PRB - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB subject to meeting OCB requirements - FFS: whether to reduce power on common PRBs - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace - PSSCH transmissions on non-overlapped resources are mapped to orthogonal dedicated PRBs for PSFCH transmission - FFS: whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR - FFS: whether to drop common PRBs if the dedicated PRBs can already satisfy OCB requirement Agreement When the SL-BWP contains multiple RB sets, support the followings: - When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion) - UE may transmit S-SSB repetition in more than one RB set - Down-select one of the followings in RAN1#114: - Alt 1: At least the power for S-SSB transmission on anchor RB set does not change due to the number of used RB sets - FFS details, e.g., whether this can be satisfied by (pre-)configuration, whether the power for S-SSB transmission on other RB set(s) also does not change due to the number of used RB sets, etc. - Alt 2: The power for S-SSB transmission on each RB set does not change due to the number of used RB sets - FFS details, e.g., whether this can be satisfied by (pre-)configuration, etc. - FFS: Locations of S-SSB repetitions in each RB set are the same as the locations of S-SSB repetitions in the anchor RB set - FFS: how to (pre)configure resources for the S-SSB repetitions - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218, R1-2306198 Agreement Regarding “Option 3-1(revised): Transmit legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement”: - Support: - Alt 3: the value of gap is (pre-)configured, and the value of N is (pre-)configured - FFS: value range for gap and N - FFS: whether N for different RB sets can be different - FFS: whether to apply any restriction on sl-AbsoluteFrequencySSB-r16 for 60 kHz - FFS: whether/how to support reducing PAPR of S-SSB transmission, at least considering the following options - Option 1: use different NIDSL across the different S-SSB repetitions to determine the initial scrambling seed of PSBCH, and the sequence shift for S-SSS and S-PSS - Option 2: phase adjustment among repetitions - Option 3: no specification impact to reduce PAPR - Option 4: use S-SSB repetition index to scramble different S-SSB repetition(s) Agreement Regarding the number and location(s) of additional candidate S-SSB occasions, support: - Option 2 (12): Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length (including possibility of being 0), etc. Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U, support the followings: - Option A3: N_ref is (pre-)configured - The value range for N_ref at least includes {10, 11} - FFS: whether to additionally include other values Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, for TBS determination and 2nd SCI overhead, in TS 38.214 Clause 8.1.3.2: - L_ref replaces sl-LengthSymbols - Value range of L_ref is {7, 8, 9, 10, 11, 12, 13, 14} symbols - NsymbPSFCH is determined in the same way as in legacy NR SL Working assumption Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings: - Use FRIVsubCH to indicate used sub-channel index(s) - FRIVsubCH is conveyed in 1st stage SCI - FRIVsubCH is calculated as below - If sl-MaxNumPerReserve is 2 then - FRIVsubCH=nsubCH,1start+i=1LsubCHRBset-1N subchannel RBset+1-i - If sl-MaxNumPerReserve is 3 then - FRIVsubCH=nsubCH,1start+nsubCH,2start⋅N subchannel RBset+1-LsubCHRBset+i=1LsubCHRBset-1N subchannel RBset+1-i2 - where - nsubCH,1start denotes the starting sub-channel index for the second resource - nsubCH,2start denotes the starting sub-channel index for the third resource - N subchannel RBset is the number of sub-channels for each RB set - LsubCHRBset is the number of used sub-channels for each RB set for each of the indicated resources - FFS: whether/how to use spare states of FRIVsubCH to indicate using non-contiguous interlaces - FFS: whether additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB - FFS: whether additionally support using bitmap Working assumption Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings: - Use FRIVRBset to indicate used RB set index(s) - FRIVRBset is conveyed in 1st SCI - FRIVRBset is calculated as below - If sl-MaxNumPerReserve is 2 then - FRIVRBset=nRBset,1start+i=1LRBset-1NRBset+1-i - If sl-MaxNumPerReserve is 3 then - FRIVRBset=nRBset,1start+nRBset,2start⋅NRBset+1-LRBset+i=1LRBset-1NRBset+1-i2 - where - nRBset,1start denotes the starting RB set index for the second resource - nRBset,2start denotes the starting RB set index for the third resource - NRBset is the number of RB sets in a resource pool - LRBset is the number of used RB sets for each of the indicated resources Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, support only option 3. - FFS other details, e.g., impacts on resource selection, PSCCH mapping, etc. - Note: - Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission - Note: PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement - Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission - Nleft : the number of remaining PRBs of a sub-channel belonging to a RB set after excluding the PRBs belonging to intra-cell guardband - NPSCCH : the number of PRBs for PSCCH transmission Agreement When neither COT initiating UE nor responding UE intends to transmit PSFCH on some PSFCH occasion(s) within a COT, to avoid COT interruption, select one or more of the followings: - Option 1: COT initiating UE or responding UE transmits PSSCH on such PSFCH occasion(s) - FFS details, e.g., how PSSCH Rx UE knows such transmission, etc. - Option 2: COT initiating UE or responding UE transmits a PSFCH-like signal on such PSFCH occasion(s) - FFS details, e.g., signaling design, etc. - Option 3: no optimization for this case Agreement Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration: - Regarding locations of candidate PSFCH occasion(s): - Down-select at RAN1#114: - Alt 1 (15): Associated PSFCH occasion(s) are within the RB set(s) occupied by PSSCH transmissions - FFS details - Alt 2 (2): PSSCH transmission and its related PSFCH occasion(s) are in the same or different RB set(s) of the same resource pool - For one PSCCH/PSSCH transmission, at least support that its associated candidate PSFCH occasion(s) are in different slots of the same RB set(s) - FFS: whether to support its associated candidate PSFCH occasion(s) are in different RB sets of the same slot Agreement Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration: - Value range of N at least includes {1, 2, 3, 4} - When N>1, N associated candidate PSFCH occasion(s) for one PSCCH/PSSCH transmission have different time and/or frequency resource with the candidate PSFCH occasion(s) for another PSCCH/PSSCH transmission, at least if these two PSCCH/PSSCH transmissions are on non-overlapped resources Agreement Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration: - Regarding UE behaviour on transmitting PSFCH: - Down-select at RAN1#114: - Alt 1: For one PSCCH/PSSCH transmission, PSCCH/PSSCH receiver UE attempts to transmit PSFCH on a candidate PSFCH occasion if and only if it fails to transmit on previous PSFCH occasion(s) due to LBT failure - Alt 2: For one PSCCH/PSSCH transmission, PSCCH/PSSCH receiver UE attempts to transmit PSFCH on a candidate PSFCH occasion if and only if it fails to transmit on previous PSFCH occasion(s) (e.g., due to LBT failure, or due to UL/SL prioritization, etc.) - Alt 3: Do not specify additional UE behavior on transmitting PSFCH due to LBT failure. - FFS: UE behaviour on receiving PSFCH - Regarding HARQ RTT restriction: - Further study whether/how to update the followings: - The minimum time gap Z=a+b between any two selected resources of a TB in case PSFCH is configured for this resource pool - The reference slot n for PUCCH transmission to report HARQ in Mode 1

This document focuses on the changes required for the physical channel design to be able to support sidelink operation in unlicensed (SL-U) spectrum.

# SL-U Resource Pools

In Rel-16 and Rel-17, NR sidelink UEs were configured with one active SL BWP which is used for both transmissions and receptions. The SL BWP further contains resource pools, where each resource pool contains multiple sub-channels across frequency and the resource pool is defined across time using continuous or discontinuous time slots. The same SL BWP and resource pool configuration was agreed to be adopted for Rel-18 in [3]. The other issues that remain are the definition of the resource pools across time while minimizing the chances of a COT being taken over by another non-3GPP UE in the case of discontinuous resource pools, along with the challenges for incorporating interlaced RB-based transmissions in a legacy SL resource pool.

## Across Time

This section is related to the FFSs on how to define the slots in a resource pool across time. One of the aspects that was discussed was the need for a time-continuous resource pool. The rationale behind defining a resource pool across time with a bitmap value of all 1s is that in the event that a resource pool is defined in a discontinuous manner, i.e., with both 0s and 1s, it opens up the possibility of non-3GPP UEs being able to access the channel. This would result in a 3GPP UE not having access to the channel within a COT due to a failed LBT since other non-3GPP UEs would be occupying the channel. This can be avoided by configuring a time-continuous resource pool with a bitmap with all 1s, and providing the 3GPP UEs with a possibility of a truly time-continuous COT.

Proposal 1: We propose that Rel-18 SL-U resource pools are configured in a time-continuous manner, using a bitmap with all 1s.

## Across Frequency

NR-U was designed to co-exist with other non-3GPP RATs such as IEEE 802.11, especially in the sub-6 GHz frequency bands. Due to this reason, NR-U supports only bandwidths that are an integer multiple of 20 MHz in order to meet the OCB and PSD regulatory requirements, where each of these 20 MHz channels are designated as a sub-band. While NR-U supports both contiguous and interlaced resource allocation schemes, for interlaced resource allocation, the basic unit of resource allocation is called an interlace, consisting of a number of equally spaced resource blocks within the 20 MHz sub-band. This is depicted in Figure 1.

A UE using interlaces in NR-U is configured with a parameter M denoting the number of interlaces, and multiple interlaces of resource blocks are defined, where each interlace consists of common resource blocks that are determined by m,M+m, 2M+m, 3M+m, …, as defined in [4]. The UE is then expected to perform an intersection operation with the resource blocks of the configured BWP. The number of interlaces is restricted to a maximum of 10 interlaces.

In order to use the concept of interlaced resources in SL-U for the transmission of PSCCH and PSSCH, it has to incorporate the fact that multiple resource pools are defined within a SL BWP, and a single resource pool does not necessarily span the entire frequency of the BWP. Each resource pool is defined with a number of sub channels, which each sub channel size defined by the number of consecutive PRBs, which can be configured to the values of 10, 12, 15, 20, 25, 50, 75 or 100 PRBs. This would open up the possibility of a resource pool bandwidth not being a multiple of the 20 MHz sub band that is mandated by regulatory requirements. Hence, a resource pool has to be configured accordingly in order to avoid wastage of resources. Furthermore, the resource pool may be smaller or equal to the bandwidth defined for the SL BWP. In this case, instead of performing the intersection operation between the RB sets and the BWP as performed in NR-U, the intersection operation can also be performed between the RB sets and the configured resource pool within the BWP.

Figure 1: Depiction of interlaces in NR-U

Proposal 2: In order to support interlaced RB-based transmissions in SL-U, we propose to perform the intersection operation between the RB sets and a resource pool within the SL-BWP.

### Definition of Sub Channel

With the agreement from the previous meetings that the resource pools will be configured to include an integer number of RB sets, there is a requirement for the sub channel sizes and resource pool bandwidths to be aligned or compatible with the number of RB sets configured for the resource pool. It is advantageous to retain the concept of sub channels and maintain a similar structure as that of resource pools defined in Rel-16/17 for ease of specification efforts, else, a new resource pool structure would need to be defined. Keeping this in mind, and in order to support interlaced RB-based transmissions, it is possible to define a sub channel with an integer multiple of number of RBs that can also match the number of RBs in an LBT sub band, on a per 20 MHz channel basis.

In this case, the size of the RB sets would be configured based on the SCS for the SL-BWP within which the resource pool is configured. This would enable a unified design for resource pools that support RB-based and interlaced RB-based transmissions. It has to be further studied how the OCB requirements can be incorporated into the sub channel design.

Proposal 3: We support the configuration of sub channels within a resource pool in Rel-18 SL-U. The number of RBs within a sub channel should be compatible with the LBT sub bands, according to the SCS.

### Support for Multiple RB Sets

In the previous meeting [2], for frequency domain resource indication for interlace RB-based PSSCH transmission, it was agreed to support that for one PSSCH transmission, the used interlace index in different used RB sets are always the same. Another aspect to be explored is the use of multiple interlace configurations for a given resource pool. This would enable the use of different configurations for reservation of resources used in future transmissions and retransmissions. Based on the way interlaces are configured in NR-U, higher layer configurations are used to determine the exact resources used for a transmission. With the use of multiple interlace configurations supported by a resource pool, it is also possible to use different interlaces for an initial transmission and a retransmission by indicating the interlace configuration in an SCI. Currently these resources are indicated using the TRIV/FRIV. This would introduce greater frequency diversity and avoid any untoward resource collisions.

Proposal 4: Study the possibility of introducing multiple interlace configurations per resource pool and its effects on the resource indication using TRIV/FRIV in SCIs.

# Slot Structures for SL-U

In Rel-16 and Rel-17, each resource pool contains multiple sub-channels across frequency and the resource pool is defined across time using continuous or discontinuous time slots. Each time slot within these resource pools contains a set of 7 or 14 symbols, with physical channels being carried within the slot. An exemplary depiction of these time slots with the physical channels can be seen in Figure 2.

A key aspect that has to be borne in mind is that the smallest unit of time used for scheduling SL transmissions is a time slot across time and a sub channel across frequency. It was agreed previously that SL-U would also adopt the same slot-based transmission procedures. Another agreement from a previous meeting supports the use of a maximum of two starting symbols within a time slot. This would make it challenging to incorporate LBT procedures as defined in NR-U, primarily because they take place at a per-symbol level.

Figure 2: Examples of slot formats with and without PSFCH

Observation 1: The smallest unit of time used for scheduling SL transmissions in Rel-16/17 is a time slot across time and a sub channel across frequency, while LBT procedures in NR-U take place at a per-symbol level.

NR-U employs a flexible slot structure, where each symbol is used for transmissions in the downlink or uplink direction, including flexible symbols that can be used for either. They are defined in [5], in Section 11.1.1, where the different supported slot formats are specified. This would allow UEs to carry LBT procedures over a few symbols, ascertain whether a given channel is available or not, and immediately begin transmissions using the flexible slot formats. After contending for a (pre-)defined number of symbols on the channel, the UEs can use it for a time period referred to as the COT. Mini-slots have also been defined in NR-U specifically to cater to the LBT taking place at the beginning of the time slot and facilitate immediate transmissions once the UE determines the channel to be available.

Observation 2: NR-U employs flexible slot structures, including mini-slots, in order to incorporate LBT procedures taking place at the beginning of a time slot.

In NR-U, channel access procedures can be carried out by the gNB for downlink transmissions, and by the UE for uplink transmissions. The different channel access procedures used by both entities are defined in [4], where the time duration of the sensing slots is randomly determined in the Type 1 access procedure, and deterministically determined in the Type 2 access procedure. In both the procedures, the basic unit for sensing is a sensing slot with a duration Tsl = 9us, and a channel is determined to be available if the received power detected for at least 4us of the sensing slot is less than an energy detection threshold. Else, the channel is adjudged to be busy or occupied.

One of the concerns of adapting the same channel access procedures to SL-U is that the sensing slot and the minimum sensing or LBT durations in NR-U across Type 1 and Type 2 are less than the length of a symbol within a time slot. For example, a 30 kHz SCS resource pool, each symbol within a time slot is 35.7us. This would require the UE to carry out LBT, check if the channel is available, and if yes, transmit within the same time slot. In the case where the existing NR sidelink time slot structures were to be used, the UE can determine that a time slot is available using LBT, but would not be able to transmit in the same time slot due to its slot structure. This is depicted in Figure 3.

Figure 3: Depiction of existing NR-U LBT procedures and Rel-16/17 time slots and sensing/resource selection procedures

The main challenge here is that Rel-16/17 sidelink UEs that carry out sensing rely on fixed slot structures and well-defined slot boundaries in order to identify the location of the PSCCH so as to decode the control information. While incorporating flexible slot structures using multiple starting symbols might be considered in Mode 1, Mode 2 would require a more cautious approach since UEs carry out sensing in order to determine the availability of a channel based on decoding the PSCCH, as per the Rel-16/17 sensing and resource allocation procedure in [6]. Furthermore, the UE would have to carry out LBT before using the scheduled resources for a transmission.

Since LBT would indicate the availability of only the current time slot, there is a need to define slot structures that are less than 14 symbols in length depending on the position of the starting symbols within the time slot at which the UE can commence its transmission after performing LBT. This would enable the UE to carry out LBT and then carry out a transmission within the same time slot. While the existing definitions of mini-slots and flexible slot structures can be carried forward for SL-U, new slot structures would need to be defined that would incorporate the SL physical channels as well.

In our understanding, the first of the two candidate starting symbols would essentially be symbol #0, while the second starting symbol can be a (pre-)configured on a per resource pool or per transmission basis. If multiple second starting symbols are allowed, it would also require an equivalent slot structure for each of these starting positions. The slot structures defined and discussed in Rel-16 [7] can be used as a starting point for the support of different slot structures.

However, it has to be borne in mind that aspects regarding the number of PSCCH occasions while using a shorter slot structure in conjunction with a full slot structure when MCSt is enabled, and their respective content have to be discussed. Furthermore, further discussion is required on the configuration of DMRS symbols, e.g., alignment of DMRS symbols in frequency domain, and the presence or absence of PSFCH symbols in these new sub slot structures.

With varying lengths of the LBT procedure, modifications would need to be carried out in the sensing and resource allocation procedures for UEs to be aware of the new slot structures and be able to determine the new positions of the PSCCH after a UE has carried out LBT. Discussions on multi-slot transmissions have been elaborated in our accompanying contribution in [8]. For slots with 2 candidate starting symbols, the location of 1st and 2nd starting symbols was agreed upon in the last meeting [2]. Some details of the positions of reference symbols and position and content of the PSCCH are yet to be discussed.

Proposal 5: For slots with 2 candidate starting symbols, further discussion is required in terms of the content of the sub slot structure, including the position of DMRS symbols, presence, or absence of PSFCH and the position and content of PSCCH.

Regarding the Tx UE behavior, in case the transmission starts from the 1st starting symbol, it was discussed in in one of the previous meetings whether the PSCCH/PSSCH transmission has 1 or 2 symbols for AGC purposes. Furthermore, the number of symbols used per AGC could simply depend on a condition, e.g., on the exact sub-slot structure used or whether feedback is enabled having a sub-slot containing the PSFCH. Thus, we believe that in the above cases, PSCCH/PSSCH transmission should not be limited to only have 1 symbol per AGC purposes. For the Rx UE behavior, this would require the UE to monitor two AGC symbols, unless it would be aware of the frame structure so that the Rx UE could drop monitoring a 2nd AGC symbol.

Proposal 6: Regarding the Tx UE behavior, if PSCCH/PSSCH starts from a 1st starting symbol, we support 1 or 2 symbols for AGC purposes, depending on a condition.

# PSFCH Transmissions and SL-HARQ in SL-U

In Rel-16/17, NR V2X introduced the support of HARQ based transmissions, where the receiving UE transmits a 1-bit HARQ ACK or NACK on the PSFCH, confirming the receipt or non-receipt of a given transmission based on whether it was able to decode the transport block successfully or unsuccessfully, respectively. The receiving UE can transmit an ACK/NACK for unicast and groupcast transmissions, as well as NACK-only for particular groupcast transmissions. The UE implicitly determines the earliest PSFCH resource to be used to transmit the HARQ feedback depending on the following two conditions:

- The transmitting UE provides a “PSFCH overhead indication” parameter in the SCI [9], which is a parameter derived from the higher layer parameter sl-PSFCH-Period [10], which is a resource pool configuration parameter. This parameter informs the receiving UE on the periodicity of PSFCH resources defined within the resource pool.

- Sl-MinTimeGapPSFCH is another higher layer parameter [9] which is a resource pool configuration parameter that defines the minimum time gap between the data transmission in the PSSCH and the feedback transmission in the PSFCH.

While the support for PSFCH format 0 was agreed previously, there are still a few challenges in implementing SL HARQ feedback on SL-U. Based on the sl-PSFCH-Period and Sl-MinTimeGapPSFCH parameters, the UE could implicitly determine the time slot on which the HARQ feedback has to be transmitted for a given data transmission. In order to reuse the same concept in SL-U, a few factors have to be considered:

- If the implicitly determined time slot with PSFCH is within the COT initiated by the transmitting UE, it is possible for the receiving UE to use COT sharing and transmit on the designated PSFCH. However, within same COT, it is possible for LBT failure to occur, if LBT is performed, for the intended time slot where the HARQ feedback was to be transmitted.

- In the case where the transmission is a blind transmission, taking place in a resource pool with PSFCH enabled, or when there is a gap in transmissions when neither the transmitting UE nor the receiving UE transmits on the PSFCH, this would result in the current COT being lost to another UE, or non-3GPP device, that occupies the channel.

The above cases would result in the receiving UE not being able to use the implicitly designated time slot with PSFCH configured for the transmission of its HARQ feedback. Hence, the receiving UE would require more than a single PSFCH occasion in order to be able to transmit the HARQ feedback successfully (Alt 1). This can be done by providing more PSFCH occasions over time and over frequency.

Over time, it is possible to configure the PSFCH in every time slot (sl-PSFCH-Period =1), but that would also come at the cost of reduced PSSCH resources being available for data transmissions in a resource pool. Hence it is possible to (pre-)configure a time duration within which the transmitting UE can expect to receive feedback for a given data transmission. This would provide the receiving UE with multiple PSFCH occasions to perform LBT and use the one occasion where it was deemed available.

Based on the discussions in the previous meetings [2] and [3], these multiple PSFCH occasions can be (pre-)configured or dynamically indicated (Option 2) to the transmitting UE. It is also possible for the time duration within which the receiving UE has to transmit the feedback for a transmission to be (pre-)configured, while the exact PSFCH resource to be used can be dynamically indicated. These additional PSFCH occasions need to be used only if the initial intended PSFCH occasion could not be used due to LBT failure. However, this would require the transmitting UE to monitor all these PSFCH occasions to be able to receive the HARQ feedback, if not explicitly indicated otherwise.

Across frequency, the receiving UE can also use different sub channels to transmit the HARQ feedback, even across different COTs. This would require the receiving UE to initiate a COT and share this with the transmitting UE, but it would be able to respect the conditions of implicitly determining the PSFCH time slot, similar to how it was used in Rel-16/17.

In both the scenarios described above, new signaling would be required to inform the receiving UE of the possible PSFCH occasions that it can use to transmit HARQ feedback, as well as for the transmitting UE to be aware of the PSFCH occasions that it needs to monitor.

Proposal 7: We propose to introduce multiple PSFCH occasions in SL-U by using

- a (pre-)configured time duration within which the receiving UE can transmit the HARQ feedback,

- dynamically indicated time slots with PSFCH configured across time, or

- different sub channels across frequency.

With the introduction of multiple PSFCH occasions, another issue is the reporting of the HARQ feedback by the transmitting UE back to the gNB when operating in Mode 1. While currently the UE uses the first possible PUCCH transmission occasion to report the HARQ feedback after the resources indicated in a grant, it might be required to revisit this because it is possible that the receiving UE was not able to transmit the HARQ feedback in the intended PSFCH time slot. This would require a redundancy to be introduced for the transmitting UE to be able to report the HARQ feedback to the gNB.

Proposal 8: We propose to study and consider the possibility of adapting the PUCCH timeline for the transmitting UE to report HARQ feedback to the gNB in SL-U, if multiple PSFCH occasions can be used by the receiving UE for transmitting the HARQ feedback.

Another aspect to be further discussed is the actions that a receiving UE can carry out in the case it faces multiple LBT failures in its attempt to transmit HARQ feedback. While it is possible for the receiving UE to be (pre-)configured with multiple PSFCH occasions, it needs to ensure that it is able to transmit the HARQ feedback depending on the priority of the transmission and the remaining PDB. It is also possible to (pre-)configure a maximum LBT failure threshold, beyond which the receiving UE has to take some action in order to ensure it can transmit the PSFCH successfully. Different possibilities and solutions have to be studied.

Proposal 9: In case of more than 1 PSFCH occasion per PSCCH/PSSCH transmission, we support that such PSFCH occasions are (pre-)configured and dynamically indicated (Option 2).

# Conclusions

The following observations and proposals have been made in this document:

Proposal 1: We propose that Rel-18 SL-U resource pools are configured in a time-continuous manner, using a bitmap with all 1s.

Proposal 2: In order to support interlaced RB-based transmissions in SL-U, we propose to perform the intersection operation between the RB sets and a resource pool within the SL-BWP.

Proposal 3: We support the configuration of sub channels within a resource pool in Rel-18 SL-U. The number of RBs within a sub channel should be compatible with the LBT sub bands, according to the SCS.

Proposal 4: Study the possibility of introducing multiple interlace configurations per resource pool and its effects on the resource indication using TRIV/FRIV in SCIs.

Observation 1: The smallest unit of time used for scheduling SL transmissions in Rel-16/17 is a time slot across time and a sub channel across frequency, while LBT procedures in NR-U take place at a per-symbol level.

Observation 2: NR-U employs flexible slot structures, including mini-slots, in order to incorporate LBT procedures taking place at the beginning of a time slot.

Proposal 5: For slots with 2 candidate starting symbols, further discussion is required in terms of the content of the sub slot structure, including the position of DMRS symbols, presence, or absence of PSFCH and the position and content of PSCCH.

Proposal 6: Regarding the Tx UE behavior, if PSCCH/PSSCH starts from a 1st starting symbol, we support 1 or 2 symbols for AGC purposes, depending on a condition.

Proposal 7: We propose to introduce multiple PSFCH occasions in SL-U by using

- a (pre-)configured time duration within which the receiving UE can transmit the HARQ feedback,

- dynamically indicated time slots with PSFCH configured across time, or

- different sub channels across frequency.

Proposal 8: We propose to study and consider the possibility of adapting the PUCCH timeline for the transmitting UE to report HARQ feedback to the gNB in SL-U, if multiple PSFCH occasions can be used by the receiving UE for transmitting the HARQ feedback.

Proposal 9: In case of more than 1 PSFCH occasion per PSCCH/PSSCH transmission, we support that such PSFCH occasions are (pre-)configured and dynamically indicated (Option 2).

# References

- RP-230077, “WID revision: NR sidelink evolution”, OPPO, 3GPP RAN#99, March 2023.

- R1-2305991, “FL summary#6 for AI 9.4.1.2 SL-U physical channel design framework“, Moderator (Huawei), 3GPP RAN1#113, Incheon, Korea, May 22nd – 26th, 2023.

- TS 37.213 v16.7.0, “Physical layer procedures for shared spectrum channel access (Release 16)”, December 2021.

- TS 38.213 v17.2.0, “Physical layer procedures for control (Release 17)”, June 2022.

- TS 38.214 v17.2.0, “Physical layer procedures for data (Release 17)”, June 2022.

- R1-1913576, “Summary#2 of discussion on PSSCH DMRS patterns and for the number of PSSCH symbols”, Samsung, RAN1#99, November 2019.

- R1-2308023, “NR Sidelink Unlicensed Channel Access Mechanisms”, Fraunhofer HHI, Fraunhofer IIS, 3GPP RAN1#114, Toulouse, France, August 2023.

- TS 38.212 v17.2.0, “Multiplexing and channel coding (Release 17)”, June 2022.

- TS 38.331 v17.1.0, “Radio Resource Control (RRC) protocol specification (Release 17)”, June 2022.