3GPP TSG RAN WG1 Meeting #115		R1-2312522

Chicago, USA, 13th – 17th November 2023

Title:	Reply LS on monitoring of paging occasions for CG-SDT with HD-FDD RedCap UEs

Response to:	R1-2304331 (R2-2304562), R1-2308812 (R4-2314464)

Release:	Rel-17

Work Item:	NR_redcap-Core, NR_SmallData_INACTIVE-Core

Source:	RAN1

To:	RAN2, RAN4

Cc:

Contact person:	Johan Bergman, johan (dot) bergman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1#115 discussed the LSs received from RAN2 and RAN4 on monitoring of paging occasions for CG-SDT with HD-FDD RedCap UEs. The discussion is captured under Issue #1 in R1-2312278.

Following the discussion, RAN1 made the following agreements (where HD-UE means HD-FDD RedCap UE):

- RAN1 confirms RAN2’s understanding that there is no need to specify restrictions on gNB configuration to avoid partial overlap between PO and CG-SDT transmission for HD-UE.

- RAN1 confirms that there is no need to specify RAN4 requirements for the case with full overlap between PO and CG-SDT transmission within the SI modification period for HD-UE.

- This means that it is up to gNB responsibility to leave at least one PO for SI update monitoring in modification window.

- Adopt the following TP for 38.213 clause 17.2:

A HD-UE does not expect to receive both dedicated higher layer parameters configuring transmission in a set of symbols and dedicated higher layer parameters configuring reception in the set of symbols. A HD-UE does not expect to receive both a Type-0/0A/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols, except Type-2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and configured-grant based PUSCH transmission as described in clause 19.1 in the set of symbols for which case the UE follows the procedure as in clause 5.1B.2.6 in [10, TS 38.133]. The UE expects to be configured with a Type-2-PDCCH CSS set configuration for PDCCH reception such that there is at least one paging occasion that does not overlap with configured-grant based PUSCH transmission as described in clause 19.1 per SI modification period.

# 2	Actions

To RAN2 and RAN4:

ACTION: 	RAN1 respectfully requests RAN2 and RAN4 to take the above into account in their future work.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #116	26th February – 1st March 2024	Athens, GR

TSG-RAN WG1 Meeting #116bis	15th – 19th April 2023			China (TBC), CN