# 3GPP TSG-RAN WG1 Meeting #115												         R1-2311075

# Chicago, USA, November 13th – November 17th, 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PUSCH selection rule for UCI multiplexing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In NR Rel-15, for UCI multiplexing on PUSCH, the following conclusion was achevied to clarify the understanding of the specification on determining a PUSCH for UCI multiplexing when there are multiple PUSCHs overlapping with a PUCCH, espescally for the case with different numerologies among PUCCH and PUSCHs in RAN1#97 meeting. Conclusion: For the issue raised in the draft CR R1-1906302, the intended UE behavior per specification is commonly understood as follows: - For UCI multiplexing, within a PUCCH group, on PUSCH, the following two steps are performed with step 1 first, then followed by step 2: - Step 1: UCI in overlapped PUCCH transmissions is multiplexed into one PUCCH resource (resource Z). This step is done per PUCCH slot. - Step 2: UCI, that doesn’t include SR, in Z is multiplexed into one PUSCH, if Z overlaps with at least one PUSCH, following the priorities (sequentially from high to low) as listed below. - First priority: PUSCH with A-CSI as long as it overlaps with Z - Second priority: earliest PUSCH slot(s) based on the start of the slot(s) - If there are still multiple PUSCHs overlap with Z in the earliest PUSCH slot(s), follow the following priorities (sequentially from high to low) - Third priority: Dynamic grant PUSCHs > PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH - Fourth priority: PUSCHs on serving cell with smaller serving cell index > PUSCHs on serving cell with larger serving cell index - Fifth priority: Earlier PUSCH transmission > later PUSCH transmission Note: The clarification applies to both cases with the same (except the second priority part) and different numerologies among PUCCH and PUSCHs. In the previous meetings, an issue of HARQ-ACK multiplexing on PUSCH in the absence of PUCCH was discussed, the following agreements were made in RAN1#107-e and RAN1#109-e meetings to address the issue. Agreement For Rel-16 with one PUSCH and no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot (both single carrier and UL CA), if the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), the UE multiplexes HARQ-ACK following the UL-TDAI into the PUSCH. Agreement For Rel-16 UEs, in the scenario with more than one PUSCH (overlapping and non-overlapping) and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), for a unified design, the following should be specified: - Selection of the candidate PUSCH for multiplexing: PUSCHs without UL-TDAI=4 in case Type 2 CB, and without UL-TDAI n.e. 1 in case of Type 1 CB within the PUCCH slot are candidates - Prioritization rules to select PUSCH for multiplexing. Prioritization rules are identical to 38.213 - Limitations for multiplexing - UE expects to multiplex HARQ-ACK on only 1 PUSCH selected based on step 2 in the PUCCH slot. - All the PUSCHs in the determined candidate set after step 1 have to satisfy Rel-15 UCI multiplexing timeline, defined with respect the starting symbol of the earliest PUSCH transmission in the candidate set. The above specified behavior is supported subject to a new Rel-16 UE capability [Multiplexing-HARQ-ACK-without-PUCCH-on-PUSCH] - FFS: the details of the capability signaling To capture the above agreements in spcification, a CR in R1-2205629 was indorsed in RAN1#109-e meeing. The issue and agreements are for the case PUCCH is absent, but the CR also involves the case there is a PUCCH overlapping with PUSCHs and defines candidate PUSCHs for UCI multiplexing for this case. According to the currect specification, when there are multiple PUSCHs overlapping with one PUCCH, the following priority order is used to determine a PUSCH for UCI multiplexing, which is different with the conclusion in Rel-15 for the case with different numerologies among PUCCH and PUSCHs. - First priority: PUSCH with A-CSI - Second priority: Dynamic grant PUSCHs > PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH - Third priority: PUSCHs on serving cell with smaller serving cell index > PUSCHs on serving cell with larger serving cell index - Fourth priority: Earlier PUSCH transmission > later PUSCH transmission |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Second priority: earliest PUSCH slot(s) based on the start of the slot(s) in the specification based on UE capability and RRC signalling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is existing implementation according to the behavior associated to this new capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9	UE procedure for reporting control information

< Unchanged parts are omitted >

When a UE transmits multiple PUSCHs on respective serving cells in a slot with reference to slots for PUCCH transmissions and the multiple PUSCHs overlap with a PUCCH carrying UCI in the slot, the UE selects all the PUSCHs overlapping with the PUCCH as the candidate PUSCHs for UCI multiplexing within the slot.

If a UE would transmit a single PUSCH scheduled by a DCI format that includes a DAI field on a serving cell in a slot with reference to slots for PUCCH transmissions without any other PUSCH that would be transmitted on any serving cell in the slot and the UE does not determine any PUCCH carrying HARQ-ACK information in the slot, or if the UE indicates the corresponding capability mux-HARQ-ACK-withoutPUCCH-onPUSCH and the UE transmits multiple PUSCHs on respective serving cells in a slot with reference to slots for PUCCH transmissions and the UE does not determine any PUCCH carrying HARQ-ACK information in the slot and at least one of the multiple PUSCHs is scheduled by a DCI format that includes a DAI field, the UE selects the single PUSCH or all the multiple PUSCHs in the slot as the candidate PUSCHs for HARQ-ACK multiplexing within the slot except for any PUSCH among the multiple PUSCHs that is scheduled by a DCI format that includes a DAI field that is equal to 4 in case the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic or with pdsch-HARQ-ACK-Codebook-r16, or is equal to 0 in case the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static.

The UE determines the PUSCH for UCI multiplexing by applying the following procedure on the candidate PUSCHs as described in this clause where the timing conditions for overlapping PUCCHs and PUSCHs in clause 9.2.5 are fulfilled:

-	If the candidate PUSCHs include a PUSCH with aperiodic CSI, and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH.

-	Else

-	If the candidate PUSCHs are in more than one slots starting at different time, and the UE indicates a corresponding capability [mux-Basedon-SlotTiming] and is provided with [multiplexing-Basedon-SlotTiming], the UE multiplexes the UCI in a PUSCH from the PUSCHs in a slot starting earliest and does not transmit the PUCCH.

-	If the candidate PUSCHs that include first PUSCHs that are scheduled by DCI formats and second PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH, the UE multiplexes the UCI in a PUSCH from the first PUSCHs and does not transmit the PUCCH.

-	If the candidate PUSCHs include PUSCHs on more than one serving cells , the UE multiplexes the UCI in a PUSCH on the serving cell with the smallest ServCellIndex and does not transmit the PUCCH. If the UE transmits more than one PUSCHs in the slot on the serving cell with the smallest ServCellIndex, the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot and does not transmit the PUCCH.

If a UE transmits a PUSCH over multiple slots or multiple PUSCHs over multiple slots that are scheduled by a DCI format 0_1, and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots of the multiple slots, and the PUSCH transmission in the one or more slots fulfills the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots. The UE does not multiplex HARQ-ACK and/or CSI information in the PUSCH transmission in a slot from the multiple slots if the UE would not transmit a single-slot PUCCH with HARQ-ACK and/or CSI information in the slot in case the PUSCH transmission was absent.

< Unchanged parts are omitted >