3GPP TSG RAN WG1 #116        		                           R1-2400751

Athens, Greece, February 26th – March 1st, 2024

Agenda item:		9.11.3

Source:			Samsung

Title:			Discussion on uplink capacity/throughput enhancement for NR-NTN

Document for:		Discussion and decision

- Introduction

At the previous RAN#102, the following Work Item Description (WID) was approved [1]. The motivation behind this is that it may need to optimize uplink performance considering that there will be large number of UE within the same spot beam in the limited frequency resource.

2. Uplink Capacity/Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Study then specify, if beneficial, DFT-s-OFDM PUSCH enhancements via Orthogonal Cover Codes (OCC) - Determine the achievable capacity improvement to be targeted taking into account realistic impairments (e.g. Doppler, time variation, phase distortion, etc) - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed - Note: The study can consider orthogonal cover codes across OFDM symbols, across slots, and/or within an OFDM symbol. - Note: the study phase is targeted to be completed by RAN#104 - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

This contribution focuses on discussing on several aspects for NR NTN uplink capacity and throughput enhancement for FR1-NTN.

- Discussion

In Rel-18 for NR NTN uplink coverage enhancement we evaluated several channels/signals to identify those channels/signals that needed further enhancements to provide the necessary coverage for NTN.  Accordingly, efforts were focused on link-level evaluation of NTN. For Rel-19 NR NTN uplink capacity enhancements we can reuse Rel-18 simulation assumptions as shown in the Appendix for link-level evaluation at least as a starting point.  For modeling frequency error, timing error, phase distortion, etc. due to Doppler and time variation, further evaluation assumptions are needed.

Proposal 1: Consider PUSCH EVM considered in Rel-18 NTN as a baseline.

The uplink enhancement that NR NTN WID asks for is applicable to all NTN scenarios including GSO and NGSO cases.  In fact this study may provide uplink enhancements that could be applicable to UEs that are operating in terrestrial networks too.  On the other hand, in order to provide “realistic” achievable capacity improvement this study must take into account realistic impairments such as high Doppler scenarios, severe timing variations, and phase distortion that happen mostly for NGSO scenarios such as LEO. For Rel-19 downlink NR NTN coverage enhancement study, it is noted that the WID already considers NGSO scenarios and specifically LEO Set-1 @ 600 km to be in priority.  For uplink study, considering the realistic impairment assumptions and the limited time for the study, it is preferable to consider NGSO scenarios and specifically LEO Set-1 @ 600 km to be in priority.

Proposal 2: Consider LEO Set-1 @600km in priority.

In [1], there are 3 scenarios to be evaluated for uplink capacity/throughput enhancement via OCC: orthogonal cover codes across OFDM symbols, across slots, and/or within an OFDM symbol. When evaluating capacity/throughput improvement via OCC, it is necessary to decide what baseline scenario should be considered. For instance, for evaluating OCC across OFDM symbols, within a symbol and across slots, Figures 1-3 can be regarded as high-level scenarios.

Figure 1. An example of evaluation scenario for OCC across symbols (left: TDM, right: OCC across symbols)

Figure 2. An example of evaluation scenario for OCC within a symbol (left: FDM, right: OCC within a symbol)

Figure 3. An example of evaluation scenario for OCC across slots (left: TDM, right: OCC across slots)

Proposal 3: Discuss baseline scenario(s) to evaluate uplink capacity/throughput performance.

Conclusion

This contribution discussed on evaluation assumptions for uplink capacity/throughput evaluation. Followings are proposals in this contribution.

Proposal 1: Consider PUSCH EVM considered in Rel-18 NTN as a baseline.

Proposal 2: Consider LEO Set-1 @600km in priority.

Proposal 3: Discuss baseline scenario(s) to evaluate uplink capacity/throughput performance.

References

[1] RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3

[2] TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

Appendix

Agreement

For coverage evaluation of PUSCH in NR NTN, the following table is assumed.

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| PUSCH duration | 14 OS |
| Repetitions | w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| PRBs/TBS/MCS for low data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK, pi/2 BPSK (optional) |
| Other parameters | Reported by companies |