3GPP TSG RAN WG1 Meeting #117	R1-2403860

Fukuoka, Japan, May 20 – 24, 2024

Agenda Item:	9.4.2.1

Source:	Futurewei

Title:	Discussion on physical layer design for Rel-19 Ambient IoT devices

Document for:	Discussion and Decision

# Introduction

During the RAN1#116-bis meeting, RAN1 reached several agreements on multiple access techniques, D2R bandwidth definitions, line encoding, OFDM-based waveforms, CRC and baseband modulation [1]. In this contribution, we discuss and present our views on different multiple access techniques, line encoding, OFDM-based OOK waveforms, and bandwidth.

# Discussion

## Device to Reader (D2R)

### Bandwidth Definitions

At the RAN1#116-bis meeting, RAN1 agreed on the following bandwidth definitions:

Agreement The following bandwidths for D2R are defined for the purpose of the study: - Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device. - FFS in agenda 9.4.2.3: how frequency resources scheduled by a reader are determined - Occupied bandwidth, Bocc,D2R: The transmission bandwidth plus the potential associated intra A-IoT guard-bands totalling Bguard,D2R - Note: this guard band is not for coexistence with NR/LTE - If/how to define guard band for coexistence between A-IoT D2R and NR/LTE is up to RAN4. - Bocc,D2R >= Btx,D2R - Possible values of each bandwidth are FFS

Additionally, RAN1 also agreed to study three multiple access techniques.

Agreement Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Agreement Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

In order to study the agreed multiple access techniques above, RAN1 needs to define an additional bandwidth term, namely the D2R system bandwidth, otherwise the list of bandwidth definitions is not complete. Hence, we provide the definition for the system bandwidth:

- D2R system bandwidth, Bsys,D2R: The total contiguous frequency resources (or spectrum) allocated by the reader for Ambient IoT D2R transmission.

The relationship between these terms can be expressed as an inequality: Bsys,D2R≥Bocc,D2R>Btx,D2R.

Proposal 1: For D2R multiple access studies, RAN1 defines the D2R system bandwidth as follows:

- D2R system bandwidth, Bsys,D2R: The total contiguous frequency resources (or spectrum) allocated for Ambient IoT D2R transmission, where

  - Bsys,D2R≥Bocc,D2R>Btx,D2R

It is worth noting that the values of Bsys,D2R and Btx,D2R may not be constant; this means, Bsys, D2R may be dynamically configured to match the population size of Ambient IoT devices within the coverage area of the reader, and Btx,D2R can be configured to meet variable data rates needed by different Ambient IoT devices. In the case of NR in-band operation, the frequency location of the D2R system bandwidth within the NR channel bandwidth is not fixed; that is, the system bandwidth can be located anywhere within the NR channel, allowing efficient spectrum sharing with NR as shown in Figure 1. Depending on the NR operating band and deployment, the system bandwidth can occupy the entire NR channel bandwidth.

For co-existence with NR technology, the total Ambient IoT spectrum Bsys,D2R can be expressed as an integer multiple of NR PRBs. Similarly, the D2R channel bandwidth Bocc,D2R can be expressed as an integer multiple of PRBs even though it is not necessary. The minimum D2R channel bandwidth Bocc,D2R is one PRB.

Figure 1: D2R System bandwidth in NR in-band operation

Observation 1: For D2R transmission, the D2R system bandwidth Bsys,D2R is a configurable parameter, which can be expressed as an integer multiple of NR PRBs.

Proposal 2: For D2R transmission, the minimum D2R occupied bandwidth Bocc,D2R can be assumed to be one PRB as a starting point.

### Multiple Access

As mentioned in the previous subsection, RAN agreed to study time-domain access, frequency-domain multiple access, and code-domain multiple access techniques.

Agreement Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Agreement Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

Code-domain multiple access achieves device separation in the code domain, which demands precise timing alignment and synchronization, and power control between Ambient IoT devices. Due to low complexity and large SFO values of the Ambient IoT devices, such accurate timing and power control are far-fetched. For example, the synchronization requirement of IS-95 CDMA system is 10-6, which cannot be met by Ambient IoT devices.

Observation 2: For D2R transmission, code-domain multiple access is not feasible.

Time-domain multiple access can be considered as the basic multiple access technique. However, it might result in spectrum underutilization, in particular when Bocc,D2R≪ Bsys,D2R which can result in a spectrum utilization of Bocc,D2R/Bsys,D2R≪ 1 because of having only one device transmitting at a time and occupying the whole system bandwidth. Further, it might not be able to fulfil the high device/connection density requirement of certain Ambient IoT use cases.

Observation 3: For D2R transmission, time-domain multiple access might result in low spectrum utilization if  Bocc,D2R≪Bsys,D2R.

For the frequency-domain multiple access, the D2R system bandwidth is divided into a number of relatively narrow orthogonal RF channels. Let us assume D2R system bandwidth Bsys,D2R and the D2R occupied bandwidth Bocc,D2R are known as illustrated in Figure 2. Then the total number of D2R RF channels ND2R is given by (considering support of single-side band, SSB-based backscattering)

ND2R=Bsys,D2RBocc,D2R

Basically, one single reader can simultaneously support ND2R devices in the D2R transmission.

.

.

.

CD2R,0

Bsys,  D2R

CD2R,1

CD2R,2

CND2R-1

Bocc,D2R

Btx, D2R

Bg,D2R

Bg,D2R

CD2R,2

fD2R,ND2R-1

fD2R,2

fD2R,0

fD2R,1

Figure 2: D2R Channelization

Each D2R RF channel is denoted by CD2R,n and defined by a carrier (center) frequency fD2R,n, where n∈{0,…,ND2R-1}. The carrier frequency of the RF channel can be a frequency in the FDD uplink or downlink frequency range. The RF channel CD2R,0 represents the lowest RF channel number with the lowest center frequency fD2R,0. Given the center frequency of the first RF channel and assume the bandwidth of each RF channel (Bocc,D2R) is the same, the center frequency of the nth D2R RF channel can be obtained as

fD2R,n=fD2R,0+Bocc,D2R×n,  		n∈{0,…,ND2R-1}

Each D2R RF channel fD2R,n can be assigned to a different Ambient IoT device for device-to-reader transmission. As such, the device should be able to transmit on the assigned D2R RF channel. Since Device 1 and Device 2a are not equipped with a local oscillator, baseband frequency shifting is used, e.g., by multiplying the CW with a square wave; refer to [2] for more details.

Observation 4: For D2R transmission, frequency-domain multiple access has higher spectrum utilization than time-domain multiple access if Bsys,D2R>Bocc,D2R,

Observation 5: For D2R transmission, frequency-domain multiple access requires Device 1 and Device 2a to be capable of shifting (or transmitting) on the assigned channel frequency.

The spectrum utilization of the frequency-domain multiple access can be further increased if it is combined with the time-domain multiple access scheme.

Observation 6: For D2R transmission, combined frequency-domain multiple access and time-domain multiple access result in the highest spectrum utilization if  Bsys,D2R>Bocc,D2R, which can fulfil the high device/connection density requirement of certain Ambient IoT use cases.

Proposal 3: For D2R transmission, support combined frequency-domain multiple access and time-domain multiple access.

Proposal 4: For D2R transmission, code-domain multiple access can be deprioritized.

### Modulation

During the RAN1#116-bis meeting, RAN1 agreed to study the following D2R modulation techniques:

Agreement Study for all devices the following for D2R baseband modulation, for potential down-selection: - OOK - Binary PSK - Binary FSK - Strive to identify one variant of Binary FSK to study further

The achievable bit rate is directly proportional to the transmission bandwidth Btx,D2R, modulation scheme, and SNR. Once the occupied bandwidth Btx,D2R is allotted, the maximum bit rate achievable depends on the spectral efficiency of the selected modulation scheme. FSK modulation is known to have lower spectral efficiency than ASK and PSK modulation techniques but can have better detection performance. Therefore, each modulation candidate has its pros and cons and as such, all modulation candidates should be studied, but ASK and BPSK can be prioritized to limit the SI scope.

Proposal 5: For D2R baseband modulation, RAN1 prioritizes OOK and binary BPSK.

## Reader to Device (R2D)

In this section, we discuss the waveform, modulation, and line encoding options for R2D link. In RAN1#116, the following were agreed for R2D modulation and line encoding [3]:

Agreement A-IoT DL study includes OOK from DL transmitter’s perspective. - For an OFDM waveform, assume OOK-1 for single-chip per OFDM symbol transmission, and OOK-4 for M-chip per OFDM symbol transmission, starting from definitions in TR 38.869. - FFS value(s) of M. - FFS: Any changes needed from the definitions in TR 38.869. - FFS: Exact definition of chip - If other DL waveforms are included, further elaboration of the transmitter’s OOK generation would be needed. Agreement For R2D, line codes studied are: Manchester encoding and pulse-interval encoding (PIE). - FFS: Mapping(s) from bit(s) to line-code codewords - FFS: Time domain definition of e.g., chips and relation to OFDM symbols, resource allocation unit, etc.

In TR 38.869, a chip was not defined but rather an OOK bit, which was understood as a coded bit in the line-code codeword, where only Manchester encoding was discussed. However, given the consideration of PIE is also one of the line encoding schemes, the definition of a chip as a coded OOK bit may not directly apply to PIE. Therefore, a chip may be defined as the coded bit or the reference interval of a line-code codeword as discussed next.

Proposal 6: For R2D transmission, adopt the definition of a chip as the coded bit or the reference interval of a line-code codeword.

n the second agreement above, the mapping from information bits to line-code codewords as well as the time definition of chips relative to OFDM symbol are left as FFS. For Manchester encoding, there are two conventions for the mapping from information bit(s) to line-code codewords. In the first convention, an information bit 1 can be mapped to the line codeword {1 0} whereas an information bit 0 is mapped to the line codeword {0 1}. In the second convention, an information bit 1 can be mapped to the line codeword {0 1} whereas an information bit 0 is mapped to the line codeword {1 0}. For Ambient IoT R2D link evaluation, one of the conventions can be agreed. Without considering CP handling, in this context, a chip can correspond to a coded bit in any of the codewords and the chip duration can correspond to the OFDM symbol duration (without cyclic prefix) divided by the number of chips per OFDM symbol M.

Proposal 7: For Manchester encoding in R2D, adopt one of the following conventions for mapping from information bits to codewords:
Convention 1: Information bit 1 to codeword {1 0} and information bit 0 to codeword {0 1}
Convention 2: Information bit 1 to codeword {0 1} and information bit 0 to codeword {1 0}

At the RAN1#116-bis meeting, the following agreement was made for R2D CP handling.

Agreement For R2D CP handling for OFDM based OOK waveform: - For potential down-selection, study among the following candidate methods - Method Type 1: Removal of CP at device without specified transmit-side - FFS: How device determines the CP location - FFS: Impact on feasibility of device SFO - FFS: relation to M, if any - Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. - FFS: Whether/how to arrange that OOK chips have equal length after CP insertion - FFS: relation to M, if any - FFS: Detail of relationship to line code codewords - FFS: Impact on feasibility of device SFO - [Other method types are not precluded] - Study of the methods should include e.g.: - CP impact on R2D timing acquisition, and decoding & performance of PRDCH - Reader and device implementation complexities - Interference between R2D and NR DL/UL if in the same NR band - Spectrum efficiency

Due to the expected low complexity of the A-IoT devices, Method Type 2 where CP insertion is handled by waveform/signal design at the transmitter, can be supported.

For example, considering PIE, a three-step mapping procedure can be used to generate the PIE signal while accounting for cyclic-prefix insertion in OFDM-based waveform generation. In the first step, an information bit 1 can be mapped to the codeword {1 0} whereas an information bit 0 is mapped to the codeword {0}. In the second step, each coded bit is mapped to a symbol (basis function) where a coded bit 1 is mapped to a symbol/function (S1) and a coded bit 0 is mapped to a symbol/function (S2). The two symbols/functions (S1 and S2) can be selected as in Figure 5 where it can be noted that the beginning and end of each of them is always high and the duration of the high value at the beginning/end can be selected to be at least equal to the cyclic-prefix duration (TCP), i.e., Tp1, Tp2≥TCP. In the third step, a number of samples (NCP) corresponding to the cyclic-prefix duration are removed from the beginning of the group of symbols/functions in an OFDM symbol. In this context, a chip can be a reference interval corresponding to a symbol/function and for this generation procedure, the chip duration/reference interval (Tchip) can be fixed for both symbols/functions and corresponds to the OFDM symbol duration (with cyclic-prefix) divided by the number of chips per OFDM symbol M. An illustration of the first two steps of the PIE generation procedure is shown in Figure 6, assuming Tp1=Tp2, where the third step combined with the cyclic-prefix insertion, as part of the OFDM generator, should result in the same waveform as in the second step.

A similar three-step mapping procedure can be used to generate the Manchester encoded signal while accounting for cyclic-prefix insertion in OFDM-based waveform generation where only the first step is updated such that the information bit 1 is mapped to the codeword {1 0} whereas the information bit 0 is mapped to the codeword {0 1}.

| - Symbol/Basis function (S1) for coded bit 1 | - Symbol/Basis function (S2) for coded bit 0 |
| --- | --- |

Figure 5: Mapping between coded bits and symbols/basis functions

Figure 6: Illustrative example of OFDM-based PIE waveform generation

Proposal 8: For PIE in R2D, adopt the following steps, as a starting point, for mapping from information bits to codewords and waveform generation with CP handling:
Step 1: Map information bit 1 to codeword [1 0] and information bit 0 to codeword [0].
Step 2: Map coded bits 1 and 0 to symbols/functions S1 and S2 in Figure 5, respectively.
Step 3: Remove NCP samples from the beginning of the group of symbols/functions in an OFDM symbol

Proposal 9: For Manchester encoding in R2D, adopt the following steps, as a starting point, for mapping from information bits to codewords and waveform generation with CP handling:
Step 1: Map information bit 1 to codeword [1 0] and information bit 0 to codeword [0 1].
Step 2: Map coded bits 1 and 0 to symbols/functions S1 and S2 in Figure 5, respectively.
Step 3: Remove NCP samples from the beginning of the group of symbols/functions in an OFDM symbol

# Conclusion

This contribution has discussed the open issues of the general aspects of physical layer design. Our observations and recommendations are as follows:

Proposal 1: For D2R multiple access studies, RAN1 defines the D2R system bandwidth as follows:

- D2R system bandwidth, Bsys,D2R: The total contiguous frequency resources (or spectrum) allocated for Ambient IoT D2R transmission, where

  - Bsys,D2R≥Bocc,D2R>Btx,D2R

Observation 1: For D2R transmission, the D2R system bandwidth Bsys,D2R is a configurable parameter, which can be expressed as an integer multiple of NR PRBs.

Proposal 2: For D2R transmission, the minimum D2R occupied bandwidth Bocc,D2R can be assumed to be one PRB as a starting point.

Observation 2: For D2R transmission, code-domain multiple access is not feasible.

Observation 3: For D2R transmission, time-domain multiple access might result in low spectrum utilization if  Bocc,D2R≪Bsys,D2R.

Observation 4: For D2R transmission, frequency-domain multiple access has higher spectrum utilization than time-domain multiple access if Bsys,D2R>Bocc,D2R,

Observation 5: For D2R transmission, frequency-domain multiple access requires Device 1 and Device 2a to be capable of shifting (or transmitting) on the assigned channel frequency.

Observation 6: For D2R transmission, combined frequency-domain multiple access and time-domain multiple access result in the highest spectrum utilization if  Bsys,D2R>Bocc,D2R, which can fulfil the high device/connection density requirement of certain Ambient IoT use cases.

Proposal 3: For D2R transmission, support combined frequency-domain multiple access and time-domain multiple access.

Proposal 4: For D2R transmission, code-domain multiple access can be deprioritized.

Proposal 5: For D2R baseband modulation, RAN1 prioritizes OOK and binary BPSK.

Proposal 6: For R2D transmission, adopt the definition of a chip as the coded bit or the reference interval of a line-code codeword.

Proposal 7: For Manchester encoding in R2D, adopt one of the following conventions for mapping from information bits to codewords:
Convention 1: Information bit 1 to codeword {1 0} and information bit 0 to codeword {0 1}
Convention 2: Information bit 1 to codeword {0 1} and information bit 0 to codeword {1 0}

Proposal 8: For PIE in R2D, adopt the following steps, as a starting point, for mapping from information bits to codewords and waveform generation with CP handling:
Step 1: Map information bit 1 to codeword [1 0] and information bit 0 to codeword [0].
Step 2: Map coded bits 1 and 0 to symbols/functions S1 and S2 in Figure 5, respectively.
Step 3: Remove NCP samples from the beginning of the group of symbols/functions in an OFDM symbol

Proposal 9: For Manchester encoding in R2D, adopt the following steps, as a starting point, for mapping from information bits to codewords and waveform generation with CP handling:
Step 1: Map information bit 1 to codeword [1 0] and information bit 0 to codeword [0 1].
Step 2: Map coded bits 1 and 0 to symbols/functions S1 and S2 in Figure 5, respectively.
Step 3: Remove NCP samples from the beginning of the group of symbols/functions in an OFDM symbol

# References

R1-2403663, Session notes for 9.4 Study on solutions for Ambient IoT (Internet of Things), Ad-Hoc Chair (Huawei)

R1-2403859, Discussion on Rel-19 Ambient IoT device architecture, Futurewei

R1-2401767, Session notes for 9.4 (Study on solutions for Ambient IoT in NR), Ad-Hoc Chair (Huawei)

EPC® Radio-Frequency Identity Generation-2 UHF RFID Standard, Release 3.0, Jan 2024

# Appendix A: RAN1#116-bis agreements for general aspects of physical layer design

Agreement

Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS.

Agreement

Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS.

Agreement

Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

Agreement

The following bandwidths for D2R are defined for the purpose of the study:

- Transmission bandwidth, Btx,D2R: The frequency resources scheduled by a reader for a D2R transmission from one device.

  - FFS in agenda 9.4.2.3: how frequency resources scheduled by a reader are determined

- Occupied bandwidth, Bocc,D2R: The transmission bandwidth plus the potential associated intra A-IoT guard-bands totalling Bguard,D2R

  - Note: this guard band is not for coexistence with NR/LTE

- If/how to define guard band for coexistence between A-IoT D2R and NR/LTE is up to RAN4.

- Bocc,D2R >= Btx,D2R

  - Possible values of each bandwidth are FFS

Agreement

For D2R, study: Manchester encoding, FM0 encoding, Miller encoding, no line coding.

- FFS: Mapping(s) from bit(s) to line-code codewords

- FFS: How to achieve small frequency shift in baseband and/or FDM(A) among devices

- Aspects to study include:

  - Spectrum shape

  - Complexity

  - Power consumption

  - BER, BLER

  - Resilience to SFO

  - If there is any relation to CFO

Agreement

A-IoT D2R study of FEC includes at least convolutional codes.

- Comparisons are encouraged to compare to the case of no FEC

- FFS details of convolutional codes, such as polynomial(s), shift-register termination, etc.

- FFS if other FEC candidates/methods will be studied.

Agreement

Study

- baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PRDCH

- baseline: using 6 bits and 16 bits CRC with polynomials from TS 38.212, or no CRC, for PDRCH

- FFS: details when different CRC lengths or no CRC may be used

- FFS: other 6 bits and 16 bits CRC with different polynomials than from TS 38.212

Agreement

Study D2R transmission in the physical layer using repetition

- Note: Discussions regarding higher-layer repetitions are up to RAN2.

Agreement

R2D study includes subcarrier spacing of 15 kHz, from the reader perspective, for OFDM-based waveform.

- Inclusion in the study of subcarrier spacing of 30 kHz is FFS.

Agreement

For R2D study OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D is ≤ [12] PRBs and is down-selected among:

- Alt 1: Including 180 kHz, 360 kHz, and FFS other values

- Alt 2: Integer multiple(s) of 180 kHz (FFS: what integer(s))

- Alt 3: Integer multiple(s) of the subcarrier spacing (FFS: what integer(s))

Agreement

For R2D CP handling for OFDM based OOK waveform:

- For potential down-selection, study among the following candidate methods

  - Method Type 1: Removal of CP at device without specified transmit-side

    - FFS: How device determines the CP location

    - FFS: Impact on feasibility of device SFO

    - FFS: relation to M, if any

  - Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n.

    - FFS: Whether/how to arrange that OOK chips have equal length after CP insertion

    - FFS: relation to M, if any

    - FFS: Detail of relationship to line code codewords

    - FFS: Impact on feasibility of device SFO

  - [Other method types are not precluded]

- Study of the methods should include e.g.:

  - CP impact on R2D timing acquisition, and decoding & performance of PRDCH

  - Reader and device implementation complexities

  - Interference between R2D and NR DL/UL if in the same NR band

  - Spectrum efficiency

Agreement

Study for all devices the following for D2R baseband modulation, for potential down-selection:

- OOK

- Binary PSK

- Binary FSK

  - Strive to identify one variant of Binary FSK to study further

# Appendix B: RAN1#116 agreements for general aspects of physical layer design

Agreement

A-IoT DL study includes an OFDM-based waveform from A-IoT R2D (reader-to-device) perspective.

- Depending on what modulation(s) are decided to be studied:

  - Study whether/how to handle CP at transmitter/device/design

- Study other characteristics of the OFDM waveform, e.g.:

  - CP-OFDM

  - DFT-s-OFDM

  - Etc.

  - The type of OFDM waveform is transparent to A-IoT device.

Other waveforms from DL transmitter’s perspective can be proposed, and further discussion will consider whether or not they are included in the study.

Agreement

A-IoT DL study includes OOK from DL transmitter’s perspective.

- For an OFDM waveform, assume OOK-1 for single-chip per OFDM symbol transmission, and OOK-4 for M-chip per OFDM symbol transmission, starting from definitions in TR 38.869.

  - FFS value(s) of M.

  - FFS: Any changes needed from the definitions in TR 38.869.

  - FFS: Exact definition of chip

- If other DL waveforms are included, further elaboration of the transmitter’s OOK generation would be needed.

Agreement

For R2D, line codes studied are: Manchester encoding and pulse-interval encoding (PIE).

- FFS: Mapping(s) from bit(s) to line-code codewords

- FFS: Time domain definition of e.g., chips and relation to OFDM symbols, resource allocation unit, etc.

Agreement

Regarding FEC, R2D with no forward error-correction code (FEC) is studied as baseline.

- Evaluations would be by comparison to this baseline

Agreement

R2D study assumes use of CRC. FFS which CRC generator polynomial(s) are assumed, and if any cases are included with no CRC.

- FFS: Association, if any, between down-selected CRC(s) and message size, considering at least false-alarm rate target

Agreement

D2R study assumes use of CRC. FFS which CRC generator polynomial(s) are assumed, and if any cases are included with no CRC.

- FFS: Association, if any, between down-selected CRC(s) and message size, considering at least false-alarm rate target

Agreement

At least the following bandwidths for R2D are defined for the purpose of the study:

- Transmission bandwidth, Btx,R2D from a Reader perspective: The frequency resources used for transmitting R2D

- Occupied bandwidth, Bocc,R2D from a Reader perspective: The frequency resources used for transmitting R2D, and potential guard band

- Bocc,R2D ≥ Btx,R2D

  - FFS: Further constraint(s) e.g. Bocc,R2D = Btx,R2D.

  - Possible values of each bandwidth are FFS