3GPP TSG RAN WG1 #117			R1-2405167

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:	9.7.1

Source: 	Qualcomm Incorporated

Title: 	ISAC Deployment Scenarios

Document for:		Discussion

- Introduction

A RAN1-led study item has been agreed in RAN#102 for channel modeling for sensing, with the following objectives [1]

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

At RAN1 #1160-bis, the following agreements were made related to ISAC deployment scenarios:

Agreement RAN1 agrees the following ISAC terminology with minor modifications as follows: For ISAC channel modelling, RAN1 uses the sensing related terminology as defined in TS22.137 or TR22.837 as a starting point for discussion purposes with the following definitions: - Sensing transmitter: the TRP or a UE that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be located in the same or different TRP or a UE as the sensing receiver. - Sensing receiver: the TRP or a UE that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be located in the same or different TRP or a UE as the sensing transmitter. - Sensing target: target that need to be sensed by deriving characteristics of the objects within the environment from the sensing signal. - Background environment: background (clutter and/or environmental objects) that are not the sensing target(s). - Mono-static sensing: sensing where a sensing transmitter that transmits a sensing signal and a sensing receiver that receives the sensing signal are co-located in the same TRP or UE. - Bi-static sensing: sensing where a sensing transmitter that transmits a sensing signal and a sensing receiver that receives the sensing signal are not co-located in the same TRP or UE. - Multi-static sensing: sensing where there are multiple sensing transmitters and/or multiple sensing receivers, for a sensing target. - Sensing signal: Transmissions on the 3GPP radio interface that can be used for sensing purposes. Agreement Any TRP and/or UE location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations. FFS: other possible sensing transmitters and receivers locations. Agreement The following table can be used by companies to propose values for each sensing target - Additional parameters/rows can be added if needed Table x. Evaluation parameter template for sensing scenarios Parameters; Value. Applicable communication scenarios. Sensing transmitters and receivers properties. Supported sensing modes. Sensing target; Outdoor/indoor. 3D mobility. 3D distribution. Orientation. Physical characteristics (e.g., size). [Unintended/Environment objects]; Types. 3D mobility. 3D distribution. Orientation. Physical characteristics (e.g., size). [Sensing area]. Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects].

In this contribution, we summarize our views on the next steps on ISAC deployment scenarios.

- Sensing target & scenario mapping table

With regards to the sensing targets and the agreed mapping table, we have the following observations:

- If the sensing target is a human indoor, we do not consider outdoor-to-indoor sensing as a likely deployment scenario, nor a viable/feasible one, at least when it comes to the level of modelling that is expected within a 3GPP framework and timeline; therefore, we prefer to not pursue it further.

- Similarly, if the sensing target is a human outdoor, with regards to the RMa scenario with ISD of 1732 m or 5000 m, it is unlikely any reasonable sensing capability will be demonstrated due to large ISD and the large pathloss; therefore, we also prefer to not pursue it further.

Therefore, we make the following proposal:

Proposal 1: Resolve the brackets in the sensing target & scenario mapping table as follows:

- For the “Human indoors” sensing target

  - do not include Umi, Uma as a scenario

- For the “Human outdoor” sensing target

  - Do not include RMa as a scenario

- Sensing modes

The deployment scenario should be independent of use cases and sensing modes. The channel modelling should be unified for any use case and sensing mode. During last meeting, there was a discussion to exclude 2 cases:

- For UAV sensing, it was discussed whether to exclude the case that a UE is the sensing transmitter or receiver. However, we think that is a likely case that either, a UAV (e.g. which is a UE) could be receiving the reflected signals, or transmitting to sense other UAVs, or that another type of UE (e.g. handheld device) is picking up the reflections from a transmitting gNB.

- For human sensing, it was discussed whether to deprioritize sensing modes where there is no TRP involvement. Such cases in which a UE is picking up reflections (e.g. UE-UE monostatic, UE-UE bi-static, UE being an RSU, etc) are very useful to be considered given the vast use-cases of interest that 3GPP should consider (e.g. in the automotive sensing case, a car is sensing another car, or an RSU is sensing another UE; in the indoor use-cases, a device is sensing through monostatic sensing an intruder, etc).

Furthermore, when it comes to the channel model for sensing, deprioritizing a subset of sensing modes for specific use-cases may not necessarily introduce any modeling simplification; instead, it might impose an unnecessary limitation on the model’s applicability. Based on the above, we make the following proposal:

Proposal 2: For ISAC use cases and sensing targets, RAN1 should consider all six sensing modes without an exception and without an explicit prioritization.

- Frequency ranges

With regards to the frequency ranges associated with a given sensing target, in a similar way as described above, when it comes to the channel model for sensing, deprioritizing a specific FR for specific use-cases may not necessarily introduce any modeling simplification; instead, it might impose an unnecessary limitation on the model’s applicability.

Proposal 3: For all the sensing targets considered, both FR1 and FR2 is considered without an exception and without an explicit prioritization.

- Detailed scenario parameters

In this Section, we provide, for each agreed sensing target, a set of initial main parameters that should be considered with regards to the parameters and the assumption used in the corresponding scenario.

  - General parameters for modelling an object & object types

With regards to the parameters for modelling a sensing target or an environment object, we believe that the same parameters should be used for both. In either case, these are just objects that can be abstracted out using a few main parameters.

Proposal 4: The general parameters that model a sensing target should be the same as that for an environment object. In other words, we can just discuss how to parametrize an “object”.

Proposal 5: Define modeling of objects as follows:

- Define families of object types, and reuse these object types across multiple scenarios.

Proposal 6: Define at least the following families of object types:

- Object type 1: UAV

- Object type 2: Human

- Object type 3: AGV

- Object type 4: Vehicle

- Object type 5: Animal

- Object type 6: Building/wall

- Object type 7: Rain

  - UAV scenarios

With regards to the evaluation parameters for UAV sensing scenarios, we make the following updates:

Proposal 7: Support the following updates in the Evaluation parameter table for UAV sensing scenarios:

| Parameters | Value |  |
| --- | --- | --- |
| Applicable communication scenarios | UMA-AV (FR1 only), UMi-AV (FR1 & FR2), RMa-AV (FR1 only) |  |
| System parameters | Carrier frequency of FR1(3.5 GHz) & FR2 (28 GHz) - For FR1, use as starting point the system parameters and the values from TR 36.777 for the RMa/Uma/Umi scenarios in Table A.1-1 - For FR2, use as a starting point the system parameters from TR 36.777 Table A.1-1 and TR 38.855 (Tables 6.1.1-1, 6.1.1-2, 6.1.1-4) with at least the following FR2-specific changes: Scenario; UMi-AV. Cell layout; Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = 200m). Carrier frequency; 28 GHz. Bandwidth; 100 MHz, 400 MHz. SCS; 120 KHz. UE antenna configuration; Multi-panel configuration 1 and panel configuration a. Total gNB Tx power; 37 dBm per panel. gNB antenna configuration; (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ. UE drop procedure; 100% outdoor uniformly distributed over the horizontal area. UE mobility; Outdoor: 3km/h. gNB Noise Figure; 7 dB. UE Noise Figure; 13 dB. |
| Sensing transmitters and receivers properties | BSs and UEs in the corresponding communication scenario |  |
| Supported sensing modes | All 6 modes: TRP monostatic, TRP bistatic, Aerial UE monostatic, Aerial UE bistatic, TRP – Aerial UE bistatic, Aerial UE – TRP bistatic |  |
| Sensing target | Outdoor/indoor | Outdoor |
|  | LOS/NLOS (Tx-UAV, and UAV-Rx) | - Alt1: LOS condition - Alt2: LOS/NLOS condition based on LOS probability equation in TR36.777 |
|  | 3D mobility | horizontal plane only: - Option 1: uniform between 3 and 160 km/h - Option 2: 3 km/h, 30 km/h, 60 km/h, 160 km/h Maximum flight height: - Option 1: up to 300 m; - Option 2: up to 600m |
|  | 3D distribution | - Uniform with up to [1, 2, …, 5] targets per sector – only for the center site - Height: Uniformly distributed between 1.5m and 300m |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Option 1: [0.15m x 0.15m x 0.05m] - Option 2: [1.5m x 1.5m x 0.5m] - Option 3: [3m x 3m x 1m] RCS modelling: - FFS |
| Unintended/Environment objects | Types | FFS |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | FFS |
| Sensing area | For TRP-monostatic and TRP-TRP bistatic: - Same as center site layout For TRP- aerial UE and aerial UE- aerial UE - X meters 3-D distance around an Areal UE |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] | - Minimum distance TRP – UAV: 10 m - Minimum distance UAV – UAV: 10 m |  |

  - Humans scenarios

With regards to the evaluation parameters for humans sensing scenarios, we make the following updates:

Proposal 8: Support the following updates in the Evaluation parameter table for humans sensing scenarios:

| Parameters | Value |  |
| --- | --- | --- |
| Applicable communication scenarios | UMI, UMA, InH, InF |  |
| System parameters | Carrier frequency of FR1 (3.5 GHz) & FR2 (28 GHz) - For InH, InF, for both FR1 and FR2, use as a starting point the systems parameters and values from TR 38.855 Scenario 1 (Indoor Office for FR1 and FR2 (Open Office)) in Section 6.1 and Tables 6.1.1-1, Tables 6.1.1-2, Tables 6.1.1-3 - For UMI, UMA, - for both FR1 and FR2, for Umi, use as a starting point the systems parameters and values from TR 38.855 Scenario 2 (UMi street canyon for FR1 and FR2 (ISD 200m) in Section 6.1 and Tables 6.1.1-1, Tables 6.1.1-2, Tables 6.1.1-4, Tables 6.1.1-5 - for FR1 only, for UMa, use as a starting point the systems parameters and values from TR 38.855 Scenario 3 (UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)) in Section 6.1 and Tables 6.1.1-1, Tables 6.1.1-5, Tables 6.1.1-6. |  |
| Sensing transmitters and receivers properties | BSs and UEs in the corresponding communication scenario |  |
| Supported sensing modes | All 6 modes: TRP monostatic, TRP bistatic, UE monostatic, UE bistatic, TRP –UE bistatic, UE – TRP bistatic |  |
| Sensing target | Outdoor/indoor | Outdoor (for Umi, Uma) Indoor (for InH, InF) |
|  | LOS/NLOS (Tx-target, and target-Rx) | - Option 1: LOS condition - Option 2: LOS/NLOS condition based on LOS probability equation in TR38.901 |
|  | 3D mobility | horizontal plane only: - Human velocity: 0, 3, 10 km/h |
|  | 3D distribution | - Uniform with up to [1, 2, …, 5] targets |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult: 0.5m x 0.5m x 1.75m RCS modelling: - FFS |
| Unintended/Environment objects | Types | Option 1: Human(s) Option 2: AGVs (InF) |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | For Option 1: Human - Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult: 0.5m x 0.5m x 1.75m For Option 2: AGV - Size (Length x Width x Height): - Option 1: 0.5m x 0.5m x 0.2m - Option 2: 0.9m x 0.6m x 1m |
| Sensing area | For TRP-monostatic and TRP-TRP bistatic: - All indoor deployment area For TRP-UE and UE monostatic/bistatic - X meters 3-D distance around a UE |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] | FFS |  |

  - Automotive vehicles scenarios

Proposal 9: Support the following updates in the Evaluation parameter table for automotive vehicles sensing scenarios:

| Parameters | Value |  |
| --- | --- | --- |
| Applicable communication scenarios | Highway, Urban grid |  |
| System parameters | Reuse the parameters and values from TR 37.885 (Sections 6.1.1, 6.1.2, 6.1.3, 6.1.4, 6.1.5), with the following notes/changes: Bandwidth; 200MHz, 400 MHz, or 800 MHz. Carrier frequency; 30 GHz. Inter-BS distance; 250m, 500m. Antenna array configuration for vehicle UE; (2,4,2,1,1), or (2,4,2,8,1) - In addition to (2,4,2,1,1) in TR 37.885, antenna array configuration of (2,4,2,8,1) can be used for vehicular UE to achieve high angular resolution sensing desirable for LRR/SRR use cases. Antenna array bearing angle; 45°, 90°, 135°, or 270° for corner or side SRR use cases. Traffic model; periodic and aperiodic sensing traffic with update rate parameters (5 to 20 fps for LRR and 20 to 50 fps for SRR). |
| Sensing transmitters and receivers properties | Define percent of UEs with sensing capability only, or communication only capability, or both communication and sensing capability - An example, radar only can be 25% or 100% of the total UE drop with remaining UE as communication only |  |
| Supported sensing modes | All 6 modes: TRP monostatic, TRP bistatic, UE monostatic, UE bistatic, TRP –UE bistatic, UE – TRP bistatic |  |
| Sensing target | Outdoor/indoor | Outdoor |
|  | LOS/NLOS (Tx-target, and target-Rx) | Support both the following options: - Option 1: LOS for Highway/urban grid scenario = 1 - Option 2: According to Table 6.2-1 |
|  | 3D mobility | - Option 1: 140 km/h in all the lanes - Option 2: (optional) Vehicle speed is 70 km/h or 250 km/h in all the lanes |
|  | 3D distribution | The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {r meter, an exponential random variable with the average of the speed * t sec}. Instead of a single value of 2 for both r and t (as used in TR 37.885), more values can be provided for both these parameters for LRR /SRR use cases: FFS on (r,t) values |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | all three types/sizes included in Section 6.1.2 of TR 37.885 |
| Unintended/Environment objects | Types | For Option 1: Human - Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult: 0.5m x 0.5m x 1.75m For Option 2: vehicles - all three types/sizes included in Section 6.1.2 of TR 37.885 |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | For Option 1: Human - Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult: 0.5m x 0.5m x 1.75m For Option 2: vehicles - all three types/sizes included in Section 6.1.2 of TR 37.885 |
| Sensing area | For TRP-monostatic and TRP-TRP bistatic: - All indoor deployment area For TRP-UE and UE monostatic/bistatic - X meters 3-D distance around a UE |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] | FFS |  |

  - Automated guided vehicles scenarios

Proposal 10: Support the following updates in the Evaluation parameter table for AGV sensing scenarios:

| Parameters | Value |  |
| --- | --- | --- |
| Applicable communication scenarios | InF |  |
| System parameters | Carrier frequency of FR1 (3.5 GHz) & FR2 (28 GHz) - For both FR1 and FR2, use a starting point the parameters and values for the InF-SH and InF-DH scenarios from TR 38.857 Section 6.1 |  |
| Sensing transmitters and receivers properties | BSs and UEs in the corresponding communication scenario |  |
| Supported sensing modes | All 6 modes: TRP monostatic, TRP bistatic, UE monostatic, UE bistatic, TRP –UE bistatic, UE – TRP bistatic |  |
| Sensing target | Outdoor/indoor | Indoor |
|  | LOS/NLOS (Tx-target, and target-Rx) | LOS/NLOS condition based on LOS probability equation in TR38.901 |
|  | 3D mobility | horizontal plane only: - AGV velocity: 5, 30 km/h |
|  | 3D distribution | - Uniform with up to [1, 2, …, 5] targets |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Option 1: 0.5m x 0.5m x 0.2m - Option 2: 0.9m x 0.6m x 1m RCS modelling: - FFS |
| Unintended/Environment objects | Types | Option 1: Human(s) Option 2: AGVs |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | FFS |
| Sensing area | For TRP-monostatic and TRP-TRP bistatic: - All indoor deployment area For TRP-UE and UE monostatic/bistatic - X meters 3-D distance around a UE |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] | FFS |  |

  - Objects creating hazards on roads/railways scenarios

With regards to the objects creating hazards on roads/railways scenarios, the following 3 scenarios were agreed to be considered further:

| Sensing Targets | scenarios |
| --- | --- |
| Objects creating hazards on roads/railways (examples defined in TR 22.837) | Highway, Urban grid, HST |

For this sensing scenario, we make the following proposals with regards to the objects that create hazards and their characteristics:

Proposal 11: Support the following updates in the Evaluation parameter table for sensing of objects creating hazards on roads/railways scenarios:

| Parameters | Value |  |
| --- | --- | --- |
| Applicable communication scenarios | Highway, Urban grid |  |
| System parameters | Reuse the parameters and values from TR 37.885 (Sections 6.1.1, 6.1.2, 6.1.3, 6.1.4, 6.1.5), with the following notes/changes: Bandwidth; 200MHz, 400 MHz, or 800 MHz. Carrier frequency; 30 GHz. Inter-BS distance; 250m, 500m. Antenna array configuration for vehicle UE; (2,4,2,1,1), or (2,4,2,8,1) - In addition to (2,4,2,1,1) in TR 37.885, antenna array configuration of (2,4,2,8,1) can be used for vehicular UE to achieve high angular resolution sensing desirable for LRR/SRR use cases. Antenna array bearing angle; 45°, 90°, 135°, or 270° for corner or side SRR use cases. Traffic model; periodic and aperiodic sensing traffic with update rate parameters (5 to 20 fps for LRR and 20 to 50 fps for SRR). |
| Sensing transmitters and receivers properties | Define percent of UEs with sensing capability only, or communication only capability, or both communication and sensing capability - An example, radar only can be 25% or 100% of the total UE drop with remaining UE as communication only |  |
| Supported sensing modes | All 6 modes: TRP monostatic, TRP bistatic, UE monostatic, UE bistatic, TRP –UE bistatic, UE – TRP bistatic |  |
| Sensing target | Outdoor/indoor | Outdoor |
|  | LOS/NLOS (Tx-target, and target-Rx) | Support both the following options: - Option 1: LOS for Highway scenario = 1 - Option 2: According to Table 6.2-1 |
|  | 3D mobility | - Option 1: 140 km/h in all the lanes - Option 2: (optional) Vehicle speed is 70 km/h or 250 km/h in all the lanes |
|  | 3D distribution | uniform |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | - Animal (Sheep/deer) - Dimensions: 1.5m x 0.5m x 1 m - velocity: 5 km/h - Pedestrian: Reuse the agreed dimensions and velocity from the “Human scenarios” - vehicle: Reuse the agreed dimensions and velocities from the “automotive scenarios” |
| Unintended/Environment objects | Types | FFS |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | FFS |
| Sensing area | For TRP-monostatic and TRP-TRP bistatic: - All indoor deployment area For TRP-UE and UE monostatic/bistatic - X meters 3-D distance around a UE |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] | FFS |  |

- RCS Information for different Object types

Radar Cross Section (RCS), is a very important parameter that describes how detectable an object is by a radar system. Essentially, it quantifies how much radar energy an object reflects back to the source. A larger RCS indicates that an object is more easily detected by radar systems.

After the object types have been defined with their associated dimensions, typical RCS values would need to be agreed. RCS depends on several factors, including the angle and the carrier frequency. For example, for UAV sensing, prior academic studies ([3],[4]) have shown that large RCS variation is observed with various angles, and that Maximum RCS is observed when sensing direction is perpendicular to the UAV plane, and  minimum RCS when sensing direction is in the UAV plane. Similarly, it was observed that RCS may experience deep fades at a few carrier frequencies.

Proposal 12: Use the following average RCS value ranges as a starting point for the following object types:

| Object type | Average RCS |
| --- | --- |
| Type 1: UAV | [-15 to 3] dBsm when facing the UAV |
| Type 2: Human | [-10 to 0] dBsm |
| Type 3: Vehicle | [10 to 20] dBsm |

- Study how to model, for any given object, RCS dependencies on the incident and the reflection angles to the object’s orientation and on the carrier frequency

- FFS: RCS value ranges for the remaining object types after their dimensions have been clarified

- Conclusion

We have discussed ISAC deployment scenarios and made the following proposals:

Proposal 1: Resolve the brackets in the sensing target & scenario mapping table as follows:

- For the “Human indoors” sensing target

  - do not include Umi, Uma as a scenario

- For the “Human outdoor” sensing target

  - Do not include RMa as a scenario

Proposal 2: For ISAC use cases and sensing targets, RAN1 should consider all six sensing modes without an exception and without an explicit prioritization.

Proposal 3: For all the sensing targets considered, both FR1 and FR2 is considered without an exception and without an explicit prioritization.

Proposal 4: The general parameters that model a sensing target should be the same as that for an environment object. In other words, we can just discuss how to parametrize an “object”.

Proposal 5: Define modeling of objects as follows:

- Define families of object types, and reuse these object types across multiple scenarios.

Proposal 6: Define at least the following families of object types:

- Object type 1: UAV

- Object type 2: Human

- Object type 3: AGV

- Object type 4: Vehicle

- Object type 5: Animal

- Object type 6: Building/wall

- Object type 7: rain

Proposal 7: Support the following updates in the Evaluation parameter table for UAV sensing scenarios:

| Parameters | Value |  |
| --- | --- | --- |
| Applicable communication scenarios | UMA-AV (FR1 only), UMi-AV (FR1 & FR2), RMa-AV (FR1 only) |  |
| System parameters | Carrier frequency of FR1(3.5 GHz) & FR2 (28 GHz) - For FR1, use as starting point the system parameters and the values from TR 36.777 for the RMa/Uma/Umi scenarios in Table A.1-1 - For FR2, use as a starting point the system parameters from TR 36.777 Table A.1-1 and TR 38.855 (Tables 6.1.1-1, 6.1.1-2, 6.1.1-4) with at least the following FR2-specific changes: Scenario; UMi-AV. Cell layout; Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = 200m). Carrier frequency; 28 GHz. Bandwidth; 100 MHz, 400 MHz. SCS; 120 KHz. UE antenna configuration; Multi-panel configuration 1 and panel configuration a. Total gNB Tx power; 37 dBm per panel. gNB antenna configuration; (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2), (dH, dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ. UE drop procedure; 100% outdoor uniformly distributed over the horizontal area. UE mobility; Outdoor: 3km/h. gNB Noise Figure; 7 dB. UE Noise Figure; 13 dB. |
| Sensing transmitters and receivers properties | BSs and UEs in the corresponding communication scenario |  |
| Supported sensing modes | All 6 modes: TRP monostatic, TRP bistatic, Aerial UE monostatic, Aerial UE bistatic, TRP – Aerial UE bistatic, Aerial UE – TRP bistatic |  |
| Sensing target | Outdoor/indoor | Outdoor |
|  | LOS/NLOS (Tx-UAV, and UAV-Rx) | - Alt1: LOS condition - Alt2: LOS/NLOS condition based on LOS probability equation in TR36.777 |
|  | 3D mobility | horizontal plane only: - Option 1: uniform between 3 and 160 km/h - Option 2: 3 km/h, 30 km/h, 60 km/h, 160 km/h Maximum flight height: - Option 1: up to 300 m; - Option 2: up to 600m |
|  | 3D distribution | - Uniform with up to [1, 2, …, 5] targets per sector – only for the center site - Height: Uniformly distributed between 1.5m and 300m |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Option 1: [0.15m x 0.15m x 0.05m] - Option 2: [1.5m x 1.5m x 0.5m] - Option 3: [3m x 3m x 1m] RCS modelling: - FFS |
| Unintended/Environment objects | Types | FFS |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | FFS |
| Sensing area | For TRP-monostatic and TRP-TRP bistatic: - Same as center site layout For TRP- aerial UE and aerial UE- aerial UE - X meters 3-D distance around an Areal UE |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] | - Minimum distance TRP – UAV: 10 m - Minimum distance UAV – UAV: 10 m |  |

Proposal 8: Support the following updates in the Evaluation parameter table for humans sensing scenarios:

| Parameters | Value |  |
| --- | --- | --- |
| Applicable communication scenarios | UMI, UMA, InH, InF |  |
| System parameters | Carrier frequency of FR1 (3.5 GHz) & FR2 (28 GHz) - For InH, InF, for both FR1 and FR2, use as a starting point the systems parameters and values from TR 38.855 Scenario 1 (Indoor Office for FR1 and FR2 (Open Office)) in Section 6.1 and Tables 6.1.1-1, Tables 6.1.1-2, Tables 6.1.1-3 - For UMI, UMA, - for both FR1 and FR2, for Umi, use as a starting point the systems parameters and values from TR 38.855 Scenario 2 (UMi street canyon for FR1 and FR2 (ISD 200m) in Section 6.1 and Tables 6.1.1-1, Tables 6.1.1-2, Tables 6.1.1-4, Tables 6.1.1-5 - for FR1 only, for UMa, use as a starting point the systems parameters and values from TR 38.855 Scenario 3 (UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)) in Section 6.1 and Tables 6.1.1-1, Tables 6.1.1-5, Tables 6.1.1-6. |  |
| Sensing transmitters and receivers properties | BSs and UEs in the corresponding communication scenario |  |
| Supported sensing modes | All 6 modes: TRP monostatic, TRP bistatic, UE monostatic, UE bistatic, TRP –UE bistatic, UE – TRP bistatic |  |
| Sensing target | Outdoor/indoor | Outdoor (for Umi, Uma) Indoor (for InH, InF) |
|  | LOS/NLOS (Tx-target, and target-Rx) | - Option 1: LOS condition - Option 2: LOS/NLOS condition based on LOS probability equation in TR38.901 |
|  | 3D mobility | horizontal plane only: - Human velocity: 0, 3, 10 km/h |
|  | 3D distribution | - Uniform with up to [1, 2, …, 5] targets |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult: 0.5m x 0.5m x 1.75m RCS modelling: - FFS |
| Unintended/Environment objects | Types | Option 1: Human(s) Option 2: AGVs (InF) |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | For Option 1: Human - Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult: 0.5m x 0.5m x 1.75m For Option 2: AGV - Size (Length x Width x Height): - Option 1: 0.5m x 0.5m x 0.2m - Option 2: 0.9m x 0.6m x 1m |
| Sensing area | For TRP-monostatic and TRP-TRP bistatic: - All indoor deployment area For TRP-UE and UE monostatic/bistatic - X meters 3-D distance around a UE |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] | FFS |  |

Proposal 9: Support the following updates in the Evaluation parameter table for automotive vehicles sensing scenarios:

| Parameters | Value |  |
| --- | --- | --- |
| Applicable communication scenarios | Highway, Urban grid |  |
| System parameters | Reuse the parameters and values from TR 37.885 (Sections 6.1.1, 6.1.2, 6.1.3, 6.1.4, 6.1.5), with the following notes/changes: Bandwidth; 200MHz, 400 MHz, or 800 MHz. Carrier frequency; 30 GHz. Inter-BS distance; 250m, 500m. Antenna array configuration for vehicle UE; (2,4,2,1,1), or (2,4,2,8,1) - In addition to (2,4,2,1,1) in TR 37.885, antenna array configuration of (2,4,2,8,1) can be used for vehicular UE to achieve high angular resolution sensing desirable for LRR/SRR use cases. Antenna array bearing angle; 45°, 90°, 135°, or 270° for corner or side SRR use cases. Traffic model; periodic and aperiodic sensing traffic with update rate parameters (5 to 20 fps for LRR and 20 to 50 fps for SRR). |
| Sensing transmitters and receivers properties | Define percent of UEs with sensing capability only, or communication only capability, or both communication and sensing capability - An example, radar only can be 25% or 100% of the total UE drop with remaining UE as communication only |  |
| Supported sensing modes | All 6 modes: TRP monostatic, TRP bistatic, UE monostatic, UE bistatic, TRP –UE bistatic, UE – TRP bistatic |  |
| Sensing target | Outdoor/indoor | Outdoor |
|  | LOS/NLOS (Tx-target, and target-Rx) | Support both the following options: - Option 1: LOS for Highway scenario = 1 - Option 2: According to Table 6.2-1 |
|  | 3D mobility | - Option 1: 140 km/h in all the lanes - Option 2: (optional) Vehicle speed is 70 km/h or 250 km/h in all the lanes |
|  | 3D distribution | The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {r meter, an exponential random variable with the average of the speed * t sec}. Instead of a single value of 2 for both r and t (as used in TR 37.885), more values can be provided for both these parameters for LRR /SRR use cases: FFS on (r,t) values |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | all three types/sizes included in Section 6.1.2 of TR 37.885 |
| Unintended/Environment objects | Types | For Option 1: Human - Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult: 0.5m x 0.5m x 1.75m For Option 2: vehicles - all three types/sizes included in Section 6.1.2 of TR 37.885 |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | For Option 1: Human - Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult: 0.5m x 0.5m x 1.75m For Option 2: vehicles - all three types/sizes included in Section 6.1.2 of TR 37.885 |
| Sensing area | For TRP-monostatic and TRP-TRP bistatic: - All indoor deployment area For TRP-UE and UE monostatic/bistatic - X meters 3-D distance around a UE |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] | FFS |  |

Proposal 10: Support the following updates in the Evaluation parameter table for AGV sensing scenarios:

| Parameters | Value |  |
| --- | --- | --- |
| Applicable communication scenarios | InF |  |
| System parameters | Carrier frequency of FR1 (3.5 GHz) & FR2 (28 GHz) - For both FR1 and FR2, use a starting point the parameters and values for the InF-SH and InF-DH scenarios from TR 38.857 Section 6.1 |  |
| Sensing transmitters and receivers properties | BSs and UEs in the corresponding communication scenario |  |
| Supported sensing modes | All 6 modes: TRP monostatic, TRP bistatic, UE monostatic, UE bistatic, TRP –UE bistatic, UE – TRP bistatic |  |
| Sensing target | Outdoor/indoor | Indoor |
|  | LOS/NLOS (Tx-target, and target-Rx) | LOS/NLOS condition based on LOS probability equation in TR38.901 |
|  | 3D mobility | horizontal plane only: - AGV velocity: 5, 30 km/h |
|  | 3D distribution | - Uniform with up to [1, 2, …, 5] targets |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Option 1: 0.5m x 0.5m x 0.2m - Option 2: 0.9m x 0.6m x 1m RCS modelling: - FFS |
| Unintended/Environment objects | Types | Option 1: Human(s) Option 2: AGVs |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | FFS |
| Sensing area | For TRP-monostatic and TRP-TRP bistatic: - All indoor deployment area For TRP-UE and UE monostatic/bistatic - X meters 3-D distance around a UE |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] | FFS |  |

Proposal 11: Support the following updates in the Evaluation parameter table for sensing of objects creating hazards on roads/railways scenarios:

| Parameters | Value |  |
| --- | --- | --- |
| Applicable communication scenarios | Highway, Urban grid |  |
| System parameters | Reuse the parameters and values from TR 37.885 (Sections 6.1.1, 6.1.2, 6.1.3, 6.1.4, 6.1.5), with the following notes/changes: Bandwidth; 200MHz, 400 MHz, or 800 MHz. Carrier frequency; 30 GHz. Inter-BS distance; 250m, 500m. Antenna array configuration for vehicle UE; (2,4,2,1,1), or (2,4,2,8,1) - In addition to (2,4,2,1,1) in TR 37.885, antenna array configuration of (2,4,2,8,1) can be used for vehicular UE to achieve high angular resolution sensing desirable for LRR/SRR use cases. Antenna array bearing angle; 45°, 90°, 135°, or 270° for corner or side SRR use cases. Traffic model; periodic and aperiodic sensing traffic with update rate parameters (5 to 20 fps for LRR and 20 to 50 fps for SRR). |
| Sensing transmitters and receivers properties | Define percent of UEs with sensing capability only, or communication only capability, or both communication and sensing capability - An example, radar only can be 25% or 100% of the total UE drop with remaining UE as communication only |  |
| Supported sensing modes | All 6 modes: TRP monostatic, TRP bistatic, UE monostatic, UE bistatic, TRP –UE bistatic, UE – TRP bistatic |  |
| Sensing target | Outdoor/indoor | Outdoor |
|  | LOS/NLOS (Tx-target, and target-Rx) | Support both the following options: - Option 1: LOS for Highway scenario = 1 - Option 2: According to Table 6.2-1 |
|  | 3D mobility | - Option 1: 140 km/h in all the lanes - Option 2: (optional) Vehicle speed is 70 km/h or 250 km/h in all the lanes |
|  | 3D distribution | uniform |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | - Animal (Sheep/deer) - Dimensions: 1.5m x 0.5m x 1 m - velocity: 5 km/h - Pedestrian: Reuse the agreed dimensions and velocity from the “Human scenarios” - vehicle: Reuse the agreed dimensions and velocities from the “automotive scenarios” |
| Unintended/Environment objects | Types | FFS |
|  | 3D mobility | FFS |
|  | 3D distribution | FFS |
|  | Orientation | FFS |
|  | Physical characteristics (e.g., size) | FFS |
| Sensing area | For TRP-monostatic and TRP-TRP bistatic: - All indoor deployment area For TRP-UE and UE monostatic/bistatic - X meters 3-D distance around a UE |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] | FFS |  |

Proposal 12: Use the following average RCS value ranges as a starting point for the following object types:

| Object type | Average RCS |
| --- | --- |
| Type 1: UAV | [-15 to 3] dBsm when facing the UAV |
| Type 2: Human | [-10 to 0] dBsm |
| Type 3: Vehicle | [10 to 20] dBsm |

- Study how to model, for any given object, RCS dependencies on the incident and the reflection angles to the object’s orientation and on the carrier frequency

- FFS: RCS value ranges for the remaining object types after their dimensions have been clarified

- References

- RP-234069, “New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR,” Nokia, Nokia Shanghai Bell. RAN #102

- R1-2101456, “ISAC Channel modeling”, Qualcomm Incorporated, RAN1#116

- A. Herschfelt et al., "Consumer-grade drone radar crosssection and microDoppler phenomenology," 2017 IEEE Radar Conference (RadarConf), 2017, pp. 0981-0985.

- O. Bshara, Y. Liu and K. R. Dandekar, "Radar Cross Section Measurement Comparison of UAVs at C-band and V-band," 2019 IEEE 20th Wireless and Microwave Technology Conference (WAMICON), 2019, pp. 1-6