3GPP TSG RAN WG1 #117		 	R1-2404792

Fukuoka, Japan, May 20th –24th, 2024

Source:	NEC

Title:	Discussion on subband non-overlapping full duplex Tx/Rx and measurement operations

Agenda Item:	9.3.1

Document for: 	Discussion and Decision

# Introduction

In the RAN #102 meeting, WI on the evolution of NR duplex operation was approved. The objectives of the WI related to SBFD operation are listed as follows [1].

For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC CONNECTED mode [RAN1, RAN2] - Study and specify, if justified, SBFD operation to UE in RRC_IDLE/INACTIVE mode for random access [RAN1, RAN2] - RAN#104 to check whether to proceed with normative work - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators.

This contribution discusses and provides our views on SBFD Tx/Rx and measurement operations.

# Discussion

## SBFD Resource Configuration

For SBFD operation, it is assumed that a gNB can schedule concurrent transmissions for UL and DL in non-overlapped sub-bands for different UEs within a TDD carrier. Meanwhile, a UE continues to follow a half-duplex operation (i.e., receive either UL or DL but not both simultaneously). This operation is expected to improve UL coverage, reduce latency, and improve system capacity compared to legacy TDD systems. However, to successfully implement SBFD operations, various resource configuration aspects need to be clarified.

For traditional NR TDD, the time domain resources are split between downlink and uplink either by semi-static TDD configuration (e.g., tdd-UL-DL-ConfigurationCommon/tdd-UL-DL-ConfigurationDedicated) and/or by dynamic TDD configuration (e.g., SFI in DCI format 2_0). Since the TDD configuration can be provided dedicatedly to a UE, the gNB can potentially schedule DL and UL for different UEs simultaneously, achieving the functionality of SBFD without requiring any signalling enhancements.

However, there are certain advantages of indicating SBFD resources to the UEs that can support SBFD operations. Using the SBFD resource configuration, many UL-DL resource conflicts can be resolved at the UE side with relatively simple implementations. For example, semi-static DL receptions (like CSI-RS) can be ignored by the UE in the UL sub-band resources. Without the SBFD-based resource indication to the UE, the gNB would need to use DCI-based preemption/puncturing to resolve any DL/UL conflict during SBFD occasions. This would not only result in higher PDCCH overhead but also reduce UE/gNB performance in case the UEs cannot timely/correctly decode the DCI indications.

Observation 1: Full duplex operation can be currently achieved in NR by providing different tdd-UL-DL-ConfigurationDedicated or SFI to different UEs in a cell; however, higher DL/UL performance gains are expected if UEs are indicated of SBFD time/frequency resources.

In relation to configuring SBFD resources, the WID indicates the specification of time and frequency resources for SBFD operation, at least for the RRC-CONNECTED UEs. However, an indication of time/frequency resources in SIB1 is still under consideration. The main motivation for supporting indication via SIB1 for SBFD resource configuration is to allow random access procedures for RRC idle/inactive mode UEs. As discussed in our companion contribution, supporting random access procedures for RRC IDLE/INACTIVE mode UEs is beneficial and needs to be specified. Hence, SBFD resource configuration should be extended to RRC IDLE/INACTIVE mode UEs. Given this, we also need to consider the impact on legacy UEs by supporting SBFD configuration within SIB1.

Proposal 1:

- Support the indication of the time and frequency resources for SBFD using SIB1.

Further, in relation to SBFD configuration, there was a discussion in RAN1#116 about the applicability of UE-specific SBFD configuration for time and/or frequency for which the following agreement was captured.

Agreement

For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier.

- FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

One aspect which is still open for discussion is the case of SBFD frequency configuration. Some companies raised the point in RAN1#116 that per BWP, UE-specific SBFD frequency configurations are needed. This argument seems reasonable as UE only receives the frequency resources for initial DL/UL BWP from cell-specific configurations (ServingCellConfigCommon), while frequency resources for other BWPs are provided dedicatedly to the UE. Hence, if we consider this design principle, providing UL subband resources for all the BWPs (common as well as dedicated) within the cell-specific configuration seems counterintuitive. Thereby, we propose that UL subband resources should be indicated for each BWP independently, where the restriction of keeping a single UL subband within a TDD carrier can be achieved using specification clauses and/or gNB implementation.

Observation 2 Cell-specific configuration in NR only provides the frequency configuration for the initial BWP, while frequency resources for other BWPs are provided dedicatedly to the UE. Hence, indicating UL subband resources for the entire cell in cell common configuration contradicts the current design principles.

Proposal 2:

- Support configuration of UL subband resources per BWP.

  - UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

  - Discuss how to ensure the restriction of a single UL subband per TDD carrier.

There was also considerable discussion in RAN1#116 about the methodology to indicate the SBFD configuration. For determining the time periodicity of the SBFD pattern, two options were down-selected for further consideration as defined below:

- Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

The main motivation for Option 2 is to allow gNB to provide different SBFD time occasions for different TDD-UL-DL pattern periods. For instance, for a TDD UL-DL pattern period, which also coincides with SSB burst/CSI-RS transmission, the gNB may configure a smaller number of SBFD symbols than other pattern periods. This could especially be helpful when the periodicity of considered reference signals is not completely aligned with the TDD UL-DL pattern period. Further, this may also allow gNB to configure more consecutive SBFD symbols. For example, out of every 4 TDD UL-DL pattern periods, gNB can configure one such TDD UL-DL pattern period with only SBFD and UL symbols. Such a type of allocation would greatly reduce the scheduling complexity of the gNB, where gNB can schedule legacy users during TDD UL-DL pattern periods where SBFD symbols are not configured, thus improving the legacy user’s performance. Hence, there may be benefits to adopting Option 2 with respect to gNB scheduling flexibility.

The remaining open point for Option-2 is how to handle the case when there are multiple TDD UL-DL pattern periods are configured. In such a case, one simple solution (which has also been proposed by other proponents of Option 2) is to keep the SBFD pattern period as an integral multiple of a sum of individual TDD UL-DL pattern periods.

Proposal 3:

- For RRC_CONNECTED mode UEs, SBFD subband time locations are configured within a period given by

  - The SBFD pattern period is integer multiple of TDD-UL-DL pattern period when single TDD-UL-DL pattern period is configured.

  - If tdd-UL-DL-ConfigurationCommon provides both pattern1 and pattern2, the SBFD pattern period is integer multiple of sum of pattern1 and pattern2 periods.

## Support of Multiple Antenna Panels

The study item discussed the antenna configuration aspects for SBFD operation. Here, it is assumed that a gNB operates with multiple antenna panels. During SBFD occasions, if separate antenna panels are used for UL receptions and DL transmission at the gNB (i.e., one antenna panel for UL and another antenna panel for DL), the gNB self-interference can be reduced. For other time occasions (non-SBFD symbols), both antenna panels can operate with a single transmission direction (UL or DL). Such a scheme ensures efficient utilization of antenna elements and thus provides higher performance gains.

Considering that many operators and network vendors currently use single antenna panels for NR operation, spatial isolation can also be extended for single antenna panel operation. For example, a single antenna panel with L antenna elements can be visualized as two separate logical antenna panels with L/2 elements each. Then, we can use the same approach for multiple panels to reach the desired isolation. This mechanism can reduce the cost of SBFD deployment for network implementations currently supporting single antenna panel-based operation.

Figure 1 SBFD operation using a single antenna panel

Proposal 4:

- For SBFD slot/symbols, discuss the requirements of operation where Tx and Rx are performed using different sets of antenna elements belonging to a single antenna panel.

Several options have been considered for using different antenna panels for simultaneous Tx and Rx; some were captured in TR 38.858, where three such options are captured. Here, we will mainly concentrate on Option 1, which is described below for reference:

SBFD antenna configuration option-1 The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD.

Figure 2 SBFD antenna configuration option-1

Compared to other options specified in the TR, this option allows the utilization of all antenna elements even when the gNB operates in DL-only or UL-only slots/symbols. Hence, this option seems more efficient.

Observation 3: Among different options considered for SBFD operation using multiple antenna panels, Option 1 most efficiently utilizes the available antenna elements.

To support this option, we must note that the number of antenna elements used for DL is different between a DL-only symbol and an SBFD symbol; the same applies to UL also. Therefore, the transmission parameters (e.g., codebook, channel estimates) are expected to differ between SBFD symbols and UL-only/DL-only symbols. From our understanding, this is a crucial point as it might impact the CSI-RS transmission and reporting procedures to account for the differences between UL-only/DL-only symbols and SBFD symbols. For example, it is naïve to assume that channel estimates determined during a DL-only symbol can be equally applicable to SBFD symbols due to a change in the antenna elements used for transmission.

Moreover, there was a similar discussion for Network Energy Savings (NES) WI, where spatial adaptation configuration was considered. The rationale is to have different sets of antenna elements available for different spatial adaptation configurations. This is similar to the assumptions considered for SBFD multi-panel operation. Hence, similar fundamentals are also applicable (e.g., CSI-RS enhancements). Thus, it is preferable also to align the discussions as much as possible and identify the differences that should be covered for SBFD.

Observation 4: The availability of different numbers of antenna elements between DL-only and SBFD symbols is expected to cause channel estimation issues between the symbol types and, hence, may require enhancements to CSI-RS configuration and measurements.

Proposal 5:

- Leverage the Rel-18 NES enhancements on CSI measurements and reporting for spatial adaptation, to enable SBFD operation using multiple antenna panels.

Furthermore, reducing the number of antenna panels during SBFD slots/symbols would also imply that PUSCH/PUCCH decoding performance might suffer compared to UL-only slots/symbols. This can occur for various reasons (e.g., change in receiving beamforming pattern, reduction in receiving diversity) depending on how the antenna elements are used in the implementation. Hence, this signifies the requirement for more robust UL receiving procedures for SBFD slots/symbols than UL-only slots/symbols. One potential solution can be using different power control parameters between SBFD slots/symbols and UL-only slots and symbols to ensure that PUSCH/PUCCH performance remains stable irrespective of the slot/symbol type.

Observation 5: The availability of different antenna elements between UL-only and SBFD symbols is expected to cause gNB UL reception power variations between the two symbol types.

Proposal 6:

- Specify different power control parameters between SBFD and non-SBFD symbols to improve UL reception performance during SBFD symbols.

## Timing Alignment Issues for SBFD

A UE is provided a cell-specific TA offset NTA offset in a legacy TDD system. It can be provided in SIB1; if not, UE assumes a TA offset of 25600Tc or 39936Tc, as defined in Table 7.1.2-2 in 38.133. The cell-specific TA offset is used to reserve enough time for UL/DL switching for gNB, as illustrated in Figure 3 below.

Figure 3 Cell-specific TA offset operation in NR

For SBFD symbols, if UE applies this TA offset value, it will cause inter-symbol/inter-slot interference, as shown in the figure below. Several companies have indicated configuring TA offset as 0 for UL transmission performed during SBFD symbols to address this issue. However, as shown in ‘b’ of Figure 4, we can see that keeping a 0 TA offset also does not solve the problem as it allows no time margin to transition from UL to DL for the gNB. Moreover, UE may still require a gap between the DL-only symbol and the SBFD symbol to consider UE-specific TA. Hence, to solve this problem, there is a need to provide a guard period before and/or after UL transmission in an SBFD symbol (in addition to keeping TA offset as 0) to allow sufficient margin time for gNB to switch its RF components from UL to DL appropriately.

Observation 6 Even if the timing alignment offset for UL transmission is set as 0 for the UL transmissions in the SBFD symbol, gNB may not get sufficient margin to switch from UL in SBFD to DL in the next symbol.

Figure 4 Different scenarios for TA offset for SBFD

Proposal 7:

- Specify SBFD-specific timing alignment offset to mitigate inter-symbol interference.

  - A guard period may need to be defined before and/or after the UL transmission in an SBFD symbol to allow sufficient margin time for UL to/from DL transition.

## Impact on Physical Layer Procedures due to SBFD

### 2.4.1 Configured Physical Channels/Signal Transmission

In existing NR specifications, UEs assume that the entire cell/BWP bandwidth is available for UL or DL transmission direction during the indicated TDD UL/DL slots. However, this assumption is not valid for the case of SBFD, where the cell bandwidth is split into UL and DL sub-bands. For any dynamic transmission, e.g., PDSCH/PUSCH, gNB can appropriately allocate resources in the DL or UL sub-band based on existing procedure, and there is not expected to be any conflict in receiving/transmitting these channels. However, for the case of semi-static physical channels (like CSI-RS/PUCCH/SRS), existing procedures (rate matching/puncturing/preemption) are not suitable to avoid conflicts where UL (or DL) is performed by UE within DL (or UL) sub-band.

Observation 7 For semi-static physical channels, the following conflicts may occur during SBFD time/frequency resources:

a) UE may try to receive DL receptions (e.g., CSI-RS) within UL sub-bands, resulting in incorrect channel estimation or reduced DL performance.

b) UE may try to perform UL transmissions (e.g., SRS) within DL sub-bands, resulting in increased interference to nearby UE receiving DL.

Current semi-static procedures for FDRA may not work to address the above conflict for the case when the same transmission resource (e.g., CG/CSI-RS/SRS) can occur either in SBFD slots/symbols or in legacy UL-only/DL-only slots/symbols because of different radio resource available between the slot/symbol types. We should not rely on dynamic preemption indications due to frequent occurrences of SBFD slot/symbol types.

There was a discussion in RAN#112 about whether, for some of these configured transmissions, the transmissions should be restricted to SBFD symbols or non-SBFD symbols only or whether transmissions should be allowed to be performed in both SBFD and non-SBFD symbols. Please see the relevant discussion below for reference:

Agreement:

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols)

- Study the following options for SBFD-aware UEs:

  - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

  - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

- UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

  - PDSCH/PUSCH/PUCCH repetitions

  - SPS PDSCH/CG PUSCH

  - TBoMS

  - Multi-PUSCH/PDSCH scheduled by a single DCI

  - Periodic/semi-persistent SRS/CSI-RS/PUCCH

  - PDCCH

Restricting the transmission of configured signals/channels (like CSI-RS, PUCCH) to only SBFD or non-SBFD symbols will increase the signaling overhead immensely, where the network shall need to provide full configuration of these signals/channels for both SBFD symbols as well as non-SBFD symbols. Allowing these configured transmissions in both SBFD and non-SBFD would require the network to provide only delta configurations for these signals/channels to avoid any conflict scenarios, which incurs less signaling overhead. Hence, we prefer to enable the transmission/reception of configured channels to both SBFD and non-SBFD symbols.

Observation 8 Restricting the transmission of configured signals/channels (like CSI-RS, PUCCH) to only SBFD or non-SBFD symbols will increase the signaling overhead immensely where the network shall need to provide full configuration of these signals/channels for both SBFD symbols as well as non-SBFD symbols.

Proposal 8:

- For configured UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols)

  - For SBFD-aware UEs, the transmissions/receptions can be in SBFD symbols and non-SBFD symbols.

Further, to support such a procedure without causing any conflicts, we need to support the SBFD-specific radio resource configuration parameters. Such channels/signals use one set of frequency resources when using UL-only/DL-only slots/symbols and a second set for SBFD slots/symbols.

Proposal 9:

- Specify support of 2 sets of frequency resource configurations for configured UL/DL channels/signals where the first set is used during UL-only/DL-only slots/symbols and the second set is used for SBFD slots/symbols. Configured UL/DL channels/signals include at least:

  - CSI-RS

  - PUCCH

  - SRS

### 2.4.2 DCI scheduled PUSCH/PDSCH without Repetition

For FDRA Type 1 PDSCH transmissions, the following agreement was reached in RAN11#116bis

Agreement

For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS, discuss and decide whether/which of the following options is supported.

- Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping.

  - Existing RB indexing and VRB-to-PRB mapping are reused

  - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only

  - FFS: DMRS sequence mapping

- Option 1-2: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping.

  - Existing RB indexing and VRB-to-PRB mapping are reused

  - The number of PRBs for TBS determination is based on the assigned PRBs as legacy

  - FFS: DMRS sequence mapping

- Option 2: Introduce new RB indexing/PRB bundle indexing to ensure VRBs are mapped to DL usable PRBs only.

  - Existing VRB-to-PRB mapping is reused

  - Legacy TBS determination method is used

  - FFS: DMRS sequence mapping

- Option 3: Modify VRB-to-PRB mapping interleaver to ensure VRBs are mapped to DL usable PRBs only.

  - Existing RB indexing/PRB bundle indexing is reused

  - If the interleaver is not enabled, Option 1-1 or Option 1-2 is used

  - Legacy TBS determination method is used

  - FFS: DMRS sequence mapping

For Type 1 PDSCH resource allocation, the resource assignments are indicated as a contiguous number of VRBs (from a starting VRB number). When interleaving functionality is enabled, the VRBs are mapped to PRBs using the following approach:

- VRBs are grouped into virtual resource-block bundles in a contiguous manner in increasing order of VRB and bundle number.

- In a similar manner, PRBs are bundled together into PRB bundles where each PRB bundle consists of contiguous set of PRBs and PRBs are assigned to PRB bundle based on increasing PRB number and bundle number.

- The interleaving procedure is applied to map a VRB bundle index (j) to a PRB bundle index (f(j)), as shown below.

  - f (j) = rC + c

  - j = cR + r

  - r = 0, 1, …, R-1

  - c = 0, 1, …, C-1

  - R = 2

  - C = [Nbundle/R]

The above inter-leaver scheme results in the pattern where consecutive VRB bundles are spaced apart by C PRB bundles. Consider the example below, the diagram shown in Figure 5.

- Let us assume BWP bandwidth is 48 PRBs for the sake of simplicity with a bundle size of 4 (i.e. 4 RBs form a single bundle)

- In this case, we will have 12 VRB and PRB bundles for the BWP. Here, RB (VRB or PRB) indexes 0-3 are mapped to RB bundle 0, RB indexes 4-7 are mapped to RB bundle 1 and so on. The value of C is determined as 6.

- Due to the interleaving functionality, following the VRB bundle index to PRB bundle index can be derived as:

  - VRB bundle index 0 is mapped to PRB bundle index 0.

  - VRB bundle index 1 is mapped to PRB bundle index 6.

  - VRB bundle index 2 is mapped to PRB bundle index 1.

  - VRB bundle index 3 is mapped to PRB bundle index 7.

Figure 5 Interleaving example for Type 1 resource allocation

Now, if we also include the UL subband into the equation and try to determine the PRB allocation for various schemes, we can understand how each scheme is performing. For this, let us assume that the UL subband (including any guard band) is constrained from PRB bundle index 5 to PRB bundle index 9. Thus, we assume that these PRB bundles cannot be used for DL transmissions during SBFD symbols.

In this case, if we evaluate the resource assignment methodology of Option-1-1 and 1-2, we can observe that VRB bundle indices 1, 3, 5, 7, and 10 are not available for DL. Hence, if UE is allocated VRB 0 to VRB 19 for DL transmission (i.e. VRB bundle index 0 to 4), only VRB index 0, 2, 4 can be utilized, which results in PRB allocation of PRB 0 to PRB 11 (i.e. PRB bundle index 0, 1, 2). This shows that Option-1-1 and 1-2 provide no interleaving gain for resource allocations even though interleaving functionality is enabled. This effect is more pronounced when the UL subband size is comparable to or larger than the DL subband size, whereas the UL subband size gets larger than the DL subband size, the interleaving benefit is not perceivable. This example is illustrated in Figure 6.

Figure 6 Impact of UL subband on Type-1 PRB allocation based on Option-1-1/1-2

Observation 9 Option-1-1/1-2 captured for Type-1 resource allocation do not allow RB interleaving benefit when UL subband size is comparable to DL subband size.

Based on the above observation, we propose that at least when frequency interleaving is enabled for Type 1 PDSCH, Option-1-1 and Option-1-2 should not be considered for down selection (i.e. only consider Option-2 or Option-3).

Proposal 10:

- Option-1-1 and Option1-2 should not be further considered for down selection, at least for the case when interleaving is enabled for Type-1 PDSCH

For the case when interleave is not enabled, Option 1-1 can be supported. That is for TB size determination for PDSCH, assigned PRBs that fall UL subband or guardband should be subtracted. As if TBS determination is based on the assigned PRBs as legacy, the bit rate will be higher especially allocated frequency across two non-adjacent DL subband and the decoding result will be effected heavily and DL performance will be reduced.

Proposal 11:

- When interleave is not enabled, Option 1-1 can be supported for FDRA Type-1 PDSCH.

Further for Option-2 and Option-3, we need to change the assumption of available DL bandwidth for interleaving. The current VRB to PRB mapping procedure currently assumes availability of entire BWP bandwidth for DL, which is not valid for SBFD symbols. Hence, the VRB to PRB mapping rules for both Option-2 and Option-3 needs to be enhanced to also consider the available DL PRBs within an SBFD symbol.

Proposal 12:

- Discuss enhancements to VRB to PRB mapping considering the availability of DL PRBs within an SBFD symbol for Option-2 and Option-3.

Enhancement for Frequency Hopping Procedure

When we consider the case of PUSCH frequency hopping, we have similar issues as defined for an interleaving procedure, where some UL resources may not be valid after performing the hopping procedure. One option considered during RAN1#112 to address these issues is configuring an offset value specific to SBFD symbols. However, even when considering different offset-based enhancements, it is still possible that the frequency resources selected for UL transmission may fall outside the UL subband bandwidth. Consider the RSstart calculation for the intra-slot frequency below, defined in TS 38.214. A similar calculation is also defined for inter-slot hopping.

If we follow the same calculation for SBFD operation, then (RBstart +RBoffset) mode NBWP can fall outside the UL subband bandwidth, as shown in Figure 7, irrespective of the value chosen for the frequency offset value. Hence, even with frequency offset enhancement, we would still need to address this conflict.

Figure 7 Frequency hopping issues for SBFD

One potential way to address this is by adapting the RB offset value if the frequency resource falls into the other subband resource. Another option is to update the RBstart calculation to consider the UL subband bandwidth size.

Proposal 13:

- Discuss enhancements to frequency hopping offset and starting RB calculation for performing UL transmission in an SBFD symbol.

Unaligned boundaries between SBFD subband(s) and PRG

PDSCH precoder Group (PRG) can be configured with wideband, 2, or 4 PRB. When the UL subband is configured in the DL BWP, it may not be aligned with PRG; for the PRG with the size of 2/4 PRB, partial PRB in one PRG may be occupied by the UL subband or guard band. In this case, determining the used precoder matrix for the PRB in the PRG should be specified. In the SI, for SBFD-aware UE, it has been agreed that if PRG is determined as wideband, study the following two options:

- Option 1: non-contiguous frequency resources across two DL subbands, but contiguous frequency resources within each DL subband can be allocated.

- Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated.

For option 1, UE can be scheduled on two non-contiguous DL subbands, which increases the scheduling flexibility. As the difference in the channel conditions in two DL subbands may be small, different RBs in two DL subbands can use the same precoder. Further, new UE behavior, such as UE not needing to do CQI calculations for the RBs in the UL subband and guard band, can be introduced.

Proposal 14:

- If PRG is determined as wideband, support Option-1- non-contiguous frequency resources across two DL subbands, but contiguous frequency resources within each DL subband can be allocated.

  - If UE is scheduled with non-contiguous PRBs in two subbands in SBFD symbols, UE may assume that the same precoding is applied to the allocated resources associated with the same TCI state or the same QCL assumption in the non-contiguous PRBs of the two DL subbands.

### 2.4.2 SPS/Configured Grant without Repetition

For SPS and configured grant transmissions, the following agreement was reached in RAN1#116bis

Agreement

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate resource allocations for SBFD symbols and non-SBFD symbols

  - FFS other separate configurations for SBFD symbols and non-SBFD symbols

- Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: An SPS PDSCH reception occasion overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid

- Option 4: Only SPS PDSCH reception occasion in one symbol type is valid and SPS PDSCH reception occasion in the other symbol type is invalid

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid for SPS PDSCH

- Other options are not precluded

For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate resource configurations for SBFD symbols and non-SBFD symbols

  - FFS type 2 CG PUSCH

  - FFS other separate configurations for SBFD symbols and non-SBFD symbols

- Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: A CG PUSCH transmission occasion overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid

- Option 4: Only CG PUSCH transmission occasion in one symbol type is valid and CG PUSCH transmission occasion in the other symbol type is invalid

- Option 5: Only the assigned PRBs within UL usable PRBs in SBFD symbols are considered to be valid for CG PUSCH

- Other options are not precluded

When we consider SPS, these resources are generally used in network implementations to support VoIP downlink traffic, which is expected to have well-defined periodicity and packet size. Hence, when network provides SPS resources to the UE it provides the resources according to the expected VoIP packet size. For such a scenario, allocating partial resources to SPS during SBFD symbols (as indicated in Option-5) may not guarantee transmission of entire VoIP packet. Hence, we think that Option 5 should not be considered further for SPS transmission.

Further, we think that SPS periodicity is generally quite relaxed (10ms, 20ms, etc.), which is expected to be aligned with the SBFD pattern period. For such a case, if an SPS occasion at time T1 occurs during non-SBFD symbol then we can expect that all other SPS occasions to also fall on non-SBFD symbols only (and same goes if SPS occasion falls on SBFD symbols). Hence, the need for more complicated procedure like Option-1 and Option-2 may not provide any practical gains.

Hence, we think that either of Option-3 or 4 should be selected for SPS transmissions. And if there is any instance where SPS occasion is not available (e.g. due to misaligned SPS periodicity and SBFD pattern period) then network can handle those occasions by using DCI based PDSCH scheduling.

Proposal 14:

- For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols, down-select between Option-3 and Option-4.

In the case of a configured grant, the situation is a little different, where CG resources can also be used to support delayed sensitive data like XR or URLLC. And, when CG resources are used for VoIP UL traffic, the absence of CG resources can lead to higher power consumption for UEs, as UEs would need to request SR for periodic data transmissions. Considering both scenarios, it is important to select a solution that can guarantee radio resources for CG irrespective of the symbol type. But again, we should avoid Option 5, which does not guarantee a minimum UL resource guarantee for CG occasions. Hence, for the case of CG, either of Option-1 or Option-2 should be considered.

Proposal 15:

- For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols, down-select between Option-1 and Option-2.

### 2.4.3 PDSCH/PUSCH with Repetitions

The multi-slot PUSCH/PDSCH transmission includes the PUSCH/PDSCH repetition transmission and one DCI scheduling multiple PUSCH/PDSCH transmission and TBoMS cases. For multi-slot PUSCHs scheduled by a single DCI, UE determines whether the corresponding PUSCH is transmitted based on the time domain resource allocation of the PUSCH and TDD configuration. If the PUSCH overlaps with an invalid symbol, e.g., the DL symbol configured by tdd-UL-DL-ConfigurationCommon/tdd-UL-DL-ConfigurationDedicated, the PUSCH transmission is canceled. Suppose SBFD operation is configured, as shown in Figure 8, where multiple PUSCHs scheduled by a single DCI are indicated to be transmitted cross symbols/slots with different duplex types. In that case, e.g., for UL symbols/slots only and SBFD symbols/slots with UL sub-bands, how to perform the UL transmission needs to be clarified.

Figure 8 Example for UL transmission scheduled on SBFD symbol with UL sub-band

RAN1#116bis discussed aspects for PUSCH and PDSCH repetitions where different repetitions can span SBFD and non-SBFD symbols. Following agreement was reached for the same.

Agreement

For PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

- Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: A PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid, e.g. the PDSCH in the slot is dropped

- Option 4: Only PDSCH in one symbol type is valid and PDSCH in the other symbol type is invalid

- Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid

- Option 6: gNB does not schedule any PDSCH in SBFD symbols in a slot to be overlapping with PRBs outside DL usable PRBs

- Other options are not precluded

- FFS: Applicable conditions

Agreement

For PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols, and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

- Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid, e.g. the PUSCH in the slot is dropped/postponed

- Option 4: Only PUSCH in one symbol type is valid and PUSCH in the other symbol type is invalid

- Option 5: For a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols, only the assigned PRBs within UL usable PRBs are considered to be valid

- Option 6: gNB does not schedule any PUSCH in SBFD symbols in a slot to be overlapping with PRBs outside UL usable PRBs

- Other options are not precluded

- FFS: Applicable conditions

The options can mainly be seen to belong to 2 separate groups: Allow repetitions across SBFD and non-SBFD symbols (Option-1, 2, 3, 5, 6) or not allow repetitions across SBFD and non-SBFD symbols (Option-4)

Given that SBFD is mainly introduced to improve the UL coverage aspects, we must allow multi-slot PUSCH to use the SBFD slots for UL transmission even if the span of multi-slot PUSCH may contain both UL-only and SBFD slots. For instance, if a PUSCH transmission with eight repetitions is scheduled, which starts from a UL slot and if there is only a single UL slot per TDD pattern period, then it is more efficient to allow transmission of the remaining PUSCH repetitions over the SBFD symbols rather than cancelling the repetitions altogether.

However, it might also make a network scheduler implementation much more complex, especially when the network needs to schedule multiple PUSCHs with repetitions (addressed to different UEs) and UL subband bandwidth is limited and may not have sufficient resources to schedule all PUSCHs. Hence, we think that we should enable additional control at the network side to enable or disable the operation of transmission of PUSCH (or PDSCH) repetitions across different symbol types.

Observation 10 Allowing PUSCH/PDSCH repetitions to span both SBFD and non-SBFD symbols can improve the coverage performance but having network control to enable/disable this functionality can significantly relax the gNB scheduler complexity.

Proposal 16:

- Multi-slot PUSCH/PDSCH spanning SBFD and UL-only/DL-only slots can be transmitted on both SBFD slots and UL-only/DL-only slots.

  - Additional network control signaling is required to indicate UE whether the PUSCH/PDSCH repetitions are valid across both SBFD and non-SBFD symbols.

This implies that at least Option 4 should be supported for both PDSCH and PUSCH repetition based on network indication.

Proposal 17:

- For PDSCH (or PUSCH) repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PDSCH (or multi-PUSCH) scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH (or PUSCH) within a slot has either all SBFD or all non-SBFD symbols, network can indicate to the UE that PDSCH (or PUSCH) is valid for only one symbol type (SBFD or non-SBFD).

Furthermore, when the network wants to schedule PDSCH/PUSCH repetitions across SBFD and non-SBFD symbols, then we need to select between one of the Option-1 2, 3, 5 and 6. Note that Options 3, 5, and 6 assume that UE can continue to use the same set of radio resources between SBFD and non-SBFD symbols. Such an assumption can again create too many restrictions on the gNB scheduler because the gNB scheduler would need to ensure resource selection such that the resources do not conflict with any other physical channel (e.g. PUCCH) whose resource allocations are also expected to vary between SBFD and non-SBFD symbols. For example, PUCCH resources are generally allocated near the edge of the UL bandwidth (i.e. edge of BWP for UL symbols and edge of UL subband for SBFD symbols); therefore, there is a high likelihood that PUSCH repetitions scheduled using Option-5 within SBFD symbols can frequently collide with the PUCCH resources which may be configured at the edge of UL subband. Hence, we prefer to keep more control at the network side in the selection of radio resources during both SBFD and non-SBFD symbols and hence prefer to support either Option-1 or Option-2.

Proposal 18:

- For PDSCH (or PUSCH) repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PDSCH (or multi-PUSCH) scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH (or PUSCH) within a slot has either all SBFD or all non-SBFD symbols, if network indicates to UE that PDSCH (or PUSCH) can be transmitted over both SBFD and non-SBFD symbols then down select between:

  - Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

  - Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

### 2.4.3 PDCCH/CORESET

The frequency allocation issues are also applicable to the CORESET configuration. Although CORESET configuration in NR is flexible, where the network can configure the frequency resources to be used for a CORESET, there is still a need for enhancement in CORESET frequency allocation for the case of SBFD. Without any enhancements, the network vendor must always configure the CORESET frequency resources within the DL subband portion to avoid CORESET overlap with the UL subband. This is inefficient as the network will not be able to utilize the frequency resources within the UL subband portion during non-SBFD slots, reducing the PDCCH capacity. From our understanding, we can increase CORESET efficiency by defining SBFD-specific and non-SBFD-specific CORESET configurations. UE selects the CORESET frequency resources depending on the symbol type. For example, two CORESET/SS groups/sets can be configured for a UE. A new parameter, such as CORESETGroupIdList, can list CORESET/SS for each CORESET/SS group. The CORESET/SS in the first CORESET/SS group with the allocated frequency resource can overlap with the UL subband and guardband, and the CORESET/SS in the second CORESET/SS group with the allocated frequency resource cannot coincide with the UL subband and guardband. UE can switch or adapt the PDCCH monitoring based on the DCI indication or the symbol type.

Proposal 19:

- Support of separate frequency resource configurations for CORESET in SBFD symbols and non-SBFD symbols for UE-specific search space.

Further, we need to discuss how an UE determines which CORESET configuration to use for receiving PDCCH when a slot may consist of both SBFD and non-SBFD symbols. This is especially relevant when a CORESET consists of more than one symbol, which may map to both SBFD and DL-only symbols, as shown in the figure below.

Proposal 20:

- Discuss the UE behavior for selecting the CORESET configuration for PDCCH monitoring in a slot consisting of SBFD and DL-only symbols.

Figure 9 CORESET overlapping with SBFD and DL-only symbols within a slot

However, such enhancements may not be applicable for CORESETs associated with common search space, which is shared with legacy UEs. For such CORESETs, we can only rely on legacy CORESET configuration and may need to specify gNB/UE behavior to transmit/receive PDCCH during SBFD operation. A more straightforward solution would be restricting the network configuration such that CORESET associated with common search space is always configured within the DL subband.

Proposal 21:

- SBFD-aware UEs shall not expect the CORESET associated with common search space to have resources outside the DL subband in an SBFD symbol.

### 2.4.4 PUCCH

For PUCCH, how to configure the PUCCH resource so that it does not overlap with the DL subband in SBFD symbols should also be discussed. There are two options for PUCCH configuration for SBFD symbols.

Option1: The configured PUCCH resource set and PUCCH resource list in each set is common or shared between SBFD symbols and non-SBFD symbols, and additional configurations (such as power control, FDRA, PHY priority, spatialRelationInfo) or offset parameters are added for each PUCCH resource ID used for PUCCH transmission on SBFD symbols.

Option 2: A PUCCH resource set, such as PUCCH-ResourceSet-SBFD, can be configured for the UE applicable during SBFD symbols, and a list of PUCCH resources is included in this set for UE to determine the PUCCH resource on the SBFD symbols.

For each of the options, how to handle PUCCH repetitions that occur over SBFD and non-SBFD symbols should be discussed. For example, for PUCCH repetition transmission, two allocation parameters set (including TPC, PRI, and beam) can be indicated within the DCI/RRC for PUCCH transmission on SBFD symbols and non-SBFD symbols; otherwise, some kind of deferring or dropping rule should be defined if the SBFD resource is not available or overlapped with DL subband for the duration of the PUCCH repetition transmission.

Further, for option 2, if separate PUCCH resources are configured for SBFD symbols, then some enhancements for the UCI multiplexing and HARQ-ACK codebook generation can also be considered for SBFD symbols. For instance, for DL symbols configured as SBFD symbols, the candidate PDSCH occasions on SBFD symbols are still counted as candidate occasions for candidate PDSCH receptions according to existing specifications and one position in the HARQ codebook is reserved for each PDSCH occasion. This can be wasteful of HARQ resources if UE is expected to perform UL during these candidate PDSCH occasions. Therefore, we need to discuss whether the PDSCH occasions on SBFD symbols should be considered when constructing a type-1 codebook.

For the above, UE ACK/NACK feedback (padding NACK or feedback the ACK/NACK) can be based on the collision cases for DL reception and the UL transmission in SBFD symbols. Besides, based on the predefined priority rule of the SPS PDSCH and the configured UL transmission, UE can determine whether these SBFD symbols are counted as candidate PDSCH occasions when constructing the type-1 codebook and determining the type-1 HARQ-ACK codebook size.

Proposal 22:

- If separate PUCCH resources are configured for SBFD symbols, the following aspects should be considered.

  - UCI multiplexing and HARQ-ACK codebook generation

### 2.4.5 CSI-RS

For CSI-RS, two issues were discussed in the SI phase; one is related to CSI-RS frequency configuration considering UL subband frequency resources and the other is related to CSI report transmission during SBFD operation.

For CSI-RS resource configuration, the argument is that due to UL subband placement, a set of frequency resources may not be available for CSI-RS transmission. Still, a UE may assume that CSI-RS resources are continuous in the frequency domain. To address this problem, two options are being discussed to enhance the CSI-RS configuration:

  - Option 1: Two contiguous CSI-RS resources that are linked

  - Option 2: One CSI-RS resource

    - Option 2-1: Non-contiguous CSI-RS resource allocation

    - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

Having one CSI-RS resource for the two DL subbands avoids the complicated discussion of the UE capability on the number of CSI-RS resources that UE can measure during SBFD symbols. Hence, we support Option 2 to make the discussion simple. Also, between Option-2-1 and Option-2-2, we slightly prefer Option-2-2 as the UE is already expected to be aware of DL subband frequency resources; hence, Option-2-2 requires minimal specification change.

Proposal 23:

- Support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s) as a baseline.

Further, for the CSI report, the CSI/PMI values included within the CSI report when the corresponding measurement was performed during the SBFD symbol should only include the measurements corresponding to DL subband resources to reduce CSI report overhead. Hence, the UE should transmit the CSI report by removing any entries corresponding to the UL subband when CSI measurement is performed during SBFD symbols. If UE and gNB are in sync on the SBFD symbol location, we understand that gNB would be aware of the CSI transmitted by the UE, and hence, no other enhancement would be required.

Proposal 24:

- UE should only include the reporting quantities (e.g., CSI/PMI) associated with the DL subband resources when the corresponding CSI measurements are performed during the SBFD symbol.

### 2.4.6 SRS

SRS is expected to be used for CLI measurement with SBFD operation. Like CSI-RS and PUCCH, the configuration for SBFD symbols for SRS transmission should be discussed. Considering the CLI influence on SBFD symbols, we may require different SRS configurations/beams during SBFD as compared to UL-only symbols, such as the time and frequency resource, the frequency hopping pattern, the power control parameters, the beam/spatial relation, etc. Besides, for DCI scheduling PUSCH and SRS, the associated SRS resources should be determined based on the symbol type (i.e., SBFD vs. UL-only).

Furthermore, the available slot concept defined for SRS needs to be clarified for SBFD operation. The available slot concept was introduced for SRS repetition transmission, where the SRS can be transmitted for 1, 2, and 4 repetitions configured by the repetition factor parameter. The available slot is not counted on the DL slot in the existing spec, which may be used for SBFD purposes. This counting method must be updated if the UL subband on SBFD symbols is configured/indicated for SRS transmission. Therefore, the specified aspects for SRS within SBFD operation should include at least the following aspects.

- For SRS transmission on SBFD symbols, the configuration of the resource should be considered.

- How to associate the PUSCH and SRS resources for SRI in the DCI within SBFD and non-SBFD symbols.

- How to address the SRS transmission collision with the DL reception(s) on SBFD symbols should be considered.

- The counting method for SRS repetition transmission must change if SRS can be transmitted on the UL subband in SBFD symbols.

Proposal 25:

- Discuss potential enhancements of SRS for SBFD operation.

### 2.4.7 Physical Channel Tx/Rx Spanning SBFD and non-SBFD symbols

During the study item, RAN1 discussed the issues related to physical channel/signal transmissions spanning both SBFD and non-SBFD symbols within a slot. There were several serious issues raised regarding allowing UE to Rx/Tx, such as channel/signal, including issues related to phase continuity, handling guard periods, impact on link adaptation, channel estimation, and power control. While RAN1#116 did make an agreement about the composition of the slot where it has been agreed that a slot can consist of both SBFD and non-SBFD symbols, it is still under discussion whether a Tx/Rx occasion can be supported which spans both SBFD and non-SBFD symbols.

The RAN1 agreement for slot composition was mainly driven based on the requirement of current NR specification, which allows a slot to be composed of different symbol types, and partly due to the requirement of transmission of PDCCH, where PDCCH transmission can be performed on legacy DL symbol, and then SBFD symbols can be immediately started to schedule actual PDSCH/PUSCH transmission. It is important to note that the given agreement does not imply that we are going to witness frequent occurrences of PDSCH/PUSCH occasions spanning both SBFD and non-SBFD symbols. Besides, the RAN1#116 agreement of configuring SBFD occasions in consecutive symbols reduces the chances of such occurrences. Hence, from our understanding, there is no significant need to address the scenario where a physical channel Tx/Rx occasion may span both SBFD and non-SBFD symbols.

Therefore, to simplify the discussion, we propose that if there is any physical channel Tx/Rx occasion that spans both SBFD and non-SBFD symbols, then the given Tx/Rx occasion shall be dropped by the UE.

Proposal 26:

- For a physical channel/signal with transmission/reception occasion mapped to SBFD and non-SBFD symbols within a slot, SBFD aware UE does not transmit or receive the physical channel/signal within the slot.

### 2.4.8 Collision Handling

Lastly, we also need to study the UE behavior for SBFD slots/symbols, where UEs can potentially receive both UL and DL transmissions. For example, the UE may receive SPS PDSCH, where a PUSCH is also scheduled a PUSCH on this resource. Note that many conflicts like these can be addressed using careful network configurations. Still, there may be some scenarios where a configured DL (or UL) transmission can conflict with DCI scheduled UL (or DL) transmission. We must identify these scenarios and specify UE behavior to resolve these conflicts properly.

RAN1 agreed to define different scenarios to study for this collision resolution, as indicated below.

- Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

  - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

- Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

  - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

- Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

- Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

- Case 5: SSB vs. dynamically scheduled or configured UL transmission

  - e.g., PUSCH, PUCCH, PRACH, SRS

- Case 6: Dynamic or semi-static DL vs. valid RO

For Case-1 and Case-2 following agreements have been reached in RAN1#116bis

Agreement

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal.

- The above does not imply link direction indication is supported

- FFS on dynamically scheduled UL transmission with repetition

Agreement

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles and timeline in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UL transmission is cancelled if cancellation timeline is met.

- The above does not imply link direction indication is supported

- FFS on dynamically scheduled DL reception with repetition

The remaining discussion is about the FFS point (i.e. handling of UL transmission/DL reception with repetitions). However, during the online discussion for Case-1, it was agreed to prioritize dynamic DL transmission even when UE may have high-priority UL traffic because it was argued that the network may decide not to schedule any dynamic DL transmission during time instances of high-priority CG traffic. However, when we consider the case of DL repetitions (especially large repetitions like 8), then there is a high possibility that the network may not always be able to avoid conflict between dynamic DL repetitions and CG traffic. In such a case, if DL transmission is prioritized, then the UE may miss the CG transmission opportunity, leading to increased latency for high-priority traffic. Hence, we think that when considering multiple DL repetitions, DL transmission should only be prioritized when there is no high-priority UL data available for transmission at the UE side. Otherwise, UE shall prioritize the UL channel.

Proposal 27:

- If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs and for dynamically scheduled DL reception with repetition, UE shall prioritize UL transmission over DL repetition when high priority UL data is available at UE for transmission, otherwise UE prioritizes DL transmission.

Similarly, in the case of UL repetitions, the repetitions can be configured to the UE for transmission of low-latency UL data. In this case, it is possible that UE may not utilize all the UL repetitions available to it. If UE decides not to use one or more UL repetitions, then it may be beneficial for UE to receive DL reception.

Proposal 28:

- If link direction indication is not supported nor provided for an SBFD symbol, for collision Case 2 (dynamically scheduled DL reception vs semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs and for dynamically scheduled UL transmission with repetition, UE shall prioritize DL transmission over UL repetition when UE decides to skip the given UL repetition occasion. Otherwise, UE prioritizes UL transmission.

When considering the case between dynamic/semi-static DL and a RACH occasion, it seems preferable to prioritize RACH. This is because RACH is generally required when UE loses UL sync with the network, and hence, UE performing DL reception (other than SSB) is pointless as UE wouldn’t anyways be able to provide any UL feedback (like HARQ) to the network. This necessitates RACH transmission over another DL Rx. Hence, the following is proposed.

Proposal 29:

- For SBFD-aware UE transmission and reception in an SBFD symbol, support following for link direction determination for Case-6.

  - UE prioritizes RACH transmission over dynamic or semi-static DL reception.

In relation to supporting UL transmission within SSB symbols, an agreement was made in RAN1#113:

Agreement

An UL subband can be configured in an SSB symbol.

- Note: It is SSB from serving cell perspective, which can be CD-SSB or NCD-SSB.

- Whether actual UL transmission can be done is for further discussion

Agreement

The following conclusion is to be captured in the TR

If SBFD-aware UEs are not allowed to transmit in the SSB symbol but are allowed to receive within the DL BWP in the SSB symbol, a negative impact on SSB detection and measurement can be avoided, but UL performance may be degraded due to fewer UL opportunities.

If SBFD-aware UE is allowed to transmit in the SSB symbol, the UE may only transmit UL in a UL subband depending on gNB scheduling, configuration, UE measurement or priority rule. There may be a negative impact on SSB detection and measurement if the SBFD-aware UE is requested to transmit in the SSB symbol.

Supporting configuration of SBFD occasions during SSB symbols allows efficient utilization of SBFD resources.

Given that SSB periodicity (20/40ms) is expected to be less frequent than TDD periodicity (<=10ms), if we configure SBFD symbols to avoid SSB occasions, we might end up with a significant number of DL slots/symbols where SBFD operation cannot be performed. Consider the example below:

- SSBs are transmitted within a time window of 2ms with a repetition of 20ms

- TDD periodicity of 5ms with DDDUU configuration (15kHz SCS)

- In this case, if we try to avoid SSB occasions, then we end up with only 4ms duration out of 20ms where the SBFD symbols can be configured, leaving a duration of 8ms of DL symbols which cannot be used for SBFD (refer to Figure 10)

Figure 10 SBFD time occasion configuration to avoid SSB occasions.

The above example indicates that many DL symbols cannot be utilized for SBFD operation if SBFD configuration is not allowed during SSB symbols. Hence, the configuration of the UL subband during SSB occasions allows a higher number of potential SBFD symbols.

The main conflict for the discussion is whether the UE should be allowed to transmit UL when UL transmission partially/fully overlaps with an SSB occasion. Allowing UL transmission during SSB symbols requires the network to determine whether a UE tries to receive an SSB during an SSB occasion. However, it is naïve to assume that the network can fully determine the UE behavior for SSB reception.

In some network implementations, it may be possible for the network to determine the occasions where the UE tries to receive the SSB. This could be the case where the network configures an intra-frequency measurement gap for UEs to complete SSB measurements. The network implicitly understands that UE will measure SSB only during measurement gap occasions. However, such types of network implementation are not universally applicable or mandated. Many network implementations still operate without configuring an intra-frequency measurement gap and may continue to do so in the future. In such cases, the occasions where UE tries to receive the SSBs are determined primarily based on UE's implementation, and the network has no/minimal knowledge of the occasions determined by the UE for SSB reception.

Further, allowing UL reception along with SSB transmission may also require complex network implementations where the network needs to ensure there are no changes in the SSB transmission characteristics between SBFD and non-SBFD symbols. This may be an issue for implementations where SSB transmission is performed using a full set of antenna elements, and RU utilizes only half of the antenna elements for DL transmission during SBFD symbols. In such a case, performing SSB transmission during SBFD symbols requires a change in antenna configuration for SSB transmission and, subsequently, SSB transmission characteristics. Therefore, considering the above set of arguments, we are okay with studying SBFD operation during SSB symbols. However, the network should have control of the occasions/scenarios where UE is allowed to perform UL transmission during SSB symbols.

Proposal 30:

- Support gNB in informing the UE whether the UE can use the SBFD occasion, which partially/completely overlaps with the SSB occasion for a UL transmission.

Further, predefined priorities can be considered when handling the conflict between remaining cases like semi-static UL and semi-static DL. For example, the predefined priority of SPS PDSCH and the configured UL transmission can be as follows:

- SPS PDSCH has a lower priority than PUCCH/PUSCH, which is carrying UCI without repetition.

- SPS PDSCH has a lower priority than Type-1 CG PUSCH in carrying URLLC traffic.

- SPS PDSCH has lower priority than PRACH/Msg3/MsgB.

- SPS PDSCH has lower priority than SRS.

- SPS PDSCH has higher priority than PUCCH/PUSCH, which carries UCI with repetition.

- SPS PDSCH has higher priority than Type-1 CG PUSCH carrying URLLC repetition transmission

- SPS PDSCH has higher priority than Type-1 CG PUSCH carrying eMBB traffic.

Proposal 31:

- The predefined priority of SPS PDSCH and the configured UL transmission given above can be supported.

We also need to consider the case of DMRS bundling. If TDW for DMRS bundling overlaps with the SBFD symbols, how to address the case should be discussed. CLI-avoiding schemes, such as power control or beam switching, may be performed in the SBFD slot, which leads to the problem of power consistency and phase continuity not being maintained across SBFD slots/symbols and non-SBFD slots/symbols. Therefore, the actual TDW cannot cross the boundary between SBFD and non-SBFD slot, and an event that PUSCH/PUCCH over multiple slots transmission transitions to/from the SBFD symbol can be defined where, if the nominal TDW for PUCCH/PUSCH multiple slot transmission is interrupted by an SBFD symbol, the nominal TDW will be separated into at least two actual TDWs.

Proposal 32:

- Specify DMRS bundling enhancement when the TDW overlaps with SBFD symbols.

### 2.4.9 SSB considerations

During SBFD PRACH, the location of the SSB block can lead to a variety of issues. When considering SBFD reading devices reading SSB located in the legacy UE device bandwidth, this may lead to the SSB falling outside of the usable DL PRB region. This would suggest that dedicated SSBs may be required for SBFD devices in order to complete PRACH in SBFD slots. These SBFD devices may have different configurations when compared to legacy UEs, so dedicated SBFD SSB will be required. The additional SBFD configurations proposed in meetings may lead to PBCH running out of room within the current configurations when we consider an SBFD slot where a ratio (α) of all blocks is classified as DL usable PRBs. The current SSB configurations allow the reading for two types of SSB blocks (12 and 20 RB in size) depending on the configurations. This will lead to an issue of SSB not fitting within the DL subband of SBFD if the α value is not big enough. This is particularly the case when SCS is large or when SBFD is operating within low BW regions like 3MHz BW.

Proposal 33:

- Separate SSBs for SBFD devices are required

Proposal 34:

- When determining the size of DL usable PRBs, considerations need to be made to ensure the SSB is able to fit within the subbands.

### 2.4.10 RSRP threshold value considerations

The indications of the SBFD status of neighbouring gNBs can be useful not only for CLI measurements but can also affect the gNB's measurement characteristics. When neighbouring gNBs are in UL mode, the RSRP of the reference signal may be lower due to the lower number of simultaneously transmitting sources. As such, the reference RSRP levels for RS measurements need to be adjusted accordingly depending on the SBFD configurations.

Proposal 35:

- When issuing RSRP threshold levels, the current SBFD status needs to be considered for measuring RS in SBFD.

# Conclusion

In this contribution, we discussed SBFD for NR duplex operation. Observations and Proposals are summarized as follows:

Observation 1: Full duplex operation can be currently achieved in NR by providing different tdd-UL-DL-ConfigurationDedicated or SFI to different UEs in a cell; however, higher DL/UL performance gains are expected if UEs are indicated of SBFD time/frequency resources.

Observation 2 Cell specific configuration in NR only provides the frequency configuration for initial BWP, while frequency resources for other BWPs are provided dedicatedly to the UE. Hence, indicating UL subband resources for the entire cell in cell common configuration contradicts the current design principles.

Observation 3: Among different options considered for SBFD operation using multiple antenna panels, Option 1 most efficiently utilizes the available antenna elements.

Observation 4: The availability of different numbers of antenna elements between DL-only and SBFD symbols is expected to cause channel estimation issues between the symbol types and, hence, may require enhancements to CSI-RS configuration and measurements.

Observation 5: The availability of different antenna elements between UL-only and SBFD symbols is expected to cause gNB UL reception power variations between the two symbol types.

Observation 6 Even if the timing alignment offset for UL transmission is set as 0 for the UL transmissions in the SBFD symbol, gNB may not get sufficient margin to switch from UL in SBFD to DL in the next symbol.

Observation 7 For semi-static physical channels, the following conflicts may occur during SBFD time/frequency resources:

Observation 8 Restricting the transmission of configured signals/channels (like CSI-RS, PUCCH) to only SBFD or non-SBFD symbols will increase the signaling overhead immensely where the network shall need to provide full configuration of these signals/channels for both SBFD symbols as well as non-SBFD symbols.

Observation 9 Option-1-1/1-2 captured for Type-1 resource allocation do not allow RB interleaving benefit when UL subband size is comparable to DL subband size.

Observation 10 Allowing PUSCH/PDSCH repetitions to span both SBFD and non-SBFD symbols can improve the coverage performance but having network control to enable/disable this functionality can significantly relax the gNB scheduler complexity.

Proposal 1:

- Support the indication of the time and frequency resources for SBFD using SIB1.

Proposal 2:

- Support configuration of UL subband resources per BWP.

  - UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

  - Discuss how to ensure the restriction of a single UL subband per TDD carrier.

Proposal 3:

- For RRC_CONNECTED mode UEs, SBFD subband time locations are configured within a period given by

  - The SBFD pattern period is integer multiple of TDD-UL-DL pattern period when single TDD-UL-DL pattern period is configured.

  - If tdd-UL-DL-ConfigurationCommon provides both pattern1 and pattern2, the SBFD pattern period is integer multiple of sum of pattern1 and pattern2 periods.

Proposal 4:

- For SBFD slot/symbols, discuss the requirements of operation where Tx and Rx are performed using different sets of antenna elements belonging to a single antenna panel.

Proposal 5:

- Leverage the Rel-18 NES enhancements on CSI measurements and reporting for spatial adaptation, to enable SBFD operation using multiple antenna panels.

Proposal 6:

- Specify different power control parameters between SBFD and non-SBFD symbols to improve UL reception performance during SBFD symbols.

Proposal 7:

- Specify SBFD-specific timing alignment offset to mitigate inter-symbol interference.

  - A guard period may need to be defined before and/or after the UL transmission in an SBFD symbol to allow sufficient margin time for UL to/from DL transition.

Proposal 8:

- For configured UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols)

  - For SBFD-aware UEs, the transmissions/receptions can be in SBFD symbols and non-SBFD symbols.

Proposal 9:

- Specify support of 2 sets of frequency resource configurations for configured UL/DL channels/signals where the first set is used during UL-only/DL-only slots/symbols and the second set is used for SBFD slots/symbols. Configured UL/DL channels/signals include at least:

  - CSI-RS

  - PUCCH

  - SRS

Proposal 10:

- Option-1-1 and Option1-2 should not be further considered for down selection, at least for the case when interleaving is enabled for Type-1 PDSCH

Proposal 11:

- When interleave is not enabled, Option 1-1 can be supported for FDRA Type-1 PDSCH.

Proposal 12:

- Discuss enhancements to VRB to PRB mapping considering the availability of DL PRBs within an SBFD symbol for Option-2 and Option-3.

Proposal 13:

- Discuss enhancements to frequency hopping offset and starting RB calculation for performing UL transmission in an SBFD symbol.

Proposal 14:

- If PRG is determined as wideband, support Option-1- non-contiguous frequency resources across two DL subbands, but contiguous frequency resources within each DL subband can be allocated.

If UE is scheduled with non-contiguous PRBs in two subbands in SBFD symbols, UE may assume that the same precoding is applied to the allocated resources associated with the same TCI state or the same QCL assumption in the non-contiguous PRBs of the two DL subbands.

Proposal 15:

- For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols, down-select between Option-1 and Option-2.

Proposal 16:

- Multi-slot PUSCH/PDSCH spanning SBFD and UL-only/DL-only slots can be transmitted on both SBFD slots and UL-only/DL-only slots.

  - Additional network control signaling is required to indicate UE whether the PUSCH/PDSCH repetitions are valid across both SBFD and non-SBFD symbols.

Proposal 17:

- For PDSCH (or PUSCH) repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PDSCH (or multi-PUSCH) scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH (or PUSCH) within a slot has either all SBFD or all non-SBFD symbols, network can indicate to the UE that PDSCH (or PUSCH) is valid for only one symbol type (SBFD or non-SBFD).

Proposal 18:

- For PDSCH (or PUSCH) repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PDSCH (or multi-PUSCH) scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH (or PUSCH) within a slot has either all SBFD or all non-SBFD symbols, if network indicates to UE that PDSCH (or PUSCH) can be transmitted over both SBFD and non-SBFD symbols then down select between:

  - Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

  - Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

Proposal 19:

- Support of separate frequency resource configurations for CORESET in SBFD symbols and non-SBFD symbols for UE-specific search space.

Proposal 20:

- Discuss the UE behavior for selecting the CORESET configuration for PDCCH monitoring in a slot consisting of SBFD and DL-only symbols.

Proposal 21:

- SBFD-aware UEs shall not expect the CORESET associated with common search space to have resources outside the DL subband in an SBFD symbol.

Proposal 22:

- If separate PUCCH resources are configured for SBFD symbols, the following aspects should be considered.

  - UCI multiplexing and HARQ-ACK codebook generation

Proposal 23:

- Support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s) as a baseline.

Proposal 24:

- UE should only include the reporting quantities (e.g., CSI/PMI) associated with the DL subband resources when the corresponding CSI measurements are performed during the SBFD symbol.

Proposal 25:

- Discuss potential enhancements of SRS for SBFD operation.

Proposal 26:

- For a physical channel/signal with transmission/reception occasion mapped to SBFD and non-SBFD symbols within a slot, SBFD aware UE does not transmit or receive the physical channel/signal within the slot.

Proposal 27:

- If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs and for dynamically scheduled DL reception with repetition, UE shall prioritize UL transmission over DL repetition when high priority UL data is available at UE for transmission, otherwise UE prioritizes DL transmission.

Proposal 28:

- If link direction indication is not supported nor provided for an SBFD symbol, for collision Case 2 (dynamically scheduled DL reception vs semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs and for dynamically scheduled UL transmission with repetition, UE shall prioritize DL transmission over UL repetition when UE decides to skip the given UL repetition occasion. Otherwise, UE prioritizes UL transmission.

Proposal 29:

- For SBFD-aware UE transmission and reception in an SBFD symbol, support following for link direction determination for Case-6.

  - UE prioritizes RACH transmission over dynamic or semi-static DL reception.

Proposal 30:

- Support gNB in informing the UE whether the UE can use the SBFD occasion, which partially/completely overlaps with the SSB occasion for a UL transmission.

Proposal 31:

- The predefined priority of SPS PDSCH and the configured UL transmission given above can be supported.

Proposal 32:

- Specify DMRS bundling enhancement when the TDW overlaps with SBFD symbols.

Proposal 33:

- Separate SSBs for SBFD devices are required

Proposal 34:

- When determining the size of DL usable PRBs, considerations need to be made to ensure the SSB is able to fit within the subbands.

Proposal 35:

- When issuing RSRP threshold levels, the current SBFD status needs to be considered for measuring RS in SBFD.

# References

- RP-234035, ‘New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)’ CMCC.

- Draft_Minutes_report_RAN1#109-e_v030

- Draft_Minutes_report_RAN1#110_v020

- Draft_Minutes_report_RAN1#110-bis-e_v010

- Draft_Minutes_report_RAN1#111_v020

- Draft_Minutes_report_RAN1#112_v010

- Draft_Minutes_report_RAN1#112b-e_v010

- Draft_Minutes_report_RAN1#113_v020

- Draft_Minutes_report_RAN1#116_v020

- Draft_Minutes_report_RAN1#116bis_v030

# Previous Agreements

RAN1#116bis

Agreement

A symbol configured as SBFD symbol via cell-specific configuration cannot be reverted to a non-SBFD symbol via any UE-specific configuration or group-common signaling.

A symbol not configured as SBFD symbol via cell-specific configuration cannot be reverted to an SBFD symbol via any UE-specific configuration or group-common signaling.

Agreement:

For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs.

Agreement

Study the feasibility and enhancements to support separate power control and/or spatial relation for SRS, PUCCH and PUSCH in SBFD and non-SBFD symbols in different slots, including repetition and non-repetition, by considering existing schemes, e.g. multi-TRP PUCCH/PUSCH repetition schemes.

Agreement

For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS, discuss and decide whether/which of the following options is supported.

- Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping.

  - Existing RB indexing and VRB-to-PRB mapping are reused

  - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only

  - FFS: DMRS sequence mapping

- Option 1-2: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping.

  - Existing RB indexing and VRB-to-PRB mapping are reused

  - The number of PRBs for TBS determination is based on the assigned PRBs as legacy

  - FFS: DMRS sequence mapping

- Option 2: Introduce new RB indexing/PRB bundle indexing to ensure VRBs are mapped to DL usable PRBs only.

  - Existing VRB-to-PRB mapping is reused

  - Legacy TBS determination method is used

  - FFS: DMRS sequence mapping

- Option 3: Modify VRB-to-PRB mapping interleaver to ensure VRBs are mapped to DL usable PRBs only.

  - Existing RB indexing/PRB bundle indexing is reused

  - If the interleaver is not enabled, Option 1-1 or Option 1-2 is used

  - Legacy TBS determination method is used

  - FFS: DMRS sequence mapping

Agreement

For cell-specific configuration of frequency locations of SBFD subbands,

- Option 1: Cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList.

  - For each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing.

Agreement

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate resource allocations for SBFD symbols and non-SBFD symbols

  - FFS other separate configurations for SBFD symbols and non-SBFD symbols

- Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: An SPS PDSCH reception occasion overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid

- Option 4: Only SPS PDSCH reception occasion in one symbol type is valid and SPS PDSCH reception occasion in the other symbol type is invalid

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid for SPS PDSCH

- Other options are not precluded

For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate resource configurations for SBFD symbols and non-SBFD symbols

  - FFS type 2 CG PUSCH

  - FFS other separate configurations for SBFD symbols and non-SBFD symbols

- Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: A CG PUSCH transmission occasion overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid

- Option 4: Only CG PUSCH transmission occasion in one symbol type is valid and CG PUSCH transmission occasion in the other symbol type is invalid

- Option 5: Only the assigned PRBs within UL usable PRBs in SBFD symbols are considered to be valid for CG PUSCH

- Other options are not precluded

Agreement

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal.

- The above does not imply link direction indication is supported

- FFS on dynamically scheduled UL transmission with repetition

Agreement

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles and timeline in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UL transmission is cancelled if cancellation timeline is met.

- The above does not imply link direction indication is supported

- FFS on dynamically scheduled DL reception with repetition

Agreement

For PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols in different slots, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

- Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: A PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid, e.g. the PDSCH in the slot is dropped

- Option 4: Only PDSCH in one symbol type is valid and PDSCH in the other symbol type is invalid

- Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid

- Option 6: gNB does not schedule any PDSCH in SBFD symbols in a slot to be overlapping with PRBs outside DL usable PRBs

- Other options are not precluded

- FFS: Applicable conditions

Agreement

For PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols in different slots, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols, and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported.

- Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols

- Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type

- Option 3: A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid, e.g. the PUSCH in the slot is dropped/postponed

- Option 4: Only PUSCH in one symbol type is valid and PUSCH in the other symbol type is invalid

- Option 5: For a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols, only the assigned PRBs within UL usable PRBs are considered to be valid

- Option 6: gNB does not schedule any PUSCH in SBFD symbols in a slot to be overlapping with PRBs outside UL usable PRBs

- Other options are not precluded

- FFS: Applicable conditions

RAN1#116

Agreement

For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier.

- FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

Agreement:

For RRC connected mode UEs, SBFD subband time locations are configured within a period. At least when only one TDD-UL-DL pattern is configured, the period is down-selected from one of the following options.

- Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- FFS: Further details

FFS: Details when two TDD-UL-DL patterns are configured

Agreement

A slot can consist of SBFD symbols and non-SBFD symbols.

Agreement:

The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.

The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier.

For semi-static indication of SBFD subband frequency location, down-select from the following options.

- Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

- Option 2: Frequency location of UL subband and the number of RBs for guardband(s), if any, are explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s).

Agreement

A slot can consist of SBFD symbols and non-SBFD symbols.

For semi-static indication of SBFD subband time location,

- When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period.

  - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon

  - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot.

  - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS.

  - FFS details

Agreement

The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid.

- RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

  - Subject to RAN4 guidance on the size of subband/guardband, if any

  - FFS reference starting RB and reference SCS

Agreement

For discussion purpose, UL subband frequency resources within active UL BWP are called UL usable PRBs and DL subband(s) frequency resources within active DL BWP are called DL usable PRBs.

For determining UL/DL usable PRBs, consider the following options.

- Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols.

- Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

Agreement

For SBFD-aware UE transmission and reception in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon,

- UL transmissions within UL usable PRBs are allowed

  - FFS SSB symbols

- DL receptions within DL usable PRBs are allowed

- UL transmissions outside UL usable PRBs are not allowed

- DL receptions outside DL usable PRBs are not allowed

  - This restriction is not applicable for CLI measurement

CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3.

RAN1 to discuss SBFD aware UE behaviors in SBFD symbols with interaction with legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0

- DCI format 2_0 cannot be used to revert SBFD symbol to non-SBFD symbol

Agreement

For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol.

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

Other options are not precluded.

Agreement:

For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary:

- Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

  - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

- Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

  - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

- Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

- Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

- Case 5: SSB vs. dynamically scheduled or configured UL transmission

  - e.g., PUSCH, PUCCH, PRACH, SRS

- Case 6: Dynamic or semi-static DL vs. valid RO

Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

RAN1#113

Conclusion At least for semi-static SBFD, in order to avoid frequent switching between SBFD and non-SBFD symbols, potential limitation on the maximum number of switching transition points between SBFD and non-SBFD symbols can be considered from the SBFD subband configuration perspective. A maximum of two switching transition points, including one switching transition point from non-SBFD symbols to SBFD symbols and one switching transition point from SBFD symbols to non-SBFD symbols within a TDD UL/DL pattern period can be considered as a starting point where the switching transition point can be aligned with slot boundary or within a slot. - (Agreement) The usage of ‘switching point’ in previous conclusions/agreements are revised to ‘transition point’ A guard period between SBFD and non-SBFD symbols may or may not be required at gNB and/or UE side depending on gNB/UE implementation and/or SBFD operation. Agreement For the three methods agreed to be studied for UE-to-UE CLI-RSSI measurement/report across downlink subbands, the following observations are agreed. - Method #1 allows flexible configuration of measurement reporting in one DL subband or two DL subbands but it consumes multiple CLI-RSSI measurement resources from the UE capability budget. - Method #2 restricts gNB configuration flexibility and does not account for whether or not the CLI is asymmetric across two DL subbands. This method does not consume multiple CLI-RSSI measurement resources from UE capability point of view. - Method #3 requires additional specification efforts to support non-contiguous CLI-RSSI resource allocation across downlink subbands. This method is similar to non-contiguous CSI-RS resource allocation. A single CLI-RSSI report based on non-contiguous CLI-RSSI resource may be sufficient. This method does not consume multiple CLI-RSSI measurement resources from UE capability point of view. Note: Above does not imply whether L1 or L2 based measurement is supported. Conclusion For a PRG that overlaps with subband boundary, if the part of DL PRG inside the DL subband can be used, better scheduling flexibility and resource utilization can be achieved, however degraded channel estimation quality in the partial PRG is expected compared to a PRG due to limited RBs in the partial PRG. - Note: UE complexity could increase if this feature is supported Agreement An UL subband can be configured in an SSB symbol. - Note: It is SSB from serving cell perspective, which can be CD-SSB or NCD-SSB. - Whether actual UL transmission can be done is for further discussion Agreement The following conclusion is to be captured in the TR If PRG is determined as wideband, better scheduling flexibility and higher DL data rate can be achieved if non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated. Compared to the case that PRG is determined as wideband and only contiguous frequency resources can be allocated, non-contiguous frequency resources across two DL subbands requires UE to handle two non- contiguous segments of contiguous RBs that may increase UE complexity for channel estimation. Agreement The following conclusion is to be captured in the TR gNB can configure a CORESET and a search space in a way such that the MOs of the search space occur in either SBFD or non-SBFD symbols, or the MOs of the search space occur in both SBFD and non-SBFD symbols but the associated CORESET does not overlap the boundary of a DL subband in SBFD symbols. If it is agreed to be beneficial that a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of a DL subband in SBFD symbols, at least the following options can be considered for SBFD-aware UE: - Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols. - Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s). - Option 3: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL subband(s). - Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s) - Option 5: Separate search spaces associated with a CORESET in SBFD and non-SBFD symbols Note: These options are applicable to at least USS. Agreement The following conclusion is to be captured in the TR - For the methods agreed to be studied for inter-UE inter-subband CLI measurement, Method #2 and Method #3 can be used for identifying the aggressor UE(s) if orthogonal resources are allocated for different aggressor UE(s); and Method #2 and #3 can at least provide higher interference signal strength than inter-subband interference leakage based measurements in Method #1. Furthermore, such measurement is not subject to inter-cell DL interference. - It is feasible for UE to measure RSRP/RSSI within UL subband if within active DL BWP and receive DL in DL subband(s) simultaneously similar as simultaneous RSRP/RSSI measurement and DL reception in Rel-16. - The existing CLI measurement and report framework can be reused to support RSRP/RSSI measurements within UL subband when UL subband is confined within active DL BWP. Agreement The following conclusion is to be captured in the TR If SBFD-aware UEs are not allowed to transmit in the SSB symbol but is allowed to receive within the DL BWP in the SSB symbol, negative impact on SSB detection and measurement can be avoided but UL performance may be degraded due to fewer UL opportunities. If SBFD-aware UE is allowed to transmit in the SSB symbol, the UE may only transmit UL in an UL subband depending on gNB scheduling, configuration, UE measurement or priority rule. There may be negative impact on SSB detection and measurement if the SBFD-aware UE is requested to transmit in the SSB symbol. Agreement The following conclusion is to be captured in the TR For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot if any, the following options for UE transmission/reception can be considered in the normative stage - Option 1: UE does not transmit or receive the physical channel/signal within the slot. - Option 2: UE can transmit or receive the physical channel/signal within the slot only under certain conditions. - The conditions may depend on at least the following: whether or not phase continuity can be maintained across SBFD and non-SBFD symbols, whether or not there are same or different transmission/reception parameters e.g. power control, spatial/QCL, UL timing etc. applied in SBFD and non-SBFD symbols, and whether or not there is a guard period between the SBFD and non-SBFD symbols, etc. - Other options are not precluded Agreement The following conclusion is to be captured in the TR For SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, it may be beneficial to have separate resources, FH parameters, UL power control parameters and/or beam/spatial relation.

RAN1#112-bis-e

Conclusion The following RAN1 observation is made: One motivation for allowing that a slot can consist of both SBFD and non-SBFD symbols is for compatibility with symbol-level TDD UL/DL configuration. Frequent switching between SBFD and non-SBFD symbols may increase the implementation complexity and interruptions of transmissions/receptions during transition. - Further study whether limitation(s) on the maximum number of switching points between SBFD and non-SBFD symbols within a slot, a TDD UL/DL pattern period, and/or semi-static SBFD configuration period (if different from TDD UL/DL pattern period) are needed - Further study scenarios a guard period between SBFD and non-SBFD symbols is required/not required and the length of the guard period if required Note: Whether or not a physical channel/signal occasion is mapped to both SBFD and non-SBFD symbols within a slot is a separate discussion. Agreement At least for semi-static SBFD, the following two options are viable solutions for frequency location configuration of DL subband(s) and guardband(s) if any. - Option 1: Frequency locations of DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). - Option 2: The number of RBs for guardband(s), if any, is explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s). Agreement If PRG is determined as wideband, study the following two options: - Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated - FFS: Precoding assumption within and across the two DL subbands - Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated The study should include the impact on UE complexity Agreement (modified on April 26th as shown in red) For UE-to-UE CLI-RSSI measurement/report across downlink subbands, study the following methods: - Method#1: separate CLI-RSSI measurement resources/reports in each DL subband - Note: supported in existing specifications - Method#2: CLI-RSSI measure/report in one DL subband only - Note: supported in existing specifications - Method#3: CLI-RSSI measurement/report based on non-contiguous CLI-RSSI resource across downlink subbands - FFS: report single or separate CLI-RSSI report(s) - FFS: details on determination of non-contiguous CLI-RSSI resource allocation Agreement: For SBFD-aware UEs, Option 1 with update is agreed for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands for better resource utilization. For an RBG that overlaps the subband boundary, - Option 1 (with update): - The Part of the DL RBG inside the DL subband can be used - The Part of the UL RBG inside the UL subband can be used Agreement For semi-static SBFD, a SBFD aware UE does not transmit UL channels/signals or receive DL channels/signals on the guardband(s) that the UE is aware of. - FFS: Measurement in guardband for the purpose of CLI measurement. Agreement For semi-static SBFD, for a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid for SBFD-aware UE. - For semi-static SBFD, for a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s). Conclusion For the two options agreed in RAN1#112 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following observations are agreed. - Option 1 can be achieved by gNB configuration or scheduling to ensure that all transmission/reception occasions are confined to either SBFD symbols or non-SBFD symbols. Alternatively, Option 1 can be achieved by additional indication or rules to determine the transmission/reception occasions are valid within one symbol type and are invalid within the other symbol type. - The frequency resources, power control and beam/spatial relation for all the transmission/reception occasions can be the same for Option 1 but may be different for Option 2. If different, it may require additional specification efforts. - Option 1 may or may not increase the transmission/reception latency if the transmission/reception in the other symbol type is postponed and may degrade the performance if the transmission/reception in the other symbol type is dropped. Option 2 may or may not reduce the transmission/reception latency and improve coverage. Agreement For inter-UE inter-subband CLI measurement, study Method#2 and Method#3 considering: - Necessity/benefit compared with measurement within DL subband - Whether/how to estimate CLI from RSRP/RSSI measurements within UL subband / guardband - Whether UE is required to measure RSRP/RSSI within UL subband and receive DL in DL subband(s) simultaneously - Whether existing CLI measurement and report framework can be reused to support RSRP/RSSI measurements within UL subband - If not, identify the potential impact Conclusion Time misalignment at gNB between UL receptions and DL transmissions due to configuration of non-zero NTA,offset at UE can lead to increased interference assuming no gNB transmit chain side impairments and no filtering of DL subband(s) in the gNB Rx chain. - FFS the case with gNB transmit chain impairments and/or filtering of DL subband(s) in the gNB Rx chain - FFS whether/how to mitigate the interference increase, including impact to legacy UEs Agreement Study the following options for SBFD operation in SSB symbols. - Option 1: UL subband cannot be configured in an SSB symbol - FFS handling of misaligned periodicities between SSB and semi-static SBFD subband time location configuration - Option 2: An UL subband can be configured in an SSB symbol - FFS whether/when and/or under which conditions an SBFD-aware UE transmits in the UL subband or may receive SSB in the symbol. Agreement Study whether the transmission/reception occasion of a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot for a UE, and whether a UE can transmit/receive in the occasion mapped to SBFD symbols and non-SBFD symbols including: - Use-case(s) including the locations and number of switching points of the SBFD and non-SBFD symbols in the slot. - Potential benefits if any - Phase continuity - Potential interruption of transmissions/receptions during transition - Required guard time if any - Potential impact on performance - Impact on link adaptation, channel estimation, and other procedures - UL transmission timing if any - Implementation complexity - Applicability for SBFD aware UE and non-SBFD aware UEs - NOTE: There are more than one scenario where a transmission overlaps SBFD and non-SBFD symbols and some may or may not face the aspects listed above - NOTE: This study doesn’t mean RAN1 agreement on a slot consisting of SBFD and non-SBFD symbols. Conclusion For the options agreed to study in RAN1#112 for frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, the following observations are agreed. - For all the options, there is no impact on CSI-RS sequence generation. - Option 1 requires additional signalling to link two CSI-RS resources in two DL subbands. - Option 2-1 requires new RRC structure to configure non-contiguous RBs for one CSI-RS resource, which may require additional signalling overhead. - Option 2-2 can reuse the existing signalling design for CSI-RS resource configuration. Option 2-2 can be used to resolve the potential unaligned boundaries between CSI-RS resource configuration and SBFD subbands - Further discussion is required on the UE complexity due to: - UE capability of maximum number of configured CSI-RS resources - Processing non-contiguous CSI-RS Agreement For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols: - Option 1: two CSI-ReportConfigs, where one is associated with SBFD symbols and the other is associated with non-SBFD symbols - Option 1-1: One CSI-ReportConfig is associated with a CSI-RS restricted to SBFD symbols only and the second CSI-ReportConfig is associated with a second CSI-RS restricted to non-SBFD symbols only; - Option 1-2: Both CSI-ReportConfigs are associated with the same CSI-RS. The CSI report associated with one CSI-ReportConfig is derived based on CSI-RS instances in SBFD symbols only. The CSI report associated with the second CSI-ReportConfig is derived based on CSI-RS instances in non-SBFD symbols only. - Option 2: one CSI-ReportConfig associated with both SBFD symbols and non-SBFD symbols - Option 2-1: One CSI-ReportConfig is associated with two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively. Separate CSI measurements are derived based on the first and second CSI-RSs respectively. - Option 2-2: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances. - FFS impact on UE CSI processing and reporting timeline Note: Whether the CSI-RS resource can be used for SBFD and non-SBFD symbols may depend on, e.g., gNB implementation of same/different antenna configuration in both symbols. Option 1-1 can be supported according to existing specification by gNB configuration of appropriate periodicities to ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. But it may restrict the gNB configuration flexibility and enhancements can be considered by additional indication or rules to determine the CSI-RS is valid within one symbol type and is invalid in the other symbol type. Option 2-2 can be supported according to existing specification to configure measurement restriction so that UE would not average CSI measurements across SBFD and non-SBFD symbols. Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, study at least the following frequency resource allocation options for PDSCH, CSI-RS, PUSCH, PUCCH, SRS for SBFD-aware UE: - Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots. - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication - Option 1-3: single FDRA configuration/indication and RB offset(s) - Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals. - Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed. Note: Different options can be studied for different signals/channels. Agreement For the case that: - The monitoring periodicity of a search space is such that different monitoring occasions in different slots occur in SBFD and non-SBFD symbols, respectively, and, - The associated CORESET overlaps the boundary of a DL subband in SBFD symbols Consider whether/how the above could be supported considering both existing tools in specifications on CORESET and search space configuration as well as at least the following options for potential enhancement for SBFD-aware UE: - Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols. - Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s). - Option 3: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL subband(s). - Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s) - Option 5: Separate search spaces associated with a CORESET in SBFD and non-SBFD symbols Note: Whether these enhancements are applicable to only USS or also CSS.

RAN1#112

Agreement For dynamic SBFD, - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are not allowed - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are allowed Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency. For each option, additional conditions may apply to determine whether the option is applicable. Agreement Study whether or not a slot can consist of both SBFD and non-SBFD symbols including - Benefits - Use cases - Scheduling flexibility - Implementation complexity - Compatibility with legacy TDD DL/UL configuration Agreement For inter-UE inter-subband CLI measurement, study at least the following methods: - Method#1: victim UE measures RSSI within DL subband - FFS: Whether SINR can be measured - Method#2: victim UE measures RSRP of aggressor UE within UL subband - Method#3: victim UE measures RSSI within UL subband - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP. Agreement: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) - Study the following options for SBFD-aware UEs: - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following: - PDSCH/PUSCH/PUCCH repetitions - SPS PDSCH/CG PUSCH - TBoMS - Multi-PUSCH/PDSCH scheduled by a single DCI - Periodic/semi-persistent SRS/CSI-RS/PUCCH - PDCCH Agreement For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary, - Option 1: - Part of the DL RBG inside the DL subband can be used - Part of the UL RBG inside the UL subband can be used - Option 2: - Part of the DL RBG inside the DL subband cannot be used - Part of the UL RBG inside the UL subband cannot be used FFS: The part of the RBG outside. Agreement For SBFD-aware UEs, study at least the following issues for PDSCH: - PRG(s) with size of 2 and 4 that overlaps with subband boundary - Wideband precoder in case of non-contiguous DL subbands Agreement: Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options: - Option 1: Two contiguous CSI-RS resources that are linked - Option 2: One CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s) Agreement: For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols): - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols Agreement: Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots: - Whether/how to have separate resources - Whether/how to have separate FH parameters - Whether/how to have separate UL power control parameters - Whether/how to have separate beam/spatial relation

RAN1#111

Agreement For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived - DL receptions within DL subband(s) are allowed in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol Agreement For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid. Agreement Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured. Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously Agreement Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following: - RBG for PDSCH RA type 0 - CSI reporting configuration - CSI-RS resource configuration - PRG of PDSCH

RAN1#110-bis-e

Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined. Agreement Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands. Agreement Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - If there are, whether/how to avoid/handle such collision cases (as second step) Agreement Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency. Agreement Study whether SBFD operation in SSB symbols is supported or not. Agreement For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline. Agreement The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1. - The UL subband can be located at one side of the carrier. - The UL subband can be located at the middle part of the carrier, subject to RAN4’s study and conclusion Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible. Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance. Send an LS to RAN4 to inform the above agreement. If RAN4 has response, it will be taken into account but in the meanwhile, RAN1 work will continue based on the above. LS on maximum number of UL subbands for duplex evolution to RAN4 is endorsed. Final LS in R1-2210671. Agreement For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

RAN1#110

Agreement: Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state. - SBFD operation Alt 1: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side. - SBFD operation Alt 2: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs - SBFD operation Alt 3: - Only time location of subbands for SBFD operation is known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. UE capability discussion is held in work item phase. Agreement: For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline. Agreement For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline. Working Assumption For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair Agreement For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol - Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol - Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB

RAN1#109e

Agreement Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation. Agreement Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation. Agreement At least study SBFD operation within a TDD carrier. Agreement The time and frequency location of subbands within a TDD carrier are not fixed in the specification. - Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier. - Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed. Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of gNB-to-gNB CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - gNB-to-gNB CLI measurement and reporting - Coordinated scheduling - Spatial domain enhancements - Advanced receiver - UE and gNB transmission and reception timing - Power control based solution - Potential enhancements to Rel-16 RIM - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for inter-gNB CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancements specific for SBFD will be discussed in 9.3.2 Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of UE-to-UE CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - Potential enhancements to UE-to-UE CLI measurement/reporting - Coordinated scheduling - Spatial domain enhancements, - Advanced Receiver - UE and gNB transmission and reception timing - Power control based solution - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for UE-to-UE CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancement specific for SBFD will be discussed in 9.3.2