3GPP TSG RAN WG1 #117                                           R1-2405002

Fukuoka, Japan, May 20th – 24th, 2024

Source:	ZTE

Title:	Discussion on ISAC deployment scenarios

Agenda item:	9.7.1

Document for:	Discussion and Decision

Introduction

In RAN#102 meeting, the following SID [1] on channel modelling for Integrated Sensing And Communication (ISAC) was agreed:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

The following agreements were agreed in RAN1#116 and RAN1#116bis [2]:

Agreement in RAN1#116 For progressing ISAC study, the following sensing targets and existing communication scenarios will be considered as a starting point: - Note1: the table below does not imply that the sensing target will be placed at positions defined for UEs and BSs in the scenarios in the right column. - Note2: the table below does not imply that UEs are necessarily placed at positions defined for UEs in the scenarios in the right column. - Note3: the existing communication scenarios are listed with the intent to use the evaluation parameters defined for those scenarios, as a starting point. Sensing Targets; scenarios. UAVs; RMa-AV, UMa-AV, UMi-AV (TR 36.777). Humans indoors; InF, Indoor Office, [Indoor Room (TR 38.808)], [UMi, UMa]. Humans outdoors; UMi, UMa, [RMa]. Automotive vehicles (at least outdoors); Highway, Urban grid, UMa, UMi, RMa. Automated guided vehicles (e.g. in indoor factories); InF. Objects creating hazards on roads/railways (examples defined in TR 22.837); Highway, Urban grid, HST. Agreement in RAN1#116bis RAN1 agrees the following ISAC terminology with minor modifications as follows: For ISAC channel modelling, RAN1 uses the sensing related terminology as defined in TS22.137 or TR22.837 as a starting point for discussion purposes with the following definitions: - Sensing transmitter: the TRP or a UE that sends out the sensing signal which the sensing service will use in its operation. A sensing transmitter can be located in the same or different TRP or a UE as the sensing receiver. - Sensing receiver: the TRP or a UE that receives the sensing signal which the sensing service will use in its operation. A sensing receiver can be located in the same or different TRP or a UE as the sensing transmitter. - Sensing target: target that need to be sensed by deriving characteristics of the objects within the environment from the sensing signal. - Background environment: background (clutter and/or environmental objects) that are not the sensing target(s). - Mono-static sensing: sensing where a sensing transmitter that transmits a sensing signal and a sensing receiver that receives the sensing signal are co-located in the same TRP or UE. - Bi-static sensing: sensing where a sensing transmitter that transmits a sensing signal and a sensing receiver that receives the sensing signal are not co-located in the same TRP or UE. - Multi-static sensing: sensing where there are multiple sensing transmitters and/or multiple sensing receivers, for a sensing target. - Sensing signal: Transmissions on the 3GPP radio interface that can be used for sensing purposes. Agreement in RAN1#116bis Any TRP and/or UE location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations. FFS: other possible sensing transmitters and receivers locations. Agreement in RAN1#116bis The following table can be used by companies to propose values for each sensing target - Additional parameters/rows can be added if needed Table x. Evaluation parameter template for sensing scenarios Parameters; Value. Applicable communication scenarios. Sensing transmitters and receivers properties. Supported sensing modes. Sensing target; Outdoor/indoor. 3D mobility. 3D distribution. Orientation. Physical characteristics (e.g., size). [Unintended/Environment objects]; Types. 3D mobility. 3D distribution. Orientation. Physical characteristics (e.g., size). [Sensing area]. Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects].

In this contribution, we provide our views on ISAC deployment scenarios.

Discussion

As an important potential key technology of 5G-Advanced, integrated sensing and communication (ISAC) has drawn extensive attention. It can provide sensing function in the current 5G communication system. To reduce costs, power consumption, and optimize resource utilization compared to two independent systems, a unified design of communication and sensing functions through signal joint design and/or hardware sharing is important. In other words, the 5G communication system transmits wireless signals to target areas or objects, and analyzes the received reflected or scattering wireless signals to obtain corresponding sensing measurement data, and further provides sensing services to third-party applications. The feasibility study report for ISAC in SA1 [3] [4] contains 32 use cases and the corresponding requirements. Among the above use cases, it is agreed that Rel-19 study in RAN side focuses only on the use cases regarding object detection and/or tracking.

There is no one-to-one mapping between sensing targets and deployment scenarios. Specifically, one deployment scenario can be configured to evaluate more than one types of sensing targets, and one type of sensing target can appear in multiple sensing deployment scenarios. For example, human may appear in either indoor office, indoor factory or in highway, and highway scenario can include both automotive vehicles and humans.

Moreover, the evaluation parameter per deployment scenario is related to sensing mode. In one deployment scenario, there may exist a combination of multiple sensing modes including at least one of TRP monostatic, TRP bistatic, TRP-UE bistatic, UE-TRP bistatic, UE monostatic, UE bistatic. If only one sensing mode is assumed, there is no need to include any evaluation parameters for unrelated sensing node.

In current communication scenario, only the parameters regarding gNB and UE are considered. Taking that as a starting point, we should further consider:

- Parameters related to sensing targets (ST): e.g., ST height, ST velocity, ST distribution, min. BS-to-target distance, min. UE-to-target distance, min. target-to-target distance.

- Modification of existing scenario parameter: e.g., ISD.

Detailed information for evaluation parameters is provided in the following sub-sections:

## 2.2.1 UAV

With the continuous expansion of the UAV market, more and more civilian UAVs have entered our lives. The commercialization of low-altitude UAVs has entered a new stage [4]. In future cities, a large number of UAVs will be used in industrial inspections, transportation, and logistics transportation and other scenarios. In the logistics industry, UAV delivery is developed very quickly and is estimated to become a nearly 10-billion-euro market.

By comparing evaluation parameters for RMa-AV/UMa-AV/UMi-AV in TR 36.777 and that for RMa /UMa /UMi in TS 38.901, we observe the following differences in Table 1:

Table 1: Evaluation parameter differences between RMa-AV/UMa-AV/UMi-AV in TR 36.777 and RMa /UMa /UMi in TS 38.901

|  | RMa-AV/UMa-AV/UMi-AV in TR 36.777 | RMa /UMa /UMi in TS 38.901 |
| --- | --- | --- |
| Cell layout (site number) | 19 or 37 or even more than 37 sites | 19 sites |
| UE height | Terrestrial UE: same as 38.901 Aerial UE: - Uniformly distributed between 1.5m and 300m - Optionally, fixed height value chosen from {50, 100, 200, 300} m. - Other optional fixed height values are not precluded. | UMi, UMa: Same as TR36.873 RMa: 1.5m |
| UE mobility | - 30 km/h for outdoor terrestrial UEs - 3 km/h for indoor terrestrial UEs - 160 km/h for aerial UEs | UMi, UMa: 3km/h |
| Min. BS - UE distance(2D) | Min. BS – Terrestrial UE distance: - UMi-AV: 10m - UMa-AV: 35m - RMa-AV: 35m Min. BS – Aerial UE distance: - UMi-AV: 10m - UMa-AV: 10m - RMa-AV: 10m | UMi: 10m UMa: 35m RMa: 35m |
| Aerial UE ratio | - Case 1: 0% (corresponding to ) used as baseline - Case 2: 0.67% (corresponding to ) - Case 3: 7.1% (corresponding to ) - Case 4: 25% (corresponding to ) - Case 5: 50% (corresponding to ) | / |
| LOS/NLOS | Terrestrial UE: LOS and NLOS Aerial UE: Modelled according to Annex B in 36.777 | LOS and NLOS |
| Channel modes | Terrestrial UE: same as 38.901 Aerial UE: According to Annex B in 36.777 | Section 7.5 of 38.901 |
| Other parameters: carrier frequency, system bandwidth, BS antenna pattern | These parameters are not included in 38.901 |  |

Generally, UAV parameters can refer to aerial UE configurations in 36.777. The following can be further considered:

- Cell layout: For UAV as sensing target, LOS probability is high especially when the height of UAV is higher than 100m. Larger ISD can be optionally considered in scenarios for UAV as sensing target. For example, for UMa scenarios, 1km-ISD can be considered.

- Sensing transmitter and receiver properties: generally, the TRP located in communication scenario can be selected as sensing transmitter and sensing receiver. Any modifications based on the TRP or UE configurations in communication scenario is optional.

- Sensing mode: For UAV as sensing target, we prefer to take TRP monostatic and TRP bistatic as first priority sensing modes since UE’s lack of transmission power and antenna elements.

  - For UE as sensing transmitter or sensing receiver, one remaining issue is whether terrestrial UE or aerial UE can be selected. If terrestrial UEs are chosen for sensing transmitter, the coverage of terrestrial UE transmitting sensing RS may not be large enough especially when the height of UAV is up to 600m. Moreover, if aerial UE with a velocity of 160km/h is chosen as sensing transmitter or sensing receiver, coverage may not be an issue but its antenna element number is limited. Spatial consistency should be specially designed since both sensing entity and sensing target move fast.

- Sensing target orientation: orientation can be taken as the direction of sensing target’s velocity.

- Sensing target physical characteristics: multiple UAV dimension are considered. Given the fact that RCS of UAV is dependent on the size of UAV, to ease the spec work, only one typical size can be chosen as baseline. From our side, we support to set a-small-size-UAV as baseline.

- Sensing area: we prefer to select one sector as sensing area to simplify the simulation complexity. To evaluate the performance of sensing, one way is to deploy sensing targets only in the center sector. To get smooth CDF curves for the simulation results, multiple drops can be done in the simulation.

According to the above analysis, we support the following proposal for UAV as sensing target:

Proposal 1: Support the following evaluation parameter for sensing scenarios for UAV as sensing target

Table 2. Evaluation parameter for sensing scenarios for UAV as sensing target

| Parameters |  | Value |  |  |
| --- | --- | --- | --- | --- |
| Applicable communication scenarios |  | UMi-AV | UMa-AV | RMa-AV |
| Cell layout |  | Hexagonal grid, 19 macro sites, 3 sectors per site ISD =200 m, optional larger ISD | Hexagonal grid, 19 macro sites, 3 sectors per site ISD =500 m, optional larger ISD (e.g., 1000m) | Hexagonal grid, 19 macro sites, 3 sectors per site ISD =1732 m, optional larger ISD |
| Sensing transmitters and receivers properties |  | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | TRP monostatic and TRP bistatic as first priority | TRP monostatic and TRP bistatic as first priority | TRP monostatic and TRP bistatic as first priority |
| Sensing target | Outdoor/indoor | Outdoor | Outdoor | Outdoor |
|  | LOS/NLOS | LOS | LOS | LOS |
|  | 3D mobility | {30, 60, 160}km/h in horizontal plane, 0 km/h in vertical plane Other velocity values in horizontal plane are FFS | {30, 60, 160}km/h in horizontal plane, 0 km/h in vertical plane Other velocity values in horizontal plane are FFS | {30, 60, 160}km/h in horizontal plane, 0 km/h in vertical plane Other velocity values in horizontal plane are FFS |
|  | 3D distribution | Horizontal: uniform Height: options from 36.777 - Uniformly distributed between 1.5m and 300m - Fixed height value chosen from {50, 100, 200, 300} m - Optional up to 600m | Horizontal: uniform Height: options from 36.777 - Uniformly distributed between 1.5m and 300m - Fixed height value chosen from {50, 100, 200, 300} m - Optional up to 600m | Horizontal: uniform Height: options from 36.777 - Uniformly distributed between 1.5m and 300m - Fixed height value chosen from {50, 100, 200, 300} m - Optional up to 600m |
|  | Orientation | Random (direction of velocity) | Random (direction of velocity) | Random (direction of velocity) |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height) - Baseline:0.15m x 0.15m x 0.1m - Other sizes are optional. | Size (Length x Width x Height) - Baseline:0.15m x 0.15m x 0.1m - Other sizes are optional. | Size (Length x Width x Height) - Baseline:0.15m x 0.15m x 0.1m - Other sizes are optional. |
| Sensing area |  | Select one center sector |  |  |
| Minimum 3D distances between Tx and sensing target |  | 10m | 10m | 10m |
| Minimum 3D distances between Rx and sensing target |  | 10m | 10m | 10m |
| Minimum 3D distances between sensing targets |  | 10m | 10m | 10m |

## 2.2.2 Human

### 2.2.2.1 human indoor

For detecting or tracking human indoor, we are fine with both indoor factory scenario and indoor office scenario. UMi and UMa scenario can only be used for human outdoor, otherwise for detecting human indoor the O2I penetration loss should be additionally considered which cause too much complexity.

Proposal 2: Support the following evaluation parameter for sensing scenarios for human indoor as sensing target

Table 3. Evaluation parameter for sensing scenarios for human indoor as sensing target

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Indoor factory | Indoor office |
| Cell layout |  | Refer to 38.857 | Refer to 38.901 |
| Sensing transmitters and receivers properties |  | Any TRP and/or UE location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP and/or UE location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | All 6 sensing modes | All 6 sensing modes |
| Sensing target | Outdoor/indoor | Indoor | Indoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | 3D mobility | 5km/h in horizontal plane (refer to 22.837) | 5km/h in horizontal plane (refer to 22.837) |
|  | 3D distribution | Uniform in horizontal plane | Uniform in horizontal plane |
|  | Orientation | Random (direction of velocity) | Random (direction of velocity) |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m |
| Sensing area |  | Indoor layout | Indoor layout |
| Minimum 3D distances between Tx and sensing target |  | 0m (2D) 3D distance can be inferred via the height of sensing transmitter and target | 0m (2D) 3D distance can be inferred via the height of sensing transmitter and target |
| Minimum 3D distances between Rx and sensing target |  | 0m (2D) 3D distance can be inferred via the height of sensing receiver and target | 0m (2D) 3D distance can be inferred via the height of sensing receiver and target |

### 2.2.2.2 human outdoor

For detecting or tracking human outdoor, we prefer to consider UMi scenario and UMa scenario.

Proposal 3: Support the following evaluation parameter for sensing scenarios for human outdoor as sensing target

Table 4. Evaluation parameter for sensing scenarios for human outdoor as sensing target

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | UMi | UMa |
| Cell layout |  | Hexagonal grid, 19 macro sites, 3 sectors per site ISD =200 m | Hexagonal grid, 19 macro sites, 3 sectors per site ISD =500 m |
| Sensing transmitters and receivers properties |  | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | All 6 sensing modes | All 6 sensing modes |
| Sensing target | Outdoor/indoor | Outdoor | Outdoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | 3D mobility | 5km/h in horizontal plane | 5km/h in horizontal plane |
|  | 3D distribution | Uniform in horizontal plane | Uniform in horizontal plane |
|  | Orientation | Random (direction of velocity) | Random (direction of velocity) |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m |
| Sensing area |  | Select one center sector |  |
| Minimum 3D distances between Tx and sensing target |  | 10m (2D) for BS-sensing target 3D distance can be inferred via the height of sensing transmitter and target | 35m (2D) for BS-sensing target 3D distance can be inferred via the height of sensing transmitter and target |
| Minimum 3D distances between Rx and sensing target |  | 10m (2D) for BS-sensing target 3D distance can be inferred via the height of sensing receiver and target | 35m (2D) for BS-sensing target 3D distance can be inferred via the height of sensing receiver and target |

## 2.2.3 Automotive vehicles

For detecting or tracking automotive vehicles, we prefer to only consider highway scenario and urban grid scenario. UMa, UMi, RMa scenarios are not typical V2X deployment scenarios and there are no roads deployed in UMi, UMa and RMa scenarios. Unlike human which can be distributed uniformly in urban scenario (either in the building or outside), automotive vehicles (e.g., car, truck, bus) should be deployed on the road rather than deployed regardless of road configuration. In our understanding, the highway and urban grid scenario can be taken as a combination of UMa/RMa scenario and road configuration. As shown in the following table:

|  | Legacy (37.886/36.885) |
| --- | --- |
| highway | If macro eNBs are deployed for Freeway case, - Option 1 (baseline): eNBs are located along the freeway 35m away with 1732m ISD in Figure A.1.3-2. - Option 2 (optional): Wrap around method of 19*3 hexagonal cells with 500m ISD in Figure A.1.3-3. |
| Urban grid | If macro eNBs are deployed for Urban case, ISD of macro eNB is 500 m and the wrap around model in Figure A.1.3-1 is used. |

If UE is the sensing transmitter or sensing receiver in highway or urban grid scenario, two types of UEs can be considered:

- UE is deployed similar as 37.885, in such case, one automotive vehicle (UE) is responsible for the detection or tracking another automotive vehicle (sensing target).

- Deploy RSU as sensing transmitter or sensing receiver, and the velocity of RSU is 0km/h. For example, RSU can be distributed along the road.

Proposal 4: If UE is the sensing transmitter or sensing receiver in highway or urban grid scenario, consider one or more of the following UE types:

- Type 1: UE is deployed similar as 37.885

- Type 2: Deploy RSU as sensing transmitter or sensing receiver

Proposal 5: Support the following evaluation parameter for sensing scenarios for automotive vehicles as sensing target

Table 5. Evaluation parameter for sensing scenarios for automotive vehicles as sensing target

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Highway | Urban grid |
| Cell layout |  | Refer to 37.885 | Refer to 37.885 |
| Sensing transmitters and receivers properties |  | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | All 6 sensing modes | All 6 sensing modes |
| Sensing target | Outdoor/indoor | Outdoor | Outdoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | 3D mobility | up to 140km/h, refer to 37.885 | up to 60km/h, refer to 37.885 |
|  | 3D distribution | Refer to 37.885 In lane The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec} | Refer to 37.885 In lane The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec} |
|  | Orientation | Along the lane | Along the lane |
|  | Physical characteristics (e.g., size) | Type 1: length 5 meters, width 2.0 meters, height 1.6 meters Optional: truck/bus length 13 meters, width 2.6 meters, height 3 meters | Type 1: length 5 meters, width 2.0 meters, height 1.6 meters Optional: truck/bus length 13 meters, width 2.6 meters, height 3 meters |
| Sensing area |  | In lane | In lane |
| Minimum 3D distances between Tx and sensing target |  | Refer to 36.885: 37m for BS-sensing target distance (2D), 3D distance can be inferred via the height of sensing transmitter and target FFS distance between sensing target and UE | Refer to 36.885 for distance between BS and sensing target FFS distance between sensing target and UE |
| Minimum 3D distances between Rx and sensing target |  | Refer to 36.885: 37m for BS-sensing target (2D), 3D distance can be inferred via the height of sensing receiver and target FFS distance between sensing target and UE | Refer to 36.885 for distance between BS and sensing target FFS distance between sensing target and UE |

## 2.2.4 AGV

For detecting or tracking AGV, only indoor factory scenario is considered.

Proposal 6: Support the following evaluation parameter for sensing scenarios for AGV as sensing target

Table 6. Evaluation parameter for sensing scenarios for AGV as sensing target

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | Indoor factory |
| Cell layout |  | Refer to 38.857 |
| Sensing transmitters and receivers properties |  | Any TRP and/or UE location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | All 6 sensing modes |
| Sensing target | Outdoor/indoor | Indoor |
|  | LOS/NLOS | LOS and NLOS |
|  | 3D mobility | 5km/h in horizontal plane |
|  | 3D distribution | Uniform in horizontal plane |
|  | Orientation | Random (direction of velocity) |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height) 0.5m x 0.5m x 0.2m |
| Sensing area |  | Indoor layout |
| Minimum 3D distances between Tx/Rx and sensing target |  | 0m (2D) 3D distance can be inferred via the height of sensing transmitter/receiver and target |

## 2.2.5 Objects creating hazards on roads/railways

According to section 5.2 (Use case on pedestrian/animal intrusion detection on a highway) and section 5.7 (Use case on sensing for railway intrusion detection) in TR 22.837, the objects creating hazards on roads/railways mainly include human and animals. As transportation plays one important role in human’s life and the traffic safety of human is essential, we think the sensing target for objects creating hazards on roads/railways should at least include human.

Proposal 7: Sensing targets for objects creating hazards on roads/railways should at least include human.

Proposal 8: Support the following evaluation parameter for sensing scenarios for human on roads

Table 7. Evaluation parameter for sensing scenarios for human on roads

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Highway | Urban grid |
| Cell layout |  | Refer to 37.885 | Refer to 37.885 |
| Sensing transmitters and receivers properties |  | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | All 6 sensing modes | All 6 sensing modes |
| Sensing target | Outdoor/indoor | Outdoor | Outdoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | 3D mobility | 5km/h in horizontal plane | 5km/h in horizontal plane |
|  | 3D distribution | Uniform in horizontal plane | Uniform in horizontal plane |
|  | Orientation | Random (direction of velocity) | Random (direction of velocity) |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m |
| Sensing area |  | In lane | In lane |
| Minimum 3D distances between Tx and sensing target |  | Refer to 36.885: 37m for BS-sensing target distance (2D), 3D distance can be inferred via the height of sensing transmitter and target FFS distance between sensing target and UE | Refer to 36.885 for distance between BS and sensing target FFS distance between sensing target and UE |
| Minimum 3D distances between Rx and sensing target |  | Refer to 36.885: 37m for BS-sensing target (2D), 3D distance can be inferred via the height of sensing receiver and target FFS distance between sensing target and UE | Refer to 36.885 for distance between BS and sensing target FFS distance between sensing target and UE |

Conclusion

In this contribution, we provide our views on details of the deployment scenarios corresponding to ISAC use cases, and we have the following proposals:

Based on the above analysis, we support the following proposal:

Proposal 1: Support the following evaluation parameter for sensing scenarios for UAV as sensing target

Table 2. Evaluation parameter for sensing scenarios for UAV as sensing target

| Parameters |  | Value |  |  |
| --- | --- | --- | --- | --- |
| Applicable communication scenarios |  | UMi-AV | UMa-AV | RMa-AV |
| Cell layout |  | Hexagonal grid, 19 macro sites, 3 sectors per site ISD =200 m, optional larger ISD | Hexagonal grid, 19 macro sites, 3 sectors per site ISD =500 m, optional larger ISD (e.g., 1000m) | Hexagonal grid, 19 macro sites, 3 sectors per site ISD =1732 m, optional larger ISD |
| Sensing transmitters and receivers properties |  | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | TRP monostatic and TRP bistatic as first priority | TRP monostatic and TRP bistatic as first priority | TRP monostatic and TRP bistatic as first priority |
| Sensing target | Outdoor/indoor | Outdoor | Outdoor | Outdoor |
|  | LOS/NLOS | LOS | LOS | LOS |
|  | 3D mobility | {30, 60, 160}km/h in horizontal plane, 0 km/h in vertical plane Other velocity values in horizontal plane are FFS | {30, 60, 160}km/h in horizontal plane, 0 km/h in vertical plane Other velocity values in horizontal plane are FFS | {30, 60, 160}km/h in horizontal plane, 0 km/h in vertical plane Other velocity values in horizontal plane are FFS |
|  | 3D distribution | Horizontal: uniform Height: options from 36.777 - Uniformly distributed between 1.5m and 300m - Fixed height value chosen from {50, 100, 200, 300} m - Optional up to 600m | Horizontal: uniform Height: options from 36.777 - Uniformly distributed between 1.5m and 300m - Fixed height value chosen from {50, 100, 200, 300} m - Optional up to 600m | Horizontal: uniform Height: options from 36.777 - Uniformly distributed between 1.5m and 300m - Fixed height value chosen from {50, 100, 200, 300} m - Optional up to 600m |
|  | Orientation | Random (direction of velocity) | Random (direction of velocity) | Random (direction of velocity) |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height) - Baseline:0.15m x 0.15m x 0.1m - Other sizes are optional. | Size (Length x Width x Height) - Baseline:0.15m x 0.15m x 0.1m - Other sizes are optional. | Size (Length x Width x Height) - Baseline:0.15m x 0.15m x 0.1m - Other sizes are optional. |
| Sensing area |  | Select one center sector |  |  |
| Minimum 3D distances between Tx and sensing target |  | 10m | 10m | 10m |
| Minimum 3D distances between Rx and sensing target |  | 10m | 10m | 10m |
| Minimum 3D distances between sensing targets |  | 10m | 10m | 10m |

Proposal 2: Support the following evaluation parameter for sensing scenarios for human indoor as sensing target

Table 3. Evaluation parameter for sensing scenarios for human indoor as sensing target

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Indoor factory | Indoor office |
| Cell layout |  | Refer to 38.857 | Refer to 38.901 |
| Sensing transmitters and receivers properties |  | Any TRP and/or UE location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP and/or UE location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | All 6 sensing modes | All 6 sensing modes |
| Sensing target | Outdoor/indoor | Indoor | Indoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | 3D mobility | 5km/h in horizontal plane (refer to 22.837) | 5km/h in horizontal plane (refer to 22.837) |
|  | 3D distribution | Uniform in horizontal plane | Uniform in horizontal plane |
|  | Orientation | Random (direction of velocity) | Random (direction of velocity) |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m |
| Sensing area |  | Indoor layout | Indoor layout |
| Minimum 3D distances between Tx and sensing target |  | 0m (2D) 3D distance can be inferred via the height of sensing transmitter and target | 0m (2D) 3D distance can be inferred via the height of sensing transmitter and target |
| Minimum 3D distances between Rx and sensing target |  | 0m (2D) 3D distance can be inferred via the height of sensing receiver and target | 0m (2D) 3D distance can be inferred via the height of sensing receiver and target |

Proposal 3: Support the following evaluation parameter for sensing scenarios for human outdoor as sensing target

Table 4. Evaluation parameter for sensing scenarios for human outdoor as sensing target

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | UMi | UMa |
| Cell layout |  | Hexagonal grid, 19 macro sites, 3 sectors per site ISD =200 m | Hexagonal grid, 19 macro sites, 3 sectors per site ISD =500 m |
| Sensing transmitters and receivers properties |  | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | All 6 sensing modes | All 6 sensing modes |
| Sensing target | Outdoor/indoor | Outdoor | Outdoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | 3D mobility | 5km/h in horizontal plane | 5km/h in horizontal plane |
|  | 3D distribution | Uniform in horizontal plane | Uniform in horizontal plane |
|  | Orientation | Random (direction of velocity) | Random (direction of velocity) |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m |
| Sensing area |  | Select one center sector |  |
| Minimum 3D distances between Tx and sensing target |  | 10m (2D) for BS-sensing target 3D distance can be inferred via the height of sensing transmitter and target | 35m (2D) for BS-sensing target 3D distance can be inferred via the height of sensing transmitter and target |
| Minimum 3D distances between Rx and sensing target |  | 10m (2D) for BS-sensing target 3D distance can be inferred via the height of sensing receiver and target | 35m (2D) for BS-sensing target 3D distance can be inferred via the height of sensing receiver and target |

Proposal 4: If UE is the sensing transmitter or sensing receiver in highway or urban grid scenario, consider one or more of the following UE types:

- Type 1: UE is deployed similar as 37.885

- Type 2: Deploy RSU as sensing transmitter or sensing receiver

Proposal 5: Support the following evaluation parameter for sensing scenarios for automotive vehicles as sensing target

Table 5. Evaluation parameter for sensing scenarios for automotive vehicles as sensing target

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Highway | Urban grid |
| Cell layout |  | Refer to 37.885 | Refer to 37.885 |
| Sensing transmitters and receivers properties |  | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | All 6 sensing modes | All 6 sensing modes |
| Sensing target | Outdoor/indoor | Outdoor | Outdoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | 3D mobility | up to 140km/h, refer to 37.885 | up to 60km/h, refer to 37.885 |
|  | 3D distribution | Refer to 37.885 In lane The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec} | Refer to 37.885 In lane The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec} |
|  | Orientation | Along the lane | Along the lane |
|  | Physical characteristics (e.g., size) | Type 1: length 5 meters, width 2.0 meters, height 1.6 meters Optional: truck/bus length 13 meters, width 2.6 meters, height 3 meters | Type 1: length 5 meters, width 2.0 meters, height 1.6 meters Optional: truck/bus length 13 meters, width 2.6 meters, height 3 meters |
| Sensing area |  | In lane | In lane |
| Minimum 3D distances between Tx and sensing target |  | Refer to 36.885: 37m for BS-sensing target distance (2D), 3D distance can be inferred via the height of sensing transmitter and target FFS distance between sensing target and UE | Refer to 36.885 for distance between BS and sensing target FFS distance between sensing target and UE |
| Minimum 3D distances between Rx and sensing target |  | Refer to 36.885: 37m for BS-sensing target (2D), 3D distance can be inferred via the height of sensing receiver and target FFS distance between sensing target and UE | Refer to 36.885 for distance between BS and sensing target FFS distance between sensing target and UE |

Proposal 6: Support the following evaluation parameter for sensing scenarios for AGV as sensing target

Table 6. Evaluation parameter for sensing scenarios for AGV as sensing target

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | Indoor factory |
| Cell layout |  | Refer to 38.857 |
| Sensing transmitters and receivers properties |  | Any TRP and/or UE location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | All 6 sensing modes |
| Sensing target | Outdoor/indoor | Indoor |
|  | LOS/NLOS | LOS and NLOS |
|  | 3D mobility | 5km/h in horizontal plane |
|  | 3D distribution | Uniform in horizontal plane |
|  | Orientation | Random (direction of velocity) |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height) 0.5m x 0.5m x 0.2m |
| Sensing area |  | Indoor layout |
| Minimum 3D distances between Tx/Rx and sensing target |  | 0m (2D) 3D distance can be inferred via the height of sensing transmitter/receiver and target |

Proposal 7: Sensing targets for objects creating hazards on roads/railways should at least include human.

Proposal 8: Support the following evaluation parameter for sensing scenarios for human on roads

Table 7. Evaluation parameter for sensing scenarios for human on roads

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Highway | Urban grid |
| Cell layout |  | Refer to 37.885 | Refer to 37.885 |
| Sensing transmitters and receivers properties |  | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations | Any TRP location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations |
| Supported sensing modes |  | All 6 sensing modes | All 6 sensing modes |
| Sensing target | Outdoor/indoor | Outdoor | Outdoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | 3D mobility | 5km/h in horizontal plane | 5km/h in horizontal plane |
|  | 3D distribution | Uniform in horizontal plane | Uniform in horizontal plane |
|  | Orientation | Random (direction of velocity) | Random (direction of velocity) |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m | Size (Length x Width x Height) 0.5m x 0.5m x 1.75m |
| Sensing area |  | In lane | In lane |
| Minimum 3D distances between Tx and sensing target |  | Refer to 36.885: 37m for BS-sensing target distance (2D), 3D distance can be inferred via the height of sensing transmitter and target FFS distance between sensing target and UE | Refer to 36.885 for distance between BS and sensing target FFS distance between sensing target and UE |
| Minimum 3D distances between Rx and sensing target |  | Refer to 36.885: 37m for BS-sensing target (2D), 3D distance can be inferred via the height of sensing receiver and target FFS distance between sensing target and UE | Refer to 36.885 for distance between BS and sensing target FFS distance between sensing target and UE |

Reference

- RP-234069 New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, Nokia, Nokia Shanghai Bell, 3GPP TSG RAN Meeting #102

- Chair notes RAN1#116bis eom0

- TS 22.137 Service requirements for Integrated Sensing and Communication; Stage 1 (Release 19)

- TR 22.837 Feasibility Study on Integrated Sensing and Communication (Release 19)

- TR 38.808 Study on supporting NR from 52.6 GHz to 71 GHz

- TS 36.777 Study on Enhanced LTE Support for Aerial Vehicles (Release 15)

- TR 38.901 Study on channel model for frequencies from 0.5 to 100 GHz

- TR 38.857 Study on NR Positioning Enhancements;

- TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR

- TR 36.885 Study on LTE-based V2X Services;