| 3GPP TSG RAN WG1 #117 | R1-2405089 |
| --- | --- |
| Fukuoka, Japan, May 20th – 24th, 2024 |  |

Agenda Item: 9.1.3.2

Source: MediaTek Inc.

Title:	Aditional study on AI/ML for CSI Compression

Document for: Discussion & Decision

# Introduction

R19 SI on AI/ML for CSI compression targets solving major obstacles ahead of adapting it as WI. This is not possible unless the issues are fully or partially resolved. In this regard, our efforts are concentrated on either designing new solutions or re-using existing ones in new scenarios/configs to reveal real potential of AI/ML models. Some of the important aspects are mentioned below.

Training feasibility and required logistics: There has been a discussion on pros and cons of different training types with no consensus on several aspects; however, there are some  common understandings that none of the existing types is universal, and their level of complexity and logistic requirement are totally different. First and foremost, the existing training types should be discussed in terms of feasibility and complexity; second, new solution may be explored in parallel to find a lightweight solution. Inspecting existing solutions also help to down select the method and reduce inter-operability test efforts. Using standardized AI/ML model ‎[3] will treat the complication of inter-vendor collaboration. However, it still needs to be discussed how to choose a specified model, and whether specifying it’s parameters are feasible or not.

Data collection: The data collection is another aspect to be finalized before R20. None of the solutions significantly addressed the major concerns around overhead. Possibility of using uplink CSI for training purposes, downlink CSI quantization level, and quantization method, all remained unanswered.

Complexity: The complexity of AI/ML models reported by companies during R18 SI indicate AI/ML models are far from practical deployments. This issue needs to be resolved during the R19 SI as one of the major barriers toward adoption of CSI compression as WI. Also, in RAN1#116‎[2], We started discussing combination of AI-based CSI compression and AI-based CSI prediction either by cascading two distinct models or designing a new model that works on TSF domain and perform both tasks jointly. This combination makes the complexity problem even more acute. If we design one model, the prediction task also will be involved in the two-sided training and temporal domain increases the size of shared datasets. If we cascade CSI compressions and prediction’s AI/ML model, this does not solve the complexity issue we had from R18. Nevertheless, it is crucial to study if the complexity reduction techniques could remarkably help or not. Using area-specific AI/ML models is another trajectory to improve complexity-performance tradeoff. We need more evaluation result in concluding its potential.

Evaluate effectiveness of complexity reduction techniques in reducing both computational and storage complexities of AI/ML models for CSI compression

Consider and evaluate eType II algorithm’s complexity as the baseline of computational complexity.

Conformance test: Given the large number of possible encoders and decoders from UE and NW vendors, it seems infeasible to test inter-operability of all in a reasonable time. This large number is impacted by number of vendors at both side, training types, number of scenarios/configs to be covered by AI/ML, etc. Thereby, this test space should be confined to a more reasonable size, thereby, ensuring  feasibility of the tests for RAN4. In RAN1 #116 ‎[2], we have initiated discussion on how to make the test burden reasonable by standardization of (a part of) AI/ML models with different levels of standardization, and option 1 in ‎[3] is regarded as beneficial solution for treating inter-vendor collaboration problem but it still may leas to many encoder-decoder pairs and large test burden.

# Temporal-spatial-frequency (TSF) domain compression

In RAN1#116, we have agreed on including temporal domain in CSI compression evaluations. How to use temporal information for compression and decompression varies on different possible designs and can be classified by agreed table below.

Agreement ‎[2] For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study: Case; Target CSI slot(s); Whether the UE uses past CSI information; Whether the network uses past CSI information. 0; Present slot; No; No. 1; Present slot; Yes; No. 2; Present slot; Yes; Yes. 3; Future slot(s); Yes; No. 4; Future slot(s); Yes; Yes. 5; Present slot; No; Yes. Note 1: For the UE, the past CSI information may include past model inputs and/or any information derived from them. For the network, the past CSI information may include past CSI feedback instances and/or any information derived from them. Note 2: For case 3 and case 4, the UE may perform prediction as a separate step or jointly with compression. Similarly, the network may perform prediction as a separate step or jointly with reconstruction. Companies to report which option is selected, the number of future slots, and whether the prediction is AI/ML-based or not. Note 3: “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds. “Present slot” refers to the slot of the most recent CSI-RS measurement used to generate the CSI report. “Future slot(s)” includes at least one slot after the present slot and may include the present slot as well. Note 4: Down-selection is not precluded.

We have evaluated case 0 as the baseline which is SF compression (see Figure ‎2-1(a)) and two possible scenarios for case 3: “case 3-AR” which uses auto-regression (AR) for CSI prediction and cascades it with AI/ML based CSI compression as shown in Figure ‎2-1(b); and “case 3-Ideal” which uses ideal CSI prediction and cascades it with AI/ML based CSI compression as shown in Figure ‎2-1(c). In both implementations we use 5 consecutive historical CSI samples starting from present CSI and use those 5 samples for prediction and CSI compression for the next 3 CSI samples in future.  We have also evaluated case 1 where 3 consecutive historical CSI samples are used, and target time slot is present slots.

- Case 0

(b) Case 3-AR: AR-based CSI prediction and CSI compression

(c) Case 3-Ideal: Ideal CSI prediction and CSI compression

Figure ‎2-1: Evaluated scenarios for TSF CSI compression

For test and train datasets, we followed assumptions agreed in RAN1#116 ‎[2]. The training dataset includes 400k CSI samples and test dataset includes 30k samples generated with 5ms periodicity, distribution option 1 (80% indoor, 20% outdoor) with 3km/h for indoor samples and 30km/h for outdoor samples. CSI reporting periodicity is also 5ms. The observation window length is 25ms which means five subsequent historical CSI samples will be used as input of AI/ML model in case 3-AR and case 3-Ideal. The further details can be found at Table ‎6-1 in the appendix.

Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following as baseline options for UE distribution: - Option 1: 80% indoor, 20% outdoor - Option 2: 100% outdoor Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 10 km/h, 20 km/h, 30 km/h, 60 km/h, 120 km/h. Assumption on O2I car penetration loss and spatial consistency follow the R18 AI based CSI prediction. Agreement For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following evaluation assumptions: - CSI-RS configuration - Periodic: 5 ms periodicity (baseline), 20 ms periodicity(encouraged) - Aperiodic (for cases with prediction): Optional, CSI-RS burst with K resources and time interval m milliseconds (based on R18 MIMO eType-II) - CSI reporting periodicity: {5, 10, 20} ms; other values are not precluded - For cases with the use of past CSI information, to report observation window, including number/time distance of historic CSI/channel measurements. - For cases with prediction, to report prediction window, including number/time distance of predicted CSI/channel.

We evaluate case 0 and case 3-AR and case 3-ideal in terms of SGCS performance. Also, represented complexity of corresponding  AI/ML models below.

Table ‎2-1: Performance and complexity of AI/ML models used in TSF CSI compression evaluations

| Case | Input slots | Output slots | SGCS-256bits | #Param [M] |  | #FLOPs [M] |  | Benchmark |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Enc. | Dec. | Enc. | Dec. | Type | Baseline result |
| Case0 | 1 (present) | 1 (present) | 0.864 (+2.2%) | 3.30 | 3.99 | 83.8 | 85.2 | R16 CB | PC5 - 0.838 PC6 - 0.853 |
| Case1 | 3 (past) | 1 (present) | 0.870 (+3.0%/+0.7%) (R16 CB/Case0) | 3.33 | 5.59 | 84 | 88 | R16 CB and case 0 |  |
| Case3-AR | 5 (past) | 3 (future) | 0.813 (+2.3%) | 3.36 | 9.81 | 85.5 | 98.4 | AR prediction + R18 CB | PC4 - 0.787/0.777/0.770 Avg. 0.778 PC5 - 0.822/0.811/0.804 Avg. 0.812 |
| Case3-Ideal | 3 (past) | 3 (future) | 0.847 (+2.4%) | 3.33 | 9.81 | 84.6 | 98.4 | Ideal prediction + R18 CB | PC4 - 0.808/0.810/0.808 Avg. 0.809 PC5 - 0.844/0.846/0.844 Avg. 0.845 |

We could observe case 0, case 1, case 3-AR and case 3-Ideal respectively show 2.2%, 1.8%, 2.3%, and 2.4% compared to its baselines on average over time slots and ParCombs. Also, we have examined feasibility of using AR for CSI compression and observed a good perf compared to ideal prediction before CSI compression. It only introduces 3.9% degradation.

In average, case 0, case 1, case 3-AR and case 3-Ideal respectively show 2.2%, 1.8%, 2.3%, and 2.4% compared to its baselines.

Autoregression based CSI prediction shows a promising performance (3.9% degradation compared to Ideal prediction) when it is cascaded by AI/ML CSI compression.

# Error tolerance of AI/ML models

In this section we test whether the errors in CSI feedback completely ruins AI/ML models’ performance or causes slight degradation. For this evaluation, we have assumed a certain number of bits in feedback will be flipped (0 to 1 or 1 to 0). The simulation assumptions are presented in Table ‎6-1. The performance of AI/ML models under various number of erroneous bits for the following three cases are evaluated.

Table ‎3-1: Evaluation cases for studying tolerance of AI/ML model against CSI feedback errors

|  | Training | Inference |
| --- | --- | --- |
| Case 0 | No error in training | No error in inference |
| Case 1 | No error in training | Unintentional error in inference |
| Case 2 | Intentional error in training | Unintentional error in inference |

Case 0 [Benchmark]: This is the benchmark where the AI/ML model is not exposed to any error in the CSI feedback at the training and inference stages.

Case 1 [Non-robust model]:  In this case, the AI/ML model is trained in conventional manner whit no CSI feedback error.  In the inference stage, it will be exposed to CSI feedback error.

Case 2 [Robust model]:  In this case, the AI/ML model is trained with intentional CSI feedback errors to learn how to leverage inter-relation of all bits in CSI feedback for accurate CSI reconstruction. In the inference, AI/ML model will be exposed to CSI feedback error too.

Table ‎3-2: Error tolerance of AI/ML models for case 0 (benchmark), case 1(non-robust model) and case 2 (robust model)

| Cases |  | 52 bits |  | 128bits |  | 256bits |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SGCS | Perf. loss | SGCS | Perf. loss | SGCS | Perf. loss |
| Case 0 |  | 0.726 | - | 0.806 | - | 0.869 | - |
| Case 1 | Err. bits = 1 | 0.687 | 5.4% | 0.792 | 1.7% | 0.865 | 0.5% |
|  | Err. bits ≤ 2 | 0.644 | 12.7% | 0.778 | 3.5% | 0.860 | 1.0% |
|  | Err. bits ≤ 5 | 0.526 | 27.5% | 0.731 | 9.3% | 0.846 | 2.6% |
| Case 2 | Err. bits = 1 | 0.693 | 4.5% | 0.798 | 1.0% | 0.871 | 0.0% |
|  | Err. bits ≤ 2 | 0.682 | 6.1% | 0.794 | 1.5% | 0.865 | 0.5% |
|  | Err. bits ≤ 5 | 0.647 | 10.8% | 0.758 | 6.0% | 0.852 | 2.0% |

Our results show that the model trained from case 1 has intrinsically error tolerance to some extent. Errors on few CSI feedback bits do not completely ruin performance of AI/ML models. If error only occurs in inference (case 1), on average for CSI feedback of 52 bits, 128bits, and 256 bits, the inference error causes 15.2%, 4.8%, and 1% SGCS degradation respectively.

Comparing the feedback of 52 bits with 1 error bit and feedback of 256 bits with 5 error bits, we observe for the same error rate, AI/ML models’ tolerance against feedback error enhances as the feedback overhead increases. We have already observed this trend for other values of error rates as well.

If error only occurs in inference (case 1), on average for CSI feedback of 52 bits, 128bits, and 256 bits, the inference error causes 15.2%, 4.8%, and 1% SGCS degradation respectively.

The following figure also depicts the impact of CSI feedback error on throughput of case 1 compared case 0 and eType II baseline with no training or inference error. Traffic is full buffer and max rank is 2. We can see even with few bits of error the AI/ML models work better than eType II  feedback. The gap between throughput gap from Case 1 and Case 0 is gradually closing  for higher overhead bits.

For the same error rates, AI/ML models’ tolerances against CSI feedback error enhances as the overhead increases.

Figure ‎3-1: Impact of AI/ML model’s error on mean throughput

Finally, we enhanced the error tolerance of AI/ML model by intentionally exposing it to errors in training stage and learning how to compensate the errors (flipped bits) in CSI feedback at the CSI reconstruction parts. The results are shown in the previous table where case 2 significantly outperform case 1 for all overheads lengths.  If error occurs in both training and inference (case 2), on average for CSI feedback of 52 bits, 128 bits, and 256 bits, the inference error causes 7.1%, 2.8%, and 0.8% SGCS degradation respectively.  It indicates introducing intentional error in CSI feedback at the training stage, makes AI/ML model more robust against CSI feedback errors.

If AI/ML model is reinforced with intentional feedback error in training, similar error in inference for CSI feedback of 52 bits, 128bits, and 256 bits causes 7.1%, 2.8%, and 0.8% SGCS degradation respectively

Evaluate the feedback error tolerance of eType II and compare it with that of AI/ML model.

# Collaborative Training and Feasible Inter-operability Test

To make inter-operability tests feasible, we have to limit the number of possible CSI generation and CSI reconstruction parts as discussed below.

Training: Training type 2 in both simultaneous and sequential forms comes with complex API requirement and excessive inter-vendor efforts; thereby making it the most difficult training method for practical deployment. Also, introducing new UE-side and/or NW-side AI/ML models raises more re-engineering burden, However, training type 3, is the sequential, and the vendors can still train their corresponding AI/ML models with minor additional logistics. NW-first and UE-first sequential separate training both are similar in terms of performance and engineering effort. We support its UE-first variant due to: 1) UE is data generation entity and has access to data required for enabling proxy-based monitoring; and 2) It has better match with the reference CSI generation parts.

Conclusion: - Conclude, from RAN1 perspective, that Option 1, if feasible for specification, eliminate the inter-vendor collaboration complexity (e.g., whether bilateral collaboration is required between vendors). - It is RAN1’s understanding that Option 1 corresponds to RAN4 options, e.g., RAN4-Option3, or RAN4-Option4. Further study and final conclusion on interoperability and RAN4 testing of the RAN4-Option3 and RAN4-Option4 is up to RAN4. Observation - Option 1 and 2 may have limited performance in the field compared to Options 3, 4, and 5, further study is needed - Option 1 and 2 may require high specification effort from RAN1 perspective. Conclusion - Deprioritize Option 2 for inter-vendor training collaboration. - Note: This deprioritization shall not affect the ongoing discussion in RAN4 on RAN4-Option3 and RAN4-Option4.

Standardization of AI/ML models: To further relax inter-operability test, one concrete step is setting reference AI/ML models in part or completely. During ‎[3], different options have been discussed and it is concluded that option 1 (Fully standardized reference model) fixes inter-vendor collaboration issue. This model serves as the reference AI/ML model, and it is not necessarily the one finally deployed in UE and NW. UE and NW uses the model to mimic its encoder/decoder mapping through a desired AI/ML model, or they perform re-engineering and optimization on top of it to fit available budget. Nevertheless, any deviation from the reference model may impact the performance and compatibility of encoder and decoder. To still ensure compatibility of two parts, there should be limit on the manipulation of reference model which will be represented by performance. Another unclear facet of this option is how area-specific AI/ML model can be implemented through option 1. On the other hand, option 3 (Standardized reference model structure + Parameter exchange between NW-side and UE-side) seems a good match for area-specific AI/ML models. The AI/ML model structure is specified, and its parameters will be trained in accordance with locally collected data at NW. The parameters can be later exchanged for re-engineering or inference at UE side. For the conformance test, however, there are many possible configs and scenario for each of which there will be one AI/ML model. The number can easily reach to hundreds if no downselection is made on possible scenario/configuration. At least as starting point, some candidate config/scenario seems necessary.

If downselection is needed, prioritize UE-first sequential separate training.

Discuss how option 1 and option 3 will ease inter-operability test efforts and also candidate scenarios/config as starting point of tests.

Discuss bounding performance deviation of AI/ML models derived from fully standardized AI/ML to ensure inter-vendor compatibility

If area-specific AI/ML models show promising performance, prioritize option 3 to make are-specific models feasible.

# Data Collection for Model Training

## Using SRS/Uplink CSI Collection  for Training NW-side Models

AI/ML model will be used in the inference stage to compress and decompress downlink (DL) CSI samples measured by UE using CSI-RS sent in the downlink. Does this necessarily mean only DL CSI samples collected from CSI-RS are useful for training? Downlink and uplink (UL) CSI samples share common semantic features. We will show an AI/ML model can be successfully trained using a dataset merely consists of UL CSI  and works well in the inference on DL CSI samples. We put one step further and show that the UL CSI can be collected in a frequency other than what is targeted for inference stage. This is because the AI/ML models for CSI compressions have natural ability of generalization over carrier frequency. To show that using UL CSI is possible for training, we have evaluated different approaches of NW-side data collection shown in the below figure.

Figure ‎5-1: Possible approaches for NW-side data collection for training

Approach 1 (DL CSI only): In the most popular approach, it is assumed that NW sends CSI-RS toward UE, and then UE measures and reports DL CSI. The reported DL CSI samples will be stored as a part of dataset for training purposes at NW.

Approach 2 (UL CSI only): We are proposing approach 2 where UE sends an SRS to NW, and NW itself measures and stores CSI samples; Albeit it will be UL CSI and it may be collected at a frequency different from target frequency in the inference stage.

Approach 3 (DL and UL CSI): This approach is essentially the mixture of previous approaches, where NW mainly relies on UL CSI in data collection, but it may receive DL CSI samples for finetuning or forming a mixed dataset.

We have evaluated the above approaches where SRS is sent with 200 MHz shift (2.2 GHz) from the frequency used for sending CSI-RS (2GHz) in the downlink. The following table shows our evaluation results which indicate feasibility of using UL CSI at NW side for training AI/ML models.

Table ‎5-1: Impact of different NW-side data collection methods on training AI/ML models

| NW Data collection | Description of training and dataset at NW | Test dataset | FB OH 52 bits | FB OH 128 bits | FB OH 256 bits |
| --- | --- | --- | --- | --- | --- |
| Approach 1 | 300k DL CSI samples | 30k DL CSI | 0.689 | 0.764 | 0.825 |
| Approach 2 | 300k UL CSI samples | 30k DL CSI | 0.694(-0%) | 0.755(-1.2%) | 0.813(-1.5%) |
| Approach 3 | 285k UL CSI, finetune with 15k DL CSI | 30k DL CSI | 0. 687(-0.0%) | 0. 756(-1.0%) | 0.815(-1.2%) |
|  | 150k+150k mixed UL/DL CSI | 30k DL CSI | 0.690(-0%) | 0.759(-0.6%) | 0.821(-0.5%) |

Using SRS for NW-side UL CSI collection is feasible even if inference is performed over DL CSI on a different carrier frequency. It only causes 0.9% SGCS degradation.

Using UL CSI for training and DL CSI for finetuning only causes 0.7% SGCS degradation on NW-side model compared to the case DL CSI samples are only used for training.

Using mixed UL and DL CSI samples for training for training NW-side AI/ML models causes 0.4% SGCS degradation compared to the case DL CSI samples are only used for training.

For NW-side AI/ML model training, NW can rely on UL CSI samples collected from SRS sent by UEs. Use of DL CSI can be limited to finetuning purposes.

# Conclusion

We have the following observation and proposals in this contribution:

- In average, case 0, case 1, case 3-AR and case 3-Ideal respectively show 2.2%, 1.8%, 2.3%, and 2.4% compared to its baselines.

Autoregression based CSI prediction shows a promising performance (3.9% degradation compared to Ideal prediction) when it is cascaded by AI/ML CSI compression.

If error only occurs in inference (case 1), on average for CSI feedback of 52 bits, 128bits, and 256 bits, the inference error causes 15.2%, 4.8%, and 1% SGCS degradation respectively.

For the same error rates, AI/ML models’ tolerances against CSI feedback error enhances as the overhead increases.

If AI/ML model is reinforced with intentional feedback error in training, similar error in inference for CSI feedback of 52 bits, 128bits, and 256 bits causes 7.1%, 2.8%, and 0.8% SGCS degradation respectively

Using SRS for NW-side UL CSI collection is feasible even if inference is performed over DL CSI on a different carrier frequency. It only causes 0.9% SGCS degradation.

Using UL CSI for training and DL CSI for finetuning only causes 0.7% SGCS degradation on NW-side model compared to the case DL CSI samples are only used for training.

Using mixed UL and DL CSI samples for training for training NW-side AI/ML models causes 0.4% SGCS degradation compared to the case DL CSI samples are only used for training.

We have also the following proposals:

- Evaluate effectiveness of complexity reduction techniques in reducing both computational and storage complexities of AI/ML models for CSI compression

- Consider and evaluate eType II algorithm’s complexity as the baseline of computational complexity.

- Evaluate the feedback error tolerance of eType II and compare it with that of AI/ML model.

- If downselection is needed, prioritize UE-first sequential separate training.

- Discuss how option 1 and option 3 will ease inter-operability test efforts and also candidate scenarios/config as starting point of tests.

- Discuss bounding performance deviation of AI/ML models derived from fully standardized AI/ML to ensure inter-vendor compatibility

- If area-specific AI/ML models show promising performance, prioritize option 3 to make are-specific models feasible.

- For NW-side AI/ML model training, NW can rely on UL CSI samples collected from SRS sent by UEs. Use of DL CSI can be limited to finetuning purposes.

# References

- 3GPP TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, Nov 2023.

- R1-2401563,  “Final summary for Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), RAN WG1 #116, Athens, Greece, Feb 26th – Mar 1st, 2024.

- R1-2403504, “Final summary for Additional study on AI/ML for NR air interface: CSI compression”, Moderator (Qualcomm), RAN WG1 #116bis, Changsha, China, April 15th – 19th, 2024

# Appendix

The following assumptions are used for evaluations in section ‎2.1. This EVM table is used for other evaluations as well unless otherwise is explicitly mentioned.

Table ‎8-1: Evaluation assumptions used for TSF CSI compression

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Dense Urban |
| Carrier Frequency | 4 GHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 |
| Antenna-port layouts at gNB | 32 ports: (8.8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenn -port layouts at UE | 4 Rx: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power | 44 dBm (20 MHz bandwidth) |
| BS antenna height | 25m |
| UE receiver noise figure | 9 dB |
| Subcarrier spacing | 30kHz |
| Simulation bandwidth | 20 MHz |
| MIMO scheme | MU-MIMO |
| CSI Feedback | Feedback period: 5ms Scheduling delay: 4 ms |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30 km/h) |
| Channel estimation | Ideal |