3GPP TSG RAN WG1 #118	 R1-2407131

Maastricht, NL, August 19th – August 23rd, 2024

Agenda item:       9.4.2.1

Source:	IIT Kanpur, Indian Institute of Technology Madras (IITM)

Title:	Discussion on General aspects of physical layer design of AIoT

Document for:     Discussion and Decision

# Introduction

In recent years, IoT has gained much attraction in the domain of wireless communications. It is expected that more things will be interconnected to increase productivity, efficiency, and the comfort of life and reduce the stress on the existing infrastructure. In most of the existing technologies, the devices are battery-powered, which have a limited life cycle or need to be charged manually. It is impossible to power all these devices or things through batteries, which leads to environmental issues and high maintenance costs.

Existing technologies are not suitable for meeting the requirements of the target use case. Hence, in Rel-18, an SA and RAN Plenary level study was conducted to study the use case and the requirements of a new IoT technology, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low-end IoT applications. In Rel-19, a study on a new device class called Ambient IoT (AIoT) is going on, which can meet the strict complexity and power consumption requirements of these use cases defined in TR 22.840.

In this contribution, we discuss the remaining issues for general aspects of physical layer design, including waveform and modulation, coding, and multiple access.

# Discussion

In the RAN1 #117 meeting, general aspects of physical layer design for AIoT devices were discussed.

In the last RAN1 #117 meeting [1], the following agreements were made for Ambient IoT device architectures as follows. Some issues remain to be further studied

Agreement Study the following regarding CP location/length determination for Method Type 1: - Alt 1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols - Alt 2: duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration - Companies are encouraged to clarify the CP removal method used and implementation aspects for the device - Evaluations are encouraged to be performed for a small value of M, e.g. 4 and a large value of M, e.g. 24, at least by comparison to the case where the CP length of each OFDM symbol is known by device - Companies should report the values of SFO, and SFO detection methods used in evaluations Agreement Study the following options regarding subcarrier orthogonality for Method Type 2: - Alt 1: Method Type 2 retains subcarrier orthogonality (i.e. CP copied from the end of an OFDM symbol) - Alt 1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same - FFS: whether this alternative applies if CP length is longer than the chip duration - Alt 1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP - Other potential methods are not precluded - Alt 2: Method Type 2 does not retain subcarrier orthogonality - Proponents to bring further details to RAN1#118 - Evaluations and discussions are encouraged to be performed for a small value of M, e.g. M = 4 and a large value of M, e.g. M = 24. - Companies should report the values of SFO, and SFO detection methods used in evaluations Agreement Define repetition types for study purposes as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times - NOTE: Equivalent to extending the duration of each chip by Rchip times Agreement For D2R, study at least block-level and bit-level repetition type 1 and type 2. Agreement For R2D evaluation purposes, the R2D waveform for DFT-s-OFDM is generated as follows: - The time domain OOK signal is the M chips of one OFDM symbol. - A chip is represented (e.g. upsampled) by L samples - Companies to report L - An N’-points DFT is performed on the samples of one OFDM symbol to obtain the frequency domain signal. - Companies to report N’, e.g. N’=128 or equal to X - Map the frequency domain signal obtained by N’-points DFT to the X subcarriers of Btx,R2D. - Companies report how to map and report X - An N-points IDFT is performed to obtain the time domain signal. - Companies to report N, and how value was selected Note: companies report whether/how CP samples are added. Agreement The study assumes the following bit to chip mapping for Manchester encoding: - bit 0→chips{10}, bit 1→chips{01} - FFS: Variant of the above for CP handling

For the Rel-19 Ambient IoT study, it is up to the working groups to decide the specific design requirements. To study the candidate physical layer signals, methods, and procedures, it is necessary to determine the KPIs for the AIoT device.

Proposal 1: RAN1 should define KPIs for further study and down selection on physical layer signals, methods, and procedures.

## 2.1 Waveform and Modulation

From the reader's perspective, a reader can work in two modes. The first one is the standalone mode, where the reader only serves the AIoT device. In this case, it is up to the reader's implementation to use an OFDM-based signal for an R2D link. In the second mode of operation, the reader is either a base station or an intermediate UE, where the reader is simultaneously serving legacy NR UEs and an AIoT UE. In this case, the reader is expected to use the same transmission chain to transmit the legacy NR signals and AIoT signals, an OFDM-based wave is required for the transmission on the R2D link. Handling the cyclic prefix (CP) for the AIoT receiver will be difficult. It will be mandatory for the AIoT device to be capable of CP identification.

### 2.1.1CP Handling

CP in R2D transmission can cause a false rising or falling edge, and if not handled, it can corrupt the transmission. In the RAN1#117 meeting, the following agreement was made to handle the CP-related issues.

Agreement Study the following regarding CP location/length determination for Method Type 1: - Alt 1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols - Alt 2: duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration - Companies are encouraged to clarify the CP removal method used and implementation aspects for the device - Evaluations are encouraged to be performed for a small value of M, e.g. 4 and a large value of M, e.g. 24, at least by comparison to the case where the CP length of each OFDM symbol is known by device - Companies should report the values of SFO, and SFO detection methods used in evaluations Agreement Study the following options regarding subcarrier orthogonality for Method Type 2: - Alt 1: Method Type 2 retains subcarrier orthogonality (i.e. CP copied from the end of an OFDM symbol) - Alt 1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same - FFS: whether this alternative applies if CP length is longer than the chip duration - Alt 1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP - Other potential methods are not precluded - Alt 2: Method Type 2 does not retain subcarrier orthogonality - Proponents to bring further details to RAN1#118 - Evaluations and discussions are encouraged to be performed for a small value of M, e.g. M = 4 and a large value of M, e.g. M = 24. - Companies should report the values of SFO, and SFO detection methods used in evaluations

In the case of the in-band operation of the reader, solutions for handling CP could be categorized between the reader-side solution and the device-side solution. Few of these solutions were discussed in the previous meetings.

Considering the device's complexity in mind it is preferred to implement the reader side solutions. Such as using separate transmitter chains for the NR and R2D transmission which is up to the reader implementation. It is expected at least for the in-band operation BS will use OOK-4 for R2D transmission, here a guard bit/bits at every OFDM symbol boundary could be used to ensure no false rising /falling edge at the OFDM symbol boundary due to CP. The no of guard bits or chips could be calculated as number of guard bits/chips=ceil(CP duration /bit or chip duration). In the case of fewer numbers of chips transmission per OFDM symbol (for a smaller value of M) may not affect the device RF-ED as the CP duration will be significantly smaller than the chip duration.

For the device side CP removal was also discussed in the previous meeting.

CP handling for the larger value of M is based on the device’s capability to drop CP no of samples, to implement such a solution for the AIoT operation SCS should be fixed and should be known to the device.

Observation 1: SCS should be fixed for the AIoT operation for device-side CP removal.

Proposal 2: Consider the following methods to handle the CP in R2D transmission.

Method for CP removal at the transmitter side.

Option 1: It could be handled on the reader side, where BS or UE acting as a reader node could use different Tx chains for the R2D signal generation and NR signal generation, leaving the issue of CP handling up to the reader’s implementation.

Option 2: In the case of M chip OOK-4-based waveform, the use of a guard bit at every OFDM symbol boundary can ensure no false rising /falling edge at the OFDM symbol boundary.

Option 3: Using OOK-1 waveform for OFDM-based transmission, i.e., when the single chip is transmitted per OFDM symbol. In this case, there would not be a transition within OFDM symbol duration.

Option 4: Some combination of option 2 and option 3.

Method for CP removal at the receiver side.

Option 1: The AIoT device could sample the R2D signal and discard the CP sample. Further study on how to convey the CP duration to the AIoT device.

Option 2: Not required for smaller M values e.g. 1,2.

### 2.1.2 Line Coding

Manchester and PIE are considered for R2D transmission each one of them has its advantages and disadvantages [3], where Manchester coding generates a self-clocking signal which helps in mitigating the effect of SFO at device reception. For AIoT R2D transmission, the mapping rule from bit to line-code codeword can be defined according to the coding scheme and code rate. Codewords of Manchester code and PIE with different code rates are shown as follows.

- For Manchester encoding:

  - 1/2 code rate: bit 0→ chips{01}, bit 1→chips{10}

  - 1/4 code rate:

    - A: bit 0→chips{0011}, bit 1→chips{1100}

    - B: bit 0→chips{0101}, bit 1→chips{1010}

  - 1/8 code rate:

    - A: bit 0→chips{00001111}, bit 1→chips{11110000}

    - B: bit 0→chips{01010101}, bit 1→chips{10101010}

- For PIE:

  - A: bit 0→ chips{10}, bit 1→chips{110}

  - B: bit 0→chips{110}, bit 1→chips{111110}

  - C: bit 0→chips{1011}, bit 1→chips{1111}

We studied the gains of different code rates for the R2D link using the Manchester coding.

Figure 1: BER Vs SNR for R2D link with different code rate

Observation 2: Using a lower code rate in R2D transmission can provide considerable gains which can further improve the R2D coverage.

Proposal 3: The following bit-to-chip mapping should be considered for Manchester and PIE coding to support multiple line code rates.

- For Manchester encoding:

  - 1/2 code rate: bit 0→ chips{01}, bit 1→chips{10}

  - 1/4 code rate:

    - A: bit 0→chips{0011}, bit 1→chips{1100}

    - B: bit 0→chips{0101}, bit 1→chips{1010}

  - 1/8 code rate:

    - A: bit 0→chips{00001111}, bit 1→chips{11110000}

    - B: bit 0→chips{01010101}, bit 1→chips{10101010}

- For PIE:

  - A: bit 0→ chips{10}, bit 1→chips{110}

  - B: bit 0→chips{110}, bit 1→chips{111110}

  - C: bit 0→chips{1011}, bit 1→chips{1111}

The following agreement on the study of different repetition types was made in the last meeting.

Agreement Define repetition types for study purposes as follows: - Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times - Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times - Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip times - NOTE: Equivalent to extending the duration of each chip by Rchip times

We studied bit and chip level repetition types for both R2D and D2R links and their effect on the link performance. We considered the following bit-to-chip mapping for the repetition study.

- For no repetition case

Single transmission  bits : [0 1] chips : [0 1 1 0]

- For repetition

  - Bit-level repetition where repletion was performed prior to the line coding

bits : [0 1]  repeated bits : [0 0 1 1]  chips : [0 1 0 1 1 0 1 0]

  - Chip-level repetition where repletion was performed after line coding

bits : [0 1]  chips : [0 0 1 1 1 1 0 0]

Observation 3: Bit-level repetition is the repetition of the codeword bit.

For a device with an RF-ED receiver, a signal with a longer chip duration helps in better edge detection especially lower ADC resolution.

Observation 4: Chip-level repetition increases the chip duration, which makes it a more suitable choice for the R2D transmission for device-side RD-ED receivers as it improves the edge detection at the receiver end.

Figures 2 and 3 show the effect of repetition on the R2D and D2R links. The following results confirm our observation

Figure 2: BER Vs SNR for chip and bit level repletion for R2D transmission

Figure 3: BER Vs SNR for chip and bit level repletion for D2R transmission

The above figure shows that chip-level repetition offers approximately 2-4 dB better link performance when compared to bit-level repetition. This improvement is due to the difference in chip duration between both the repetition schemes.  Additionally, chip-level repetition leads to a simpler receiver implementation

Proposal 4: Chip-level repetition should be considered compared to bit-level repetition for coverage enhancement.

The following table captures the effect of repetition and the M chips per OFDM signal on the data rate of the R2D and D2R links.

Table 1: Value(s) of M and the corresponding data rate for OOK-4

| M | Data rate (kbps) |  |  |
| --- | --- | --- | --- |
|  | Without code | With Manchester (R=1/2) | With Manchester (R=1/4) |
| 1 | 14 | 7 | 3.5 |
| 2 | 28 | 14 | 7 |
| 4 | 56 | 28 | 14 |
| 8 | 112 | 56 | 28 |

Observation 5: The target data rate using the Manchester code could be achieved with the lower M value and the higher repletion factor.

The

following agreement on the use of line coding for R2D transmission was made in the previous meetings.

Agreement For R2D, line codes studied are Manchester encoding and pulse-interval encoding (PIE). - FFS: Mapping(s) from bit(s) to line-code codewords - FFS: Time domain definition of e.g., chips and relation to OFDM symbols, resource allocation unit, etc.

Figure 4: BER Vs SNR for Manchester and PIE encoding for R2D transmission

Observation 6: PIE encoding may lead to a non-constant transmission length for the same information bits due to the different waveform durations for bit ‘1’ and bit ‘0’.

Observation 7: In Manchester, each ON/OFF has a duty cycle of 50% which could be used at the device to mitigate the SFO-related issues.

Given the power consumption of the Ambient IoT in the SID, a type 1 device has a peak power consumption of ~1 µW, and a type 2 device has a peak power consumption of a few hundred µW, and the device complexity is lower than the NB-IoT device. The last meeting discussed simple-to-implement line codes, such as Manchester code and Pulse Interval Encoding, which were considered to meet the strict power consumption and complexity requirements of AIoT devices.

In the Pulse Intervals encode (PIE), the bit 0 and bit 1 are transmitted with different lengths of signal duration. This leads to random signal transmission time and may not be suitable for high-traffic scenarios. Further, unlike PIE, Manchester decoding is not dependent on thresholding; the decoder design for Manchester coding is simpler based on comparing the left and right sides of the envelope.

Observation 8: PIE can ensure high reliability, though it is sensitive to synchronization errors.

Proposal 5: For the R2D link, the Manchester code should be prioritized for AIoT devices.

## 2.2 Multiple Access

The frequency domain, time domain, and code domain multiplexing in ambient IoT should be studied to cater to the high device density.

### 2.2.1 Multiple Access for R2D Link

According to the device architectures discussed in our companion contribution [2], considering the device complexity of device type 1, the passive device has no active RF components. It is designed to operate in a static frequency band. Device type 2 can have higher complexity and power consumption, enabling it to have active RF components that provide an additional degree of freedom. Further, type 2 devices could tune to various frequencies. In our understanding, both time division multiplexing TDM and frequency division multiplexing FDM should be considered to serve multiple AIoT devices in the network on the R2D link.

Proposal 6: Both Time Division Multiplexing and Frequency Division Multiplexing should be studied for the method of Multiple Access on the R2D link.

Given the complexity and power constraints of the AIoT device, decoding CDM-ed information would not be possible for the AIoT on the R2D link.

Observation 9: Code division multiplexing is unsuitable for the AIoT device on the R2D link.

### 2.2.2 Multiple Access for D2R Link

According to the device architectures discussed in our companion contribution [2], considering the device complexity of Device Type 1, the passive device has no active RF components. It is designed to operate in a static frequency band. Device type 2 can have higher complexity and power consumption, enabling it to have active RF components that provide an additional degree of freedom. Further, type 2 devices could tune to various frequencies. In our understanding, time division multiplexing TDM, frequency division multiplexing FDM, and code division multiplexing CDM should be considered for catering to multiple AIoT devices on the D2R link.

In the previous meeting, the following agreements were reached regarding multiplexing on the D2R link.

Agreement Study time-domain multiple access of D2R transmissions. Further details, including pros/cons, are FFS. Agreement Study frequency-domain multiple access of D2R transmissions, at least by utilizing a small frequency-shift in baseband. Further details, including pros/cons, are FFS. Agreement Whether code-domain multiple access is feasible and necessary for D2R transmissions for all devices is FFS.

For the contention-based AIoT transmission on the D2R link, TDM could work fine in low-device density, but as the device density increases, the inventory query cycle time will increase significantly, ultimately increasing latency. Slotted-Aloha access was agreed to be studied for AIoT, but in this approach, the device may need to decode the response corresponding to every slot period, which may impact the device availability due to the limited energy storage.

Proposal 8: At least slotted Aloha-based TMA should be considered for D2R transmission. Further studying the impact of the TDM approach on device availability.

To cater to high device density deployment, FDM can be considered either a small frequency shift using baseband processing or a larger frequency shift using dedicated hardware could be introduced. Further details on multiplexing devices need to be studied like whether the users to be multiplexed around a single carrier like a group or device will be modulated on separate carriers (CWs) to achieve FDM.

Proposal 9: Frequency Division Multiplexing should be considered for D2R transmission. Further studying the details of device multiplexing on the carrier.

Code division multiplexing could further increase the capacity of the system without increasing the device complexity significantly. An AIoT device could be programmed with a unique orthogonal code, which could be used to make transmitted data orthogonal. This approach could be especially beneficial in the case of contention-free D2R transmission.

Proposal 10: For contention-free D2R transmission, code domain multiplexing should be considered.

# Conclusion

This paper provides the following observations and proposals:

Observation 1: SCS should be fixed for the AIoT operation for device-side CP removal.

Observation 2: Using a lower code rate in R2D transmission can provide considerable gains which can further improve the R2D coverage.

Observation 3: Bit-level repetition is the repetition of the codeword bit.

Observation 4: Chip-level repetition increases the chip duration, which makes it a more suitable choice for the R2D transmission for device-side RD-ED receivers as it improves the edge detection at the receiver end.

Observation 5: The target data rate using the Manchester code could be achieved with the lower M value and the higher repletion factor.

Observation 6: PIE encoding may lead to a non-constant transmission length for the same information bits due to the different waveform durations for bit ‘1’ and bit ‘0’.

Observation 7: In Manchester, each ON/OFF has a duty cycle of 50% which could be used at the device to mitigate the SFO-related issues.

Observation 8: PIE can ensure high reliability, though it is sensitive to synchronization errors.

Observation 9: Code division multiplexing is unsuitable for the AIoT device on the R2D link.

Proposal 1: RAN1 should define KPIs for further study and down selection on physical layer signals, methods, and procedures.

Proposal 2: Consider the following methods to handle the CP in R2D transmission.

Method for CP removal at the transmitter side.

Option 1: It could be handled on the reader side, where BS or UE acting as a reader node could use different Tx chains for the R2D signal generation and NR signal generation, leaving the issue of CP handling up to the reader’s implementation.

Option 2: In the case of M chip OOK-4-based waveform, the use of a guard bit at every OFDM symbol boundary can ensure no false rising /falling edge at the OFDM symbol boundary.

Option 3: Using OOK-1 waveform for OFDM-based transmission, i.e., when the single chip is transmitted per OFDM symbol. In this case, there would not be a transition within OFDM symbol duration.

Option 4: Some combination of option 2 and option 3.

Method for CP removal at the receiver side.

Option 1: The AIoT device could sample the R2D signal and discard the CP sample. Further study on how to convey the CP duration to the AIoT device.

Option 2: Not required for smaller M values e.g. 1,2.

Proposal 3: The following bit-to-chip mapping should be considered for Manchester and PIE coding to support multiple line code rates.

- For Manchester encoding:

  - 1/2 code rate: bit 0→ chips{01}, bit 1→chips{10}

  - 1/4 code rate:

    - A: bit 0→chips{0011}, bit 1→chips{1100}

    - B: bit 0→chips{0101}, bit 1→chips{1010}

  - 1/8 code rate:

    - A: bit 0→chips{00001111}, bit 1→chips{11110000}

    - B: bit 0→chips{01010101}, bit 1→chips{10101010}

- For PIE:

  - A: bit 0→ chips{10}, bit 1→chips{110}

  - B: bit 0→chips{110}, bit 1→chips{111110}

  - C: bit 0→chips{1011}, bit 1→chips{1111}

Proposal 4: Chip-level repetition should be considered compared to bit-level repetition for coverage enhancement.

Proposal 5: For the R2D link, the Manchester code should be prioritized for AIoT devices.

Proposal 6: Both Time Division Multiplexing and Frequency Division Multiplexing should be studied for the method of Multiple Access on the R2D link.

Proposal 8: At least slotted Aloha-based TMA should be considered for D2R transmission. Further studying the impact of the TDM approach on device availability.

Proposal 9: Frequency Division Multiplexing should be considered for D2R transmission. Further studying the details of device multiplexing on the carrier.

Proposal 10: For contention-free D2R transmission, code domain multiplexing should be considered.

# 4. References

[1] 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #117, Fukuoka City, Fukuoka, Japan May 20th – May 25th, 2024.

[2] 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #116bis, Changsha, China, April 15th – April 19th, 2024.

[3] EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard