3GPP TSG RAN WG1 #118	R1-2406915

Maastricht, NL, August 19th – 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on CAPC value for PSFCH+S-SSB for SL-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For SL transmissions including PSCCH/PSSCH, when Type 1 channel access is used for COT initiation, the CAPC value is determined based on PSCCH/PSSCH as defined in 38.300. For SL transmissions including only PSFCH or only S-SSB, when Type 1 channel access is used for COT initiation, the CAPC value is always 1. However, this does not mean that PSFCH transmission itself is assocaited with CAPC value = 1 or S-SSB transmission itself is associated with CAPC value = 1. As a result, CAPC value for PSFCH transmission + S-SSB transmission cannot be determined. For instance, at slot n, PSFCH is transmitted, and then S-SSB transmission is transmitted at slot n+1. There is no other following transmissions. Clear rule to initiate a COT for this case should be added in spec. Note that a CAPC value is associated with a PSCCH/PSSCH based on 16.9.9.2 of 38.300. Meanwhile, there is no clear description of CAPC association for each of PSFCH and S-SSB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PSFCH is associated with CAPC value = 1. S-SSB is associated with CAPC calue = 1. With these changes, CAPC value for PSFCH + S-SSB is 1, based on the following description. When a UE applies Type 1 channel access procedure to initiate a channel occupancy for multiple SL transmissions over one slot or multiple consecutive slots, the highest CAPC value among the associated CAPC values with the multiple SL transmissions is used for performing the Type 1 channel access procedure.. |
| Consequences if not approved: |  | CAPC value for this case is undefined and therefore UE does not determine CAPC value for this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.5	Sidelink channel access procedures

A UE operating in sidelink resource allocation mode 1 or mode 2 and performing SL transmission(s) on channel(s) shall perform the procedures described in this clause for the UE to access the channel(s) on which the transmission(s) are performed.

In this clause, transmissions from a UE are considered as separate SL transmissions, irrespective of having a gap between transmissions or not, and XThresh for sensing is adjusted as described in clause 4.5.5 when applicable.

A UE can access a channel on which SL transmission(s) are performed according to one of Type 1 or Type 2 SL channel access procedures as described in clauses 4.5.1 and 4.5.2, respectively.

When a UE applies Type 1 channel access procedures to transmit SL transmission(s), the applicable channel access priority class (CAPC) is defined in Table 4.5-1.

When a UE applies Type 1 channel access procedures to transmit SL transmission(s) including PSSCH with user plane data and associated PSCCH, the UE determines the corresponding SL channel access priority class p  in Table 4.5-1 following the procedures described in Clause 16.9.9.2 in [9].

A PSFCH transmission or a S-SSB transmission is associated with the channel access priority class p=1 in Table 4.5-1. When a UE applies Type 1 channel access procedures to transmit SL transmission(s) including only PSFCH or only S-SSB transmission(s), the UE shall use the channel access priority class p=1 in Table 4.5-1.

A UE shall not transmit on a channel for a Channel Occupancy Time that exceeds Tslmcot,p where the channel access procedure is performed based on the channel access priority class p  associated with the UE transmissions, as given in Table 4.5-1.

When a UE applies Type 1 channel access procedure or multi-channel access procedure to initiate a channel occupancy for multiple SL transmissions over one slot or multiple consecutive slots, the highest CAPC value among the associated CAPC values with the multiple SL transmissions is used for performing the Type 1 channel access procedure or multi-channel access procedure.

If a UE fails to access the channel(s) prior to an intended SL transmission(s), Layer 1 notifies higher layers about the channel access failure and the channel(s) that the UE fails to access.

<Unchanged parts omitted>