3GPP TSG RAN WG1 #118		 	R1-2407222

Maastricht , NL, Aug 19th – 23rd, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on NR-NTN uplink capacity/throughput enhancement

Agenda Item:	9.11.3

Document for: 	Discussion/Decision

Introduction

Following objective was included in Rel-19 NR NTN WID regarding UL capacity/throughput enhancement as updated in RAN#104 meeting [1].

Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution, our views on DFT-s-OFDM PUSCH enhancements via Orthogonal Cover Codes (OCC) for UL capacity/throughput enhancement for FR1-NTN are provided.

Discussion

For Rel-19 NR NTN study, the capacity performance on uplink is to be optimized through multiplexing techniques, where DFT-s-OFDM PUSCH enhancements via OCC is mainly considered. Whether/how to support DFT-s-OFDM PUSCH enhancements via OCC for UL capacity/throughput enhancement for FR1-NTN is discussed from following aspects.

Potential OCC techniques

Following agreement was made in RAN1#117 meeting [2]:

Agreement For the normative phase, at least one of the OCC techniques will be specified: - Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 or 4 - Inter-symbol(s) time domain OCC with OCC length 2 or 4 - Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4 - FFS Combination of OCC techniques including multiplexing of 8 UEs - FFS Use of OCC techniques with TBoMS - FFS Backward compatibility with non-Rel-19 UEs Conclusion OCC with PUSCH can support at least multiplexing of 2 or 4 UEs and achieve up to 2 or 4 times capacity gains respectively, when repetitions are used. Note: the actual gain may be less due to e.g. intra/inter cell interference

For Rel-19 NTN UL capacity enhancement study, it was agreed to support at least one of the following OCC techniques: Inter-slot time-domain OCC with PUSCH repetition Type A, Inter-symbol(s) time domain OCC, and Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4). It was also concluded in the last meeting that all the OCC techniques can support multiplexing of 2 or 4 UEs and achieve up to 2- or 4-times capacity gains with negligible BLER performance degradation. Thus, further down selection should be made mainly based on the specification impact and design/implementation complexity for each OCC technique. The pros and cons of each OCC techniques are analyzed and summarized as follows.

Table I: Pros and Cons of different OCC techniques

| OCC type | Pros | Cons |
| --- | --- | --- |
| Inter-slot time domain OCC | Not impacted by large delay spread | Degradation by large doppler spread if channel is not constant |
|  | Less/small impact on OCC design considering UCI multiplexing, TBS determination | Only applicable for PUSCH repetition or TBoMS case; Impact on OCC design considering inter-slot FH |
| Inter-symbol time domain OCC | Not impacted by large delay spread | May have degradation by large doppler spread if channel is not constant |
|  |  | Impact on OCC design considering FH/ UCI multiplexing /TBS determination |
| Intra-symbol pre-DFT OCC | Not impacted by large doppler spread | May have degradation by large delay spread |
|  | Less spec impact considering FH | Impact on OCC design consider UCI multiplexing /TBS determination |

For time-domain OCC (i.e., both inter-slot and inter-symbol), the UL performance may not be impacted by large delay spread and may degrade due to large doppler spread if channel is not constant. For intra-symbol pre-DFT OCC, the UL performance may not be impacted by large doppler spread and may degrade due to large delay spread if channel is not constant. The impact on performance of PUSCH when different OCC schemes are applied can be compared based on simulation results.

On the other hand, the design complexity and specification effort of different OCC schemes should be considered. For inter-slot OCC, the impact on OCC design considering inter-slot frequency hopping is relatively small, besides, PUSCH transmission with repetition is typical in NTN scenario, which means there’s no clear drawbacks for inter-slot OCC scheme for PUSCH. Besides, there’s no specification impact on TBS determination, and the issue in UCI multiplexing can be resolved with small design complexity and specification effort. For intra-symbol pre-DFT OCC schemes, the issues in UCI multiplexing and TBS determination can be resolved with small design complexity and specification effort, and the OCC design is not impacted by frequency hopping. For inter-symbol OCC, considering impact of intra-slot frequency hopping, UCI multiplexing and TBS determination for OCC design, the mechanisms are complicated and large specification effort is required. Thus, inter-symbol OCC scheme is not preferred, and at least one of inter-slot time-domain OCC and intra-symbol pre-DFT OCC scheme can be supported for PUSCH.

Proposal 1:

Support at least one of following OCC techniques with code length 2 or 4:

- Inter-slot time-domain OCC with PUSCH repetition Type A and/or TBoMS

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4)

Regarding combination of OCC techniques, the disadvantages of both time domain and frequency domain OCC should be jointly considered, and the design complexity may be large considering the DMRS structure. The main motivation of combination of OCC techniques will be multiplexing of more than 4 UEs. However, for multiplexing of 8 UEs, the DMRS structure becomes NOT single-symbol DMRS but double-symbol DMRS in the current specification. When a front-loaded DMRS and an additional DMRS are assumed, the number of DMRS symbols in the transmission is 4 instead of 2. This means that the number of symbols available for UL-SCH is reduced and large BLER performance degradation is expected. Thus, the combination of OCC techniques should be deprioritized.

Proposal 2:

Combinations of OCC techniques is not supported.

Regarding TBoMS for PUSCH with OCC, if inter-symbol time domain OCC or intra-symbol pre-DFT OCC is applied, no additional specification impact and design complexity is introduced. Also, for inter-slot time domain OCC, TBoMS is applicable. Appropriate OCC mapping discussed in the next section can improve BLER performance compared to non-optimal OCC mapping case and repetition without TBoMS case. Then, there is no reason to preclude TBoMS.

Proposal 3:

Support OCC for PUSCH with TBoMS.

Mapping of OCC

Following agreement was made in RAN1#116bis meeting:

Agreement RAN1 to at least further study the potential specification aspects on OCC techniques: - TBS calculation / Rate matching - UCI multiplexing - RV cycling across repetitions - Frequency hopping, e.g. intra /inter slot - OCC indication/configuration - Power control - FFS other aspects

The mapping of OCC for the potential OCC techniques should be studied considering the abovementioned aspects.

Regarding the inter-slot time-domain OCC, how to assign each bit of an OCC sequence (e.g., +1 from [+1, +j, -1, -j]) to repetitions should be studied. For example, if OCC length equals to the repetition factor, each repetition is assigned with one bit of OCC sequence; if OCC length is smaller than repetition factor, mapping of the OCC bits and repetitions should be studied, as illustrated below.

Fig1. Example of mapping of inter-slot OCC

Moreover, the impact of RV needs to be considered, i.e., the slots which mapped to one OCC sequence shall use same RV to ensure orthogonality.

Regarding TBoMS for PUSCH with inter-slot time domain OCC, following two options as shown in Fig2 can be considered regarding the mapping of the OCC bits and TBoMS repetitions.

Fig2. Example of mapping of inter-slot OCC in case of TBoMS

Link level simulation is performed, and the simulation assumptions are shown in appendix. The three cases of inter-slot OCC length 2 without TBoMS, with TBoMS mapping option1 and with TBoMS mapping option 2 are evaluated, and the simulation results for 2-slot TBoMS and 4-slot TBoMS are shown in Fig3 (a) and (b), respectively. For option 1, there’s no additional specification impact and design complexity. However, it can be found that in case of 2-slot TBoMS, OCC with TBoMS mapping option 1 will be over 3 dB worse than OCC without TBoMS at 2% BLER, and in case of 4-slot TBoMS, OCC with TBoMS mapping option 1 is around 3 dB worse than OCC without TBoMS at 2% BLER. The reason is option 1 suffers from larger impact of doppler spread as well as large phase variation due to frequency offset, and the BLER performance will be worse. For option 2, it can achieve equal or even slightly better performance compared to the inter-slot OCC without TBoMS case, which means that TBoMs with appropriate OCC mapping, i.e., option 2, experiences the same impact of doppler spread and phase variation due to frequency offset as the case without TBoMS and thereby it can improve BLER performance compared to non-optimal OCC mapping case. It should be mentioned that more TBoMS gain should be observed when some simulation parameters are different. Thus, option 2 mapping should be supported when PUSCH with inter-slot OCC applies TBoMS at the same time.

- OCC length2, 2-slot TBoMS, 2 repetitions           (b) OCC length2, 4-slot TBoMS, 2 repetitions

Fig3.Performance of inter-slot OCC in case of TBoMS

Observation 1:

For inter-slot OCC length 2 with TBoMS option1, in case of 2-slot TBoMS, the BLER performance will be over 3 dB worse than OCC without TBoMS at 2% BLER; in case of 4-slot TBoMS, the BLER performance is around 3 dB worse than OCC without TBoMS at 2% BLER.

Observation 2:

For inter-slot OCC length 2 with TBoMS option 2, the BLER performance is equal to or even slightly better compared to the inter-slot OCC without TBoMS case.

Proposal 4:

For PUSCH inter-slot time-domain OCC, the inter-slot OCC is assigned to repetitions as follows:

- If OCC length = repetition factor, each repetition is assigned with one bit of OCC sequence

- If OCC length < repetition factor, study the mapping of the OCC bits and repetitions

  - E.g., for OCC length = 4 and repetition=8, the mapping of the OCC bits can be [w0 w1 w2 w3 w0 w1 w2 w3], or [w0 w0 w1 w1 w2 w2 w3 w3]

- The slots which are associated with one OCC sequence (i.e., [w0 w1 w2 w3] ) are assigned with the same RV.

Proposal 5:

For inter-slot time-domain OCC with TBoMS, discuss how to assign OCC sequence.

For PUSCH with inter-slot time domain OCC, legacy TBS determination can be applied without enhancement. Considering UCI multiplexing, in legacy, when PUCCH is transmitted without repetition, and the PUCCH is overlapped with PUSCH, UCI in the PUCCH is multiplexed on the PUSCH only in the overlapped slot. To ensure the orthogonality, enhancements are necessary for conditions to perform UCI multiplexing for PUSCH with OCC, e.g., UCI is multiplexed on the PUSCH in the overlapped slot and spread to PUSCH with the same set of OCC code as overlapped slot, as illustrated in Fig4.

Fig4. Example of UCI multiplexing for PUSCH with inter-slot OCC of code length 2 [w0 w1]

Proposal 6:

For UCI multiplexing of PUSCH with inter-slot time-domain OCC, UCI is multiplexed on the PUSCH in an overlapped slot and on other PUSCHs that are associated with the same set of OCC code as the overlapped slot.

Regarding intra-symbol pre-DFT OCC, the OCC sequence can be assigned across subcarriers or across RBs. As code length of 2 or 4 is supported, following mapping alternatives can be considered:

Alt1. OCC assigned within a PRB, where a code of an OCC sequence (e.g., +1 or -1) is assigned for each set of subcarriers which consist of 6 or 3 subcarriers (divisor of 12). In this case, there is no need to specify the maximum number of RBs since there is no impact on OCC design. Although scheduling with more PRBs with OCC is not meaningful, decision is up to NW scheduler.

Alt2. OCC assigned across PRBs, where a code of an OCC sequence (e.g., +1 or -1) is assigned for each RB or each set of subcarriers with non-divisor of 12 (e.g., 9). For example, when 4 PRBs are scheduled with OCC length 4, each code of an OCC sequence is applied to each PRB. For this alt, the maximum number of RBs should be restricted since it has impact on OCC design in specification. Basically, OCC is used for better capacity but OCC design such as the exampled one does not bring any benefit compared to simple FDMA. 1 PRB scheduling per UE is enough rather than 4 PRBs scheduling with OCC length 4 for 4 UEs.

Alt1 is preferable for simpler specification impact, and the motivation to support Alt2 is unclear.

Proposal 7:

For PUSCH with intra-symbol pre-DFT OCC, a code of an OCC sequence (e.g., +1 or -1) is assigned within a PRB and assigned for each set of subcarriers which consists of 6 or 3 subcarriers.

For PUSCH with intra-symbol pre-DFT OCC, frequency hopping can be applied without enhancement. Meanwhile, the impact on TBS determination and UCI multiplexing should be studied, e.g., with intra-symbol OCC the number of subcarriers for PUSCH in TBS determination and UCI multiplexing can be considered as (NSCPUSCH/M) instead of NSCPUSCH, where M is OCC length. Then after mapping data/UCI for the first set of (NSCPUSCH/M) subcarriers, block-wise spreading can be applied for the remaining set(s) as in PUCCH generation for PUCCH format 4, as illustrated in Fig5.

Fig5. Example of PUSCH with intra-symbol OCC of code length 2 [w0 w1]

Proposal 8:

For PUSCH with intra-symbol pre-DFT OCC, in TBS determination and UCI multiplexing, NSCPUSCH is replaced by NSCPUSCH/M, where M is OCC length.

- Data and (if any) UCI are mapped to the first set of (NSCPUSCH/M) subcarriers, and then block-wise spreading is performed for the remaining set(s).

Orthogonal code generation

The OCC can be generated by various of techniques, e.g., Walsh-Hadamard matrix, DFT sequence , etc., to ensure orthogonality and minimize cross-correlation between OCC codes assigned to different users. For OCC for PUSCH, the design of OCC generation for DMRS can be considered as baseline. In current specifications, Walsh matrix is applied to generate OCC for DMRS for PDSCH with benefits of simple design. For example, with OCC length of 4, the OCC generated by Walsh matrix can be each row of following matrix.

A (4) = 111-1111-1111-1-1-1-11

Meanwhile, for PUSCH DMRS, Walsh matrix is applied to OCC length of 2 and DFT sequence is applied to OCC length of 4. The reason of adopting DFT sequence is less implementation complexity, better robustness against large delay spread, etc. For example, with OCC length of 4, the OCC generated by cyclic shift can be each row of following matrix.

B (4) = 1jj-1-j-1-j1  -1-j -j   1j1j-1

In our view, both of the sequences have benefits and either one of the legacy sequence i.e., Walsh sequences and DFT sequence as defined in TS38.211 can be adopted as baseline to generate OCC for PUSCH in Rel-19 NTN.

Proposal 9:

Legacy sequences (i.e., Walsh sequences and DFT sequence as defined in TS38.211) are used as baseline for generating OCC for PUSCH.

  - Determination of applied OCC

Regarding the determination of applied OCC, the length of OCC (i.e., 2 or 4) can be configured/indicated by the NW. For OCC with length N, N OCC sequences can be predefined and each of them is defined with an index. Which OCC index is allocated for each PUSCH transmission also can be configured/indicated by NW. As multiple values can be supported for the OCC related parameters (e.g., OCC length, OCC index), dynamic indication has benefits of flexibility, while leads to larger overhead. Thus, whether the indication of OCC related parameters is semi-static or dynamic can be studied.

Proposal 10:

Study whether the indication of OCC related parameters, e.g., OCC length, OCC index, is performed in a semi-static manner or in a dynamic manner.

Conclusion

Observation 1:

For inter-slot OCC length 2 with TBoMS option1, in case of 2-slot TBoMS, the BLER performance is over 3 dB worse than OCC without TBoMS at 2% BLER; in case of 4-slot TBoMS, the BLER performance is around 3 dB worse than OCC without TBoMS at 2% BLER.

Observation 2:

For inter-slot OCC length 2 with TBoMS option 2, the BLER performance is equal to or even slightly better compared to the inter-slot OCC without TBoMS case.

Proposal 1:

Support at least one of following OCC techniques:

- Inter-slot time-domain OCC with PUSCH repetition Type A

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4)

Proposal 2:

Combinations of OCC techniques is not supported.

Proposal 3:

Support OCC for PUSCH with TBoMS.

Proposal 4:

For PUSCH inter-slot time-domain OCC, the inter-slot OCC is assigned to repetitions as follows:

- If OCC length = repetition factor, each repetition is assigned with one bit OCC

- If OCC length < repetition factor, study the mapping of the OCC bits and repetitions

  - E.g., for OCC length = 4 and repetition=8, the mapping of the OCC bits can be [w0 w1 w2 w3 w0 w1 w2 w3], or [w0 w0 w1 w1 w2 w2 w3 w3]

- The slots which are associated with one OCC sequence (i.e., [w0 w1 w2 w3]) are assigned with same RV.

Proposal 5:

Support TBoMS for inter-slot time domain OCC.

Proposal 6:

For UCI multiplexing of PUSCH with inter-slot time-domain OCC, UCI is multiplexed on the PUSCH in the overlapped slot and spread to PUSCH with the same set of OCC code as overlapped slot.

Proposal 7:

For PUSCH with intra-symbol pre-DFT OCC, a code of an OCC sequence (e.g., +1 or -1) is assigned within a PRB and assigned for each set of subcarriers which consists of 6 or 3 subcarriers.

Proposal 8:

For PUSCH with intra-symbol pre-DFT OCC, in TBS determination and UCI multiplexing, NSCPUSCH is replaced by NSCPUSCH/M, where M is OCC length.

Proposal 9:

Legacy sequences (i.e., Walsh sequences and DFT sequence as defined in TS38.211) are used as baseline for generating OCC for PUSCH.

Proposal 10:

Study whether the indication of OCC related parameters, e.g., OCC length, OCC index, is performed in a semi-static manner or in a dynamic manner.

References

[1] RP- 241659 , “ Moderator summary on Rel-19 NR NTN”, Moderator (RAN1 Vice-Chair), Jun. 2024.

[2] Draft Report of 3GPP TSG RAN WG1 #117, R1-2405685, “Feature lead summary #5 of AI 9.11.3 on NR-NTN uplink capacity and throughput”, Moderator (MediaTek), May 2024

- Appendix: Link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Center frequency | 2 GHz |
| SCS | 15 kHz |
| Channel model | NTN-TDL-C |
| UE speed | 3 km/h |
| PRB allocation | 2 |
| PUSCH mapping type | Type A with 14 OFDM symbols |
| DM-RS configuration type | Type 1 |
| DM-RS Length | 1 |
| DM-RS symbol | [2, 11] |
| FDM b/w data and DMRS | No |
| TBS | 184 |
| Modulation | QPSK |
| Repetition and TBoMS | w/o TBoMS: 4, 8 repetitions w/ TBoMS: 2 repetitions of 2, 4-slot TBoMS |
| OCC length | 2 (i.e., 2 UEs multiplexing) |
| OCC sequence | Walsh sequences |
| TO | Uniform selection from [-0.94us, 0.94us], where 0.94us=29Ts |
| FO | Uniform selection from [-0.1 ppm, +0.1 ppm], Variation of frequency error is negligible. |
| Channel estimation | Real |