3GPP TSG-RAN WG1 Meeting #118	R1-2407244

Maastricht, Netherlands, August 19 - 23, 2024

Agenda Item:	8.1

Source:	Moderator (Huawei)

Title:	FL summary#3 for AI 8.1 SL-U physical channel design framework (EOM)

Document for:	Discussion and decision

# Introduction

The latest R18 WID on sidelink evolution (RP-232789) includes the following objective regarding support of sidelink on unlicensed spectrum (SL-U):

- Study (it was concluded that the principle of NR-U channel access mechanisms is feasible for SL) and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

This contribution provides discussions related to SL-U physical channel design framework (AI 8.1), including summary of contributions, FL’s proposals, discussions, outcome of this meeting, etc.

[118-R18-Maintenance] To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc – Chair

# Issues

## Issue#1: SL bandwidth part and resource pool

### Issue#1-1: Determination of sidelink symbol for SL-U

Background:

Summary

  - CATT R1-2406337 gave a draft CR on determination of sidelink symbol for SL-U, where CATT proposes that in TS 38.214 clause 8.1.2.1, clarify that the sidelink symbols are determined by higher layer parameter sl-StartingSymbolFirst and sl-LengthSymbols if sl-StartingSymbolFirst and sl-StartingSymbolSecond are provided.

FL’s view

  - A proposal is given to reflect the above.

Proposal 1-1

Draft CR#1-1 in Section 4.5 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM | NO | No difference is found. |
| NEC | Yes | Support. The CR clarify the stating symbol when 2 candidate starting symbols are configured |
| CATT/CICTCI | Yes | The CR is to clarify when two starting symbols are configured, the sidelink symbols should be determined by higher layer parameter sl-StartingSymbolFirst rather than sl-StartSymbol. |
| vivo | comment | The CR seems too lengthy and a little redundant to other places in the spec. Maybe it is better to simply list the parameters here and to refer to the RRC spec for the meaning. |
| LGE | OK |  |
| OPPO | Yes |  |

## Issue#2: Slot structure

No issues.

## Issue#3: PSCCH/PSSCH

### Issue#3-1: PSSCH decoding behavior

Background:

Summary

  - OPPO R1-2406217 gave a draft CR for correction of PSSCH decoding behavior, where OPPO proposes to add the condition for UE to perform decoding PSSCH transmission starting from the second candidate starting symbol based on associated SCI formats 2-A, 2-B, 2-C and 2-D detection results. This is captured as Draft CR#3-1A below.

  - ZTE R1-2406678, Huawei R1-2405864, Docomo R1-2406917 analysed same issue, and proposed different draft CRs, which are captured as Draft CR#3-1B, 3-1C, 3-1D below, respectively.

FL’s view

  - A proposal is given to reflect the above.

Proposal 3-1

Down-select one of following:

Alt 1: Draft CR#3-1A in Section 4.1 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Alt 2: Draft CR#3-1B in Section 4.2 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Alt 3: Draft CR#3-1C in Section 4.3 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Alt 4: Draft CR#3-1D in Section 4.4 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM |  | Note that Alt 4 is discussing a (relevant but) different issue, which is PSCCH monitoring behavior for sensing. |
| NEC | Alt.1 | Alt2/3: "within a resource pool" has no necessity to be repeated everywhere, other changes are similar to Alt.1, we are OK with either. Alt.4: These behaviors regarding 2 candidate starting symbols is defined in PSSCH decoding behaviors but not sensing procedure. |
| CATT/CICTCI | Alt.1+Alt.3 | Prefer to merge Alt.1 and Alt.3. The former add the condition that the decoding should be based on the detected SCI and PSSCH resource configuration, while the latter emphasize not only the PSSCH transmission starting from second starting symbol should be decoded, the PSSCH transmission starting from first starting symbol should also be decoded. Both modifications are valid. We suggest the following wording: In any slot without PSFCH symbols within a resource pool, the UE attempts to decode PSSCH transmission starting from the first candidate starting symbol provided by sl-startingSymbolFirst, and attempts to decode, subject to UE capability, PSSCH transmission starting from the second candidate starting symbol provided by sl-StartingSymbolSecond according to the detected SCI formats 2-A, 2-B, and 2-C, and associated PSSCH resource configuration configured by higher layers, if sl-StartingSymbolFirst and sl-StartingSymbolSecond are provided. |
| vivo | NO | Does not see the need to capture the R16 behaviour. |
| LGE | Alt 2+4 | According to the PSSCH decoding part, even if there is a single starting symbol, the specification says “can decode”. For consistency, we are fine with having Alt 2. Meanwhile, we also need to update PSCCH monitoring for 2 candidate starting symbols to avoid ambiguity. |
| Qualcomm | comments | The difference between Alt 2 & 3 seems to be the “can decode” and “attempt to decode”. Tend to prefer “can decode” For Alt. 1, we don’t really see the confusion even if we don’t add the “according to the detected SCI formats 2-A, 2-B, and 2-C”, but we are OK to discuss it further. |
| OPPO | Alt 1 | We think “ in a resource pool” is not needed, it is very clear that PSCCH/PSSCH decoding is within a resource pool. Regarding “according to the detected SCI formats 2-A, 2-B, and 2-C”, we think it is necessary. In the above paragraphs, “according to the detected SCI formats 2-A, 2-B, 2-C and 2-D” is used in the description. While SCI format 2-D which is introduced for SL pos, is not applicable to SL-U, therefore, it is necessary to clarify that the PSSCH decoding is according the detected SCI formats 2-A, 2-B, 2-C. |

### Issue#3-2: Guardband PRB handling for contiguous RB based resource allocation

Background:

Summary

  - ZTE R1-2406680 gave a draft CR for correction on contiguous RB based resource allocation in TS 38.214, where ZTE proposes to clarify that in case of more than 1 sub-channel is used for PSSCH transmission, when the highest sub-channel(s) of PSSCH overlaps with intra-cell guard band PRBs only, the UE can transmit PSSCH on the PRBs belonging to the allocated sub-channel(s) except for the intra-cell guard band PRBs overlapping with the highest allocated sub-channel(s).

FL’s view

  - A proposal is given to reflect the above.

Proposal 3-2

Draft CR#3-2 in Section 4.8 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

| Company | Agree? | Comments |
| --- | --- | --- |
| DCM | Yes | But a clearer text may be better. |
| NEC | No | Current TS is clear that the PRBs belongs to guard band PRB region cannot be used for PSSCH. |
| CATT/CICTCI |  | Unclear whether the illustrated case is a common case. |
| vivo | NO | Current text is clear enough. |
| Qualcomm | No | Seems the current text is OK |

### Issue#3-3: Resource allocation in frequency domain in TS 38.214

Background:

Summary

  - ZTE R1-2406681 gave a draft CR for correction on resource allocation in frequency domain in TS 38.214, where ZTE proposes to remove following redundant part in section 8.1.2.2.

    - “If the highest sub-channel(s) of a PSSCH would overlap with a single RB set and the highest PRB in the highest sub-channel overlaps with intra-cell guardband PRBs, or if the highest sub-channel(s) of a PSSCH would overlap with intra-cell guardband PRBs only, the RBs corresponding to the intra-cell guard band PRBs in the highest sub-channel(s) are not available for the PSSCH.”

FL’s view

  - A proposal is given to reflect the above.

Proposal 3-3

Draft CR#3-3 in Section 4.9 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

| Company | Agree? | Comments |
| --- | --- | --- |
| NEC | No | No issues to keep it. |
| CATT/CICTCI | Yes | Indeed redundant part. |
| vivo | No |  |
| LGE | No | In our understanding, the current specification already covers such a case as well. That’s why we put “(s)”. |
| Qualcomm | No |  |

## Issue#4: PSFCH and SL-HARQ

### Issue#4-1: PSFCH resource mapping for contiguous RB resource pool in TS 38.213

Background:

Summary

  - ZTE R1-2406677 gave a draft CR for correction on PSFCH resource mapping for contiguous RB resource pool in TS 38.213, where ZTE proposes to add a limiting that only the subchanels whose all PRBs are located in RB set k are considered for PSFCH resource allocation.

  - Huawei R1-2406987 also analyse this issue and gave same text proposal.

FL’s view

  - A proposal is given to reflect the above.

Proposal 4-1

Draft CR#4-1 in Section 4.7 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

| Company | Agree? | Comments |
| --- | --- | --- |
| NEC | Agree | In our understanding, as a result, the specifical sub-channel A are not mapped to any PSFCH resource, the PSFCH can be associated with the adjacent full sub-channel B. (A will be used with B together for PSSCH) |
| CATT/CICTCI | Yes |  |
| vivo | No | The existing spec may not be optimal, i.e., some sub-channels without linking to possible PSFCH is still mapped, but anyway it is workable. |
| LGE | OK |  |
| Qualcomm | Yes |  |
| OPPO | Yes |  |

### Issue#4-2: PSFCH power control

Background:

Summary

Following is copied from RAN1#117 chair’s note: Down-select one of following: - Alt1: Draft CR#4-3A in R1-2405393 is endorsed for TS 38.214. - Alt2: Draft CR#4-3B in R1-2405394 is endorsed for TS 38.214. - Alt3: Draft CR#4-3A in R1-2405393 and Draft CR#4-3B in R1-2405394 are not pursued in Rel-18. Conclusion: Discuss at the next meeting.

  - NEC R1-2406535, Huawei R1-2405844 support Alt 1 above

    - NEC: “Support Alt1: Draft CR#4-3A in R1-2405393 is endorsed for TS 38.214. Otherwise, it should be clarified that NTx,PSFCH equals i=1KMi.”

  - OPPO/ZTE in R1-2406216 proposes a draft CR which is similar to Alt 2 above.

    - It seems the difference between OPPO/ZTE’s draft CR in R1-2406216 and Alt 2 above is that: Alt 2 above has following 3 sentences in red box

      - @OPPO/ZTE/all: in R1-2406216, due to lack of following 3 sentences in red box, it seems the relevant parameters  NPSFCH,oneinterlace1 and NPSFCH,one,kinterlace1 are not correctly defined? Please check.

FL’s view

  - A proposal is given to reflect the above.

  - Note: This word document corrupted multiple times when I tried to incorporated draft CR#4-2A and 4-2B into it (I assume they are just too large and there are too many math symbols). Thus, I prepared separate documents for these two draft CRs, please check ftp draft folder to find them.

Proposal 4-2

Down-select one of following:

Alt 1: Draft CR#4-2A in Section 4.10 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

Alt 2: Draft CR#4-2B in Section 4.11 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

| Company | Agree? | Comments |
| --- | --- | --- |
| NEC | Alt.1 |  |
| CATT/CICTCI | Alt.1 |  |
| vivo | No | For Alt1, the spec only requires the UE to select at least max1,i=1KMi PSFCH, in which case the current spec is correct. The CR is to optimize the PC for the case that UE select more PSFCHs that locate in another orphan RB set, which is not an essential issue. Anyway if at the end the Tx power is exceeding the max tx power, the UE can anyway drop some of them until the number is equal to max1,i=1KMi. For Alt2, the removal is not needed. The current spec is not wrong. Removing the text actually make the spec confusing, as the P_psfch,k(i) is introduced but not defined. |
| LGE | Alt 2 | As we know, for common interlace PSFCH transmission, P_psfch,k(i) does not have any meaning and it is not used for the actual PSFCH TX power control. According to the specification, we have another paragraph to allocate a power on common PRB and dedicated PRB. For Alt 2, we may further modify following text. A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH or a power on each PRB for PSFCH transmission(s), on all the resource pools in PSFCH transmission occasion i on active SL BWP b of carrier f as. |
| OPPO | Alt 2 | In case of sl-TransmissionStructureForPSFCH = ‘commonInterlace’ , only the total transmission power of PRB of common interlace can be determined, while not for each PSFCH, therefore, it is preferred to remove the description of PPSFCH,ki. We are OK to add the red part suggested in the FL’s analysis. |

## Issue#5: S-SSB and synchronization

### Issue#5-1: S-SSB Transmission in Non-Anchor RB Set

Background:

Summary

  - Samsung R1-2406634 gave a draft CR for correction of S-SSB Transmission in Non-Anchor RB Set, where Samsung proposes to add S-SSB transmission in non-anchor RB set when anchor RB set is outside the resource pool.

  - Huawei R1-2405845 Section 2.2.1 points out no need for specification change with following analysis:

    - Huawei: We noted in existing specification, it’s stated that a UE attempts to transmit at least S-SS/PSBCH blocks in the slots including S-SS/PSBCH blocks in the anchor RB set regardless the anchor RB set is within its resource pool or not, and UE behavior for transmitting S-SSB on non-anchor RB set is not precluded. Therefore, the issue is already addressed from existing specification and no specification impact is needed.

FL’s view

  - A proposal is given to reflect the above.

Proposal 5-1

Down-select one of following:

Alt1: Draft CR#5-1 in Section 4.6 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

Alt2: Draft CR#5-1 in Section 4.6 of R1-24xxxxx (to be FLS Tdoc#) is not pursued in Rel-18.

| Company | Agree? | Comments |
| --- | --- | --- |
| NEC |  | Neutral, tend to agree Huawei's view |
| vivo | OK | OK to discuss. We see the motivation to solve this issue. |
| LGE |  | In our understanding, it will try to allow the case the UE transmit SSB on the non-anchor PRB without transmitting SSB on the anchor PRB. If is it correct understanding, it will cause additional specification work such as TX power control mechanism where the power on the anchor PRB is used to decide the power on the non-anchor PRB. Moreover, to me, if we allow this case, it seems like that we support another form of the garbage signal. If it does not intend to transmit S-SSB on non-anchor PRB without S-SSB TX on anchor PRB, we can discuss it. |

## Issue#6: Others

No issues.

If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |

# Collection of Proposals

## [Closed] Proposals before Monday online

FL’s comment:

- To help delegates have an overview, the following is a collection of Proposals. Online session will probably go with this order.

- Please provide comment in the above box for each proposal.

  - If you want to propose draft CR, please also give all 3 CR fields, i.e., “reason for change”, “summary of changes”, “consequences if not approved”. Otherwise, companies may feel it’s hard to understand and FL may not add it in next update.

==

Proposal 1-1

Draft CR#1-1 in Section 4.5 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Proposal 4-1

Draft CR#4-1 in Section 4.7 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

Proposal 4-2

Down-select one of following:

Alt 1: Draft CR#4-2A in Section 4.10 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

Alt 2: Draft CR#4-2B in Section 4.11 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

Proposal 3-1

Down-select one of following:

Alt 1: Draft CR#3-1A in Section 4.1 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Alt 2: Draft CR#3-1B in Section 4.2 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Alt 3: Draft CR#3-1C in Section 4.3 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Alt 4: Draft CR#3-1D in Section 4.4 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Proposal 3-2

Draft CR#3-2 in Section 4.8 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Proposal 3-3

Draft CR#3-3 in Section 4.9 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Proposal 5-1

Down-select one of following:

Alt1: Draft CR#5-1 in Section 4.6 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

Alt2: Draft CR#5-1 in Section 4.6 of R1-24xxxxx (to be FLS Tdoc#) is not pursued in Rel-18.

## [Closed] Proposals for Monday online

Proposal 1-1

Draft CR#1-1 in Section 4.5 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

FL’s note about above proposal:

- Support: NEC, CATT, LGE, OPPO

- Not support: Docomo, [vivo]

  - Docomo does not see the difference, and Vivo feels the CR is too lengthy.

  - @CATT, please have some offline with Docomo and Vivo and improve the CR if needed.

Proposal 4-1

Draft CR#4-1 in Section 4.7 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

FL’s note about above proposal:

- Support: NEC, CATT, LGE, QC, OPPO

- Not support: vivo

  - Vivo feels the CR is not necessary since current spec still works.

  - @ZTE, please have some offline with Vivo and improve the CR if needed.

Proposal 4-2

Down-select one of following:

Alt 1: Draft CR#4-2A in R1-2407245 is endorsed for TS 38.213.

Alt 2: Draft CR#4-2B in R1-2407246 is endorsed for TS 38.213.

FL’s note about above proposal:

- Support Alt1: NEC, CATT.

- Support Alt2: LGE, OPPO.

- Draft CR in Alt 2 is updated to include the sentences in red box in Section 2.4.2, based on FL’s analysis and OPPO’s comment in Section 2.4.2,

- Vivo feels both Alts are not needed.

- LGE gave following red change on Alt 2. But FL feels it may not be correct to have following red sentence since PPSFCH,k(i) for a PSFCH transmission k, not for a PRB.

Proposal 3-1-1

Down-select one of following:

Alt 1: Draft CR#3-1A in Section 4.1 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Alt 2: Draft CR#3-1B in Section 4.2 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Alt 3: Draft CR#3-1C in Section 4.3 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Alt X: Draft CR#3-1X in Section 4.12 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

FL’s note about above proposal:

- FL adds Alt X by merging Alt 1, 2, 3. FL suggests Alt X. Alt 1, 2, 3, X are copied below for quick check.

- As per Docomo’s reply, FL puts Alt 4 as a separate proposal 3-1-2

- Alt 1:

- Alt 2:

- Alt 3:

- Alt X:

Proposal 3-1-2

Alt 4: Draft CR#3-1D in Section 4.4 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

Proposal 3-2

Down-select one of following:

- Alt1: Draft CR#3-2 in Section 4.8 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

- Alt2: Draft CR#3-2 in Section 4.8 of R1-24xxxxx (to be FLS Tdoc#) is not pursued in Rel-18.

FL’s note about above proposal:

- Support draft CR: Docomo (prefers a clearer text)

- Not support draft CR: NEC, vivo, QC

- Unsure: CATT (may not be common case)

- @ZTE: Please try to have some offline with above companies to improve the CR if needed.

Proposal 3-3

Down-select one of following:

- Alt1: Draft CR#3-3 in Section 4.9 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

- Alt2: Draft CR#3-3 in Section 4.9 of R1-24xxxxx (to be FLS Tdoc#) is not pursued in Rel-18.

FL’s note about above proposal:

- Support: CATT

- Not support: NEC, vivo, LGE, QC

- @ZTE: Please try to have some offline with above companies to improve the CR if needed.

Proposal 5-1

Down-select one of following:

Alt1: Draft CR#5-1 in Section 4.6 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

Alt2: Draft CR#5-1 in Section 4.6 of R1-24xxxxx (to be FLS Tdoc#) is not pursued in Rel-18.

FL’s note about above proposal:

- Alt1:

- Alt2: HW, NEC,

- Ok to discuss: vivo

- Needs some clarification: LGE

- @Samsung: please try to have some offline with above companies to improve the CR if needed.

==

If you have any comments about proposals above, please provide in box below.

| Company | Comments |
| --- | --- |

## [Closed] Proposals before Thursday online

Proposal 4-1

Draft CR#4-1 in Section 4.7 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

FL’s note about above proposal:

- Support: NEC, CATT, LGE, QC, OPPO, Samsung

- Not support: vivo

  - Vivo feels the CR is not necessary since current spec still works.

  - @ZTE, please have some offline with Vivo and improve the CR if needed.

Proposal 4-2

Down-select one of following:

Alt 1: Draft CR#4-2A in R1-2407245 is endorsed for TS 38.213.

Alt 2: Draft CR#4-2B in R1-2407246 is endorsed for TS 38.213.

FL’s note about above proposal:

- Support Alt1: NEC, CATT, Samsung

- Support Alt2: LGE, OPPO.

- Draft CR in Alt 2 is updated to include the sentences in red box in Section 2.4.2, based on FL’s analysis and OPPO’s comment in Section 2.4.2.

- Vivo feels both Alts are not needed.

- LGE gave following red change on Alt 2. But FL feels it may not be correct to have following red sentence since PPSFCH,k(i) for a PSFCH transmission k, not for a PRB.

Proposal 3-2

Down-select one of following:

- Alt1: Draft CR#3-2 in Section 4.8 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

- Alt2: Draft CR#3-2 in Section 4.8 of R1-24xxxxx (to be FLS Tdoc#) is not pursued in Rel-18.

FL’s note about above proposal:

- Support draft CR: Docomo (prefers a clearer text), Samsung

- Not support draft CR: NEC, vivo, QC

- Unsure: CATT (may not be common case)

- @ZTE: Please try to have some offline with above companies to improve the CR if needed.

==

If you have any comments about proposals above, please provide in box below.

| Company | Comments |
| --- | --- |
| Samsung | Proposal 4-1: No matter Understanding 1 or Understanding 2, some clarification is needed in the spec. Proposal 4-2: We don’t need to delete the equation (which is not broken actually), and only fixing the typo from previous CR, so we prefer Alt 1. Proposal 3-2: We support the draft CR. |

## [Closed] Proposals for Thursday online

Proposal 4-2

Down-select one of following:

Alt 1: Draft CR#4-2A in R1-2407245 is endorsed for TS 38.213.

Alt 2: Draft CR#4-2B in R1-2407246 is endorsed for TS 38.213.

FL’s note about above proposal:

- After discussions among FL, LGE, ZTE, it seems Alt 2 has some small issue, i.e., for common interlace design, if we remove the equation for determining PPSFCH,k(i), then PPSFCH,k(i) is undefined for common interlace design and the following part in red box needs to be fixed. So FL suggests to go with Alt 1 and fix the “max1,i=1KMi --> NTx,PSFCH” issue first. The issue of removing the equation for determining PPSFCH,k(i) can be considered later if needed.

Proposal 4-1

Draft CR#4-1 in Section 4.7 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.213.

FL’s note about above proposal:

- Support: NEC, CATT, LGE, QC, OPPO, Samsung

- Not support: vivo

- Based on Monday online, it’s clear that different companies have different understanding of the spec, so it’s better to have a CR to fix the spec. So far, no new CRs are received so the proposal remains unchanged.

Proposal 3-2 (may postpone to next meeting)

Down-select one of following:

- Alt1: Draft CR#3-2 in Section 4.8 of R1-24xxxxx (to be FLS Tdoc#) is endorsed for TS 38.214.

- Alt2: Draft CR#3-2 in Section 4.8 of R1-24xxxxx (to be FLS Tdoc#) is not pursued in Rel-18.

FL’s note about above proposal:

- Support draft CR: Docomo (prefers a clearer text), Samsung

- Not support draft CR: NEC, vivo, QC

- Unsure: CATT (may not be common case)

- After discussion between ZTE and FL, it’s not very clear this is a common case or corner case. So probably RAN1 can postpone the CR and companies can further check, e.g., do some calculation considering sub-channel size, RB set size, and guardband size, to see whether this is common case or corner case.

# Collection of Draft CRs

## [Closed, endorsed 3-1B] Draft CR#3-1A for “Issue#3-1: PSSCH decoding behavior”

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | Rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PSSCH decoding behavior |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If sl-startingSymbolFirst and sl-startingSymbolSecond are provided and there is no PSFCH resource within a slot, UE can decode PSSCH starting from the second candidate starting symbol only when associated SCI formats 2-A, 2-B, 2-C and 2-D are correctly detected. Current description “UE attempt to decode PSSCH transmission…” is not clear. It seems that no matter whether there is associated SCI formats 2-A, 2-B, 2-C and 2-D detected, UE needs to decode PSSCH transmission starting from the second candidate starting symbol. Furthermore, “attempt to” is changed to “can” which follow the desctiption in above paragraphs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the condition for UE to perform decoding PSSCH transmission starting from the second candidate starting symbol based on associated SCI formats 2-A, 2-B, 2-C and 2-D detection results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE need to decode PSSCH transmission even when there is no SCI formats 2-A, 2-B, 2-C and 2-D detected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

8.3	UE procedure for receiving the physical sidelink shared channel

For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

A UE is required to decode neither the corresponding SCI formats 2-A, 2-B, 2-C, 2-D nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support.

In any slot without PSFCH symbols, the UE can, subject to UE capability, decode PSSCH transmission starting from the second candidate starting symbol provided by sl-StartingSymbolSecond according to the detected SCI formats 2-A, 2-B, and 2-C, and associated PSSCH resource configuration configured by higher layers, if sl-StartingSymbolFirst and sl-StartingSymbolSecond are provided.

<Unchanged parts are omitted>

## [Endorsed] Draft CR#3-1B for “Issue#3-1: PSSCH decoding behavior”

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.214 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PSSCH transmission decode behaviour in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1.UE only decode PSSCH transmission in a slot within a resource pool, not any slot. 2.if sl-startingSymbolFirst and sl-startingSymbolSecond are provided, for SL slot without PSFCH symbols, both the first candidate starting symbol provided by sl-startingSymbolFirst and the second candidate starting symbol provided by sl-startingSymbolSecond can be used to decode PSSCH transmission, this is not captured in current spec. - ‘attempts to decode PSSCH ’ behaviour is not aligned with legacy R16 PSSCH receiving ‘can decode PSSCH ’ behaviour |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Limit that UE would decode PSSCH transmission starting from the second candidate starting symbol in any slot without PSFCH symbols within a resource pool. 2.Capture the missing part of PSSCH decode according to the first candidate starting symbol provided by sl-startingSymbolFirst for SL slot without PSFCH symbols. 3. change ‘attempts to decode’ behaviour to ‘can decode’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - UE may decode PSSCH transmission in a S-SSB slot or non sidelink slot. - It is not clear how to determine the first candidate starting symbol for SL slot without PSFCH symbols in RAN1. - R18 PSSCH receive behaviour is not aligned with legacy R16 behaviour . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

8.3	UE procedure for receiving the physical sidelink shared channel

For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

A UE is required to decode neither the corresponding SCI formats 2-A, 2-B, 2-C nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support.

In any slot without PSFCH symbols within a resource pool, the UE can decode PSSCH transmission starting from the first candidate starting symbol provided by sl-startingSymbolFirst, and can decode, subject to UE capability, PSSCH transmission starting from the second candidate starting symbol provided by sl-startingSymbolSecond , if sl-startingSymbolFirst and sl-startingSymbolSecond are provided.

<Unchanged parts are omitted>

## [Closed, endorsed 3-1B] Draft CR#3-1C for “Issue#3-1: PSSCH decoding behavior”

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PSSCH transmission decode behaviour in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - UE only decodes PSSCH transmission in a slot within a resource pool, not any slot. This is not reflected in the spec. - If sl-startingSymbolFirst and sl-startingSymbolSecond are provided, for SL slot without PSFCH symbols, the first candidate starting symbol provided by sl-startingSymbolFirst and the second candidate starting symbol provided by sl-startingSymbolSecond can be used for PSSCH transmission, and thus UE needs to decode them. This is not captured in current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Limit that UE would decode PSSCH transmission starting from the second candidate starting symbol in any slot without PSFCH symbols within a resource pool. - Capture the missing part of PSSCH decode according to the first candidate starting symbol provided by sl-startingSymbolFirst for SL slot without PSFCH symbols. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - UE may decode PSSCH transmission in a S-SSB slot or non sidelink slot. - It is not clear how to determine the first candidate starting symbol for SL slot without PSFCH symbols in RAN1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Impact Analysis: No backward compatible issue is expected from the CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

8.3	UE procedure for receiving the physical sidelink shared channel

For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

A UE is required to decode neither the corresponding SCI formats 2-A, 2-B, 2-C, 2-D nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support.

In any slot without PSFCH symbols within a resource pool, the UE attempts to decode PSSCH transmission starting from the first candidate starting symbol provided by sl-startingSymbolFirst, and attempts to decode, subject to UE capability, PSSCH transmission starting from the second candidate starting symbol provided by sl-StartingSymbolSecond, if sl-StartingSymbolFirst and sl-StartingSymbolSecond are provided.

<Unchanged parts are omitted>

## [Closed, not pursued] Draft CR#3-1D for “Issue#3-1: PSSCH decoding behavior”

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on sensing with two starting symbols |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the clause 8.1.4 of 38.214, sensing behavior is described for mode 2 resource allocation. Which slot shall be monitored is determined in step 2 for each of full sensing and partial sensing. However, detailed UE behavior in case that two starting symbols are (pre-)configured is not described in the clause and any other specification. In R18 SL, two starting symbols are available at each slot and thus there are two PSCCH occasions in such a slot. In this case, some UE may monitor PSCCH from both the first starting symbol and the second starting symbol, but other UE may monitor PSCCH from only the first starting symbol by UE implementation. Expected UE behavior should be clarified, and TX UE should not skip monitoring any PSCCH occasion in identified monitoring slots. Note that for cases where a single starting position is available, there is no ambiguity. When monitoring slots are determined, UE shall monitor PSCCH at the slots as described in step 2. For example, in full sensing case, PSCCH shall be monitored in every slot within [n –T0,n–Tproc,0SL). Exceptions are only TX/RX overlapping cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarifed that UE performs monitoring PSCCH starting from both the first starting symbol and the second starting symbol in non-PSFCH slot, if two starting symbols are (pre-)configured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may skip monitoring PSCCH from the second starting symbol. Reservation information transmitted by other UE from the second starting symbol is missed in this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

8.1.4   UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts are omitted>

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×Preserve''SL, where t'ySL is a slot of the selected candidate slots and Preserve' is Preserve converted to units of logical slot according to clause 8.1.7. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

The value of Preserve corresponds to sl-PBPS-OccasionReservePeriodList if (pre-)configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList.

The UE monitors sensing occasion(s) determined by sl-Additional-PBPS-Occasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than t'y0SL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured. t'y0SL is the first slot of the selected Y candidate slots of PBPS.

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled and Prsvp_TX≠0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot t'y0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. The value of M is (pre-)configured with the sl-CPS-WindowPeriodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowPeriodic is not (pre-)configured, M equals to 31.

When the UE performs at least contiguous partial sensing and if Prsvp_TX=0, the contiguous partial sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at least M consecutive logical slots before t'y0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than t'y0SL, where t'y0SL is the first slot of the selected Y'candidate slots. The value of M is (pre-)configured with the sl-CPS-WindowAperiodic. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. If sl-CPS-WindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when PrsvpTX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter sl-PartialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given periodicity corresponding to Preserve, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots.

In the monitoring slots without PSFCH symbols, the UE shall monitor PSCCH transmissions starting from the first candidate starting symbol provided by sl-startingSymbolFirst, and shall monitor PSCCH transmissions starting from the second candidate starting symbol provided by sl-startingSymbolSecond, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided.

<Unchanged parts omitted>

## [Endorsed] Draft CR#1-1 for “Issue#1-1: Determination of sidelink symbol for SL-U”

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | draft | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on the determination of sidelink symbol for SL-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, CICTCI |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the sidelink symbol determination in SL-U, If two candidate starting symbols for PSCCH/PSSCH transmission are supported, the sidelink symbols should be determined by higher layer parameter sl-StartingSymbolFirst rather than sl-StartSymbol. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 8.1.2.1, clarify that the sidelink symbols are determined by higher layer parameter sl-StartingSymbolFirst and sl-LengthSymbols if sl-StartingSymbolFirst and sl-StartingSymbolSecond are provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to determine the sidelink symbols when two candidate starting symbols are supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.2.1	Resource allocation in time domain

The UE shall transmit the PSSCH in the same slot as the associated PSCCH.

The minimum resource allocation unit in the time domain is a slot.

The UE shall transmit the PSSCH in consecutive symbols within the slot, subject to the following restrictions:

-	The UE shall not transmit PSSCH in symbols which are not configured for sidelink. A symbol is configured for sidelink, according to higher layer parameters sl-StartSymbol and sl-LengthSymbols, where sl-StartSymbol is the symbol index of the first symbol of sl-LengthSymbols consecutive symbols configured for sidelink, except when sl-StartingSymbolFirst and sl-StartingSymbolSecond are provided for a SL-BWP. If sl-StartingSymbolFirst and sl-StartingSymbolSecond are provided for the SL-BWP, a symbol is configured for sidelink, according to higher layer parameters sl-StartingSymbolFirst and sl-LengthSymbols, where sl-StartingSymbolFirst is the symbol index of the first symbol of sl-LengthSymbols consecutive symbols configured for sidelink.

-	Within the slot, PSSCH resource allocation starts at symbol sl-StartSymbol+1, except when sl-StartingSymbolFirst and sl-StartingSymbolSecond are provided for a SL-BWP. If sl-StartingSymbolFirst and sl-StartingSymbolSecond are provided for the SL-BWP, there are 2 candidate starting symbols, given by sl-StartingSymbolFirst and sl-StartingSymbolSecond respectively, for PSSCH transmission for slots without PSFCH symbols; and there is one starting symbol, given by sl-StartingSymbolFirst, for PSSCH transmission for slots with PSFCH symbols. PSSCH resource allocation starts at the next symbol after each candidate starting symbol. In a slot, the UE may use the second candidate starting symbol, provided by sl-StartingSymbolSecond, only if it fails to access the channel prior to the first candidate starting symbol provided by sl-StartingSymbolFirst.

<<< UNCHANGED PARTS OMITTED >>>

## [Endorsed modified version] Draft CR#5-1 for “Issue#5-1: S-SSB Transmission in Non-Anchor RB Set”

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Correcting S-SSB Transmission in Non-Anchor RB Set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the case that the anchor RB set is outside the RB set(s) associated with the resource pool, transmitting S-SSB in anchor RB set may not be efficient. For example, consider the scenario in the below figure, wherein RB set #0 is the anchor RB set, and RB set #1 and #2 are non-anchor RB sets and associated with the resource pool. If a UE initiates a CO from slot #s-1 for PSSCH transmission, and in slot #s+1 to continue the transmission within the resource pool and CO, wherein slot #s is for S-SSB transmission, then based on current specification, the UE shall at least attempt to transmit S-SSB in the anchor RB set (i.e., RB set #0), and since the CO is only for RB set #1 and #2, the UE has to initiate a new CO over at least RB set #0 to attempt to transmit the S-SSB in the anchor RB set, then for slot #s+1, the UE needs to switch back to RB set #1 and #2 to initiate a new CO for the continued PSSCH transmission. In this sense, in order to transmit S-SSB in the anchor RB set, the UE may not be able to maintain the CO, which makes it less meaningful to support S-SSB in non-anchor RB set. One way to resolve or mitigate this issue could be up to RAN2 discussion – try to (pre-)configure the anchor RB set to be within the resource pool, but such scheme may not always work since the anchor RB set is defined per SL BWP, while the resource pool can semi-statically change (e.g., due to consistent LBT failures). From RAN1 perspective, for the case anchor RB set is outside the RB set(s) associated with the resource pool, S-SSB on non-anchor RB set(s) only shall also be supported, and it’s up to UE’s implementation to choose to maintain the CO or transmit S-SSB on the anchor RB set. Meanwhile, since current specification on power control of S-SSB is from configuration point of view, i.e., no restriction that S-SSB(s) on anchor RB set has to be transmitted, then there is no need to modify current specification for power control and the existing mechanism can work well with the draft CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add S-SSB transmission in non-anchor RB set when anchor RB set is outside the resource pool. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not clear for UE behavior of transmitting S-SSB when anchor RB set is outside the resource pool. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Impact Analysis: No backward compatible issue is expected from the CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

16.1		Synchronization procedures

============================= Unchanged Text Omitted =================================

For operation with shared spectrum channel access, a UE attempts to transmit at least S-SS/PSBCH blocks in the slots including S-SS/PSBCH blocks in the anchor RB set. The UE may alternatively attempt to transmit S-SS/PSBCH blocks only in the non-anchor RB set(s) associated with the resource pool, if any, if the anchor RB set is not included in the RB set(s) associated with the resource pool, and the channel access procedure [15, TS 37.23] for transmitting the S-SS/PSBCH blocks fails on the channel including the anchor RB set. The UE applies CP extension to the first symbol of an S-SS/PSBCH block and within the first one or two symbols before the first symbol of the S-SS/PSBCH block according to an index [4, TS 38.211] provided by sl-CPE-StartingPositionS-SSB. The UE assumes PRB(s) in an intra-cell guard band [6, TS 38.214] are not used for transmission of S-SS/PSBCH blocks.

============================= Unchanged Text Omitted =================================

For operation with shared spectrum channel access and for each slot that includes S-SS/PSBCH blocks, a UE is provided, by sl-NumOfAdditionalSSSBOccasion, a number NadditionalS-SSB of additional candidate S-SS/PSBCH block transmission occasions. When the UE determines to transmit S-SS/PSBCH blocks on additional candidate S-SS/PSBCH block transmission occasions, the UE attempts to transmit S-SS/PSBCH blocks at least in the anchor RB set. The UE may alternatively attempt to transmit S-SS/PSBCH blocks only in the non-anchor RB set(s) associated with the resource pool, if any, if the anchor RB set is not included in the RB set(s) associated with the resource pool, and the channel access procedure [15, TS 37.23] for transmitting the S-SS/PSBCH blocks fails on the channel including the anchor RB set. When NadditionalS-SSB>0, for S-SS/PSBCH block with index iS-SSB, the UE determines indexes of slots that include the additional candidate S-SS/PSBCH block transmission occasions as NoffsetS-SSB+NintervalS-SSB+1⋅iS-SSB +( NgapS-SSB+1)⋅(iS-SSB+1), where

-	NgapS-SSB is a slot gap, provided by sl-GapOfAdditionalSSSB-Occasion, for determining the additional candidate S-SS/PSBCH block transmission occasions, and

-	iS-SSB is an index of the additional candidate S-SS/PSBCH block transmission occasions, with 0≤iS-SSB≤NadditionalS-SSB-1.

============================= Unchanged Text Omitted =================================

## Draft CR#4-1 for “Issue#4-1: PSFCH resource mapping for contiguous RB resource pool in TS 38.213”

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.213 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PSFCH resource mapping for contiguous RB resource pool in TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For operation with shared spectrum channel access, when sl-PSFCH-Type is not provided, the description of the ‘subchannel in RB set k’ is not clear in the current TS 38.213. As shown in the following figure, only partial PRBs of subchannel i is within RB set k. It is unclear whether or not subchannel i was included in RB-set k for PSSCH-PSFCH mapping in the current TS 38.213. There are two kinds of understandings: - Understanding 1: Subchannel i was included in RB set k for PSSCH-PSFCH mapping; - Understanding 2: Subchannel i was not included in RB set k for PSSCH-PSFCH mapping; It is suggested that RAN1 clarify which of the above understandings should be adopted in the TS 38.213. And in RAN1#113, it was agreed: Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, support only option 3. - Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission So it is reasonable to not allocate PSFCH resource in RB set k to the special subchannels which are partially overlapped with RB set k. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a limiting that only the subchanels whose all PRBs are located in RB set k are considered for PSFCH resource allocation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the change is not approved, whether and how to allocate PSFCH resource in RB set k to the special subchannels which are partially overlapped with RB set k is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

16.3.0	UE procedure for transmitting PSFCH with control information

<Unchanged parts are omitted>

For operation with shared spectrum channel access, when sl-PSFCH-Type is not provided and within RB-set k, for the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, a UE determines a set of MPRB, set,  kPSFCH, n PRBs in a resource pool based on the n-th indication provided by sl-PSFCH-RB-SetList or sl-RB-SetPSFCHList for PSFCH transmission with HARQ-ACK information or conflict information, respectively. The UE expects that different PRBs are (pre)configured for conflict information and HARQ-ACK information. For a number of Nsubchk sub-channels where all PRBs of each sub-channel are located in RB-set k and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, set,  kPSFCH, n PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubchk⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, set,  kPSFCH, n is a multiple of Nsubchk∙NPSSCHPSFCH.

<Unchanged parts are omitted>

## Draft CR#3-2 for “Issue#3-2: Guardband PRB handling for contiguous RB based resource allocation”

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.214 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on guardband PRB handling for contiguous RB based resource allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB', and if more than 1 sub-channel is used for PSSCH transmission, when the highest sub-channel(s) of PSSCH overlaps with intra-cell guard band PRBs only, it could be ambiguous on which PRBs the UE can transmit PSSCH. Following current specification, “when the highest sub-channel(s) of PSSCH overlaps with intra-cell guard band PRBs only, the UE can transmit PSSCH on the PRBs belonging to the allocated sub-channel(s) except for the intra-cell guard band PRBs overlapping with the highest sub-channel(s).” it seems the two highlighted parts of “the highest sub-channel(s)” indicate the same sub-channel(s), it could be a misunderstanding that only the PRBs in the highest subchannel should be excluded for PSSCH transmission. But actually, the intra-cell guard band PRBs overlapping with the highest two sub-channels should be excluded, as the PRBs marked in shadow in below figure. RB set 2 Guard band#2 RB set 3 Guard band#1 RB set 1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that in case of more than 1 sub-channel is used for PSSCH transmission, when the highest sub-channel(s) of PSSCH overlaps with intra-cell guard band PRBs only, the UE can transmit PSSCH on the PRBs belonging to the allocated sub-channel(s) except for the intra-cell guard band PRBs overlapping with the highest allocated sub-channel(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case that the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB', and more than 1 sub-channel is used for PSSCH transmission, when the highest sub-channel(s) of PSSCH overlaps with intra-cell guard band PRBs only, UE may misuse some of the PRBs overlaps with intra-cell guard band PRBs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8	Physical sidelink shared channel related procedures

<Unchanged parts are omitted>

For operation with shared spectrum channel access for frequency range 1, a sidelink resource pool can be (pre-)configured to include integer number of RB sets, and the lowest RB of the sidelink resource pool is aligned with the lowest RB of lowest RB set in the resource pool, and the highest RB of the sidelink resource pool is aligned with the highest RB of highest RB set in the resource pool. A UE can be provided with intra-cell guard bands according to the higher layer parameter sl-IntraCellGuardBandsSL-List or according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] when the higher layer parameter intraCellGuardBandsSL-List is not configured. The intra-cell guard band PRBs between any two adjacent RB sets can be used only for PSSCH transmission, if and only if, the UE has successfully performed channel access procedure in both adjacent RB sets, and the UE uses both of these RB sets for PSSCH transmission. If the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB', and if more than 1 sub-channel is used for PSSCH transmission, when the highest sub-channel of PSSCH overlaps with a single RB set and the highest PRB in the highest sub-channel overlaps with intra-cell guard band PRBs, the UE can transmit PSSCH on the PRBs belonging to the allocated sub-channel(s) except for the intra-cell guard band PRBs within the highest sub-channel. If the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB', and if more than 1 sub-channel is used for PSSCH transmission, when the highest sub-channel(s) of PSSCH overlaps with intra-cell guard band PRBs only, the UE can transmit PSSCH on the PRBs belonging to the allocated sub-channels except for the intra-cell guard band PRBs overlapping with the allocated sub-channels and higher than the highest RB set for PSSCH transmission.

<Unchanged parts are omitted>

## [Not pursued] Draft CR#3-3 for “Issue#3-3: Resource allocation in frequency domain in TS 38.214”

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.214 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on resource allocation in frequency domain in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The description of the PRBs available for PSSCH transmission in case of the higher layer parameter transmissionStructureForPSCCHandPSSCH set to ‘contiguousRB' has been captured in section 8, the following text in section 8.1.2.2 is redundant, and inconsistent with the description in section 8 due to the loss of some conditions. If the highest sub-channel(s) of a PSSCH would overlap with a single RB set and the highest PRB in the highest sub-channel overlaps with intra-cell guardband PRBs, or if the highest sub-channel(s) of a PSSCH would overlap with intra-cell guardband PRBs only, the RBs corresponding to the intra-cell guard band PRBs in the highest sub-channel(s) are not available for the PSSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the redundant part in section 8.1.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior is not consistent in section 8 and 8.1.2.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

8.1.2.2	Resource allocation in frequency domain

The resource allocation unit in the frequency domain is the sub-channel.

The sub-channel assignment for sidelink transmission is determined using the "Frequency resource assignment" field in the associated SCI.

The lowest sub-channel for sidelink transmission is the sub-channel on which the lowest PRB of the associated PSCCH is transmitted.

For operation with shared spectrum channel access for frequency range 1, the lowest RB set for sidelink transmission is the RB set on which the lowest PRB of the associated PSCCH is transmitted.

If a PSSCH scheduled by a PSCCH would overlap with resources containing the PSCCH, the resources corresponding to a union of the PSCCH that scheduled the PSSCH and associated PSCCH DM-RS are not available for the PSSCH.

When PSSCH is transmitted on multiple RB sets, the corresponding PSCCH is located on the sub-channel with the lowest index of the RB set with the lowest index within the multiple RB sets.

For operation with shared spectrum channel access for frequency range 1, if the higher layer parameter sl-TransmissionStructureForPSCCHandPSSCH is set to 'interlaceRB',

-	the lowest index of the RB set allocation to the initial PSSCH transmission is indicated via the field "Lowest index of the RB set allocation to the initial transmission" of the DCI format 3_0.

-	the starting RB set index of the initial PSSCH transmission of the sidelink configured grant Type 1 is indicated via the higher layer parameter sl-StartRBsetCG-Type1.

<Unchanged parts are omitted>

## Draft CR#4-2A for “Issue#4-2: PSFCH power control”

See separate document on ftp.

## Draft CR#4-2B for “Issue#4-2: PSFCH power control”

See separate document on ftp.

## [Closed, endorsed 3-1B] Draft CR#3-1X for “Issue#3-1: PSSCH decoding behavior”

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | Rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PSSCH decoding behavior |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If sl-startingSymbolFirst and sl-startingSymbolSecond are provided and there is no PSFCH resource within a slot, UE can decode PSSCH starting from the second candidate starting symbol only when associated SCI formats 2-A, 2-B, 2-C and 2-D are correctly detected. Current description “UE attempt to decode PSSCH transmission…” is not clear. It seems that no matter whether there is associated SCI formats 2-A, 2-B, 2-C and 2-D detected, UE needs to decode PSSCH transmission starting from the second candidate starting symbol. Furthermore, “attempt to” is changed to “can” which follow the desctiption in above paragraphs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the condition for UE to perform decoding PSSCH transmission starting from the second candidate starting symbol based on associated SCI formats 2-A, 2-B, 2-C and 2-D detection results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE need to decode PSSCH transmission even when there is no SCI formats 2-A, 2-B, 2-C and 2-D detected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

8.3	UE procedure for receiving the physical sidelink shared channel

For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

A UE is required to decode neither the corresponding SCI formats 2-A, 2-B, 2-C, 2-D nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support.

In any slot without PSFCH symbols within a resource pool, the UE can decode PSSCH transmission starting from the first candidate starting symbol provided by sl-startingSymbolFirst, and can decode, subject to UE capability, PSSCH transmission starting from the second candidate starting symbol provided by sl-startingSymbolSecond according to the detected SCI formats 2-A, 2-B, and 2-C, and associated PSSCH resource configuration configured by higher layers, if sl-startingSymbolFirst and sl-startingSymbolSecond are provided.

<Unchanged parts are omitted>

# Conclusions

The outcome of this meeting is in Annex B.

# References

- R1-2405844	Correction on PSFCH power control	Huawei, HiSilicon

- R1-2405845	Discussions on remaining issues of R18 NR sidelink from RAN1#117	Huawei, HiSilicon

- R1-2405864	Correction on PSSCH transmission decode behaviour in TS 38.214	Huawei, HiSilicon

- R1-2406151	Clarification on COT sharing flag in 38.212	vivo

- R1-2406152	Clarification on channel occupancy sharing information in 37.213	vivo

- R1-2406213	Draft CR for correction on interlace RB-based transmission in partial sensing	OPPO

- R1-2406214	Draft CR for indication of remaining channel occupancy duration	OPPO, Qualcomm

- R1-2406215	Draft CR for correction on CAPC value for PSFCH and S-SSB	OPPO

- R1-2406216	Draft CR for correction on PSFCH power control	OPPO, ZTE, Sanechips

- R1-2406217	Draft CR for correction on PSSCH decoding behaviour	OPPO

- R1-2406336	Draft CR on the contention window adjustment procedures for SL-U	CATT, CICTCI, OPPO

- R1-2406337	Draft CR on the determination of sidelink symbol for SL-U	CATT, CICTCI

- R1-2406535	Remaining issues on PSFCH power control	NEC

- R1-2406634	Draft CR for Correcting S-SSB Transmission in Non-Anchor RB Set	Samsung

- R1-2406676	Correction on IUC in co-existence case in TS 38.214	ZTE, Sanechips

- R1-2406677	Correction on PSFCH resource mapping for contiguous RB resource pool in TS 38.213	ZTE, Sanechips

- R1-2406678	Correction on PSSCH transmission decode behaviour in TS 38.214	ZTE, Sanechips

- R1-2406679	Supplementary higher layer parameter for section 8 in TS 38.214	ZTE, Sanechips

- R1-2406680	Correction on contiguous RB based resource allocation in TS 38.214	ZTE, Sanechips

- R1-2406681	Correction on resource allocation in frequency domain in TS 38.214	ZTE, Sanechips

- R1-2406915	Draft CR on CAPC value for PSFCH+S-SSB for SL-U	NTT DOCOMO, INC.

- R1-2406916	Draft CR on multi-channel access vs PSFCH prioritization for SL-U	NTT DOCOMO, INC.

- R1-2406917	Draft CR on sensing with two starting symbols	NTT DOCOMO, INC.

- R1-2406918	Maintenance of resource selection in MAC layer for SL-U	NTT DOCOMO, INC.

- R1-2406987	Determination of PSFCH resources for a PSSCH	Huawei, HiSilicon

- R1-2406988	Correction on CPE starrting position for PSFCH	Huawei, HiSilicon

- R1-2406997	Correction on COT sharing information processing delay	Huawei, HiSilicon

# Annex A: Contact Information

Please provide/update your contact information here so that delegates can better communicate with each other if needed.

| Company | Name | Email |
| --- | --- | --- |
| Huawei | Xiang Mi | Shawn.mixiang@huawei.com |
| HiSilicon | Fan Yang | james.yangfan@huawei.com |
| LGE | Daesung Hwang Seungmin Lee | daesung.hwang@lge.com edison.lee@lge.com |
| CATT, GOHIGH | Rui Zhao Shupeng Li Xiaoran Wen | zhaorui@goghigh.com.cn lsp@catt.cn wenxiaoran@gohigh.com.cn |
| OPPO | Zhenshan ZHAO Kevin Lin | zhaozhenshan@oppo.com kevin.lin@oppo.com |
| vivo | Zichao Ji | jizichao@vivo.com |
| ZTE,Sanechips | Yuzhou Hu | hu.yuzhou@zte.com.cn |
| Spreadtrum | Mimi Chen | mimi.chen@unisoc.com |
| Intel | Salvatore Talarico | salvatore.talarico@intel.com |
| CMCC | Pengyu Ji Jingwen Zhang | jipengyu@chinamobile.com zhangjingwen@chinamobile.com |
| Panasonic | Ayako Iwata | iwata.ayako@jp.panasonic.com |
| xiaomi | Wensu Zhao Qun Zhao | zhaowensu@xiaomi.com zhaoqun1@xiaomi.com |
| NTT DOCOMO | Shohei Yoshioka | shohei.yoshioka@docomo-lab.com syouhei.yoshioka.py@nttdocomo.com |
| Sharp | Liqing Liu | liu.liqing@sharp.co.jp |
| Nokia/NSB | Naizheng Zheng | naizheng.zheng@nokia-sbell.com |
| Ericsson | Ricardo Blasco Jose Leon Calvo | name.surname at company . com jose.leon.calvo@ericsson.com |
| Transsion | Xingya Shen | xingya.shen@transsion.com |
| InterDigital | Aata El Hamss | Aata.elhamss@interdigital.com |
| Lenovo | Xiaodong Yu Karthikeyan Ganesan Alexander Golitschek | Yuxd1@lenovo.com kganesan@lenovo.com aelbwart@lenovo.com |
| Samsung | Hongbo Si | hongbo.si@samsung.com |
| Futurewei | George Calcev | gcalcev@futurewei.com |
| Qualcomm | Giovanni Chisci Chih-Hao Liu Stelios Stefanatos | gchisci@qti.qualcomm.com chihliul@qti.qualcomm.com sstefana@qti.qualcomm.com |
| NEC | Zhaobang MIAO Jin YANG | miao_zhaobang@nec.cn yangjin@labs.nec.cn |
| WILUS | Minseok Noh | minseok.noh@wilusgroup.com |
| MediaTek | Tao Chen | tao.chen@mediatek.com |
| Fraunhofer | Tom Wirth | thomas.wirth@hhi.fraunhofer.de |
| China Telecom | Jing Guo | guojing6@chinatelecom.cn |
| Sony | Kazuyuki Shimezawa | kazuyuki.shimezawa@sony.com |
| Johns Hopkins University APL | Vijitha Weerackody | vijitha.weerackody@jhuapl.edu |
| National Spectrum Consortium | Amitav Mukherjee | Amitav.Mukherjee@nationalspectrumconsortium.org |
| ETRI | Junhyeong Kim | jhkim41jf@etri.re.kr |

# Annex B: Outcomes of RAN1 meetings

The agreement list becomes very long now. To reduce the length of FLS, FL puts RAN1 agreement list for R18 NR SL-U PHY channel design until RAN1#118 into a separate Tdoc R1-2407247.