3GPP TSG RAN WG1 #118bis			R1-2409045

Hefei, China, October 14th – 18th, 2024

Title:	LS on Rel-18 RAN1 UE features list for NR after RAN1#118bis

Response to:	-

Release:	Rel-18

Work Items:	NR_MIMO_evo_DL_UL, NR_pos_enh2, Netw_Energy_NR, NR_netcon_repeater, NR_NTN_enh, NR_Mob_enh2, NR_SL_enh2, NR_redcap_enh, NR_MC_enh, NR_XR_Enh, NR_FR1_lessthan_5MHz_BW, NR_DSS_enh, NR_BWP_wor, NR_cov_enh2, TEI18

Source:	RAN WG1

To:	RAN WG2

CC:	RAN WG4

Contact Person:

Name:	Hiroki Harada, Ralf Bendlin

E-mail Address:	hiroki.harada.sv@nttdocomo.com, ralf_bendlin@labs.att.com

Attachment:	               R1-2409043.zip (Updated RAN1 UE features list for Rel-18 NR after RAN1#118bis)

1. Overall Description:

RAN1 has continued to discuss the Rel-18 RAN1 UE features list for NR and would like to share the latest version with RAN2 and RAN4 in the attachment R1-2409043.

For NR_MIMO_evo_DL_UL for FG 40-7-1g-2, RAN1 made following agreement.

Agreement: Endorse the following TP for TS 38.306 in RAN1 and send an LS to RAN2 codebookParameter8TxPUSCH-r18 Indicates whether the UE supports codebook-based 8Tx PUSCH. The UE shall include codebook-8TxBasic-r18 to indicate basic features of 8Tx PUSCH codebook. This capability signalling comprises the following parameters: - maxNumberPUSCH-MIMO-Layer-r18 defines the maximum number of PUSCH MIMO layers for codebook based PUSCH. - maxNumberSRS-Resource-r18 defines the maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook' for codebook-based 8Tx PUSCH. - srs-8TxPorts-r18 defines SRS 8 Tx ports—codebook. Value 'noTDM' indicates noTDM. Value 'both' indicates TDM and noTDM. This parameter only applies to codebook2-8TxPUSCH-r18, codebook3-8TxPUSCH-r18, and codebook4-8TxPUSCH-r18. A UE that supports codebook-8TxBasic-r18 must support of at least one of codebook1-8TxPUSCH-r18, codebook2-8TxPUSCH-r18, codebook3-8TxPUSCH-r18, and codebook4-8TxPUSCH-r18. - codebook1-8TxPUSCH-r18 comprises the following parameters: - codebookN1N4-r18 indicates whether the UE supports (N1, N2) codebook-based 8Tx PUSCH—codebook1. Value ng1n4n1 corresponds to (4,1) codebook, value ng1n2n2 corresponds to (2,2) codebook, value both corresponds to both codebooks. - srs-8TxPorts-r18 defines SRS 8 Tx ports for codebook1—codebook. Value 'noTDM' indicates noTDM. Value 'both' indicates TDM and noTDM. - codebook2-8TxPUSCH-r18 indicates whether the UE supports codebook-based 8Tx PUSCH—codebook2. - codebook3-8TxPUSCH-r18 indicates whether the UE supports codebook-based 8Tx PUSCH—codebook3. - codebook4-8TxPUSCH-r18 indicates whether the UE supports codebook-based 8Tx PUSCH—codebook4. The UE optionally indicates ul-FullPwrTransMode0-r18 to indicate whether the UE supports UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation. The UE optionally indicates ul-FullPwrTransMode1-r18 to indicate whether the UE supports UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation. The UE optionally indicates ul-FullPwrTransMode2-r18 to indicate whether the UE supports UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation. The UE indicates the maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2. NOTE 1: A UE that supports ul-FullPwrTransMode2-r18 supports at least full power operation with single port. The UE optionally indicates ul-SRS-TransMode2-r18 to indicate whether the UE supports SRS configurations with different number of antenna ports per SRS resource for mode 2. The UE indicates a 3-bit bitmap, where the leading / leftmost bit (bit 0) corresponds to whether SRS resource can be configured with 1 port. The next bit (bit 1) corresponds to whether SRS resource can be configured with 2 port. The rightmost bit (bit 2) corresponds to whether SRS resource can be configured with 4 port. A UE supporting ul-SRS-TransMode2-r18 shall also indicate support of ul-FullPwrTransMode2. NOTE 2: An SRS resource set supported by the UE for uplink full power Mode 2 must contain at least an 8 port SRS resource. NOTE 3: Any of the above values of ul-SRS-TransMode2-r18 can be used if ul-FullPwrTransMode2-r18 is reported as value n2 or n4. The UE optionally indicates tpmi-FullPwrCodebook2-r18 to indicate which TPMI group(s) delivers full power when UE is capable of and configured with 8 Tx codebook based PUSCH operation with codebook2. Value first indicates the TPMI group corresponding to only the first coherent antenna port group 0. Value second indicates the TPMI group corresponding to only the second coherent antenna port group 1. Antenna port group is defined in Table 6.3.1.5-8 of TS 38.211. A UE supporting tpmi-FullPwrCodebook2-r18 shall also indicate support of ul-FullPwrTransMode2..

RAN1 would like to thank RAN2 for the LS in R1-2407603/R2-2407601. RAN1 would like to inform RAN2 about the following RAN1 understanding.

Agreement: - Send a reply LS to RAN2 to inform the following RAN1’s understanding. - FG45-3a/4a, 45-5/5a/6 do not need to be indicated separately for intra and inter-frequency LTM cell switch. - If UE supports ltm-InterFreq-r18, the support of above FGs implies the support for inter-frequency scenarios in addition to intra-frequency scenarios. If UE only supports ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18, the support of above FGs implies the support for intra-frequency scenarios. - Same understanding is applicable to FG45-3/4 as well

2. Actions:

To RAN WG2

ACTION: RAN1 respectfully asks RAN2 to take into account above information and the RAN1 NR UE features in the attachment for designing corresponding capability signalling in Rel-18.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #119				November 18 to 22, 2024	Orlando, US

TSG-RAN WG1 Meeting #120				February 17 to 21, 2024		Athens, Greece